IALA Guideline

Gnnnn

Buoy Tender Activities

Edition 1.0

Date (of approval by Council)

urn:mrn:iala:pub:gnnnn

Revisions to this document are to be noted in the table prior to the issue of a revised document.

| Date | Details | Approval |
| --- | --- | --- |
|  | First Issue |  |

1.	Introduction	6

1.1.	Responsibilities of the Competent Authority (AtoN)	6

2.	Aims and Objectives	6

2.1.	Scope of the GuidEline	6

2.2.	Essential Components of the Guideline	6

2.3	Other consideration of the guideline	8

3.	Common requirements	8

3.1.	Regulatory compliance	8

3.2.	Mission planning	8

3.3.	Safety Procedures	9

3.4.	Risk Assessment	9

3.5.	Buoy deck machinery and equipment	9

3.6.	Task Responsibility	10

3.7.	Buoy Technical Characteristics	11

3.8.	General Buoy Operating Procedure	11

3.9.	Special Operations	11

4.	DEFINITIONS	12

5.	abbreviations	12

6.	references	13

7.	Further reading	13

List of Tables

Table 1	Essential Components of the Guideline	5

Introduction

Buoy tender activities are an important aspect of the management of Marine Aids to Navigation (AtoN). The safety risk due to the lifting of heavy loads is higher than other marine activities. As a result, there are obvious differences between buoy tender operations and other marine vessel operations. This guideline puts forward suggestions on some factors that should be considered for buoy tender activities.

Buoy tenders are typically equipped with either a crane or an A-frame, with winches onboard for handling of the mooring arrangements. The operations undertaken by buoy tender crews are unique (different?) from those of other marine vessels. They are complex in nature and often with increased safety risk. Buoy tender crews not only conduct general sailing manoeuvres and other activities associated with safe navigation and seamanship, they also may carry out high risk service AtoN operations.

Responsibilities of the Competent Authority

As a key participant in front line management and maintenance of AtoN, determining, creating and delivering the necessary guidelines for the crew or workers of the buoy tender is the responsibility of the competent authority or contracted third-party service providers.

Aims and Objectives

Provide guidance to competent authorities and third parties to manage buoy tender activities, provide safe procedures for buoy tender operations, and assist the personnel responsible for buoy tender activities by identifying safety risks, operational procedures, and responsibilities.

Scope of the GuidEline

Provide operating safety parameters for buoy tender activities including the retrieval, deployment, positioning, maintenance and other special operations contained in Table 1.

Essential Components of the Guideline

The guideline contains safety and operational guidance for buoy tenders. It contains information on safety procedures, risk management, environmental considerations and buoy tender operations.

Table 1

| Item | Content | Description |
| --- | --- | --- |
| 1 | Regulatory compliance | Competent authority regulations, other legal requirements, Insurance and Liability, Vessel Seaworthiness, and Crew Competence. |
| 2 | Mission planning | Making a work plan, preparation of machinery, collection and analysis of weather/sea/traffic information, pre-departure check. |
| 3 | Safety procedures | Preparation before operation. Crew licensing and certification, fire safety and emergency drills. |
|  |  | Personal protection equipment as identified by Member State. |
| 4 | Risk management | Complete risk assessment checklist. |
|  |  | Ensure correct vessel and/or equipment for operation. |
| 5 | Environmental considerations | Ensure that you identify risks and restrictions specific to certain environmental conditions. |
| 6 | Buoy tender deck machinery, equipment and maintenance | Introductions to buoy tender deck machinery and equipment, including crane, winches, rigging equipment, mooring tools, and rigging inspections. |
| 7 | Task responsibility | Introductions to basic task information and responsibility of bridge, deck and engine department. |
| 8 | AtoN technical characteristics | Introductions to basic AtoN characteristics relating to buoy tender operations. |
|  |  | Introductions to buoy mooring systems |
| 9 | General buoy operation procedure | Loading and unloading procedure. |
|  |  | Buoy retrieval procedure. |
|  |  | Maintenance and inspection of buoy and mooring system. |
|  |  | Positioning methods and procedures. |
|  |  | Buoy deployment procedure. |
| 10 | Special operations | Each special operation is unique, and each has its own procedures and safety concerns. Diving operations, retrieving sunken buoys and other objects, servicing fixed AtoNs such as lighthouses at sea and sector lights, leading lines, and lights and beacon verifications and aids to navigation surveys are some examples of special operations. |
|  |  | These operations are not typically routine and therefore require extra precautions and considerations. |
|  |  | Each evolution should be planned in advance as the case may be and include risk mitigations. |
|  |  | Competent authorities and third-party service providers may have standardized methods that must be followed. |

- 2.3	Other consideration of the guideline

In recent years, with the development of technology, the number of marine economic activities has been increasing. The rise of economic activities such as offshore wind farms and aquaculture have brought new challenges to the activities of buoy tenders.  Vessel traffic density and volume have also increased, resulting in more operational and navigational risk, more AtoNs to service, and more private AtoNsrequiring additional supervision and inspection, with these tasks being assigned to the buoy tender.

Common requirements

Regulatory compliance

Follow international and domestic maritime legal requirements, regulations, and competent authority policies.

Verify that crew and vessel are properly insured and understand the terms of their liability.

Inspect buoy tenders for seaworthiness in accordance with relevant maritime laws and regulations.

Personnel competencies and skills must be qualified according to government requirements. These may include International Maritime Organization’s (IMO) Standards of Training, Certification and Watchkeeping for Seafarers (STCW).

Mission planning

The planning process should begin with the competent authority or contracted service provider and include:

Determine the boundaries of the working area.

Consider the physical and geographical conditions of the working area and the coordinates AtoN setting (depth, type of ground, tide level, ice condition, current).

Account for the presence of restrictions related to flora and fauna in the area (including dangerous ones), navigation regimes in water areas and nature reserves.

Determine the type of AtoN, mooring system, and signal system.

Determine the type of buoy tender.

Plan the route and time for it into account hydrometeorological conditions and daylight hour.

Coordinate with port authorities and other competent authorities concerned.

Review any requirements for:

Waste Management

Oil Spill Prevention regulations

Restrictions on the introduction of invasive species

Mobilize and load AtoN Equipment.

Prepare machinery and load adequate fuel and consumables for the operation.

Collect and analyse weather/sea/traffic information.

Conduct voyage planning.

Coordinate with port authorities, competent authorities, and if necessary, the VTS.

Complete pre-departure checks.

Consider the physical and geographical conditions of the working area and the coordinates AtoN setting (depth, type of ground, tide level, ice condition, current).

Account for the presence of restrictions related to flora and fauna in the area (including dangerous ones), navigation regimes in water areas and nature reserves.

Determine the type of AtoN, mooring system, and signal system

Determine the type of vessel (Buoy tender).

Plan the route and time for it into account hydrometeorological conditions and daylight hour

  - Coordinate with port authorities and other competent authorities concerned.

Review any requirements for:

Mobilize and load AtoN Equipment.

- Prepare machinery and load adequate fuel and consumables for the operation.

- Collect and analyse weather/sea/traffic information.

- Conduct voyage planning.

- Coordinate with Port Authorities, Competent Authorities, and if necessary, the VTS.

- Complete pre-departure checks.

Safety Procedures

Decide what personal protective equipment (PPE) the buoy tender personnel are required to use according to national regulations and environmental conditions.

Determine the safest operational procedures according to national regulations and good seamanship.

Provide task-oriented training (for example, on the job training for new joiners or inexperienced personnel) in accordance with the organisation’s standard operating procedures.

Conduct regular emergency drills and operational simulations according to national regulations.

Conduct safety and risk mitigation briefings according to the organisation’s safety management system.

- Determine the safest operational procedures according to national regulations and good seamanship.

Provide task-oriented training (for example, on the job training for new joiners or inexperienced personnel) in accordance with the organisation’s standard operating procedures.

Conduct regular emergency drills and operational simulations according to national regulations.

- Conduct safety and risk mitigation briefings according to the organisation’s safety management system.

Risk Assessment

Identify common risks related to the task and risks associated with particular operations

Conduct health and safety risk assessment and determine any mitigation measures.

Conduct specific briefings for the personnel involved in the operations.

Provide simulation if necessary.

Conduct health and safety risk assessment and determine any mitigation measures.

Conduct specific briefings for the personnel involved in the operations.

Provide simulation if necessary.

BUOY DECK MACHINERY AND EQUIPMENT

Make sure that all buoy tender deck machinery and equipment are maintained and inspected according to national regulations and those of the competent authority.

Ensure that buoy tender deck machinery and equipment is rated to perform the buoy and mooring operation.

Buoy tender deck cargo, machinery, and equipment should be properly secured during operations and transit.

Determine if operations in hours of darkness are necessary, and consider the buoy tender deck is equipped with sufficient lighting and other safety requirements before conducting these operations.

Ensure that buoy tender deck machinery and equipment is rated to perform the buoy and mooring operation.

Buoy tender deck cargo, machinery, and equipment should be properly secured during operations and transit.

Determine if dark time operations are necessary, and the buoy tender deck is equipped with sufficient lighting and other safety requirements before conducting these operations.

TASK RESPONSIBILITY

Master’s responsibilities:

Master shall maintain command, control, and organization of the operation through the Bridge, Deck and Engine crews.

Bridge crew responsibilities:

Route planning.

Safe navigation and manoeuvrings of the ship.

Maintain ship’s log.

Operate internal and external communications.

Give orders to execute, pause and secure the specific AtoN operation.

Position the buoy and verify the assigned position with the physical position.

After launching and fixing the position of the buoy prepare a buoy report with the necessary photos and position information.

Verify Sector Lights, Leading Lights, and other fixed AtoNs.

Ensure crew is wearing proper PPE.

Ensure proper functioning of navigational equipment.

Notify national Marine Safety Information authority of any changes to the AtoN characteristics.

Deck crew responsibilities:

Prepare fixed and floating AtoN equipment, including structures, mooring systems, and signal equipment.

Load and secure deck cargo.

Test tools and deck machinery, to include cranes, winches, and loose cargo gear, prior to operations and verify they are in good condition.

Responsible for following procedure for specific deck machinery.

Test internal communications.

Prepare crew and equipment before conducting moorings maintenance and buoy deployment and recovery.

Follow deck safety procedures and work as a group to achieve the operating goal.

Verify identification of the buoy with assigned number.

Conduct inspection of the buoy including mooring systems and signaling equipment before and after launching the buoy.

Repair and replace components as required.

Deploy buoy and mooring system.

Engine crew responsibilities:

Maintain readiness of all machinery propulsion and buoy tender deck equipment to include:

Hydraulics.

Winches.

Auxiliary such as electrical power, etc.

Responsibility of onboard shore-based personnel:

Comply with onboard safety management procedures.

According to their scope of personal duty, provide information assistance, communication, task transmission, technical support, resource provision, etc.

According to their purpose onboard, participate in tender operations, such as equipment installation, document recording, buoy position determination, etc.

Provide advice to the master on the implementation of operations but shall not interfere with the master`s independent judgment.

Bridge crew responsibilities:

Route planning.

Safe navigation and manoeuvrings of the ship.

  - Operate internal and external communications.

  - Give orders to execute, pause and secure the specific AtoN operation.

  - Position the buoy and verify the assigned position with the physical position

After launching and fixing the position of the buoy prepare a buoy report with the necessary photos and position information.

Ensure crew is wearing proper PPE.

Ensure proper functioning of navigational equipment.

Notify national Marine Safety Information authority of any changes to the AtoN characteristics.

Deck crew responsibilities:

- Prepare fixed and floating AtoN equipment, including structures, mooring systems, and signal equipment.

- Load and secure deck cargo.

- Test tools and deck machinery, to include cranes, winches, and loose cargo gear, prior to operations and verify they are in good condition.

- Responsible for following procedure for specific deck machinery

- Test internal communications.

- Prepare crew and equipment before conducting moorings maintenance, and buoy deployment and recovery.

- Follow deck safety procedures and work as a group to achieve the operating goal.

- Verify identification of the buoy with assigned number.

- Conduct inspection of the buoy including mooring systems and signaling equipment before and after launching the buoy.

- Repair and replace components as required.

- Deploy buoy and mooring system

BUOY TECHNICAL CHARACTERISTICS

For detailed information about the buoy technical characteristics, please refer to the IALA Maritime Buoyage System (MBS (R1001) and other relevant IALA Recommendations and Guidelines (see references section 6).

SPECIAL OPERATIONS

Each special operation is unique, and each has its own procedures and safety concerns. These operations are not typically routine and therefore require extra care and consideration. Each evolution should be planned in advance as the case may be and include risk mitigations. Competent authorities may have standardized methods that must be followed, included but not limited to:

Large buoy (light ship) operation

Sunken Buoy Recovery

Drifting Buoy Recovery

Beached Buoy Recovery

Obstructions and Hazards to Navigation

Diving Operations

Drones and Remotely Operated Vehicles (ROVs) with or without cables

Hydrographic and AtoN Surveys

Construction of fixed AtoN Structures Verification of sector, leading lines, lights and beacons

Operating in Strong Currents and Heavy Seas

Towing Operations

Ice Breaking

Alongside operations (fixed mark/offshore structure) Marking operations in risk areas (i.e. sunken wreck, shoaling, etc.)

DEFINITIONS

The definitions of terms used in this Guideline can be found in the International Dictionary of Marine Aids to Navigation (IALA Dictionary) at http://www.iala-aism.org/wiki/dictionary and were checked as correct at the time of going to print.  Where conflict arises, the IALA Dictionary should be considered as the authoritative source of definitions used in IALA documents.

abbreviations

AtoN	Marine Aids to Navigation

IMO	International Maritime Organization

ROV	Remotely Operated Vehicle

STCW	Standards of Training, Certification and Watchkeeping for Seafarers

VTS	Vessel Traffic Services

WWA	World-Wide Academy

References

IALA Recommendation R1021 Marine Aids to Navigation Awareness for Mariners

IALA Recommendation R1001 Maritime Buoyage System

IALA Guideline G1035 Availability and Reliability of Aids to Navigation ‐ Theory and Examples

IALA Guideline G1186 Overview of a Floating Aton

IALA Guideline G1151 Maintenance of AtoN Structures

IALA Guideline G1077 Maintenance of AtoN

IALA Guideline G1066 Design of Floating AtoN Moorings

IALA Guideline G1092 Safety Management for AtoN Activities

IALA Guideline G1173 AtoN Training and Awareness for Mariners

IALA Guideline G1018 Risk Management

IALA Guideline G1036 Environmental Management in AtoN

IALA Guideline G1120 Disaster Recovery

- ANNEX A - GENERAL BUOY OPERATING PROCEDURE examples including pictures

Loading and Unloading Procedure

- Follow competent authority instructions on cargo handling.

Retrieval Procedure

- Verify correct buoy.

- Approach buoy based on environmental conditions (wind, tide, etc.)

- Capture and connect to the buoy.

- Lift the buoy on deck.

- Secure buoy to the deck.

- Retrieve the mooring assembly to the deck.

Inspection

- General inspection of buoy hull and lighting equipment.

- Conduct a general overview of mooring assembly during lifting procedure.

- Secure the mooring chain before lifting the sinker.

- Remove marine growth from the buoy hull and appendages.

- Inspect mooring assembly (chain, shackles, swivels, sinker for wear and thickness).

- Verify thickness and wear of mooring components.

- Replace worn components that measure below or will measure below organization standards before the next scheduled maintenance.

- Inspect buoy prior to deployment.

Deployment

- Ensure buoy is secured during transit to deployment position.

- Use proper positioning system to deploy the buoy and mooring assembly on charted/assigned position.

- Deploy the buoy and mooring assembly by following safety considerations.

- Inspect buoy after deployment.

- Verify and record deployed position.