Input paper: ARM21-8.5.2

Input paper for the following Committee(s): 			Purpose of paper:

check as appropriate

X ARM	□  ENG		□  PAP					X  Input

□  ENAV	□  VTS							□  Information

Agenda item 			            8.5

Technical Domain / Task Number 2	            Task 7.1.14

Author(s) / Submitter(s)			            CHINA MSA

# Proposed Amendments to the Maritime Service description for the“MS-2 – Aids to Navigation” Service Based on S-125 Server Testing

# SUMMARY

This input paper intends to share China's experience in the application test of AtoN information service based on S-125, and proposes revision suggestions for Maritime Service description for the "MS-2 – Aids to Navigation" Service.

## Purpose of the document

In respond to the global trend of intelligent and digitalized development in the shipping industry, China has conducted an application test of the S-125 AtoN (Aids to Navigation) information service . This report aims to share the experience based on S-125 server testing and propose revision suggestions for Maritime Service description for the "MS-2 – Aids to Navigation" Service, including enhancing data filtering capabilities, supporting incremental updates, and establishing a two-way feedback mechanism, so as to improve the practicality, real-time performance, and interactivity of AtoN information services.

## Related documents

- S-125: Marine Aids to Navigation (AtoN)

- Maritime Service description for the“MS-2 – Aids to Navigation” Service

# Background

With the development of the global shipping industry moving towards greater intelligence and digitalization, the demand for AtoN information services is constantly evolving. The traditional AtoN data release methods have gradually failed to meet the needs of modern shipping, especially in terms of real-time information updates, accuracy, and interoperability. Future AtoN services will rely on network technology to provide more dynamic, real-time, and reliable AtoN data services, enabling ships at sea to obtain the latest AtoN status and position updates at any time, thereby improving navigation safety and efficiency.

Recognizing this trend,  IALA explicitly proposed in the 20th ARM meeting to promote the development of technical specifications for AtoN information services to ensure that the AtoN information is provided in a standardized and unified manner. To this end, WG2 has initiated Task ARM-7.1.14 "Development of technical service specifications for the provision of AtoN information", aiming to develop and improve the service guideline Maritime Service description for the“MS-2 – Aids to Navigation” Service.

The core goal of this service specification is to standardize all aspects of AtoN information services, from information content modeling to service process modeling, and then to specific service goal setting, ensuring that AtoN data can be accurately and transmitted to the shipboard through shore-based platforms in real time. Through this service method, the shipping industry can greatly improve the efficiency of obtaining AtoN information and enhance the crew's response speed to changes in AtoN status.

Therefore, China conducted an AtoN information service test based on the S-125 standard, which included the deployment of a shore-based test platform for AtoN services and the application of terminal ECDIS to obtain S-125 AtoN information through shore-based services.

# discussion

## S-125 AtoN Information Service Test

The S-125 AtoN information service test mainly includes two parts: firstly, establishing the S-125 AtoN information service shore-based testing platform; secondly, the ECDIS connects to the service API of the shore-based testing platform via 4G/5G networks to complete application function verification. The shore-based test platform for S-125 AtoN information service mainly has the following three main functions:

- S-125 AtoN Information List: The shore-based test platform can import AtoN data organized in a fixed format, automatically convert the data into the format required by the S-125 product specification, and display it. After logging in to the platform, administrator users can maintain the AtoN data list;

S-125 Data List of Shore-based Test Platform

S-125 AtoN Information Editing: After logging in to the test platform, administrators can edit relevant AtoN information;

The Shore-based Test Platform Enables Editing and Modification of S-125 AtoN Information

S-125 AtoN Information Service API: The shore-based test platform provides API service interfaces, which mainly include service subscription interface, S-125 data query interface and S-125 dataset acquisition interface. Users can obtain S-125 AtoN information data by calling these service interfaces.

- On the ECDIS terminal, it is realized that the subscription of S-125 AtoN information service invoked by the S-125 shore-based test platform via 4G/5G network on board, can successfully query S-125 AtoN information data and download S-125 datasets through the interface of the shore-based test platform.

- After subscribing to the service, the specified S-125 AtoN dataset can be obtained through the data acquisition dialog box on ECDIS based on parameters such as regional scope, AtoN data type, and AtoN status.

Parameter Settings for Obtaining S-125 Dataset on ECDIS

- After successfully obtaining the S-125 dataset, it is displayed and rendered on ECDIS.

S-125 Data Obtained by ECDIS Through Shore-based Test Platform

## Proposals

During the testing, we found that crew members need to quickly locate specific types of navigation aids (such as "light buoys") or prioritize identifying navigation aids with abnormal statuses such as "temporary changes" during night navigation and in complex water areas. To reduce manual filtering operations by the crew, shorten the time for obtaining key information, and lower the risk of human errors, we have added navigation aid type and status filtering parameters to theS-125 test platform, enabling ECDIS terminals to accurately acquireS-125 datasets based on category and status dimensions. Accordingly, it is recommended to supplement the filtering function requirements for navigation aid types and statuses in the MS2-R002 clause of Table 4 "Functional Requirements for the MS-2 AtoN Information Service" in the Maritime Service Description for the "MS-2 – Aids to Navigation" Service.

In this test, data was transmitted to ECDIS terminals in GML format. Approximately 3,000 navigation aid samples generated a file size of about 6M. The test results showed that full data transmission has the following problems: first, large bandwidth is consumed in a single transmission ; second, the transmission time in 4G/5G offshore environments is long; third, in ocean-going or severe weather conditions, high packet loss rate of maritime networks leads to increased retransmission probability of large data packets, which easily causes failures such as "data acquisition timeout" on the terminal. Therefore, it is recommended to adopt the optimization scheme of "compression before transmission" in practical applications to improve data transmission efficiency and stability.

- To solve the problems above, a provisional amendments for Maritime Service description for the "MS-2 – Aids to Navigation" Service is given in the annex.

# References

None

# Action requested of the Committee

- The Committee is requested to:

- 1	Note this paper.

- 2	It is recommended that ARM consider the suggestions for modifying relevant content in the Maritime Service Description for the "MS-2 – Aids to Navigation" Service.

# ANNEX

In light of the test results of this S-125 service application, we propose to add and revise the content in Maritime Service description for the "MS-2 – Aids to Navigation" Service.

- The functional requirement MS2-R002 requirement text is suggested to add the description of filtering by AtoN type and AtoN status: It is recommended to revise the requirement text of functional requirement MS2-R002 from "The end-users can request S-100-compliant dataset(s) by name or based on a point, line, polygon geometry, or complete service content. Service providers will respond with current data relevant to the request but will not subdivide datasets to less than that defined by the authoritative source of the data." to "The end-users can request S-100-compliant dataset(s) by name or based on a point, line, polygon geometry,AtoN type,AtoN status or complete service content. Service providers will respond with current data relevant to the request but will not subdivide datasets to less than that defined by the authoritative source of the data";

Table 4 : Functional Requirements for the MS-2 AtoN Infortmation Service

| Requirement Type | Requirement Id | Requirement Name | Requirement Text | Feature Identifier |
| --- | --- | --- | --- | --- |
| Functional | MS2-R002 | Request for dataset(s) | The end-users can request S-100-compliant dataset(s) by name or based on a point, line, polygon geometry, AtoN type, AtoN status or complete service content. Service providers will respond with current data relevant to the request but will not subdivide datasets to less than that defined by the authoritative source of the data. | F001 |

- In the non-functional requirement MS2-NF007, in addition to specifying the data transmission format, it is recommended to add content such as that data should be transmitted in compressed format and the compression method should be clearly indicated. It is recommended to revise the requirement text of non-functional requirement MS2-NF007 from "The service should make the data available in portable machine-readable formats (e.g. XML/JSON)." to "The service should make the data available in portable machine-readable formats (e.g. XML/JSON) and shall transmit the data in compressed format, with the compression method (e.g., gzip) clearly indicated.".

Table 5: Non-Functional Requirements for the MS-2 AtoN Information Service

| Requirement Type | Requirement Id | Requirement Name | Requirement Text | Feature Identifier |
| --- | --- | --- | --- | --- |
| Non-functional | MS2-NF007 | Portability | The service should make the data available in portable machine-readable formats (e.g. XML/JSON)and shall transmit the data in compressed format, with the compression method (e.g., gzip) clearly indicated. | F012 |

- Functional Requirement MS2-R003 mentions that users can obtain automatic update pushes of datasets through subscription services. Considering the network conditions during ocean voyages, users should have a way to clearly know the status of automatic updates, such as whether the current automatic update is successful and whether the local dataset has successfully obtained each automatic update. Therefore, it is recommended to add Functional Requirement MS2-R007, as follows:

Table 4 : Functional Requirements for the MS-2 AtoN Infortmation Service

| Requirement Type | Requirement Id | Requirement Name | Requirement Text | Feature Identifier |
| --- | --- | --- | --- | --- |
| Functional | MS2-R007 | Data update status | The service must be able to inform users of the status of automatic updates for the currently subscribed dataset and whether the current dataset is the latest one, along with other relevant information. | F006 |

Leave open if uncertain