# PERFORMANCE REQUIREMENTS FOR 
Radiocommunications SERVICE IN A VTS

### INTRODUCTION

In 1997 the IMO Maritime Safety Committee adopted Regulations for Vessel Traffic Services (VTS) that have since been included in SOLAS Chapter V (Safety of Navigation) as Regulation 8-2.This Regulation specifies the responsibilities of contracting governments to arrange for the establishment of VTS in certain vulnerable areas under their control.

### BACKGROUND

The performance requirements placed on the CCTV service varies depending on traffic density, levels of VTS, special regional features and the coverage of the VTS area. The purpose of this document is to describe the general performance requirements of these sensors.

Special surveillance cameras such as infrared are not covered by this Manual, as they are considered to be outside of the scope of a VTS.

- OBJECTIVES OF cctv EQUIPMENT

CCTV information may be integrated into VTS applications to provide the operator a real-time assessment of the situation in the VTS area of responsibility.  Information collected from this equipment can be provided to ships to assist in assessing the waterway conditions.

### RELATED REQUIREMENTS INVOLVED

In this section all additional applicable IMO requirements for the respective navigational equipment should be listed. This may be:

Relevant SOLAS requirements; SOLAS Chapter V (Safety of Navigation) as Regulation 8-2

General resolutions, such as resolution A.694(17) - General Requirements for Shipborne Radio Equipment forming Part of the Global Maritime Distress and Safety System (GMDSS) and for Electronic Navigational Aids;

Resolutions with more specific requirements, such as:

Resolution A.688(17) - Code on Alarms and Indicators;

Resolution MSC.39(63) - Adoption of amendments to the Code on Alarms and Indicators.

NMEA 0183 – Standard for Marine Electronic Devices (IEC equivalent)

### DEFINITIONS

As defined by WMO.

### FUNCTIONAL REQUIREMENTS

6.1	Characteristics of the CCTV

Cameras may be used either in low traffic density areas where this is a more cost effective solution to radar or in conjunction with radar as an additional sensor, depending on the level of risk.

The camera should be capable of automatically tracking a vessel manually selected by the VTS Operator via the VTS Screen, if required by the VTS Authority. In addition, there should be the possibility for the VTS Operator to manually de-select the automatically acquired target and manually select another target and/ or area of the acquired target, in order for the VTS Operator to perform specified monitoring tasks, such as pilot embarkation/disembarkation.

Where more than one camera is installed to cover a VTS area it is desirable for the output from both to be fused together to provide one composite picture.

6.2	Reliability, accuracy, range, and resolution

The reliability, accuracy,range, resolution, and units of the measurements should satisfy the minimum requirements as determined by WMO.

6.3	Malfunctions, warnings, alarms and indications

In this section reference should be made to relevant requirements of resolution A.686(17) and resolution MSC.39(63). Depending on the individual type of the navigational equipment requirements in addition to resolutions A.686(17) and MSC.39(63) should be determined if necessary.

6.4	Software requirements

Not applicable

### OPERATIONAL REQUIREMENTS

7.1 Ergonomy

Not applicable

7.2	Operational controls

Not applicable.

7.3	Detection Performance

Depending upon the circumstances, cameras should be capable of identifying the type and possibly the name of the vessels concerned.

The type of vessel should be capable of being identified normally at a minimum range of 3 Nm from the camera location, but this will depend on the individual circumstances, including local topography. The identity of a vessel by shape, color and other features should be capable of being determined normally at a minimum range of 1 NM, but again this will depend on the individual circumstances. These ranges are based on where the nominal visibility is in excess of 10 NM.

.

7.4	Recording and Replaying of Data

The data should be recorded automatically and capable of being replayed onto a separate replay system, if required by the VTS Authority.

7.5	Software requirements

Not applicable

7.6	Availability

The requirements for the availability of CCTV is a matter for the VTS Authority to determine.

### DESIGN AND INSTALLATION

8.1	Durability and resistance to environmental conditions

Electronics installed externally should be in an environmental enclosure .  As regards environmental conditions, the IEC requirements should be applied as far as relevant.

8.2 	Interference

CCTV equipment is industrial equipment and therefore complies with applicable international standards and regulations.  Refer to the IEC requirements (IEC 60945) as far as relevant.

8.3	Power supply

IEC requirements should be applied as far as relevant.  In remote locations, authorities should consider use of alternative power (e.g., solar panels, wind vanes, etc.) in addition to generators.

8.4	 Installation

Requirements concerning the installation of CCTVs, wiring and the arrangement of the equipment in the VTC should be determined.  Operational requirements will determine where CCTVs are to be located and how many are required.  Sites for CCTVs should be selected based upon optimizing the view of the waterway relevant to the VTS.  Other considerations include availability of power, protection against vandalism, housing, and collocation with existing VTS, AtoN, or other suitable infrastructure

'IEC 529 "Degrees of protection provided by enclosures (IP Code)"

IEC 721-3-6 "Classification of environmental conditions, Part 3: Classification of groups of environmental parameters and their severities; Ship environment"

IEC 60945 "Maritime navigation and radiocommunication equipment and systems - General requirements, methods of testing and required test results"

8.5 Maintenance

Requirements in addition to IMO Assembly Resolution A.694(17) concerning maintenance should be determined if necessary. Siting considerations for sensors should include the maintenance,repair, and accessibility requirements.

### INTERFACING

The data to be interfaced for the CCTV service are described in Section 6.

For the interfacing of CCTV services to VTS equipment, several different standards are in use.  Among those standards, the Standard for Marine Electronic Devices, NMEA 0183, has been applied for these applications.

For the interface between a VTS and its users, CCTV video should follow standardized data exchange formats, e.g., ?????????XML.  (See Guidelines on the Exchange of Real Time Data between VTS Systems.)  In addition, a time stamp and source should be provided.

### BACK-UP AND FALL-BACK ARRANGEMENTS

Depending on the individual type of the equipment, requirements concerning back-up and fall-back arrangements should be determined if necessary.

### SAFETY PRECAUTIONS

Depending on the individual type of the equipment, requirements in addition to resolution A.694(17) concerning safety precautions should be determined if necessary.

### MARKING AND IDENTIFICATION

Depending on the individual type of the equipment, requirements in addition to resolution A.694(17) should be determined if necessary, e.g., for the identification of software.

### DOCUMENTATION

In this section the contents of the documentation (e.g. Operator's Manual, Installation Manual, Service Manual) necessary for the operation of the equipment should be determined. The documentation should contain the following items:

General:

manufacturer, type of equipment and the serial number; system description;

specification of the equipment; supplementary equipment;

Installation:

physical characteristics of the equipment;

requirements concerning installation (e.g. supply voltage and frequency, earthing, power consumption, heat dissipation);

safety information;

Operation:

operational controls, functions and presentation of information; startup and shutdown procedures;

tests, failures, calibration procedures and alarms; interfaces and software description;

of updating procedures; safety information;

Failure Elimination, Maintenance and Service:

required tools and materials for failure elimination and maintenance on board; access for maintenance;

preventive maintenance;

replacement, repair, failure elimination, adjustment; information concerning external service.

### MISCELLANEOUS

Not applicable

3. RADIO COMMUNICATION

  - Very High Frequency (VHF)

It is common for the VTS to have its own independent VHF network, for the use within specifically designated VHF Channels.

This network may comprise of one or more VHF Channels in different sectors of the VTS Area.

The VTS Authority may require specific VHF Channels to be designated by the National Radio Authority for specific types of operations.

The VHF equipment must comply with national and international regulations.

  - Long Range Communication

In the case where a VTS Authority requires pre-arrival information, the normal maritime communication systems should be used and therefore an independent network is not required.

  - Radio Direction Finder (RDF)

A number of VTS Authorities require RDF receivers to receive information on the identity of the vessels.

In order to ensure accurate identification the use of two or more separate RDF bearing stations are required.

Bearing accuracy should be better than +/- 2 degrees.

All bearings should be automatically displayed on the chosen screen when the signal has been received after a delay of no more than 3 seconds. The bearings should remain visible on the chosen screen as long as the vessel is transmitting a signal.

It is foreseen that this requirement may decrease as AIS usage by VTS Authorities increases, but will not become obsolete as, depending on national requirements, some vessels may not be equipped with AIS.

RDF is not suitable of being used for continuous tracking.

  - Communication with Allied Services

VTS Centres should be equipped with the ability to communicate with allied services by the use of reliable and secure communication networks. It was usual for a VTS Centre to be equipped with a point to point line with MRCCs for this purpose. However, this means have been found to be inadequate and unnecessarily expensive to establish and maintain. Therefore it is recommended that VTS Centres should be equipped with a digital switched network, with caller identification.