As a result of the accelerated phasing in of the carriage requirement in SOLAS Chapter V Automatic Identification Systems (AIS) are being rapidly installed on board vessels.

Monitoring of AIS messages has shown that a significant number of vessels have already been equipped with AIS. However, this monitoring has also shown many vessels are transmitting incomplete or incorrect data. The causes seem to be mostly:

- Incorrect setting of static data, such as the MMSI number and the vessels’ dimensions, during the initial installation of the AIS on board

- Omission of updating the voyage related data, such as the destination, estimated time of arrival (ETA) and changes in navigational status, by the crew

- Incorrect input or processing of dynamic data, such as the position, course, speed and heading, from the vessels’ sensors

- Inconsistent naming of some data, such as the vessels’ name and destination

The transmission of such incomplete or incorrect data can be considered as ‘teething’ problems due to the rapid introduction of AIS. However without remedial action this could well become a permanent phenomenon jeopardising the usefulness of AIS, also for the VTS.

Due to the minimum keyboard display required and the present lack of presentation of AIS data on radar or ECDIS on board many vessels the vessel might not be readily aware it is transmitting incomplete or incorrect AIS data.  The VTS, when equipped with AIS and presentation of the vessel data on a visual display, can more easily detect such errors.

In order to contribute to improving the usefulness of AIS, the VTS could and should inform the vessels when incorrect data transmission of AIS data by a vessel is detected, as far as practicable.  This will also benefit the usefulness of AIS for the VTS, further contributing to the safety of navigation and the protection of the marine environment.