IALA Long-range Tracking Seminar

‘Tracking all the way?’

Seminar Report

Executive Summary

The IALA Long-range Tracking Seminar – Tracking all the Way? – was held from Nov. 3-5, 2004 at the Coast Harbourside hotel, Victoria, BC, Canada.  The workshop was attended by over 80 delegates including delegates from more than 20 countries.

The Seminar provided an opportunity for discussion on issues surrounding long-range tracking of vessels, and included presentations from technology providers.  The presentations identified many benefits and challenges of long-range tracking, noting that global tracking of certain vessels is presently taking place and has been for a number of years.

The Seminar concluded that global tracking of vessels is a reality, however there is a concern that many local and regional solutions are developing with little international coordination.  As technology exists to respond to global tracking of vessels, there is a need for policy development, including functional specifications.

The Seminar identified 11 conclusions and 3 recommendations (Section 10 refers).

Table of Contents

Introduction	7

Sesssion 1 - Opening	8

Session 2 - Requirement for Long-range Tracking	9

Chairperson – Mike Sollosi, USCG	9

USCG Requirements for Long-range Tracking; J. High, USCG	9

Developments of Long-range Tracking Requirements in IMO (MSC / NAV), K. Fisher, MCA	9

IMO - COMSAR and Long-range Tracking, U. Hallberg, SMA	10

We know where we are, what next? J. Taylor, NLB	10

Discussion – Session 2	10

Session 3 – Benefits and Challenges	11

Chairperson – Duncan Glass, THLS	11

The ship owners’ view of LRIT – P. Hinchliffe, ICE	11

Port Authority Perspective C. B. Teo, MPA, Singapore	11

LRIT Benefits to SAR Organizations, G. Kinney	12

Potential Benefits to Shipping Operations, E. Monteiro	12

Session 4 – Benefits and Challenges	13

Chairperson – Duncan Glass, THLS	13

Potential Forensic uses of Vessel Long-Range Tracking, C. Allen, U. of Washington	13

Long-range Tracking, a Seagoing Master’s Viewpoint, M. Scott	13

Long-range and MISNA, E. Page, MISNA	13

AIS Live and Long-range Tracking R. Trainor, ICAN / AIS Live	13

Discussion – Sessions 3 and 4	14

Session 5 – Technology of Long-range Tracking	14

Chairperson – Ed LaRue, USCG	14

Using Satellite Systems for Long-range Identification and Tracking, A. Fuller, IMSO	14

Long-range ID and Tracking, B. Mullan, INMARSAT	14

Maritime Fleet Tracking Solutions, J. Longson, Polestar	15

MFC, our Experience in Long-range Tracking, A. Iglesias, VMS	15

Session 6 – Technology of Long-range Tracking	16

Chairperson – Ed LaRue, USCG	16

Future Ship Monitoring Technologies, W. Van der Heijden / C. Glansdorp	16

Tracking Data and Data Distribution – The Fisheries Experience, R. Salama, TerraVision	16

LRIT and Intelsat, J. Thomas, Intelsat	16

THISS’s LRIT Solution and the Global Stakeholder, N. Hart, THISS	16

Iridium,  D. Wigglesworth, Iridium	16

Discussion – Session 5 and 6	17

Session 7 – Harmonizing the Process	17

Chairperson – Neil Trainor, AMSA	17

Integrating LRIT into VTS, Phil Woodhams	17

Satellite AIS G. Molessa, USCG	17

Preliminary Results of Long-range Questionnaire	18

Discussion – Session 7	19

Session 8 – Displays and Demonstrations	19

Session 9 – Putting it all together – Regional / International Experiences in Long-range Tracking	20

Chairperson – Jillian Carson-Jackson, IALA	20

Long-range Tracking for Maritime Applications – Australia’s Experience, N. Trainor, AMSA	20

CVTS, G. May CCG	20

USCG, M. Sollosi, USCG	20

‘Tracking all the Way? T. Kruuse, IALA	20

Session 10 - Closing Session	21

Chairperson – Mr Torsten Kruuse, Secretary General – IALA	21

Conclusions and Recommendations	21

Observations	22

Concluding remarks	22

Seminar Closing	22

Annex 1 – Graphic Representation for Vessel Monitoring	23

Annex 2 – ‘Tracking all the way?!’ Keynote Speech	24

Annex 3 – VTS2004 Conclusions and Recommendations	27

Annex 4 – Long-range Tracking Seminar Program	29

Annex 5 – Benefits and Challenges for Long-range Tracking	32

Annex 6 - Long-range Seminar Participant List	34

‘Tracking all the Way’ IALA Long-range Tracking Seminar

# Introduction

A seminar on the possibilities of increased safety, security and environmental protection through Long-range Tracking of vessels was held at the Coast Harbourside Hotel, Victoria, BC, Canada from Nov. 3-5, 2004.  The workshop was attended by over 80 delegates including delegates from more than 20 countries.

The objectives of the Seminar were to:

- Identify benefits and possible drawbacks to authorities, ship owners and ports through the long-range tracking of vessels;

- Identify technology available, or soon to be available to enable long-range tracking of vessels;

- Ensure harmonization of technology used for long-range tracking of vessels; and

- Promote discussion on long-range tracking.

The seminar provided an opportunity for open discussion between stakeholder groups through the presentation of user requirements, technologies available and possible means of implementation.

Delegates were asked to complete a questionnaire in order to better identify the scope of the discussions.  This questionnaire was made available to participants on the CD that was prepared.

The seminar was organized as a follow-up to the 10th International VTS Symposium, held in Hong Kong, SAR, Peoples’ Republic of China in February 2004.  VTS2004 concluded that safety and security issues highlighted the need to improve communications between VTS Centres, other Authorities and stakeholders concerned with the safety of shipping, ports and waterways.  The Symposium also noted that there may be advantages in safety, security and traffic management if long-range automatic ship reporting were to be used by ships and shore authorities.

In response to the conclusions of the Symposium a number of recommendations were formulated, including:

VTS2004 Recommendation 1:

Consideration should be given to developing a Recommendation on communication networks for 'common information sharing' between a VTS, appropriate MRCC, Port Authorities and other stake-holders of a waterway as well as adjacent VTS Centres.  The Recommendation should also address the means of ensuring the security of such communications.

VTS2004 Recommendation 4:

Studies should be made into the possible advantages in safety; security and traffic management if long-range ship reporting were to be available for use by ships and shore based authorities.

VTS2004 Recommendation 5

Limiting the mandatory use of VTS to National waters minimises the value of VTS as a tool for improving safety at sea and reducing pollution of the marine environment.

* * * * *

The complete list of Conclusions and Recommendations from the 10th International VTS Symposium, VTS2004, are provided in Annex 3.

The Programme of the Seminar is attached at Annex 4 and a list of the participants is given at Annex 6.

The presentations were divided into topic specific sessions designed to provide background information on the requirements for long-range tracking, the benefits and challenges of long-range tracking, the technologies presently available and under development for long-range tracking and examples of how this information could be put in place.  The presentations, as provided during the seminar, were included on the Seminar CD.

# Session 1 - Opening

Mr T. Kruuse, Secretary General, IALA, opened the Seminar and invited Mr T. Tebbs, Regional Director General, Canadian Coast Guard, to speak on behalf of the Host Organization.  Mr Tebbs welcomed the participants to Victoria, and stressed the importance of the objectives of the workshop.  In reply Mr Kruuse thanked the Canadian Coast Guard for the kind invitation to hold the seminar in Victoria and, on behalf of IALA, welcomed all participants.

Mr. Kruuse then provided an introduction to the concept of Long-range Tracking, noting the evolution of tracking requirements.  He identified the proposed requirement for 2000 NM reporting (96 hr reporting), indicating that this requirement would virtually include all waters, leading to a logical conclusion of point-to-point vessel monitoring.  A copy of graphic representation as presented at the symposium is provided in Annex 1.

Ms. J. Carson-Jackson gave a review of the results of the 10th International VTS Symposium – VTS2004, noting the conclusions and recommendations relevant to the seminar.  She also provided an overview of the objectives of the seminar, and the planned organisation of the meeting.  Participants were then asked to respond to the questionnaire prepared by the Seminar Steering Committee.  This questionnaire was used to focus the objectives of the Seminar to the requirements of the participants.

Mr. Kruuse then introduced Mr. C. Davidson, President, IALA.  Mr. Davidson provided a keynote address on the topic ‘Tracking all the way’.  Mr. Davidson noted that there are a number of parties interested in global tracking of vessels and that various technologies have already been developed to respond to these requirements.  The question of ‘Tracking all the way’ is no longer the issue, the issue is how to track vessels from point of departure to point of arrival.  He noted that such a system would need to be simple to implement while maintaining the integrity and security of the information.  As well, there is a requirement to identify what information should be provided, to whom, and how.  A copy of Mr. Davidson’s speech is included in Annex 2.

An electronic copy of Mr. Davidson’s keynote presentation was included on the Seminar CDS.

# Session 2 - Requirement for Long-range Tracking

## Chairperson – Mike Sollosi, USCG

### USCG Requirements for Long-range Tracking; J. High, USCG

Mr. Jeff High is the Director, US Coast Guard Maritime Domain Awareness (MDA) Program integration Office.  His presentation identified that MDA is "the effective understanding of anything in the marine environment that could adversely affect America’s security, safety, economy, or environment."

The objective of the Coast Guard’s Maritime Security mission is to reduce maritime security risks.  MDA is about recognizing threats, assessing vulnerabilities, and then disseminating accurate information so that decision makers at all levels can take the appropriate actions, such as protective measures (interdiction).

Within the program, Tracking includes the surveillance, detection, classification and identification of vessels.

Initially, SOLAS vessels will be tracked and a track history of vessels will be maintained. International and national legislations will be required to ensure compliance with LRIT requirements.  In the long term, the US intends to track any mobile platforms in US waters.

### Developments of Long-range Tracking Requirements in IMO (MSC / NAV), 
K. Fisher, MCA

Mr. Kim Fisher is Head of Navigation and Communication for the Maritime and Coastguard Agency, which is an executive agency of the Department for Transport in the UK.  He attended the IMO meetings which developed the requirements for security and the ISPS Code and chaired the technical groups that developed the Ship Security Alert System and Long-range Tracking.  In his presentation he noted that long-range tracking discussions began in IMO at the 22nd Assembly in Nov. 2001.  This Assembly took place shortly after the 9/11 incident, and it was noted at the Assembly in Resolution A.924(22) that the IMO should undertake on a high priority basis a review of security measures and take prompt action.

The concept of ‘Long-range Identification and Tracking’ was at brought forward during MSC75 in May 2002.  It was noted that the system is intended to enhance the security of coastal States by providing information about vessel traffic in a timely manner to enable the State to take any appropriate action.

At MSC76 in Dec. 2002 a draft resolution was prepared.  Further discussion at the Diplomatic Conference in Dec. 2002 resulted in two resolution of direct relevance to long-range tracking.

- Resolution 3 – Invites IMO to carry out as a matter of urgency, an impact assessment of the proposals to implement the long-range identification and tracking of ships and, if found necessary, develop and adopt appropriate performance standards and guidelines for long-range ship identification and tracking systems.

- Resolution 10 - Urges Governments to take action at a national level; Invites Governments to encourage their ships to prepare to respond to Inmarsat C polling or to other available systems; Requests Governments to consider all aspects including misuse and confidentiality.

Resolution 10 urges countries to ask their ships to respond to INMARSAT –C polling. However no countries have taken up the initiative.  The COMSAR working group is continuing work on the development functional requirements for long-range tracking.

### IMO - COMSAR and Long-range Tracking, U. Hallberg, SMA

Mr. Urban Hallberg is an advisor on SAR and VTS with the Swedish Maritime Administration and Shipping Inspectorate.  He is the coordinator for the implementation of the EU traffic monitoring directive and Chairman of IO COMASAR subcommittee.

Mr. Hallberg noted that development of functional requirements for long-range tracking is still under way at COMSAR.  Worldwide tracking is something that will come as the need to know about vessel movements is increasing.  The COMSAR Working Group is working under a worldwide long-range tracking concept.  A fragmented approach to ship reporting would complicate the establishment of national, regional and international standards and procedures.

COMSAR 8 noted that there are a lot of issues to be discussed and proposals to be developed since no existing system alone is capable to provide a global coverage.  In addition, COMSAR 8 felt that special consideration should be given to ships in Sea Area AI provided with a ship borne AIS to be exempted from Long-range Tracking.  The Working Group also determined that, due to the worldwide concern over the integrity of AIS ship borne data that the system is not well suited for a security system and that long-range tracking should not interface with AIS.

The Working Group also felt that Long-range Tracking data should be shared with SAR & Rescue organizations as the information could save lives.  Considerable work still needs to be done by COMSAR to advise MSC on long-range tracking.

Mr. Hallberg noted that the results of the seminar could be of benefit to COMSAR.  He asked the participants to identify what can be done to assist in the process, with a specific request to IALA to continue promoting discussion as they have done in this seminar.

### We know where we are, what next? J. Taylor, NLB

James Taylor developed the theme of ‘We know where we are, what next’ by drawing parallels in development of the road and aircraft transportation sectors and their use of tracking devices to increase safety and improve efficiency.  He stressed that the marine sector will not be able to wait much longer in using similar technology.  Worldwide traffic is increasing and security, safety and environmental risks are increasing.  Long-range tracking systems will have to be simple, robust and universally available.  They will have to supplement and mesh with existing VTS and water-space management systems; in time they may replace them.  He also referenced the keynote speech and agreed that there will need to be careful consideration in the development of the system to identify the trade-off between transparency and vulnerability.

Mr. Taylor provided a word copy of his presentation, which was included on the Seminar CD.

## Discussion – Session 2

The issue of AIS unreliability of data was of concern to participants and it was suggested that international organizations responsible for AIS should take measures to ensures that data in AIS cannot be tampered with.  The integration of a GPS /GNSS receiver into AIS ship borne equipment (built into the unit) was discussed as reliance on external positioning devices has been generating data errors.

A general view was expressed that total visibility of traffic by means of tracking devices such as AIS and LRIT will provide greater security.

# Session 3 – Benefits and Challenges

## Chairperson – Duncan Glass, THLS

### The ship owners’ view of LRIT – P. Hinchliffe, ICS

Commander Peter Hinchliffe is the General Manager (Marine) of the International Chamber of Shipping.  He provided a presentation on the ship owner’s view, noting commercial aspects, experience with AIS and LRIT principles and the possibility of LRIT as a mandatory requirement.

With regards to commercial aspects, Commander. Hinchliffe noted that business needs can be served by LRIT, and in fact this is already being done by some companies.  The experience with AIS has led industry to oppose LRIT or, at best, be sceptical.  Some key elements in the introduction of long-range tracking would be to:

- Ensure no cost to ship owner

- No additional carriage requirements (make use of existing carriage requirements)

- Limit message to time, position and identification

- Clearly identify who receives data, acts upon the data, and why to ensure rules and priorities are set

- Establish an international body, accountable to IMO, to collect, collate and distribute data

The presentation also made reference to the Australian proposal to MSC79 (MSC79-5-12) and noted that ICS fully supports this proposal.

### Port Authority Perspective C. B. Teo, MPA, Singapore

Chee-Beng Teo provided an overview of the port of Singapore, noting that Singapore traffic includes 135, 000 vessels representing 954, 000, 000 tons a year.  Within this context, Mr. Teo noted the many benefits of long-range tracking:

- Enhance security

- Improve monitoring

- Improve efficiency

- Benefit SAR

- Improve situational awareness

He also expressed some concerns:

- Cost to shipowner, coastal/port state to support International tracking

- Misuse of data by unauthorized users for ship targeting

- Training, proliferation of reporting requirements

He noted the presentations of Mr. Fisher and Mr. Hallberg, and the fact that further work is needed at IMO to make long-range tracking a reality.

### LRIT Benefits to SAR Organizations, G. Kinney

Captain Kinney is on the Board of Directors of the Association For Rescue At Sea (AFRAS), and the Council of the International Lifeboat Federation (ILF), which is a worldwide non-governmental organization of Search and Rescue Services and a recognized expert in Search and Rescue, Maritime Security and Coastal Surveillance.  His presentation indicated that ‘LRIT’ could present a significant improvement in quantity and quality of ‘long-range’ information available for SAR.  Specific benefits of long-range tracking to SAR:

- Information readily available to SAR Controller with minimal effort

- Awareness of all SOLAS vessels in area

- Greater position accuracy

- More frequent update rate

- Passive – No reporting action required of vessel

- Near Global coverage

- Facilitates much quicker response

The presentation also noted that SAR requires the right information, at the right time, in the right places, to be proactive if possible, or reactive if necessary.

### Potential Benefits to Shipping Operations, E. Monteiro

Captain Monteiro is a Fellow of the Nautical Institute and is actively involved with various shipping activities on the West Coast of Canada.  His presentation identified potential benefits of long-range tracking for the shipping industry.

He specifically noted that the benefits of long-range tracking for the mariner would be limited.  The main benefits would be for the shore side, specifically for Ship Owners, Charters, Shippers, Stevedores and Government Agencies.  The presentation also looked at possible agencies for collecting long-range tracking information, including VTS; a Central Agency or a Commercial Operator.

The main challenges identified included process and content for information sharing.

# Session 4 – Benefits and Challenges

## Chairperson – Duncan Glass, THLS

### Potential Forensic uses of Vessel Long-Range Tracking, C. Allen, U. of Washington

Craig Allan is a professor of law at the University of Washington and provided a presentation on the forensic uses of LRIT.  His presentation concentrated on the legal issues surrounding long-range tracking, with specific reference to the forensic use of long-range tracking:

  - Hit and run

  - Mystery spills

  - Fishing violations

  - Routing violations

  - General crimes

He also noted there is not immediate international economic benefit for long-range tracking and, as such, it will have to be required by a governing body it if is to be fully implemented.

### Long-range Tracking, a Seagoing Master’s Viewpoint, M. Scott

Capt. Scott provided a Master’s Viewpoint on long-range tracking.  His presentation made reference to the discussions to date, and the requirement to think the process through carefully before proceeding.  He noted that there are many aspects to long-range tracking that could be of benefit, but there are many areas where the process can go wrong as well.

The viewpoint of master mariners will be critical to ensuring practical application of long-range tracking, and their experiences must be including in any discussions on implementing global tracking of vessels.

### Long-range and MISNA, E. Page, MISNA

Ed Page provided an overview of MISNA – Marine Information System of North America – with emphasis on the concept of long-range tracking.  He noted that some vessels are already being tracked, using MISNA, and that there is no technical reason why this cannot be extended to all SOLAS vessels.

The presentation noted that MISNA is a broker for information by collecting, analysing and then disseminating as required.

### AIS Live and Long-range Tracking R. Trainor, ICAN / AIS Live

While not a long-range tracking system, AIS-Live has been discussed in many forums related to long-range tracking.  Rob Trainor provided the presentation on behalf of HITT.  He noted that AIS is a broadcast system, and that the AIS information is in the public domain.  The use of information gained through a system such as AIS for third party use was discussed, and this could be of relevance to any further discussions on long-range tracking.

## Discussion – Sessions 3 and 4

Discussions highlighted the many benefits, and challenges, of long-range tracking.  These were developed into a table, included in Annex 5.  In addition, discussion included what can be done to encourage ship owners to adopt long-range tracking:

- Global tracking for SAR response

- Possible reduction for insurance

- Reduced port fees

- Slotted arrival times for more efficient port operations

# Session 5 – Technology of Long-range Tracking

## Chairperson – Ed LaRue, USCG

### Using Satellite Systems for Long-range Identification and Tracking, A. Fuller, IMSO

Andy Fuller provided a historical background and context of LRIT.  He focused on the question - what is long-range? (how far is ‘long’?)  He also noted that if you don’t track a vessel from point of departure to point of arrival (i.e. ‘all the way’) you cannot know what that ship has done during the voyage.

A number of questions were raised on security technical requirements, passive vs active systems, costs and who should pay.  Issues on accounting and billing were raised, as well as a requirement for quality control, integrity and security of data transmitted.

He noted that it may not be necessary to have global providers, rather regional providers working together under a governing body.  IMSO is an international body that could run / oversee the operation of a global tracking system.

### Long-range ID and Tracking, B. Mullan, INMARSAT

Brian Mullan identified a generic process that could be implemented for long-range tracking that would ensure timely, relevant information provided on a global basis.  The proposed solution is identified below:

The need for multilateral agreements between administrations and identification of who will perform the operational integrity monitoring were identified.  A system and timetable was proposed and it was pointed out that data management does not need to be run by governments.  He noted that many systems for long-range tracking of specific vessels are in existence and, if a global system is not put in place in the very near future we may end up with various regional, incompatible systems.

Proposed timetable for introduction of long-range tracking:

| MSC 79 agrees to adopt global long-range tracking | Dec. 2004 |
| --- | --- |
| MSC 79 instructs COMSAR 9 | Dec 2004 |
| COMSAR 9 completes requirements | Feb. 2005 |
| MSC 80 endorses COMSAR 9 output | June 2005 |
| Long-range tracking introduced for existing equipment | Dec 2006 |
| Long-range Tracking introduced for new equipment | Dec 2007 |

### Maritime Fleet Tracking Solutions, J. Longson, Polestar

Julian Longson provided a summary of Pole Star and PupleFinder®.  He demonstrated how LRIT is currently being implemented and pointed out that the technical solution is available however it is a very complex problem from a political/regulatory perspective.  He noted that any long-range tracking solutions should be a private/public system with an independent overseeing organization.

The presentation made reference to the introduction process for AIS, noting that we need to learn from the AIS experience and move foreword carefully to ensure a system that is robust, secure and fully auditable.

### MFC, our Experience in Long-range Tracking, A. Iglesias, VMS

Angel Iglesias provided a presentation on the FLMS – Fleet Localisation and Monitoring System.  His presentation provided an overview of Spain’s experience in long-range tracking over the past 8 years.  He noted that over 2000 fishing vessels are monitored to enforce fishing policies.  The operation of the system was described and examples were given.

The presentation included an overview of the equipment used for the monitoring.  He noted that the technology is available for global tracking of vessels, and companies are ready to respond to the challenge of implementing a system for SOLAS vessels.

# Session 6 – Technology of Long-range Tracking

## Chairperson – Ed LaRue, USCG

### Future Ship Monitoring Technologies, W. Van der Heijden / C. Glansdorp

Wim Van der Heijden and Cees Glandsorp introduced the EMBARC and MarNIS Projects providing the context and objectives of each project.  The precautionary principle and Risk Assessment at Sea was explained, along with the effect of age on vessels.

The presentation included technologies for long-range tracking, including the possibility of using AIS information as input for a satellite based long-range tracking system.  In addition to information provided by ships, the possibilities for alternatives means of monitoring vessels in an area of concern was also presented.  These included IKONOS satellite optical images and remote sensing synthetic aperture radar (SAR).

The presenters stressed that the future should combine existing technologies to provide one reliable picture.

### Tracking Data and Data Distribution – The Fisheries Experience, R. Salama, TerraVision

Rama Salama presented an overview of the TerraVision service and systems, linking them to the objectives of the seminar.  The issues related to long-range tracking were discussed, such as costs, data sterilisation caused by infrequent polling and and operational issues.  The question was raised on how to standardize the various technologies currently available.

While stressing that the technology is available for long-range tracking, it stated that users of the system need to have a clear idea of what is expected from the system, balanced with acceptable costs.

### LRIT and Intelsat, J. Thomas, Intelsat

Janice Hart provided an overview of Intelsat, noting that they are a wholesale network solution.  She noted that standards need to be developed – once they have been developed industry will offer the solution.

### THISS’s LRIT Solution and the Global Stakeholder, N. Hart, THISS

Nick Hart described THISS, with specific reference to the objectives of the seminar.  He noted that, from a technological perspective, long-range tracking can, and has been, achieved.  Technology is available to provide cost effective solutions.  What is required is a clear indication of what the objective of what the shipping industry requires – the technology can then respond.

### Iridium,  D. Wigglesworth, Iridium

David Wigglesworth provided an overview of Iriduim.  He identified experience with aviation tracking solutions, and the value added elements that could be included in long-range tracking equipment.  The importance of properly defining all the requirements for long-range tracking was stressed. Once it has been determined what is needed, the technology will provide it.  The presentation highlighted the fact that the technology is available, that global tracking of certain types of vessels is already going on.

What needs to be identified is:

- How will information be managed?

- What will be in place to ensure authentication of information?

- What are the rules and policies surrounding long-range tracking? And

- Who pays?

## Discussion – Session 5 and 6

It was noted that the presentations in Sessions 5 and 6 were of great value in identifying options for long-range tracking.  The requirement for functional specifications, and not technical specifications was stressed.  The technology exists for global tracking of vessels, what is required are policies and rules in place to ensure a global, rather than a regional or local approach.

The definition of security was discussed, and it was noted that ship owners need to feel comfortable that information is not made available to their competitors.  Requirements for global tracking of vessels were also discussed, and it was stated that this should be developed by IMO.  The proposal from Inmarsat (Brian Mullan) was noted as a reasonable and workable approach, and discussion included how to move forward with development of a global tracking system.

Questions included:

- What format will the data be in and will it be available to the ship owners?

- Will LRIT evolve to the point that security aspects can be removed from AIS, so that it can concentrate on collision avoidance?

- What format will the information be in and who will set the standard for the format?

- Who could be the International Oversight body to specify the format and settle on a standard for service providers and users?

# Session 7 – Harmonizing the Process

## Chairperson – Neil Trainor, AMSA

### Integrating LRIT into VTS, Phil Woodhams

Phil Woodhams provided a presentation on the integration of long-range tracking into the Australian REEFREP system.  The presentation provided practical information on a long-range tracking system, noting the technological and regulatory issues faced when setting up the system.

The presentation concluded by looking at future integration with other systems for Ship Information, Pilot Information; Fisheries (VMS), Port Authority Information; Port State Control and Customs.

### Satellite AIS G. Molessa, USCG

George Molessa provided a presentation on the long-range tracking system for the USCG.  The process would see AIS signals sent to a satellite, with data filtering onboard the satellite to lessen bandwidth requirements and meet reporting requirements.  The processed AIS data is then sent from the satellite to GES vai ORBCOMM data link.  The data is then routed through a Network Control Center (NCC) for quality check, satellite maintenance, etc.  The information then goes to the USCG, where it is incorporated in Command and Control (C2) systems and distributed to other users.

## Preliminary Results of Long-range Questionnaire

Neil Trainor provided a presentation on the preliminary results of the Long-range Quesitonnaire.  Key results included

‘For what purpose(s) do you think Long-range Tracking should be used – Government Agencies / Competent Authorities’

The main purposes identified included for Search and Rescue (15%), Ship Reporting Systems (SRS) (13%), Marine Domain Awareness (11%); VTS (11%).

‘For what purpose(s) do you think Long-range Tracking should be used – Maritime Industry’

The main purposes identified were Fleet Tracking (24%); Route Monitoring (21%) and Scheduling (18%).

## Discussion – Session 7

Participants noted the results of the questionnaire with interest, and requested further information on analysis.  It was noted that more detailed analysis will be done, and that the questionnaire will be made available on the CD of the Seminar.

Participants were able to re-submit their questionnaire if they felt it will be of benefit.

# Session 8 – Displays and Demonstrations

Participants were encouraged to provide displays / demonstrations on long-range tracking equipment and trials.  Demonstrations were provided by:

Arne Mikkelsen – Nauplios (Kongsberg)

Edward Page – ASUTS

Henry Nel – TRANSAS USA Inc.

Mark Johnson- SHINE MICRO, Inc.

Terry McLeod – OCEANVISION using TERRAVISION

Greg Flessate – ORBCOM

Phil Woodham – HAS

Geoff Douglas- ABSOLUTE

# Session 9 – Putting it all together – Regional / International Experiences in Long-range Tracking

## Chairperson – Jillian Carson-Jackson, IALA

### Long-range Tracking for Maritime Applications – Australia’s Experience, N. Trainor, AMSA

The presentation was prepared by Neil Trainor, John Young and Mahesh Alimchandani of AMSA and identified how long-range tracking is being integrated in AUSREP and REEFREP.  It noted the use of Inmarsat C for near-real time reporting and the benefits realized through the integration of long-range tracking into a coastal VTS system.  These benefits include route monitoring and better planning for shipping operations and AtoN services within the region.

### CVTS, G. May CCG

Gordon May provided an overview of the Co-operative VTS System for the west coast of Canada and the US.  He indicated that this system has been in place since 1979, and has provided benefits on many levels.  He related the experience of setting up the CVTS system to the concerns expressed during the workshop, noting that the CVTS system took 10 years of planning and coordination and over three years of intergovernmental negotiations to set up.

### USCG, M. Sollosi, USCG

Mike Sollosi provided an overview of the seminar from the USCG perspective.  He noted the key concerns as expressed by the seminar, by responding to the questions of ‘Where?’; ‘Who?’; ‘How often?’; and ‘How?’.

The presentation identified that the next steps include a need to establish regulations and define functional and performance requirements.  He noted that technical requirements are not needed, as technology exists for global tracking of vessels.  He also observed that security of the data at all levels is paramount – on board the vessels, during transmission, in the database and in the administration.  The system must be a low or no cost to the ship, and it must be a global system to avoid a proliferation of unilateral regional efforts.  He noted that there is a need to limit the messages to what is necessary both in content and reporting interval.

In conclusion, he identified that long-range identification and tracking is going to happen – but it must be done right!

### ‘Tracking all the Way? T. Kruuse, IALA

In closing the technical sessions Mr Kruuse provided a review of the week’s discussions.  He referred to the keynote presentation of Mr. Clive Davidson, specifically identifying those areas that had been focused on during the Seminar - a system simple to implement while maintaining the integrity and security of the information; the requirement to identify what information should be provided, to whom, and how.

# Session 10 - Closing Session

## Chairperson – Mr Torsten Kruuse, Secretary General – IALA

## Conclusions and Recommendations

The Seminar reached the following draft Conclusions:

- Global tracking of vessels is currently taking place by some ship owners and maritime authorities.

- There was agreement to have mandatory global tracking of certain types of vessels.

- Operational use of Global tracking of vessels can open possibilities for more efficient management of maritime traffic.

- Global tracking of vessels should be technology neutral.  The Functional requirements must be clearly identified.

- There is a need for an approval procedure for system providers for the global tracking of vessels.

- Global tracking of vessels should simple to implement, with minimum requirement for additional equipment on board, maximizing the use of existing carriage requirements.

- The system data shall be protected from source to end user.

- Data transmission costs should be borne by the end-users.

- The end users should have the capability to determine the reporting rate in real-time and on demand.

- Information in the global tracking messages should support a number of associated maritime activities, including search and rescue (SAR), Ship Reporting Systems (SRS), Vessel Traffic Services (VTS), protection of the environment, and security.

- Information from the global tracking of vessels should be sent to data management centre(s) that collect and disseminate the information.  There should be an overseeing intergovernmental agency accountable to the international community that assures the integrity of the data management centres – IMSO should be considered for this role.

The Seminar agreed the following draft Recommendations

- IMO member states participating in this seminar should put forward a joint paper recognizing the results of this seminar to COMSAR 9 by Dec. 6th 2004.

- IALA should prepare an information paper with the results of this seminar for IMO – COMSAR 9.

- IALA should continue to contribute to the development of a global vessel tracking system.

* * * * *

## Observations

Although not identified as an objective of the seminar, AIS was brought out in many areas of discussion.  It was agreed that the following observations be included in the report:

  - AIS is a tool that can be used for maritime safety, maritime security and tracking and should be developed to fulfil these requirements.

  - Shipborn use of AIS will be improved with integrated operational displays replacing the MKD.

# Concluding remarks

Torsten Kruuse closed the discussion by noting areas where further work will be required in the very near future and encouraged all delegates to follow up on the action items that were developed during the seminar.

The results of the IALA Long-range Tracking Questionnaire were also discussed further, and he noted that a more detailed review of the results will be carried out by relevant IALA Committees during the next round of meetings (march/april 2005).

# Seminar Closing

Mr. Kruuse congratulated all speakers on the excellence of their presentations and expressed gratitude once again to the Canadian Coast Guard for co-hosting this event.

Mr Kruuse also thanked the exhibitors, noting that the presentations on technology aspects clearly identified the capability for long-range tracking from a technical viewpoint.

Finally Mr Kruuse expressed the appreciation of IALA to all participants for the support they continued to give not only to this particular event, but also to the Association in general.  He highlighted the short notice that was necessary in order to arrange this seminar in a timely manner, and thanked participants for re-arranging their schedules to attend the event.

# Annex 1 – Graphic Representation for Vessel Monitoring

Graphic representation of waters outside of the 200 nautical mile limit (in green)

Graphic Representation of waters outside 2000 nautical mile limit (in green)

# Annex 2 – ‘Tracking all the way?!’ Keynote Speech

Mr. Clive Davidson,

President, IALA

CEO, AMSA

This seminar’s theme is long-range identification and tracking (LRIT) and in particular we will be examining the available technologies and the scope for harmonisation to meet a variety of uses.

Long-range tracking is high on the International Maritime Organisation’s   agenda for development of appropriate measures to enhance maritime domain awareness.  IMO will be considering in 2005 appropriate technologies and management arrangements that should be incorporated into mandatory obligations for ship operators and Governments.

This seminar provides a valuable opportunity to provide considered technical advice from IALA to assist IMO in that task.

There are already a number of parties with a legitimate interest in the long-range tracking of ships:

  - Commercial bodies, who want to know where their own ships and cargoes are, the status of the ship and the condition of its cargo;

  - Port authorities, and ships’ agents, wanting to know in advance what ships are expected and the services they will require;

  - Customs, immigration and quarantine services, port State control agencies, all looking at their separate risk profiles for ships to be inspected;

  - Navigational services, providing information and related ship reporting and VTS services for safety of navigation purposes;

  - Search and Rescue services, especially those with large SAR areas like Australia’s, who wish to prepare for emergency responses;

  - Environmental protection agencies, wishing to protect their coasts and the oceans from potential pollution;

  - Fisheries management authorities, regulating the long term sustainability of the world’s fisheries, particularly the battle against illegal fishing on the high seas and in remote areas; and

  - Security agencies, including armed forces, from flag States, port States and coastal States, dealing with the ever present threats of piracy and now the more recent threats from terrorism.

Many different systems have been developed to cater for these interests, both commercial and government owned and operated.  They vary in the type of technology used, the application to different types of ships or areas of operation, and in the frequency and costs of reporting a ship’s position and related information.

The proliferation of systems has the potential to overburden the ship’s crew with different reporting requirements, as well as raising costs for operators and governments in both equipment and operational terms.

With discussion at IMO about the potential use of LRIT information for security, safety and pollution prevention as well as search and rescue purposes, there appears to be scope to rationalise the provision of vessel position data for these multiple IMO sponsored systems.   There is also scope to incorporate commercial interests into an integrated system, with appropriate access controls to the data.

In addition to providing a better picture of vessel movements from a domain awareness perspective, key features of long-range tracking should be that:

  - ship reporting should be as efficient as possible from the vessel’s perspective and

  - a vessel should not be required to report the same data to multiple systems, especially if those systems use different technologies.

Where possible, a vessel should be required to report the data only once within a defined area or time period and the data should then be shared between reporting systems for their respective purposes.

To reduce reporting burdens on crew, the more automated the reporting the better. Similarly, the various reporting requirements around the world should be capable of being met by the minimum range of equipment on board ships, with out a need for operators to fit extra equipment.

The principal challenge is to organise a grouping of interests that can most efficiently generate a single picture of shipping movements, including interoperability of technologies and data sharing systems, whilst providing appropriate levels of access and protection of data confidentiality and security from unauthorised uses.

We also must be mindful of the legal basis for an integrated tracking system, particularly ensuring consistency with the international law of the sea regarding rights and responsibilities of ships, and of coastal, port and flag States.

  - In this regard, there are already many interpretations of customary and international law concerning rights of innocent passage.

  - With new global security concerns, and increasing concern for pollution prevention and coastal environmental protection, there may be a need to consider the meaning of “innocent passage” if a ship is not prepared to advise on its identity, location, deviations from its intended course, what it is carrying, who is on board and so on.

The aviation sector for many years has openly provided information on aircraft movements, providing a comprehensive picture of an individual aircraft’s position, identity and nature. . Data is shared regularly among flight control centres and with other relevant interested parties.  This has long been held as an important contribution to aviation safety.

Debate to date in the maritime sector has concentrated on areas around coastal states or a defined time or distance from the relevant port when a ship has to commence reporting.  This would still leave some areas of the oceans uncovered, leading to potential gaps in awareness of ships’ movements and possible interactions between ships at sea.

In the modern world, particularly in the current climate of enhanced security concerns, there is no reason why we shouldn’t be working to ensure that we have more complete knowledge of shipping movements.

  - The technology and organisational capacity already exists.

  - What is needed is the collective will to do it.

Legitimate commercial operators should have nothing to fear about transparency of knowledge of where their ships are and in declaring their positions to government agencies.   There are several benefits for them in terms of improved protection by security agencies that are alert and prepared to respond to attacks on ships, as well as the obvious benefits from efficient processing on arrival at ports and responses to emergencies.

Nevertheless, there is a need to protect position data from unauthorised access and tampering, which needs to be built into any system of reporting.

In conclusion, we are looking for this seminar to chart a way forward for long-range ship tracking and information sharing that caters for many different users, that is efficient in the application of technology and minimises the operational burdens on ships’ crews, and that allows for transparent and efficient sharing of information while also protecting data integrity and preventing access by unauthorised users.

I am confident that you will rise to this challenge and can provide useful and comprehensive guidance to the IMO in its future deliberations.

Thank you.

# Annex 3 – VTS2004 Conclusions and Recommendations

The VTS2004 Symposium reached the following draft Conclusions:

- Safety and security issues have highlighted the need to improve communications between VTS Centres, other Authorities and stake-holders concerned with the safety of shipping, ports and waterways.

- The response time and interruptions to traffic flow resulting from incidents occurring in a VTS area could be significantly improved by the provision of facilities capable of rapidly analysing the effect of the incident and indicating appropriate actions that might be taken by VTS Personnel.

- There is concern at the prospect of two different shipborne AIS systems operating in a port or waterway where each system is used by a substantial percentage of the traffic.

- There may be advantages in safety, security and traffic management if long-range automatic ship reporting were to be used by ships and shore authorities.

- Limiting the mandatory use of VTS to National waters minimises the value of VTS as a tool for improving safety at sea and reducing pollution of the marine environment.

- Concern was expressed at the standard of installation of AIS equipment on board some ships and the limitations of the MKD (Minimum Keyboard and Display). Concern was also expressed about the lack of knowledge on the operation and use of shipborne AIS equipment.  These are causing problems with the use of the system and could result in low confidence in AIS information.

- Experience with implementing training in accordance with Recommendation V-103 has confirmed the value of the Recommendation and its associated documentation.

- Vessel Traffic Management and Information Services (VTMIS) are likely to have an impact on the traditional concept of VTS.

- There is a need for the availability of Electronic Navigation Charts (ENC) to provide Competent and VTS Authorities with accurate and up to date information.

The VTS2004 Symposium agreed the following draft Recommendations

- Consideration should be given to developing a Recommendation on communication networks for “common information sharing” between a VTS, appropriate MRCC, Port Authorities and other stake-holders of a waterway as well as adjacent VTS Centres.  The Recommendation should also address the means of ensuring the security of such communications.

- Studies should be made into the use of risk models and/or simulation techniques to provide a proactive system in VTS Centres to assist Operators when it is necessary to take appropriate actions with traffic as a result of a collision, grounding or other incident within a VTS area.

- Urgent studies should be made into the risks involved and the difficulties that might arise through the use of alternatives for maritime shipborne AIS systems in ports and waterways where there is a substantial mix of ocean-going ships and inland waterway craft.

- Studies should be made into the possible advantages in safety, security and traffic management if long-range ship reporting were to be available for use by ships and shore based authorities.

- Studies should be made into areas of the world in international waters and straits used for international navigation where mandatory traffic management would significantly improve the safety of shipping transiting the area or reduce the pollution caused by the marine traffic.  The studies should also investigate the feasibility of establishing traffic management in the areas and identify the extent to which its effectiveness would be limited by not being a mandatory system.

- 6.1	IALA should draw the attention of IMO to the problems resulting from incorrect installation of AIS equipment and of the need to improve the display of AIS information on ships.

- 6.2	IALA should consider the provision of information on its website containing the setting up procedure and correct operation of shipborne AIS equipment.

- 6.3	Competent and VTS Authorities should consider initiating actions to improve the quality of AIS information.

- Competent Authorities should consider introducing training of VTS Personnel in accordance with IALA Recommendation V-103 and its associated documentation.

- IALA should consider developing guidance on VTMIS for Competent and VTS Authorities.

- IALA should work closely with IHO on the availability of Electronic Navigation Charts (ENC), responding to the present and future demand for display of information from different sensors.

# Annex 4 – Long-range Tracking Seminar Program

Tuesday, Nov. 2nd, 2004

1600 – 1800 –Registration – Coast Harbourside Hotel

1800 – 2000 - Welcome Reception – Coast Harbourside Hotel, Ballroom - Sponsored by Carmanah Technologies

Wednesday, Nov. 3rd, 2004

| Time | Activity |  | Presenter | Chairman |
| --- | --- | --- | --- | --- |
| 0830-0930 | Registration |  |  |  |
| 0930-1030 | Session 1 – Opening - CCG - IALA - Overview of Seminar aims |  | Terry Tebb, Regional Director, CCG T. Kruuse, Secretary General, IALA J. Carson-Jackson, IALA | T. Kruuse |
|  | Keynote Address |  | C. Davidson, AMSA |  |
| 1030 - 1100 | Coffee Break |  |  |  |
| 1100– 1230 | Session 2 – Requirement for Long-range Tracking M. Sollosi |  |  |  |
|  | - Security Requirements - Developments of Long-range Tracking Requirements in IMO (MSC / NAV) - IMO - COMSAR and Long-range Tracking - Aids to Navigation Authority Perspective |  | J. High, USCG K. Fisher, MCA U. Hallberg, SMA J. Taylor, NLB |  |
| 1230-1400 | Lunch |  |  |  |
| 1400-1530 | Session 3 – Benefits and Challenges |  |  | D. Glass |
|  | - International Chamber of Shipping Perspective - Port Authority Perspective - Long-range Tracking and Safety - Long-range Tracking, a Seagoing Master’s Viewpoint | P. Hinchliffe, ICS C. B. Teo, MPA, Singapore G. Kinney, Lockheed Martin M. Scott, Master Mariner |  |  |
| 1530-1600 | Coffee Break |  |  |  |
| 1600-1730 | Session 4 – Benefits and Challenges |  |  | D. Glass |
|  | - Legal Aspects of Long-range Tracking - Potential Benefits to Shipping Operations - Long-range and MISNA - AIS Live and Long-range Tracking | C. Allen, U. of Washington E. Monteiro – FNI E. Page, MISNA R. Trainor, ICAN |  |  |
| 1730-1800 | Overview of Benefits and Challenges – Questions / Discussions |  |  | D. Glass |

Free Evening – Facilities are available for evening sessions as required.  Please contact Secretariat to arrange evening discussion sessions.

Thursday, Nov. 4th, 2004

| Time | Activity | Presenter |  |  |  |  | Chairman |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0900-1030 | Session 5 – Technology of Long-range Tracking - Existing |  |  |  |  |  | E. LaRue |  |  |
|  | - International Maritime Satellite Organisation - INMARSAT - Polestar - VMS |  | A. Fuller, IMSO B. Mullan, INMARSAT J. Longson, Purplefinder A. Iglesias, VMS |  |  |  |  |  |  |
| 1030-1100 | Coffee Break |  |  |  |  |  |  |  |  |
| 1100-1230 | Session 6 – Technology of Long-range Tracking - Developing |  |  |  |  |  | E. LaRue |  |  |
|  | - Future Ship Monitoring Technologies - Tracking Data and Data Distribution – The Fisheries Experience - THISS - LRIT and Intelsat - Iridium |  |  | W. Van der Heijden / C. Glansdorp, MarNIS R. Salama, TerraVision N. Hart J. Thomas D. Wigglesworth |  |  |  |  |  |
| 1230-1300 | Overview of Technologies– Questions / Discussion |  |  |  |  |  | E. LaRue |  |  |
| 1300-1400 | Lunch |  |  |  |  |  |  |  |  |
| 1400-1530 | Session 7 – Harmonizing the Process |  |  |  |  | N. Trainor |  |  |  |
|  | - A harmonized approach to Long-range Tracking - OrbComm – USCG approach |  |  |  | P. Woodhams G. Molessa, USCG |  |  |  |  |
|  | Presentation of Questionnaire Results |  |  |  |  | N. Trainor |  |  |  |
|  | Discussion Session on Harmonizing the Approach to Long-range Tracking |  |  |  |  | N. Trainor |  |  |  |
| 1530-1600 | Coffee Break |  |  |  |  |  |  |  |  |
| 1600-1730 | Session 8 – Display and Demonstration |  |  |  |  |  |  |  |  |
|  | Participants are encouraged to visit the displays and view demonstrations presented by Technology Providers. |  |  |  |  |  |  |  |  |

Evening Activity – 1800 - 2300 Bus Tour of Victoria and Workshop Banquet 
(Dress – Semi-Formal / Business Attire)

Friday, Nov. 5th, 2004

| Time | Activity |  | Presenter | Chairman |
| --- | --- | --- | --- | --- |
| 0900-1030 | Session 9 – Putting it all together |  |  | J. Carson-Jackson |
|  | Regional / International Experiences in Tracking - AMSA - USCG - CVTS - ‘Tracking all the Way?’ |  | N. Trainor M. Sollosi G. May T. Kruuse |  |
| 1030-1100 | Break |  |  |  |
| 1100-1200 | Session 10 – Conclusions Presentation Conclusions and Recommendations | J. Carson-Jackson |  | T. Kruuse |
|  | Closing of the workshop |  |  |  |

A CD of the Seminar will be prepared during the final session.  Please remember to collect your CD from the Seminar Reception Desk.

# Annex 5 – Benefits and Challenges for Long-range Tracking

The following is a representation of the discussion on Benefits and Challenges for Long-range Tracking.  It is not intended to be all-encompassing.

| Benefits | Challenges |
| --- | --- |
| Ships - SAR response - Efficiency of operations | Ships - How will information be sent – should not be requirement for new equipment (use existing)? - How often will information be sent, what is impact on OOW work load? - Who pays? |
| Ship Owners - Efficiency of operations – in port - Efficiency of operations – fleet operations - Lower insurance? | Ship Owners: - How to implement? - How to ensure compatibility? - How to ensure confidentiality / security of information? - Who pays? |
| Government Agencies - SAR; - Pollution; - Customs; - Enforcement - Incident review - AtoN requirements (from historical data) | Government Agencies: - Who is responsible? - Who pays? - Who sets regulations / policies? - How is system security maintained? - How /Where is information collected, analysed, disseminated? |
| Port Authorities - Port Efficiency – Pilot boarding / Stevedores / etc. | Port Authorities - How to integrate into existing systems? - Who sets policy / standards? - Who pays? |

Some general concerns included:

Confidentiality and integrity of information;

How to make best use of existing technology;

Who / How to set standards for use (functional, not technical)

Legal concerns regarding third party use of data, UNCLOS and limits of authority.

# Annex 6 - Long-range Seminar Participant List

| Country | Name | Organisation / Address | Telephone | Fax | E-mail |
| --- | --- | --- | --- | --- | --- |
| AUSTRALIA | DAVIDSON, Clive | AMSA | (61) 2 62 79 50 39 | (61) 2 62 79 58 13 | kxo@amsa.gov.au |
| AUSTRALIA | JOHNSON, Brian | Australian Maritime Systems | (61) 736 334 100 | (61) 732 681 781 | btj@marsys.com.au |
| AUSTRALIA | SALAMA, Rameri | TerraVision Pty., Ltd. | (61) 8 9364 5788 | (61) 8 9364 9545 | rameri@sat.com.au |
| AUSTRALIA | TRAINOR, Neil | AMSA | (61) 7 3120 7422 | (61) 7 3120 7420 | neil.trainor@amsa.gov.au |
| AUSTRALIA | WOODHAMS, Philip | HSA Systems | (1) 613 727 0658 | (1) 613 727 9465 | philip.woodhams@amec.com |
| AUSTRALIA | YOUNG, John | AMSA | (61) 2 62 79 57 10 | (61) 2 62 79 57 57 | john.young@amsa.gov.au |
| AUSTRIA | MOSER, Thomas | Frequentis GmbH | (43) 1 81150 2615 | (43) 1 8450 2399 | thomas.moser@frequentis.com |
| CANADA | CAMPANA, Ellister | DND | (1) 250 363 1060 | (1) 250 363 2976 | campana.enb@forces.gc.ca |
| CANADA | CHERNOFF, Rick | Canadian Coast Guard | (1) 601 775 8858 |  | chernoffr@pac.dfo-mpo.gc.ca |
| CANADA | DESPAROIS, Michel | Canadian Coast Guard | (1) 613 990 3031 | (1) 613 996 89 02 | desparoism@dfo-mpo.gc.ca |
| CANADA | DRABIT, Mimi | Carmanah Technologies | (1) 250 380 00 52 | (1) 250 380 00 62 | mdrabit@carmanah.com |
| CANADA | HANLON, Jim | Seimac Ltd. | (1) 902 468 3007 | (1) 902 468 3009 | info@seimac.com |
| CANADA | KOLACZ, Richard | COM DEV Ltd. | (1) 519 622 2300 | (1) 519 622 1691 | richard.kolacz@comdev.ca |
| CANADA | FLOTRE, Al | IMPA | (1) 604 688 0291 | (1) 604 688 5250 | bccp@bcpilots.com |
| CANADA | McLEOD,Terry | Oceanvision Systems, Inc. | (1) 604 649 3550 |  | admin@oceanvision.com |
| CANADA | MAY, Gordon | Canadian Coast Guard |  |  | Mayg@dfo-mpo.gc.ca |
| CANADA | MONTEIRO, Ed | Nautical Institute | (1) 604 312 4502 | (1) 604 467 2370 | EGMONTE@aol.com |
| CANADA | RYAN, Sam | Canadian Coast Guard | (1) 613 998 15 28 | (1) 613 998 92 58 | ryans@dfo-mpo.gc.ca |
| CANADA | STEELE, James | Nautical Institute, British Columbia (Press Repr.) | (1) 250 592 6188 | (1) 250 592 6188 | jamessteeleconsulting@shaw.ca |
| CANADA | TRAINOR, Robert | ICAN Ltd. | (1) 250 897 1920 | (1) 604 648 8393 | rtrainor@ican.nf.net |
| CANADA | ZANDVLIET, Marc | DND | (1) 250 363 5365 | (1) 250 363 2590 | zandvliet.m@forces.gc.ca |
| CHINA | BAO, Jianbo | China MSA | (86) 10 65 29 2891 | (86) 10 65 29 2893 | bjb@vip.sina.com |
| CHINA | XU, Yansong | China MSA | (86) 10 6529 2891 | (86) 10 6529 2893 | bjb@vip.sina.com |
| DENMARK | CHRISTENSEN, Henrik | Thrane & Thrane | (45) 39 55 88 06 | (45) 39 55 88 88 | hch@tt.dk |
| DENMARK | FUSSEL, Jan | Thrane & Thrane | (45) 39 55 82 22 | (45) 39 55 88 88 | jaf@tt.dk |
| DENMARK | HEINSDORF, Jens | Thrane & Thrane | (45) 39 55 83 48 | (45) 39 55 88 88 | jh@tt.dk |
| DENMARK | PEDERSEN, Jens | Terma Elektronik AS | (45) 20 435 357 | (45) 87 436 001 | jcp@terma.com |
| DENMARK | SVENDSEN, Thinus | Thrane & Thrane | (45) 39 55 88 00 | (45) 39 55 88 88 | ts@tt.dk |
| GERMANY | BREUNING, Rolf | EADS - Defense & Communications Systems | (49) 731 392 5950 | (49) 731 392 7128 | rolf.breuning@eads.com |
| GERMANY | JUSTEN, Ingo | Frequentis GmbH | (49) 6103 30086 37 | (49) 6103 30086 19 | ingo.justen@frequentis.com |
| GERMANY | STADTELMEYER, Klaus | EADS - Defense & Communications Systems | (49) 731 392 4826 | (49) 731 392 7182 | klaus.stadtelmeyer@eads.com |
| HONG KONG | LAU, Eric | Marine Department, HKSAR | (852) 2852 4539 | (852) 2542 3199 | wk_lau@mardep.gov.hk |
| ITALY | GIOMI, Alessandro | Alenia Marconi Systems | (39) 0641504388 | (39) 0641504487 | agiomi@amsjv.it |
| ITALY | PELLIZZARI, Piero | Italian Coast Guard | (39) 65 508 4578 | (39) 65 508 4990 | tei_pellizzari@hotmail.com |
| JAPAN | SODA, Yasushi | Japan Association of Marine Safety | (81) 3 3502 2233 | (81) 3 3581 6136 | yasushi-soda@nikkaibo.or.jp |
| KENYA | BANAFA, Adnan | Kenya Ports Authority | (254) 41 311 409 | (254) 41 220 424 | abanafa@kpa.co.ke |
| MALAYSIA | BAHARI, Zakaria | Light Dues Board | (60) 3 33 46 76 02 | (60) 3 31 685 289 | zakaria@mot.gov.my |
| NETHERLANDS | DE WITT, Witte | XANTIC | (31) 651 20 39 21 | (31) 704 46 56 24 | witte.dewit@xantic.net |
| NETHERLANDS | GLANSDORP, Cees | Marine Analytics BV | (31) 10 22 01 330 | (21) 65 31 28 120 | maran@luna.nl |
| NETHERLANDS | VAN DER HEIJDEN, Wim | TNO-FEL | (31) 70 374 0332 | (31) 10 374 0654 | vanderheijden@fel.tno.nl |
| NETHERLANDS | WILLEMS, Reinier | XANTIC | (31) 6 53 22 95 35 | (31) 70 34 30 900 | reinier.willems@xantic.net |
| NORWAY | MIKKELSEN, Arne | Kongsberg Seatex AS |  | (47) 73 51 50 20 | arne.mikkelsen@kongsberg.com |
| PANAMA | STEC, Henry | Panama Canal Authority | (507) 272 44 20 | (507) 272 7199 | hstec@pancanal.com |
| SCOTLAND | TAYLOR, James | Northern Lighthouse Board | (44) 131 473 31 00 | (44) 131 220 20 93 | jamest@nlb.org.uk |
| SINGAPORE | HART, Nicholas | Thiss Technologies Pty., Ltd. | (65) 62 72 3001 | (65) 62 72 75 72 | nhart@thiss.net |
| SINGAPORE | TEO, Chee Beng | IMPA | (65) 63 75 17 10 | (65) 63 75 17 19 | kennethl@mpa.gov.sg |
| SPAIN | IGLESIAS, Angel | Sainsel Sistemas Navales | (34) 95 49 35 346 | (34) 95 49 35 306 | angel.iglesias@sainsel.abengoa.com |
| SWEDEN | HALLBERG, Urban | Swedish Maritime Administration | (46) 11 19 11 17 | (46) 11 19 12 30 | urban.hallberg@sjofartsverket.se |
| TANZANIA | SWAI, Raymond | Tanzania Harbours Authority | (255) 22 212 1696 | (255) 22 211 3938 | dets@tanzaniaports.com |
| UK | FISHER, Kim | Maritime & Coast Guard Agency | (44) 2380 329 143 | (44) 2380 329 204 | kim.fisher@ties.itu.int |
| UK | FULLER, Andy | IMSO | (44) 207 728 1378 | (44) 207 728 1172 | andy_fuller@imso.org |
| UK | GLASS, Duncan | Trinity House Lighthouse Service | (44) 207 481 6900 | (44) 207 480 7662 | duncan.glass@thls.org |
| UK | HINCHLIFFE, Peter | International Chamber of Shipping | (44) 207 417 2853 |  | Peter.hinchliffe@marisec.org |
| UK | LONGSON, Julian | POLE STAR - Maritime Fleet Tracking Solutions | (44) 207 313 7400 | (44) 207 7313 7401 | longson@purplefinder.com |
| UK | MULLAN, Brian | INMARSAT | (44) 207 728 1464 | (44) 207 728 1752 | brian_mullan@inmarsat.com |
| UK | SCOTT, Martin | Nautical Institute | (33) 4 93 26 42 53 | (33) 4 93 26 42 53 | emartinscott@yahoo.com |
| USA | ALLAN, Craig | University of Washington, School of Law |  |  | challen@u.washington.edu |
| USA | PETERS, Andrew | North America CLS | (1) 206 855 28 80 | (1) 206 201 82 31 | apeters@nacls.com |
| USA | ASCHEMEYER, Manny | Marine Exchange of Southern California | (1) 310 519 31 27 | (1) 310 732 06 45 | mannya@mxsocal.org |
| USA | CARUSO, Nick | L-3 Communications | (1) 941 379 1630 | (1) 941 377 5591 | nick.caruso@L-3com.com |
| USA | CHEN, Paul | Barcoview, LLC | (1) 503 614 0559 | (1) 503 690 1525 | paul.chen@barco.com |
| USA | DOUGLAS, Jeffrey | Absolute Software Inc. | (1) 617 444 86 20 | (1) 617 777 86 06 | jeff@absolutesw.com |
| USA | FLESSATE, Greg | ORBCOMM | (1) 703 433 6507 | (1) 703 433 6400 | flessate.gregory@orbcomm.com |
| USA | HIGH, Jeff | US Coast Guard | (1) 202 267 6157 | (1) 202 267 4700 | jhigh@comdt.uscg.mil |
| USA | HOLT, Guy | NOAA OFFICE for Law Enforcement | (1) 907 586 9353 |  | guy.holt@NOAA.gov |
| USA | JOHNSON, Mark | Shine Micro, Inc. | (1) 360 437 25,3 | (1) 360 437 2483 | mark@shinemicro.com |
| USA | KAMARA, Dieudonné | Telenor Satellite Services | (1) 301 838 7747 | (1) 301 838 7824 | dieudonne.kamara@telenor-usa.com |
| USA | KINNEY, Gabe | Lockheed Martin | (1) 315 456 39 11 | (1) 315 456 04 39 | gabriel.o.kinney@lmco.com |
| USA | LA RUE, Ed | US Coast Guard | (1) 202 267 04 16 | (1) 202 267 48 26 | elarue@comdt.uscg.mil |
| USA | LOCKERBY, Ronald | Thrane & Thrane | (1) 757 463 9557 | (1) 757 463 95 81 | rdl@tt.dk |
| USA | MARKLE, Robert | RTCM | (1) 703 527 2000 | (1) 703 351 9932 | rmarkle@rtcm.org |
| USA | MOLESSA, George | US Coast Guard | (1) 202 267 12 75 | (1) 202 267 10 67 | gmolessa@comdt.uscg.mil |
| USA | NEL, Henry | Transas USA | (1) 206 838 3000 | (1) 206 838 7900 | hnel@transasusa.com |
| USA | PAGE, Edward | Marine Exchange of Alaska | (1) 907 463 2607 | (1) 907 463 2593 | edpage@mxak.org |
| USA | PESCHEL, Rudy | US Coast Guard (Retired) | (1) 305 361 7839 |  | SpesAgent@aol.com |
| USA | SHELVER, Clayton | SARSINC |  |  |  |
| USA | SOLLOSI, Mike | US Coast Guard | (1) 202 267 15 39 | (1) 202 267 48 26 | msollosi@comdt.uscg.mil |
| USA | THOMAS, Janice | INTELSAT | (1) 202 243 5048 | (1) 202 944 7982 | janice.thomas@intelsat.com |
| USA | WIGGLESWORTH, David | Iridium Satellite LLC | (1) 301 571 6242 | (1) 301 571 6250 | david.wigglesworth@iridium.com |
| USA | SPRUNT, Douglas | Tideland Maritime Systems | (1) 503 744 0672 | (1) 503 744 0675 | dsprunt@tidelandsignal.com |