INMARSAT-C AS AN LRT TOOL – AUSTRALIA’S EXPERIENCE IN THE GREAT BARRIER REEF REGION

Purpose

- To provide preliminary information on Australia’s experience with the adoption of Automated Position Reporting via Inmarsat-C in the Great Barrier Reef Region

- To raise awareness of emerging issues related to adaptation of existing technologies for LRT purposes.

### Discussion

Globally, there is a growing demand from various agencies for LRT to enable them to fulfil their individual roles ranging from navigation safety and SAR to environmental protection, prevention of crime and security.  The information needs range across a continuum from a simple daily position report to close monitoring of a ship’s movement and its cargo. Some complex needs are being addressed in layered surveillance designs related to ‘risk ‘.

The Great Barrier Reef and Torres Strait Vessel Traffic Service (REEFVTS) is one of many measures implemented to reduce the risk of environmental damage by shipping in the Great Barrier Reef and Torres Strait. The region is a navigationally challenging Particularly Sensitive Sea Area, which is remote, sparsely populated and subject to sporadic, severe tropical weather conditions.

To effectively monitor ships transiting throughout the 350,000 sq. km. region satellite technology has recently been adopted to compliment radar and AIS installations. As the region is classified as Sea Area A3 for GMDSS purposes and transiting ships are required to be equipped with Inmarsat-C terminals.

Since 1 December 2004, the mandatory ship reporting system has been upgraded and three new requirements introduced.  Ships now provide:

- Pre-Entry Reports

- 15-minute position updates via Inmarsat-C Automated Position Reporting

- Comprehensive route information

Further information on REEFVTS can be found on the AMSA website - http://www.amsa.gov.au/shipping_safety/ausrep_and_reefrep/changes_to_reefrep.asp

The REEFVTS Traffic Information Module integrates this information with other key information sources (radar, AIS, etc) to provide accurate ship encounter predictions to transiting ships.  If so desired, traffic updates are automatically forwarded to individual ships via Inmarsat-C Email.  The upgraded services also allow for provision of navigational assistance information to ships where close track monitoring indicates a ship may be standing into danger.

In relation to use of Inmarsat-C, REEFVTS objectives are to:

- Remotely program Inmarsat C terminals and monitor ships without imposing any burden on the watch-keeping officers

- Automatically provide ships with updates of Ship Traffic Information

- Immediately alert ships in potential emergency situations

Australia’s experience with LRT via Inmarsat C has been encouraging and well received.  REEFVTS provides transiting ships with significantly more accurate traffic and encounter information and VTS operators with an enhanced surface picture.

However, the adoption of Automated Position Reports via Inmarsat C for LRT has also revealed a number of emerging issues.  Addressing these issues, where possible, would significantly improve the capability of the technology to provide a simple and easily managed solution for LRT.  The issues fall into the following categories and are detailed in Annex 1:

- Issues relating to the Inmarsat-C system

- Issues relating to Inmarsat-C terminals

- Issues relating to ship staff

These issues are presumably only now being identified, as the functionality to deliver close track monitoring has not been exploited in the maritime sector until recently.  In addition it would appear that application of the existing technology for LRT purposes would be enhanced through the availability of a more robust data reporting structure and reduced delays in delivery of messages.

# Annex 1. Issues Identified

| Issue | Comments | Possible solutions |
| --- | --- | --- |
| - Global DNID Management | Where the surveillance area encompasses more than one Ocean Region such as Australia’s SAR region that encompasses areas in the IOR and POR. Global DNIDs would: - Eliminate the need for duplication in downloading multiple DNIDs and programming polls, and - Negate the difficulties associated with deleting DNIDs from Ocean Regions other than the one currently logged into. | Adoption of a Global DNID would have major benefits for both the vessels and shore stations wishing to receive position reports |
| - Losses in Data Reports | It has been observed that while some terminals transmit automated data reports like clockwork, others sometimes appear to behave erratically. This undermines the ability to track vessels accurately. | The data reporting structure could be assessed with a view to making the processes more robust |
| - Delays or non-delivery of messages | Currently, an Inmarsat-C message sent to a ship could remain undelivered for over 30 minutes without the sender being aware | An assessment of the message delivery structure with a view to reducing delivery times and improving notification of non-delivery |
| - Costs of LRT via Inmarsat-C | If LRIT via Inmarsat-C were to be considered for applications beyond VTS, with provision for AIS type information, the costs of delivery could be prohibitive | An abbreviated or compressed proprietary messaging system could be developed Await the IMO functional specifications |
| - Deficiencies in functionality of First Generation Terminals | - ‘First generation terminals’ is a term used to describe the Inmarsat-C terminals released in the early 1990s. It is estimated that between 5-10% of terminals currently being used by ships are first generation units - The functionality of these units is considerably less than later models. In particular: - They cannot be remotely programmed to forward position reports at prescribed intervals. While some models can be manually programmed on board the vessel this is not true for all. - Most have only one DNID - Re-starting a unit can result in the DNID and programming being lost | Terminals that are not remotely programmable could be phased out over a 12-month period. |
| - Manual updating of time/position | Some Inmarsat-C units have not been installed with automated internal/external GPS feed and must be manually updated with both time and position. Under GMDSS this is every four hours | Automated internal/external GPS feed to terminal could be mandated by flag state over a 12-month period. However, it might be the case that automated GPS input is already a condition of Class. |
| - Terminals subject to barring | It is not possible to track a terminal via APR if the terminal has been barred by the LES. This generally happens for non-payment of service charges. | An unbarred terminal could be included in conditions of entry and/or mandated by flag state |
| - Limited technical standardisation in data reporting | The technical specifications allow for a fair degree of latitude in design of LES-MES data reporting structures. This results in variations in terminal responses to polling commands | The technical specifications need to be standardised. |
| - Ship staff – lack of knowledge | - A significant, unquantified proportion of ship’s navigating officers do not have sufficient knowledge of Inmarsat-C. Up to a third provide incorrect terminal details. | Education campaign to increase knowledge of: - Updating GPS info - Selection of Datum - Deletion of DNIDs - Configuration of terminals - Logging-on. In REEFVTS the ship’s terminal must stay logged-on to the POR - Deletion of log files Amending the GMDSS Operator’s Course by IMO |