Performance requirements for Automatic Identification Systems(AIS) in VTS

Table of Contents

1	Introduction	4

1.1	Abbreviations	4

1.2	Definitions and Clarifications	4

1.3	References	4

2	General Provisions	6

2.1	Objectives of AIS	6

3	AIS Base Station as a sensor	8

3.1	Equipment	8

3.1.1	Antennas	8

3.1.2	GPS	8

3.1.3	DGPS	8

3.1.4	EGNOS/WAAS	8

3.2	Interface requirements	8

3.3	Firmware upgrade	9

3.4	Base Station configuration	9

3.4.1	Standard messages for configuration	9

3.4.2	Issues not covered by standard messages	9

3.5	Slot map allocations	9

3.6	Redundancy	10

3.6.1	Within the AIS Base Station	10

3.6.2	External Base Station Controller	10

3.6.3	External software process	10

3.7	Coverage aspects	10

3.8	Long Range Tracking	10

4	AIS Network as a sensor	11

4.1	Equipment	11

4.2	Interface requirements	11

4.3	Reliability of services	11

4.4	Security of data	11

4.5	Validity of data	11

4.6	Domain awareness	11

5	Operational aspects in relation to the VTS	12

5.1	Use of virtual MMSI	13

5.2	Use of the VTS Targets message	13

5.2.1	Limitations to the use of VTS Targets message	13

5.3	Waypoints and Route Plan	14

5.3.1	Ship Waypoints and/or Route Plan Report	15

5.3.2	Advice of Waypoints and/or Route Plan of VTS	15

5.4	Graphical presentation	Error! Bookmark not defined.

5.4.1	Symbol usage	Error! Bookmark not defined.

5.4.2	Interaction with radar tracks	Error! Bookmark not defined.

5.5	Data Validity	16

5.5.1	Potential hazards	17

5.5.2	Data handling problems	17

5.5.3	Data fusion problems	18

6	Security	18

# Introduction

AIS is intended to enhance: safety of life at sea; the safety and efficiency of navigation; and the protection of the marine environment. SOLAS regulation V/19 requires that AIS exchange data ship-to-ship and with shore-based facilities. Therefore, the purpose of AIS is to help identify vessels; assist in target tracking; simplify information exchange (e.g. reduce verbal mandatory ship reporting); and provide additional information to assist situation awareness. In general, data received via AIS will improve the quality of the information available to the OOW, whether at a shore surveillance station or on board a ship. AIS should become a useful source of supplementary information to that derived from navigational systems (including radar) and therefore an important ‘tool’ in enhancing situation awareness of traffic confronting users.

## Abbreviations

AIS		Automatic Identification System

PSS		Physical Shore Station

LSS		Logical Shore Station

ASM 		AIS Service Management

BSC		Base Station Controller

SOTDMA	Self Organised Time Domain Multi Access

FATDMA	Fixed Allocation Time Domain Multi Access

## Definitions and Clarifications

- AIS Transponder

- AIS Network

  - PSS (Physical Shore Station)

    - BS(AIS Base Station)

    - BSC(Base Station Controller)

  - LSS(Logical Shore Station)

  - ASM(AIS Service Management)

## References

| [1] | IMO MSC. 74(69), Annex 3 | IMO Recommendation on Performance Standards for a Universal Shipborne Automatic Identification System (AIS) |
| --- | --- | --- |
| [2] |  | IMO SOLAS Convention Chapter V |
| [3] |  | ITU Radio Regulations, Appendix S18, Table of Transmitting Frequencies in the VHF Maritime Mobile Band |
| [4] | ITU-R M.1371 | ITU Recommendation on the Technical Characteristics for a Universal Shipborne Automatic Identification System (AIS) Using Time Division Multiple Access in the Maritime Mobile Band |
| [5] | IEC Standard 61993 Part 2 | Universal Shipborne Automatic Identification System (AIS) Operational and Performance Requirements, Methods of Testing and required Test Results. |
| [6] |  | IALA Recommendation on the Technical Clarifications of Recommendation ITU-R M.1371-1 |
| [7] | IALA Recommendation A-124 | IALA Recommendation on AIS Shore Stations and Networking Aspects Related to the AIS Service |
| [8] | Resolution A.694(17) | General Requirements for Shipborne Radio Equipment forming Part of the Global Maritime Distress and Safety System (GMDSS) and for Electronic Navigational Aids |
| [9] | Resolution A.529(13) | Accuracy Standards for Navigation |
| [10] | Resolution A.688(17) | Code on Alarms and Indicators |
| [11] | Resolution MSC.39(63) | Adoption of amendments to the Code on Alarms and Indicators |

# General Provisions

## Objectives of AIS

AIS should:

- Provide automatically to appropriately equipped shore stations, other vessels and aircraft information, including the craft’s identity, type, position, course, speed, navigational status and other safety-related information

- Receive automatically such information from similarly fitted vessels and craft

- Monitor and track vessels

- Exchange data with shore based facilities.

- Assist in ensuring the highest possible level of security and efficiency for the vessel traffic in the designated area

The requirements should not be applied to cases where International agreements, rules or standards provide for the protection of navigational information.  AIS should be operated taking into account the guidelines developed by the Organization.

## Application

These performance standards specify the requirements for AIS for use in a Vessel Traffic Service.  The AIS should improve the safety of navigation by assisting in the efficient navigation of ships, protection of the environment, and operation of Vessel Traffic Services (VTS), by satisfying the following functional requirements:

- in a ship-to-ship mode for collision avoidance;

- as a means for littoral States to obtain information about a ship and its cargo; and

- as a VTS tool, i.e. ship-to-shore (traffic management).

The AIS should be capable of providing to ships and to competent authorities, information from the ship, automatically and with the required accuracy and frequency, to facilitate accurate tracking.  Transmission of the data should be with the minimum involvement of ship's personnel and with a high level of availability.

The installation, in addition to meeting the requirements of the Radio Regulations, applicable ITU-R Recommendations and the general requirements as set out in resolution A.694 (17), should comply with the following performance standards.

Mandating AIS carriage and establishing a network to receive, process and distribute the AIS signals received from vessels will enhance safety and security and will improve a shore facility’s ability to manage traffic.  The shore facility that receives and processes the AIS signal can vary in size and mission ranging from a small boat station, a Vessel Traffic Service (VTS), a Captain of the Port (COTP), or a regional data fusion center.  Some shore facilities may need to act on the information, another may need to monitor AIS and maintain an information database or archive.  For these reasons, a nationwide network must be scalable and adaptable to a range of demands and requirements.

The network must also be capable of information exchange and distribution among several users ashore and afloat.  Government agencies outside the VTS and other commercial maritime interests all may have justifiable needs for AIS data.

Vessel Traffic Services have been established in various ports to respond to documented navigation safety needs, and the contributions of AIS as a VTS tool are well understood.  Therefore, as a matter of priority, these ports should be outfitted with AIS receiving and transmitting capability as soon as practicable.  Other ports, where there is no compelling federal need to operate a VTS, will want to take advantage of AIS capabilities in order to meet other operational demands.  AIS receiving and transmitting capability will be established in these places based on operational needs and exigency.

# AIS Base Station as a sensor

## Equipment

In addition to the Base Station itself, there is additional equipment necessary to install it.

### Antennas

The Base Station needs to be connected to a VHF antenna for reception and transmission of AIS data. Depending on the setup, one antenna, or individual antennas might be used. The specification for the base station does not include connection of two antennas to the unit, but this can be handled by external equipment.

### GPS

A GPS antenna is necessary to provide GPS information to the Base Station. Typically the AIS Base Station vendors supports a range of GPS antennas.

### DGPS

AIS supports retransmission of Differential GPS corrections, therefore one or more DGPS reference stations might be installed to enhance the accuracy in a VTS area.

### EGNOS/WAAS

The satellite based European Global Navigation Overlay System (EGNOS) and Wide Area Augmentation System(WAAS) will enhance GPS accuracy, availability and integrity by using a network of reference and monitor stations in Europe (EGNOS) and North America (WAAS). Geostationary satellites are used for the data distribution.

## Interface requirements

The Base Station will be interfaced to the VTS system via the hardware interface provided by the manufacturer of the unit. The primary use of of the unit is to receive AIS data as received from mobile units, but there are additional important interface requirements that need to be supported such as how the unit supports firmware upgrades, base station configuration and data exchange. The following interfaces are common:

- Serial

A connection to the unit through a serial connection is the traditionally most common way of interfacing. The interfaces in use are RS232 and RS422/485. The differences between these interfaces are solely hardware related where the balanced (RS422/485) has a better transmission characteristic, while RS232 are more commonly supported for computer equipment.

- Ethernet

Ethernet support is becoming widely supported either directly from the Base Station or via Base Station controllers. The support for Ethernet directly out of the base station simplifies the set up of a site, and has advantages over serial as there is no need to keep a hardware connection (like a serial cable) from the unit to another converting device. Currently the support differs between the manufacturers as some requires an extra device in addition to the base station to support Ethernet, some support only one protocol (UDP or TCP) while some manufacturers supports configuration of the protocol on the interface port.

## Firmware upgrade

The AIS Base Station consists of a software part handling the device. As a great portion of this is to follow the standard protocols, the firmware needs to be upgraded when changes to the protocols are issued. As the base stations might be located at remote sites, there might be a requirement of being able to do the firmware upgrade from a centralised location instead of having to go to each site for upgrade. As the different manufactures have implemented the support for firmware upgrade very differently, this might be a criterion for selection of base station. On the other hand, where there are one or just a few base stations located nearby the control centre, the requirement for remote firmware upgrade is less important.

## Base Station configuration

The base stations needs to be configured before taken into use. There are different proprietary solutions for the AIS base stations regarding configuration, but there are also common messages defined in the AIS protocol supporting configuration of the AIS base stations.

### Standard messages for configuration

### Issues not covered by standard messages

## Slot map allocations

Figure 1 : Slot map

The AIS communication is based on SOTDMA (Self Organised Time Domain Multi Access) and FATDMA (Fixed Allocation Time Domain Multi Access) protocols. As the base stations are using FATDMA, they have to be configured according to a slot map setup. When the base station has been configured with a slot map, it will transmit its own information according to this setup, regardless of other units in the area. The slot map setup is very important in areas where two base stations are overlapping in coverage. If they have slot maps with overlapping areas for transmission, they will transmit at the same time, and the vessels will not be able to receive the information, at least not from both base stations.

Even if a base station is installed for use only by the VTS, it is important to know the setup for any nearby base stations to avoid overlapping transmission slots.

## Redundancy

For high availability systems, redundancy is an important feature, and is supported differently by the vendors. From a system point, the different solutions should be regarded as similar, but the different approaches might be important to certain systems.

### Within the AIS Base Station

### External Base Station Controller

### External software process

## Coverage aspects

## Long Range Tracking

# AIS Network as a sensor

The purpose of an AIS network is to combine AIS information from several base stations, and provide this information to internal or external users. An AIS network might involve several services for maintenance and control of data, but for the VTS an AIS Network will ideally look like an AIS base station.

## Equipment

AIS Base Station, VHF antennas, remote power control units, UPS etc.

## Interface requirements

The implementation of the AIS Network will to some extent dictate the interface requirements from the VTS. Even if the data should be based on the standard AIS protocol, there will typically be requirements for authentication of users of the AIS Network. This implies login procedures for the VTS to be able to access data.

## Reliability of services

## Security of data

## Validity of data

## Domain awareness

# Operational aspects in relation to the VTS

## General requirements

Depending on the individual type of the navigational equipment, requirements in addition to resolution A.694(17) should be determined, such as settings (e.g. continuous, variable and default settings), maximum permissible time for operation, information to be displayed at all times and dedicated display area.
The AIS will provide timely, relevant and accurate information to users to support their decision-making processes.  The AIS also provides information to VTS and allied services to support their missions.

a. Basic information needs of the VTS.  Representative users identified the following as the basic information needs for vessel tracking and port security related missions.

- Up-to-date knowledge and/or information regarding the route to be transited.

- Timely, relevant, and accurate information about other vessels within the area that might affect safety, security, or the decision making of the operational commander.

- Timely information about emergency and environmental conditions that might affect safety or the decision making of the operational commander.

- Where required, the transmission of relevant information to the mariner in a manner that does not distract from the task at hand, particularly in narrow, confined channels where there is heavy traffic.

- Existing navigation aids and tools, pilotage systems, navigation management systems, and regulations provide information to the mariner but these systems require integrity monitoring to ensure the information they impart is accurate.

b.   AIS Watchstander Decision Support System.  The AIS service shall provide automatic vessel position reports and movement information as it is received at remote sites throughout the service area.

c. Integrated Operations.  Where applicable, AISs will support regional Vessel Traffic Services between adjoining VTSs, both domestic and foreign.

d. Incident Analysis Support.  The AIS service shall support incident response and analysis.

- Immediately after notification of an incident, the AIS service shall alert vessels in or planning to enter the area of concern with particulars of the incident.

- The AIS service will provide for review of supporting information and events that occurred before, during, and after an incident.

- The AIS service will be able to replay a recorded incident.

- The AIS service will have the ability to generate and execute scenarios that simulate events to assist in analysis, planning, and readiness.

e. Operations Support.  The AIS service may support port operations by providing information to appropriate shore facilities.  In support of incident response, the AIS service will coordinate with the port authority and provide information about traffic and the corresponding situational information.  In cooperation with the port authority, the system will monitor safety and security zone boundaries that are established in connection with an incident.

f. Other Agency Support.  The AIS service will provide information to other agencies and allied services when a particular vessel of interest has been identified.

## Use of virtual MMSI

Every base station have their own MMSI number. This number defines the AIS base station. When operating from a VTS, the VTS might have its own virtual MMSI number. This will enable the VTS to send messages to vessels using one MMSI regardless of the number of base stations connected, and even when operating through an AIS Network. The feature has to be supported through either the AIS Network or the software used by the VTS vendor when sending data from the VTS.

## Use of the VTS Targets message

The set of standard AIS messages comprises a message called VTS Targets, which contains information on targets derived by means other than AIS. Typically this will be targets that are tracked by a VTS-system based on radars. Sometimes the VTS Targets message is also referred to as VTS Footprint.

The AIS message for VTS Targets can be filled with information for up to 7 targets, each comprising:

- Target id (MMSI, IMO number or Callsign)

- Latitude

- Longitude

- Course (COG)

- Speed (SOG)

- UTC time stamp

A VTS target should only be included in the message if it contains latitude and longitude values, while all the other data elements may be unknown.

A VTS Targets message can either be sent as an addressed message to one or more target(s), or as a broadcast message sent to all targets that are inside the VHF coverage area of one or more AIS base station(s).

An example of a case where the VTS Targets message can be useful is when a ship’s radar is malfunctioning. If this ship is equipped with an AIS transponder that is connected to an ECDIS/ECS, and it is inside a VTS-area then the VTS operator can send for instance all the radar tracked targets that are inside a circle around this specific ship. The onboard chart display will then show all the targets around this ship, both the ones with AIS transponder and the ones without.

A VTS-system should allow the operator to define an area (like a circle with a given radius around a specific ship) in which the system should pick out all targets that do not carry an AIS transponder to form a list of so-called VTS targets. The area could be a circle around a ship, a circle around a given position or a defined polygon. If an area contains more than 7 VTS Targets the VTS-system would send several consecutive VTS Targets messages, each containing maximum 7 targets. The VTS Targets messages could be sent as a one-shot or as periodical updates.

### Limitations to the use of VTS Targets message

Number of targets, number of messages recommended to transmit during a time period limits the usability.

## Short safety-related messages

Short safety-related messages are fixed or free format text messages addressed either to a specified destination (MMSI) or all ships in the area. Their content should be relevant to the safety of navigation, e.g. an iceberg sighted or a buoy not on station. Messages should be kept as short as possible. The system allows up to 158 characters per message but the shorter the message the more easily it will find free space for transmission. At present these messages are not further regulated, to keep all possibilities open.

Operator acknowledgement may be requested by a text message.

Short safety-related messages are only an additional means of broadcasting maritime safety information. Whilst their importance should not be underestimated, use of such messages does not remove any of the requirements of the Global Maritime Distress and Safety System  (GMDSS).

The operator should ensure that he displays and considers incoming safety-related messages and should send safety-related messages as required.

According to SOLAS regulation V/31 (Danger messages)

“The master of every ship which meets with dangerous ice, a dangerous derelict, or any other direct danger to navigation, or ...is bound to communicate the information by all the means at his disposal to ships at his vicinity, and also to the competent authorities...”.

Normally this is done via VHF voice communication, but “by all the means” now implies the additional use of the AIS short messages application, which has the advantage of reducing difficulties in understanding, especially when noting down the correct position.

The AIS Service implemented in the VTS should support sending safety messages, whether it is implemented through an AIS Network or directly to specific AIS Base Stations.

Confidentiality

When entering any data manually, consideration should be given to the confidentiality of this information, especially when international agreements, rules or standards provide for the protection of navigational information.

## Available Data from AIS

Vessel Data.  Vessel data will be transmitted over the VHF marine band and will include data elements as required by international regulation.  Vessel static data will be reported as a vessel enters an AIS service area and can be updated through AIS polling techniques.  Transit-specific data will be provided continuously according to the reporting scheme defined in the international standard and will include vessel type, vessel position, speed over ground (SOG), and course over ground (COG).

Voyage Data.  The AIS service will take advantage of the vessel AIS data to monitor mandatory participants and selected vessels within the service area to ensure their compliance with regulations or adherence to stated intentions.

Anchorage Data.  The AIS service will obtain information about mandatory participant and selected vessels in or planning to enter or depart coastal zone anchorage areas, and will continuously monitor these areas.  Vessels in anchorage areas will continue to provide AIS updates at the intervals defined by international standard.

Special Activity Data.  The AIS service may obtain information on and monitor the presence of activities in the coverage area that affect port security and vessel traffic flow.

Oceanographic and Hydrographic Data.  The AIS may gather and disseminate relevant meteorological, oceanographic and hydrographic information within the AIS service area.

Port Geography and Port Structures.  The AIS service will be used to provide timely information updates on coastal zone and port geography, waterway lock and bridge openings, and shore-based marine facilities as they relate to maritime safety and security throughout the AIS service area.  Geographic data will contain parameters normally included on navigational charts.

Aids to Navigation.  The AIS will obtain and monitor the status of aids to navigation throughout the AIS service area and will report status information to users.

Rules and Regulations.  The AIS service will be used to monitor rule and regulation compliance by mandatory participant and selected vessels.

Incident Data. The AIS service may be used to communicate relevant traffic incident information to service users and maritime participants operating within service area.

Record Data.  The AIS service will record vessel track data to support port safety and security mission procedures.  The AIS service will support incident analysis to clarify the causal factors and logical sequence of events surrounding an incident, and overall mission analysis to improve maritime safety and security.

## Waypoints and Route Plan

The set of standard AIS messages comprises 2 messages called ‘Ship Waypoints and/or Route Plan Report’ and ‘Advice of Waypoints and/or Route Plan of VTS’.

### Ship Waypoints and/or Route Plan Report

This message can be used by a ship to report its waypoints and/or its route plan. The message can include up to 14 next waypoints (in the order of the intended passage), and/or a route specified by a textual description. A waypoint is given by its latitude and longitude values.

A ship can send this message as an addressed message to one or more other ships/recipients or as a broadcast to all other ships/recipients within VHF range of the ship. A VTS could be a typical recipient of such messages. The VTS-system should, as an option, be able to present the waypoints graphically on the chart. If a textual route indication is received this should be presented in relation to the ship specific (voyage-related) data.

This message can be useful for a VTS-operator for surveying or planning the traffic situation. It can also be used as a basis for the operator to instruct the ship to correct its planned route.

### Advice of Waypoints and/or Route Plan of VTS

(Waypoints and Route Plan are currently not approved by IMO as a binary message)

This message can be used by a VTS-operator to advice a ship to follow a specified navigation route, given by a set of waypoints and/or by a textual route name. The message can include up to 12 advised waypoints and/or a route specified by a textual description. A waypoint is given by its latitude and longitude values. If waypoints are transmitted, then a recommended turning radius may be transmitted for each waypoint.

A VTS-operator can send this message as an addressed message to or more ships or as a broadcast to all ships within VHF range of one or more AIS base stations. The VTS-system should offer functionality for easy and efficient definition of an advised route to be sent by this AIS message, for instance by being able to define the waypoints directly in the chart.

This message is useful for a VTS-operator for advising ships to use certain sailing routes within the VTS area. This is also useful for navigation crews approaching a harbour or a strait for the first time. In order to be really user friendly the chart displays on both the VTS-centre and the ship must be interfaced to the AIS-system.

## Graphical presentation

The presentation of AIS-data can be done in several ways, spanning from the target list on a typical MKD-unit (Minimal Keyboard and Display) that is the minimum required user interface for an AIS transponder to the more advanced chart- and symbol based displays on modern VTS- and ECDIS-systems.

It should be noted that there may be different requirements related to the graphical presentation at a shore-based VTS-system than at an onboard ECDIS/ECS systems. The primary focus for the users of these two kinds of systems is different although overlapping. A VTS-operator is most concerned about the overall traffic situation inside his/her responsibility area, and the detection of any deviances from the normal and safe navigation of any of the ships. The captain or mate is most concerned about his/her own navigation in relation to the other ships and the fairway (above and under sea-level) in a given sector a certain distance ahead.

### Symbol usage

The IALA Guidelines on AIS as a VTS Tool (December 2001) includes a description of the recommended AIS target symbols, but these are originally intended for the onboard ECDIS/ECS systems. As the guideline points out the given symbols are not necessarily adequate in the VTS context. The main difference is that a VTS-operator may prefer a much wider range of information than is necessary onboard a ship. For example traffic management may necessitate the use of symbols which depict different types and sizes of vessels. Alternatively, it may be necessary to show which vessel have pilots embarked, and which do not.

A VTS-system should include a configurable set of AIS target symbols that corresponds to the following 6 categories:

- Vessels with Class A AIS transponder

- Vessels with Class B AIS transponder

- AIS base stations

- SAR aircrafts with AIS transponder

- AtoNs with AIS transponder

- VTS Targets

When a new target shall be displayed the VTS-system can select one of the 6 categories by looking at the type of AIS messages that is received. When the category of a new target is established the VTS could also select among a list of targets dependent on the type of ship as given by for instance the AIS-message for Ship Static and Voyage Related Data. If the VTS-system comprises a vessel database, the type of target can also be looked up from there based on the target ID (MMSI, IMO no or Callsign). This means that dedicated symbols can be used for different ship types like passenger ship, cargo ships, tanker, high-speed crafts, tugs, etc.

A relative indication of the length and beam (or even the draught and height over keel for 3D graphical presentations) can be given similarly by using AIS-provided data optionally supplemented by database lookups. Such size indications must be scaled according to the range (usage level) of the current chart view.

The symbol orientation on the chart should be according to the heading indicator, and the speed-vector should indicate the course (COG), the speed (SOG) and the rate of turn (if applicable). The length of the speed-vector should depict where the target will be positioned after a certain amount of time, i.e. xx minutes from current time where xx should be easy to change when ranging in or out in the chart.

A target should be marked as lost when no AIS-messages have been received from that target the last 200 seconds, and the symbol should be removed from the chart after 6 minutes. A target is marked as lost by adding a solid line across the symbol or by replacing the symbol by an x-sign. No speed-vector or rate of turn indicator will be shown for lost targets.

The symbol should get a foreground colour and a background colour to code the following types of situations:

- System calculated warnings and alarms for a target

- Operational statuses for a target as configured at a given installation

A symbol should start to flash when an alarm is initiated, and continue to do so until an operator acknowledges the alarm. Red foreground colour should be used to indicate an alarm, and the colour should remain as long as the alarm situation is present.

### Interaction with radar tracks

A target that is tracked by radar and also carries an AIS transponder will normally be displayed with one symbol based on a fusion of the dynamic information received from the two sensor types. However, the user should have the option to override the integration and display the input from each sensor with two different symbols. It should also be possible to, for a selected target, decide whether to use both sensors integrated, both sensors independently, only AIS input or only radar input. It should be clearly indicated if the user has chosen to override the default functionality, which is the integration (fusion).

## Data Validity

Implementation of AIS in several VTS’es has indicated instances of erroneous data received from AIS Mobile units. This is an important issue as it implies caution has to be taken when using AIS data in a VTS environment.

### Potential hazards

As erroneous data occurs, this might be hazardous to a system. Data has been seen to have the correct checksums and transmits on the AIS channels. However, decoding of the data leads to useless information where the messages are proved to not follow the AIS standard. In some instances messages are also found to include more bits than it should. As data systems interacting with AIS data relies on the standard protocol, there are potential hazards when receiving this type of data.

### Data handling problems

If data received from the AIS is being used for further data processing in management and information systems, the data content needs to be accurate. Automatic processing using wrong data will most certainly lead to a system containing useless data. Until further regulations dictate what data shall be correct in the AIS mobile units, caution has to be takes when using AIS data for processing.

### Data fusion problems

In a VTS environment, the data fusion and presentation is important for the safety within the area. When erroneous information occurs, this might inflict on the operators ability to get a correct view of the situation. Typical situations are when certain on board sensors are malfunctioning, or the AIS mobile has been misconfigured. This could lead to potential dangerous situations where the Vessel reports a wrong position or heading. The information from a radar based tracking system is important as a confirmation of the AIS information received. Even if the different systems might not have the same coverage, a VTS should not rely solely on AIS for critical areas of the VTS.

## Use of binary messages for data exchange

Number of passengers, height over keel, VTS Targets, Waypoints and/or Route Plan, and optionally additional locally defined messages.

# Security