DRAFT GUIDELINES ON THE EXCHANGE OF REAL TIME DATA BETWEEN VTS SYSTEMS

# Introduction

There is an increasing interest in exchanging data between VTS systems, although it needs to be recognised that it will form only a part of the overall data flow into and out of a VTS.  These systems may be locally or regionally grouped and will be using a variety of sources (including external databases) and which may be site dependent / application specific.

Even if several VTS systems have been interfaced, there is currently no standard available covering the different types of data available in a VTS.  To make a standard describing data exchange between VTS systems, it is important to choose a way forward that will take into account the ever evolving needs of new types of information required to be exchanged.  In addition, a static description will only lead to proprietary solutions needed to cater for different types of data not covered by it.  However, it is necessary to create a baseline so that interfaces between different vendors will work together without further refinement.

# Objective

This paper provides guidance on the content and type of real time data which may be exchanged between VTS systems and does not seek to address the wider scope of all information likely to be exchanged.

# Data exchange

It is highly desirable to exchange validated data to avoid operational conflicts between VTS systems.  It is essential that the system network architecture prevents data loops.

# TYPES / CLASSES OF DATA THAT MAY BE EXCHANGED

These may be contained in the following types of data :

## Real Time Track information

This is dynamic information related to a target being followed by a tracking system, which may be derived from multiple sensors and other sources.  Currently, the most common sources are radar and AIS.  Position, course and speed over ground, and identification with a few additions comprise the parameter set involved in tracking.

The message is described in the table below. The message purpose is to enable continuous tracking and handover between the adjacent VTS systems.

The following messages are identified

- Dynamic target data

- Basic static target data

The output is based on XML but may be compressed before sending to clients. The compression algorithm needs to be defined. Gzip?

Extended static information e.g. sailingplans are not part of this interface but require a different interface probably identical to Safesea Net.

### Dynamic data

| Header |  |  |  |
| --- | --- | --- | --- |
| Timetag | mm.ss | Minutes and seconds from UTC timestamp | 00.00 59.59 |
| Message ID |  |  |  |
| Datablock |  |  |  |
| Target ID type | M, I, C | MMSI, IMO or Callsign | Char |
| Target ID data | <string> | either MMSI, IMO number or Callsign | up to 9 char |
| Target ID internal | <string> | internal track id | ?? |
| Target timetag | mm.ss | Minutes and seconds from UTC timestamp | 00.00 59.59 |
| Target source | A,R,F,P,O | AIS, Radar, Fused, Predicted, Other | Char |
| Target quality | L,M,H | Low, Medium, High | Char |
| Target position | pDDD:MM.mmmm, pDDD:MM.mmmm | p=+/- Degrees,minutes, fraction of minutes | string? |
| Target SOG | KKK.k | speed over ground (knots) | digit |
| Target COG | DDD.d | course over ground 0-360 deg | digit |

### Basic static data

Static message (taken from AIS msg. 5) (to be sent prior to the first dynamic message, and then every 6th minute):

- MMSI

- IMO-No

- Callsign

- Name

- Type of ship and cargo

- Length

- Beam

- Draught

- Destination

- ETA

- Version no (for the interface)

The area of interest (zone or line) will be defined by the serving side.

When a ship enters the zone, or passes the line, the latest static information is sent as a static message to the other VTS. Then you start to send the dynamic messages for this ship at the senders defined update rate. And continue with sending the static message for this ship every 6th minute.

We need to analyse the data traffic and the requirements for data bandwidth, and suggest a minimum bandwidth.

We also need to decide upon what to do when you get into data congestion, i.e. queuing.

### Considerations relating to track data exchange

Track data needs special consideration because it is intended to be integrated into the receiving processing system and so latency is critical.

Track data exchange is used to :

- Extend the coverage of own system;

- Avoid repeated identification of targets which have already been identified by the neighbouring system.

The following data set has been identified as typical for the integration, correlation and display in the receiving VTS system as well as for the storage in an associated database system :

- Ship identification (e.g., IMO number or MMSI)

- Name of vessel;

- Call Sign;

- Position (Latitude / Longitude);

- Course (COG);

- Speed (SOG);

- Heading (if available from AIS);

- Time stamp (time must be synchronized between VTCs);

- Data source;

- Quality figure.

From the list above, the minimum data set is considered to be 1, 4, and 8.

A quality figure is an assessment of the reliablity of the transmitted track data.  There is currently no existing standard for such a quality figure.  However, as a preliminary indicator, the sensor data source should suffice.

## Meteorological and hydrological information

This information can vary according to the different types of sensors and information, although NMEA has provided a form of standardisation.

## Digitised radar video data

The radar video data is used both for presentation and as the basis for tracking in a VTS system.  The tracking system should take care of all targets of interest in a system and therefore the exchange of radar video between VTSs generally is not required.  However, there might be different levels of interest, i.e. a commercial operator may look for something different to a military / security focused operator.

## Voyage related and static ship information

Although not real time data, voyage related and static ship information are subject to  change and may be useful to adjacent VTSs. This information is not necessarily restricted to that provided by AIS and may be application-specific to regional or national requirements.  For a comprehensive list of AIS data elements, refer to ITU-R Recommendation M.1371-1.

# TECHNOLOGIES

There are several non-proprietary and internationally recognized methods of data exchange, including NMEA, EDI, XML, TCP/IP, as well as NATO standards.  A format suitable for real time data exchange needs to be selected.

The eXtensible Markup Language -- has emerged as a standard for data representation and exchange. The basic ideas underlying XML are simple: tags on data elements identify the meaning of the data, rather than, e.g., specifying how the data should be formatted, and relationships between data elements are provided via simple nesting and references.  Servers and applications encoding their data in XML can quickly make their information available in a simple and usable format, and such information providers can interoperate easily. Information content is separated from information rendering, making it easy to provide multiple views of the same data.

Using XML schemas for message validation will make the transfer of data easier. The schemas can be made part of the standard.  Messages can be checked against schemas, ensuring the validity of the data.

# Abbreviations and definitions

AIS	Automatic Identification System

COG	Course Over Ground

EDI	Electronic Data Interchange

IMO	International Maritime Organisation

ITU-R	International Telecommunications Union – Radiocommunications Bureau

NATO	North Atlantic Treaty Organisation

NMEA	National Marine Electronics Association

SOG	Speed Over Ground

TCP/IP	Transmission Control Protocol / Internet Protocol

VTS	Vessel Traffic Services

XML	eXtensible Markup Language

- References

- IALA VTS Manual, 2002	3.3.4.2

- IALA VTS Manual, 2002	4.3.2

- ITU-R M.1371-1 Technical Characteristics of Automatic Identification Systems using Self-organizing Time Division Multiple Access

- Regional Directives, e.g. EU Directive 2002/59

- XML - Recommendation of W3C dated 4 Feb 2004