# CHAPTER 13:   VTS INFORMATION

1301  Introduction

The purpose of this chapter is to draw attention to the requirements for promulgating information about vessel traffic services and to provide advice as to the information that might be considered appropriate for publication.

## 1302  Requirement

The IMO has set out the requirement for the publication of VTS information as follows:

The VTS Authority should, in a timely manner, provide mariners with full details of the requirements to be met and the procedures to be followed in the VTS area. This information should include the categories of vessels required or expected to participate; radio frequencies to be used for reporting; areas of applicability; the times and geographical positions for submitting reports; the format and content of the required reports; the VTS authority responsible for the operation of the service; any information, advice or instructions to be provided to participating ships; and the types and level of services available. This information should be published in the appropriate nautical publications and in the "World VTS Guide".                                                     IMO Resolution A.857(20)

In setting this requirement it was recognized that VTS information is published in a variety of nautical publications, including the IALA/IMPA/IFSMA/IAPH/IHMA World VTS Guide, that are widely made available to the mariner, and by individual VTS authorities.  In the latter case the data provided may refer to only the local VTS area.

It is important that mariners have ready access to the range of information and procedures that they may be required to adhere to when entering or passing through a VTS area.

## 1303  Promulgation of Information

VTS authorities are advised to consider the extent and means of publishing information about the services that they provide.  In particular, VTS authorities should ensure that all potential marine users of their services are fully briefed about the facilities available and the requirements to report information about vessels and their movements when approaching or entering a VTS area.  It is recommended that VTS Authorities should maintain up-to-date entries in the World VTS Guide, the appropriate Lists of Radio Signals and marine publications about ports and port entry.  Additionally, it is recommended that other, more immediate means of promulgation should be considered: such as a website, e-mail or other text transfer media, recognizing that an appropriate degree of security may be required to avoid the possibility of malicious action.

## 1304  Information

The following list of headings is intended as a guide to the type of information that might need to be promulgated:

| Title of VTS Description VTS area VTS sectors VTS centre Communications Reporting Reporting points Callsigns Hours of operation Radar surveillance Types of Service VTS Procedures Accident procedures Information Broadcasts Amplifying Notes Diagrams | The name of the VTS or VTS area. Brief overview of the services and whether participation is compulsory or mandatory and to which vessels it applies. Define the area boundaries Where an area is sub-divided into separate operational sectors provide details of boundaries Define location (s) and details of VTS centre(s) (Location, Telephone, Fax and e-mail identities) VHF channels to be used. Other communications channels Language, or languages that may be used. Circumstances when SMCP is required Details of reports required and when these should be made. Identify geographical locations at which reports are to be made. Callsigns to be used where this differs from the VTS name. Sector or local area callsigns. Where services are not provided on a continuous basis specify hours of operation. Radar coverage and, if appropriate, its availability when less than ‘continuous’. Types of service provided (INS, NAS or TOS) Procedures for vessels entering, transiting and departing VTS area and/or VTS sectors. Procedures for vessels involved in specific accidents or emergencies. Details of the schedule, content and purpose of local broadcasts to shipping. Local regulations and practice, and other relevant items not included above Include diagrams to indicate key features of the VTS area, anchorages and berths, with particular reference to dangers and areas to avoid. |
| --- | --- |

# 1305  World VTS Guide

VTS authorities that have entries in the World VTS Guide are requested, in addition to providing the information listed above, to forward information in the format shown at Annex A to this chapter.

Annex A - Categorisation of VTS

| 1. Service Availability |  |
| --- | --- |
| X = 24 hours |  |
| Y – Other, for example Code Number | Explanation |
| 1 | 24 hours except holidays |
| 2 | 24 hours weekdays only |
| 3 | All others (between the hours specified) |

An entry of Y3(0900-1700) indicates a service that is provided only between 0900 and 1700 daily.

| 2. Traffic Image Generation |  |
| --- | --- |
| Indicated by “T” and code number from table |  |
| Code Number | Explanation |
| 1 | Automatic Identification System (AIS) |
| 2 | Real-time Tracking |
| 3 | Radar |
| 4 | Closed Circuit Television (CCTV) |
| 5 | Visual |
| 6 | Radio Direction Finder (RDF) |
| 7 | VHF Radio Position Reporting (no other sensors) |
| 8 | Satellite Position Reporting |
| 9 | Satellite Surveillance |

An entry of T136 indicating that the service provided incorporates the following: AIS; RADAR, and RDF.

| 3. VTS type of Service |  |
| --- | --- |
| Information Service | Indicated by INS |
| Navigation Assistance Service | Indicated by NAS |
| Traffic Organisation Service | Indicated by TOS |

| 4. Availability to Allied Services |  |
| --- | --- |
| A service that includes information exchange with other VTS or allied services. | Indicated by AS |

Example of the Code

## “VTS/Y3(0900-1700)/T136/INS+TOS/AS”

This example describes a VTS that is available from 0900 to 1700 daily; that builds its traffic image using AIS, radar and RDF, that provides information and traffic organisation services, and has communications with other VTSs or allied services.