VTS – Committee TWG

VTS21 Intersessional Group

# Data Exchange between VTS Systems

1. Definitions:

Vessel Traffic Services System (VTS-System): This paper  defines a VTS System as closed system operated from one or more Vessel Traffic Centres in charge of vessel Traffic services for the designated area.

2. Abstract:

This paper describes the data exchange between VTS Systems and not between Vessel Traffic Centres.

The data exchange between VTS—Systems shall support a number of different applications  using different data sets. These may be composed of track data, ships data, meteorological data , hydrological data. For that reason a data exchange between VTS –Systems should not define application specific sets of data. Instead, the user composes the application specific data set himself.  This principle only works if every data object in the data set

- has a unique identifier

- has unique path leading to the data object

the that is known to both the sender and the recipient.  In that case each data object of the set can be addressed separately. Only the application specific data will be transmitted. No need to define a data set for every application or to transmit long data set that may be needed.

The two variants A and B with the corresponding standards are compared in general.

The data exchange for variant a in described in more detail

An interface between systems is fully specified by the definition of the layers in the  ISO Open System Interconnection (OSI) model. This paper refers to the ISO /OSI model.

3. Requirement for data exchange:

- Some application need near real time data exchange (latency below 1 second)

- Some applications  need  to exchange extensive amount of data – data file exchange

- No proprietary exchange format wanted

#### Comparison of XML and IEC61162-1

XML:

XML has the capability for a defined data-exchange in the net-world. For the data-exchange in a net-world it needs a lots of definition and rules do deal even with short messages.

The data-format is defined in a tree structure. Both sender and recipient use this structure as a key to decode the messages.  The receiving parser reads the message, checks the tree structure and interprets the data. Access to a single tree-element is possible.  To address a single element the entire tree path has to be transmitted.

IEC 61162 (NMEA is a derivative of  IEC 61162-1):

The IEC 61162 is for local, point to point and low data rate usage. The header and trailer is shorter than more universal XML- network frame. The standardisation for the AIS –Service   interface bases on the IEC 61162 principles. But the data presentation is in ASCII instead of the binary format of NMEA. The ASCII- presentation adds a certain amount of data, compared to binary. In the future AIS-service interface each data object will have its identifier, so that each data object can be directly addressed.  The identifier is sufficient and not so  byte consuming tree path information like in XML is required. The ASCII presentation can be displayed on any standard terminal without decoding.

# Description for IEC 61162

1. Application, Presentation, Session Layer:

A. Principle:

A.1. Data Object Identifier:

Each data object gets an identifier - the data object identifier (DOI). The DOI is the address of each data object.

The data object identifier is predefined by the data exchange partners. During the data exchange the DOI and the value of the data object are transmitted. The recipient can identify the data object by knowing the DOI.

The data set for a specific application is combined to string of data objects in the following manner:

“ DIO:value, DOI:value;value, DOI:value” .

The string is framed by some header and trailer.

A.2 Data  Classes:

The data objects should be structured in classes, subclasses, data objects and data elements rather then to define them randomly.

Example for tree structure (XML-like) of data classes:

Data Classes:

-  Traffic Object data class  ( def.: all object on water with relevance to traffic, e.g. vessel, floatings, buoys...)

Subclasses:

- Dynamic data class

- Voyage related data class

- Static data class

- Chart element data class:  (def.: static geographical abstract object e.g. restricted areas, designated routes, traffic separation schemes, radar lines, way points)

Subclasses:

- Polygons, Sets of geographical positions

- Identifier for geographical object

- Met data class:

- Meteorological data objects

- Hydrological data class

- Hydrological data objects

- Containers for user defined data ( files or free text)

The coding of a DIO is described below:

First digit:	Class

Second digit	1. subclass

Third digit	2. Subclass

Fourth	digit:    3.Subcals

Fifth digit:	Data Object

5. Application layer

A. Data frame:

For the application layer definition of the data frame see details in IEC 61162-1  (NMEA is a derivate of IEC 61162-1). The data frame of message comprises of a header and a trailer . The header comprises of the Msg Id, Msg Sequence No, TX-Time, sender Id , recipient Id. The trailer includes a CR,LF and  checksum.

6. Presentation layer:

The data presentation is ASCII-Code. All data of one message is lined up in one string and while terminated by a CR, LF presented as one line of text. The message can be dumped to a standard terminal and is readable on an HMI.

7. Session Layer#3

t.b.d.

# XML or IEC 61162  (The description of layer #1 thru #4 are independent from XML or IEC 61162 use )

8. Transport Layer#4:

It may be necessary for the data exchange partners to agree on a security standard  to prevent illegal access to private data or data of commercial value for safety or security purposes. Encryption methods or private nets should be considered.

9. Network Layer#3:

On the link layer a standard should be recommended

10. Link Layer#2

On the link layer a standard should be recommended

11. Physical layer#1:

The physical connection between VTS Systems is very much dependent on local requirements- no recommendation

## To do:

- Decision for XML , IEC 61162 or other

- Recommendation for layers 1 thru 5

- Finalise the Data model  for   dynamic data and define each DOI(IEC 61162 only)

- Finalise the Data model  for	voyage data and define each DOI (IEC 61162 only)

- Finalise the Data model for	static data and define each DOI (IEC 61162 only)

- Finalise the Data model  for	met data and define each DOI (IEC 61162 only)

- Finalise the Data model  for	hydro data and define each DOI (IEC 61162 only)

- Finalise the Data model for	chart data and define each DOI (IEC 61162 only)

- Recommendation on often used applications

- Define reference standards on data object presentation ( ASCII, UNI-code)

- Define reference standards on data object attributes  (units..)presentation)

Author:  Dirk Eckhoff, Dez.VT, WSD Nord u. Nordwest , 01.02.05

Revision: Juergen Meine, Altlas Elektronik

Attachment 1

12. Example for Application “Track Data of Single Traffic Object”

Description of  a Message:

{Header Fields, Data Fields, Trailer Field}

Details:

$

Header Field: 		$Msg-SequenceNo,SenderID,ReceiverID,Msg-GenerationTime

Data Field: 		,Data-Field1, Data-Field2, ..., Data-Field n,

Trailer field: 		*Checksum, Terminal Commands

More detailed:

Header Field:  	$Msg-SequenceNo,SenderID,ReceiverID,Msg-GenerationTime

Data Field:  ,		DOI1:Value, DOI2:Value, DOI3:Value,Value,Value,  ..., DOIn:Value

Trailer field: 		*Checksum<CR>,<LF>

More detailed:

$ <Header>,

110000:20051231;23595999,

TrackGen Time: date;time

113121:12345678910,

TO- Id : IMO No

113111:89595999;359595999,

Pos	: Lat, 		Long

113118:99;

Track Source: Radar#19

113119:9

Track Quality:9

<Trailer>

The bytes  used for frame and data object identification: 6 x 6 (Header+ Trailer), 6 digit per DOI, 1 Digit =6 Bit (ASCII):

6 x 6 + 5 x 6 x 6 Bit  /  8 =  28 Byte

Bytes for used for the Values:  40 Byte

Examples for Applications:

- Basis Traffic Object Static and Voyage related data

- Basic Track Data

- Basic track Table

#### Application - Basic Real Time Data for a single Traffic Object

$<Header>

Track Generation Time Stamp

Traffic Object ID

Track ID

Position (Lat / Lon)

COG	?

SOG	?

HDG	?

ROT	?

Track Service

#### Application – Track table for max. n Traffic Object

(Predefine max. number of Traffic Object in table )

$<Header>

Time Stamp for  the Generation of the Track table	(for the total table or for each object?)

### Traffic Object ID #1

Track ID #1

Position #1

Track Source #1

Quality #1

### Traffic Object ID #2

Track ID #2

Position #2

Track Source #2

Quality #2

### Traffic Object ID #n

Track ID #n

Position #n

Track Source #n

Quality #n