VTS22/OPSWG/WP10rev1

VTS Manual Chapter 8

# 8	Planning and Organisation of VTS

Chapter 7 identified the methodology for determining the need for VTS. This Chapter addresses the issues involved in planning the subsequent organisation of a VTS.

Geography

The following consideration will need to be taken into consideration establishing the limits of the VTS Area and its division into VTS sectors:

  - Local geography: The local geography will be the determining influence on the size of the area to be covered by a VTS.  In the case of ports these vary enormously in their geography.  Some ports are extremely simple and are little more than an indentation in the coast protected by breakwaters.  Entry and exit is through a passage between the breakwater heads, which give direct access to the open sea.  Vessels are only restricted in their freedom to manoeuvre as they pass through the breakwater and into the port itself.  At the other extreme are estuarial ports, often far from the open sea with long approaches encumbered by shallow, shifting sandbanks.  Vessels using these ports will be restricted navigationally and possibly be unable to anchor or reverse course over long stretches of their passage.

  - Traffic separation schemes: The existence or addition of traffic separation schemes within or adjacent to the VTS area may be to be taken into account.

  - Anchorages: Consideration should be given to the designation of anchorages or anchorages areas.

  - Local Conditions:  The prevailing weather, in particular visibility and wind together with the tidal range and stream, may impose difficulties on the ability to navigate safely.  Together with the local geography, they determine the degree of navigational difficulty likely to be encountered by a vessel. An appreciation of these physical factors, plus any interface with local or regional services, leads to a first assessment of the VTS facilities that would be of most benefit to vessels navigating in the area.

  - Numbers of vessels and types:  The numbers of vessels, including local traffic, and their type is significant.  A simple count of vessels, although of value, is not sufficient.  The vessels need to be considered with regard to their size, type, equipment, manoeuvrability, spatial distribution and cargo so that the optimum service meeting the needs of all users and without placing unnecessary constraints on the movement of any of the vessels can be identified.

  - Commercial factors:  Any VTS must make due allowance for any potential conflict between safety and commercial operation and pre-empt such conflicts before they arise.  Ports must operate in an efficient and timely manner and meet the needs of their users, but this must be done without impinging on the safe operation of the port.  The distribution of ship arrivals and departures may be an important factor influencing the port management resources.  Unannounced arrivals and departures can have a considerable and adverse effect on the viability of a port.  Some ports, such as ferry ports and container terminals, operate to a schedule which has to be maintained in virtually all weathers.

  - Other Activities: Naval operations, oil and gas production and recreational activities may take place within the area to be covered by a Vessel Traffic Service.  These activities will also influence the operation of the scheme and must be taken into account.  A good working relationship needs to be established and maintained with other users of the area.

  - Offshore Structures.  Offshore structures, particularly the increasing pressure to site windfarms close to navigable channels may need to be considered in the management of vessel traffic.  The impact of such structures on both shore based and marine radars should be carefully considered.

Hydrography

The size of the VTS area and the proximity of hazards and dangers will be  key considerations in assessing the positional and navigational accuracy requirements in a VTS system. Advice and recommendations on navigational accuracy requirements are documented in IMO …[reference to be added].

Modern digital charting offers the opportunity of providing greater accuracy and the choice on the level of detail that is provided on the background to the traffic image.  Guidelines on the use of symbology  is provided in IALA “ Guidelines ….”. Care should also be taken to ensure that all data inputs such as AIS and radars are aligned to a common datum.

Type of VTS/ Service Categories

Having taken into account the geographical area, traffic density and  traffic pattern, the VTS Authority will need to consider the type of VTS and category of service (information service, navigational assistance service and/or traffic organisation service) to be provided. The types of VTS and categories of service are described in Chapter 5.

Allied Services

This is a supporting activity of the VTS intended to increase the safety and efficiency of the traffic, the protection of the environment and the effectiveness of the VTS without adding to the reporting burden of the vessel.

Co-operation with allied services is both safety and efficiency orientated.  It should be a continuous process and is of particular importance in cases where a VTS sailing plan is to be established and action agreement between services is required.  Procedures for the co-operation between parties should be established.

Incidental co-operation with emergency services such as Search and Rescue and Pollution Control should be conducted in accordance with pre-established contingency plans in which the procedures for the co-operations are laid down and responsibilities established.

Co-operation between vessel traffic services can be of particular interest where two such services share a common border because they may need to coordinate jointly with the master of a ship when the VTS sailing plan is being agreed.  In other cases it should be recognised that the exchange of data between vessel traffic services could give advance notice of arrivals thus relieving the reporting burden of vessels.  It could also provide an Administration with valuable information on future traffic and cargo flows in its intermediate sea area.

Operational Management

The provision of the following capabilities may need to be considered in the planning and organization of a VTS

- Marine Communications.  The number of sectors will determine the requirement for frequency allocation in a limited VHF marine communications band.  Application to regulating authorities will be required and consideration should be given to frequency allocations in adjacent areas to minimise interference.

- Prohibited or Dangerous areas. Vessel traffic may need to be kept clear of areas of ecological significance or other hazards. This may influence the routing of traffic and the key points for surveillance and traffic monitoring.

- Places of Safety. It may be prudent to identify potential “Places of Safety” to cater for marine emergencies at local and national level.

- Separation. Safety of navigation can be enhanced in particularly sensitive areas or confined/restricted waters through separation techniques. This may be achieved by:

  - Time separation. Time separation is achieved by a vessel having exclusive use of a certain area or a restricted passage for a given time span.  The time slots may  be allocated as part of a VTS sailing plan.

  - Distance separation. Distance separation is a method whereby vessels are given a minimum distance between each other in order to transit the whole or certain areas and restricted passages.  The separation distances to be maintained are allocated and monitored by the VTS centre and may differ depending upon the categories of vessels or the cargo which is carried.  Overtaking restrictions and/or minimum passing distances may be part of this method of traffic organisation.

- Emergency and Incident Management.  Configuration of a VTS Centre should take into account the need to manage incidents and emergencies.  Issues that should be addressed include:

  - Workstation(s).  Provision should be made for additional staff to manage the specific incident whilst the VTS continues with the primary traffic management function.  This may be in the form of dormant workstations or a plan to reconfigure existing positions to make best use of the facilities available.

  - Planning.  Contingency plans and action sheets should be prepared.

  - Liaison.  Consideration should be given to the links that may be necessary with emergency services including the coastguard.

  - Training.   Contingency plans should be exercised.

- Pollution Control.  Pollution is a specific concern resulting form an incident or emergency that may have far reaching consequences for a port or coastal VTS.  In addition to the measures above, consideration my need to be given to the control of pollution.  Prevention measures may include special regulations and controls for vessels carrying Hazardous cargos which should be addressed in the planning of the VTS.

Internal Organization

Having identified the VTS Area, the number of Sectors and the Types of Service to be provided, the manning of the VTS can then be addressed.  The number of Sectors and the shift patterns will dictate the number of VTS Operators required and the complexity of the VTS will determine the need for a VTS Supervisor.

Other functions such as the management of Allied Services may be carried out from the VTS Centre and additional personnel may be required to handled these additional tasks in order to prevent VTS Operators from being diverted from their primary responsibility for the Safety of Navigation.

The VTS organisation must be firmly backed by documented administrative processes and operational procedures.  These are covered more fully in later chapters.

Legal  Basis

The legal framework for VTS is covered in Chapter 2.  From these international  and national regulations including matters such as Traffic Separation Schems, the legal basis for the VTS at a local level will need to be determined.

All VTS personnel should be aware of the legal basis under which they are operating.  In planning a VTS, the powers and authority delegated to individual operators will need to be established by the VTS Authority.  The following will need to be addressed:

- The type of service that may be offered by individual operators.  This is particularly relevant in respect of Navigational Assistance Services which may require specific authorisation.

- To whom the power to issue compulsory directions has been delegated if any.

- How to process and to whom to report infringements of regulations.

- Powers of enforcement.

- VTS and Operator liability