IALA Guideline

Maritime Service Portfolios:
digitising maritime services

Edition 1.0

Document date

Revisions to this IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

1.	INTRODUCTION	10

1.1.	General Description	10

1.2.	Purpose	10

1.3.	Implementation	10

1.3.1.	Change process	10

1.4.	Technical Issues	12

2.	Governing body, SERVICE PROVIDERS & STAKEHOLDERS	16

2.1.	Definitions	16

2.2.	Responsible service providers [to be decided later]	16

3.	Defined sea areas for MSP's	17

4.MARITIME SERVICES	18

4.1.	MS 1 VTS Information Service (INS)	18

4.1.1.	Submitting Organisation	18

4.1.2.	Description of the Digital Maritime Service	18

4.1.3.	Purpose	19

4.1.4.	Operational Approach	19

4.1.5.	User Needs	19

4.1.6.	Information to be Provided	20

4.1.7.	Associated Technical Services	20

4.1.8.	Relationship To Other DMSs	20

4.2.	MS 2 Navigational Assistance Service (NAS)	22

4.2.1.	Submitting Organisation	22

4.2.2.	Description of the Digital Maritime Service	22

4.2.3.	Purpose	22

4.2.4.	Operational Approach	23

4.2.5.	User Needs	23

4.2.6.	Information to be provided	24

4.2.7.	Associated Technical Services	25

4.2.8.	Relation to other DMSs	25

4.3.	MS 3 Traffic Organization Service (TOS)	25

4.3.1.	Submitting Organisation	25

4.3.2.	Description of the Digital Maritime Service	25

4.3.3.	Purpose	27

4.3.4.	Operational Approach	27

4.3.5.	User Needs	27

4.3.6.	Information to be provided	28

4.3.7.	Associated Technical Services	28

4.3.8.	Relation to other MSPs	28

4.4.	MS 4 Local Port Service (LPS)	29

4.4.1.	Submitting Organisation	29

4.4.2.	Description of Digital Maritime Service	29

4.4.3.	Purpose	29

4.4.4.	Operational Approach	29

4.4.5.	User Needs	29

4.4.6.	Information to be Provided	29

4.4.7.	Associated Technical Services	29

4.4.8.	Relation to other Digital Maritime Services	30

4.5.	MS 5 Maritime Safety Information service (MSI)	31

4.5.1.	Submitting Organisation	31

4.5.2.	Description of Digital Maritime Service	31

4.5.3.	Purpose	31

4.5.4.	Operational Approach	32

4.5.5.	User Needs	34

4.5.6.	Information to be Provided	34

4.5.7.	Associated Technical Services	34

4.5.8.	Relation to other Digital Maritime Services	34

4.6.	MS6 Pilotage service [IMPA]	35

4.6.1.	Submitting Organisation	35

4.6.2.	Description of Digital Maritime Service	35

4.6.3.	Purpose	35

4.6.4.	Operational Approach	35

4.6.5.	User Needs	35

4.6.6.	Information to be Provided	35

4.6.7.	Associated Technical Services	35

4.6.8.	Relation to other Digital Maritime Services	35

4.7.	MS 7 Tugs service	36

4.7.1.	Submitting Organisation	36

4.7.2.	Description of Digital Maritime Service	36

4.7.3.	Purpose	36

4.7.4.	Operational Approach	36

4.7.5.	User Needs	36

4.7.6.	Information to be Provided	36

4.7.7.	Associated Technical Services	36

4.7.8.	Relation to other Digital Maritime Services	36

4.8.	MS 8 Vessel shore reporting	37

4.8.1.	Submitting Organisation	37

4.8.2.	Description of Digital Maritime Service	37

4.8.3.	Purpose	37

4.8.4.	Operational Approach	37

4.8.5.	User Needs	37

4.8.6.	Information to be Provided	38

4.8.7.	Associated Technical Services	38

4.8.8.	Relation to other Digital Maritime Services	38

4.9.	MS 9 Telemedical Assistance Service (TMAS)h	38

4.9.1.	Submitting Organisation	38

4.9.2.	Description of Digital Maritime Service	38

4.9.3.	Purpose	38

4.9.4.	Operational Approach	38

4.9.5.	User Needs	39

4.9.6.	Information to be Provided	39

4.9.7.	Associated Technical Services	39

4.9.8.	Relation to other Digital Maritime Services	39

4.10.	MS 10 Maritime Assistance Service (MAS)	40

4.10.1.	Submitting Organisation	40

4.10.2.	Description of Digital Maritime Service	40

4.10.3.	Purpose	40

4.10.4.	Operational Approach	40

4.10.5.	User Needs	40

4.10.6.	Information to be Provided	40

4.10.7.	Associated Technical Services	40

4.10.8.	Relation to other Digital Maritime Services	40

4.11.	MS 11 Nautical Chart Service	41

4.11.1.	Submitting Organisation	41

4.11.2.	Description of Digital Maritime Service	41

4.11.3.	Purpose	41

4.11.4.	Operational Approach	42

4.11.5.	User Needs	42

4.11.6.	Information to be Provided	43

4.11.7.	Associated Technical Services	43

4.11.8.	Relation to other Digital Maritime Services	44

4.12.	MS 12 Nautical publications service	45

4.12.1.	Submitting Organisation	45

4.12.2.	Description of Digital Maritime Service	45

4.12.3.	Purpose	45

4.12.4.	Scope	46

4.12.5.	Objective	46

4.12.6.	Operational Approach	47

4.12.7.	User Needs	47

4.12.8.	Information to be Provided	48

4.12.9.	Associated Technical Services	48

4.12.10.	Relation to other Digital Maritime Services	48

4.13.	MS 13 Ice navigation service WMO	49

4.13.1.	Submitting Organisation	49

4.13.2.	Description of Digital Maritime Service	49

4.13.3.	Purpose	49

4.13.4.	Operational Approach	49

4.13.5.	User Needs	50

4.13.6.	Information to be Provided	54

4.13.7.	Associated Technical Services	54

4.13.8.	Relation to other Digital Maritime Services	54

4.14.	MS 14 Meteorological information service [WMO]	55

4.14.1.	Submitting Organisation	55

4.14.2.	Description of Digital Maritime Service	55

4.14.3.	Purpose	55

4.14.4.	Operational Approach	56

4.14.5.	User Needs	57

4.14.6.	Information to be Provided	59

4.14.7.	Associated Technical Services	59

4.14.8.	Relation to other Digital Maritime Services	59

4.15.	MS 15 Real-time hydrographic and environmental information services	60

4.15.1.	Submitting Organisation	60

4.15.2.	Description of Digital Maritime Service	60

4.15.3.	Purpose	60

4.15.4.	Operational Approach	61

4.15.5.	User Needs	61

4.15.6.	Information to be Provided	61

4.15.7.	Associated Technical Services	61

4.15.8.	Relation to other Digital Maritime Services	61

4.16.	MS 16 Search and Rescue (SAR) Service	62

4.16.1.	Submitting Organisation	62

4.16.2.	Description of Digital Maritime Service	62

4.16.3.	Purpose	62

4.16.4.	Operational Approach	62

4.16.5.	User Needs	62

4.16.6.	Information to be Provided	62

4.16.7.	Associated Technical Services	62

4.16.8.	Relation to other Digital Maritime Services	62

4.17.	MS 17 Aids to Navigation services (AtoN)	64

4.17.1.	Submitting Organisation	64

4.17.2.	Description of Digital Maritime Service	64

4.17.3.	Purpose	64

4.17.4.	Operational Approach	64

4.17.5.	User Needs	64

4.17.6.	Information to be Provided	64

4.17.7.	Associated Technical Services	64

4.17.8.	Relation to other Digital Maritime Services	64

4.18.	MS19 PNT and augmentation services	64

4.18.1.	Submitting Organisation	64

4.18.2.	Description of Digital Maritime Service	64

4.18.3.	Purpose	64

4.18.4.	Operational Approach	64

4.18.5.	User Needs	64

4.18.6.	Information to be Provided	64

4.18.7.	Associated Technical Services	65

4.18.8.	Relation to other Digital Maritime Services	65

6.	RELEVANT ASSOCIATED IMO GUIDELINES	72

6.1.	Guidelines on SQA and HCD	72

6.2.	Guidelines on Display of navigation information from communications	72

6.3.	Guidelines on test beds reporting	72

7.	LIST OF PUBLICATIONS THAT CAN BE DIGITAL	72

8.	ACRONYMS To be checked	72

ANNEX A	MS 1 INFORMATION SERVICE Template	74

8.1.	74

List of Tables

Table 1	Responsible Authorities	7

# INTRODUCTION

## General Description

When developing the IMO e-Navigation strategy to improve safety and efficiency of sea transport it became clear that information services provided from shore to ships are an essential part of this initiative. In order to best describe, structure and implement those services, IMO introduced the concept of “Marine Service Portfolios” (MSPs).

IA “Maritime Service Portfolio (MSP)” defines and describes  a set of operational and technical Maritime Services and their level of service provided by a stakeholder in a given sea area, waterways or ports, as appropriate. (NAV 57/6 para 23)

IMO has identified a preliminary list of 16 Maritime Services (MS). Under its remit, IALA recognised that additional MSs were needed for items such as AtoNs and PNT, which have been added to these guidelines.

In parallel to the development of MSs’ content, the IMO is working on Guidelines for harmonized display of navigation information received via communication equipment. These Guidelines aim to develop standards and specifications which will automatically display on equipment onboard MSs’ information received from a recognized an official mean of communication. This interoperability will allow a national authority to select, among e-Navigation recognized means of communication, those adapted to its own situation and capacity.

## Purpose

This Guidelines describe the operational aspects regarding the IMO e-navigation solution on maritime information services provided from shore to ship and displayed on a graphical display onboard the ship.

The descriptions are provided in cooperation between recognized international organizations and make the basic fundament for further global technical harmonization.

This guideline is mainly for providers of Maritime Services defined in MSPs to understand what is expected by the maritime community if a dedicated provider of such services is declaring the availability of an MS in their jurisdiction. It provides the basic information on the defined MSs. It describes the purpose and operational needs to be achieved with the MS as well as a short explanation of the MS. It also includes references to other MSs, which may be associated to the specific MS.

This guideline helps providers to integrate new information services, understand the operational aspects and references to the technical aspects on order to migrate from conventional to digital services but does not include technical specifications necessary for the harmonized implementation of those MSs. Those will be defined elsewhere through the respective competent bodies, but are referenced in this document for easy access. It rather provides the guidance on the overarching expectations for a service provider of a given MS.

The services described in this guideline are intended for IALA guidance, but are coordinated with international organizations responsible for a domain and can therefore also be used for other organisations and authorities planning to implement a set of services as a portfolio.

## Implementation

The Maritime Services described within this guideline can be implemented in full or in part, based on individual service providers local circumstances.

### Change process

To be successful, the transformation of existing services into digital services needs to follow a defined process. See Figure 1 below:

In the beginning there needs to be an assessment of the current situation as well as a clear definition of the intended future situation. This assessment includes review of the organization, the processes and the outputs of the two states: Current and Future.

The result of this assessment will be the input to further analysis. During the analysis phase all relevant information need to be taken into consideration. The categories of information to be analysed are technical and procedural information as well as existing experience and guidelines. The analysis phase will document all information classes of relevant and the expected outcomes in accordance with the initial assessment of the intended end state. Both a Change Process Plan as well as a Change Process Timeline will have to be created. Once those two plans are available, the execution can start in accordance to those plans.

Figure 1

For example: If a certain Competent Authority decides to change or develop a service to digital information (implementing an MS), it should take into account the following steps:

Assess the services being delivered by traditional means and the actual delivery platforms (e.g. VHF network);

Identify the services under own responsibility and the eventual need for new delivery platforms.

Identify the services to be delivered in the new format (MS) and the timeline for the implementation

Identify the providers of information for the selected MS

Establish a common workable platform between all providers. This platform must be able to allow interoperability between the different providers and systems. It must also be established a working protocol to guarantee the responsibility levels and the flow of information.

Coordinate the intended work with MS users and organizations creating tools for utilizing MS output to ensure usability by intended end users.

Implement the onshore structures (infrastructures, hardware and software) necessary for the delivery of MS portofolio.

Guarantee an experimental faze in order to allow assess the resilience of the system.  When the Competent Authority decides that the service is mature enough, it can be delivered to the mariners and be used as primary means for the delivery of that information. However, due to the existence of different levels of equipment on board of vessels, both services must be provided simultaneously.

During the transformation procedures, it is necessary to continuously evaluate the process in order to see if the service can be improved, optimized, identify gaps and implement the necessary changes. This is a continuous process, and it must be complemented with a permanent monitoring of the system (e.g. feedback from mariners and other stakeholders)

Figure 2

## Technical Issues

According to the situation, information provided electronically (digital format) could complement and/or replace voice communication. However, critical and urgent situations will most likely be better addressed through radio voice communication, which ensures that the distress call has reached out and that a proper response is triggered. For non-critical situations, information can be provided in a digital format but care should be taken to ensure that all ships have the capacity to receive and display properly the information.

Given that neither shore-based authorities nor all ships will have modernized their equipment to the digital format level at the same time, a transition period from the current provision of Information Services to the Digital Service format must be envisaged. This transition period will also allow to both ship and shore authorities to get familiar with the new technology and have confident into it.

It is also important to consider that the provision of Maritime Services in many areas is not limited to SOLAS ships but includes as well other type of vessels (domestic, fishing, pleasure craft, etc.). These vessels will need a similar period of transition, which can be longer according to the financial investment that might be required. Thus, the transition period will require that both ways of communication, radio and digital, be maintained in parallel to ensure safety of navigation, and until the Digital format proves to be as efficient and reliable for all situations (figure x).

Figure 3 – Illustration of the provision of information services over time.

Each existing MS and future new MS should make use of the methodology in Annex A for mapping and identifying what to be digital information in the future. This form will also identify existing product specifications in the s-100 registry and the GAPs that has to developed in addition.

Also, many Services are produced by other national authorities (see figure 4 below) and the conversion of information such as ice, meteorological and hydrographic data to the digital format will gradually change the role of VTS centres with respect to these data. In some cases, the digitalization process will allow to transmit directly the information to mariners without necessarily passing through a VTS centre. However, these situations will require that VTS centres have a constant access to these data in order to monitor its status and to provide the information to ships experiencing electronic problems.

Currently, there are situations where the exchange of information between shore authorities and ships is done electronically.

For example:

Provision of VTS services such as INS can be done electronically without radio voice communication;

Meteorological, hydrographic and other type of environmental data can be broadcasted directly to ships through AIS Application Specific Message (AIS ASM), without passing by a VTS centre; and

The content of the VHF communication can be transmitted electronically and be displayed as a text in parallel to voice communication.

Figure 4: Illustration of the historical communication process between VTS onshore and ship.

Over the last decades, the common practice was to “push” information to ships. This allowed VTS to have a knowledge of the status of this information and to trigger a notification procedure whenever a problem was noted. Digitalization of information introduces new ways to communicate with ships and although it will still be possible to push it through communication systems such as AIS and VDES, the capacity of ships  to “pull” it directly from internet through Web services will also exist. In both cases, the assistance of VTS may not be required. This new situation  may decrease the situational information awareness  of VTS as they won’t know whether or not information has been received by ships. In order to keep pace with the digital transformation of information service, VTS will need to have access and monitor the status of information transmitted directly to ships. Hence, if ever there is a technological failure and information service cannot be communicated to ships, VTS will be in a position to notify as well mariners as the service producer, or to provide the information to the ship.

Even though information in the near future could be made accessible directly to ships, VTS will remain the primary contact with ships for urgent and important messages, including as a back-up for electronic failure. The figure 5 illustrates some of the current and future possibilities with respect to shore  ship communication.

Figure 5: Illustration of some current and future communication processes between shore and ship.

Implementation of MS1 by a national authority will require a strong coordination with all other MS producers to clarify the mean of communication that wil be used to transmit their own  service information to mariners, and to determine how the VTS will be kept informed. Development of national procedures detailing how shore  ship communication will proceed will be essential to maintain safety to navigation.

# Governing body, SERVICE PROVIDERS & STAKEHOLDERS

## Definitions

- MSP governing body which defines and maintains the overall architecture of the MSPs, endorses the definition and scope of individual Maritime Services, ensures interoperability and consistency, etc. (the IMO/IHO HGDM could be the initial basis for defining further that structure;

- Service definition owner which proposes the definition to the governing body and then implement the agreed definition through technical specifications

- Service provider responsible for delivering an operational service according to the relevant specifications;

- Service producer refers to a national authority responsible of the collection or creation of an information or data.

- User which makes use of the information provided by the service. In some cases (i.e. MSI service), there may be a need to distinguish between the provider of the information content (i.e. a NAVAREA coordinator) and the provider of the communication infrastructure/service (i.e. SafetyNET).

- Technical service specification owner  refers to the body responsible for developing and maintaining the technical specification(s) of a service, based on the corresponding service definitions [by way of example: for VTS Information Service, technical service specification owners could be IALA and the IHO]

## Responsible service providers

In each country there will be authorities responsible for providing information services.  The table below offers examples of authorities responsible in each case, which can be different between countries.

Responsible authorities may require service providers to deliver the operational service.

Responsible Authorities

| MS No | Identified Services | Identified Responsible Service Provider |
| --- | --- | --- |
| 1 | VTS Information Service (INS) | VTS Authority |
| 2 | Navigational Assistance Service (NAS) | VTS Authority |
| 3 | Traffic Organisation Service (TOS) | VTS Authority |
| 4 | Local port Service (LPS) | Local Port/Harbour Authority |
| 5 | Maritime Safety Information (MSI) Service | National Competent Authority |
| 6 | Pilotage service | Pilotage Authority/Pilot Organization |
| 7 | Tug Service | National Competent Authority; Local Port/Harbour Authority |
| 8 | Vessel Shore Reporting | National Competent Authority and appointed service providers |
| 9 | Telemedical Assistance Service (TMAS) | National health organization / dedicated health organization |
| 10 | Maritime Assistance Service (MAS) | Coastal/Port Authority / Organization |
| 11 | Nautical Chart Service | National Hydrographic Authority / Organization |
| 12 | Nautical Publications service | National Hydrographic Authority / Organization |
| 13 | Ice navigation Service | National Competent Authority Organization |
| 14 | Meteorological information service | National Meteorological Authority Public Institutions |
| 15 | Real time hydrographic and environmental information service | National Hydrographic and Meteorological Authorities |
| 16 | Search and Rescue Service | SAR Authorities |

# Defined areas for A PORTOFOLIO OF MS’s

This guideline describes the operational aspects of each maritime service.

Services (or a portfolio of services) will be provided depending on the service type and the mandate that a service  provider has for its provision in different geographic areas.  Each service provider should ensure that the service area is clearly defined and publicised.

# Flow chart of developing a new Maritime Service

The schedule of the flow chart below reflects a recommended process of steps to be     followed, with the following clarification

These descriptions of maritime services, using the templates, should contain references to relevant international Standards, Recommendations and Guidelines, which may contain criteria for the eventual implementation of these maritime services as well as identified user requirements for data and information;

The domain coordinating bodies will submit the descriptions of maritime services, using the template in Annex xx  to the Organization as the recipient (3), using the IMO Secretariat as a mailbox for automated registration and forwarding to the right IMO body.

The assigned IMO forum (eg HGDM) will study and evaluate the submitted descriptions of maritime services, using the templates, and identify potential harmonized data models (5) and finally report to NCSR (6) with a request for consideration (7) to incorporate, where appropriate, an approved description of a maritime service, using the template, into a relevant IMO instrument (e.g. Resolution, Guideline or Circular) (8);

Once the IMO instrument has been finalized and consequently, in accordance with IMO procedures, be approved by MSC, the Organization can provide guidance to the member states and the domain organizers (relevant NGO's and IGO's) to their members (9) on how to implement maritime services.

# MARITIME SERVICES

## MS 1 VTS Information Service (INS)

### Submitting Organisation

IALA VTS Committee

### Description of the Maritime Service

A VTS Contracting Government or Governments or the competent authority should establish and set the objectives for the types of services, areas and categories of vessels required or expected to participate.

The VTS Information Service is defined by IMO as “a service to ensure that essential information becomes available in time for on-board navigational decision-making” (Res. A857(20)).

The information service is provided by broadcasting information at fixed times and intervals or when deemed necessary by the VTS or at the request of a vessel, and may include for example reports on the position, identity and intentions of other traffic; waterway conditions; weather; hazards; or any other factors that may influence the vessel's transit .(IMO Res A.857(20))

In some VTS areas the provision of VTS Information Service is not limited to SOLAS vessels and it should in include all participating vessels. Other means to promulgate safety critical information to vessels unable to receive digital information should be considered.

IALA Guideline No. 1089 defines the “Provision of Vessel Traffic Services”(INS, TOS & NAS).

Examples of the types of information that may be provided by the VTS operating an Information Service

| Information related to: | Examples: |
| --- | --- |
| Navigational situations (including traffic and route information) | Position, identity, destination of vessels and the intention of other traffic; Amendments and changes in promulgated information concerning the VTS area such as boundaries, procedures, radio frequencies, reporting points; the mandatory reporting of movements; Limited maneuverability that may impose restrictions on the navigation of other vessels, or any other potential hindrances; Suspension or change of routes; etc. |
| Navigational warnings | Dangerous wrecks, obstacles not otherwise promulgated, diving operations, vessels not under command, etc. |
| Meteorology | Information that will include the speed and direction of the prevailing wind, direction and height of the waves, visibility, atmospheric pressure, the formation of ice, etc. |
| Meteorological warnings | Gale, storm, tsunami, restricted visibility, etc. |
| Hydrography | Information that will include factors such as the stability of the seabed, sea depth, the accuracy of surveys, tidal ranges, tidal streams, prevailing currents and swell, etc. |
| Electronic navigational aids | The availability of electronic navigational aids such as: GNSS, Loran, LRIT, DGPS, AIS, RACON etc. |
| Other information | Port information, pilot or tug request, cargo information, health condition, PSC, ISPS, etc. |

### Purpose

An VTS Information Service provides relevant information at appropriate times and on request for the promulgated VTS area. An Information Service involves maintaining a traffic image and allows interaction with traffic and response to developing traffic situations. (IALA Guideline No.1089 defines the " "Provision of Vessel Traffic Services" (INS, TOS & NAS))

The purpose is to provide VTS Information Service (INS) digitally to create means to reduce administrative burden and information overload, reduce miscommunication due to external interference, simplify work procedures, and increase navigational safety.

Information provided digitally (non-verbal means) could complement and/or replace verbal/voice communication. The steps to achieve this transition to digital information exchange will vary in different areas and for different types of vessels. Depending on the categories of the Information Service in annex A, (MS 1, Information Service Template) and its discoverability, new equipment may be required both on board and on shore side. Details about information exchange available should be published by the VTS authority

### Operational Approach

Information provided (non-verbal means) could complement and/or replace verbal/voice communication.

The digitalization of information will diversify the communication means between shore authorities and vessels and will affect VTS procedures regarding information provision. VTS may no longer be the focal or relay point for all vessel-shore authority-vessel communications or the other way around VTS could have a lager information-coordinating role.

Even though information in the near future could be made accessible directly to vessels, VTS will remain the primary contact with vessels for urgent and important messages, including as a back-up for electronic failure.

### User Needs

In IMO resolution A.857(20) are examples of information that can be provided to vessels. The user case below is based on the information from table 3

#### User Case – Vessel Arrival

Before the vessel arrives to the VTS area, a data collecting system on board to send all details regarding the arrival via relevant infrastructure to the VTS. The VTS collects the vessel's data directly into its system, and automatically updates the vessel's pre-registered data. Both vessel and VTS use chart systems as a graphic interface to present details that are of interest to the voyage, such as reporting point line and VHF channel.

Upon arrival to the VTS area the vessel use a data collecting system on board, and sends Actual Time of Arrival via relevant infrastructure to VTS.

The example is generic and simplified through an example for description purposes only. Actions and template categories may differ for different countries. Content in the column named "Template Info (technical)"is pending submissions from relevant stakeholders.

The categories of services and the associated details are listed in annex 1, DMS 1 Information Service template.

| Time | Vessel Action | VTS Action | Template Info (category) | Template Info (technical) |
| --- | --- | --- | --- | --- |
| 01:00 | Provides pre-arrival info | Replying with information on weather | Environmental | Annex 1 |
| 02:00 | Enters VTS Area, provides sailing route | Traffic information to vessels | Traffic and Route information |  |
| 02:30 | Passes reporting point line | Provides information on current, wave height, etc. | Hydrographical information |  |
| 03:00 | Requires port information | Provides quay details | Traffic and Route information |  |
| 03:30 | Passes second reporting point | Provides operational information on AtoNs | Navigation Hazards |  |
| 04:00 | Vessel along side | Gives information on wind speeds, visibility | Environmental |  |

### Information to be Provided

See Annex 1, MS 1 Information Service template

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |
| Voyage Information Service | urn:mrn:stm:service:specification:sma:vis | The service supports exchange of voyage plans, text messages and area messages |  | IEC? |
| Weather Service |  |  |  |  |

[to be filled in later]

Note: Annex 1 could be complimented with required information regarding this table

### Relationship To Other MSs

MS1 has relationships with other MSs where it affects the VTS

Examples may be different depending on the coastal state arrangements.

| Description | Examples of data that could be used in MS 1 |
| --- | --- |
| MS 2 VTS NAS | See Annex 2 MS 2 Navigation Assistance Service Template |
| MS3 VTS TOS | See Annex 3 MS 3 Traffic Organisation Template |
| MS 4 Local Port Service | Delays, obstruction, cargo operations, port availability and anchorage are in the port, ISPS state Marsec level |
| MS 5 Maritime Safety Information | All information depending on the structure of MSI |
| MS 6 Pilotage Service | Pilot orders and Updates |
| MS 7 Tug Service | Tug order and updates |
| MS 8 Vessel Shore reporting | Notification of arrival, dangerous cargo etc. |
| MS 9 Telemedical | Delays |
| MS 10 Maritime Assistance Service | Notifications, routing, places of refuge |
| MS 11 Nautical Chart Service | Local area updates, chart updates |
| MS 12. Nautical Publications Service | Updates to publications |
| MS 13 Ice Navigation Service | Ice routes, ice conditions, ice breaking assistance |
| MS 14 Meteorological Service | VTS area weather |
| MS 15 Real Time Hydro and Information Service | Horizontal and vertical Tidal information in VTS area, available water column |
| MS 16 Search and Rescue Service | Search Pattern and vessel of opportunity |
| MS 17 |  |
| MS 18 |  |

## MS 2 Navigational Assistance Service (NAS)

### Submitting Organisation

IALA VTS Committee

### Description of the Maritime Service

A VTS Contracting Government or Governments or the competent authority should establish and set the objectives for the types of services, areas and categories of vessels required or expected to participate.

The VTS Navigational Assistance Service is defined by IMO as “a service to assist on-board navigational decision-making and to monitor its effects. (IMO Res.A857(20))

The navigational assistance service is especially important in difficult navigational or meteorological circumstances or in case of defects or deficiencies. This service is normally rendered at the request of a vessel or by the VTS when deemed necessary. (IMO Res.A857(20))

IALA guideline 1089 gives guidance on the delivery of the three different types of services provided by a VTS; Information Service (INS), Traffic Organization Service (TOS) and Navigational Assistance Service (NAS).

Examples of the types of information that may be provided by a VTS operating a NavigationalAssistance Service

| Information related to NAS | Examples |
| --- | --- |
| Request and identification | availability of NAS, start and end of NAS; request for ship identification such as position, course made good and speed over the ground; status of ship's equipment; etc. |
| Navigational information (including position and course information) | Examples provided to an individual vessel: provide range and bearing from fixed objects, fairway/channel or way‐points; proximity to navigational hazards, etc. provide information related to navigating into a channel/fairway/lane (i.e.track is parallel/diverging/converging with/from/to reference line); etc. |
| Advice (or instruction) | advise (or instruct) a ship to alter the course, speed; advise (or instruct) to keep clear from area/position, close up/drop backon/from vessels; etc. |
| Warning | Diverging from the recommended track towards dangerous wrecks, obstacles not otherwise promulgated; diving operations; vessels not under command; etc. |

### Purpose

The purpose is to provide information related to Navigational Assistance Service (NAS) digitally to create means to reduce administrative burden and information overload, reduce miscommunication due to external interference, simplify work procedures, and increase navigational safety.

Information provided digitally (non-verbal means) could complement voice communications in time critical situations and in addition partly replace voice communications in non-time critical situations.

The steps to achieve this transition to digital information exchange will vary in different areas and for different types of vessels. Depending on the categories of the Navigational Assistance Service in annex 2, (MS 2, Navigation Assistance Service Template) and its discoverability, new equipment may be required both on board and on shore side. Details about digital information exchange available should be published by the VTS authority.

### Operational Approach

VTS will remain the primary contact with vessels for urgent and important messages necessary for the decision-making for the vessels, even though information in the near future could be made accessible directly to vessels from other stakeholders. MS2 Navigational Assistance Service should be delivered only by VTS authorities

Information provided digitally (non-verbal means) could complement voice communications in time critical situations and in addition partly replace voice communications in non-time critical situations.

Note: Example of time critical situation:

Risk of grounding; In addition, to voice communications, vessel can be provided with an electronic route recommendation.

Note: Example of non-time critical situation:

Assist a vessel to an anchoring position; provide the vessel with an electronic route recommendation without voice communications.

The identity of the vessel receiving Navigational Assistance Service should be assured. Also, other items listed in the IALA Guideline 1089 On Provision of Vessel Traffic Service, annex B should be taken into consideration for digital transmission of information.

All information related to this service should be displayed in real time. Measures should be taken to ensure that the information is received and acknowledged.

### User Needs

The user cases below are based on the information from table X

Each user case is generic and simplified through example for description purposes only. Actions and template categories may differ for different countries. Content in the column named "Template Info (technical)" is pending submissions from relevant stakeholders.

For example:

Pre arrival reporting can be done digitally without voice communication for update of route of voyage plan in order to avoid collisions, groundings and assist in safe navigation.

The content of the voice communication can be provided digitally and be displayed as text in parallel / in addition to voice communication.

#### User Case - vessel approaching VTS area

Vessel approaches VTS area according to received voyage plan. The VTS application observes and analyses the situation. It alerts the VTS operator, who then confirms that the vessel has deviated from its route. For example based on statistical data/deep learning, the application then provides decision support to the VTS operator on the next course of action. The VTS operator validates and acknowledges the advice, and informs, warns and if necessary instructs / advises the vessel to change course via voice communication. The proposed solution is also presented on the vessel's own navigation system. The VTS operator confirms that the vessel has changed course accordingly to the solution. The VTS application continues to monitor the vessel's voyage. It will alert the VTS operator if new deviation occurs.

The categories of services and the associated details are listed in annex 2, MS 2 Navigational Assistance Service template.

The use case is generic and simplified through example for description purposes only. Actions and template categories may differ for different countries. Content in the column named "Template Info (technical)" is pending submissions from relevant stakeholders.

| Time | Vessel Action | VTS Action | Template Info (category) | Template Info (technical) |
| --- | --- | --- | --- | --- |
| 01:00 | Approaches VTS area | Receive voyage plan and monitor vessels progress | Traffic and Route Information |  |
| 01:30 | Deviates from her route | Informs, warns and advises/instructs the vessel to change course/speed | Navigational advice |  |
| 01:35 | Changes Course | Confirms that vessel has changed course and is no longer in danger |  |  |

#### User Case – vessel approaches shallow waters

The VTS operator concludes that the vessel wanted to make a lee for the pilot boat. Course given by the pilot boat would put the vessel into an even worse situation regarding shallow waters. VTS interrupted the boarding operation and gave instruction and guided the ship into a safe area for the embarkation of the pilot.

The VTS requests that the ship states its intention in the VTS area. In response the ship provides its sailing route. In the template, this action is categorised as Navigational information.

The VTS gives a warning when the ship has run into shallow waters, upon which the ship confirms that the information has been received. In the template, this action is categorised as Navigational warning.

The VTS advices the ship to change course. The ship confirms. In the template, this action is categorised as Navigational advice.

The VTS instructs the ship to alter its course, upon which the ship confirms. In the template, this action is categorised as Navigational instruction.

| Time | VTS Action | Ship Action | Template Info (category) | Template Info (technical) |
| --- | --- | --- | --- | --- |
| 00:00 | Question: What is your intention? | Provides sailing route | Navigation Information |  |
| 02:00 | Warning: You are running into danger - shallow waters... | Confirms | Navigational warning |  |
| 02:10 | Advice: Recommended course to make good... | Confirms | Navigational advice |  |
| 02:20 | Instruction: Alter course to south-west... | Confirms | Navigational advice |  |

The VTS application indicate to the VTS operator the situation and suggests an appropriate decision. The VTS operator selects the correct decision from the VTS application and provides it digitally to the vessel .The vessel verifies the decision and takes appropriate action.

### Information to be provided

Information may be provided digitally, in general, but some situations will be time-critical.  Such time-critical information may be best provided by voice. On a case by case, ad hoc and time critical information may be provided using agreed methods.  For details, please see  Annex 2, MS 2 Navigation Service Template.

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |
| Voyage Information Service | Urn:mrn:stm:service:specification:sma:vis | The service supports exchange of voyage plans, text messages and area messages |  | IEC ? |

### Relation to other MSs

MS 2 has relationships with other MSs where it affects the VTS:

Examples may be different depending on the coastal state arrangements.

| Description | Examples of data that could be used in MS 2 |
| --- | --- |
| MS 1 VTS IS | See Annex 1 MS 2 Information Service Template |
| MS3 VTS TOS | See Annex 3 MS 3 Traffic Organisation Template |
| MS 4 Local Port Service | Delays, obstruction, cargo operations, port availability and anchorage are in the port, ISPS state Marsec level |
| MS 5 Maritime Safety Information | All information depending on the structure of MSI |
| MS 6 Pilotage Service | Pilot orders and Updates |
| MS 7 Tug Service | Tug order and updates |
| MS 8 Vessel Shore reporting | Notification of arrival, dangerous cargo etc. |
| MS 9 Telemedical | Delays |
| MS 10 Maritime Assistance Service | Notifications, routing, places of refuge |
| MS 11 Nautical Chart Service | Local area updates, chart updates |
| MS 12. Nautical Publications Service | Updates to publications |
| MS 13 Ice Navigation Service | Ice routes, ice conditions, ice breaking assistance |
| MS 14 Meteorological Service | VTS area weather |
| MS 15 Real Time Hydro and Information Service | Horizontal and vertical Tidal information in VTS area, available water column |
| MS 16 Search and Rescue Service | Search Pattern and vessel of opportunity |

## MS 3 Traffic Organization Service (TOS)

### Submitting Organisation

IALA VTS Committee

### Description of the Maritime Service

A VTS Contracting Government or Governments or the competent authority should establish and set the objectives for the types of services, areas and categories of vessels required or expected to participate.

Traffic Organization Service (TOS) is defined by IMO as “a service to prevent the development of dangerous maritime traffic situations and to provide for the safe and efficient movement of vessel traffic within the VTS area” (IMO Res.A857(20)).

The traffic organisation service concerns the operational management of traffic and the forward planning of vessels movements to prevent congestion and dangerous situations, and is particularly relevant in times of high traffic density or when the movement of special transports may affect the flow of other traffic. The service may also include establishing and operating a system of traffic clearances or VTS sailing plans or both in relation to priority of movements, allocation of space, mandatory reporting of movements in the VTS area, routes to be followed, speed limits to be observed or other appropriate measures which are considered necessary by the VTS authority. (IMO Res. A.857(20)

IALA Guideline No. 1089 defines the Provision of Vessel Traffic Services (INS, TOS & NAS).

Examples of types of information that may be provided by the VTS within a Traffic Organization Service:

| Information related to: | Examples |
| --- | --- |
| Traffic clearance | Give authorization under conditional circumstances to a vessel when: prior to or entering a VTS area; departing from a berth or an anchorage position within a VTS area; entering into a fairway within a VTS area; or prior to commencing a manoeuvre that may be detrimental to safe navigation. Examples of conditions: a VTS sailing plan before entering a VTS area; lock and bridge passage planning; report position at determined reporting point/line/pilot station; use a second fairway in case of bad visibility/weather; use a tug boat in case of strong wind; dredging or compass swing in confined waterway. |
| Anchorage | Examples of anchorage situations: organizing the movements to/from an anchorage position/area; assignment of an anchorage position; assisting vessels into anchorage position. |
| Enforcement | Examples of enforcement: speed limits; adherence to rules regarding traffic routeing measures; pilotage requirements; other traffic regulations and possibly local by‐laws |
| Waterway (sea, channels and fairway) management | Examples of management measures: the use of one‐way traffic as an alternative of two way traffic, depending on the dimensions of ship or the weather conditions; organizing other traffic when a vessel has passed point of no return; slot management to allocate ships in a time window; organizing the traffic concerning vessel dimensions in comparison to fairway restrictions; instruct vessels when overtaking is not permitted; establish and organise ship safety zones in case of particular operations; establish and organise exclusion zones; instruct vessels to keep clear from special areas/positions; organizing the traffic as regards to meteorological, hydrographical or other restrictions such as visibility, wind speed, current, sea state and under keel clearance. |

### Purpose

The purpose is to provide information related to Traffic Organisation Service (TOS) digitally, to create means to reduce administrative burden and information overload, reduce miscommunication due to external interference, simplify work procedures, and increase navigational safety.

All information provided digitally can complement and/or replace verbal/voice communication and would be distributed in real time to the vessel

The steps to achieve this transition to digital information exchange will vary in different areas and for different types of vessels. Depending on the categories of the Traffic Organisation Service in annex 3, (MS 3, Traffic Organisation Service template.) and its discoverability, new equipment may be required both on board and on shore side. Details about digital information exchange available should be published by the VTS authority.

Utilization of electronic communication may apply to elements of the Traffic Organization Service that are not implying risky situations. For instance, AIS Application Specific Messages (IMO SN.1/Circ.289) can be used to provide information related to:

### Operational Approach

A Traffic Organization Service should be responsible for separating traffic in the interest of safety. This separation could be defined in space, time and/or distance.

Enforcement may also be carried out within a Traffic Organization Service where the VTS should monitor adherence to applicable rules and regulations and to take appropriate action where required and within the authority of the VTS. (IALA Guideline 1089 On Provision of Vessel Traffic Service )

Examples:

Slot management; provide vessels digitally with priority of arrival and distance between two vessels

Traffic clearance; provide vessels digitally with permission to proceed, impose conditions or deny entry

Route information; provide vessels digitally with recommended route information

Traffic information; vessel provide VTS digitally their intentions, such as over taking of another vessel

Information regarding restricted or no go area, the content (draft, closed fairway/port/quay etc.) can be provided digitally to vessels without using voice communication.

All information provided digitally can complement and/or replace verbal/voice communication and would be distributed in real time to the vessel

### User Needs

The user case below are based on the information from table 5

The user case is generic and simplified through example for description purposes only. Actions and template categories may differ for different countries. Content in the column named "Template Info (technical) " is pending submissions from relevant stakeholders.

#### User case – vessel leaves quay

Vessel sends planed ETD digitally to VTS where it is presented in the VTS application and the VTS operator takes action and instructs vessel digitally to remain alongside for five more minutes. The instructions are graphically displayed in applications, acknowledged and provided to other traffic via digital and/or verbal means for non MSP-ready vessels. The application alerts operator on upcoming traffic conflicts and advice on a solution, which is acknowledged and transmitted to vessels.

Detailed information can be found in the annex 3, MS 3, Traffic Organisation Service Template.

| Time | Ship Action | VTS Action | Template Info (category) | Template Info (technical) |
| --- | --- | --- | --- | --- |
| 00:00 | Requests permission to leave quay | Instruction: Remain alongside. Permission to leave in five minutes | Traffic clearance |  |
| 02:00 | Vessel passing reporting point before entering fairway A | Instruction: Provides sequence slot due to other traffic | Waterway management |  |
| 02:10 | Vessel had exceeded speed limit in the fairway | Instruction: speed limit in fairway 8 knots, keep safe speed | Enforcement |  |
| 02:20 | Requests anchorage | Provides position for anchorage | Waterway management |  |

### Information to be provided

See Annex 3, MS 3, Traffic Organisation Service template

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other MS

MS3 has relationships with other MSs where it affects the VTS:

This may be different depending on the coastal state arrangements.

| Description | Examples of data that could be used in MS 2 |
| --- | --- |
| MS 1 VTS IS | See Annex 1 MS 2 Information Service Template |
| MS2 VTS NAS | See Annex 2 MS 3 Navigation Assistance Service Template |
| MS 4 Local Port Service | Delays, obstruction, cargo operations, port availability and anchorage are in the port, ISPS state Marsec level |
| MS 5 Maritime Safety Information | All information depending on the structure of MSI |
| MS 6 Pilotage Service | Pilot orders and Updates |
| MS 7 Tug Service | Tug order and updates |
| MS 8 Vessel Shore reporting | Notification of arrival, dangerous cargo etc. |
| MS 9 Telemedical | Delays |
| MS 10 Maritime Assistance Service | Notifications, routing, places of refuge |
| MS 11 Nautical Chart Service | Local area updates, chart updates |
| MS 12. Nautical Publications Service | Updates to publications |
| MS 13 Ice Navigation Service | Ice routes, ice conditions, ice breaking assistance |
| MS 14 Meteorological Service | VTS area weather |
| MS 15 Real Time Hydro and Information Service | Horizontal and vertical Tidal information in VTS area, available water column |
| MS 16 Search and Rescue Service | Search Pattern and vessel of opportunity |

## MS 4 Local Port Service (LPS)

### Submitting Organisation

International Harbour Master Association (IHMA)

### Description of the Maritime Service

A port authority is a hub from which several types of maritime and terrestrial services are provided. A port is generally not the final destination of goods, it therefore needs to link with other mode of transportation (rail, truck, smaller ships) in order to ensure the fluid movement of goods to market. Port’s infrastructure and facilities are important links in the chain of intermodal transportation and to be capable to offer timely services, a port needs information on a ship’s transit from departure until time of arrival.

The size of a port and its status (local, regional, national or international) will influence the type and amount of services provided. These services include both maritime and shore sectors.

Table X.X - Examples of port services

| Maritime Services | Shore Services |
| --- | --- |
| Notification/Documentation/Reporting | Berthage |
| Navigation (anchorage, pilotage, tug, etc.) | Cargo Operations |
| Safety & Security | Fresh Water/Electricity/Waste/Repairs/Medical facilities/Shipping Agent/etc. |
| Nautical information | Bunkering |
| Communication | Linesmen |
| Weather | Intermodal transportation |

The combined services contribute to safe navigation, protection of the environment and the efficient movement of goods and people.

Figure x. Brief summary of services that may be provided by a port.

### Purpose

The Local Port Service aims facilitating the exchange of information and coordination among key stakeholders during each phase of a ship transit. To promote an optimal coordination and efficient operations all along the transportation chain, the exchange of information must be done in a timely manner. This allows all actors involved in LPS to take specific actions at the right moment either to respond or to adapt to new situations, such as delay, premature departure/arrival, incident, etc.

All actors involved in a ship transit will benefit of a common information platform. Access to timely information at each phase of a transit will improve:

- Just in time arrival and departure

- Reduction in waiting time for berthing (anchorage)

- Reduction of fuel consumption and greenhouse gases

- Predictability for delivery of cargo to market

- Work schedules of services

A common communication platform will facilitate the fluid movement of cargo and persons, and contribute to a better and more transparent decision making process.

### Operational Approach

An efficient LPS must allow all key stakeholders to access information they need during a ship transit. Sharing a common operational picture will optimize the decision-making process of each actor and facilitate adjustment whenever a situation is changing. A basic operational approach for LPS would be the development and utilization of a common electronic information platform.

This platform will require each actor to enter its own relevant data in a timely manner for the benefit of other actors (see Figure xx). For sensitive and/or confidential data, restrictions can be set to provide access only to actors that are requiring it.

Figure xx. Examples of actors and data exchange that can contribute to a common LPS platform. Status of actions taken by each actor is illustrated by the lights (green, yellow and red).

Apart from the sensitive information, all other data should be accessible to all participants involved in the transportation chain in order to create the common operational picture. Given that an input from one actor might be critical for others’ decision and influence their inputs, a management process would need to keep the platform updated in real-time and to inform actors of status or give notification if required data is omitted. The port authority or another organization could assume the responsibility of monitoring the platform and the status of data entries.

Figure 6 Terms for Vertical Measurement of vessels, depths and elevations. Identified in the IHMA and UKHO port information project.

### User Needs

Provision of LPS is designed to improve coordination and exchange of information among stakeholders involved in a ship transit. The nature of this information is variable and has several purposes, for example safety, efficiency, protection of environment, fluidity of goods to market, etc. As such, user needs cover both maritime and shore information from departure to the time of arrival.

- Table X.X – Examples of types of information provision that LPS can cover

| Information related to: | Examples |
| --- | --- |
| Arrival and Berthing | Examples of Arrival and Berthing information: - Port allocation - ETA / ATA - Time slot, with slot management to allocate ships in a time window; - Mooring information - Draft and limitations - Safety and limitations of traffic concerning vessel dimensions in comparison to fairway restrictions - Local warnings and restrictions |
| Port information | Examples of port information: - ISPS information - Water supply - Crane and other cargo handling arrangement - Waste handling - Fuel arrangement - Immigration - Custom clearance - Pilot information - VTS information - Reporting formalities |
| Anchorage | Examples of anchorage situations: - organizing the movements to/from an anchorage position/area; - assignment of an anchorage position; - assisting vessels into anchorage position. |
| Enforcement | Examples of enforcement: - speed limits; - adherence to rules regarding traffic routeing measures; - pilotage requirements; - other traffic regulations and possibly local by‐laws |

### Information to be Provided

See Annex 4, MS 4 Local Port Service template

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Maritime Services

This may be different depending on the coastal state arrangements.

| Description | Examples of data that could be used in MS 4 |
| --- | --- |
| MS 1 VTS IS | VTS area, Type of VTS service, Contact to VTS, Information from sensors, radar, AIS, met, CCTV |
| MS2 VTS NAS | Information about vessel undertaking NAS |
| MS3 VTS TOS | Information about timeslots / ETA, route advisories |
| MS 5 Maritime Safety Information | All Safety related information |
| MS 6 Pilotage Service | Local Pilot boarding place and stations, Pilot regulations Information from pilot booking system |
| MS 7 Tug Service | Tug operation, Available tug capacity |
| MS 8 Vessel Shore reporting | ETA/ATA, Cargo information, Incidents reports, Dangerous gods, Waste, Number of persons onboard |
| MS 9 Telemedical | Local address of medical centre and communication and Capacity |
| MS 10 Maritime Assistance Service | Information about vessels requesting relevant MAS Incidents Notifications, places of refuge |
| MS 11 Nautical Chart Service | Chart update information Hinge resolution charts for the port Notice to Mariners Accuracy coverage characteristics Contour line |
| MS 12. Nautical Publications Service | Local descriptions and publications. |
| MS 13 Ice Navigation Service | Ice forecast, Ice charts, Icebreaker services |
| MS 14 Meteorological Service | All meteorological information |
| MS 15 Real Time Hydro and Information Service | Sea state, Current, Wave, Moving sandbanks |
| MS 16 Search and Rescue Service | SAR situations and operations in local areas Search and rescue capacity SAR communication capability |

## MS 5 Maritime Safety Information service (MSI)

### Submitting Organisation

Local Met/Hydro organisation

### Description of Maritime Service

The Global Maritime Distress and Safety System (GMDSS) as described in SOLAS Chapter IV defines the seventh functional requirement as: 'Every ship, while at sea, shall be capable of transmitting and receiving maritime safety information'.

### Purpose

The MSI service is an internationally co-ordinated network of broadcasts of Maritime Safety Information from official information providers, such as:

National Coastal administration or shipping authority, for navigational warnings;

National Meteorological Offices, for marine weather warnings and forecasts;

Rescue Co-ordination Centres (RCCs), for shore-to-ship distress alerts;

The International  Ice Patrol, for Oceanic ice hazards.

- SOLAS Chapter V, regulations 4 through 7 governs the contracting government’s responsibilities with regards to providing MSI information.

Examples of Maritime Safety Information Service are listed in Table x.

Maritime Safety Information Service

| Information related to: | Examples: |
| --- | --- |
| Impediments to shipping and areas to avoid | Dangerous wrecks, obstacles not otherwise promulgated, diving operations, cable laying operations, vessels not under command, etc. The routes of large unwieldy tows; Drifting hazards (including derelict vessels, ice, mines, containers, other large items, etc.); The establishment of offshore structures in or near shipping lanes and the positioning of rigs; Areas where search and rescue (SAR) and anti-pollution operations are being carried out (for avoidance of such areas) Military practice areas. |
| Status of navigation aids | Casualties to lights, fog signals, buoys and other aids to navigation affecting main shipping lanes; Establishment of major new aids to navigation or significant changes to existing ones. |
| Other urgent safety-related information | unexpected alteration or suspension of established routes; acts of piracy and armed robbery against ships; tsunamis and other natural phenomena, such as abnormal changes to sea level; New or amended mandatory ship reporting systems or maritime regulations affecting ships at sea; Significant malfunctions or changes to maritime communications systems. World Health Organization (WHO) health advisory information; security-related requirements. |
| Marine weather warnings and forecasts | Information that will include the speed and direction of the prevailing wind, direction and height of the waves, visibility, atmospheric pressure, the formation of ice, etc. Gale, storm, tsunami, restricted visibility, etc. |

### Operational Approach

The joint IHO/IMO/WMO Publication S-53 states that the Maritime Safety Information Service of the GMDSS is the internationally and nationally coordinated network of broadcasts containing information which is necessary for safe navigation, received on ships by equipment which automatically monitors the appropriate transmissions, displays information which is relevant to the ship and provides a print capability.  This concept is illustrated in figure 4.5-1.

Figure 4.5-1 The maritime safety information service of the Global Maritime Distress and Safety System  (Source: S-53)

*Note that search and rescue information is outside of scope for this MS.

Additionally, local and regional governments may provide MSI information in other formats such as through a website, push e-mail service and social media.

Within GMDSS, navigational warnings promulgation is done in defined areas that are managed by area coordinators as illustrated on figure 4.5-2.

Figure 4.5-2 NAVAREAs for coordinating and promulgating navigational warnings under the World-Wide Navigational Warning Service (Source: S-53)

Marine meteorological warnings are, within GMDSS, promulgated in defined areas which are under the coordination of defined area coordinators. Within other distribution channels, other areas and operators are possible.

Figure 4.5-3 METAREAs for coordinating and promulgating meteorological warnings and forecasts within the GMDSS (Source: S-53)

### User Needs

Two principal methods are used for broadcasting maritime safety information in accordance with the provisions of the International Convention for the Safety of Life at Sea, 1974, as amended, in the areas covered by these methods, as follows:

NAVTEX: broadcasts to coastal waters; and

SafetyNET: broadcasts which cover all the waters of the globe except for Sea Area A4, as defined by IMO resolution A.801(19), annex 3, as amended.

Additionally, HF NBDP may be used to promulgate maritime safety information in areas outside Inmarsat or NAVTEX coverage (SOLAS regulation IV/7.1.5).

Ships are required to be capable of receiving maritime safety information broadcasts for the area in which they operate in accordance with the provisions of the International Convention for the Safety of Life at Sea, 1974, as amended. The NAVTEX receiver should operate in accordance with the technical specifications set out in Recommendation ITU-R M.540, as amended. The SafetyNET receiver should conform to the Maritime Design and Installation Guidelines (DIGs), annex B, issue 6 of April 2008 published by Inmarsat. In Sea Area A4, outside of the coverage of NAVTEX, where MSI is received using HF NBDP, the HF NBDP receiver should operate in accordance with the technical specifications set out in Recommendation ITU-R M.688, as amended, and should meet the performance standards adopted by IMO resolution A.700(17), as amended.

Future MSI services should provide information in formats and by means that allow it to be better integrated with other systems on board, especially ECDIS. IHO is developing the S-124 standard which is expected to be the next generation MSI exchange standard. It is envisioned that these will interact in various forms with the ENC within the ECDIS.

Figure x.x.x : MSI can be portrayed on graphical display by using common standards such as s-124 and s-411 etc will

### Information to be Provided

See Annex 5, MS 5 Maritime Safety Service

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Digital Maritime Services

Information related to ENC data such as updates to the status of navigation aids will supplement information that is part of MS 11 and DMS 17. Anti-piracy warnings transmitted as MSI will overlap with MS 20 and will probably be summary information that is expanded on in MS 20 services. Meteorological information such as forecasts will overlap information that is part of MS 14 and MS 15, and again will potentially be extracts or summaries of information that is provided in more detail as part of those services.

## MS6 Pilotage service [IMPA]

### Submitting Organisation

International Maritime Pilots Association / Local Pilots

### Description of Digital Maritime Service

Seafarers seeking pilotage should have easy access to information regarding the pilot service provided in a specific area. Information such as local regulations, contact, support, meeting point, limitations, pilot booking systems, guidance etc can be easy and user-friendly accessible by electronic standardizes means.   Efficient pilotage depends, among other things, upon the effectiveness of the communications and information exchanges between the pilot, the master and the bridge personnel and upon the mutual understanding, each has for the functions and duties of the other.

Establishment of effective co-ordination between the pilot, the master and the bridge personnel, taking due account of the ship's systems and equipment available to the pilot, will aid a safe and expeditious passage.

### Purpose

The purpose is to share all necessary information related to the pilot service when planning an operation before the pilot embarque and during the operation, by using modern technology and common standards.

Efficient pilot operations depend on, among other things, the effectiveness of the communications and information exchanges between relevant stakeholders.

Good information exchange between shore and vessel regarding pilot is crucial for safe and efficient operations.

### Operational Approach

Pilot organizations providing pilot service in an area should provide information to vessels in a digital and easy accessible way. The information can be portrayed as a layer at the ECDIS or in an graphical display.

Examples of information can be:

| Information related to: | Examples |
| --- | --- |
| General information | Examples of information: - Pilot services required in the area - Regulations - limitations - Response time - Pilot booking system - Contact information to pilot organization/ operator - Mandatory needs for tug assistance - Safety procedures and requirements - Mooring information - Pilot meeting point - Information about pilot fee |
| Operational information | Examples of tug information: - Pilot identity - ETA / ETD - Contact to pilot boat - Position of vessels or station - Assisting methods - Required manoeuvring space - speed limits - Communication - Exchange of PPU information |

### User Needs

Position exchange, pilot request, meeting points, special regulations, special manoeuvring, time, cost and fees, tug information.  A pilot application (My Pilot Service), exchange of passage plan, route exchange, pilot identity and any exemptions from pilotage requirements.

### Information to be Provided

See Annex 6, MS 6 Pilotage Service

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Maritime Services

MS 6 has relationships with other MSs where it affects the Pilot operation and contribute to safe and efficient operations.

Examples may be different depending on the local conditions and type of vessel / operation.

| Description | Examples of data that could be used in MS 7 |
| --- | --- |
| MS 1 VTS IS | VTS area, Type of VTS service, Contact to VTS, places of refugees, local regulations, limitation, visibility, information regarding traffic in the area |
| MS2 VTS NAS | Pilot need information if NAS is conducted |
| MS3 VTS TOS | Traffic clearance, time slots, |
| MS4 Local Port Service | Birthing information, timeslots, security, local regulations, supply, assistance, port contact information |
| MS 5 Maritime Safety Information | Warnings, no-go areas, diving operations, exercises |
| MS 7 Tug service | Tug allocation and direction, capacity |
| MS 8 Vessel Shore reporting | Vessel information, cargo information, people onboard |
| MS 9 Telemedical | Not relevant |
| MS 10 Maritime Assistance Service | Contact information, places of refuge |
| MS 11 Nautical Chart Service | Local area updates, chart updates |
| MS 12. Nautical Publications Service | Digital information from nautical publications relevant for the operation |
| MS 13 Ice Navigation Service | Ice chart, ice conditions, information regarding icebreaker service / assistance, Ice routes |
| MS 14 Meteorological Service | All information regarding weather in the area |
| MS 15 Real Time Hydro and Information Service | All information from real time sensors. |
| MS 16 Search and Rescue Service | Salvage information, drifting pats, SAR areas, rescue capacities in the area. |

## MS 7 Tugs service

### Submitting Organisation

TBA

### Description of the Maritime Service

Tug services range from small vessels with limited capacity and service in ports and rivers to oceangoing vessels built for complex operations and salvage. This service contributes to the safety of navigation, protection of the environment, and efficiency of marine transportation by conducting different type of operations such as:

- transportation (personnel and staff from port to anchorage);

- ship assistance (ex: mooring);

- salvage (grounded ships or structures);

- shore;

- towage (harbour/ocean);

- escort;

- oil spill response.

The need of tug services differ from port to port, type of vessel and cargo. In some cases, information about a tug service capacity and/or availability may be difficult to obtain due to communication deficiency. This maritime service is dedicated to improve information availability of tug service.

Tug service would encompass all kinds of tugs, such as

- conventional;

- azimuth stern drive;

- tractor;

- rotor.

| Information related to: | Examples information shared in a Tug service |
| --- | --- |
| Deep sea information | Examples of information: - Contact information for tug vessel / operator - Safety procedures and regulations - Available resources - Working hours |
| Local port or river information | Examples of information: - Contact information for tug vessel / operator - Mooring and berthing information - Available resources - Working hours |
| Tug information | Examples of tug information: - Type of tug - Capacity - Size - Assistance services - Response time - Contact information - Working hours |

Table xx – Examples of information that can be shared in a Tug service

### Purpose

Tug service aims to facilitate access to all necessary information required by ships heading to port in order to optimize transit times and promote efficient movement of goods and persons by using modern technology and common standards.

Effective communications and exchange of information between relevant stakeholders would contribute to efficient tug service. Exchange of information electronically would significantly contribute to the improvement of this service.  For example, notifying a ship officer, in advance, about tug availability in a port could lead the ship to adapt its speed accordingly and in some cases, may avoid the need to anchor the ship. The types of information which can be exchanged include:

- ETA (request);

- confirmation requests;

- updates on transit status and tug availability;

- updates among stakeholders;

- standardized messages to overcome language barriers.

### Operational Approach

Tug service is a key element of the marine transportation chain and well-coordinated procedures and communication means should be in place to ensure fluid movement of ships.

Alike the Local Port Service, utilization of a common platform to exchange information electronically and keep users updated on a regular basis about the status of operations would significantly improve this service, as well for the ships’ operator as for the tugs owners. The tug service does not substitute to current operational procedures but rather aims mainly to improve the communication aspect involved in a ship request:

- Ship’s size

- Number of tugs required

- Time the service is required

- Time the tug could be on site

- Estimated duration of operations

- End of operations.

- Access to this information electronically would enhance the awareness of a ship’s time-stamp.

- Figure xx. Example of an electronic communication platform for all actors involved in the tug service.

- Increased connectivity through sharing of harmonized digital information regarding tug services in ports, rivers or deep-sea will enhance the efficiency through just-in-time services and reduce the human factor errors, such as language barriers or outdated information such as in analogue publications. It will also enhance the efficiency and access to the information in a fast and easy-to-use-manner.

### User Needs

### An easy and timely access to tug services information is critical to ensure the fluidity in the transportation chain. The main information required from this service are:

- Capacity

- Availability

- Time of response

- Status of operations

- Duration of operations

### In return, the tug services should be regularly updated on the ship’s ETA/ATA in order to plan its operations accordingly. Also, and in the event of an unanticipated change, the tug service and ship officers must be capable to communicate easily with each other in order keep both parties inform about the evolving situation and make proper decisions. Consequently, an easy communication link must be part of the user needs and this communication link would benefit as well to all other actors.

Figure x.x.x: Example portrayal of tugs (in green) engaged in an advanced pilotage operation

### Information to be Provided

See Annex 7, MS 7 Tug Service

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |
| S-211 Port Collaborative Decision Making (PortCDM) |  | Standard for information exchange between actors in ports, including tugs. |  | IALA |

### Relation to other Maritime Services

| Description | Examples of data that could be used in MS 7 |
| --- | --- |
| MS 1 VTS IS | VTS area, types of VTS service, VTS contact information, places of refuge, local regulations, limitation, visibility, information regarding traffic in the area |
| MS2 VTS NAS | Not relevant |
| MS3 VTS TOS | Traffic clearance, time slots |
| MS4 Local Port Service | Birthing information, time slots, security, local regulations, supply, assistance, port contact information |
| MS 5 Maritime Safety Information | Navigational Warnings, Meteorological Information and other urgent safety-related information. |
| MS 6 Pilotage Service | Pilot regulations, contact information, request procedures |
| MS 8 Vessel Shore reporting | Vessel information, cargo information, crew information |
| MS 9 Telemedical | Not relevant |
| MS 10 Maritime Assistance Service | Contact information, places of refuge |
| MS 11 Nautical Chart Service | Charting information, chart updates |
| MS 12. Nautical Publications Service | Digital information from nautical publications that is relevant for the operations at hand |
| MS 13 Ice Navigation Service | Ice chart, ice conditions, information regarding icebreaker service / assistance, Ice routes |
| MS 14 Meteorological Service | Information regarding weather in the area |
| MS 15 Real Time Hydro and Information Service | Information from real time sensors. |
| MS 16 Search and Rescue Service | Salvage information, drifting parts, SAR areas, and rescue capacities in the area. |

## MS 8 Vessel shore reporting

### Submitting Organisation

### Description of Maritime Service

This service provides information from shore to ship about two different reporting regime.

The first regime is the Vessel Shore Reporting (VSR) as prearrival reporting. This service can stretch form providing information and guidelines on reporting formalities and when, what and how to report in a specific port to a full exchange of information in a Single Window ship reporting system.

The second regime is linked to a voyage in an area where a Ship Reporting System (SRS) is established.

In SRS areas, shore authorities can provide more automated and efficient reporting systems by using technology such as AIS and VDS in combination whit common data structures and product specifications.

The Vessel Shore Reporting (VSR) service aims to provide information and guidelines on reporting formalities required by national authorities. Information required can contain the following elements:

- Marine security regulations

- Vessel Traffic Services zones regulations

- Custom and immigration regulations

- Port state regulations

- Health and veterinary regulations

- Environmental regulations

- Etc.

Many of these elements will be addressed by the IMO Facilitation Committee who is developing the specifications for the Single Window Reporting System, which is linked to e-Navigation Strategy Implementation Plan Solution 2 - Means for standardized and automated reporting -.

IMO e-navigation solution 2 regarding automated ship reporting is one of the most important solutions to reduce the Mariners workload (amount of time spent on preparing and submitting reports to shore-based authorities). To achieve this, reports should be automatically generated as much as possible from on-board systems.

In a ship reporting system (SRS), MSC 43(64) recalls that communication between a shore-based authority and a participating ship should be limited to information essential to achieve the objectives of the ship reporting system (SRS). The initial report required from a ship entering the system should generally be limited to:

- ship's name

- call sign

- IMO identification number if applicable, and

- position.

The existing resolution is based on Regulation 11 of SOLAS Chapter V as follows:

“Ship reporting systems contribute to safety of life at sea, safety and efficiency of navigation and/or protection of the marine environment. A ship reporting system, when adopted and implemented in accordance with the guidelines and criteria developed by the Organization pursuant to this regulation, shall be used by all ships, or certain categories of ships or ships carrying certain cargoes in accordance with the provisions of each system so adopted.

The Organization is recognized as the only international body for developing guidelines, criteria and regulations on an international level for ship reporting systems. Contracting Governments shall refer proposals for the adoption of ship reporting systems to the Organization. The Organization will collate and disseminate to Contracting Governments all relevant information with regard to any adopted ship reporting system.”

Although the Regulation refers to safety of life at sea, safety and efficiency of navigation and/or protection of the marine environment, resolution MSC.43(64) states that other supplementary information may also be requested in the initial report if justified to ensure the effective operation of the Ship (MSC.43(64) Annex para 2.2.1.4).

This information may include the intended movement of the ship through the area covered by the reporting system and any defects or otherwise as well as the general categories of any hazardous cargoes on board (MSC.43(64) Annex para 2.2.1.4)

The Shipreporting prosess from a vessel to a Vessel Ship reporting (VSR) include normally a web solution and a onshore representative for the vessel operator or owner.  The agency act on behalf of the master or owner and provide information to relevant authorities.

A future e-navigation solution 2 regarding the report from ship to shore expect to reduce this timeconsuming operation.

Information required to be transmitted as part of a mandatory SRS is generally transmitted to the VTS centre, or to the relevant authority, via VHF voice communication. Some SRSs accept also reports given using AIS, internet-based reporting systems, e-mail, fax, SATcom, mobile phone, or a combination of these communication means.

Figure XX – Examples of reporting points from Bay of Fundy

Mandatory information required can be transmitted either to the VTS centre or the relevant Coastal authority through different communication means.

### Purpose

The majority of ports require a number of reports from ships before arriving at the port. At least, information is required according to the IMO FAL conviction. However, a large number of ports require different additional information that is used by different authorities. This means that the vessel must report the same information several times. In addition, good and understood information about local reporting requirements may cause the vessel to report incorrect or insufficient information to the port state.

The purpose is to:

- Secure submission and distribution of all reports required by all shore-based authorities in the harmonized standard at the required timeframe.

- Reduce the administrative burden on board the ship and ashore in respect to ship reporting requirements and achieve the use of common standards and harmonization

- Reduce the number of human errors and lack of information related to ship reporting and transmission of pre-arrival / SRS information between relevant authorities and involved stakeholders in port and coastal state and vessel.

- Provide real-time access to information to relevant stakeholders in a secure manner.

MSC 43(64) states that the objectives of a ship reporting system (SRS) should be based upon:

“the improvement of the safety of life at sea, the safety and efficiency of navigation and/or to increase the protection of the marine environment. They may or may not be operated as part of a vessel traffic service.”

As such, the VSR service contributes to the traffic situational awareness of a Coastal administration either by being informed of the ships heading into its waters or the ones already transiting  it. Based on the information collected, a Coastal administration can initiate an intervention plan if required.

An additional purpose of the VSR service would be to reduce the administrative works on board by automating as much as possible reporting processes. This would allow the officer of the watch to dedicate more time for navigational oriented responsibility.

### Operational Approach

Electronic systems for reporting should use the same protocols and product specifications in a Single Window solution to send digital pre-arrival information such as the FAL forms and other regional/national requirements as defined in SOLAS V reg 11.2 for ship reporting systems. This will ensure a common harmonized platform for all ship reporting systems.

National Competent authorities should provide information about reporting formalities and make sure that all information on aspects regarding reporting is easy to understand, accessional and even automated for the master or operator.

Examples of information can be:

| Information related to: | Examples |
| --- | --- |
| Reporting regulations | Examples of information: - What to report. The pre-arrival information may consist of ship particulars, arrival notice, crew and passenger lists, crew and passenger effects declarations, stores list, IMDG information, waste declaration, different ship’s certificates, ports of call list, dangerous cargo declaration and manifests, vaccination list, narcotic list, ship’s money declaration, etc. - When What to report (e.g. 24h, 48h, 72h, 96h before arrival) - To who (e.g. immigrations, police, port master, etc.) |
| Reporting tools | Examples of tug information: - Web, App etc - Communication - Guidelines |
| Shore receivers And support | Examples of assistance: - Contact information - Support information - Information about local reporting aid / support such as coastal radio stations and agency’s |

The type of information required and reporting periods may differ from country to country and create some confusion on the ships’ side if the information requested is not clearly stated. Also, the reporting periods in some situations start as far as 96 h before a ship enters into a Coastal administration waters, posing a challenge with respect to the communication mean that can be used to provide the information all along the transit.

Currently, the following communication systems are normally used to report information:

- AIS

- internet-based reporting systems

- e-mail

- fax,

- SATCOM

- mobile phone, or

- a combination of these systems

As the scope, transmission capacity, and data format of each of these systems differ, the possibility to develop standardized protocols, product specifications in order to be capable to automate the collection of data on board and communicate it to shore-based authorities might constitute a real issue. The reporting information may be packaged with a route exchange format (e.g. S-412).

Figure XX – Example of current communication systems used to report information

Some of these reporting systems are not using digital format which prevents to provide an automated service. The first step of an operational approach would be the utilization of a communication system that can transmit digital data. Secondly, and given reporting requirements differ among Coastal administration, a library containing information required by each administration should be developed and complying with the standardized product specifications principle. Specific requirements requested by an administration would be part of the form pertaining to this country.

Figure XX – Example of electronic communication systems using a standardized product specifications library

Basic information such as the ship name, call sign, IMO number and MMSI, etc., can be automatically collected from the ship’s AIS system and transmitted through terrestrial or satellite AIS

### User Needs

Ship’s masters face considerably administrative burden to comply with different mandatory Vessel Ship reporting requirement and the transmission of pre-arrival information and  reporting in a Ship Reporting Systems (SRSs). Different procedures, data formats, documentary requirements and formalities are usually applied depending on the country, ship reporting system or port of destination. One of the main challenges is that the ship has to communicate in different manners through different systems.

Provide ICT tools for shipboard and shore-based personnel to streamline the processes and procedures associated with submittal, generation and distribution of required reports, including retrieval of information from other ship systems (Ballast Management, Waste Management System, Emission Control System, Navigation System, etc) and from shore-based sources (cargo and passenger booking offices, crewing agents, stevedores, etc).

This, in turn, requires building and maintaining a library of required reports that are uniquely identified and characterized by their requirements for format, deadline, content, etc.  The eco-system also requires developing and maintaining an S-100 Product Specification for CMDS that can be used to generate all required reports in the library.  Lastly it requires that ships’ systems that generate reporting information be certified to be compliant with an international machine-to-machine interface standard or ship network standards such as IEC 61162 series.

Figure x.x.x BAREP SRS between the Russian federation and Norway

### Information to be Provided

See Annex 8, MS 8 Vessel Shore Reporting

### Associated Technical Services

To be capable of generating this information automatically and transmit it automatically, a realistic operational implementation of the VSR service would require the involvement of both: Coastal administration and shipowners. Coastal administration would need to develop an international library of ship reports that are uniquely identified and characterized by their requirements for format, deadline, content, etc. This library would be part of the S 10x register and standardized product specifications (e.g. S-127 and S-421) would be developed and maintained in order to generate the required reports.

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |
| S-127 Maritime Traffic Management |  | Marine Traffic Management (MTM) datasets describe the availability and reliability of vessel traffic services, pilotage, routing measures, and ship reporting systems. This includes their service areas, services offered and instructions for contacting or utilizing these services. |  | IHO |
| S-421 Route Plan based on S-100 |  | S-421 specifies the content, structure, and metadata needed for creating a fully S-100 compliant Route Plan Information and its portrayal within an S-100-based Application. |  | IEC |

### Relation to other Maritime Services

Digital information from the ship reporting system using common standards is beneficial for several stakeholders and actors involved.

In addition to authorities ( e.g border police, immigration, defence, coastguard, custom etc) claiming the information, the list below gives example on services that can benefit from an access to ship reporting information

| Description | Examples of data that could be used from MS 10 |
| --- | --- |
| MS 1 VTS IS | Type of vessel, nationality, MMSI, IMO no, contact information |
| MS2 VTS NAS | Draft, cargo, |
| MS3 VTS TOS | ETA / ATA, ISPS information, purpose for arrival |
| MS4 Local Port Service | The majority of the information in a Ship reporting system is useful and can be reused an effective and transparent port operation |
| MS 6 Pilotage Service | Ship reporting systems can exchange information with the pilot system. Easy access to information can be important for the pilot. An digital pilot requesting / booking system connected to the ship reporting system will increase the efficiency |
| MS 10 Maritime Assistance Service | Information about cargo, dangerous gods and people onboard can reduce time before assistance and contribute to allocate right resources for the actual situation. |
| MS 13 Ice Navigation Service | Information from the ship reporting system can contribute to a more tailormade and effective Icebreakers service. |
| MS 16 Search and Rescue Service | Salvage information, drifting pats, SAR areas, rescue capacities in the area. |

## MS 9 Telemedical Assistance Service (TMAS)

### Submitting Organisation

### Description of the Maritime Service

According to the IMO/ILO resolution 164 the TMAS centre should be able to provide medical advice for seafarers 24 h/day, 365 days/year. TMAS should be permanently staffed by physicians qualified in conducting remote consultations and who are well versed in the particular nature of treatment onboard ship.

Within the maritime medicine the prevailing view has for a long time been that a standardization of the TMAS services is both necessary and wanted. This would firstly enhance the quality of the medical practice, and secondly, a standardization of reporting and registering of medical events will make a much better basis for advancement. MSC.1/Circ.1218  MSC/Circ.960

### Purpose

TMAS can be delivered in all sea areas, but differences in bandwidth depending on available types of transmission techniques will result in different levels of service.

Seafarers has some basic, but limited, training and skills in medical treatment. Sharing of digital information for diagnostic will increase the rate for right treatment.

To provide seafarers with easy and fast access to a single point of contact with the TMAS centre is essential.

Sharing of information between TMAS centre and medically responsible personal onboard during treatment.

The purpose of TMAS is to give decision support and advice to medically responsible personal onboard in cases of medical emergency where treatment cannot wait. This is today mostly done by voice communication using VHF, MF or short wave radio, or mail. In a future e-navigation scenario, the quality of remote diagnostics and even treatment can be expected to improve.

In addition to prevailing TMAS provisions, real time monitoring of the patients current health status will be important. The parameters monitored may differ in different systems, but may include hart rate, blood oxygen saturation and pressure. As an example see the telemetric medical information screen below.

An example of a typical telemetry screen: top (green) curve is hart rate (Beats Per Minute from two redundant connectors), below (in red) is Ambulatory Blood Pressure (ABP) in mmHg, the third curve is Peripheral Capillary Oxygen Saturation (SPO2), the oxygen saturation of the blood (normal is 95-100 %), at the bottom of the screen are Noninvasive Blood Pressure (NBP) and the patient’s temperature in degrees Celsius.

Further needs might involve real-time video chat between the remote specialist doctor and the patient as well as examinations with remotely controlled cameras. Also extensive, detailed, cardiovascular (hart) data for a period of time should be able to be collected and sent as compressed files for further monitoring by the specialist doctor.

### Operational Approach

One may envision that in the not too distant future, what today is very expensive medical equipment could be installed on (at least some) ships, e.g. large cruise vessels. Examples of such equipment could be:

[short version] ultrasonography, computer tomography (CT) and surgical operation robots. Such equipment will then be remotely operated from land given reliable radio connections.

Diagnostic ultrasonography
is used to see internal body structures such as tendons, muscles, joints, vessels and internal organs. Its aim is often to find a source of a disease or to exclude any pathology. The practice of examining pregnant women using ultrasound is called obstetric ultrasound, and is widely used.

The European Space Agency has already tested a long-distance robotic ultrasound system for remote operation by distant specialists. An assistant onboard simply holds the device against the patient and the ultrasound expert can move the probe as if present in the examination room, rather than thousands of kilometres away. They control the device in real time using a joystick, based on ultrasound imagery relayed back.

CAT
Computed tomography, more commonly known as a CT or CAT scan, is a diagnostic medical test that, like traditional x-rays, produces multiple images or pictures of the inside of the body.

Remote surgery (also known as tele-surgery)
Remote surgery allows the surgeon to remotely perform surgery either using direct tele-manipulator or through computer control. The surgeon does not have to be present, but can be anywhere in the world, leading to the possibility for remote surgery.

### User Needs

Today

- A digital health emergency monitoring system allowing logged text interchange between a ship and medical specialist at a shore hospital including

- Voice communication not only with the medical officer onboard, but also with the patient in the medical bay

- Real-time video for visual examination by the remote specialists using video

- Real-time transmission at length of medical telemetry

- Transmission of compressed data packages of e.g. EEG data

In the future (examples)

- Transmission of data and remote control of ultrasonography

- Transmission of data and remote control of computer tomography and the like imaging techniques

- Transmission of data and remote control of robotic surgery

Figure x.x.x remote control of robotic surgery exist onshore already

https://www.welchallyn.com/content/dam/welchallyn/documents/upload-docs/Product-Literature/Brochure/MC11587%20Telemetry%20MedSurg.pdf

http://www.esa.int/Our_Activities/Space_Engineering_Technology/Long-distance_ultrasound_exams_controlled_by_joystick

### Information to be Provided

See Annex 9, MS 9 Telemedical Assistance Service

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Digital Maritime Services

## MS 10 Maritime Assistance Service (MAS)

### Submitting Organisation

### Description of the Maritime Service

IMO Resolution A.950(23) on Maritime Assistance Services (MAS) specifies that the circumstances of a ship’s operation that involve a MAS are not those requiring rescue of persons Three situations can arise:

	the ship is involved in an incident (e.g., loss of cargo, accidental discharge of oil, etc.) that does not impair its seakeeping ability but nevertheless has to be reported;

	the ship, according to its master’s assessment, is in need of assistance but not in a distress situation (about to sink, fire developing, etc.) that requires the rescue of those on board; and

	the ship is found to be in a distress situation and those on board have already been rescued, with the possible exception of those who have remained aboard or have been placed on board to attempt to deal with the ship’s situation.

If, however, in an evolving situation, the persons on board find themselves in distress, the involvement of the MRCC and not the MAS will have priority. As such, the MAS is responsible only for receiving and transmitting communications and monitoring the situation.

The establishment of a MAS should not necessarily entail the setting up of a new organization. The functions of the MAS could, at the discretion of the Administration, be discharged by an existing organization, preferably an MRCC, or alternatively a harbour master’s office, a coast guard operations centre (if one exists) or another body.

Coastal States are requested to notify IMO of the existence and details (call numbers, call signs, etc.) of their MAS, in accordance with the predetermined format (A 23/Res.950, appendix to annex 2). The IMO Secretariat will periodically publish the collected particulars in a circular. National organizations that disseminate nautical information are invited to publish such particulars.

### Purpose

The primary mission of the MAS is to manage communications between the coastal State, ship's officers requiring assistance, and other responsible maritime organizations: fleet owners, salvage companies, port authorities, brokers, etc. As an intermediary, the main purposes of the MAS are:

•	 receiving the reports, consultations and notifications required by the IMO instruments referred to in Annex 1 to the present resolution;

•	monitoring the ship. situation if a report, as referred to in paragraph (a) above, discloses an incident that may cause the ship to be in need of assistance;

•	serving as the point of contact between the master and the coastal State concerned, if the ships situation requires exchanges of information between the ship and the coastal State but is not a distress situation that could lead to a search and rescue operation;

•	serving as the point of contact between those involved in a marine salvage operation undertaken by private facilities at the request of parties having a legitimate interest in the ship and the coastal State, if the coastal State concerned decides that it should monitor all phases of the operation

### Operational Approach

A 23/Res.950 indicates that a MAS should be operational on a 24-hour basis and that it should be possible for the English language to be used in exchanges between a ship in need of assistance and a MAS. Also, MASs should be authorized by their respective Governments to exchange with each other information concerning reports received and situations involving ships which may be in need of assistance.

The most common events requiring MAS are (IMO A23/Res. 949):

- Fire

- Explosion

- damage to the ship, including mechanical and/or structural failure

- collision

- pollution

- impaired vessel stability

- grounding.

Foreseeable consequences (including in the media) of the different scenarios envisaged with regard to safety of persons and pollution, fire, toxic and explosion risks.

The development of a database containing the key services contact data and availability of their respective resources would help move towards the digitalization of the MAS. Also, the utilization of templates listing the type of incidents that mostly occurred and their level of severity could facilitate ship – shore based communication.  The exchange of information on key services, their contact details, capabilities and geographic service areas can be developed as an S-100-based product specification. For the ship side, digital nautical publications in the S-12x-series of product specifications, such as S-123 (Marine Radio Service), can contain MAS information.

Figure xx illustrates briefly the different functions of a MAS.

Figure xx. Illustration of some functions provided by a MAS

### User needs

- Table X.X – Examples of types of information provision that MAS can cover

| Information related to: | Examples |
| --- | --- |
| General information | Examples of information: - competent MAS - roles and responsibilities of authorities and responders (fire fighting capability) - Responsible SAR coordination centre - Contact information to MRCC / operator - Ongoing operations in an area |
| MAS information | Examples Assistance information position, contact information, capacity etc: - Information about available resources for lightering - Information about available resources for pollution combating and recovery - Information about available resources for lightering - Information about available resources for towage - Information about available resources for stowage - Information about available resources for salvage - Information about available resources for storage. - Contingency planning - Evacuation facilities - Type of operation - What and when - Vessels involved - Communication |

In some these situations, the ship’s assistance may require the designation of a place of refuge in order to limit the navigational impacts, protect the environment or facilitate repairing the ship. In that case, an assessment of risks related to the identified event should be done taking into account:

1.	Environmental and social factors:

- safety of those on board

- threat to public safety

- pollution caused by the ship

- sensitive habitats and species

- facilities available

- etc.

2.	Natural conditions:

- Prevailing winds and sea conditions

- Tides and tidal currents

- Bathymetry

- Etc.

### Information to be Provided

See Annex 10, MS 10 Maritime Assistance Service

### Associated Technical Services

Regarding communication facilities, Res. 950 recommends to use a s a basis COMSAR/Circ.18, Guidance on minimum communication needs of maritime rescue co-ordination centres (MRCCs). This Circular identifies the following communication systems:

- Telephone links

- Fax links

- Telex link

- VHF and MF with DSC

- Ordinary VHF

- Special networks : examine the possibility of linking with existing networks (Administration, armed forces telephone networks), in particular the aeronautical fixed telecommunication network (AFTN), which provides the link with the aeronautical rescue organization.

- Inmarsat-C ship earth station or other satellite service providers.

- Etc.

S-123 (Marine Radio Service) data products contain contact details for MRCC.

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other the Maritime Services

| Description | Examples of data that could be used in MS 10 |
| --- | --- |
| MS 1 VTS IS | Local sensor information such as CCTV, Radar, AIS. Regulations. Other traffic. |
| MS 2 VTS NAS | Exchange of routs, directions, navigation advices and assistance |
| MS3 VTS TOS | Information regarding regulations and other traffic |
| MS 4 Local Port Service | Port availability and anchorage are in the port, services related to the MAS situation |
| MS 5 Maritime Safety Information | All information depending on the structure of MSI |
| MS 6 Pilotage Service | Contact information for pilotage, Pilot assistance, Pilot request |
| MS 7 Tug Service | Tug capacity, Contact information, Tug order and updates |
| MS 8 Vessel Shore reporting | Information about the reporting formalities, local regulations, contact information, Notification about dangerous cargo, number of persons onboard, arrival and departure information etc. |
| MS 9 Telemedical | Contact Information |
| MS 11 Nautical Chart Service | Local area updates, chart updates, Notice to Mariners |
| MS 12. Nautical Publications Service | Tidal tables, Notice to Mariners, List of lights, Information Updates to publications |
| MS 13 Ice Navigation Service | Ice routes, ice conditions, ice breaking assistance |
| MS 14 Meteorological Service | Weather information, visibility, wave information |
| MS 15 Real Time Hydro and Information Service | Horizontal and vertical Tidal information in an area, real time current, real time wave form sensors in an area |
| MS 16 Search and Rescue Service | Search and Rescue contact information, SAR capacity. SAR areas aon operations. |

## MS 11 Nautical Chart Service

### Submitting Organisation

Hydrographic Office

### Description of the Maritime Service

The aim of the nautical chart service is to safeguard navigation at sea by providing information such as nature and form of the coast, water depth, tides table, obstructions and other dangers to navigation, location and type of aids to navigation and is most often signatories to IMO SOLAS where the responsibilities of a hydrographic service is defined

### Purpose

Contracting Governments to SOLAS V undertake to arrange for the collection and compilation of hydrographic data and the publication, dissemination and keeping up to date of all nautical information necessary for safe navigation. In particular, Contracting Governments undertake to co-operate in carrying out, as far as possible, the following nautical and hydrographic services, in

the manner most suitable for the purpose of aiding navigation:

- to ensure that hydrographic surveying is carried out, as far as possible, adequate to the requirements of safe navigation;

- to prepare and issue nautical charts, sailing directions, lists of lights, tide tables and other nautical publications, where applicable, satisfying the needs of safe navigation;

- to promulgate notices to mariners in order that nautical charts and publications are kept, as far as possible, up to date; and

- to provide data management arrangements to support these services.

Contracting Governments undertake to ensure the greatest possible uniformity in charts and nautical publications and to take into account, whenever possible, relevant international resolutions and recommendations, which means the appropriate resolutions and recommendations adopted by the International Hydrographic Organization (IHO).

IHO facilitate co-ordination of the hydrographic office activities to the greatest possible degree in order to ensure that hydrographic and nautical information is made available on a world-wide scale as timely, reliably, and unambiguously as possible.

In nearly all countries, chart data is currently available to end-users only on a commercial basis, either directly from the hydrographic office, via commercial data vendors, or chart agents. This model is expected to continue for the foreseeable future. Updates are available by both physical media (CD/DVD) and online, over the Internet or by e-mail. Online update services are accessible either directly via off-the-shelf tools (e.g., Web browsers) or via custom applications or software modules in applications.

Vendors supply value-added data (VAD) supplied by vendors (this means VAD that is intrinsically chart data, not an overlay of other information). Such information is considered unofficial data.

Raster charts services also continue to be available and there is expected to be a continued demand during the transition period at least.

Examples of Nautical Chart Service is listed in Table x.

Table x – Nautical Chart Service

| Information related to: | Examples: |
| --- | --- |
| Navigational charts | Paper charts Raster Navigational Charts (RNC) Electronic Navigation Charts (ENC) |
| Chart catalogue | Catalogue of available products |
| Bathymetric charts | Bathymetric Information with greater details than normally given in a navigational chart |
| Notice to Mariners | Regular updating service for products issued by the individual Nautical Chart Service. |
| Hydrography | Information that will include factors such as the stability of the seabed, sea depth, the accuracy of surveys, tidal ranges, tidal streams, prevailing currents and swell, etc. |

### Operational Approach

A hydrographic service produces and distribute paper charts, ENCs and other products to safeguard navigation at sea. These services are carried out as defined in the IHO publications M-3 and for ENCs the WEND (Worldwide Electronic Navigational Chart Database) principles (M-3, Resolutions of the IHO – Resolution 1/1997 (as amended)) which note that:

“The purpose of WEND is to ensure a world-wide consistent level of high-quality, updated official ENCs through integrated services that support chart carriage requirements of SOLAS Chapter V, and the requirements of the IMO Performance Standards for ECDIS.”

The IHO S-65 standard note that hydrographic offices are responsible for:

- The preparation and provision of digital data and its subsequent updating for waters of national jurisdiction.

- Ensuring that, mariners, anywhere in the world, can obtain fully updated ENCs for all shipping routes and ports across the world and that their ENC data are available to users through integrated services.

- Assuring the high quality of its ENC services through the use of a Quality Management System that is certified by a relevant body as conforming to a suitable recognised standard; typically this will be ISO 9001.

- Ensuring compliance with all relevant IHO and IMO standards and criteria (including IHO S-57, IHO S-52, or their replacements).

- Providing timely updates to the ENC for the mariner; these should be at least as frequent and timely as those provided by the Contracting Government for the correction of paper charts.

Similar arrangements and principles apply to nautical charts and other nautical products (e.g. S-102 high definition bathymetry) made available by hydrographic offices.

[This section reuses material from S-65. S-65 contains this copyright statement: In the event that this document [S-65] or partial material from this document [S-65] is reproduced, translated or distributed under the terms described above, the following statements are to be included:

“Material from IHO publication [reference to extract: Title, Edition] is reproduced with the permission of the IHO Secretariat (Permission No ……./…) acting for the International Hydrographic Organization (IHO), which does not accept responsibility for the correctness of the material as reproduced: in case of doubt, the IHO’s authentic text shall prevail. The incorporation of material sourced from IHO shall not be construed as constituting an endorsement by IHO of this product.”]

### User Needs

Nautical chart service should offer both paper charts as well as digital charts in raster and vector formats for use in ECDIS or ECS, depending on vessel classification. Nautical charts should be made available as widely as possible utilizing various distribution channels, including direct distribution, agents and other service providers. ENC may be distributed via a RENC, which distribute them onward with the distributor network.

Figure 4.11-1 - General principle of ENC distribution (Source: S-65)

A chart catalogue service should be included as a part of a Nautical Chart service to give service users a quick reference guide to products offered by the nautical chart service.

IHO is developing the next version of ENC, called S-101. It should be expected that a new ECDIS or ECS system may be needed to utilize these next generation ENC. For future nautical chart services, the commercial model of distribution is expected to continue. Chart vendors provide for licensing and payment on a when-needed basis even now, and this will continue to be a requirement. The technical specifications of this service must support licensing, payment (both subscription-based and spot payments), and data protection.

Requirements for future services in this portfolio include faster turnaround for the process between mariner reports to agencies to the issuance of chart updates, making it easier to include more types of relevant information with reports such as information in mixed media, and reporting methods that are better integrated with ENCs. For example, initial generation of a hydrographic report on an ECDIS followed by automatic transfer of the initial report to a back-of-bridge system and thence to the appropriate hydrographic office.

Provision should be made for value-added data (VAD) and value-added services (VAS) supplied by vendors (this means VAD that is intrinsically chart data, not an overlay of other information), including SENC distribution.

Raster charts services also continue to be available and there is expected to be a continued demand during the transition period at least.

### Information to be Provided

See Annex 11, MS 11 Nautical Chaart Service Service

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Maritime Services

MS 12 and DMS 13 will supplement MS 11 data for navigation purposes. MS 5 will supplement it with up-to-date information on the status of navigation aids, obstructions to shipping and other events that impact navigational safety.

## MS 12 Nautical publications service

### Submitting Organisation

### Description of the Maritime Service

The aim of the nautical publication service is to promote navigation awareness and safe navigation of ships. Nautical publications include, tidal currents, aids to navigation system, buoys and fog signals, radio aids to marine navigation, chart symbols, terms and abbreviations, sailing directions and notices to mariners and local notices. Nautical publications information often includes information necessary for navigation that cannot be included in a chart service, for example because it consists of blocks of text, or information of use to the mariner but not essential to route monitoring. When the S-100 based product specifications are in full use, the line between ENC and many nautical publications will be based on content rather than format, since both will be available as feature-based datasets.

### Purpose

A nautical publication service is normally provided by a hydrographic service by a nation that is most often a signatory to IMO SOLAS where the responsibilities of a hydrographic service is defined as follows

Contracting Governments to SOLAS V undertake to arrange for the collection and compilation of hydrographic data and the publication, dissemination and keeping up to date of all nautical information necessary for safe navigation. In particular, Contracting Governments undertake to co-operate in carrying out, as far as possible, the following nautical and hydrographic services, in the manner most suitable for the purpose of aiding navigation:

- to ensure that hydrographic surveying is carried out, as far as possible, adequate to the requirements of safe navigation;

- to prepare and issue nautical charts, sailing directions, lists of lights, tide tables and other nautical publications, where applicable, satisfying the needs of safe navigation;

- to promulgate notices to mariners in order that nautical charts and publications are kept, as far as possible, up to date; and

- to provide data management arrangements to support these services.

Contracting Governments undertake to ensure the greatest possible uniformity in charts and nautical publications and to take into account, whenever possible, relevant international resolutions and recommendations, which means the appropriate resolutions and recommendations adopted by the International Hydrographic Organization (IHO).

IHO facilitate co-ordination of the hydrographic office activities to the greatest possible degree in order to ensure that hydrographic and nautical information is made available on a world-wide scale as timely, reliably, and unambiguously as possible. Moreover, the IHO publication M-3 contains technical resolutions and recommendations that define how nautical publications shall be produced to be compliant with SOLAS (NAUTICAL PUBLICATIONS AND THE SOLAS CONVENTION, 3/2002 as amended).

While they have been largely book-based until now, IHO is actively working on making several of them more ECDIS-friendly in the sense of turning them into feature-based datasets.

Nautical publications are currently nearly always provided as printed and digital paper publications that are nearly always distributed via commercial channels. Updates and corrections especially to the status of navigation aids and lights are often distributed via agency web sites as well as the distribution channels for the originals.

Examples of Nautical Publications Service are listed in Table x.

Table x – Nautical Publications Service

| Information related to: | Examples: |
| --- | --- |
| Transits and routeing | Routes in constricted shipping lanes or Routeing measures, traffic separation schemes, and shipping lanes |
| Summary information about port facilities | Depth alongside berths, and quay lengths Cargo handling facilities at specified terminals and berths |
| Variations from charted information | Tendencies toward silting at river mouths, shifting sandbanks, etc., that may degrade the accuracy of charted information |
| Marine radio services | Geographic availability of services Frequencies and channels used and broadcast schedules Type of traffic supported – Weather forecasts, radiofax, telemedical assistance, etc. |
| Protected area information | Locations of marine protected areas Restrictions and regulations applicable within specific areas |
| Prevailing natural conditions | Seasonal hazardous conditions Periodic (e.g., tide-related) or irregular hazardous conditions |
| Regulatory information | Laws and regulations applicable in specific locations. Laws and regulations applying to vessels of specific dimensions or carrying specified cargo Local rules regarding use of specific pilot boarding places by vessels exceeding specified dimensions or carrying hazardous cargo |
| Services | Waste disposal, repair, bunkering, collection of ship pollutants such as oily wastes Pilot services contact information and notice times |
| Navigation aids | List of Lights |
| Tide information and forecasts | Tide tables, tide stream atlases |
| Planning | Routeing guides |
| Controlled areas | VTS contact information |

### Scope

MS12 can be delivered in all sea areas (1-6).

### Objective

A hydrographic service produces and distribute nautical publications to promote navigation awareness and safe navigation of ships. These services are carried out as defined in the IHO publication M-3. Technical resolution 1/2002 gives the following exhaustive list of nautical publications, but note that other publications may also be added.

Distance Tables

List of Buoys and Beacons

List of Lights

List of Radio Signals

List of Symbols, Abbreviations and Terms used on Charts

Mariners’ Handbooks

Notices to Mariners

Routeing Guides

Sailing Directions

Tidal Stream Atlases

Tide Tables

Moreover, M 3 states that nautical publications may be in printed form or digital form (Technical resolution 2/2002 as amended). The digital form may be a digital representation of the printed version, such as is often the case with PDF files, and it may be in other forms such as XML. M-3 classifies the different types of nautical publications in the following manner;

NP1 – Printed paper publications

NP2 – Digital publications based upon existing paper publications

NP3 – Digital dataset(s) fully compatible with ECDIS that serve the purpose otherwise provided by NP1 or NP2.

It should be noted that Data Specifications for NP3 have yet to be finalised and IHO is working on developing S-100 based product specifications that are functionally equivalent to the paper chart versions These are often referred to as the S-12x series of standards.

### Operational Approach

### User Needs

Users should be trained in understanding the use of nautical publications.

SOLAS V, Regulation 27 require that nautical publications must be adequate for the intended journey, and always be kept up to date. Keeping the nautical publications updated should be done using the means provided by the issuing hydrographic office or approved service provider. A shore-based Internet service that is kept up-to-date with carriage requirements and information about publication updates would help masters and navigation officers in ensuring that digital publications on board are adequate and up-to-date for an intended voyage. Such a service might accept ports of call for a voyage, vessel and cargo characteristics, and return a catalogue of required publications and their latest revision information, to be compared by on-board software aid to digital publications currently installed, resulting in generation of a compliance report for review and action by the navigation officer.

NP2 nautical publications may be used back of bridge or even on a tablet computer. NP3 is expected to interact in various degrees with the next generation ENC within the ECDIS or ECS system, though some will not be included in an ECDIS at all – e.g., routeing guides.

Short update cycles are preferable – but must be consistent with the nature of the content and the needs of the vessel. For example, port information for a destination on the current voyage is higher priority than Chart No. 1 or the List of lights – and information about changes to the status of a navaid are higher priority than most other information.

Updates should be in a form that can be applied automatically to on-board datasets.

Automated aids for reporting discrepancies should be provided.

Consumers of the service should have the ability to obtain up-to-date information pertaining to planned voyage, not just in the present area.

The services should be capable of working within multiple levels of bandwidth limitations, which will vary depending on whether the vessel is on the high seas, approaching a harbour, engaged in loading/unloading operations in port, etc. Performance should degrade gracefully with reductions in bandwidth, e.g., for low-bandwidth channels transfer of large graphics files may be deferred or replaced with lower-quality files.

Pictorial information needs to be transferred, but consistent with bandwidth limitations, if any, e.g., deferred updates for non-essential graphics, or transmission of lower-quality graphics.

Services should be compatible with WFS and WCS (Web Coverage Service) standards for providing up-to-date information about individual features, features within specified spatial constraints, and coverage data within specified spatial constraints. This will need to be integrated with licensing and subscription-based access at least, probably data protection as well.

In cases where information exists in narrative form, the information will need to be summarized or condensed by a producer before it is disseminated.

Some information (such as text from sailing directions) will have to be converted from narrative to geographic form.

### Information to be Provided

See Annex 12, MS 12 Nautical Publications Service

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Maritime Services

May give overviews and summarize information in several other MSs, for example MS1, MS13, climatological information from MS14, MS6, MS3, etc. Information in this MS supplements charted data in MS 11. Ice navigation (MS 13) will reuse certain information such as radio services information. The advent of S-100 and creation of S-100-based products including data traditionally in nautical publications will blur the line between MS11 and MS 12.

## MS 13 Ice navigation service

### Submitting Organisation

World Meteorological Organisation

### Description of the Maritime Service

To provide ice navigation information digitally to ships

### Purpose

- The World Meteorological Organization’s Manual on Marine Meteorology (WMO No. 558) defines the procedures for marine meteorological information text bulletins involving ice-related components in High Seas areas, and Coastal, Offshore and Local Waters.

- SOLAS V Regulation 5 outlines obligations for the provision of weather information suitable for shipping consisting of forecasts including ice conditions and hazards.

- The IMO Polar Code (MSC, MEPC) outlines the environmental information requirements for ships operating in polar waters.  The IMO Polar Code reinforces operating guidelines for the hazard of Ice Accretion and Ice Waters (sea-ice conditions), and introduces new operating guidelines for hazards related to Low Air Temperature, and defines the Polar Service Temperature for equipment performance.

SOLAS V, regulation 6 governs the ice patrol service in the North Atlantic stating that “Ice Patrol contributes to safety of life at sea, safety and efficiency of navigation and protection of the marine environment in the North Atlantic. Ships transiting the region of icebergs guarded by the Ice Patrol during the ice season are required to make use of the services provided by the Ice Patrol.” This service is currently carried out jointly by USA and Canada.

- The IMO Assembly Resolution A.1051(27) outlines the functions of the Worldwide Met-Ocean Information and Warning Service. The Worldwide Met-Ocean Information and Warning Service (WWMIWS) provides met-ocean Maritime Safety Information (MSI) including sea-ice conditions and hazards to mariners.

- The standards for the portrayal of ice conditions on Electronic Navigational Chart systems are documented within S-411.

WMO publication 574 (Sea Ice Information Services of the World) has comprehensive information on National Ice Services.

### Operational Approach

Examples of Ice Navigation Services are listed in Table x.

Table x – Ice Navigation Service

| Information related to: | Examples: |
| --- | --- |
| Ice conditions | Near real-time and forecasts of: - Sea ice concentrations - Sea ice stage of development (type or thickness) - Iceberg location and drift - Limit of all known sea ice or icebergs |
| Ice reports and bulletins | - Text summaries of ice conditions |
| Routeing aid | - Recommended routes - Icebreaker assistance |

The most important features of sea ice which affect marine operations are:

(a)	the amount of ice present, I.e. concentration usually measured as tenths of the sea surface covered by ice;

(b)	ice thickness, referred to as stage of development which is related to ice age;

(c)	form of ice, i.e. whether it is fast ice or pack ice, floe size, and the amount of ridging; and

(d)	ice movement.

The position of icebergs at specified times are required with information about their estimated size; and speed and direction of movement.

Warnings of ice accretion highlight areas where the accumulation of ice on the superstructure and deck equipment of vessels may seriously affect safety and operational efficiency.

Information about extremely low air temperatures is important for the safety of workers, whilst historical information about cold air temperatures enables planning and ship design based on the Polar Service Temperature guidelines.

#### Ice charting

Some 20 nations around the world offer an ice information service, Services may provide ice forecasts once a day for a period of 24 to 144 hours. These are tactical forecasts and may provide advice on difficult ice conditions forming or dissipating, the general motion of the pack, opening and closing of leads, etc. They are strongly influenced by meteorological prediction and should always be used in concert with the weather forecast.

Other longer-range predictions – those covering periods from 7–10 days to 30 days and seasonal predictions – are  based on climatological,  analogue or statistical methods.

#### Vessel Escort and Ice breaking

Icebreaking and support services may be available to ships transiting ice-covered waters. Coast Guards or other national agencies may operate Ice Operations Centres. These Centres generally provide up-to-date ice information, suggest routes for ships to follow through or around ice, and co-ordinate icebreaker assistance to shipping. Ice Operations Centres are generally in contact with icebreakers at all times and monitor progress of shipping within their area of responsibility. Ice Operations Centres may also provide Recommended Ice Routing services, such as routing maps.

### User Needs

Activity based information requirements

| Information related to | Examples |
| --- | --- |
| En-route or at sea | Broad, area-based forecasts Higher detail in complex waterways Increased interest in synoptic features and movement Longer forecast lead-time essential |
| Entering, transiting and exiting a port | Point (small area) based forecasts, High spatial and temporal detail Real-time observations Focus on short-term lead-times |
| At berth | Forecasts of changes to ice conditions |
| Planning a trip | Focus on short-term timeframes, as well as longer forecast lead-times Forecasts and warnings Specific details on timing of wind changes or hazardous weather leading to changes in ice conditions Focus on forecast details for specific areas or routes |
| Vessel and equipment design | Historical values of low air temperatures and water temperatures Focus on ocean and sea routes |

Formats

Ice information may be provided in a number of formats to meet user requirements.  The following descriptions outline some of the benefits and constraints for each format:

- Map

  - Map displays provide highly detailed information across defined spatial domains, and if provided as a time sequence, a user can study the evolution of the weather, ice and ocean elements during specific time periods.  Colours may be used to highlight hazards or important conditions to operations. Maps may be produced in an image format, or provided as a web map service, gridded or S-100 compatible file for display by on-screen software.

- Text

  - Text products provide short summaries and broader detail for a defined area and time period.  These text products may be simpler to interpret for most users, and can be used for marine radio broadcasts.  Text products generally have a small file size for internet dissemination to mariners at sea.

- Voice

  - Voice products may be transmitted as audio or by video accompanied by other formats.

- Table

  - Information displayed as a table is usually for a specific location, so a user will get the benefit of detailed information over a period of time for that location but may lose context of what is happening over nearby areas.

- Grids

  - Gridded information may be integrated into decision support systems and situational awareness tools, or interrogated to output customized information for the particular marine activity or operating risk threshold.

#### Examples

Figure xx: Example of iceberg analysis.

Figure xxExample of a Recommended Ice Route in the Gulf of St. Lawrence (source ”Ice Navigation in Canadian Waters” – Canadian Coast Guard )

Figure xx - Example of Sea Ice satellite service (www.polarview.aq)

Figure xx – A MODIS satellite image taken on March 17, 2015 of the Gulf of St. Lawrence. The image highlights compacted first-year ice along Anticosti Island as well as young ice and grey-white ice off the coast. (Image credit: NASA)

### Information to be Provided

See Annex 13, MS 13 Ice Navigation Service Service

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Maritime Services

MS 11, MS 14: MS 14 information in particular sea temperature charts and meteorological information are essential for mariners to make informed use of ice charts. MS 13 data may be overlays to MS 11 (chart data) in navigation systems.

## MS 14 Meteorological information service

### Submitting Organisation

World Meteorological Organization

### Description of the Maritime Service

To provide meteorological information digitally to ships

### Purpose

- The World Meteorological Organization’s Manual on Marine Meteorology (WMO No. 558) defines two types of marine meteorological information:

- Forecasts and warnings for the High Seas; and

- Forecasts and warnings for coastal, offshore and local areas (including ports and harbours).

- SOLAS V Regulation 5 obligates contracting parties to produce and distribute to shipping warnings about severe weather such as gales, storms and tropical cyclones, and to produce and provide other weather information suitable for shipping consisting of data, analyses, warnings and forecasts of weather, waves and ice.

- The IMO Assembly Resolution A.1051(27) outlines the functions of the Worldwide Met-Ocean Information and Warning Service. The Worldwide Met-Ocean Information and Warning Service (WWMIWS) provides Maritime Safety Information (MSI) to mariners in the form of marine forecast and warning products. The WWMIWS is coordinated across the worlds’ oceans through 21 defined areas, called METAREAs. Ships receive the MSI products via marine communication systems such as SafetyNet and NAVTEX, which form part of the Global Maritime Distress and Safety System (GMDSS).

- The IMO Polar Code (MSC, MEPC) outlines the environmental information requirements for ships operating in polar waters.  The IMO Polar Code reinforces operating guidelines for the hazard of Ice Accretion and Ice Waters (sea-ice conditions), and introduces new operating guidelines for hazards related to Low Air Temperature, and defines the Polar Service Temperature for equipment performance.

- Weather routeing services are provided in accordance with SOLAS Chapter V, Regulation 34, and IMO Resolution A.893 and IMO MSC/Circular 1063 – Minimum Standards for Provision of Weather Routeing Services, outlines the minimum characteristics for a service.  The SOLAS Chapter V, Regulation 5 states that met-ocean services shall be issued by the National Meteorological Service, and this would imply that WMO and its Members should oversee weather routeing services and standards as well.

- The standards for the portrayal of met-ocean conditions on Electronic Navigational Chart systems are documented within S-412.

- Details of service availability, broadcast times and radio frequencies for services provided to vessels at sea are maintained in the WMO publication: WMO No. 9, Volume D, Information for Shipping.

Examples of Meteorological Information Service is listed in Table x.

Table x – Meteorological Information Service

| Information related to: | Examples: |
| --- | --- |
| Wind | Wind speed, direction, gust information Real-time values from instruments or satellite |
| Waves | Forecast wave height, direction, period Real-time values from buoys or satellite |
| Atmospheric conditions | Forecast temperature, squalls, cloud, rainfall Real-time values from instruments or satellite |
| Ocean | Forecast surface temperature, currents, salinity Forecast sub-surface temperature and currents Real-time values from instruments and satellite |
| Weather systems | Mean Sea Level Pressure contours System features such as cold fronts, tropical cyclones, low pressure centres, high pressure centres Satellite images |
| Dangerous weather | Warnings about location, strength, and movement of storms Warnings about fog or phenomena causing reduced visibility, ice accretion, cold air temperature, squalls |
| Bulletins and forecasts | Surface weather analysis, synoptic features with barometric pressure Forecasts of wind, waves, weather |
| Polar Service Temperature | Historical values for ocean and port areas |
| Low Air Temperature | Forecasts of hazard areas Historical values for ocean and port areas |
| Ship observations | Receipt of reports from ships in the Voluntary Observation System Transmission of information extracted from received ship reports to shipping |

MS14 can be delivered in all sea areas (1-6).

### Operational Approach

Broadly speaking, marine meteorological services have two functions:

- To serve international shipping, fishing and other marine activities on the high seas; and,

- To serve the various activities which take place in coastal and offshore areas, ports and on the coast.

SOLAS convention (Chapter V, Regulation 34, and within Annex A.24 Voyage Planning) describes how vessels should prepare for their trip and route and therefore their information requirements.  The Annex specifically outlines to small vessels the importance of:

- checking the weather forecast for the journey;

- knowing the tides;

- knowing the vessel limitations for the expected weather and wave conditions.

SOLAS V Regulation 5 describes the underlying obligations for weather services, i.e., conveying warnings about severe weather and other weather information useful for shipping, and facilitating weather reports by ships and their distribution as needed for the safety of navigation.

In general, the impact which could result from a meteorological condition depends on its severity and on the sensitivity of a particular activity or operation to that condition. Similarly, meteorological phenomena can make recreational activities and the work of fishing and shipping fleets much more difficult or hazardous.

Marine operations are sensitive to environmental conditions. Generally, extreme values of waves, wind and obstructions to visibility increase the risk to the safety of the vessel or sea structure and to the persons involved in the operation. Less extreme values, even if safety is not threatened, will affect the efficiency, effectiveness or comfort of the operation. The usefulness of a warning or a forecast depends on the accuracy of the prediction, the format and communication platform that the information is delivered, its timeliness, i.e. the number of hours or days in advance of the event that the forecast can be provided, and the ability of the user to react to the information.

### User Needs

#### Activity based information requirements

| Information related to | Examples |
| --- | --- |
| En-route or at sea | Broad, area-based forecasts Higher detail in complex waterways Increased interest in synoptic features and movement Longer forecast lead-time essential |
| Entering, transiting and exiting a port | Point (small area) based forecasts, High spatial and temporal detail Real-time observations Warnings of reduced visibility, squalls Focus on short-term lead-times |
| At berth | Warnings of squalls, thunderstorms Forecasts of general weather conditions |
| Planning a trip | Focus on short-term timeframes, as well as longer forecast lead-times Increased interest in synoptic features and movement Forecasts and warnings Specific details on timing of wind changes or hazardous weather Focus on forecast details for specific areas or routes |

#### Formats

Weather information may be provided in a number of formats to meet user requirements.  The following descriptions outline some of the benefits and constraints for each format:

- Map

  - Map displays provide highly detailed information across defined spatial domains, and if provided as a time sequence, a user can study the evolution of the weather and ocean elements during specific time periods.  Maps may be produced in an image format, or provided as a web map service, gridded or S-100 compatible file for display by on-screen software.

- Text

  - Text products provide short summaries and broader detail for a defined area and time period.  These text products may be simpler to interpret for most users, and can be used for marine radio broadcasts.  Text products generally have a small file size for internet dissemination to mariners at sea.

- Voice

  - Voice products may be transmitted as audio or by video accompanied by other formats.  There may be time limits imposed for radio broadcasts, and consideration should also be given to the reception quality on board vessels and impact on a mariner’s ability to interpret the information whilst doing other duties if the broadcast is too long.  These constraints have an impact on the information provided in the text product that it is based on.

- Table

  - Information displayed as a table is usually for a specific location, so a user will get the benefit of detailed information over a period of time for that location but may lose context of what is happening over nearby areas.

- Grids

  - Gridded information may be integrated into decision support systems and situational awareness tools, or interrogated to output customized information for the particular marine activity or operating risk threshold.

#### Examples

- Examples of products include surface analysis, wind and wave forecasts and analyses.

Figure 4.14-1. Surface analysis, West Atlantic. (NOAA)

Figure 4.14-2. Wind and wave forecast. (NOAA)

Figure 4.14-3. Wind and wave analysis (NOAA)

### Information to be Provided

See Annex 14, MS 14 Meteorological information service

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Digital Maritime Services

MS 5 for communicating warnings about extreme weather. Complements certain services in MS13, namely ice charts.

## MS 15 Real-time hydrographic and environmental information services

### Submitting Organisation

TBA

### Description of the Maritime Service

The real time hydrographic and environmental information service is essential to safeguard navigation at sea and protect the environment.  The services provided include:

current speed and direction;

wave height;

marine habitat and bathymetry;

- Sensor networks such as ODAS, wave radar, water level gauges, anemometers, current gauges, etc., are widespread and utilized in providing data for these services. This data is often made available on web sites, web mapping services, and other appropriate Internet locations, for example Web sites of major ports or from VTS. The format in which processed data from such sensors is available to the public appears to be variable.

Examples of Real-time Hydrographic and Environmental Information Service are listed in Table x.

Table x – Real-time Hydrographic and Environmental Information Service

| Information related to: | Examples: |
| --- | --- |
| Current speed and direction | databuoy information via AIS Surface current snapshots |
| Surface conditions | Wave heights via radar |
| Environmental conditions | Temperature, pressure, tendencies (rising/falling) Wind speed and direction Visibility |
| Water column | Depths, salinity, temperatures |

In general MSP 15 services can be delivered in all sea areas (1-6). Specific services may be limited to areas where the type of hydrographic and environmental information conveyed by the service is collected.

### Purpose

The main objective of services in this portfolio is similar to MS 14, and consists of conveying hydrographic and environmental information to shipping where needed, including in the middle of ocean voyages, robustly under different conditions especially severe weather, in a timely fashion, and in a form, that is of maximum practicable utility to mariners and conveys the maximum appropriate information. Information must be conveyed by means and formats that are easy for ships to receive. IHO M-3 2/1962 as amended recommend that the results of oceanographic observations be communicated for the maximum utilization by all marine scientific and hydrographic users.

The transmission of environmental conditions by ships and the distribution of information gathered from weather reports to shipping, is also an objective.

Local and regional services may transmit hydrographic and environmental information with AIS-ASM, specifically messages 6 and 8.

IHO is developing a product specification for surface currents named S-111, which is intended for use in ECDIS.

### Operational Approach

### User Needs

Real-time hydrographic and environmental information is distributed in a variety of ways, including radio, AIS-ASM, and internet.

For accurate onboard display of AIS messages, receiving vessels must be equipped with a proper Class A AIS device.

Internet access and possibly a user account is required to access real-time hydrographic and environmental information via internet.

### Information to be Provided

See Annex 15, DMS 15 Real time hydrographic and environmental information services

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardization Body |
| --- | --- | --- | --- | --- |

### Relation to other Maritime Services

MS1 for communication. MS5 for communicating warnings about extreme cases. MS14 for meteorological information.

## MS 16 Search and Rescue (SAR) Service [NORWAY] +IMRF+sweden

### Submitting Organisation

### Description of the Maritime Service

The 1979 Convention was aimed at developing an international SAR plan, so that, no matter where an accident occurs, the rescue of persons in distress at sea will be co-ordinated by a SAR organization and, when necessary, by co-operation between neighbouring SAR organizations.

Following the adoption of the 1979 SAR Convention, IMO's Maritime Safety Committee divided the world's oceans into 13 search and rescue areas, in each of which the countries concerned have delimited search and rescue regions for which they are responsible.

Provisional search and rescue plans for all of these areas were completed when plans for the Indian Ocean were finalized at a conference held in Fremantle, Western Australia in September 1998.

Parties to the Convention are required to ensure that arrangements are made for the provision of adequate SAR services in their coastal waters. Parties are encouraged to enter into SAR agreements with neighbouring States involving the establishment of SAR regions, the pooling of facilities, establishment of common procedures, training and liaison visits. The Convention states that Parties should take measures to expedite entry into its territorial waters of rescue units from other Parties.

The Convention then goes on to establish preparatory measures which should be taken, including the establishment of rescue co-ordination centres and subcentres. It outlines operating procedures to be followed in the event of emergencies or alerts and during SAR operations. This includes the designation of an on-scene commander and his duties.

In an SAR situation, effective use of time, allocation of vessel and persons in distress and priority of the best available rescue resources is crucial.

Several actors are normally involved in an SAR operation. Manually exchange of information can be misunderstood. Wrong positions, SAR areas end other information can make the different be the different between life or lost of life.

The e-navigation process on user needs revealed that lack of language skills was a major challenge.

Digital exchange and sharing of information will reduce the human errors. The MRCC can get more aducate and timely access to relevant information.

The MRCC can deliver information about the situation and the SAR area to a graphical display onboard and share the same information and situation picture with all partners involved.

### Purpose

SAR of human lives will always be a challenge when it comes to time and resources.  Information for SAR coordination such as SAR areas, the vessels involved and drift predictions can be shared electronically.  This will make it easier for all units to get up-to-date information the search operation underway.

Information from several services will be useful to the SAR coordinator’s capacity to reduce time and make use of the appropriate resources.

Information about vessels in the area, ice information, meteorological information, MSI, cargo information, capacities such as medical, fire fighting, tide and current, ship reporting information such as persons on board, incidents etc.

### Operational Approach

The SAR service is responsible for assisting, coordinating search and rescue operations at sea. In maintaining a state of full readiness, the Services may assist the following search and rescue functions:

The crew and passengers of vessels in distress;

Victims of maritime and aircraft accidents or incidents.

The SAR services must also coordinate the evacuation of seriously injured or ill person from a vessel at sea when the person requires medical treatment sooner than the vessel would be able to get him or her to a suitable medical facility.

The Services may also be pro-actively involved in activities such as:

Information collection, distribution, and coordination;

Monitoring towing operations;

Monitors and evaluates levels of risk from Maritime Safety Information (MSI) broadcasts to ensure an immediate response in case of life threatening situations developing;

Monitoring vessels not under command;

Pollution reports and vessels aground.

### User Needs

| Information related to: | Examples |
| --- | --- |
| General information | Examples of information: - Responsible SAR coordination centre - Contact information to MRCC / operator - Ongoing operations in an area |
| SAR information | Examples of tug information: - Type of operation - What and when - Position - Number persons missing - Dangers such as dangerous substances - In raft, lifeboat, persons in water etc - Datum - SAR area - Vessels involved - on-scene commander - Communication |
| Assistance | Examples of assistance: - Position of vessels or station - Capacity, Bollard pull (BP), - Other salvage and oil recovery capacities - Response time - Connection information - Intended route / towing plan - speed limits - Maneuvering limits - Communication |

Figure x-x-x: Example on common sharing of digital Sears And Rescue information

### Information to be Provided

See Annex 16, MS 16 Search and Rescue Service

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Digital Maritime Services

## MS 17 Aids to Navigation services (AtoN)

### Submitting Organisation

### Description of the Maritime Service

### Purpose

### Operational Approach

### User Needs

### Information to be Provided

Annex 17, MS 17 Aids to Navigation Services (AtoN)

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Maritime Services

## MS19 PNT and augmentation services [UK GLA]

### Submitting Organisation

### Description of Maritime Service

### Purpose

### Operational Approach

### User Needs

### Information to be Provided

Annex 18, MS 18 PNT and Augmentation Services

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Maritime Services