Input paper:   ENG22-3.1.1.1

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	X  ENG		□  PAP					x Input

□  DTEC	□  VTS							□  Information

Agenda item 			n.n

Technical Domain / Task Number 2	Task 2.4.1

Author(s) / Submitter(s)			…William Dunning…

# Update to G1036 to include greenhouse gas emissions calculations guidance

# Summary

This paper presents a draft inclusion to IALA guideline G1036 – Environmental management in aids to navigation. This draft section includes a guide to member states on greenhouse gas (GHG) emissions calculating in line with the international best practise GHG protocol standards.

Taking the assumption of a member state with no existing systems deployed to account for and report on their greenhouse gas emissions, it provides a systematic approach to defining, collating, calculating and reporting on an organisations emissions in a principled manner.

If adopted it would enable a comprehensive and uniformed approach, paving the way for IALA wide understanding of its emissions.

## Purpose of the document

I expect this committee to review this input paper and where appropriate and agreed, adopted its contents into IALA guideline G1036.

## Related documents

IALA guideline G1036 – Environmental management in aids to navigation

# Background

During the IALA Sustainability Workshop held in October 2025 it was made apparent that there is significant disparity between member states on how greenhouse gas emissions are calculated. Some hold advanced levels of understanding with others as yet unsure where to start.

IALA guideline G1036 – Environmental management in aids to navigation introduces the topic of carbon foot printing and although a good place to start it does offer more guidance on what to measure, whereas some member states will need guidance on how to calculate GHG emissions.

# Discussion

It would be useful to update the existing Guidance G1036 – Environmental management in aids to navigation, to include further guidance on how to calculate carbon emissions in a more structured approach so that member states have a uniform and systematic approach to emissions measuring and reporting. This would enable better understanding when communicating sustainability topics amongst member states and compatibility, which is required in order to evidence progress and sharing of meaningful best practice activities.

One such approach could be to present the Greenhouse Gas Protocol standards for emissions accounting and reporting. These global standards are widely seen as best practice in the sector and are used amongst some existing member states.

A potential guideline inclusion referencing the principles and sentiment of those GHG protocol standards and some applicability to member states has been included in Annex 1.

I would welcome feedback on the inclusion of these principles and steps within the guidance G1036 and time within the workplan to develop it further so that it captures the language and delivery required to be as accessible to member states as possible.

# References

GHG Protocol Corporate Accounting and reporting Standard, revised version

GHG Protocol Technical guidance for Calculating Scope 3 emissions, version 1.0

# Action requested of the Committee

The Committee is requested to:

- Note the guidance presented, feedback on its suitbalitiy and action its inclusion into guidance where appropriate

- Annex 1

# Proposed inclusion to G1036

# Introduction

## Principles

As is common within standards, there are principles that underpin these guidelines to ensure that the outputs are of the expected quality and to ensure they produce a true and fair account of an organisations GHG emissions. They are; relevance, consistency, completeness, accuracy and transparency.

### Relevance

The data collected needs to provide value to those that use it for decision making. Appropriate data set within the appropriate boundaries should be dependant on the characteristics of the organisation, the needs of that organisation and the indented use for the data.

### Consistency

To track and compare GHG emissions over time, to identify trends and assess performance the consistent application of accounting approaches and calculation methodologies is essential. Information needs to be complied in a manner that ensures it is internally consistent and comparable over time.

### Completeness

All relevant emissions sources within the organisation’s boundary must be accounted for so a meaningful and comprehensive inventory is complied. In practice, a lack of data may be a limiting factor. It is important that this is documented and justified, so the potential impact of the exclusion or lack of quality can be assessed by the end users.

### Accuracy

Data needs to be collected in a manner that minimises uncertainty. It needs to be precise and within agreed thresholds so that it can be assured and relied upon. Reporting on data management can promote this credibility.

### Transparency

Information should be recorded, complied and analysed in a manner than enables internal review and external verification, it should be sufficient to enable a third party to derive the same results if provided with the same source data. Accessibility to the processes, procedures, assumptions and limitations of the data provides a clear context for emissions reporting and is considered a bed-rock to good practice.

# 5 steps for ghg EMISSIONS calculations

A true and fair account of an organisations GHG emissions can be achieved in 5 simple steps. They are:

  - Set organisational boundaries

  - Set operational boundaries

  - Set a baseline year

  - Identify and calculate GHG emissions

  - Report of emissions

These can be adopted in any size organisation and to whatever level of granularity the organisation has the capabilities to achieve.

## Set organisational boundaries

Organisations vary in their size and structure, they could include wholly own entities, incorporated or non-incorporated ventures and subsidiaries amongst other formats. In setting organisational boundaries, an organisation selects an approach for consolidating their GHG emissions and consistently applies this within their accounting and reporting. There are two main approaches: equity share and the control approaches.

### Equity share

An organisations accounts for the GHG emissions from operations according to its share of equity in the operation. Typically, this is aligned with the organisation’s percentage ownership of that operation.

### Control approach

Under the control approach an organisation accounts for 100% of the GHG emissions from operations over which it has control. Control can be defined in either financial or operational terms and organisations must choose between the two.

Financial control

Under the financial control approach, an organisation accounts for 100% of the emissions from operations where it has the authority to direct financial policies for economic gain.

Operational control

Under the operational control approach, an organisation reports 100% of the emissions from operations over which it has the authority to introduce and enforce policies, even if it doesn’t own the entity.

Organisations need to decide on the best approach suited to their business activities as it will have an impact on the size and data requirement of their accounting and reporting, there may be an instance when both (control and equity) is appropriate, but they must be reported separately.

By defining this an organisation understands is accountability better which allows it to target its efforts where it has direct influence, making reductions more feasible and measurable.

## Set operational boundaries

Once achieved an organisation should set its operational boundaries, this involves identifying emissions associated with its operations, categorising them as direct or indirect emissions and choosing the scope of emissions for which they will report.

A vital step in reporting, this enables an organisation to define the origin of their emissions providing a holistic view of their emissions across their value chain, enabling more targeted management of its emissions.

Based around ownership there are 3 scopes for which emissions are categorised: Scope 1, Scope 2 and Scope 3.

### Scope 1

Otherwise known as direct emissions, this covers emissions from sources owned or controlled by an organisations, such as fuel in boilers and vehicles.

### Scope 2

Referred to as indirect emissions, this covers emissions from the generation of purchased electricity, steam, heat or cooling that is purchased off site and consumed by an organisation.

### Scope 3

Somewhat ambiguous, this scope encompasses everything else in the value chain, capturing emissions an organisation is responsible for outside of Scope 1 and Scope 2. This is almost always the largest category within an organisation, due to the breadth of its scope. This is visualised in Figure 1.

Figure 1, GHG Protocol Scopes

Scope 3 is considered as optional to capture, although as an organisations maturity expands they require the oversight this data capture provides and so is widely adopted, although in fragmented form. Often comprising of Scope 3 categories that have readily available datasets.

These scopes are commonly used to communicate an organisations emissions and are also used as the basis for reduction targets. For example, in the United Kingdom, the UK government use Scope 1 and 2 reduction targets as interim goals for achieving net zero by 2050.

## Setting a baseline year

To reliably grade performance, organisations need to track emissions over time and to achieve this need a datum from which progression is scored against. In emission reporting this datum is referred to as the baseline year and is an annual emissions amount quantified over 1 calendar or financial year of operating.

### Choosing a baseline

Organisations should choose a year at the earliest relevant point in time for which they have reliable data for. It should be a year where your current operations align with the operational requirements of that time and be a year without an either abnormally high or abnormally low footprint (caused by significant restructuring/pandemics/closures e.ct).

### Recalculating a baseline

As an organisation changes it may be required to recalculate your baseline and baseline year. This may be due to significant restructuring, a move to more outsourcing of services, or divestitures due to changing requirements. A recalculation could also be as a result of updates to calculation methodologies or more accurate data collection methods, or due to a discovery of data errors with a magnitude that materially impact the current baseline. Finally some regions may impose changes through legislative requirements, for example the EU European Sustainability Reporting Standards mandate an update every 5 years.

## Identifying and calculating GHG emissions

Once an organisations boundaries have been clearly defined they can begin to calculate GHG emissions. These also can be broken down into 5 clear steps:

- Identify sources

- Select a calculation approach

- Collect data and choose emissions factors

- Apply calculations

- Roll up

### Identify sources

The first step is to further categorise the GHG emissions sources defined when setting your operational boundaries. This involves expanding the granularity of your Scope 1, Scope 2 and Scope 3 categories so that specifically developed methodologies can be used to calculate emissions based on sector and source category with higher accuracy.

Scope 1

Typically in Scope 1, emissions are attributed to four main sources:

Stationary combustion: from the combustion of fuels in stationary equipment like boilers, furnaces and generators

Mobile combustion: from the combustion of fuel in transport equipment, like cars, vans, planes and vessels

Process emissions: from the industrial, chemical or physical processing of materials, for example from surface treatment finishing like plating or from calcination in the production of concrete

Fugitive emissions: from intentional or unintentional releases of gases from plant and machinery, for example from air conditioning systems or fire suppression systems

An organisation should undertake an exercise to identify its direct emissions that fall into each of the four source categories mentioned above. Referencing your organisational boundaries here is important to ensure you only capture what you are responsible for.

Scope 2

Next you should identify all the sources of purchased electricity, heat or steam that you procure as an organisation. This information should be readily available from your finance departments or can be supplied upon request from your utility provider.

Scope 3

This step invoices identifying all the other indirect emissions from your organisations upstream and downstream activities within your value chain, otherwise not captured in Scope 1 or Scope 2. The GHG protocol presents 15 sub-categories that are mutually exclusive, which if captured comprehensively provides an organisation with a systematic framework to measure their emissions in its entirety. They are:

- Purchased goods and services

- Capital goods

- Fuel-and-energy-related activities not included in Scope 1 or Scope 2

- Upstream transportation and distribution

- Waste generated in operations

- Business travel

- Employee commuting

- Upstream leased assets

- Downstream transportation and distribution

- Processing of sold products

- Use of sold products

- End-of-life treatment of sold products

- Downstream leased assets

- Franchises

- Investments

Further description and guidance on calculation methodologies for these sub-categories can be found here: Scope3_Calculation_Guidance_0.pdf

Organisations should prioritise identifying and calculating their Scope 1 and Scope 2 emissions before investing significant resources into identification of Scope 3 emissions.

### Select a calculation approach

The most common and accessible approach for calculating emissions is through the use of emissions factors. Emissions factors are coefficients that describe the rate to which an activity releases greenhouse gases and are a key instrument in quantifying the environmental impact of the activities, services or products that an organisation creates. The use of these factors enables organisations to use readily available data on their organisations activity, known as activity data, to measure emissions release. These could include:

            - Fuel consumed

            - Energy used

            - Distance travelled

Other methods include direct measurement of gases releases, although accurately containing and measuring concentration and flow rate is difficult and un-common for most organisations.

### Collect data and choose emission factors

Using the GHG sources identified in section 2.4.1 organisations should now identify the potential activities that can be attributed to those, in turn determining points from which you can begin to measure your impact.

Scope 1

Calculations may come from the purchased quantities of fuel like natural gas, oil, petrol or diesel. This data may be found in utility bills, purchase orders or invoices.

Scope 2

Data points you can use could be found in metered electricity consumption, primarily from utility bills or invoices.

Scope 3

These emissions may be calculated from activity data such as fuel use, passenger miles, weights of materials or volumes of production.

Choosing emission factors

Emissions factors have a regional influence because they reflect local variables. The fuel mix of energy production, efficiency of power generation plants and locality of fuels or materials that are consumed all play into the particular emissions burden of a country.

Due to this it’s important to select emission factors relevant to your organisations locality, with many governments releasing official conversion factors to be used by organisations residing in country. In some cases, source specific emissions factors are available, these are common in specialist fuel industries and are preferred over more generic regional emissions averages.

### Apply calculations

How you collate your data and the tools use utilise to do so is largely dependant on your organisation, its existing systems, resourcing available and ambitions for the outputs.

Specialist tools are available that provide varying levels of high-end functionality, however automated spreadsheets with manual inputs for emissions factors can be as accurate as high end off the shelf softwares and can be deployed with a fraction of the cost.

This is where an IALA developed tool could be delivered.

### Roll up

To finalise this 5 step process your data needs to be collated and summarised into a format that can be used for internal decision making. It may be likely that this data comes from multiple facilities and organisation departments and to reduce the risk of human errors in data collection and to reduce the reporting burden you may wish to integrate GHG reporting within existing reporting processes, utilising their reporting formats and cadence to ensure it is comparable with data from around the organisation.

There are two basic approaches you may wish to adopt, centralised and decentralised.

Centralised

Here all subsidiaries, facilities or departments of an organisation report activity or fuel use data to the corporate level, where calculations are undertaken.

Decentralised

Here all subsidiaries, facilities or departments of an organisation collect activity or fuel data and calculate emissions and report this to the corporate level.

With both approaches the operational level staff will be responsible for the initial collection of data, with the difference occurring in where the emissions are calculated.

The centralised approach offers greater consistency of reporting and control over data management but can often be slower to enact and have less engagement at local levels. Whereas the decentralised approach offers flexibility and speed in decision making, encouraging local entities to identify emissions hot spots earlier and act upon them, but can lead to overall inconsistencies in data management at an organisational level, more complex coordination requirements and higher resourcing challenges.

Choosing an approach will be dependent on the volume of data you have available and the level of detail you organisation deems necessary to make decisions from.

## Reporting on emissions

Once you have determined how your data is shared internally, you may want to shift focus to how you report your emissions externally. Its not mandated that public GHG emissions reporting should follow any specific format, although its contents may be governed by regional legislations, as is the case is the case for European Union member states.

For member states, not-governed by regional legislation and looking for international best practise they may wish to follow the GHG Protocol Corporate Standard. If so, you must include:

Organisation information

- A description of the organisational boundaries chosen, including your chosen consolidation approach

- The operational boundaries chosen (and if including Scope 3, a list of the categories chosen)

- The reporting period for which the report covers

Emissions information

- The total Scope 1 and Scope 2 emissions and break down of each scope’s emissions separately

- Emissions data in tonnes of Co2 equivalent (CO2e)

- Your chosen baseline years data in a comparable format to the year you’re reporting on

- A brief summary of the methodologies used to calculate your emissions

- Any explicit exclusions (sites, operations or services) and explanations for doing so

In line with the same standard, you may wish to optionally include some additional information, including:

- Scope 3 emissions from activities defined as in section 2.4.1

- Further sub-division of data, by source types or activity types

- Communication of performance against any targets your organisation has set

- Any intensity ratios your organisation may use (emissions per kwh, or emissions by tonnes of material produced or waste)

- Any sustainability initiatives you as an organisation are employing to reduced GHG emissions production

- GHG emissions data from previous years before the stated reporting year, so multi-year trends can be seen

- An outline of any external assurance provided and associated verification statements

# Conclusion

Once these steps have been followed member states will have:

- Defined what parts of their organisation they are measuring

- Defined the emissions sources within their organisation that they are measuring

- Defined the baseline year for which they are measuring performance against

- Calculated their GHG emissions

- Reported their GHG emissions

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain