RECOMMENDATION Normative

R0112 Leading Lights

Edition 2.0

June 2026

urn:mrn:iala:pub:rnnnn:ed2.0

Revisions to this document are to be noted in the table prior to the issue of a revised document. The latest edition of the Recommendation is the only version in force unless the Recommendation is explicitly revoked by the Council.

| Date | Revision details | Approval |
| --- | --- | --- |
| June 1998 | Edition 1.0 |  |
| December 2005 | Edition 1.1 Entire document. Reformatted to reflect IALA documentation hierarchy |  |
| September 2020 | Edition 1.2 Editorial corrections. |  |
| June 2026 | Edition 2.0 Entire document to be aligned with the revised Guideline G1023 | Council 04 |

THE COUNCIL

RECALLING

the aim and objectives of the International Organization for Marine Aids to Navigation (the Organization) with respect to the improvement and harmonization of Marine Aids to Navigation (AtoN) worldwide for the benefit of the maritime community and the protection of the marine environment,

Article 8 of the Convention of the Organization regarding the responsibilities of the Council;

RECOGNIZING the need to provide a harmonized approach on leading lights during nighttime and during day time;

RECOGNIZING FURTHER that such a harmonized approach should include requirements on the nature and significance of each condition;

HAVING CONSIDERED the proposals made by the Engineering and Sustainability Committee;

APPROVES Recommendation R0112 on Leading Lights, set out in the Annex;

INVITES Members of the Organization and AtoN authorities and other appropriate stakeholders worldwide to implement the provisions of this Recommendation; and

REQUESTS the Engineering and Sustainability Committee or other Committees as the Council may direct, to keep this Recommendation under review and to propose amendments as necessary.

Annex  Requirements for the Performance of Leading LiNES

Member States, AtoN authorities and other appropriate stakeholders should use the following requirements for the performance of leading lines.

Leading lines guide ships along narrow fairways by providing a continuous series of straight lines (the useful segment). To ensure safe and effective navigation, authorities must ensure that no obstructions prevent the mariner from seeing the leading marks from any point within the useful segment, considering both lowest tide levels and observer eye heights.

Leading lights used at night must comply with the following photometric and geometric conditions:

Minimum Illuminance: At any point in the useful segment, the illuminance at the eye of the navigator produced by each of the lights must be at least 1 x 10^-6 lux without background lighting. This requirement increases to 2 x 10^-5 lux in areas with substantial background lighting.

Acquisition Region: In the acquisition region, the illuminance at the eye produced by at least one of the lights must be at least 0.2 x 10^-6 lux to ensure mariners can easily locate the line before turning.

Equality of Illuminance: The illuminance at the eye of the navigator produced by the front and rear lights should be as equal as possible. Under favorable circumstances, the ratio of illuminance between the two lights should be maintained between 0.5 and 2.0.

Prevention of Glare: To prevent glare, the maximum illuminance at the eye produced by either light must not exceed 0.1 lux. If the background is very dark, this maximum should be reduced to 0.01 lux.

Separation of Lights: The navigator must perceive a distinct vertical separation between the front and rear leading lights at all points along the useful segment.

The use of leading lines during the day may rely on daymarks, daytime lights, or a combination of both.

Daytime Lights: Daytime lights provide a superior signal compared to daymarks in poor conditions. When lights are used during the day, the required minimum illuminance at the eye must be increased significantly to 1 x 10^-3 lux to overcome background sky luminance.

Daymarks: When daymarks are utilized, they should be designed to maintain sharp contours at the longest required distances. Rectangular shapes are considered optimal. The size of the daymark must subtend a minimum horizontal angle of at least 1 minute of arc (1') in the observer's eye, though 3' is preferred for optimal shape recognition.

The leading line system must be sensitive enough for the mariner to easily detect cross-track deviation.

Cross-Track Factor: The sensitivity of the leading line should be evaluated using the Cross-Track Factor (CTF). Authorities should aim for a CTF between 10% and 30% (Excellent to Good) to ensure safe navigation, avoiding systems that yield a CTF over 75%.

Design Calculans: All detailed calculations regarding vertical difference angles, luminous intensity thresholds, standard deviations for bearing differences, and required heights must be conducted in accordance with the formulae provided in Guideline G1023.