Input paper:  ENG22-3.1.1.3

Input paper for the following Committee(s):		Purpose of paper:

（Select as appropriate）

☐ ARM	X  ENG		☐ PAP			X  Input

☐ DTEC	☐ VTS					☐ Information

Agenda item 			 n.n

Technical Domain / Task Number	 Task 2.1.4

Author(s) / Submitter(s)			 China Maritime Safety Administration (China MSA)

# Draft Guideline G1023 on Design of Leading Lines

# summary

Based on the discussion at the 21st session of the ENG Committee, China MSA continued the intersessional work with Estonian Transport Administration, revised the guideline and drafted the content to be moved to IALA Wiki for further review by the 22th session of the Committee.

## Purpose of the document

The attached document is the draft guideline relevant to the task of  updating the relevant IALA publications relating to leading lines, with the aim of pushing forward the task.

## Related documents

IALA. (2005) Guideline G1023 The Design of Leading Lines

# background

Updating the relevant IALA publications relating to leading lines is a task of IALA 2025-2027 work plan, which is led by China MSA. At the ENG21 meeting, the Working Group reviewed the draft guideline. During the intersessional period, China MSA, together with Estonian Transport Administration, continued to revise the guideline.

# discussion

## Revision of the draft guideline

### Non-editorial Changes

- As multiple different names for leading marks appear in the draft guideline, in order to unify the terminology used in the guideline, opinions are solicited from all ENG delegates regarding all names mentioned in the guideline as well as proposed name put forward by the delegates, with a view to determining the final terminology for leading marks.

Present options:

-Front Lead/Read Lead (as presented)

-Front Mark/Rear Mark

-Front Tower/Rear Tower

-Front Leading Mark/Rear Leading Mark.

Based on the survey results, ‘Front Leading Mark/Rear Leading Mark’ is confirmed as the standard term. Therefore all instances of the term in the guideline are replaced with “leading mark”.

- Given that this guideline focuses only on the design of leading line itself, not on channel design, the navigational factors related to leading line design in Chapter 3 “Background” are deleted and moved into IALA Wiki.

- Delete Chapter 4 ‘Background’ and move it to IALA Wiki because it describes how the IALA technical documents on leading line evolved.

- The description in Chapter 3 “Background” stating that it will often be necessary to observe at least one of the leading marks or lights within the ‘acquisition area’ will remain unchanged. The sentence “the mariner must be able to see both leading marks or lights within the ‘acquisition area’”should be deleted. In actual navigation, vessels in the ‘acquisition area’ may be unable to observe both the front and rear leading marks simultaneously due to topographic obstruction or insufficient visibility, etc. “simultaneous visibility” could result in an excessively large acquisition area for some waterways, thereby increasing design and construction costs.

- Because all the calculations in this guideline are based on the useful segment of the leading line, so change “Channel Segment” to “Useful Segment”, and “Channel Width W” to “Useful Channel Width” in Figure 3, Section 4.1.

- Replace “Channel” with “Useful Segment” in the guideline. The reason is as stated above.

- Revise the reference datum for the definition of the observer’s height in Figure 6, Section 4.2, from “sea level” to “MHW or MLW”, if not explicitly specified, it should default to MHW. Since leading marks are all designed based on the mean high water(MHW) level. However, under certain special circumstances, such as when calculating the minimum vertical angle difference between the front and rear leading marks, the observer’s height should be referenced to the mean low water (MLW) level to account for the worst-case scenario.

- In Section 4.4, replace the required minimum illuminance at the eye of the observer under the two conditions of minor and substantial background lighting from 10  and 100  with 2  and 20  respectively. This is to ensure that they are consistent with the minimum illuminance to use with different background luminance specified in Table 2 of IALA Guideline G1148 <Determination of Required Luminous Intensity for Marine Signal Lights>, as Table 2 was proven to be applicable to Leading Lights at the 21th session of ENG Committee Meeting.

- Revise the calculation formulae for the initial value of R in Section 5.1, changing Rinitial=a+C1000*W3.4∙a+C-1=L1000∙W3.4∙L-1 to Rinitial=M+C1000*W3.4∙M+C-1=L1000∙W3.4∙L-1in accordance wih the formula in EXCEL tool . In addition, supplement the background for obtaining the initial value of R, i.e., assume the CTF value is 20%, indicating a “very good” leading line, which comes form the original version of G1023.

- Revise the reason for not needing to calculate the geographical range limit of the rear light in Section 5.9.2.2 from “the blur calculation of front and rear light” to “the blur calculation of front and rear light and the geographical range calculation of front light”.

- Revise the title of Section 5.10.3.1 from “Using Retroreflectors in Place of Lights” to “Retroreflective Daymarks”. This is because the retroreflective material is panted on darmark, so it is essentially still a daymark, which can be substitute for light at night.

### Editorial Changes

- Move the “MTR” in Figure 5, Section 4.2 from left side to right side to align with that in Figure 8, Section 5.3, so as to avoid confusion.

- In Section 5.2.4, clearly specify the ratio refers to illuminance rather than intensity to be set to 1 in the list of factors which should be considered.

- Revise the threshold value of γmin which leads to blurring from 1.5∙10-6 rad to 1.5∙10-3 rad (≈5'), which is a common experienced value.

- In Section 5.7, delete the symbol % in the calculation formulae of CTF to make it normative, explain its unit in the explanation of the formulae.

- In Section 5.7, change the title of Table 1 from “CTF Classification” to “CTF Suitability Assessment”. Meanwhile, revise the explanation for CTF exceeding 75% from “unworkable” to “not useful to the mariner” to make the description more accurate.

- In Section 5.8, replace ‘dU=1 M=1852 m’ with ‘dU=1 M’ to avoid confusing the readers.

- Change the symbol index of the front daymark length  LFL,DM,sel to LFDM,sel and make the similar modification for the rear daymark as daymark belongs rather to leading mark not to light. Meanwhile, add the terms FDM and RDM in Section 4.5. In addition, revise LFL,DM,sel and LRL,DM,sel in Figure 6 of Section 4.2 to LFDM,sel and LRDM,sel.

- Revise the position of S in Figure 12 of Section 5.9.1.3 from “the distance between the obstruction and the front light” to “the distance between the obstruction and the near end of the useful segment” , so as to be consistent with the formulas below Figure 12 and the definition of S in the spreadsheet.

- Replace Hv with Hv,min in Formula (39) and Formula (40) of Section 5.9.2.1, Formula (43) and Formula (44) of Section 5.9.2.4 as these formulas are calculated for the lowest height of the observer which ensures the calculation is based on the worst situation so the result will be appropriate to all the situations.

- Revise Formula (42) in Section 5.9.2.3 by changing the denominator from  M+C to C. This is because at the near end of the useful segment, x has changed from M+C to C. In addition, the explanation preceding this formula has also been corrected to state that x=M instead of x=C at the near end.

- Replace Hv with Hv,min in Formula (43) and Formula (44) of Section 5.9.2.4  maintain consistency with the Excel design tool, i.e., to account for the worst-case scenario.

- Revise Formula (46) in Section 5.9.2.5 from ‘HRL,min8=12∙LRL,DM,sel+Hv-MTR+M+R∙HFL,sel+MTR-HvM+C+α∙M+R∙R’ to ‘HRL,min8=12∙LRDM,sel+Hv,min-MTR+M+RS∙Hobs-Hv,min+MTR+α∙M+R∙M+R-S’ as this formula should be evolved from Formula (44) in Section 5.9.2.4.

For details, please refer to the attached Draft Guideline G1023 on Design of Leading Lines.

## Draft of the content to be removed to IALA Wiki

- The history of leading line design.

- Sensitivity of a leading line.

- Simplified intensity calculation.

- Initial channel layout.

For details, please refer to the attached Additional and useful information about the design of leading line (to be removed to IALA Wiki in future).

# reference

- IALA. (2005) Recommendation R0112 Leading Lights

- IALA. (2005) Guideline G1023 The Design of Leading Lines

- IALA. (2001) Design of Leading Lines Program (Excel Workbook)

- IALA. (2021) Guideline G1148 Determination of Required Luminous Intensity for Marine Signal Lights

# Action required of the committee

The Committee is invited to consider the attached draft Guideline on Design of Leading Line.