IALA Guideline

G1023

Design of Leading Lines

Edition 2.0

Date (of approval by Council)

urn:mrn:iala:pub:g1023:ed2.0

Revisions to this document are to be noted in the table prior to the issue of a revised document.

| Date | Details | Approval |
| --- | --- | --- |
| December 2001 | 1st Edition | December 2001 |
| December 2005 | 1.1: Entire document reformatted to reflect IALA Documentation Hierarchy | December 2005 |
| 2026 | Edition 2.0 More specific information on the design principle, the geometry and the photometry are included. The hidden equations in the spreadsheet are extracted to be visible as mathematical equations, so as to preserve the knowledge and information of leading line design. |  |

1. Introduction	6

2. Purpose	6

3. Background	7

4. Definitions and Parameters	7

4.1. Geometry, Plan View	7

4.2. Geometry, Side View	8

4.3. General overview of design process	9

4.4. Luminous intensity calculations	10

4.5. OTHER TERMS	11

5. Design equations	12

5.1. Position of leading marks	12

5.2. Luminous intensity	13

5.2.1. Minimum Luminous Intensity	13

5.2.2. Maximum Luminous Intensity	13

5.2.3. Design Luminous Intensity of Front Light	14

5.2.4. Design Luminous Intensity of Rear Light	14

5.3. Vertical Difference Angle	15

5.4. Minimum Vertical Difference Angle	16

5.5. Horizontal Difference Angle	17

5.6. Sensitivity of a Leading Line	18

5.7. Cross-Track Factor	19

5.8. Geographical Range	19

5.9. Heights	20

5.9.1. Front Light	20

5.9.2. Rear Light	22

5.10. Daymarks	24

5.10.1. Shape of Daymarks	24

5.10.2. Colour of Daymarks	24

5.10.3. Dimensions of Daymark	25

6. additional considerations	28

6.1. Lights	28

6.1.1. Fan versus Pencil Beam Lights	28

6.1.2. Beam Width of Pencil Beam Lights	28

6.1.3. Additional Lights	29

6.2. Daymarks versus Daytime Lights	29

6.3. Colour and Flashes	30

6.4. Placement of leading marks	30

6.5. Lantern Placement	30

7. Abbreviations and Technical Quantities	32

7.1. LATIN	32

7.2. Greek	35

8. references	35

List of Tables

Table 1 Cross_Track Factor Suitability Assessment	19

List of Figures

Figure 1 Geometry of a leading line and its appearance for observer, when off centreline	5

Figure 2 Geometry of a leading line and its appearance for observer, when at centreline	5

Figure 3 Channel parameters (plan view)	7

Figure 4 Observer's parameters (plan view)	7

Figure 5 Vertical plane (side view)	7

Figure 6 Heights of lights and daymarks	8

Figure 7 Iterative Leading Line Design Cycle	9

Figure 8 Vertical difference angle (γ)	15

Figure 9 'Apparent magnitude of front and rear light'	16

Figure 10 Horizontal difference angle (θ)	17

Figure 11 Bearing and elevation difference	17

Figure 12 Minimum height of front light considering an obstruction	20

Figure 13 A typical rectangular daymark of leading line	23

Figure 14 Examples of non-rectangular dayboards with the front and the rear daymark are mirrored	23

Figure 15 Comparison of calculated daymark lengths	25

Figure 16 The final version of figure yet to be drawn	26

Figure 17 Dayboards using semi-transparent material	27

Figure 18 Beam Width of Leading Lights	28

Figure 19 Placement of leading marks	29

Introduction

A leading line is a straight line produced by the alignment of two fixed aids to navigation used to mark straight sections of fairways or channels. The aids to navigation producing a leading line can be daymarks or lights or consist of both and are called leading marks. For the remainder of this document the two navigational marks are referred to as the “front leading mark” and the “rear leading mark”. The part of the line intended for navigational use is called the “useful segment” of the leading line. The end closer to the front lead is called the “near end” and the end further from the front lead is called “far end” of the useful segment.

The design of a leading line has to consider:

Geometric parameters and the layout of the leading line;

The luminous intensity of the lights;

Dimensions of the daymarks.

The performance of a leading line can only be guaranteed when all parameters work together. Therefore, extensive and iterative calculations are usually necessary. Figures 1 and 2 provide a general overview of the geometry of leading lines.

Geometry of a leading line and its appearance for observer, when off centreline

Geometry of a leading line and its appearance for observer, when at centreline

Purpose

The purpose of this document is to provide guidance on the principles of leading lines and the calculation used in their design. Guidance on fairway design is provided in IALA Guideline G1078 The Use of AtoN in the Design of Fairways [10].

Background

Leading lines allow ships to navigate with precision along straight sections of a route providing visibility is sufficient to see the leading marks/lights.

It is common practice to guide ships along narrow fairways by means of leading lines or, more generally, a series of leading lines in succession, the useful segments of which form a continuous series of straight lines. The availability of suitable sites for the leading marks may influence the selection of ship courses in natural, narrow fairways or in the layout of artificial channels. The use of existing buildings or structures may be possible as locations for installation leading marks.

To ensure safe and effective navigation toward the useful segment of a leading line,  it will often be necessary to observe at least one of the leading marks or lights within the ‘acquisition area’. the mariner must be able to see both leading marks or lights while still approaching from the seaward side. This area is referred to as the “acquisition region”. The size of the acquisition region must be considered in the calculation of the main line.

This guideline is only related to the inaccuracy of the aid to navigation proper, leaving out the different 'nautical margins' that should be taken into account but which depend on local conditions and can only be determined after considering each particular case.

Definitions and Parameters

Variables required for the design of a leading line are described in this section. The channel is idealised to a horizontal rectangle, which is symmetrical about the centreline (see Figure 3).

The front leading mark is abbreviated FL and the rear leading mark is abbreviated RL.

SI Standard units are used:

Lengths and heights in metres [m]

Illuminance in lux [lx]

Luminous intensity in candelas [cd])

Angles are in radians, unless otherwise stated. In many cases the angles are small and therefore some approximations are valid (αradians≈sinα≈tanα and cosα≈1).

IHO Chart Specification [5] unit used are:

Nautical mile (M) [1 M=1852 m]

All dimensions are named explicitly; for example, instead of writing ex the term exdU is used. This will work for x both in metres and in nautical miles: e1000 m1 M=e1000 m1852 m= e0,54 M1 M=e0,54.

This is done for distance with dU=1 M=1852 m, height hU=1 m and illuminance EU=1 lx.

For all equations, SI units are used unless stated otherwise.

Geometry, Plan View

Channel parameters (plan view)

C	Length of useful segment

L	Distance to front structure from far end of useful segment (L=C+M)

M	Distance to front structure from near end of useful segment

R	Distance between leading line structures

W	Useful channel width

Observer's parameters (plan view)

x	Distance of observer (vessel) from front tower (parallel to leading line)

y	Off-axis distance of observer (perpendicular to leading line)

θ	Bearing difference (horizontal angular difference between front and rear light)

Geometry, Side View

The height of the observer on the vessel is referenced to both mean high water (MHW) and mean low water (MLW) sea level (at the vessel’s location along the useful segment. All other heights are referenced to MHW. Mean sea level (MSL) can be used in place of MHW or MLW when the mean tidal range (MTR) is small or zero (MTR<0.3 m).

Vertical plane (side view)

Heights of lights and daymarks

Hv	Height of observer (on vessel) above MHW or MLW,  if not explicitly specified, it should default to MHW.

HFL	Height of front light above MHW or MSL

HRL	Height of rear light above MHW or MSL

Hshaw	Safe height above water

LFDM	Vertical length of the front daymark

LRDM	Vertical length of the rear daymark

MHW	Mean high water

MLW	Mean low water

MSL	Mean sea level

MTR	Mean tidal range

ZFL	Height of front mark obscured behind earth’s curvature

ZRL	Height of rear mark obscured behind earth’s curvature.

γ	Vertical difference angle between front and rear light

General overview of design process

Throughout the leading line design process, the designer will calculate minimum and maximum values for parameters such as heights and luminous intensity which set the range of viable solutions. These minimum and maximum calculations will then lead to recommended value calculations.  Once a recommended value is obtained, then a design value is selected. The design or selected value may be the recommended value or another value between the minimum and maximum values. The calculations provide a specific value that may not be practical, prudent or cost effective in constructing the leading line. For example, the recommended height of the front leading mark is 4.2 m. Instead, the designer selects a front leading mark height of 5 m which is a standard tower design for their nation. As design value are chosen, the calculations are completed with the design values to ensure the leading line system is effective.

Iterative Leading Line Design Cycle

Luminous intensity calculations

Detailed information on the luminous intensity calculations needed for the design of leading lights can be found in IALA Guideline G1148 Determination of Required Luminous Intensity for Marine Signal Lights [3]. Based on this guideline the following parameters must be determined in the calculation of leading lights.

EFL	Illuminance at the eye of the observer produced by the front light

ERL	Illuminance at the eye of the observer produced by the rear light

Er	Required minimum illuminance at the eye of the observer

- The required minimum illuminance at the eye of the observer depends on background lighting of the leading lights.  Er=10−6 lx is used for situations without background lighting, Er=2∙10−65 lx for minor and Er=2∙10−54 lx for substantial background lighting. For a daytime light the required illuminance is Er=10−3 lx.

Emax	Allowed maximum illuminance at the eye of the observer

The allowed maximum illuminance at the eye of the observer is needed to avoid glare.  Emax=0.01 lx is used for situations without background lighting, Emax=0.1 lx with background lighting. It is very unlikely to cause glare with daytime lights, so there is no need to check for and no maximum value for the illuminance is defined for daytime lights.

IFL	Luminous intensity of the front light

IRL	Luminous intensity of the rear light

Vmin	Minimum Meteorological Visibility

- The minimum meteorological visibility should be selected by the designer based on the worst visibility conditions under which the leading lights must be usable, navigational requirements, practical limitations, etc. For guidance in selecting the optimal value for minimum meteorological visibility please refer to Guideline G1148.

Vdsg	Design Meteorological Visibility

Design visibility is used to establish the recommended ratio of luminous intensitiesy of the front and the rear light. As a practical consideration, use of a fixed value Vdsg=10 M=18 520 m is recommended.

Vmax	Maximum Meteorological Visibility

The potential for glare produced by the lights is calculated with the maximum meteorological visibility. Typically, Vmax=20 M=37 040 m is chosen.

OTHER TERMS

Some symbols are defined in individual chapters and their meaning is explained in the context. For these the following predefinitions are used.

E	illuminance

H	height

I	luminous intensity

L	(vertical) length of a daymark, distance from the front mark to the far end

r	ratio (ratio of intensities)

Rg	geographical range

S	distance between an obstruction and front light (parallel to centreline)

V	visibility

T	transimissivity

Windex	extension or width, when used with index, usually for the daymark

x	distance (along to centreline)

y	distance (perpendicular to centreline), off-axis distance

Z	dip of horizon

α	factor for dip calculation

ϕ	angle used to describe horizontal divergence

The following indices are used.

DM	daymark

d	detection

dsg	design

far	far, referencing the far end of the channel

FL	front light

FDM	front daymark

initial	initial iteration value

max	maximum

mid	middle, referencing the middle of the channel

min	minimum

min1, min2…	minimum according to the requirement referenced

near	near, referencing the near end of the channel

obs	obstruction

rec	recommended

RL	rear light

RDM	rear daymark

sel	selected

shaw	safe height above water

v	vessel, referencing the observer at the vessel

Design equations

Position of leading marks

As a starting point, input values for the rectangular dimensions of the useful segment (C, W) must be defined.

When the locations of the leading marks (front and rear) are not prescribed by geographical or other restrictions the following formulas can be used for initial determination of their positions in relation to the channel to be marked.

- Distance from the near end to the front tower:

Minitial=C5=0.2∙C	(1)

- Distance between the towers:

Rinitial=Ma+C1000∗W3.4∙Ma+C−1=L1000∙W3.4∙L−1	(2)

Explanation for Rinitial:

For the calculation of the initial value of Rinitial it is assumed that the observer is able to detect that they are distance YD=0.1∙W apart from the leading line at the far end of the useful segment, which means the cross track factor (CTF) is 20%, inferring a ‘very good’ leading line (see Table 1). Furthermore, it is assumed that the elevation difference at this position is γ=1.5∙10−3 rad (≈5') and the bearing difference becomes θ=0.34∙10−3 rad (≈1').

Luminous intensity

The intensity needs the input of a distance x, a visibility V and a threshold for the illuminance E at the eye of the observer and is calculated by Allard’s Law as follows:

I=E∙x2∙0.05−xV	(3)

When an intensity is known, the illuminance at the eye of the observer is:

E=I∙x−2∙0.05xV	(4)

Minimum Luminous Intensity

The required minimum intensity of the front and rear lights is calculated for the minimum meteorological visibility (worst atmospheric condition), required illuminance at the eye of the observer and for the maximum distance from each tower to the far end of the useful segment. The minimum luminous intensity should be calculated for night and for day, if the lights are also to be used during daylight. In equations 5 and 6, Vmin and Er are derived from G1148:

- Minimum intensity of the front light:

IFL,min=Er∙M+C2∗0.05−M+CVmin	(5)

- Minimum intensity of the rear light:

IRL,min=Er∙R + M+C2∗0.05−R+M+CVmin	(6)

Where：

Er	required minimum illuminance at the eye of the observer;

Vmin	minimum meteorological  visibility;

M+C	distance from far end of the useful segment to the  front leading mark;

R+M+C	distance from far end of the useful segment to the rear leading mark.

Maximum Luminous Intensity

To avoid glare the luminous intensity should not exceed a certain maximum value. The maximum allowed intensity is calculated for both front and rear lights at the near end of the useful segment. In equations 7 and 8, Vmax and Emax are derived from G1148:

- Maximum intensity of the front light:

IFL,max=Emax∙M2∙0.05−MVmax	(7)

- Maximum intensity of the rear light:

- IRL,max=Emax∙R+M2∙0.05−R+MVmax	(8)

Where：

Emax	allowed maximum illuminance at the eye of the observer;

Vmax	maximum  meteorological visibility;

M	distance from near end of the useful segment to front light;

R+M	distance from near end of the useful segment to rear light.

Design Luminous Intensity of Front Light

The designer of the leading line may choose any intensity for the front light (IFL, dsg), as long as it is between the minimum and maximum value. The design luminous intensity should consider the presence of background lighting. G1148 provides alternative values of E to use in certain background lighting conditions. Most commonly the minimum luminous intensity is selected.

Design Luminous Intensity of Rear Light

The illuminances at the eye of the navigator provided by the respective lights within the useful segment should be as equal as possible. An intensity ratio is used to determine the design intensity of the rear light based on the design intensity of the front light. Allard’s law and the ideal ratio are used to determine the design luminous intensity of the rear light.

The illuminance values for the lights at the design visibility Vdsg :

EFL=IFL∙x−2∙0.05xVdsg	(9)

ERL=IRL∙R+x−2∗0.05R+xVdsg	(10)

The ratio of illuminances is:

EFLERL=IFLIRL∙R+x2x2∙0.05xVdsg0.05R+xVdsg=IFLIRL∙R+x2x2∙0.05−RVdsg

The ideal ratio for the illuminance should be equal to one, i.e.,  EFLERL=1, so the ratio of the intensity r:

- r=IRLIFL=R+x2x2∙0.05xVdsg0.05R+xVdsg=R+x2x2∙0.05−RVdsg

However, the ratio depends on the observer's distance from the (front) light x. Since the ideal ratio can only be achieved at one location along the useful segment, it is recommended that the ratio IRLIFL be  calculated at least twice, first for a position in the middle of the channel and secondly for a position at the far end.

These ratios are evaluated to determine a recommended ratio from which the rear light design luminous intensity is calculated. It is generally preferred to set the ratio to 1 at the far end to provide a strong signal to the mariner in the acquisition range. The following factors should also be considered in setting the ratio of illuminance and the corresponding selection of the rear light design luminous intensity:

- For longer leading lines, it may be necessary to set the ratio of illuminance to 1 at, or closer to, the middle of the useful segment to avoid glare or blurring of the lights at the near end.

- The predominant ship types and height of the various observers should be considered in selecting the location along the useful segment to set the ratio of illuminance  to 1.

- As per IALA Recommendation R0112 on Leading Lights [1], under favourable circumstances, the ratio of illuminance may be permitted to vary between 0.5 and 2.0. This ratio may sometimes deviate substantially from 1 ranging from 0.1 to 10 or even from 0.01 to 100 in special cases, for instance when an existing light is used as a part of new leading lights or when the distance between the lights is large when compared to the maximum distance to the front light. This necessitates increasing the minimum values of the elevation difference and affects the sensitivity slightly, but the lights remain usable.

In the leading lines spreadsheet, the ratio of illuminance is calculated for every 10% increment of the useful segment length.

Middle of the useful segment:

The distance from the middle of the useful segment to the front light (FL) is:	XFL,mid=M+C2

- the rear light (RL) is:	XRL,mid=R+M+C2

With these equations the ratio r becomes:

rmid=IRLIFL=XRL,mid2XFL,mid2∙0.05XFL,midVdsg0.05XRL,midVdsg=R+M+C22M+C22∙0.05−RVdsg	(11)

Far end of the channel:

The distance from the far end of the useful segment to the front light (FL) is:	XFL,far=M+C

- the rear light (RL) is: 	XRL,far=R+M+C

With these the ratio of intensities becomes:

rfar=IFLIRL=XFL,far2XRL,far2∙0.05XRL,farVdsg0.05XFL,farVdsg=M+C2R+M+C2∙0.05RVdsg	(12)

The ratio between the luminous intensities of front and rear light is then selected as follows:

r=IRLIFL=rmid    for rmid < 2rfar2rfar     for rmid ≥ 2rfar	(13)

- And the design luminous intensity of the rear light, IRL, dsg :

IRL,dsg=r∙IFL,dsg	(14)

During daylight the luminous intensity of a light needs to be magnitudes higher than at nighttime. This may result in complex lighting and power requirements. Therefore, design luminous intensity is not calculated for daytime lights. Instead, it is assumed that the design luminous intensity will be equal to or above the minimum luminous intensity.

Vertical Difference Angle

It is essential that the navigator perceives a vertical separation between the front and rear leading lights at all points along the useful segment of the leading line. The vertical angle refers to the angle between a light and the observer’s horizontal plane. The vertical difference angle γ is defined as the difference between the vertical angles of the front and rear lights as seen from the observer’s position. This angle plays a critical role in the correct design of a leading line, as it affects the mariner’s ability to distinguish the two lights. Vertical difference angle is calculated at Mean Low Water (MLW), since this condition typically results in the smallest vertical separation, which can challenge visibility and recognition of the leading line.

Equations 10 to 11 below are derived with reference to Figure 8, which illustrates the geometric relationships involved.

Vertical difference angle (γ)

The vertical angles are:

γFL≈tanγFL=HFL−ZFL+MTR−Hvx	(15)

γRL≈tanγRL=HRL−ZRL+MTR−Hvx+R	(16)

At longer distances, the apparent height of a light above the horizon is reduced by the part of the mark that is obscured Z by the earth’s curvature. The value of Z depends on the distance (x) between the observer and the light.

ZFL≈α∙x2	(17)

ZRL≈α∙x+R2	(18)

where α=6.75∙10−8m−1.

The vertical difference angle γ (vertical angle between front and rear light as seen from the observer) is then:

- γ=γRL−γFL=HRL−ZRL−Hv+MTRx+R−HFL−ZFL−Hv+MTRx →

- γ=HRL−α∙x+R2−Hv+MTRx+R−HFL−α∙x2−Hv+MTRx →

]’			γ=HRL−Hv+MTRx+R−HFL−Hv+MTRx−α∙R	(19)

where α=6.75∙10−8m−1.

The vertical difference angle will vary in practical use based on the current sea level, the observer height and the distance.

Minimum Vertical Difference Angle

For navigating along a leading line, i.e. being able to notice deviation from the line, leading line (lights and daymarks) must appear vertically separated to the mariner, i.e. the rear light clearly above the front mark. There is a minimum value γmin for the vertical difference angle, at which the lights are very close but can still be clearly seen as two separated lights.

The minimum required vertical difference angle (γmin) between the lights depends on the illuminance at the eye of the observer generated by the lights. In general, all maritime lights are seen under a viewing angle below the eye resolution (approx. 1'=0.291∙10−3rad). However, the lights appear larger or smaller for the observer's eye according to their brightness. (This effect is known from astronomy as the 'apparent magnitude (of a star)'.)

'Apparent magnitude of front and rear light'

To account for that effect and calculate the minimum required vertical difference angle γmin , the luminous intensity of front and rear light must be selected.

selected luminous intensity FL:	IFL,sel

selected luminous intensity RL:	IRL,sel

The illuminance at the eye of the observer is then calculated for very good viewing conditions (Vmax=20 M, Tmax=0.861) and for several equally spaced positions along the channel.

illuminance from FL:

EFL=IFL,sel∙x−2∙0.05xVmax	(20)

illuminance from RL:

ERL=IRL,sel∙x+R−2∙0.05x+RVmax	(21)

The minimum difference angle is then:

γmin=2.4−0.06∙logERLEFL+0.26∙logERLEFL2+

logE+EU∙0.2−0.02∙logERLEFL−0.02∙logERLEFL2∙10−3rad	(22)

Where：

E+=maxEFL,ERL                  for nighttimemaxEFL,ERL10000     for daytime;

EU=1 lx.

Generally, a γmin less than 1.5∙10−3 rad (≈5') leads to blurring and should be avoided.

Horizontal Difference Angle

The horizontal angle describes the horizontal angle between the direction to a light from the observer and a line parallel to the leading line. The horizontal difference angle θ is the horizontal angle between front and rear light seen from the observer. The horizontal difference angle is calculated as the difference between the horizontal angles of the front and rear light.  It depends on the lateral distance of the vessel from the leading line.

The equations necessary for the calculations are derived from Figure 10.

Horizontal difference angle (θ)

The horizontal angles are:

θFL≈tanθFL=yx

θRL≈tanθRL=yx+R

The horizontal difference angle θ (horizontal angle between front and rear light as seen from the observer) is then:

θ=θFL−θRL=yx−yx+R=y∙Rx∙(x+R)	(23)

The off-axis distance is calculated as follows:

y≈θ∙x∗R+xR=θ∙x∙1+xR	(24)

Sensitivity of a Leading Line

Sensitivity of a leading line is the magnitude of deviation from the leading line needed for an observer to detect with certainty that the lights are not vertically aligned. Off-axis distance shows absolute sensitivity, cross-track factor shows relative sensitivity compared to width of the channel. Smaller detection deviation has a higher sensitivity and larger detection deviation has a lower sensitivity.

Sensitivity of a leading line depends mostly on the distance between the leading marks (longer distance yields a higher sensitivity). To a smaller extent, the elevation difference of the lights/daymarks impacts the leading line sensitivity (the closer the lights/daymarks appear to each other vertically the higher the sensitivity).

Bearing and elevation difference

Bearing difference θdfor detection of not being on the leading line based on elevation difference γ (vertical difference angle) is calculated using the formulae:

θd=maxθ'1,θ'2	(25)

Where:

θ'1=0.16∙10−3+0.12∗γ                           for γ≤5∙10−3 rad 0.31∙10−3+0.09∗γ     for  5∙10−3<γ≤20∙10−3 rad;

θ'2=0.224∙γmin	(see Section 5.4).

- Note: Prior to calculating θd,  γ must calculated at MHW using the equations below:

- γ=γRL−γFL=HRL−ZRL−Hvx+R−HFL−ZFL−Hvx →

- γ=HRL−α∙x+R2−Hvx+R−HFL−α∙x2−Hvx →

γ=HRL−Hvx+R−HFL−Hvx−α∙R	(26)

where α=6.75∙10−8m−1.

Cross-Track Factor

Cross-track factor CTF is a measure of sensitivity of a leading line. The cross-track factor is calculated as the ratio of the off-axis distance the navigator can detect with certainty that the vessel is not on the leading line, yd, divided by the half-width of the channel, expressed as a percentage.

From equations (24) yd is:

yd≈θd∙x∙1+xR	(27)

The CTF then becomes:

CTF=yd0.5∗W	(28)

where CTF is the cross-track factor in percent.

For a leading line design, the cross-track factor varies with the observers position (x) in the useful segment and the observer's height Hv. Cross-track factor suitability assessments are provided in Table 1.

Cross_Track Factor Suitability Assessment

| CTF | Suitability | Interpretation |
| --- | --- | --- |
| over 75% | Not Acceptable | Range must be improved, or it will be not useful to the mariner. |
| 50% - 75% | Poor | Decrease the cross-track factor if physically possible. |
| 30% - 50% | Fair | Decrease the cross-track factor only if moderate cost involved. |
| 20% - 30% | Good | Decrease the cross-track factor only if little cost involved. |
| 15% - 20% | Very Good | Do not expend more funds to decrease the cross-track factor. |
| 10% - 15% | Excellent | The cross-track factor should not be less than 10% at the far end of the channel. |

Geographical Range

The geographical range Rg of a light or a daymark is calculated by the following equation.

Rg=2.03 dU∙HhU+HvhU	(29)

Where：

H	height of the light or lower end of the daymark;

Hv	height of the observer on vessel;

hU=1 m	unit for height;

dU=1 M=1852 m	unit for distance (1 nautical mile).

Remark: Written with hidden dimensions, the equation is: Rg=2.03∙H+Hv.

When a required geographical range is given, then the minimum height of a light or a daymark achieving that range can be derived from:

H=Rg2.03∙dU−HvhU2∙hU	(30)

Remark: Written with hidden dimensions, the equation is: H=Rg2.03−Hv2

Heights

There are several requirements for the height of the front and rear light. Each requirement leads to a different minimum required height in the form of Hlight≥Hmin. Each minimum height resulting from individual requirements is given an index to identify it according to its requirement. The height of the lights are significantly controlled by the minimum observer height Hv=Hv,min for which the leading line is designed. It is assumed that for observer heights lower than Hv,min that the leading line will not work for parts or all the useful segment.

Front Light

Safe Height above Water

- To avoid damage from waves or vandals or obstruction by vegetation, the front light HFL has to be above a safe height above water (Hshaw):

HFL≥HFL,min1=Hshaw	(31)

- When a daymark with length LFDM,sel is used, the lower end of the daymark shall be above Hshaw and the light above the upper end of the daymark: HFL,lower_end_daymark≥Hshaw.

- In this case the minimum front light height is:

HFL≥HFL,min2=Hshaw+LFDM,sel	(32)

Geographical Range

The height of the front light above mean high water must be sufficient to be seen above the horizon i.e. its geographical range must exceed the distance from the front mark to the far end of the channel Rg=M+C.

HFL≥HFL,min3=Rg2.03 dU−HvhU2∙hU	(33)

- When a daymark with length LFDM,sel is used, the lower end of the daymark must be high enough above mean high water so that its geographical range exceeds the distance to the far end of the channel.

- HFL,lower_end_daymark=Rg2.03 dU−HvhU2∙hU
When the light is positioned at the upper end of the daymark, the daymark length must be added to the minimum front light height.

HFL≥HFL,min4=Rg2.03 dU−HvhU2+LFDM,sel∙hU	(34)

Avoiding Occlusion by Obstruction

When an obstruction is between the front light and the observer and it seems to be relevant for the visibility of the front light, a calculation is necessary to estimate whether the front light is occluded. The height Hobs and the distance S between the obstruction and the near end of the useful segment are needed for this calculation. HFL,min is only calculated for the observer at mean low water which is the worst case situation for obstruction.

Minimum height of front light considering an obstruction

Considering presence of an obstruction from Figure 12, the minimum height of the front light HFL,min can be derived.

Hobs−Zobs−Hv−MTRx−M+S=HFL,min−ZFL−Hv−MTRx →

HFL,min=xx−M+S∙Hobs−Zobs−Hv+MTR+ZFL+Hv−MTR

The dip of horizon for obstruction and front light is:

Zobs=α∙x−M+S2

ZFL=α∙x2

- where α=6.75∙10−8m−1.

The minimum height for the front light is then:

HFL,min=xx−M+S∙Hobs−Hv+MTR+Hv−MTR−α∙x∙x−M+S+α∙x2 →

HFL,min=Hv−MTR+xx−M+S∙Hobs−Hv+MTR+α∙x∙M−S

The minimum height for the front light is calculated for x=M+C (far end) and x=M (near end) for the lowest height of eye Hv,min :

Far end:

HFL,min5=Hv,min−MTR+M+CC+S∙Hobs−Hv,min+MTR+α∙M+C∙M−S	(35)

Near end:

HFL,min6=Hv,min−MTR+MS∙Hobs−Hv,min+MTR+α∙M∙M−S	(36)

When the front light has a daymark with length LFDM,sel, it is assumed that the lower end of the daymark must not be obstructed and that the light will be at the upper end of the daymark. In this case the selected length of the daymark LFDM,sel is added to the minimum front light height.

Far end:

HFL,min7=LFDM,sel+Hv,min−MTR+M+CC+S∙Hobs−Hv,min+MTR+α∙M+C∙M−S	(37)

Near end:

HFL,min8=LFDM,sel+Hv,min−MTR+MS∙Hobs−Hv,min+MTR+α∙M∙M−S	(38)

Selected Front Light Height

The recommended minimum height HFL,rec is the maximum value of the minimum heights calculated above. With this value, all requirements stated before in this chapter are fulfilled.

However, a different value for the height may be chosen, which is called HFL,sel.

Rear Light

The calculation of the rear light height requires that the front light height has already been selected (HFL,sel).

Blur

The rear light must be high enough so that the vertical difference angle in whole useful segment is larger than γmin (Section 5.4), to avoid blurring of the lights.

γmin=HRL−Hv+MTRx+R−HFL,sel−Hv+MTRx−α∙R →

HRL,min=x+R∙γmin+HFL,sel−Hv+MTRx+α∙R+Hv−MTR

The minimum height of the rear light considering blur is calculated for x=M+C (far end) and x=M (near end) for the lowest height of eye Hv,min:

Far end:

HRL,min1=M+C+R∙γmin+HFL,sel−Hv,min+MTRM+C+α∙R+Hv,min−MTR	(39)

Near end:

HRL,min2=M+R∙γmin+HFL,sel−Hv,min+MTRM+α∙R+Hv,min−MTR	(40)

Geographical Range

From georaphical range calculation of the front light, (incorporating horizon dip due to the curvature of the Earth) and the blur calculation (ensuring the rear light is seen above the front light),  it is guaranteed that the rear light will always appear above the horizon as well. So there is no need to check the rear light for geographical range. However, for the list of lights and the charts the geographical range of the rear light may be calculated separately.

Avoiding obscuring of the Rear Daymark by Front Light

At the far end of the channel (x=M+C) the entire daymark area of the rear leading mark must appear above the front light.

Far end:

HRL,min3=LRDM,sel+Hv−MTR+M+C+R∙HFL,sel+MTR−HvM+C+α∙M+C+R∙R	(41)

At the near end of the channel (x=CM) it is not necessary that the entire daymark is visible, because it appears much larger to the observer. An accepted compromise is that one half of the rear light daymark is visible above the front light.

Near end:

HRL,min4=12∙LRDM,sel+Hv−MTR+M+R∙HFL,sel+MTR−HvM+C+α∙M+R∙R	(42)

Avoiding Occlusion by Obstruction, without Daymark

The equations for the front light (Section 5.9.1.3) are valid for the rear light with the following transitions:

HFL,min		→	HRL,min

M		→	M+R

This leads to:

HRL,min=Hv−MTR+xx−M−R+S∙Hobs−Hv+MTR+α∙x∙M+R−S

Far end:

HRL,min5=Hv,min−MTR+M+R+CC+S∙Hobs−Hv,min+MTR+α∙M+R+C∙M+R−S	(43)

Near end:

HRL,min6=Hv,min−MTR+M+RS∙Hobs−Hv,min+MTR+α∙M+R∙M+R−S	(44)

Avoiding Occlusion by Obstruction, with Daymark

At the far end of the channel (x=M+CC+M+R) the entire daymark area of the rear leading mark must appear above the obstruction. In this case the full height of the daymark is added to the minimum rear light height.

Far end:

HRL,min7=LRDM,sel+Hv,min−MTR+M+R+CC+S∙Hobs−Hv,min+MTR+α∙M+R+C∙M+R−S	(45)

At the near end of the channel (x=M) it is not necessary that the entire daymark is visible, because it appears much larger to the observer. An accepted compromise is that one half of the rear light daymark is not obscured by the obstruction.

Near end:

HRL,min8=12∙LRDM,sel+Hv,min−MTR+M+RS∙Hobs−Hv,min+MTR+α∙M+R∙M+R−S	(46)

HRL,min8=12∙LRDM,sel+Hv,min−MTR+M+R∙HFL,sel+MTR−HvM+C+α∙M+R∙R

Selected Rear Light Height

The recommended minimum height HRL,rec is the maximum value of the heights calculated before. With this value, all requirements stated before in this chapter are fulfilled.

However, a different value for the height may be chosen, which is called HRL,sel.

Daymarks

Shape of Daymarks

Shape of daymarks of leading marks is not restricted by MBS and there are different shapes used worldwide. However, rectangular is considered the optimal shape for daymarks as this maintains sharp contours of silhouette at longest distance.

A typical rectangular daymark of leading line

Trapezoidal and triangular daymarks can be used for leading lines with shorter ranges. At longer ranges daymarks of these shapes are less recognizable than rectangular daymarks of the same size. However, at the background of city buildings, trapezoidal and triangular daymark can be more distinguishable than rectangular daymarks. Square daymarks should be avoided, as their shape becomes unrecognizable at the shortest distance.

Examples of non-rectangular dayboards with the front and the rear daymark are mirrored

As a rule, front and rear daymarks have the same shape. In exceptional cases daymarks with different shapes can be used.

Colour of Daymarks

Most typical colour scheme of daymarks of leading line is three vertical stripes, the middle one being in contrasting colour with the outer ones. Width of the central stripe is usually 1/4 to 1/3 of the width of the daymark.

Colour of daymarks is important to make them visible against their background. As a rule, colour contrast plays a role at distances up to 2-4 km from the structure. At longer distances, due to haze, the colour of a structure is less discernible to the observer and the main factor becomes the luminous contrast. So, for light background daymarks should be in a dark colour and for dark background the daymark should be in a light colour. In case of three stripes, the colours recommended in the table should be the outer ones and the central one should be in a contrasting colour with them. Table 1 gives recommended colours for daymarks observed against some typical backgrounds.

Table 1	Recommended colour of daymarks depending on the colour of the background

| Background of a daymark | Recommended colour for the daymark |
| --- | --- |
| Green vegetation | White, yellow |
| Sky | Red, black |
| Sea | White, yellow |
| Yellow sand | Black, white |
| Dark rock | White, yellow |
| Snow | Red, black |

Dimensions of Daymark

According to G1094 Daymarks for AtoN, a daymark must form an angle of least 1’ for it to be visible. To distinguish more complicated shapes the daymarks must form an angle of at least 3’.

For rectangular dayboard shape recognition is not critical. At great distance it is acceptable if the observer will see the dayboard as a small single colour vertical bar, i.e. at a horizontal angle of 1’. The central stripe blends with the outer ones because width of the dayboard is already smaller than the eye resolution. When approaching along the leading line, the stripes become distinguishable and allow for more accurate aligning of the leads.

In some countries standardized ratios of length and width of rectangular daymarks (e.g. 1:1.5, 1:2 or even up to 1:5) are used. However, different ratios can be used. With width determined by the minimum subtense angle (at least 1’), the ratio determines the height of daymarks.

For triangular, trapezoidal or other shapes, all dimensions must be at least 3’. For better conspicuity this value can be increased, e.g. to 6’ for any shapes.

The size of a daymark depends on its required visual range – the longer the useful segment, the greater the size of the daymark. When considering geometry only, the dimensions of a daymark should be proportional to the required range. The atmosphere will reduce the contrast of daymarks for long ranges according to 'Koschmieder's law'. The loss of contrast will reduce the range of the daymark which can be compensated by increasing the daymarks size exponentially with the daymark range. However, this may result in impractically large daymark sizes, so the method based on subtense angle i.e. angle the daymark covers in the observer’s eye as shown below is recommended.

Both length and width of a daymark are calculated with the formula:

LDM,rec or WDM,rec=0.291∙10−3∙a∙X	(47)

Where：

0.291∙10−3	value of 1’ in radians;

a	required subtense angle of a dimension of the daymark in angular minutes(‘);

X=XFL,far	distance from front light to far end of the useful segment;

X=XRL,far	distance from rear light to far end of the useful segment.

The graph on the Figure 15 shows dimensions of daymarks calculated for different ranges and different subtense angles.

Comparison of calculated daymark lengths

Practical Sizing Considerations

- 3’ is preferred length for rectangular or for any dimension for other shapes for most leading light designs.

- 6’ may be preferred in certain conditions, like in areas of reduced visibility or at major fairways where increased conspicuity is needed.

- 1’ is minimum width for rectangular daymarks. Less than this may not allow full use of the daymark without the use of binoculars.

- While providing a better signal to the mariner, larger daymarks require taller and stronger towers which will increase construction costs.

Retroreflective Daymarks

Some nations use retroreflective sheets for the daymark. The retroreflective materials can be used as a substitute for lights at leading marks on small craft routes and other less important fairways. They can be useful as back up for damaged lights on other fairways too. The daymarks are intended to be illuminated with a vessel’s searchlight.

Calculation of the required area of retroreflective materials should be done according to the IALA Guideline G1145 Application of Retroreflecting Material on AtoN.

Retroreflective materials are positioned so that they cover up to ¾ of the middle stripe so it covers ¼ of the total area of the dayboard. Due to not-perfectly-white or the colour being different from the rest of the middle stripe, too large reflectors can degrade the daytime conspicuity of dayboards.

To provide maximum possible vertical angle between the reflectors for avoiding blurring, the reflector of the front mark starts from the lower edge of the dayboard and reflector of the rear mark from the upper edge of the dayboard.

Additional considerations

For creating the most favourable conditions of illumination the planking of daymarks  may be made at 10-20°angle backwards from the direction of the leading line as seen on Figure 16. Alternatively, the whole daymark can be tilted backwards.

The final version of figure yet to be drawn

Conspicuity of leading marks observed against the sun can be enhanced by using semi-transparent material that enables the sun to illuminate the daymarks from back, as seen at Figure 17.

Dayboards using semi-transparent material

additional considerations

Lights

Fan versus Pencil Beam Lights

The lanterns of a leading line may consist of fan or pencil beam lights. Fan beam lights (large horizontal beam width) or omnidirectional lights are preferred when the required ranges are small and when the light should be visible far off of the channel centreline. The use of omnidirectional lanterns also eliminates the need for passing lights (the lights showing the position of an AtoN for ships passing it) on towers located in navigable waters.

However, an omnidirectional light needs much more energy and may cause glare, when vessels are passing very close to it. In this case, it is better to choose pencil beam lights.

Beam Width of Pencil Beam Lights

There is often confusion regarding the effect of a narrow spread lens on the sensitivity of a leading line. The beam width of the optic has nothing to do with the lateral sensitivity of a leading line. The beam width is of some concern when a narrow spread lens is used, as care must be taken to ensure that the minimum intensities identified as necessary are provided over the full width of the far end of the channel. The angle that need to be subtended at the far end of the useful segment ϕ, (see Figure 18) is given by:

ϕ≈WX	(48)

Where：

ϕ	in radians;

X	distance from the light to the far end of the channel in metres;

W	the width of the channel in metres.

Remark:

Written in degree, the equation is: ϕ in degree≈57,3 degreeradian∙WX

The equation is used for front light (position XFL,far) and rear light (position XRL,far), which result in two different values ϕFL and ϕRL.

Beam Width of Leading Lights

The need for acquisition of the leading lights prior to turning onto a leading line should also be considered when selecting the beam width of the leading lights. In order to reach the first useful segment of the leading line it will often be necessary to observe at least one of the leading lights in a region to seaward and/or to either side of the channel. In some countries the leading lights have to be visible at least 3 and better if 5 ship lengths before ships have to start the turn. This is called the 'acquisition region'. The selected leading lights should have a beam width (and intensity) sufficient to cover the desired acquisition region.

Additional Lights

If the design does NOT use omnidirectional optics at night, and the structure is located in navigable waters, it may be necessary to add omnidirectional lights to the structure. These additional lights should be mounted where they will not be blocked by the structure.

Additional lights should be mounted at a low enough height to ensure that the lights will be visible to vessels with a low height of eye. As an example, if a leading line has a front tower height of 8 m and a rear tower height of 25 m, the additional light for the front tower can be mounted directly above the front leading light. The additional light should display the same characteristic as the leading light, and should be synchronized with the leading light.

On towers greater than about 12 m, the additional lights may be designated as passing lights and installed at a lower level than the leading light. Installation of a passing light requires two optics, as the structure will partially occlude the output from each lantern. Due to potential to mislead a mariner consideration should be given to the passing lights be of different colour and character than leading lights.

Daymarks versus Daytime Lights

Traditionally leading lights, particularly those powered by batteries, were switched off during daytime, when the signal was provided by daymarks. Efficiency improvements in optics combined with solar power have allowed expanded use of daytime leading lights, even when mains power is not readily available. The following are some points to consider when deciding on the daytime signal:

Daymarks are simple. Having no moving parts they require little maintenance and so are more reliable than lights. Smaller daymarks are also easy to maintain, with no special training required for servicing personnel.

Daytime leading lights provide a superior signal. In poor conditions they can be seen further than daymarks. Furthermore, substituting lights for large daymarks may result in less costly tower structures and foundations. Daytime lights, however, require more complex lighting and power systems, which will increase hardware costs and the technical demands on the servicing personnel. However, the higher initial equipment costs will likely be more than offset by reduced structural costs.

Colour and Flashes

It is advantageous to use the same colour and synchronized flashes for front and rear leading light of a leading line. In the presence of rival lights this will highlight that these two lights belong together.

When a channel is marked by a series of leading lines in succession, each leading line should have a different flash character than the nearby ones.

If the lights are not synchronized the length of flashes and periods of the front lights could be shorter than these of the rear light.

Duration of the flashes should not be less than 0.5 s.

Length of the flashes of the rear light should be selected based on maximum overlapping of the duration of the flashes of the leading lights. The narrower the channel the more the flashes must be seen simultaneously.

Fixed (F) characteristics should be used sparingly, if at all. Lights displaying a fixed characteristic, especially white light signals, can be difficult to identify against even minimal background lighting. Furthermore, flashing lights displaying a character with a three second flash duration provide approximately 96% of the intensity of a fixed light signal, yield longer lantern service intervals, have lower power consumption, and provide greater conspicuity than the fixed light signal.

Placement of leading marks

Placement of the leading marks determines the axis of the leading line. Placement of leading marks along the centreline should be within ±3 meters of the desired position, while the lateral error in placement of the towers to either side of the true centreline should be limited to approximately ±0.3 meters. See Figure 19.

Placement of leading marks

Lantern Placement

Proper lantern placement is necessary to ensure that blur will not occur during nighttime and that each light or group of lights will be viewed as a single source. All heights are measured to the centre of the (set of) lantern(s). When using both day and night leading lights, the lower optic on the front tower and the upper optic on the rear tower should be the nighttime lights to keep them further apart as their possibility of blurring is higher. The only exception is if the front tower uses an omnidirectional nighttime light, which should then be mounted above the daytime light(s). Multiple lanterns for daytime lights should be installed as close to each other as possible to ensure the lights are viewed as a point source.

Abbreviations and Technical Quantities

LATIN

| Abbreviation | Explanation | Chapter |
| --- | --- | --- |
| a | required subtense angle of a dimension of the daymark in angular minutes(‘) | 5.10.3 |
| C | Length of useful segment | 4.1 |
| CTF | Cross-track factor | 5.7 |
| dU | unit distance (dU=1852m=1M) | 4 |
| E | illuminance | 5.2 |
| EFL | Illuminance at the eye of the observer produced by the front light | 4.4 |
| Emax | Allowed maximum illuminance at the eye of the observer | 5.2.2 |
| Er | Required minimum illuminance at the eye of the observer | 5.2.1 |
| ERL | Illuminance at the eye of the observer produced by the rear light | 4.4 |
| EU | Unit illuminance (EU=1 lx) | 4 |
| E+ | maximum illuminance at the eye either from FL or RL | 5.4 |
| IFL | Luminous intensity, front light | 4.4 |
| IRL | Luminous intensity, rear light | 4.4 |
| H | Height | 4.5 |
| HFL | Height of front light above MHW or MSL | 4.2 |
| HFL,min1 | Minimum height of front light (height above safe water) | 5.9.1.1 |
| HFL,min2 | Minimum height of front light with a daymark (height above safe water) | 5.9.1.1 |
| HFL,min3 | Minimum height of front light (geographical range) | 5.9.1.2 |
| HFL,min4 | Minimum height of front light with a daymark (geographical range) | 5.9.1.2 |
| HFL,min5 | Minimum height of front light (obstruction, far end) | 5.9.1.3 |
| HFL,min6 | Minimum height of front light (obstruction, near end) | 5.9.1.3 |
| HFL,min7 | Minimum height of front light with a daymark (obstruction, far end) | 5.9.1.3 |
| HFL,min8 | Minimum height of front light with a daymark (obstruction, near end) | 5.9.1.3 |
| HFL,rec | Recommended height of the front light | 5.9.1.4 |
| HFL,sel | Selected height of the front light | 5.9.1.4 |
| Hobs | Height of an obstruction above MHW or MSL | 5.9.1.3 |
| Hv | Height of observer (on vessel) above vessel waterline | 4.2 |
| HRL | Height of rear light above MHW or MSL | 4.2 |
| HRL,min1 | Minimum height of rear light (blur, far end) | 5.9.2.1 |
| HRL,min2 | Minimum height of rear light (blur, near end) | 5.9.2.1 |
| HRL,min3 | Minimum height of rear light with a daymark (front light not obscuring, far end) | 5.9.2.3 |
| HRL,min4 | Minimum height of rear light with a daymark (front light not obscuring, near end) | 5.9.2.3 |
| HRL,min5 | Minimum height of rear light (obstruction, far end) | 5.9.2.4 |
| HRL,min6 | Minimum height of rear light (obstruction, near end) | 5.9.2.4 |
| HRL,min7 | Minimum height of rear light with a daymark (obstruction, far end) | 5.9.2.5 |
| HRL,min8 | Minimum height of rear light with a daymark (obstruction, near end) | 5.9.2.5 |
| HRL,rec | Recommended height of the rear light | 5.9.2.6 |
| HRL,sel | Selected height of the rear light | 5.9.2.6 |
| Hshaw | Safe height above water | 5.9.1.1 |
| hU | Unit height hU=1 m | 4 |
| I | luminous intensity | 5.2 |
| IFL,max | Maximum luminous intensity, front light | 5.2.2 |
| IFL,min | Minimum luminous intensity, front light | 5.2.1 |
| IFL,dsg | Design luminous intensity, front light | 5.2.3 |
| IFL,sel | Selected luminous intensity, front light | 5.4 |
| IRL,max | Maximum luminous intensity, rear light | 5.2.2 |
| IRL,min | Minimum luminous intensity, rear light | 5.2.1 |
| IRL,dsg | Design luminous intensity, rear light | 5.2.4 |
| IRL,sel | Selected luminous intensity, rear light | 5.4 |
| L | (vertical) length, usually for the daymark | 4.5 |
| L | Distance to front structure from far end of useful segment | 4.1 |
| LDM | Vertical length of a daymark | 5.10.1 |
| LDM,rec | Recommended vertical length of a daymark | 5.10.3 |
| LFDM | Vertical length of the front daymark | 4.2 |
| LFDM,sel | Selected vertical length of the front daymark | 5.9.1.1 |
| LRDM | Vertical length of the rear light daymark | 4.2 |
| LRDM,sel | Selected vertical length of the rear daymark | 5.9.2.5 |
| M | Distance to front structure from near end of useful segment | 4.1 |
| Minitial | Initial value of M for iteration process | 5.1 |
| MHW | Mean high water | 4.2 |
| MLW | Mean low water | 4.2 |
| MSL | Mean sea level | 4.2 |
| MTR | Mean tidal range | 4.2 |
| R | Distance between leading line structures | 4.1 |
| Rinitial | Initial value of R for iteration process | 5.1 |
| Rg | geographical range | 5.8 |
| r | ratio of intensities | 5.2.4 |
| rmid | ratio of intensities, value in the middle of the useful segment | 5.2.4 |
| rfar | ratio of intensities, value in the far end of the useful segment | 5.2.4 |
| S | distance between an obstruction and the near end of the useful segment | 5.9.1.3 |
| Tmax | Maximum transimissivity | 5.4 |
| V | visibility | 5.2 |
| Vdsg | Design Meteorological visibility | 5.2.4 |
| Vmax | Maximum Meteorological visibility | 5.2.2 |
| Vmin | Minimum Meteorological visibility | 5.2.1 |
| W | Channel width | 4.1 |
| WDM | Width of a daymark | 5.10.1 |
| WDM,rec | Recommend width of a daymark | 5.10.3 |
| x | Distance of observer (vessel) from front tower | 4.1 |
| XFL,mid | Distance from front light to middle of the useful segment | 5.2.4 |
| XFL,far | Distance from front light to far end of the useful segment | 5.2.4 |
| XRL,mid | Distance from rear light to middle of the useful segment | 5.2.4 |
| XRL,far | Distance from rear light to near end of the useful segment | 5.2.4 |
| y | Off-axis distance of observer (perpendicular to leading line) | 4.1 |
| yd | Off-axis distance the navigator can detect with certainty that the vessel is not on the leading line | 5.7 |
| Z | dip of horizon | 5.3 |
| ZFL | Dip of horizon at front light | 5.3 |
| ZRL | Dip of horizon at rear light | 5.3 |

Greek

| Abbreviation | Explanation | Chapter |
| --- | --- | --- |
| α | factor for calculation of dip of horizon (α=6.75∗10−8m−1) | 5.3 |
| ϕ | Required angle of subtense of a leading light at the far end of channel. | 6.1.2 |
| ϕFL | Required angle of subtense of front light at the far end of channel. | 6.1.2 |
| ϕRL | Required angle of subtense of rear light at the far end of channel. | 6.1.2 |
| θ | Horizontal difference angle (horizontal angle between front and rear light) | 5.5 |
| θD | Bearing difference for sensitivity | 5.6 |
| θFL | Horizontal angle, front light | 5.5 |
| θRL | Horizontal angle, rear light | 5.5 |
| θ1 | Partial result for sensitivity | 5.6 |
| θ2 | Partial result for sensitivity | 5.6 |
| γ | Vertical difference angle | 5.3 |
| γFL | Vertical angle, front light | 5.3 |
| γmin | Minimum vertical difference angle | 5.4 |
| γRL | Vertical angle, rear light | 5.3 |

references

IALA Recommendation R0112 on Leading Lights, December 2005

IALA Guideline 1023 The Design of Leading Lines, December 2001

IALA Guideline G1148 Determination of Required Luminous Intensity for Marine Signal Lights

Système International d'Unités (International System of Units) published by Bureau International des Poids et Mesures (International organization established by the Metre Convention), www.bipm.org

Regulations of the IHO for International (INT) Charts and Chart Specifications of the IHO, Edition 4.9.0, 2021

United States of America, US Coast Guard, Instruction M16500.4, Range Design, 1980

Report of the Proceedings of the International Technical Conference on Lighthouses and other Aids to Navigation , Paris 1993, published by Trinity House 1936

IALA Guideline 1094 Daymarks for Aids to Navigation, Edition 2.0, June 2016

United States Coast Guard Aids to Navigation Manual - Technical, U.S. Department of Homeland Security, Apr 06 2010

IALA Guideline G1078 The Use of AtoN in the Design of Fairways