Introduction

When revising the leading line guideline, the task group have provided some additional and useful information about the design of leading line which are not include in the guideline. To prevent the loss of these information, this document has been created to record it which is now saved in the file-share and will be moved to IALA  Wiki for future reference.

Background

Although leading lines play an important role for navigation in narrow channels, there was no internationally published design methodology or specification before 1998. The appearance and the design of leading lines is therefore very different at each nation.

The first standardization was published by IALA with Recommendation E-112 in 1998 e. The recommendation included basic requirements on the position, height and luminous intensity of leading lights. As a design methodology was missing, a spreadsheet program for leading line calculation was created and the manual for the use of the spreadsheet became IALA Guideline 1023 in 2001 [2]. In addition to the methodology from the Recommendation, ethe Guideline includes a methodologies for sizing daymarks. Both recommendation and guideline are mainly influenced by the standard design of the United States Coast Guard.

Since then, the IALA documents have been widely used for the design of new leading lines. However, many leadings lines were built long before 1998 and some nations still use their national design tools or specifications, when a leading line must be changed or newly built.

When using a leading line, the determination of the “usable width of a channel” or of the axis to be marked in a natural channel for each particular case requires taking into account not only of the inaccuracy (sensitivity) of the relevant aid to navigation (leading line), but also different 'nautical margins', such as those resulting from the breadt

Sensitivity of a Leading Line

The IALA Recommendation of 1998 classified the sensitivity (precision) of a leading line into three categories. However, these categories have never been used strictly. The IALA guideline and the spreadsheet use the "Cross Track Factor" (CTF) instead. The CTF is calculated with the formulae of the second category.

Categories:

The standard deviation of the bearing difference of the two lights, when the observer has the impression that the two lights are vertically in line, should be calculated using the formulae:

θQ=maxθ1,θ2	(1)

Where:

θ1=0.05∙10−3rad+0.038∙γ                           for γ≤5∙10−3 rad 0.1∙10−3rad+0.028∙γ     for  5∙10−3<γ≤20∙10−3 rad;

θ2=0.07∙γmin	(see IALA Guideline G1023 Section 5.4).

The magnitude of the minimum value of the bearing difference of the two lights, when an off-axis deviation in a definite direction is considered as detected with certainty by the observer, should calculated using the formulae:

θD=maxθ'1,θ'2	(2)

Where:

θ'1=0.16∙10−3+0.12∗γ                           for γ≤5∙10−3 rad 0.31∙10−3+0.09∗γ     for  5∙10−3<γ≤20∙10−3 rad;

θ'2=0.224∙γmin	(see Section 5.4).

In the interest of safety, the axis of the leading lights be established in such a manner that for the type of ships and navigation conditions considered, the ship may safely depart from the axis up to a point where the bearing difference as observed by the navigator reaches the following value in radians:

θM=maxθ''1,θ''2	(3)

Where:

θ''1=0.25∙10−3rad+0.19∙γ                           for γ≤5∙10−3 rad 0.5∙10−3rad+0.14∙γ     for  5∙10−3<γ≤20∙10−3 rad;

θ''2=0.35∙γmin.

SIMPLIFIED INTENSITY CALCULATION

Introduction

The minimum required luminous intensity Imin of a marine signal light is traditionally calculated with the Allard's law (see IALA Guideline G1148).

Imin=Emin∙dmax2∙0.05−dmaxVmin	(4)

Where：

Imin	the minimum required intensity (cd);

Emin	the minimum required illuminance at the eye of the observer (lx);

dmax	distance of farthest point of the required range to the AtoN light (m);

Vmin	the minimum meteorological visibility in the area of interest (m).

The minimum required illuminance at the eye of the observer was standardized to Emin=2∙10−7lx in 1933 at an international conference [7]. This illuminance value is linked to the physiological threshold of human eye and results in very low intensities at short ranges. For example, a light with a range of 2 M and a minimum visibility of Vmin=10 M would get an intensity of 5 cd only.

required intensity of Leading Lights

It was commonly accepted that to ensure their usability, leading lights should be much more intense than the all other lights and therefore the minimum required illuminance for leading lights was increased five times to Eminleading line=10−6lx by IALA. This value was introduced in the Recommendation E-112 in 1998.

With this convention the required minimum intensity for leading lights (black curve inline at Figure 1) becomes five times higher than the other lights (blue curve inline at Figure 1).

Required intensities when calculated with different input for V and E

In addition, to provide for worse visibility weather, it would be prudent to choose a minimum visibility lower than 10 M, which will increase the required intensity as well. This is shown in Figure 2 with a visibility of Vmin=4 M (red curve inline at Figure 2).

When the IALA Guideline was developed in 2001, it was recognized that all these measures in calculations were not enough to represent all the existing lights, which have been proven to provide suitable intensities for decades. The calculated minimum required intensities in the ranges about 4 000 m and 7 000 m were still below 1 000 cd, but existing lights for these ranges were found to be about 10 000 cd or even 100 000 cd.

To solve this contradiction a new value called 'recommended intensity' was introduced in the IALA Guideline 2001. This recommended intensity is ten times the minimum value (red dotted line in Figure 2).

Recommended Intensity for a meteorological visibility Vmin=10 M

IALA Guideline G1148

An alternative and more controlled procedure to get higher than plain minimum values for required or recommended luminous intensity that correspond better to possible existing lights is available in IALA Guideline G1148, which presents measures for situations where when background illumination and rivals lights have to be considered. The guideline recommends the minimum required illuminance value 2∙10−6 lx (factor 10) for minor and 2∙10−5 lx (factor 100) for considerable background lighting. Both were already introduced in the 2001 IALA Guideline on leading lines. Guideline G1148 also presents the option to calculate the factor for/ the effect of background illumination based on measurement of it.

Application of Guideline G1148 requires more work on identifying the background illumination and the rival lights. This is necessary, when problems concerning the visibility of marine signal lights have been reported and there is a need of action.

Simplified Method

The German Administration investigated its exiting leading lights and found that all methods, which still have the Allard's Law as a basis, do not really represent the actual correlation between required ranges of lights and the intensities used on them (shown with blue dashed line in Figure 3) which has caused no problems/complaints from users and seems to work fine .

Luminous intensity of German leading lights

For short range lights  the actual intensity is much higher than the ones calculated according to '2001 IALA Guideline', even when the minimum visibility is about 4 M (① in Figure 3). In these cases the lights are very often nearby a harbour or a city with a lot of background illumination and rival lights.

For long range lights the existing intensities are below the values calculated according to '2001 IALA Guideline' (②in Figure 3). Background illumination and rival lights do not play an import role in these cases, but instead glare at near end of useful segment and high costs are relevant.

Based on the existing lights, a simplified empirical equation for determining recommended luminous intensity for leading lights was be derived.

Irec,sim=Irec,M∙XFardU3	(5)

Where：

Irec,M=1000 cd		recommended luminous intensity for XFar=1852 m=1 M;

dU=1852 m=1 M	the minimum required illuminance at the eye of the observer (lx).

There is some arbitrariness in this this equation and the coefficients were chosen to keep the equation simple, but it corresponds very well to existing German leading lights.

The calculation is done for the front light and the rear light intensity is harmonized with that using the 'illumination ratio calculation' of the '2001 IALA Guideline'.

The equation does not require selecting a meteorological visibility or estimating background illumination or rival lights. However, the resulting intensities have been proven acceptable for decades considering background illumination and rival lights indirectly. A curve for recommended intensity according to the simplified method (blue curve) is shown in Figure 4.

When problems are reported with intensities calculated with this method, it is advised to use the methods of IALA Guideline G1148 [3].

Recommended luminous intensity according to the simplified method shown with blue curve

Initial CHANNEL LAYOUT

The initial channel layout is to establish a beginning position for the leading line towers when they are not otherwise constrained. For these preliminary calculations it is necessary to assume values for vertical separation of the lights, cross-track sensitivity and a distance from the near end of the useful segment to the front tower.

Bearing difference

Refer to IALA Guideline G1023, bearing difference θdfor detection of not being on the leading line based on elevation difference γ (vertical difference angle) is calculated using the formulae:

θd=0.16∙10−3+0.12∗γ for γ≤5∙10−3 rad	(6)

Before adopting the final luminous intensities, a preliminary investigation of appropriate sites is undertaken, it should be based on a value of γm=1.5∙10−3 rad (≈5') .

Assuming γm=1.5×10−3 radians and substituting in Formulae (6),

θd=0.16∙10−3+0.12∗(1.5∙10−3) →

θd=0.34mrad

Off-AXIS DISTANCE

Refer to IALA Guideline G1023, the off-axis yd at which a mariner can detect with certainty that a vessel is not on the leading line:

yd≈θd∙x∙(1+xR)	(7)

Where：

R	the distance between the front and rear light;

x	the distance from front light to the observer.

The distance between near end of useful segment and front light, M, is assumed to be 0.2C (C is the length of the useful segment). This value is based on design experience, but may be deemed too low if background lighting or low visibility requires bright lights, which could cause glare at the near end of the useful segment.

SEPERATION OF FRONT AND REAR LIGHTS

By identity, both sides of Formulae (7) can be divided by 12W :

yd12W=θd∙x∙(1+xR)12W	(8)

yd12W is defined as the ‘cross-track factor’ in IALA Guideline G1023. For these preliminary calculations, a cross-track factor of 20% is assumed, inferring a ‘very good’ leading line (see Table 1 in IALA Guideline G1023).

Assuming yd12W=0.2 and substituting in above equation:

0.20=yd12W=θd∙x∙(1+xR)12W →

0.10W=θd∙x∙(1+xR) →

W10θd∙x=1+xR →

R=xW10θd∙x−1

Assuming the observer is at the far end of the useful segment, so x=M+C, θd=0.34mrad, then:

R=M+C1000W3.4∙(M+C)−1	(9)