Input paper:  ENG22-3.1.1.4

Input paper for the following Committee(s):		Purpose of paper:

（Select as appropriate）

☐ ARM	X  ENG		☐ PAP			X  Input

☐ DTEC	☐ VTS					☐ Information

Agenda item 			 n.n

Technical Domain / Task Number	 Task 6.3.1

Author(s) / Submitter(s)			 China Maritime Safety Administration (China MSA)

# PROPOSAL FOR THE REVISION OF GUIDELINE “G1008 REMOTE CONTROL AND MONITORING OF MARINE AIDS TO NAVIGATION”

# Summary

The first Council of the International Organization for Marine Aids to Navigation (IALA)  approved the Committee's work plan for the period 2025-2027. Guideline “G1008 Remote Control and Monitoring of Marine Aids to Navigation” requires further revision and review. In accordance with the revisions to the guidelines discussed at the 21st ENG meeting and the work arrangements for the 22nd meeting, the China Maritime Safety Administration has further proposed amendments to the draft of the guideline “G1008 Remote Control and Monitoring of Marine Aids to Navigation” for the consideration of the ENG Committee.

## Purpose of the document

This document, incorporating the revision direction and approach from ENG 21st Session, focuses on cyber security, hardware protection, data storage, system integration, maintenance, and training for RCMS System. It aims to supplement and refine Chapters 5 to 15 of the draft Guideline “G1008 Remote Control and Monitoring of Marine Aids to Navigation”. It refines the schematic diagram of the main components of the RCMS system discussed at ENG 21st Session and adds textual explanations; In line with the ENG Committee's revision approach for cyber security pre-design, incorporate relevant sections from Guideline G1182 Cyber security Guidelines from the International Association of Marine Aids to Navigation and Lighthouse Authorities to supplement cyber security content in chapters covering communication links and equipment design; enhance the newly added hardware protection chapter from the ENG21 meeting; integrate content from the deleted ‘Future Development Trends’ chapter into the ‘Integration with Other Systems’ chapter as per ENG21 decisions; Further refinement of unamended sections, providing revision proposals concerning monitoring parameter categories, data storage, maintenance and testing, training, etc.; add a new chapter titled “System Acceptance and Performance Evaluation; aiming to assist the revision of the guidelines at ENG22.

## Related documents

G1008 Remote Control and Monitoring of Marine Aids to Navigation (Edition 2.1, June 2009)

G1182 Cybersecurity Guidelines from the International Association of Marine Aids to Navigation and Lighthouse Authorities

Demonstration course L2 10.12  "Introduction to Remote Monitoring of AtoN"

# Background

The guideline G1008 Remote Control and Monitoring of Marine Aids to Navigation was first issued in 1998. Rapid technological advancements and widespread adoption significantly reduced telemetry and telecontrol costs. It was revised in 2005 to incorporate IALA documentation, underwent comprehensive updating and review in 2009 to reflect monitoring sector developments since its inception, and was re-edited and proofread in 2022. The ENG21 session in 2025 reviewed proposals concerning this guide since the ENG13 session and introduced new revisions to its overall structure. Certain sections addressing telemetry and telecontrol cybersecurity and hardware protection remain under content revision and refinement.

# Discussion

## Supplementary Refinements to Chapter 5 "General Concepts of Telemetry and Remote Control Systems"

## 3.1.1 Collation and Supplement of the Schematic Diagram for Main Components of the Telemetry and Remote Control System

The schematic diagram of the main components of the telemetry and remote control system in Chapter 5 was discussed at the  ENG21 Meeting, and a preliminary sketch was finalized. Based on this draft  and the meeting discussions, the final schematic diagram has been prepared for the reference of the ENG Committee. The schematic diagram and its textual description are as follows:

This diagram illustrates the communication architecture of the telemetry and remote control system, covering the processes of data collection, network transmission, and user interaction. The system connects the front-end and backend through various communication media, supports multi-source data fusion and standardized interfaces, and thereby enables efficient remote monitoring and management.

Figure 1 Communication Architecture of Remote Control and Monitoring System

## 3.1.2 Supplementary Content on Cybersecurity in Section 5.2 "Design Considerations"

In accordance with the revision concept of prioritizing cybersecurity design discussed at the 21st ENG Meeting, and with reference to Guide G1182 "Cybersecurity Guidelines from an IALA Perspective", it is proposed to add content related to cybersecurity access control in Section 5.2 "Design Considerations". Specifically, it is recommended that  a new subsection "Access Control" be added after 5.2.5 "Communications" in the original text, with the following suggested content:

• Access Control

Access control systematically restricts  users, programs, and systems from accessing resources based on the principle of least privilege. As a crucial measure to mitigate lateral movement attacks and reduce internal risks, it shall be designed synchronously at the network, physical, and application levels, including but not limited to the following aspects:

- Network Segmentation and Data Center Isolation: The operational network shall be logically or physically isolated from the office network. In the communication network, strictly enforce IP segment isolation for links connecting multiple data centers, remote stations, and monitoring centers to ensure secure data transmission paths.

- Secure Isolation in Virtualized Environments: When deploying government or customer-specific clouds using physical server virtualization, ensure effective isolation between different tenants or business systems through security groups, virtual networks, and resource pool policies, balancing data security and flexible scalability.

- Monitoring and Resilience Design for Distributed Architectures: Implement monitoring of the distributed operation of codes and services based on a geographically distributed server architecture. This design not only improves processing capacity but also serves as a key to achieving high availability and disaster recovery. It is necessary to monitor the status of each node to ensure rapid detection of local failures and seamless service switching.

- Principle of Least Privilege: Strictly limit remote access permissions, granting only the minimum privileges required for employees and third parties to complete their tasks. Implement time restrictions and activity monitoring/recording for remote access.

- Physical Access Control: Enforce strict physical access control for cabinets housing critical electronic equipment, VTS centers, remote sensor stations, etc., such as locking and using sensors to detect unauthorized entry.

- Interface and Function Restrictions: Disable all unnecessary network ports, wireless interfaces, and software functions not essential to business operations.

## 3.1.3 Supplementary Section 5.2.6 "Hardware Protection"

Section 5.2.6 "Hardware Protection" was added at the 21st ENG Meeting, but only the considerations for hardware and circuit protection were listed without detailed elaboration. The following content is recommended to be supplemented:

• Circuit Protection

Overcurrent Protection: Adopt a hierarchical configuration for overcurrent protection. Install circuit breakers, fuses, overcurrent protection relays, etc., in the circuit, and match thermal relays for inductive loads.

Overcurrent Prevention Mechanism: Deploy self-resetting fuses, overcurrent detection chips, etc., to promptly cut off the circuit when the current exceeds the limit.

• Lightning Protection

A multi-level lightning protection system shall be established. Lightning protection devices shall be equipped, and a low-impedance grounding system shall be constructed, with an emphasis on protecting parts vulnerable to lightning induction, such as solar panel feeder lines and long-distance signal lines. Navigation mark metal components and lines shall be connected through surge protectors; additionally, surge protectors shall be installed at power, communication, and signal interfaces, and at least three-level lightning protection shall be implemented for microelectronic equipment.

• Physical Protection

Sealed Structure Design: Adopt a sealed structure for equipment and battery compartments, equipped with sealing rings, moisture-proof silica gel, desiccants, etc., using sealing technology.

Vibration Resistance: Select products with high vibration resistance, configure shock absorption brackets, and secure internal components via elastic mounting methods.

• Environmental Protection

Materials and components with excellent corrosion resistance, superior insulation performance, and high-level waterproofing shall be prioritized, which shall meet the requirements of the IP66 waterproof rating and the ISO 12944 C4 outdoor corrosion protection rating.

## Supplementing Chapter 7, "Monitoring – General Issues," with Cybersecurity-Related Content

3.2.1 Supplementing Chapter 7.2, "Monitoring Parameter Categories"

It is recommended to add telemetry and remote control system status monitoring to the monitoring parameter categories in Chapter 7.2. It is also recommended to add system status monitoring content after the individual status section in the original Chapter 7.2, "Monitoring Parameter Categories." Suggested additions:

• System Status: Abnormal behavior monitoring, log collection and analysis, penetration testing and vulnerability scanning, threat intelligence utilization. This can proactively discover potential threats, achieve proactive defense, and rapid incident response, contributing to beacon monitoring and operation.

3.2.2 Supplementing Chapter 7.3.2, "Coordination of Communication Links"

Based on the revision ideas from the ENG21 meeting, and referencing Guideline G1182, "Cybersecurity Regulations from the Perspective of the International Association of Marine Aids to Navigation and Lighthouse Authorities," cybersecurity-related content should be added to chapters such as communication link and equipment design. It is recommended to add link encryption content to Chapter 7.3.2, "Coordination of Communication Links." Suggested additions:

• 7.3.3 Link Encryption

Link encryption is a core technical means to ensure the confidentiality and integrity of data during transmission, effectively preventing communication from being eavesdropped on or tampered with. To ensure the communication security of IALA's critical infrastructure, encryption mechanisms should be established during the design and deployment phases. The following additional measures are recommended:

- Data Transmission Encryption: Implement strong encryption for all IP-based communication links to ensure the confidentiality and integrity of data during transmission. International standards such as SECOM (IEC 63173-2) shall be complied with.

- Wireless Communication Protection: Enable modern encryption protocols for wireless management interfaces such as Wi-Fi, Bluetooth, and RF. These wireless interfaces should be disabled when not performing maintenance to reduce exposure.

- Communication Integrity Assurance: Apply encryption and authentication throughout critical communication links to prevent data from being eavesdropped on or tampered with during transmission.

## Supplement and Improve Chapter 12 "Display, Storage, and Control"

3.3.1 Supplement and Improve Section 12.3 Data Storage

Section 12.3 Data Storage was revised at the 21st ENG Meeting. This section describes two storage methods, namely paper and electronic media, with electronic storage having become the core method for data retention. Its continuous and stable operation relies on the health status of the system. Communication links, remote stations, and monitoring devices affect the normal collection, transmission, and storage of data by the telemetry and remote control system. However, this section does not elaborate on the methods for evaluating the health status of such systems. It is recommended that the following content be added:

• Communication Link Quality Assessment

- Key Indicator Monitoring: Collect core parameters (e.g., link bandwidth, packet loss rate) in real time and set corresponding thresholds. The indicator system shall be adaptable to different link configurations.

- Dual-Link Redundancy: Configure dual-link redundancy to enable automatic switching in case of malfunctions, ensuring transmission continuity.

- Regular Verification Mechanism: Conduct regular link load tests to check connectivity and verify link capacity and operational stability.

• Remote Station Reliability Assessment

- Hardware Health Monitoring: Deploy sensors on core hardware (e.g., industrial control computers, power supplies) to monitor operating parameters in real time. Predict service life by analyzing historical fault data.

- Software and Deployment Environment Assessment: Regularly review system logs and apply security patches. Configure protective devices (e.g., lightning protection, climate control) for the deployment environment.

- Redundancy Backup and Emergency Drills: Provide standby units for core equipment, implement local dual backup for key data, and conduct regular fault-override drills.

• Monitoring Device Reliability Assessment

- Equipment Performance Calibration and Verification: Regularly calibrate sensors and other devices. Conduct comprehensive performance tests periodically and mandatorily replace equipment beyond its service life.

- Data Consistency Verification: Establish a cross-comparison mechanism for data from multiple devices. Identify and troubleshoot abnormal data entries.

- Fault Early Warning and Rapid Response: Configure alarm rules for equipment anomalies. Provide troubleshooting guidance upon alarm triggering and maintain a spare parts inventory.

3.3.2 Supplement and Improve Section 12.5 Security of Computer Systems

With reference to Guide G1182 Cybersecurity Guidelines from the International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA), it is recommended to improve the system access control content in Section 12.5 Security of Computer Systems by adding the following:

Identity authentication serves as the first line of defense for cybersecurity access, strictly verifying the legitimacy of users and devices. Establishing a reliable authentication mechanism is the foundation for implementing granular permission management and data traceability. Recommended measures include but are not limited to:

- Strengthen User Authentication:Enforce strict user identity authentication for all programmable or manageable systems. Disable default credentials and mandate the use of strong passwords with regular updates.

- Multi-Factor Authentication (MFA): Implement MFA for high-privilege access scenarios, such as remote access and privileged operations.

- Device and Source Authentication: Implement source authentication mechanisms for sensors and data-sending devices to ensure data originates from trusted and legitimate sources.

- 4.Centralized Identity Management: Consider adopting a centralized authentication database to uniformly manage user credentials and permissions.

## Supplement Chapter 13 "INTEGRATION WITH OTHER SYSTEMS"

Based on the revised ideas from the ENG21 meeting, the "Future Development" chapter was deleted and its content was integrated into other chapters. The chapter 13 "Integration with Other Systems" was supplemented. A new Section13.5 "Standardization Strategies for System Integration" was added. It is recommended to include the communication protocol standardization from the original chapter 16 "Future Development", as well as the standardization requirements for data and open API, to achieve the goals of integrating with other systems and business processes, seamless cross-system interoperability, full-link closed-loop data flow, and secure controllability.

It is recommended that the following additions be made:

• Data format: Unified basic encoding and format selection, consistent time and space, lightweight structured, fault-tolerant verification, extensible, and compatible with protocols/API.

• Communication protocol: Select protocols based on scenarios, unify transmission rules, frame structure, and heterogeneous adaptation rules.

• Open API: Unified API architecture, standardize interface design, invocation, security, and operation and maintenance rules.

## Supplement Chapter 14"MAINTENANCE AND TESTING"

The ENG21 meeting has not yet revised Chapter 14.6 "Maintenance Philosophy". However, the ENG21 meeting did revise the Section 3.1 "Purpose". It was mentioned that when the  authority formulates the maintenance plan, they should attach great importance to and fully utilize the remote diagnosis and data capabilities of RCMS, systematically plan and deploy CBM and PdM strategies, by precisely maintaining to extend the Mean Time Between Failures (MTBF) and shorten the Mean Time to Repair (MTTR); and based on data analysis, optimize operation and maintenance resources and reduce costs. It is suggested that the content of "Condition-Based Maintenance" and "Predictive Maintenance" be reflected in Chapter 14.6 "Maintenance Philosophy", and it is recommended to add:

Maintenance personnel can utilize the real-time equipment operation data collected by the RCMS, such as battery voltage, current, temperature, and equipment alarms, to remotely assess the status of the navigation beacons, enabling rapid fault location and preliminary diagnosis. This provides data support for implementing condition-based maintenance (CBM) and predictive maintenance (PdM).

- The core of Condition-Based Maintenance (CBM) lies in scheduling maintenance based on the actual health status of the equipment, rather than fixed time periods. RCMS continuously monitors key parameters and can issue warnings when there are quantifiable signs of performance decline in the equipment (such as continuous battery voltage below the threshold or systematic decline in solar charging efficiency), enabling maintenance actions to precisely match the equipment's needs and avoiding over-maintenance or insufficient maintenance.

- Predictive Maintenance (PdM) is an advancement on the basis of CBM. It utilizes data analysis and modeling techniques to predict the specific time and type of equipment failures that may occur in the future. By conducting in-depth analysis of the long-term accumulated data from RCMS (such as correlating environmental temperatures with equipment failure rates and establishing life prediction models for key components), the operation and maintenance team can plan spare parts in advance, arrange personnel, and take proactive intervention before failures occur, thereby maximizing the availability of navigation aids and optimizing the total life cycle cost.

## Supplement Chapter 15 "DOCUMENTATION AND TRAINING"

The section 15.2 "Training" only mentions the training content for the operation of the RCMS. Refer to the demonstration course L2 10.12 "Introduction to Remote Monitoring of AtoN". It is recommended to add training on cyber security after the "configuration of base station system software" in section 15.2. The suggestions for addition are as follows:

• Basics of cyber security;

• Risk assessment;

• Emergency response and handling;

## Add a new chapter titled "Quality Assurance and Performance Improvement"

The current G1008 Remote Control and Monitoring of Marine Aids to Navigation focus on system planning and design, but lack specific, actionable international recommendations regarding quality control throughout the system lifecycle, performance baseline establishment, and continuous improvement mechanisms. When implementing RCMS systems, member states primarily rely on national standards or vendor specifications. This reliance may lead to challenges in cross-border data exchange, system compatibility comparisons, and best practice sharing, thereby impacting the overall coordinated effectiveness of global navigational aids services.

This proposal integrates China's practical experience in constructing and accepting RCMS systems. Drawing upon the Chinese industry standard Technical specification of remote monitoring and control system for the aids to navigation(JT/T 788-2023), it provides a quality management framework spanning the entire system lifecycle, encompassing methods for establishing performance baselines, key performance indicators, testing methodologies, and data-driven continuous improvement mechanisms. This chapter complements the existing G1008 Remote Control and Monitoring of Marine Aids to Navigation Chapter 14, “Maintenance and Testing,” extending the quality management perspective beyond post-deployment maintenance. It incorporates quality considerations into the system's design, development, integration, and delivery phases, and extends backward to establish a continuous improvement cycle based on performance data. Together, these elements form a comprehensive quality assurance system spanning deployment through operations and maintenance.

It is suggested to add a new chapter titled “Quality Assurance and Performance Improvement” between Chapter 13 “Integration with Other Systems” and Chapter 14 “Maintenance and Testing” in the latest draft guidelines, with the following content:

## 3.7.1 General Overview

Core Content of This Chapter:

1. Three-Stage Acceptance Process: Establish a standardized “Factory Acceptance – Site Acceptance – Final Acceptance” process to achieve comprehensive quality control throughout the entire lifecycle from equipment manufacturing and on-site installation to long-term operation.

2. Seven Key Performance Metrics: Cover system availability, communication performance, data integrity, control reliability, alarm performance, power adaptability, and environmental adaptability, with quantifiable targets (e.g., system availability ≥99.5%, communication delay ≤30 seconds).

3. Six Core Test Components: Include communication link, data acquisition, remote control, system redundancy, power adaptability, and environmental adaptability testing to ensure stable system operation across diverse real-world scenarios.

4. Continuous Performance Monitoring Mechanism: Establish a three-tier assessment system (monthly, quarterly, annual) combining trend analysis and root cause analysis to achieve closed-loop management and continuous optimization of system performance.

5. Standardized documentation templates: Provide standardized templates for acceptance test plans, test case sheets, and acceptance reports to enhance the standardization, traceability, and international compatibility of the acceptance process.

## 3.7.2 Establish Performance Baseline

Before a system is formally put into operation, its performance baseline must be established through a rigorous testing process. This baseline serves as the reference standard for all subsequent maintenance and evaluation activities, and as the basis for determining whether the system meets the expected quality level.

The establishment of the system performance baseline should be divided into three phases: factory acceptance, on-site acceptance, and final acceptance. The specific process is as follows:

| Stages | Steps | Description |
| --- | --- | --- |
| Factory Acceptance | Hardware and software functional testing, interface protocol verification, documentation review | Performed prior to equipment shipment to validate compliance with technical specifications under controlled conditions, minimizing on-site commissioning risks. |
| On-site Acceptance | Installation and commissioning inspection, communication link testing, control function validation, environmental adaptation testing | Conducted after on-site installation to verify system integration and functional integrity within actual physical and electromagnetic environments. |
| Final Acceptance | Continuous operation testing (≥30 days), performance metric evaluation, user training acceptance, acceptance report preparation | Following the system trial operation period, a comprehensive assessment based on operational data is performed to obtain performance metrics under typical loads. This formally confirms the system meets acceptable quality standards before handover to the operations and maintenance team |

Table 1 Three Stages of Establishing  Performance Baseline

## 3.7.3 Key Performance Indicators

The following table defines the key performance indicators used to establish performance baselines and for subsequent long-term monitoring. These metrics serve not only for final acceptance but also as data sources for “condition-based maintenance(CBM)” and “predictive maintenance(PdM)” as described in Section 3.7 of this proposal. When long-term monitoring data indicates sustained deviation from these baselines, preventive maintenance or troubleshooting procedures should be initiated.

| Indicators Category | Specific Metrics | Requirement | Measurement Method |
| --- | --- | --- | --- |
| System Availability | Overall System Availability | ≥ 99.5% | Monthly Statistics (Uptime/Total Time) |
| System Availability Communication Performance | Data Transmission Success Rate | ≥ 98% | Percentage of Successfully Transmitted Packets |
|  | Communication Latency (End-to-End) | ≤30 seconds (alarm type) | Simulation Testing and record End-to-End Latency |
| Data Integrity | Data Loss Rate | ≤ 1% | Compare sent and received packet counts |
| Control Reliability | Remote Command Execution Success Rate | ≥ 99% | Record control command issuance and execution results |
| Alarm Performance | Alarm Accuracy Rate | ≥ 98% | Simulate fault scenarios and evaluate alarm trigger accuracy |
| Power Supply Adaptability | Power Switchover Time | ≤ 10 seconds | Simulate main power failure and record backup power switchover time |
| Environmental Adaptability | Temperature/Humidity/Salt Spray Testing Passed | Meets Design Requirements | Conducted high/low temperature, humidity/heat, salt spray/corrosion, waterproofing, and vibration tests on field stations and base station equipment respectively, documenting their functional status and structural integrity. |

Table 2 key performance indicators

## 3.7.4 Testing Methods and contents

## Test Methods

• Simulation Testing: Use simulation platforms to replicate various faults and abnormal scenarios;

• Field Testing: Conduct functional and performance verification at actual AtoN sites;

• Stress Testing: Simulate high-concurrency data flows (e.g., simultaneous alarms from multiple sites) and command issuance scenarios;

•Long-Term Operational Testing: Maintain continuous system operation for over 30 days, recording stability metrics, memory leaks, performance drift, and related data.

## Test contents

Including but not limited to:

• Communication link testing: Simulate scenarios such as signal interruption, weak signals, and interference to validate link redundancy and recovery capabilities;

•  Data acquisition accuracy testing: Compare field measurements with system display values; errors should remain within the device's tolerance range;

• Remote control functionality testing: Verify the accuracy and security of control commands such as switching, adjustment, and reset;

• System redundancy and failover testing: Simulate primary equipment failure to validate the automatic switchover capability of the backup system;

• Power supply adaptability testing: Verify system stability under different power modes (mains, solar, battery);

Environmental adaptability testing: Includes high/low temperature, humidity/heat, salt spray, vibration, and other tests.

## Synergy with G1008 Remote Control and Monitoring of Marine Aids to Navigation Chapter 14: “Maintenance and Testing”

The performance monitoring activities defined in this chapter aim to provide decision-making support for the maintenance work described in G1008 Remote Control and Monitoring of Marine Aids to Navigation Chapter 14, forming a closed-loop improvement process:

1. Daily Monitoring and Data Collection: RCMS automatically records KPI data defined in “14.3 Key Performance Indicators” and generates monthly/quarterly performance reports.

2. Analysis and Diagnostics: Regularly evaluate monitoring data to identify abnormal trends or recurring failures. When monitoring data exceeds the concern thresholds defined in “14.3 Key Performance Indicators”, early warnings should be triggered.

• For example: A sustained decline in battery voltage may trigger the “condition-based maintenance(CBM)” procedure;

• For example: An increase in data packet loss rate may prompt inspection of the communication module.

3. Procedures and Execution:

• Minor/Potential Issues: IInitiate the routine maintenance procedures detailed in G1008 Remote Control and Monitoring of Marine Aids to Navigation Chapter 14;

• Critical/Urgent Issues: Initiate the breakdown / event maintenance procedures detailed in G1008 Remote Control and Monitoring of Marine Aids to Navigation Chapter 14;

4. Verification and Feedback: After maintenance or upgrades are completed, validate their effectiveness, update system performance baselines, and continuously optimize monitoring thresholds.

## 3.7.5 Long-Term Performance Monitoring and Evaluation

After system deployment, a continuous performance monitoring mechanism should be established, including:

• Monthly/Quarterly Performance Reports: Track KPIs such as system availability, communication performance, and alarm accuracy, comparing them against baseline metrics;

• Quarterly Health Assessments: Conduct trend analysis and root cause analysis of failures to identify potential systemic risks;

• Annual comprehensive assessment: Integrate user satisfaction surveys and system upgrade recommendations to produce an annual quality report;

• Incident-specific analysis: Conduct in-depth analysis of major or recurring failures, develop corrective actions, and track closure.

## 3.7.6  Acceptance Documentation Templates

It is recommended to provide the following templates in the appendix:

• Acceptance Test Plan: Defines test scope, methodology, resources, and schedule (planning phase prior to acceptance);

• Test Case Matrix: Documents test steps, expected outcomes, and actual results in detail (execution phase);

• Acceptance Report: Summarizes test results, facilitates acceptance decisions, and records outstanding issues (results consolidation and decision phase);

• Performance Evaluation Sheet: Used during long-term operational monitoring to track KPI achievement and trend analysis.

# References

- G1008 Remote Control and Monitoring of Marine Aids to Navigation (Edition 2.1, June 2009)

- G1182 Cybersecurity Guidelines from the International Association of Marine Aids to Navigation and Lighthouse Authorities

- Demonstration course L2 10.12 "Introduction to Remote Monitoring of AtoN"

# Action requested of the Committee

The ENG Committee is invited to consider the recommendations in the guideline “G1008 Remote Control and Monitoring of Marine Aids to Navigation” and take actions as appropriate.

## SQuality Assurance and Performance Improvement”

# Appendix A: System Performance Baseline Establishment Test Example

| Test Item | Test Method | Acceptance Criteria | Test Result | Remarks |
| --- | --- | --- | --- | --- |
| Data Acquisition | Simulate sensor outputs to verify data reception and display | Data reception rate ≥ 99.9% | □ Passed □ Failed |  |
| Alarm Triggering | Simulate fault signals to verify alarm generation and notification | Alarm delay ≤ 10 seconds | □ Passed □ Failed |  |
| Remote Control | Send commands such as switching, adjustment, and reset to verify execution results | Execution success rate ≥ 99% | □ Passed □ Failed |  |
| Communication Redundancy | Simulate primary link interruptions to verify backup link switchover time | Switching time ≤ 30 seconds | □ Passed □ Failed |  |
| Power Switching | Simulate primary power failures to verify backup power switchover time | Switching time ≤ 10 seconds | □ Passed □ Failed |  |

Form A-1 Functional Test Form

# Appendix B: System Performance Evaluation Indicator Examples

| Evaluation Items | Current Value | Target Value | Results | Trend Analysis |
| --- | --- | --- | --- | --- |
| System Availability Rate | 99.7% | ≥ 99.5% | □ Yes □ No | Stable |
| Data Transmission Success Rate | 98.5% | ≥98% | □ Yes □ No | Good |
| Data Packet Loss Rate | 0.8% | ≤ 1% | □ Yes □ No | Good |
| Remote Command Success Rate | 99.2% | ≥ 99% | □ Yes □ No | Good |
| Alarm Accuracy Rate | 98.8% | ≥ 98% | □ Yes □ No | False alarms require attention |

Form B-1 Monthly System Performance Evaluation Form