Input paper:   ENG22-3.1.1.9

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	X  ENG		□  PAP					X  Input

□  DTEC	□  VTS							□  Information

Agenda item 			n.n

Technical Domain / Task Number 2	Visual and Physical AtoN / Task 2.1.6

Author(s) / Submitter(s)			Naehyuk Yoo, Chungjin Lee (Korea Institute of Aids to Navigation)

# Use of Reference Light for Outdoor Photometric Measurements

# Summary

ENG Working Group 1 (WG1) is currently drafting a new guideline under Recommendation R0203, titled Measurement of Marine Lights Performance. Annex B of this draft guideline describes detailed outdoor telephotometry methods, which currently include two approaches.

• Outdoor light range measurements with a goniometer, and

• In situ lighthouse measurements using prisms to obtain vertical beam profiles.

These methods provide valuable means of determining the angular distribution and vertical beam profiles. In addition, there is scope to further strengthen the guideline by including a method that employs a calibrated reference light, traceable to national standards as a comparator.

## Purpose of the document

This input paper proposes to supplement Annex B by adding this third method: the use of a calibrated reference light positioned adjacent to the light under test. This method enables reliable determination of luminous intensity and effective intensity under outdoor conditions, particularly where long-distance telephotometry may be affected by atmospheric conditions or low detector sensitivity.

## Related documents

IALA Recommendation R0203 Definition of marine signal lights terms and measurement

# Discussion

The two methods currently described in Annex B address angular displacement measurements and beam profile analysis. A further complementary method is to determine peak intensity and effective intensity by comparison with a reference light.

The proposed method involves:

Installing a reference light of known luminous intensity, calibrated against national standards, in close proximity to marine light under test.

Conducting measurements at a fixed observation distance (typically several nautical miles), where both the light under test and the reference light are alternately observed using a photometer or luminance meter.

Calculating the luminous intensity of the lighthouse light by comparing its measured signal to that of the reference light, using the ratio method.

Deriving the effective intensity of flashing lights using an established formula (e.g. Schmidt-Clausen method).

This approach provides an additional benefit in field conditions:

It reduces reliance on highly sensitive photometers at extreme distances.

It offers a practical solution for routine inspection of large lighthouse optics.

It aligns with good practice by using a nationally traceable reference source.

It is therefore proposed that the guideline be broadened to include this third outdoor photometry option, alongside the two existing methods.

## Measurement Method

### Measurement site

The measurement site shall be selected at an appropriate distance, typically several nautical miles, and always beyond the crossover distance of the light under test, taking into account its luminous intensity. The site may be adjusted according to weather, sea conditions, and the height of the light. For improved accuracy, land-based sites may be chosen when feasible, considering local conditions and accessibility.

### Measurement bearing

Prior to measurement, the measurement team shall determine and communicate the measurement site and bearing to the light operation team. When measurements are taken from a vessel underway, this information may be transmitted via radio communication. During the measurement, the reference position and the vessel's track shall be continuously monitored to ensure correct distance and bearing, and all details shall be recorded.

### Measurement operations

Before starting, the measurement team shall install all equipment, check wiring and operation status, and conduct a trial run until stable operation is ensured.

The luminance meter and light signal waveform analysis system shall be installed so that the optical axis of the instruments is positioned at a consistent height above ground level, typically at eye level.

The photomultiplier tube (PMT) of the receiver shall be pre-observed for an appropriate period prior to measurement to stabilize its output to the specified level.

Before commencing, the date, measurement distance, weather, visibility, bearing, and equipment operation status shall be recorded.

During measurement, the first detection of the light signal, the observed bearing, and the measured distance from the reference position shall be recorded.

The lens number of the lighthouse optic responsible for each flash shall be recorded.

Care shall be taken to maintain constant PMT sensitivity. If atmospheric transmittance changes cause a drop in the reference value, the PMT shall be readjusted and the measurement at that point repeated.

At each site, sufficient data shall be collected to ensure statistical reliability. As a guideline, multiple sets of data should be obtained at each site (one "set" being defined as capturing both the lighthouse light and the reference light through the optical axis of the luminance meter). The exact number of sets may be determined according to national practice and measurement objectives.

### Installation of the reference light

  - Prior to measurement, the light operation team shall replace the lighthouse lamp with the test lamp (including ballast, if discharge type) and ensure that the light source center coincides with the optic focal point.

  - The reference light shall be installed horizontally on the gallery floor or nearby structure or another suitable structure, in a position that does not overlap with the lighthouse light.

  - Before measurement, all equipment shall be installed, wiring and operation checked, and trial operation performed until stable.

  - The lighthouse light and the reference light shall be switched on at rated voltage before measurement to stabilize their characteristics.

  - The rotation period of the reference light shall be adjusted so that its flashes can be alternately measured with those of the lighthouse light.

  - During measurement, the light operation team shall inform the measurement team whether the observed flash originates from the lighthouse optic or from the reference light.

  - Both the lighthouse lamp and the reference lamp shall be continuously powered at rated voltage during the measurement.

  - After completion, the operation team shall dismantle the equipment and verify that the lighthouse light functions normally.

Reference Light in Korea

### Data analysis

Measurement data shall be analysed with dedicated software. For each lens panel of the lighthouse optic and for the reference light, the peak intensity, mean intensity, effective intensity, flash duration, form factor, and luminous intensity waveform shall be processed and tabulated.

## Calculation Method

The peak intensity of the lighthouse light is determined by comparing its measured peak signal with that of the reference light, according to the following ratio:

Io=Iref×YX

Where:

Io: Peak intensity of the lighthouse light

Iref: Known luminous intensity of the reference light (calibrated to national standards)

X: Peak signal of the reference light (V)

Y: Peak signal of the lighthouse light (V)

The effective intensity of flashing lights is then calculated using the Schmidt-Clausen formula:

Ie=τ×Io(C/F)+τ

Where:

Ie: Effective intensity

τ : Flash duration

C : Observer's time constant

F : Form factor of the flash profile

# References

Draft new guideline on the Measurement of Marine Lights Performance

# Action requested of the Committee

The Committee is requested to:

- Consider the addition of a third outdoor photometric method to the draft new guideline on the Measurement of Marine Lights Performance.

Provide guidance on how this method may be incorporated and standardised within Recommendation R0203.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain