Input paper:   ENG22-3.1.2.1

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	X  ENG		□  PAP					□  Input

□  DTEC	□  VTS							X  Information

Agenda item 			n.n

Technical Domain / Task Number 2	n.n

Author(s) / Submitter(s)			Inmaculada Armengol (GMV), Héctor Llorca (GMV), Marcos López (GMV), Marcelo Meneses (GMV), Adrián Blanco (GMV), Carmen Aguilera (EUSPA), Karel Callewaert (EUSPA), Ciro Gioia (EC)

# Galileo High Accuracy (HAS) enabled maritime & IWW receiver (ISLET): objective, activities and expected added value

# Summary

ISLET (Galileo HAS enabled maritime receiver) project has been launched by the EUSPA with the main objective of designing, developing, testing and demonstrating a multi-constellation (Galileo and GPS) and multi-frequency (E1/E5, L1/L5, E6 bands) maritime and IWW (Inland Waterways) receiver that integrates Galileo HAS (High Accuracy Service) and is suitable to be embedded within the vessel navigation equipment. The receiver will also include the implementation of OSNMA (Open Service Navigation Message Authentication) processing capabilities.

This document aims to present the project, including its main objectives, the target applications, the solutions to be developed, the different verification and validation activities, and the expected added value.

For additional information about the ISLET project please contact Carmen Aguilera (EUSPA) and Inmaculada Armengol (GMV).

## Purpose of the document

The objective of this paper is to make IALA ENG Committee members aware of the main objectives of EUSPA’s ISLET project, currently under development by a GMV led consortium, including its activities, as well as the main added value that the project outcomes are expected to provide to the maritime and IWW community and IALA members.

# Background

## Introduction to the ISLET project

The aim of the ISLET project is to develop a Galileo HAS enabled receiver for Maritime and IWW navigation. It is co-funded by EUSPA in the context of its Fundamental Elements Scheme, and materialized in a consortium led by GMV Aerospace and Defence (GMV), in partnership with:

- Teledyne FLIR Transponder Tech AB (FLIR, formerly known as SAAB Transpondertech AB)

- GMV Innovating Solutions SRL

- Tresco Engineering BV

This enterprise, which kicked off back in June 2025 and will have a duration of two years, has just overcome the critical design review milestone (CDR, T0+9) and is fully immersed in the development phase, as portrayed in the project planning below.

ISLET project high level planning.

The project solutions will include multi-frequency and multi-constellation capabilities, exploiting new GNSS differentiators such as Galileo HAS and OSNMA for accurate and resilient navigation in the maritime and IWW domains. They will provide an alternative without some of the challenges that other solutions like RTK (Real-Time Kinematics), EGNOS (European Geostationary Navigation Overlay Service) or radio beacons DGPS (Differential GPS) face in terms of coverage or the need for additional resources.

Besides the activities associated with the own development of the solution (i.e. design, testing) and business or dissemination, the project will also work on standardization for the introduction of high accuracy in maritime and IWW standards, identifying gaps and opportunities and engaging in relevant working groups from recognized organizations.

## Introduction to Galileo HAS

The introduction of Galileo HAS is one of the latest innovations that the Galileo system currently provides in the context of GNSS. Galileo HAS is a global positioning enhancement service that improves positioning accuracy for a wide range of users and industries, compared to Open Service (OS) GNSS.

Galileo HAS enables Precise Point Positioning (PPP) worldwide by providing corrections to orbits, clock and measurement biases through the E6-B signal and via Internet access. Unlike other private products, in this case the service is provided free of charge. In addition, Galileo HAS also provides a reference algorithm to reach the target performance, which is a horizontal accuracy of around 20 cm (95%) and a vertical accuracy of about 40 cm (95%) in nominal conditions, beyond the meter-level accuracy offered by OS GNSS services. Additionally, not only broadcasting directly through Galileo satellite signals, but also providing accessibility over the internet, gives users flexibility in how they choose to use the service.

The introduction of Galileo HAS offers advantages in a set of use cases that were previously reserved for private PPP services, Differential GNSS (DGNSS) and other systems beyond standalone GNSS. Table 1 includes a comprehensive summary of competitor technologies, current applications and main challenges. These new use cases span a large number of industries, from agriculture to drones, logistics or, in ISLET’s case, maritime and IWW navigation.

GNSS has been widely used in the maritime and IWW industries for many years now. For instance, IMO recognised Galileo as part of WWRNS (World-Wide Radio Navigation System) back in 2016. For maritime applications, standards for OS services are already available (IEC-61108). However, while Galileo OS can provide a valid positioning option for many applications, there are certain use cases that require higher accuracy than what the baseline services can provide. Although there are many reliable alternatives for this kind of service, most of them make use of a combination of one or more GNSS constellations plus an additional system, which may be a paid service or a separate infrastructure, unlike Galileo with its High Accuracy Service.

Galileo HAS may outperform baseline GNSS receivers at a fraction of the cost and difficulty of implementation with respect to other high accuracy alternatives, while offering worldwide coverage. These features make Galileo HAS an asset for the use cases that will be tested throughout the ISLET project.

Competitor Technologies & Challenges

| Competitor Technology | Description/Summary | Preliminary Challenges | How Gal HAS service overcomes challenges |
| --- | --- | --- | --- |
| RTK | RTK positioning service provides cm-level accuracy in certain geographic coordinates using GNSS satellite signals & ground-based reference stations. | Very limited coverage & based on land infrastructure. | Worldwide coverage on SiS (see the note below). |
| Radio beacon DGPS | DGPS beacon service enhances the accuracy of standard GPS signals by providing real-time corrections through a network of ground-based radio beacons. Widely used in maritime. | Limited coverage, based on land infrastructure. Best case Accuracy 0.5m. | Worldwide coverage on SiS (see the note below). Horizontal accuracy to dm order. |
| EGNOS | The European Geostationary Navigation Overlay Service (EGNOS) is a satellite-based augmentation system that enhances the accuracy and reliability of GPS signals across Europe. It provides real-time corrections and integrity information. | Limited coverage in urban areas and higher latitudes. Best case Accuracy 0.5m. | Worldwide coverage on SiS (see the note below). Horizontal accuracy to dm order. |
| Private satellite corrections provider | Private satellite correction services provide high-accuracy GNSS positioning by delivering real-time corrections through a network of satellites and ground-based reference stations. It should be noted that the private sector enables centimeter-level positioning accuracy, whereas Galileo HAS targets decimeter-level performance. | Pay-per-use scheme and expensive. | Free of charge use. |
| Public satellite high accuracy corrections | High accuracy correction services available as additional functionalities of public GNSS constellations. Main examples are MADOCA (JAXA) and PPP-B2b (BeiDou) | Limited coverage in some areas | Worldwide coverage on SiS. |

# Target applications

The ISLET project is conceived to develop solutions to support a wide range of maritime and IWW applications that require a high level of positioning accuracy in navigation. After a thorough assessment of the state of the art and user requirements and demands for different high accuracy use cases, target applications have been identified both in maritime and IWW fields.

The design and development of the ISLET solutions will be focused on meeting the requirements of these selected applications. Nevertheless, it should be noted that this does not mean that the developed solutions can be used for other Maritime and IWW applications.

## Maritime applications

In the maritime field, two different applications have been selected: hydrography (bathymetry) and buoyage/AtoN (Aids to Navigation) placement.

Hydrography (bathymetry)

Hydrography applications are used to collect, manage and analyse data about the physical features of oceans, seas, coastal and inland waters. Bathymetry involves measuring the depth of water in oceans, seas, lakes, and rivers. It provides crucial information about underwater topography. Bathymetric data resemble topographic maps for ENC, using contour lines to connect points of equal depth (isobaths). This bathymetric information helps skippers, captains and pilots safely navigate water, particularly in ports to avoid grounding.

In terms of user requirements, since it is not a safety-of-life application, the requirements for using equipment for these applications are based primarily on their compliance with accuracy specifications (decimetric level), no specific certificate is required. In terms of integrity, only information quality monitoring is needed.

Hydrography was identified as a suitable application for ISLET as its requirements may be covered by developing a solution that integrates Galileo HAS into a PPP algorithm and includes the monitoring of navigation information quality. This application will be addressed by GMV and FLIR and takes form in the shape of ISLET Receiver-P (see section 4.1).

The added value is not just in terms of high accuracy, but also because alternative positioning technologies may be unavailable or unreliable in certain conditions. For instance, DGPS and RTK solutions become ineffective in offshore environments, where the distance from coastlines and reference stations increases significantly or when internet connection is unavailable in remote regions while Galileo HAS Signal in Space remains available.

Buoyage/AtoN placement

These operations involve the planning, deployment and maintenance of buoys, beacons, lights and other navigational aids that guide vessels safely though waterways. This application ensures channels, hazards and restricted areas are clearly marked, based on traffic needs and hydrographic conditions.

Similarly to hydrography, it is not a safety-of-life application and the driver for requirement definition is compliance with accuracy specifications and the monitoring of navigation information quality. Hence, this application will be covered by the same solution: ISLET Receiver-P (see section 4.1).

## IWW applications

For inland waterways, the chosen application is navigation on small narrow channels with some automation level. The space available for navigation on IWW is generally more limited than in maritime applications. Navigating on small narrow channels presents unique challenges due to the reduced fairway width and close ship encounters. TGAIN (steering control) and collision avoidance systems can be helpful, however they should be used with caution in this application, due to limited awareness and restricted manoeuvrability. For these reasons, performing this operation requires more accurate positioning systems that allow for better adjustment of manoeuvres. And especially when this operation is performed with some automation level (the accuracy offered by systems such as DGPS or EGNOS beacons is not sufficient).

In terms of user requirements, positioning accuracy (metric) is not as demanding as the maritime applications described in the previous section. Oppositely, in this case users have a greater need for operational integrity, valuing features such as an integrity status indicator based on protection levels. Concerning regulation, at the technical level of equipment, the ES-TRIN regulation specifies that if an external GNSS receiver is connected to equipment such as ECDIS, it must have type approval from the IEC 61108 series.

This use case was identified as a suitable application for ISLET as its requirements may be covered by developing a solution that integrates Galileo HAS into a Single Point Positioning (SPP) algorithm and includes operational integrity (through the IBPL algorithm). This application will be addressed by GMV and Tresco and takes form in the shape of ISLET Receiver-S (see section 4.2).

Alike the maritime case, the added value is not just in terms of accuracy. Systems such as RTK-based land corrections could achieve the required level of accuracy, but the coverage offered is limited. In addition, rivers are highly dynamic, and narrow areas may appear at many different points along their course. This would entail expenditure and investment in land infrastructure, complicating the use of these systems. Considering the characteristics of this application, it seems very interesting to have a satellite-based system such as Galileo HAS.

Moreover, in the IWW domain, users are price-sensitive and hence may be more interested in paying a small delta for a Galileo HAS enabled receiver (and then profiting from the service for free) as opposed to constantly having to rely on expensive RTK corrections or other private services. This presents a very interesting opportunity to take advantage of ISLET Receiver-S. Since the expected accuracy with this mode would be sufficient to meet the accuracy requirements and, at the same time, allow for a more cost-effective solution than a receiver with the PPP and Galileo HAS.

# ISLET receiver solutions

To cover the aforementioned applications, two different solutions have been designed and are introduced in this section: ISLET Receiver-P and ISLET Receiver-S.

## ISLET Receiver-P

ISLET Receiver-P (IR-P) is the solution that targets the maritime applications (hydrography and AtoN placement operations) and implements an Operational Mode (OM) that consists of a PPP algorithm with Galileo HAS and information quality monitoring. This operational mode has been defined as the PPP OM.

The PPP OM leverages the Galileo HAS corrections obtained through Galileo E6 SiS. These corrections are fed, along with GPS and Galileo signals, into a PPP navigation library that implements a Kalman filter to obtain high accuracy positioning. It will be suited for the maritime target applications, as they require high levels of accuracy (in the order of decimetres).

The design of the ISLET Receiver-P is based on a modular approach, with its main components being:

GNSS antenna.

GMV OEM GNSS receiver: GMV will develop a multi-constellation (GPS + Galileo), triple-band (E1, E5, E6 and L1, L2, L5) OEM GNSS receiver. The aim is to build on the knowledge and developments acquired by GMV in the Asgard project, integrating the PPP navigation library with Galileo HAS capabilities.

FLIR NAV Sensor: FLIR’s next generation navigator platform is derived from the proven FLIR R5 NAV Mk III. The navigator hardware and software will be refined iteratively, with the current design focused on providing an IMO compliant platform suitable for professional maritime environments. Multiple configuration options remain planned to support varying installation conditions and operational requirements.

Control and Display Unit (CDU): it is based on the FLIR CDU architecture with its existing hardware and an ISLET-specific firmware. The firmware development continues to emphasize reliability, intuitive operator interaction, and support for required maritime functions and workflows, aligned with the broader system design.

ISLET Receiver-P design scheme.

## ISLET Receiver-S

ISLET Receiver-S (IR-S) is the solution that targets the IWW application (navigation on small narrow channels with some automation level) and implements an Operational Mode (OM) that consists of an SPP algorithm with Galileo HAS and operational integrity via GMV solution IBPL. This operational mode has been defined as the SPP OM.

The SPP OM also leverages the Galileo HAS corrections obtained through Galileo E6 SiS. In this case, these corrections are fed, along with GPS and Galileo signals, into an SPP navigation library. It will be suited for the IWW target application, as it requires meter level accuracy, which is less demanding compared to maritime target applications.

The design of the ISLET Receiver-S is based on a modular approach, with its main components being:

- GNSS antenna.

GMV OEM GNSS receiver: in this case including the SPP navigation library instead of the PPP.

GMV enclosure, with a service-friendly design to feature the corresponding interfaces and simplify deployment and maintenance.

Tresco ECDIS: a PC running Tresco Navigis will be connected to the GMV OEM GNSS receiver through the enclosure. This SW has been developed in compliance with the latest Inland ECDIS standards defined by CESNI, in line with the most recent version of ES-RIS.

ISLET Receiver-S design scheme.

# Validation activities, including real operations

The high-level V&V (Verification and Validation) approach of the project is presented in Table 2. Verification will start by verifying each individual component going into the ISLET receivers on its own (FLIR NAV Sensor, GMV OEM GNSS Receiver, CDU, ECDIS). Once the components have been verified and judged to work as expected, they will be integrated into each prototype. In the case of ISLET Receiver-P, it will be integrated by including the GMV OEM GNSS Receiver on FLIR NAV Sensor. Then it will be coupled with the CDU, completing the IR-P prototype. As for ISLET Receiver-S, integration is achieved by including the GMV OEM GNSS Receiver in GMV enclosure, and after that connecting it to TRESCO PC with ECDIS installed.

After integration of the individual components into the two ISLET Receivers, the integration and unit verification will take place to ensure that the components are working as expected after integration. In parallel to this activity, environmental tests will be performed on the OEM GNSS Receiver. Additionally, at the same time, preliminary performance tests will be carried out to confirm that performance is as expected.

Once the initial verification testing has been performed, the results have been analysed and the ISLET receivers are deemed ready for validation tests, the verification phase can be considered concluded, and the validation testing phase can be started.

One of the main objectives of the validation activities is to validate the requirements of the ISLET receivers. This will mainly be done through a performance laboratory campaign in which the ISLET receivers will be analysed both in static and dynamic settings. Once the equipment has been validated at the laboratory, a benchmarking campaign will be carried out with other high accuracy technologies in order to compare the performance with other solutions.

Finally, dynamic campaigns will be carried out to test ISLET receivers in environments close to real operations, on board of vessels. Two campaigns have been planned: one focused on the target maritime applications, and the other focused on the target IWW application.

It is worth mentioning that, after the V&V activities have concluded successfully, a final demonstration is foreseen to showcase the performance of the developed solutions.

ISLET high level verification and validation approach

| Phase | Type of tests | Component tested | Premises/Location |
| --- | --- | --- | --- |
| Verification | Integration Test campaign | GNSS receiver board Sensor board Chassis and connectors Control and display unit | GMV, FLIR and TRESCO facilities |
|  | Environmental tests | GNSS receiver board | GMV Laboratory |
|  | Preliminary Performance tests | GNSS receiver board | GMV and FLIR Laboratories |
| Validation | Requirement and performance - Static & dynamic laboratory testing | ISLET receiver | GMV Laboratories (Spain and Romania) |
|  | Benchmarking test campaign in the laboratory | ISLET receiver | GMV Laboratories (Spain and Romania) |
|  | Dynamic test campaign on a vessel | ISLET receiver-P | Focused on Hydrographic application: Hydrographic vessel and/or RoRo Ferry vessel. (Spanish and/or Sweden/Denmark sea) |
|  |  | ISLET receiver-S | Focused on Inland application: Merchant Inland vessel / remote control vessel (Belgium and/or Netherland Inland waterways) |

# Expected added value of project outcomes

It is expected that the ISLET project will fulfil its requirements of developing a new multi-constellation, multi-frequency, Galileo HAS and OSNMA enabled solution suitable to be embedded within the vessel navigation equipment as the main positioning source.

It is important to point out that the implementation and exploitation of Galileo HAS processing capabilities is expected to be a key added value for some maritime and IWW domains, as many applications demand high accuracy positioning for navigation. Currently, alternative technologies lack the availability of a service with the coverage (worldwide) and price (free of charge) of Galileo HAS, making the approach of ISLET receivers an interesting solution for those types of applications.

# References

- N/A

# Action requested of the Committee

The Committee is requested to:

- Note the information within this paper,

- provide the opportunity of briefly presenting this paper at ENG22 and discussing the matter at an appropriate time, and

include the proposed paper into the IALA documentation if so considered by the discussion.

# ANNEX – Acronyms

Acronyms

| Acronym | Definition |
| --- | --- |
| ARM | Aids to Navigation Requirements and Management |
| CDR | Critical Design Review |
| CDU | Control and Display Unit |
| CESNI | European Committee for Drawing up Standards in the field of Inland Navigation |
| DGNSS | Differential GNSS |
| DGPS | Differential GPS |
| DTEC | Digital Technologies Committee |
| EC | European Commission |
| ECDIS | Electronic Chart Display and Information Systems |
| EGNOS | European Geostationary Navigation Overlay Service |
| ENC | Electronic Navigation Chart |
| EUSPA | European Union Agency for the Space Programme |
| GNSS | Global Navigation Systems |
| GPS | Global Positioning System |
| HAS | High Accuracy Service |
| IALA | International Organization for Marine Aids to Navigation |
| IBPL | Isotropy-Based Protection Level |
| IEC | International Electrotechnical Commission |
| IMO | International Maritime Organization |
| IR-P | ISLET Receiver-P |
| IR-S | ISLET Receiver-S |
| ISLET | galIleo haS enabLEd mariTime receiver |
| IWW | Inland Waterways |
| JAXA | Japanese Aerospace Exploration Agency |
| MADOCA | Multi-GNSS Advanced Demonstration tool for Orbit and Clock Analysis |
| OEM | Original Equipment Manufacturer |
| OM | Operational Mode |
| OS | Open Service |
| OSNMA | Open Service Navigation Message Authentication |
| PAP | Policy Advisory Panel |
| PC | Personal Computer |
| PPP | Precise Point Positioning |
| PVT | Position, Velocity and Time |
| ES-RIS | European Standard for River Information Services |
| RoRo | Roll-on/roll-off |
| RTK | Real-Time Kinematics |
| SPP | Single Point Positioning |
| SiS | Signal in Space |
| TGAIN | Track Guidance Assistant for Inland Navigation |
| TRIN | Technical Requirements for Inland Navigation Vessels |
| VTS | Vessel Traffic Service |
| V&V | Verification & Validation |
| WWRNS | World-Wide Radio Navigation System |

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain

Galileo_HAS_SDD_v1.0.pdf