Input paper:   ENG22-3.1.2.13

Input paper for the following Committee(s): 	                              		Purpose of paper:

□  ARM	X  ENG		□  PAP					X  Input

□  DTEC	□  VTS							□  Information

Agenda item 			n.n

Technical Domain / Task Number 2	Radionavigation services / 7.1.1, 7.1.2

Author(s) / Submitter(s)			Younghoon Han, Sewoong OH, Tae Hyun Fang, Kiyeol Seo (KRISO), Woo Gyoung Park, Seung-Cheol Lee (MOF)

# Draft data model design of the S-241 PNT station almanac

# Summary

Following the decision to integrate S-200 series product specifications in the PNT domain, this paper proposes a draft data model for the S-241 PNT Station Almanac. The draft integrates the existing S-240, S-246, and S-247 data models and incorporates newly identified R-Mode station almanac. The Committee is invited to review the development of the S-241 data model and to discuss the newly included R-Mode-related station information.

## Purpose of the document

To share the draft S-241 data model and to discuss the newly added R-Mode station almanac as well as next development step.

## Related documents

S-100	IHO Universal Hydrographic Data Model (Edition 5.2.0, June 2024)

S-97	IHO Guidelines for Creating S-100 Product Specifications (Edition 1.1.0, June 2020)

S-240	DGNSS Station Almanac

S-246	eLoran Transmitting Station Almanac

S-247	Differential eLoran Reference Station Almanac

ENG21-3.1.2.7 Proposal for the Design Concept of an Integrated Data Model for S-200 PNT Station Almanac

# Background

The IALA ENG Committee has developed test versions (Edition 1.0) of S-200 series product specifications for the exchange of maritime information in the PNT domain. Regarding PNT Station Almanac, S-240, S-246, and S-247 were developed. However, considering similarities among product specifications and the efficiency of maintenance, it was decided to integrate the data models in the PNT domain. S-241 has been identified as the integrated product specification for PNT station almanac data, and its development is currently underway.

# Discussion

## Status and scope of PNT station almanac

IALA developed test versions (Edition 1.0) of S-240, S-246, and S-247 for the production and exchange of station information related to DGNSS and eLoran within the S-200 PNT framework. Subsequently, R-Mode station information was newly identified as part of PNT station almanac data. Accordingly, S-241 will include not only existing DGNSS and eLoran reference station information but also R-Mode station information. R-Mode stations are categorized as MF R-Mode, VDE-TER R-Mode, ASM-TER R-Mode. Table 1 presents the list of PNT reference stations to be covered in S-241. The Committee is requested to review the terminology.

S-241 PNT station almanac list

| System | Station | Remarks |
| --- | --- | --- |
| DGNSS | DGNSS station(S-240) |  |
| eLoran | eLoran transmitting station(S-246) |  |
|  | Differential eLoran reference station(S-247) |  |
| MF R-Mode | MF R-Mode transmitting station | DGNSS station is jointly utilized |
|  | Differential MF R-Mode reference station |  |
|  | MF R-Mode Monitor Station |  |
| VDE-TER R-Mode | VDE-TER R-Mode transmitting station |  |
| ASM-TER R-Mode | ASM-TER R-Mode transmitting station |  |

## Identify the R-Mode station almanac data

R-Mode reference stations are classified into MF R-Mode, VDE-TER R-Mode, and ASM-TER R-Mode. MF R-Mode consists of; Transmitting station, Differential MF R-Mode station and Monitor station. Draft proposals for detailed data elements to be included in the MF R-Mode station almanac and VDE-TER R-Mode station almanac are provided in Table 2 and Table 3, respectively. The Committee is requested to review these proposals. In addition, cooperation from the Committee is requested for the development of ASM-TER almanac data.

Draft S-241 MF R-Mode station almanac data spread sheet

| Category | Data | Description | Format | Unit |
| --- | --- | --- | --- | --- |
| MF R-Mode Transmitting Station Almanac | Version | Version of the reference |  |  |
|  | Name of issuing organization | Name of issuing organization who provide the data |  |  |
|  | Issue number of data |  |  |  |
|  | Date of issue |  | YYYY,MM,DD |  |
|  | Country |  |  |  |
|  | Transmitting Station name |  |  |  |
|  | Transmitting Station ID |  |  |  |
|  | Geographical position | The latitude and longitude of the transmitting station antenna position to 1cm precision |  | degree(eight decimal places) |
|  | Nominal range |  |  |  |
|  | Signal power | Signal power of transmitting station |  | kW |
|  | Station health | Normal = 1, Not monitored = 2, No information available = 3, Do not use = 0 |  |  |
|  | Status | Only DGNSS available = 1, Only R-Mode available = 2, DGNSS and R-Mode available = 3, Not available = 0, Testing = 4 |  |  |
|  | Navigation message types | R-Mode navigation message type 55, submessage 1 to 6, No message = 0 |  |  |
|  | Bit rate |  |  | bps |
|  | Signal Frequency |  |  | kHz |
|  | dR-Mode station ID |  |  |  |
|  | Monitor station ID |  |  |  |
|  | CW frequency spacing | CW frequency spacing from the MSK signal frequency |  | Hz |
|  | CW frequency status | Low CW available = 1, High CW available = 2 , Low and High available = 3, Not available = 0 |  |  |
|  | Available bandwidth |  |  | kHz |
|  | Power ratio of CW and MSK (CW1/MSK/CW2) | Sum of the ratio to be 1 | X,X,X |  |
|  | RMST source | UTC = 1, GNSS = 2, eLoran = 3, Other = 4 |  |  |
| MF R-Mode Monitor Station Almanac | Version | Version of the reference |  |  |
|  | Name of issuing organization | Name of issuing organization who provide the data |  |  |
|  | Issue number of data |  |  |  |
|  | Date of issue |  | YYYY,MM,DD |  |
|  | Country |  |  |  |
|  | Monitor Station name |  |  |  |
|  | Monitor Station ID |  |  |  |
|  | Geographical position | The latitude and longitude of the transmitter position to 1cm precision |  | degree(eight decimal places) |
|  | Status | Station available = 1, Not available = 0 |  |  |
| Differential MF R-Mode reference station Almanac | Version | Version of the reference |  |  |
|  | Name of region |  |  |  |
|  | Name of issuing organization | Name of issuing organization who provide the data |  |  |
|  | Issue number of data |  |  |  |
|  | Date of issue |  | YYYY,MM,DD |  |
|  | Name of dR-Mode station | The name of the associated dLoran |  |  |
|  | dR-Mode station ID |  |  |  |
|  | Number of ASF maps | The number of ASF maps served by the dR-Mode |  |  |
|  | ID numbers of ASF maps | The ID numbers of the ASF maps served by the dR-Mode | XXXX,XXXX, … |  |
|  | dLoran position | The latitude and longitude of the dLoran position to 1mm precision | latitude, longitude | degree(eight decimal places) |
|  | Number of transmitters | The number of transmitters served by the dR-Mode |  |  |
|  | Transmitting Station ID |  | XXX,XXX, … |  |
|  | Nominal ASF value | The nominal ASF value of the transmitting station | XX.XX,XX.XX, … | Microseconds(two decimal places) |
|  | Status | Station available = 1, Not available = 0 |  |  |

Draft S-241 VDE-TER R-Mode station almanac data spread sheet

| Category | Data | Description | Format | Unit |
| --- | --- | --- | --- | --- |
| VDE-TER R-Mode Transmitting Station Almanac | Version | Version of the reference |  |  |
|  | Name of issuing organization | Name of issuing organization who provide the data |  |  |
|  | Issue number of data |  |  |  |
|  | Date of issue |  | YYYY,MM,DD |  |
|  | Region |  |  |  |
|  | Transmitting Station name |  |  |  |
|  | Transmitting Station ID |  |  |  |
|  | Geographical position(horizontal) | The latitude and longitude of the transmitting station antenna position to 1cm precision |  | degree(eight decimal places) |
|  | Geographical position(vertical) | The height of the transmitting station antenna position to 1cm precision |  | meters(two decimal places) |
|  | Status | Available = 1, Not available = 0 |  |  |
|  | Signal power | Signal power of transmitter |  | W |
|  | Signal Frequency |  |  | MHz |
|  | Available bandwidth |  |  | MHz |
|  | Navigation message types | R-Mode navigation message types 1~6, No message = 0 |  |  |
|  | γfactor of ranging sequences | 1 = 0 , 0.7 = 1, 0.3 = 2, 0 = 3 |  |  |
|  | Navigation message interval |  |  |  |
|  | RMST source | UTC = 1, GNSS = 2, eLoran = 3, Other = 4 |  |  |

## Draft S-241 PNT station almanac data model

A data model is required that can accommodate MF R-Mode, VDE-TER R-Mode, and ASM-TER R-Mode in addition to DGNSS and eLoran. To define common attributes of PNT stations; A PNT Station Almanac feature type has been introduced. Each PNT station inherits these common attributes.

Furthermore, an eLoran Station Almanac feature type was defined to share common attributes of eLoran stations. An MF R-Mode Station Almanac feature type was defined to share common attributes of MF R-Mode stations.

In existing S-240, S-245, and S-246 data models, the Radio Station feature type is linked to PNT station information. However, in the S-241 data model, each PNT station type is defined as a feature type that directly includes coordinate information. Based on this approach, the draft S-241 data model has been developed, as illustrated in Figure 1.

Proposed S-241 data model

At ENG22, it is proposed to:

- Review the draft S-241 data model: feature classification and inheritance/association relationships of the PNT Station Almanac

- Review the prosed R-Mode station almanac data

The R-Mode station almanac data will be finalized through Committee discussions and then incorporated into the data model.

# Action requested of the Committee

The Committee is requested to consider this input paper, and take actions as appropriate.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain