Input paper:   ENG22-3.1.2.2

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	X  ENG		□  PAP					□  Input

□  DTEC	□  VTS							□  Information

Agenda item 			n.n

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)			Andrea Sánchez (GMV), Héctor Llorca (GMV), Marcos López (GMV), Marta Cueto (GMV), Florin Mistrapau (GMV), Daniel Karbach (WSV), Yves Hacha (TRESCO), Falk Bethke (BM), Csaba Hargitai (HAC), Gergely Mező (RSOE), Carmen Aguilera (EUSPA), Marta Wolska (EC)

# GNSS & EO Requirements for Automated Inland Waterways Navigation: Gap Analysis and Pilot Tests Preparation

# Summary

This paper aims to provide a summary of the activities performed in the AVIS project, starting with an analysis of the user requirements established for different GNSS and Earth Observation services and a comparison with their current performance. This paper also shows the implementation process of the prototype that will be used in future pilots to extract navigation data for analysis, with a preliminary round of laboratory results for the different services considered. The planning and definition of four different pilots is also presented.

## Purpose of the document

The objective of this paper is to make IALA ENG Committee members aware of the progress of the AVIS project in the prototype implementation based on the GNSS and Earth Observation requirements previously identified and the preparation of the pilots that will serve as a basis to obtain navigation data to study the added value of EU Space Data for automated navigation on Inland Waterways.

## Related documents

- ENG20 input paper: “GNSS & EO Requirements for Automated Inland Waterways Navigation”, presented at the 20th ENG Committee Meeting, IALA, April 2025, ENG20-3.1.2.1.

# Background

The Automated Vessels on European Inland Waterways (AVIS) project, fully funded by the European Union, aims to analyse how the use of European Union’s space systems (such as EGNSS and Copernicus Earth Observation) could improve the accuracy and reliability of navigation and forecasting in European IWW for Automated Vessels. This analysis includes the definition, design and implementation of a safety prototype, which will be used in four pilots across different EU IWW corridors. All the information and data provided in this paper results from this project.

AVIS is formed by a consortium led by GMV, a company that has relevant experience in GNSS and Earth Observation systems. To perform the best possible study, this consortium also integrates some of the most important experts in the IWW sector in the form of national inland authorities and private companies such as WSV, TRESCO, BM, HAC, RSOE and EY.

# introduction to GNSS and Copernicus requirements

Inland waterway transport is a key component of the transportation of goods, cargo, and people within Europe. It is a world that has experienced important transformations in recent years due to new technologies being introduced, which have advanced the digitalisation of the field. With the idea of autonomous vessels being as close as ever, there is a need to improve the reliability and accuracy of the navigation systems used for inland waterway transportation.

Modern navigation in inland waterways relies on Global Navigation Satellite Systems (GNSS) like GPS and Galileo, which are essential in the positioning of vessels. On top of that, Earth Observation technologies can be used to provide alerts to the navigation and update the electronic nautical charts. As such, the AVIS project aims to integrate European GNSS and the Copernicus program for additional safety, accuracy, and reliability in the navigation of autonomous vessels on European IWW.

The information shown in this paper builds upon the requirement definition and analysis performed in a previously presented study (See related documents). In this study, requirements were established based on the IWW navigation operations selected and the degree of automation that was aimed for.

Based on the levels established by the CCNR [3], the realistic target for AVIS was determined to be an Automation Level (AL) 3, which is defined as the level in which a vessel should be able to perform all the dynamic navigation tasks expected of an autonomous vessel, but where a crew is still expected on board to monitor and intervene in case of failure.

- Several parameters were also identified as key for this requirement definition based on the IWW navigation operations selected for this study. The resulting list of requirements is too extensive to be included in this document. The focus now is set on providing the criteria selected for requirement definition, as a relevant background for the following gap analysis and prototype design and implementation. The full analysis can be found in the AVIS D2.1 document, “User requirements for Automated Navigation” [1].

# EGNSS and Copernicus requirements GAP analysis

With the user needs identified and defined, the next step was to analyse if they could be met with the current EGNSS and Copernicus services.

## EGNSS Requirements Gap Analysis

The potential EGNSS services that are identified to help reach the defined requirements are:

- Galileo Open Service (OS): open and free service for positioning and timing provided by the Galileo constellation.

- Galileo Open Service Navigation Message Authentication (OSNMA): complements the Galileo OS by delivering authenticated data.

- Galileo High Accuracy Service (HAS): additional data provided to increase the accuracy of the solution provided by Galileo and GPS constellations.

- European Geostationary Navigation Overlay Service (EGNOS): European satellite-based augmentation system (SBAS) that aims to increase the accuracy of GNSS satellites’ signals.

As such, the expected performance from the various EGNSS services was thoroughly studied and compared with the requirements set for the different groups of operations and different automation levels.  It is to be highlighted that the services’ theoretical performance used for this analysis is taken directly from the official documents published by each of the services’ providers. From the list of requirements presented in the IALA ENG20 input paper “GNSS & EO Requirements for Automated Inland Waterways Navigation” (see related documents),  a significant part were observed to be achievable with current services’ performances. However, the focus of this section is placed on the set of requirements that cannot be met with the current theoretical performance of the EGNSS services considered. Table 1 collects all the identified gaps in the requirements, that is, the requirements that are not met and the current gap to meet them based on the current performance of EGNSS services under nominal conditions [2].

Table 1 	EGNSS requirements gaps collection

| Parameter | Requirement | Current Performance | Gap % |
| --- | --- | --- | --- |
| Availability | 99.80% | 90% | 9.82% |
| Horizontal Position ¦ Accuracy ¦ Group 3 ¦ AL3 | 0.1 m | 0.5 m | 400% |
| Time Between Authentications (TBA) ¦ AL0-AL2 | 10 s | 30 s | 200% |
| Time Between Authentications (TBA) ¦ AL3 | 6 s | 30 s | 400% |
| Vertical Position ¦ Accuracy ¦ Group 3 ¦ AL0-AL2 | 0.1 m | 0.6 m | 500% |
| Vertical Position ¦ Accuracy ¦ Group 3 ¦ AL3 | 0.1 m | 0.6 m | 500% |
| SOG ¦ Accuracy ¦ Group 1 ¦ AL3 | 1% | 2% | 100% |
| SOG ¦ Accuracy ¦ Group 2 ¦ AL3 | 0.2 km/h | 0.37 km/h | 85% |
| SOG ¦ Accuracy ¦ Group 3 ¦ AL3 | 0.2 km/h | 0.37 km/h | 85% |
| COG ¦ Accuracy ¦ Group 1 ¦ AL3 | 0.3º | 1º | 233.33% |
| COG ¦ Accuracy ¦ Group 2 ¦ AL3 | 0.17º | 1º | 488.24% |
| COG ¦ Accuracy ¦ Group 3 ¦ AL3 | 0.07º | 1º | 1328.57% |
| HDT ¦ Accuracy ¦ Group 2 ¦ AL3 | 0.17º | 0.25º | 47.06% |
| HDT ¦ Accuracy ¦ Group 3 ¦ AL3 | 0.07º | 0.25º | 257.14% |
| ROT ¦ Accuracy ¦ All Groups ¦ AL0-AL2 | 1º/min | 30º/min | 2900% |
| ROT ¦ Accuracy ¦ Group 1 ¦ AL3 | 1º/min | 30º/min | 2900% |
| ROT ¦ Accuracy ¦ Group 2 ¦ AL3 | 0.5º/min | 30º/min | 5900% |
| ROT ¦ Accuracy ¦ Group 3 ¦ AL3 | 0.3º/min | 30º/min | 9900% |

These results indicate that:

- Most of the requirements that are not met with current EU Space services are for higher automation levels (AL3). Therefore, using EGNSS services alone for autonomous navigation will not be enough.

- The availability of PNT data varies depending on the satellite visibility at a certain area and time. As such, the required availability of PNT data is not always a guarantee and falls short of the requirement set for AL3.

- The TBA requirements are not met for any automation level. As such, the frequency of OSNMA authentications would need to be increased.

- Currently, GNSS services cannot achieve the vertical position accuracy required for Group 3 activities at any automation level.

- In general, the ROT requirements have been identified as the furthest from being achievable with current GNSS services.

## Copernicus Requirements Gap Analysis

The Copernicus services that are identified as potentially capable of aiding in automated navigation are the detection of the river edge, measurements of river water depth and river discharge, detection of objects, and flood forecasting.

As with the EGNSS services, a theoretical analysis was performed on the difference between the current performance of these services for the requirements established for each group of user needs and automation level. Table 2 shows the gap collection for the requirements that cannot be met based on the current performance of the different Copernicus services.

Table 2 	Copernicus requirements gaps collection

| Parameter | Requirement | Current Performance | Gap % |
| --- | --- | --- | --- |
| River Edge ¦ Service Error ¦ AL0-AL2 | 0.1 m | 20 m | 19900% |
| River Edge ¦ Service Error ¦ AL3 | 0.05 m | 20 m | 39900% |
| River Depth ¦ Service Error ¦ Group 1 ¦ AL0-AL2 | 0.1 m | 0.35 m | 250% |
| River Depth ¦ Service Error ¦ Group 1 ¦ AL3 | 0.05 m | 0.35 m | 600% |
| River Depth ¦ Service Error ¦ Group 2 ¦ AL0-AL2 | 0.05 m | 0.35 m | 600% |
| River Depth ¦ Service Error ¦ Group 2 ¦ AL3 | 0.05 m | 0.35 m | 600% |
| Object Detection ¦ Service Error ¦ AL0-AL2 | 2 m | 10 m | 400% |
| Object Detection ¦ Service Error ¦ AL3 | 2 m | 10 m | 400% |

It can be concluded from this table that the service error for Copernicus services has a large gap when compared to current performances. This error is particularly significant in the river edge service. This is due to the optical resolution of Copernicus satellites’ imagery, with the smallest pixel size being 10x10m. To close this gap, the pixel size should be reduced, but an increase in the optical resolution of the satellites would lead to an increase in the revisit time, making it more challenging to meet requirements for temporal resolution. This might make it difficult to use Copernicus for automated navigation.

# AVIS Prototype for pilots Testbed

The aim of the AVIS project is to study the added value of EU Space Data and services for the automation of vessels on European IWW. As part of the study, several pilots will be performed where the purpose will be to record as much data as possible to analyse in post-processing. This section describes the design and implementation of the prototype that will be used in said pilots.

There are three main layers into which the prototype can be conceptually divided:

- The EU Space Data layer recompiles all the information that can be obtained from these services, both from EGNSS and Copernicus.

- The AVIS Safety layer processes this data to generate alerts and any other services that can be used for decision making for safe navigation.

- The data from the EU Space Data layer and the AVIS Safety layer can be used in the IWW layer, such as to provide the calculated route to the vessel’s components for automated navigation. This can be done using the Track Guidance Assistants in Inland Navigation (TGAIN). On top of that, this layer uses the aforementioned information to display the results on the Inland Electronic Chart Display and Information System (ECDIS).

Their interaction can be seen in Figure 1, which presents the architecture of the prototype from a high-level point of view.

Figure 1 AVIS high-level view layer architecture

This provides the high-level architecture and a conceptual overview of the AVIS prototype. This approach is thought to allow for flexibility at different automation levels. Since the focus of the project is on the study of the added value of EU space data, the prototype setup is focused on the maximisation of the data that can be recorded on the pilots for further analysis. As such, there are three main sources of data that can be considered: the EGNSS part, the Copernicus part and the IWW layer part (more concretely, the ECDIS). These will be explained more in detail in this section. This prototype can also make use of other sensors that, while not technically a part of it, will be used for the pilots due to their extensive use in the world of IWW navigation.

## EGNSS

This is the layer of the prototype that is made up of all the elements that collect information from EGNSS services that is accessible to all users. There are several pieces of equipment that are part of the EGNSS prototype part:

- GNSS Antennas: this is a critical component in the reception of GNSS signals. For this prototype, two antennas are required since it allows for the calculation of the heading of the vessel.

- ASGARD receivers: these are GNSS receivers developed by GMV and which comply with different maritime regulations. They allow for dual-frequency multi-constellation settings, as well as OSNMA configuration. Two receivers of these characteristics will be used in the prototype for redundancy and to assess different services and features.

- Auxiliar devices for RINEX recording: these are other GNSS receivers that can be set up in parallel to the ASGARD receivers and will be used to save the GNSS observations in RINEX format (i.e., a standardised file format used to store raw satellite data) to carry out the post-processing study.

- SBAS receiver: it was decided to use an extra receiver able to receive EGNOS corrections. A commercial receiver was selected for this with capacity to process EGNOS in real time and apply SBAS corrections for higher accuracy.

## Copernicus

The Earth Observation layer does not depend on any receiver. The information that can be obtained from the Copernicus services comes from Earth observation satellites (i.e., Sentinel-1 for radar and Sentinel-2 for optical imagery) or from land-based sensors and is stored in a database that can be accessed by the prototype via API.

This section aims to define the modular design of the Copernicus prototype, which constitutes the ad-hoc services. An overview of the services and their modules is shown in Figure 2.

Figure 2 AVIS prototype design for ad-hoc services

From here, several modules can be developed. For each of the ad-hoc services identified in section 4.2, a module is established that can process satellite data, generate alerts, and store the results. Apart from those, two other modules are developed:

- Copernicus Interface: it queries Sentinel-1 and Sentinel-2 periodically to gather data for a set of predefined Areas of Interest (AOI).

- Geodatabase: the outputs of each module described above are stored in the geodatabase and can be accessed via API.

For these modules, historical data is used to train a model for the each of the pilot sites. Historical optical data is used to flag the river edge and objects detected in their respective modules, and gauge data is used to train models to predict the river depth and discharge.

## IWW layer

ECDIS is a fundamental part of this layer. It is a piece of software that allows to visualise the Electronic Nautical Charts (ENCs), shows all sensor information in a user interface (position, time, course, speed, heading) using data from EU Space Data services and other on-board sensors, communicates with services like Copernicus, offers route-planning options, etc.

ECDIS can also be easily integrated with other systems used in IWW navigation and will thus be used in the pilots to integrate with:

- EGNSS for accurate PNT.

- Copernicus for extra chart overlays and alerts.

- ROT-regulator to demonstrate TGAIN together with EGNSS and Copernicus.

- Onboard radar to overlay the radar image on top of the chart.

## AVIS Prototype Solution Design

With these three parts explained, the complete solution can be shown.

Figure 3 AVIS prototype preliminary design

The idea is for the EGNSS and Copernicus parts to work independently and provide information to the main laptop. The main laptop will have the ECDIS software installed, and will use the inputs from the EU Space Data parts to display information such as position, alerts, etc.

Other sensors that are used in IWW navigation and could be used in the pilots to provide extra information are a compass receiver, mobile AIS receiver, and radar.

# AVIS pilots

The AVIS project allows for the obtention of experimental data to assess the EU Space Data and services’ added value for automated vessels on European IWW via the execution of four different pilots.

## River areas identified

The pilot areas identified are the Rhine and Moselle rivers in Germany, the Middle Danube area in Hungary, the Lower Danube in Romania, and the Guadalquivir in Spain.

### Rhine, Moselle

The Rhine is one of the busiest waterways in the world. In Germany alone, 70% of all waterway transportation is carried through the lower section of the Rhine, spanning from Cologne to Emmerich. The Moselle is another river that flows into the Rhine in the city of Koblenz. It is also significantly busy and has a large number of locks and bridges, making it a good candidate to assess some of the navigation operations considered.

### Middle Danube

Hungary is a country that is entirely located in the heart of the Danube Basin. Its capital city, Budapest, is split in the middle by the river Danube. As such, the Danube is an important international inland navigation route and key in the transportation of goods, services, and people. Therefore, this pilot will focus on the navigation along the middle section of the Danube, across the city of Budapest.

### Lower Danube

The lower section of the Danube runs through Romania and serves as a vital transport route in the area, as it connects the country with important production, sales, and acquisition markets in Europe. The route considered in this pilot spans the length of the Danube-Black Sea Canal and stretches from the port city of Constanta to Cernavoda. This route includes some locks and bridges.

### Guadalquivir

The Port of Seville is the only inland maritime port in Spain. As such, it is key in the transport of goods within the country and Europe, with a large number of sea and land connections. The are of interest for this pilot is the entire navigable part of the Guadalquivir, from the lock at the city of Seville to the mouth of the river in Sanlúcar de Barrameda.

## Navigation operations and EU-Space Data services identified

The navigation operations selected for each pilot depend on the area of interest and the vessel identified for the pilot. A summary of the Inland Waterway Navigation operations selected for the pilots is presented in Table 3.

Table 3 Summary of the operations selected for each of the pilots

| Navigation operation | Rhine Pilot | Middle Danube Pilot | Lower Danube Pilot | Guadalquivir Pilot |
| --- | --- | --- | --- | --- |
| ECDIS navigation, Information and/or Navigation modus | X | X | X | X |
| Track Guidance Assistants in Inland Navigation |  | X |  |  |
| General navigation on free-flowing rivers | X | X |  |  |
| Navigation on small narrow channels |  |  | X | X |
| Congestion controlled rivers and canals | X |  | X |  |
| Bridge passing | X | X | X |  |

The intention is to try and assess all of the EGNSS and Copernicus services at each of the pilots. However, this will depend on the operations that are selected for specific pilots since not all operations require all the services that can be provided via GNSS or Earth Observation satellites.

## Pilot Calendar

The idea is to carry out the pilots throughout the year 2026, in increasing order of complexity regarding the navigation operations considered for each of them. This is done to acquire the experience necessary to successfully solve unpredicted issues that may arise during the pilots. On top of that, it will allow for the results of a pilot to be used to improve the planning of the following one. The tentative dates for each of the pilots are shown in Figure 4. The only known dates are for the first pilot, which is the one in Guadalquivir.

Figure 4 Tentative Pilot Calendar

As it can be seen, the Guadalquivir Pilot has already been carried out and there are still three more pilots to be executed. Following the calendar, it is expected that the results from the pilot will be available in the following months, so no results on this first pilot are included in this paper. This paper provides some results obtained in preliminary laboratory tests as shown in the following section.

# Preliminary Laboratory results

With the prototype defined, some preliminary tests are performed in the laboratory to assess that everything works correctly and that all the pieces of equipment are integrated appropriately before going to the pilots.

## EGNSS Results

The verification of the EGNSS part comes from the assessment of the different EGNSS services.

Galileo OS and OSNMA services are assessed with the ASGARD receiver. This receiver is capable of processing Galileo OS to provide user positioning. The same receivers have been run with OSNMA active, and the results indicate that they can process OSNMA and authenticate the signal in real time. Raw GNSS data is used alongside SPP (single point positioning, a positioning algorithm with low computational cost used in satellite navigation) and PPP (precise point positioning, a more computationally expensive positioning algorithm that allows for higher accuracy) algorithms and HAS corrections to assess the performance of these services. As per the EGNOS corrections, tests have been conducted on the SBAS receiver to verify that EGNOS is processed correctly.

It was found in this preliminary assessment that all of the services work as expected. With this verification in a controlled environment, all of the services are ready to be used and analysed in a real environment such as the pilots defined above. In these pilots, the real performance of the EGNSS services can be assessed.

## Copernicus Results

The verification of Copernicus services is done at a modular level for each of the AOIs (Areas Of Interest) investigated.

The river edge, object detection, river depth, and river discharge modules are tested with the use of a test dataset. It is verified that all of these modules are well implemented and that they work and provide the expected information with high precision.

While these results show great potential for these services, the high spatial resolution and a revisit time of three days make these services better suited for monitoring instead of real-time navigation control. These services can be integrated with Inland Waterways Authorities’ decision-support systems.

## IWW ECDIS Software Results

The implementation of the ECDIS software involves the visualisation of Copernicus data on the ECDIS display. Copernicus data is set as an input to this software, which can then be seen on the display as layers on top of the ENC. Currently, the satellite imagery, river extent, river extent alert and object detection have been implemented on this software.

The results of this implementation are seen in Figure 5, where the semi-transparent blue mask depicts the river edge flag, the objects circled in orange show the river edge alerts and the objects circled in yellow are the direct result of the object detection layer.

Figure 5 Visualisation of Copernicus data layers on the ECDIS software implementation

## Prototype Integration

One final point to assess is the integration of all the elements of the prototype. The results indicate that:

- The main laptop is capable of running the ECDIS software, which can successfully use data from the different receivers for positioning, and Copernicus data for module visualisation.

- The outputs of the different individual devices can be logged even if the ECDIS software is actively using these outputs.

- The main laptop is capable of running all the required processes in parallel.

# Conclusions and further activities

This paper presents the progress of the AVIS project from a previously presented study (see related documents), where the EGNSS and Copernicus requirements and user needs were identified. A gap analysis was performed to assess how feasible it was to reach the requirements established with the current services provided by EGNSS and Copernicus. In this study, it was seen that the requirements for lower automation levels were more easily achievable with current services, but for higher automation levels these were more challenging.

With all this information, a prototype was defined in a way that maximises the amount of data that can be obtained from different devices. An overview of the prototype was given, with the different sources of data defined and their integration briefly described.

Four pilots were defined and planned throughout the course of the year 2026 to obtain some experimental data for analysis. For each of these, an area of interest is defined, alongside the operations selected and EGNSS and Copernicus services to be used.

Before the actual testing, it was relevant to verify that all the services worked correctly and to obtain preliminary results for their performance. It was concluded that the prototype was well integrated and that all the devices could work well in parallel. The receivers could use the Galileo OS, and they could also correctly authenticate the signal in real time with OSNMA active, as expected. On top of that, PPP and SPP algorithms were successfully integrated with HAS corrections to make use of the service and the SBAS receiver was observed to correctly receive and process EGNOS corrections. It was also concluded that Copernicus services were well implemented for the areas of interest, and that they were well integrated with the ECDIS software.

As for the future work expected from the project, the pilots will be performed throughout the year and the data obtained will be thoroughly analysed in post-processing. This will be used to work on the improvement of the prototype and to draw more global conclusions for the project and answer the research question posted.

# References

AVIS D2.1. “User Requirements for Automated Navigation”, 2024. https://avisproject.gmv.com/wp-content/uploads/2025/06/AVIS_GMV_TN_D2.1_v1.0.pdf

AVIS D2.2. “Analysis on Capacity Of Galileo, EGNOS and Copernicus to Contribute to the Operations and Identification of Gaps”, 2025. https://avisproject.gmv.com/wp-content/uploads/2025/06/AVIS_GMV_TN_D2.2_v2.0.pdf

CCNR, “International definition of levels of automation in inland navigation”, 2022. https://www.ccr-zkr.org/files/documents/AutomatisationNav/DefinitionAutomatisation_en.pdf

# List of Acronyms

| Acronym | Definition | Acronym | Definition |
| --- | --- | --- | --- |
| AIS | Automatic Identification System | HAS | High Accuracy Service |
| AL | Automation Level | HDT | Heading |
| API | Application Programming Interface | IWW | Inland Waterways |
| AVIS | Automated Vessels on European Inland Waterways | OS | Open Service |
| CCNR | Central Commission for the Navigation of the Rhine | OSNMA | Open Service Navigation Message Authentication |
| CMEMS | Copernicus Marine Environment Monitoring Service | PNT | Positioning, Navigation and Timing |
| COG | Course Over Ground | PPP | Precise Point Positioning |
| ECDIS | Electronic Chart Display and Information Systems | RINEX | Receiver Independent Exchange Format |
| EFAS | European Flood Awareness System | ROT | Rate Of Turn |
| EGNOS | European Geostationary Navigation Overlay Service | SBAS | Satellite-Based Augmentation System |
| EGNSS | European GNSS | SOG | Speed Over Ground |
| ENC | Electronic Navigational Charts | SPP | Single Point Positioning |
| EO | Earth Observation | TBA | Time Between Authentications |
| GNSS | Global Navigation Satellite System | TGAIN | Track Guidance Assistant for Inland Navigation |
| GPS | Global Positioning System |

# Action requested of the Committee

- The Committee is requested to:

- To review the input paper.

- To provide feedback, if considered, to the authors.

- To be able to make a presentation in next ENG22 committee to share the information provided in this paper and potentially include some preliminary results of the first pilot test.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain