Input paper:   ENG22-3.1.2.4

Input paper for the following Committee(s):		Purpose of paper:

（Select as appropriate）

☐ ARM	☒  ENG		☐ PAP			☒ Input

☐ DTEC	☐ VTS					☐ Information

Agenda item 			 n.n

Technical Domain / Task Number 2	 Task 7.1.2

Author(s) / Submitter(s)			 China Maritime Safety Administration (China MSA)

# Data Model of S-241 Positioning, Navigation and Timing (PNT) Station Almanac Product Specification

# summary

This paper proposes a standardized, compatible, and scalable data model framework for the S-241 "PNT Station Almanac Information Product Specification" based on IALA Committee Work Plan 2025-2027 and S-200 series product specification development tasks adopted in the International Organization for Marine Aids to Navigation (IALA) . The model adopts the International Hydrographic Organization (IHO) S-100 geospatial information standard as its core framework, compatible with existing IALA station almanac specifications including S-240 (DGNSS), S-246 (eLoran), and S-247 (differential eLoran). By systematically organizing multi-source data classification and coding rules for PNT stations, establishing a standardized element catalog, and designing a normalized dataset coding structure, it provides essential technical support for the official formulation of the S-241 product specification.

## 1.1  Purpose of the Document

This paper aims to submit the S-241 data model research findings to the 22nd Session of the AtoN Engineering and Sustainability Committee (ENG22) as a working paper for review and discussion by member states. It illustrates the refinement the technical framework of the S-241 draft "PNT Station Almanac Information Product Specification" by introducing critical components such as data classification, coding, and element catalogs, etc, to advance the evolution of the draft Product Specification from a framework document to a practical and feasible standard.

## 1.2  Related documents

[1]IALA Committee Work Plan 2025-2027

[2] C03-10.5.1 ENG21 Meeting Summary Report

[3] C03-10.7.1 Report on the Development of Product Specifications for S-200

[4] S-240 DGNSS Station Product Specification (Version 1.2.0)

[5] S-245 eLoran ASF data (Version 1.0.0)

[6] S-246 eLoran Station Product Specification (Version 1.0.0)

[7] S-247 Differential eLoran Station Product Specification (Version 1.0.0)

# 2   Background

As the core infrastructure of maritime navigation systems, PNT service is vital for ensuring vessel navigation safety and improving maritime traffic efficiency. To advance the standardization and interoperability of global maritime PNT systems, IALA has incorporated S-241, the 'PNT Station Almanac Information Product Specification,' into the development of the S-200 series product specifications (see task item 7.1.2).

At the ENG21, China Maritime Safety Administration submitted the S-241 draft product specification as a working document for discussion. The ENG21 decided to further develop the S-241 data model and refine technical details. The S-241 PNT Station Almanac Product Specification should effectively cover the almanac specifications of single navigation system stations such as S-240 (DGNSS), S-246 (eLoran), and S-247 (differential eLoran) to meet the information management requirements of integrated and multi-source PNT stations.

By integrating the IHO S-100 standard framework, IALA G1106 guidelines, and the existing station almanac specificaion systems, together by consolidating multi-source navigation system data, this approach establishes a standardized, compatible, and scalable S-241 data model by utilizing multiple dimensions of data such as basic attributes, location information, equipment parameters, operation status, and metadata.

# 3  discussion

3.1 Overall Framework and Core Parameters of the S—241 Data Model

This data model adopts an architecture of "Data Classification and Encoding Guide (DCEG) + Feature Catalogue Builder(FCB)+Feature Catalogue (FC) + Data Association Rules", covering S-241 development requirements. The overall framework and core parameters are as follows:

Figure 1 PNT Station Almanac Overview

3.2  Overall Framework of Data Model

The data model is based on the IHO S-100 standard, using the three-layer architecture of "input layer—core processing layer—output layer" with the input layer covering the multi-source PNT station data, and the core processing layer realizing data classification, coding and element definition.

3.2.1 Core Parameter Table

Table 1 Core Parameters of the S-241 Data Model

| module | Item | Item Description |
| --- | --- | --- |
| input layer | basic data of the station | Unique identification (ID), name, affiliated institution, country/region, and construction/activation time of the station, etc. |
|  | geographical data | WGS84 latitude and longitude, geometric type (GM_Point), vertical datum, position accuracy, etc. |
|  | equipment technical parameters | navigation system type, signal frequency, power, antenna parameters, transmission delay, differential correction flag, etc. |
|  | running/operation status data | working status, health level, sync mode, data update cycle, etc. |
|  | metadata | data quality, coverage, collection source, version information, etc. |
| core processing layer | data classification, coding system and definition of elements | 5 major categories and 32 subcategories, using a hierarchical coding system of "characters + numbers" to generate DCEG |
| output layer | standardized elements catalog | 72 elements (with 28 core elements + 44 extended elements), specification of data types, value ranges, and required/optional attributes |
|  | data set encoding structure | GML 3.2.1 compatible format, including header information, basic data blocks, system-specific blocks, metadata blocks, and geometric data blocks |
|  | data association rule | using ID as the core association key to establish internal data consistency and external multi-system compatibility associations |
|  | standardized data set | GML files compliant with S-100 Part 10b, with file names following the rule of 241CCCCXXXXXXXX_EEE.GML |

3.2.2 Detailed Design

(1) Based on the IALA's existing station almanac coding logic and PNT multi-source data characteristics, the data classification and coding system categorizes data into 5 major categories and 32 subcategories.

(2) Based on the classification coding system, a dual-track standardized element catalog comprising "core elements + extended elements" is constructed and generated in XML format.

(3) The dataset encoding structure and association rules strictly comply with the IHO S-100 standard, utilizing the Geospatial Markup Language (GML) 3.2.1 format. The file structure is organized by following the sequence of: header information → basic data blocks → system-specific blocks → metadata blocks → geometric data blocks, with the coordinate precision maintained to 7 decimal places and character encoding by UTF-8.

(4) Data association rule implementation:

Internal correlation: various types of data such as basic data, device parameters, and operation status are maintained consistent through IDs.

External integration: by reusing the encoding rules of S-240 and S-246, the seamless integration with system station data is achieved.

Cross-product integration: through the establishment of connections between IDs and Electronic Navigational Charts (ENC) as well as Aids to Navigation (S-201), the integration of multiple maritime data products is fully supported.

# 4  references

[1]S—100 IHO S—100 Universal Hydrographic Data Model (Edition 5.2.0)

[2]S—97 IHO Guidelines for Creating S—100 Product Specifications

[3]ISO 8601:2004 Data Elements and Interchange Formats — Representation of Dates and Times

[4] ISO 19136:2007 Geographic Information — Geography Markup Language (GML)

[5]G1106 IALA Producing an IALA S—200 Series Product Specification

# 5  Action requested of the committee

The Committee is requested to review the relevant proposals and take appropriate action.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain