Input paper: ENG22-3.1.2.7

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	X  ENG		□  PAP					X  Input

□  DTEC	□  VTS							□ Information

Agenda item			n.n

Technical Domain / Task Number	Radionavigation services

Author(s) / Submitter(s) 			Sulgee Park, Sanghyun Park (KRISO, R.O.K.), Youngmin Lim, 
			Seungcheol Lee (MOF, R.O.K.)

# Working Draft of Guideline on Ground-based GNSS Precise Positioning for Maritime Service

# ABSTRACT

At the 21st IALA ENG Committee meeting (ENG 21, October 2025, Dublin, Ireland), the Republic of Korea submitted an information paper (ENG21-3.1.2.4) entitled "Towards an IALA Guideline for Maritime Ground-based GNSS Precise Positioning Services: Lessons from the Republic of Korea's POINT Service," presenting the operational status and pilot performance of Korea's ground-based GNSS precise positioning service (POINT: Precise POsitioning and INTegrity monitoring). Following the deliberations at ENG 21, the Committee formally adopted the development of a new IALA Guideline on Ground-based GNSS Precise Positioning for Maritime Service as a new task.

As a follow-up to that decision, the Republic of Korea has prepared a working draft structure of the proposed Guideline for consideration at ENG 22 (April 2026). This paper presents the proposed table of contents (ToC), proposes an assignment of drafting responsibilities to interested member states, and outlines a drafting schedule targeting completion of the final Guideline by April 2027, with a view to presenting it at the 21st IALA Conference (8-12 November 2027, Mumbai, India).

# PURPOSE

The purpose of this document is to:

•	Present a proposed table of contents for the Working Draft of the Guideline on Ground-based GNSS Precise Positioning for Maritime Service;

•	Propose an assignment of drafting responsibilities for each section to interested member states and organizations; and

•	Present a drafting plan and timeline for the completion of the final Guideline by April 2027, in time for presentation at the 21st IALA Conference in November 2027.

# BACKGROUND

Ground-based GNSS precise positioning services — including Differential GNSS (DGNSS), Real-Time Kinematic (RTK), Network RTK (e.g., VRS, MAC, and FKP), and Precise Point Positioning (e.g., PPP-RTK and PPP) — have been widely deployed to support maritime navigation. Compared to satellite-based PPP services, ground-based precise positioning systems offer distinct operational advantages, particularly for coastal and port environments: centimeter-level accuracy, fast convergence time (seconds rather than minutes), relatively low deployment costs by leveraging existing national geodetic networks, and high suitability for near-shore maritime autonomous applications.

The Republic of Korea has been at the forefront of developing such infrastructure through the POINT project (Precise POsitioning and INTegrity monitoring), conducted from April 2020 to December 2024. Research and development outcomes were transferred to the Korean National Maritime PNT Office (NMPO) under the Ministry of Oceans and Fisheries in December 2024. A pilot operational phase commenced in December 2024.

The POINT service pilot operation (January-August 2025) demonstrated exceptional performance:

•	Horizontal positioning accuracy (rms): consistently within 3.1 cm (target: 5 cm)

•	Vertical positioning accuracy (rms): consistently within 3.7 cm (target: 10 cm)

•	Service availability: 99.8% or above (during normal network operation)

•	Integrity monitoring: 100% - no false alarms or missed detections throughout the pilot period

These proven results provided the technical basis for Korea's proposal at ENG 21 to develop a dedicated IALA Guideline for ground-based GNSS precise positioning services. The proposal was formally adopted as a new task by the Committee.

At ENG 21, the comparison between ground-based and satellite-based augmentation was also presented, as summarized in Table 1 below.

Table 1. Comparison of Ground-based and Satellite-based Augmentation

| Feature | Ground-Based Augmentation | Satellite-Based Augmentation |
| --- | --- | --- |
| Augmentation Data Delivery | Terrestrial Communication (LTE, 5G, etc.) or Broadcast Media (DMB, UHD) | GNSS Signals (L-band, etc.) |
| Data Transmission Rate | Min. 1 Mbps (High Bandwidth) | Under 2,000 bps (Low Bandwidth) |
| Positioning Performance | Centimeter-level accuracy; Fast convergence (seconds) | Decimeter-level accuracy; Slow convergence (minutes) |
| Service Coverage | Coastal and Near-shore areas | International Oceans (Global Coverage) |
| Deployment Cost | Relatively low initial construction cost | Very high initial construction and maintenance cost |

The two types of precise positioning service will necessarily coexist, with the preferred option adoption determined by the specific environmental and performance requirements of different maritime applications. A dedicated IALA Guideline for ground-based services is therefore essential to complement the ongoing development of the Guideline on GNSS Satellite-based PPP Maritime Service, and to ensure a cohesive, comprehensive set of international standards for all high-precision GNSS maritime services.

Existing IALA guidance documents — such as IALA Guideline 1127 (Systems and Services for High Accuracy Positioning and Ranging) and IALA Guideline 1152 (SBAS Maritime Service) — address some related aspects; however, no dedicated guideline currently exists for ground-based precise positioning services within the maritime domain. Furthermore, the existing draft framework for the satellite-based PPP guideline can serve as a direct reference, enabling IALA to build upon established frameworks efficiently, without starting from scratch.

# DISCUSSION

## Proposed Table of Contents

The following table of contents is proposed for the Working Draft of the Guideline. The structure is informed by the parallel work on the IALA Guideline on GNSS Satellite-based PPP Maritime Service and is adapted to reflect the specific characteristics and requirements of ground-based systems, incorporating lessons learned from the POINT service.

IALA Guideline GXXXX  |  Ground-based GNSS Precise Positioning for Maritime Service  |  Edition 1.0 (Working Draft)

1. Introduction

1.1 Scope of the Document

1.2 Structure of the Document

2. Reference Requirements

2.1 IMO Resolution A.1046(27) Requirements

2.2 IMO Resolution A.915(22) Requirements

2.3 Relevant IALA Guidelines and Recommendations

3. Ground-based GNSS Precise Positioning Systems

3.1 Overview of Ground-based Precise Positioning Techniques

3.1.1 DGNSS

3.1.2 RTK / Network RTK (VRS, MAC, FKP, etc.)

3.1.3 PPP-RTK / PPP

3.2 Comparison with Satellite-based Augmentation Services

3.3 Existing and Planned Ground-based Precise Positioning Systems for Maritime Use

3.4 System Architecture

3.4.1 GNSS Reference Station Network

3.4.2 Data Processing and Control Centre

3.4.3 Data Dissemination Infrastructure (LTE/5G, DMB/UHD, etc.)

3.4.4 User Segment

3.5 Correction Parameters and Data Formats (RTCM/Compact SSR, OSR/SSR, etc.)

3.6 Service Coverage

4. Service Performance Parameters

4.1 Positioning Accuracy

4.2 Convergence Time and Initialization

4.3 Signal Availability

4.4 Service Availability

4.5 Service Continuity

4.6 Integrity (Alert Limits, Time to Alarm and Integrity Risk)

4.7 Service Coverage Area

5. Maritime Service Provision Scheme

5.1 Service Provider Responsibilities

5.1.1 Operation and Maintenance

5.1.2 Performance Verification

5.1.3 Publication of Service Information

5.1.4 Working Agreements

5.2 Aids to Navigation Authorities

5.3 End Users

5.4 MSI Provider

6. Maritime Application Scenarios

6.1 Port and Harbour Operations (incl. automated docking & cargo handling)

6.2 Coastal and Restricted Water Navigation

6.3 Maritime Autonomous Surface Ships (MASS)

6.4 Offshore Operations

6.5 Survey and Hydrographic Operations

6.6 Search and Rescue

7. Acronyms

8. References

Annex A: Overview of Existing and Planned Ground-based Systems by Country/Region

## Proposed Assignment of Drafting Responsibilities

To facilitate the efficient development of the Guideline, it is proposed that interested member states and organizations take primary drafting responsibility for specific sections. The table below presents a proposed assignment, subject to the Committee's agreement and the willingness of members to participate. Member states wishing to contribute to any section are invited to express their interest at ENG 22.

Table 2. Proposed Assignment of Drafting Responsibilities

| Section | Title | Proposed Lead | Supporting Members |
| --- | --- | --- | --- |
| 1 | Introduction | Republic of Korea | All |
| 2 | Reference Requirements | Republic of Korea | All |
| 3.1 | Overview of Ground-based Techniques | Republic of Korea | TBD |
| 3.2 | Comparison with Satellite-based Services | Republic of Korea | TBD |
| 3.3 | Existing and Planned Systems | All interested members | TBD |
| 3.4 | System Architecture | TBD | TBD |
| 3.5 | Correction Parameters & Data Formats | TBD | TBD |
| 3.6 | Service Coverage | TBD | TBD |
| 4 | Service Performance Parameters | TBD | TBD |
| 5 | Maritime Service Provision Scheme | TBD | TBD |
| 6.1 | Port and Harbour Operations | TBD | TBD |
| 6.2 | Coastal and Restricted Water Navigation | TBD | TBD |
| 6.3 | MASS | TBD | TBD |
| 6.4 | Offshore Operations | TBD | TBD |
| 6.5 | Survey and Hydrographic Operations | TBD | TBD |
| 6.6 | Search and Rescue | TBD | TBD |
| Annex A | Country/Region System Overviews | All interested members | TBD |

* TBD entries are to be confirmed at ENG 22 (April 2026) based on member state expressions of interest. The Republic of Korea stands ready to contribute its operational expertise and data from the POINT service to support any section of the Guideline.

## Drafting Plan and Schedule

The IALA ENG Committee meets twice per year, typically in April and October. The following inter-sessional drafting schedule is proposed, with the objective of producing a finalized draft Guideline by April 2027 for presentation at the 21st IALA Conference (8-12 November 2027, Mumbai, India).

Table 3. Drafting Plan and Schedule

| Milestone | Target Date |
| --- | --- |
| ENG 22 (April 2026): Agreement on ToC, section assignments, and drafting plan | April 2026 |
| Submission of initial section drafts by assigned lead members | July 2026 |
| Consolidation and internal review of draft by Korea (as proponent) | August 2026 |
| Circulation of consolidated draft to all interested members for comment | September 2026 |
| ENG 23 (October 2026): Review of consolidated draft; agreement on revisions | October 2026 |
| Incorporation of ENG 23 comments; preparation of revised draft | January 2027 |
| Circulation of revised draft to all members for final comment | February 2027 |
| Incorporation of final comments; completion of final draft Guideline | April 2027 |
| ENG 24 (April 2027): Endorsement of final draft Guideline | April 2027 |
| 21st IALA Conference (8–12 November 2027, Mumbai, India): Presentation and publication of Guideline | November 2027 |

# PROPOSAL

It is recommended that the ENG Committee:

•	Review and endorse the proposed table of contents for the Working Draft of the Guideline on Ground-based GNSS Precise Positioning for Maritime Service;

•	Invite member states with an interest in contributing to the development of this Guideline to identify their preferred section(s) and confirm their participation at ENG 22; and

•	Endorse the proposed drafting schedule, targeting completion of the final draft Guideline by April 2027 and its presentation at the 21st IALA Conference (8-12 November 2027, Mumbai, India).

# ACTION REQUESTED OF THE COMMITTEE

The Committee is invited to:

(a) Agree on the proposed table of contents for the Guideline on Ground-based GNSS Precise Positioning for Maritime Service;

(b) Confirm the assignment of drafting responsibilities for each section of the Guideline, based on expressions of interest from member states at ENG 22; and

(c) Endorse the inter-sessional drafting plan and the overall schedule, with a target of completing the final draft Guideline by April 2027 for presentation at the 21st IALA Conference in November 2027.