Input paper: ENG22-3.1.2.8

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

X ARM			X  ENG		□  PAP					□  Input

X  DTEC			X  VTS							X  Information

Agenda item 				7.2

Technical Domain / Task Number 2	N/A

Author(s) / Submitter(s)			Secretariat

# Overview of the RIN Maritime GNSS Interference Report (2026)

# Summary

This paper provides information to the Committee on the UK’s Royal Institute of Navigation (RIN) Maritime GNSS Interference Report, launched on 29 January 2026. It summarises findings relating to GNSS jamming and spoofing and highlights the implications for VTS, including recommended technical, operational, training and reporting measures.

## Purpose of the document

This document is submitted for information. The Committee is invited to note the findings and take action as appropriate.

## Related documents

RIN Maritime GNSS Interference Report (2026)

# Background

In August 2025, the Royal Institute of Navigation established a working group to examine the effects and mitigations associated with GNSS disruption in the maritime sector. GNSS interference has been raised repeatedly at International Organizations such as the IMO and IALA and is increasingly recognised as a significant safety and security concern.

The report, launched on 29 January 2026, assesses the maritime industry’s reliance on GNSS and identifies vulnerabilities within navigation, communications and timing-dependent systems. Of particular relevance to VTS are the impacts of spoofed or jammed GNSS data on AIS, RADAR correlation, timing synchronisation and the maintenance of an accurate traffic image.

The full report is available at: https://rin.org.uk/RIN_Maritime_Report

# Discussion

The RIN report concludes that intentional interference with GNSS, including jamming, meaconing and spoofing, is increasing in frequency and severity. Seventy‑five percent of mariners surveyed believe the situation is deteriorating. A number of maritime casualties have been linked to GNSS interference, including groundings and collisions.

For VTS, GNSS interference creates challenges in maintaining situational awareness, ensuring continuity of service and supporting mariners whose systems may be presenting misleading positional or timing information. The report identifies a range of technical and procedural actions relevant to VTS operations.

## VTS‑relevant recommendations from the RIN report

### Technical Measures

- Integrate multiple sensors (RADAR, AIS, CCTV) to maintain the traffic image during GNSS interference;

- Conduct dependency analyses to identify VTS systems linked to GNSS position or timing;

- Consider installing detection systems and alarms for GNSS interference;

- Evaluate equipment that uses GNSS for timing to assess operational impact;

- Consider installing two GNSS receivers in fixed positions to provide an indication of jamming/spoofing;

- Fit shielded antennas to reduce low‑angle interference;

- Consider technologies such as signal authentication/encryption where feasible.

### Operational Procedures

- Include GNSS interference as a risk within the VTS Safety Management System (SMS);

- Maintain competence in voice communication procedures to compensate for unreliable AIS data;

- Verify the quality of non‑AIS traffic images and conduct routine equipment checks including RADAR calibration;

- Develop local procedures for GNSS interference situations such as reducing traffic flow, increasing separation, or deploying additional pilots;

- Consider deploying additional VTS personnel or engineering support during interference events;

- Inform internal staff and stakeholders as necessary;

- Issue warnings to mariners (e.g., via VHF, MSI broadcasts) when interference is detected;

- Require mariners to report GNSS interference experienced in the VTS area;

- Ensure vessels are aware of VTS capabilities to support them during degraded navigation events.

### Training and Awareness

- Conduct regular simulation exercises involving jamming/spoofing scenarios;

- Train VTS personnel to recognise indications of interference (e.g., AIS/RADAR discrepancies, sudden AIS jumps);

- Train staff in switching tracking sources and applying fallback procedures.

### Collaboration and Reporting

- Ensure GNSS interference events are promptly reported to national authorities;

- Consult and implement relevant international guidance;

- Apply IALA standards for best practice in technology, operations and VTS training.

# References

Royal Institute of Navigation, Maritime GNSS Interference Report, 2026.

# Action requested of the Committee

The Committee is requested to:

Note the information provided in this paper and take action as appropriate.