|  |  | ENG22-4.4.2 |
| --- | --- | --- |

# INTERNATIONAL									03.12.2025

# ITU

# Report on ITU-R WP5B meeting 18 to 27 November 2025

Note by the IALA representative Stefan Bober

#### INTRODUCTION

ITU-R Working Party 5B (WP 5B) - Maritime mobile service including Global Maritime Distress and Safety System (GMDSS); aeronautical mobile service and radiodetermination service - held its meetings from 17th to 27th November 2025 as physical meeting in Geneva. This was the fourth meeting in the study cycle 2023 to 2027. Mr. Stefan Bober represented IALA.

IALA has a specific interest in the maritime mobile service including the Global Maritime Distress and Safety System (GMDSS) and the radiodetermination service, with particular emphasis on the development of VHF Data Exchange System (VDES), Automatic Identification System (AIS), Autonomous Maritime Radio Devices (AMRD), VHF digital voice, VDES R-Mode and e-Navigation.

The meeting of the WP5B-3 maritime focussed on the revision of ITU Recommendations and the two ITU study questions, namely study questions on “VHF digital voice” and on “VDES R-Mode”. WP5B-3 maritime has no direct agenda item to prepare for WRC-27. However, WP5B-3 maritime is involved in several other agenda item at WRC-27, e.g. WRC-27 agenda item 1.12, current and future use of the 1645.5 -1646.5 MHz band.

#### issues related to IALA work addressed during ITU-R WP 5B

##### Revision of Recommendation ITU-R M.2092-1 VHF data exchange system – VDES

WP 5B finalized the work on the revision of Recommendation ITU-R M.2092-1. The group reviewed three input documents from USA, China and Denmark.

Some technical clarifications resulting from prototype testing and work at IEC, as well as some minor editorial corrections, were incorporated in the document. The technical clarifications include the updates on the transmission of short data messages with ACK, the terrestrial bulletin board, the usage conditions and operating channels for simplex VDE terminals, the definition of the terminal capabilities, the improved description of the digital signature and the introduction of digital certificate fingerprint. A proposal on blocking requirement has been withdrawn.

The revision of Recommendation ITU-R M.2092-1 was agreed by WP5 B and forwarded to ITU-R Study Group 5 (SG5). At its meeting on 1st December 2025 SG5 has initiated the procedure for simultaneous adoption and approval by correspondence for the document. The publication of Recommendation ITU-R M.2092-2 “Technical characteristics for VHF data exchange system in the maritime mobile service” is expected in early 2026.

A liaison note was drafted to inform IMO, IALA, IEC and CIRM about the finalisation of the revisions for VDES, AIS and NAVDAT.

-> 	IALA is invited to note the review the document.

##### Revision of Recommendation ITU-R M.1371-5 Automatic Identification System - AIS

WP5B finalized the revision of Recommendation ITU-R M.1371-5. The group reviewed input documents from USA, China and IMO.

The following topics were discussed: Update in Table A7-8 for Message 5 list of ship type, the IMO number description, inclusion of VDES indicator in Message 24B, update of message 28, clarify on the use of message 5 for SAR aircraft station, clarify the relation of message 1 and message 14 for burst transmissions, a reference to AMRD Group B in Rec ITU-R M.2135, clarify the use of MPP in Message 14 for unique identifier for locating devices, clarify the flow chart for first frame phase and for continuous operation phase and clarify description for intentional slot reuse. Editorial corrections have been made to the text and corrections to the figures.

The revision of Recommendation ITU-R M.1371-5 was agreed by WP5 B and forwarded to ITU-R Study Group 5 (SG5). At its meeting on 1st December 2025 SG5 has initiated the procedure for simultaneous adoption and approval by correspondence for the document. The publication of Recommendation ITU-R M.1371-6 is expected in early 2026.

-> 	IALA is invited to note the review the document.

##### New Report ITU-R M. [VDES R-MODE] - Impact of the possible introduction of a range mode on the VHF data exchange system

WP 5B continued work on a new report on the impact of the possible introduction of a range mode in the VHF data exchange system. The aim of this report is to describe the impact of R-Mode VDES.

The changes include clarifications on System Architecture and Integrity, definitions for ‘Base Station’, ‘Transmitter’, ‘Far-field Monitoring Station (FMS)’, and ‘Monitoring and Control Station (MCS), Signal Processing and Resilience, concept of processing gain and Advanced Network Operation, the amendments introduce planned co-channel operation as a network strategy.

The status of the document was upgraded to preliminary draft revision.

->	IALA is invited to contribute to the development of this report.

##### New Report ITU-R M. [DIGITAL VOICE] – Comparative analysis report of DMR and dPMR for VHF digitization

WP 5B started to draft a new report on the comparative analysis report of DMR and dPMR for VHF digitization. The group considered one input document and agreed that the analysis could support the study on VHF digitalization. This new report describes DMR as an alternative technology for VHF digital voice. DMR uses 12.5 kHz bandwidth (2 channels in one 25 kHz channel). DMR is used in terrestrial communication, it is base station centric and typical used for duplex channels. DMR  is possible for peer to peer mode without base station.  DMR is more efficient than DPMR and the standard is updated regularly. However, DMR needs time synchronization (timing source), which might be an issue for hand held and Class B equipment.

Contributions on technical studies, in particular the timing source aspect of DMR technology, are encouraged to be provided. In addition, noting the anticipation of inputs to this document in the next WP5B meeting.

->	IALA is invited to contribute to the development of this report.

##### Revision of Recommendation ITU-R M.585-9 Assignment and use of identities in the maritime mobile service

WP5B finalised the revision of Recommendation ITU-R M.585-9. Several input documents were reviewed. The following topics were discussed:

In section 4, the types of AIS AtoNs provided by the AIS Message 21 “Aids-to-Navigation Report and AIS” and Message 28 “Aids-to-Navigation Report (Single-slot message)” were introduced. The use of the sixth digit to distinguish between certain specific uses of the MMSI was clarified as optional.

A supplementary 12-character identity format for devices using freeform maritime identities, i.e. AIS-SART, MOB and EPIRB-AIS was introduced. With the growing number of such devices, the original 2-digit manufacturer identification resources which is part of the 9-digit device identity are exhausted. Therefore, a supplemental device ID information MPP is incorporated into the 9-digit identity to create a 12-character maritime identity in the following format:

9172T3X4X5M6P7P8Y9Y10Y11Y12

where T3 = device type; X4X5 = manufacturer ID 01 to 99; M6 = supplementary manufacturer ID suffix alphanumeric character; P7P8 = supplementary sequence number prefix alphanumeric character; Y9Y10Y11Y12 = the sequence number 0000 to 9999. The proposed revisions also include editorial revisions to improve the clarity of this recommendation.

The revision of Recommendation ITU-R M.585-9 was agreed by WP5 B and forwarded to ITU-R Study Group 5 (SG5). At its meeting on 1st December 2025 SG5 has initiated the procedure for simultaneous adoption and approval by correspondence for the document. The publication of Recommendation ITU-R M.585-10 is expected in early 2026.

-> 	IALA is invited to note the review the document.

##### Revision of Recommendation ITU-R M.2010-2 (NAVDAT system in 500 kHz) and ITU-R M.2058-1 (NAVDAT HF)

WP 5B finalized the revision of Recommendation ITU-R M.2010-1 and ITU-R M.2058-1. The proposed revision includes updates of references to IMO documents, clarification of pre-scan procedure, changes of the coding for broadcasting in selective area mode, improvements to the LDPC codes and updates on the description of NAVDAT frequency usage.

The revision of Recommendation ITU-R M.2010-2 and Recommendation ITU-R M.2058-1 was agreed by WP5 B and forwarded to ITU-R Study Group 5 (SG5). At its meeting on 1st December 2025 SG5 approved the document. The publication of Recommendation ITU-R M.2010-3 and Recommendation ITU-R M.2058-2 is expected in early 2026.

##### Liaison statement to WP4C regarding WRC-27 Agenda Item 1.12

WG 5B-3 invites WP 4C to consider system characteristics of Recommendation ITU-R M.1184-3, in the adjacent bands of 1 645.5-1 646.5 MHz, in its studies under WRC-27 agenda item 1.12 and accordingly prepared a draft liaison statement to WP4C with IMO in copy for information.

##### Revisions of Report ITU-R M.2231-1 (Use of Appendix 18 to the Radio Regulations for the maritime mobile service)

WP 5B continued the revision of the Report ITU-R M.2231-1 to update the Report on the use of Appendix 18, to include the updates on Appendix 18 in recent WRCs and revisions to the “Maritime frequency arrangements” and “Maritime use in RR Appendix 18” sections

##### Revision of Recommendation ITU-R M.628-5 (search and rescue radar transponders)

WP 5B started the revision of Recommendation ITU-R M.628-5 Technical characteristics for search and rescue radar transponders. to update the requirements on the effective receiver sensitivity of Radar SART and include pulse compression solid state radar in this recommendation.

##### Revision of Report ITU-R M.2027 - Engineering guidance for shore based facilities for GMDSS

WP 5B started the revision of Report -R M.2027 Engineering guidance for operators to upgrade shore based facilities to operate the Global Maritime Distress and Safety System in the A1, A2 and A3/A4 sea areas to update the sea area related text and include NAVDAT system in this report to align with the outcome of GMDSS modernization.

##### Revisions of Recommendation ITU-R M.1080 (Digital selective calling system DSC)

WP5B continued the revision of Recommendation ITU-R M.1080 “Digital selective calling system enhancement for multiple equipment installations” to update the requirements on the 10th digit for DSC audible alarms to avoid alarm on multiple radio installations.

##### Revision of Recommendation ITU-R M.541-11 (Operational procedures for DSC)

WP 5B started the revision of revisions of Recommendation ITU-R M.541-11 Operational procedures for the use of digital selective calling equipment in the maritime mobile service to further clarify international callings through DSC and list the specific frequency bands used by ACS in the recommendation in line with the latest Radio Regulations..

##### Revisions of Report ITU-R M.2231-1 (Use of Appendix 18 to the Radio Regulations for the maritime mobile service)

WP 5B continued the revision of the Report ITU-R M.2231-1 to update the Report on the use of Appendix 18, to include the updates on Appendix 18 in recent WRCs and revisions to the “Maritime frequency arrangements” and “Maritime use in RR Appendix 18” sections

#### RELATED DOCUMENTS

- Preliminary draft revision of Recommendation ITU-R M.2092-1 (VDES)

- Preliminary draft revision of Recommendation ITU-R M.1371-5 (AIS)

- Preliminary draft revision of Recommendation ITU-R M.585-9 (maritime identities)

- WD towards new report on the impact of the possible introduction of a R-Mode on the VDES

- WD towards new report on Comparative analysis report of DMR and dPMR for VHF digitization

#### IALA IS REQUESTED TO

- IALA is requested to note the report on ITU-R WP5B meeting and to act accordingly.