Input paper:   ARM22-6.3.1

Input paper for the following Committee(s): 	                            Purpose of paper:

（Select as appropriate）

☒ ARM	☐  ENG		☐ PAP		                  	☒ Input

☐ DTEC	☐ VTS			         	               	☐ Information

Agenda item  			6.3.1

Technical Domain / Task Number 	Task 1.2.5

Author(s) / Submitter(s)			 China Maritime Safety Administration (China MSA)

# Proposal on formulating the Guideline on Use of Simple IoT Sensors on Physical AtoN

# Summary

In accordance with the work plan (2025-2027) of IALA committee, the guideline on use of simple IOT sensors on physical AtoN is currently under revision. Based on the latest meeting discussion results, the China Maritime Safety Administration has supplemented and improved parts of the first and fifth chapters, compiled and revised the relevant contents of the third and fourth chapters of the draft guideline. And China MSA would like to keep undertaking the subsequent related work under the instruction of the committee and submit the proposed amendments to the committee for consideration.

## Purpose of the document

This paper aims to provide assistance in promoting the revision of the chapter in the guideline of on Use of Simple IoT Sensors on Physical AtoN.

## Related documents

[1] IALA-Committee-work-programme-2023-2027

[2] Draft guideline on Use of Simple IoT Sensors on Physical AtoN.

# Background

At the IALA ARM 21 meeting, the China Maritime Safety Administration participated in the compilation of the content of the third chapter of the guideline on Use of Simple IoT Sensors on Physical AtoN and was included in the draft guideline. However, there are still some chapters of this guideline that need to be complied and discussed. China MSA has compiled relevant chapters in accordance with the latest discussion results.

# Discussion

Based on the outcomes of the latest meeting, some contents of the first and fifth chapters of the draft guideline have been supplemented and improved, and the relevant contents of the third and fourth chapters have been complied and improved.

## Chapter One--AIMS AND OBJECTIVES

As the remote control centre is closely associated with IoT sensors and data communications as an integrated whole, it is recommended to add a description of the requirements for the remote control center in Chapter 1, clearly stating the two-way interaction capability of sensors: It shall be specified that sensors must be capable of receiving and executing remote commands (e.g., parameter modification, forced reporting), rather than only transmitting data in one direction.

## Chapter there--TECHNICAL REQUIREMENTS OF IOT SENSORS AND COMMUNICATION MODULES ON ATON

### Position Monitoring Requirements

Add differentiated position data uploading requirements to balance data validity and equipment power consumption: For floating AtoN in key areas such as important waterways and hub port areas, the position data uploading interval shall support remote adjustment; where conditions permit, the uploading interval shall be automatically shortened after detecting buoy drift or mooring failure for accurate tracking.

### Environmental Data Acquisition Requirements

Add content related to time correlation to balance adaptability, accuracy and cost: for scenarios requiring multi-source data fusion and time sequence trend analysis, sensors supporting NTP or GNSS time synchronization shall be preferred to ensure the time sequence alignment of data; time synchronization function is not mandatory for other scenarios, and time traceability shall be ensured through regular calibration of the local clock of the device.

### Device Integration And System Interface Requirements

### Device integration requirements

Device compatibility: The device should be capable of interconnecting with devices of different brands and models, supporting multiple communication protocols to adapt to various application scenarios. Meanwhile should comply with electromagnetic compatibility (EMC) standards to avoid mutual interference with radar, AIS and other equipment, surrounding environmental equipment and homogeneous sensors.

Data integration and standardization: The goal of data integration is to achieve non-redundant storage and efficient access to data. All collected data should be converted into a unified standard format to ensure data quality and availability.

Protocol support: The device should support multiple communication protocols to facilitate data interaction with different gateways and cloud platforms. Equipment integration needs to solve the problem of interface with devices of various models and protocols.

Customization of business logic: During the integration process, it may be necessary to customize the application logic based on specific business requirements to achieve functions that are more flexible.

### System interface requirements

The core of the IoT interface lies in ensuring high security and low power consumption, while achieving stable and efficient two-way data interaction and command control between a large number of devices and the cloud through lightweight and asynchronous communication protocols.

- Validity

The interface of the Internet of Things (IoT) system shall accurately and timely implement its functions to ensure the correctness of data transmission and command execution. Specifically as follows:

a)  The interface shall ensure the real-time transmission of data and guarantee that the system can respond promptly to all kinds of events.

b)  The interface shall ensure accurate data transmission and guarantee that the information obtained by the receiving end is error-free.

- Reliability

The interface of the Internet of Things (IoT) system should be capable of maintaining stability and continuity under the corresponding working environment and operating conditions. Specifically as follows:

a)  The interface shall operate continuously and stably under the corresponding working environment and load conditions, and the fault-free operation time should meet the predetermined requirements.

b)  The interface shall have a fault-tolerant mechanism to ensure operability when some systems or components are unavailable.

c)  The interface should be equipped with redundancy. When there is a failure in the interface connection, a backup connection should be enabled to maintain the normal operation of the interface.

- Security

The interface of the Internet of Things (IoT) system should have the ability to protect the transmitted data from unauthorized access, tampering or loss. Specifically as follows:

a)  The interface shall ensure the authenticity and integrity of data transmission and guarantee that the receiving end acquires all true information.

b)  According to different confidentiality requirements, the interface shall adopt corresponding encryption technology to protect data transmission and prevent data from being stolen or tampered with during the transmission process.

c)  The interface shall implement strict identity verification and authorization mechanisms to ensure that only authorized users can access sensitive data and operate the interface;

d)  The interface should be equipped with a security audit and monitoring mechanism, which can promptly detect and respond to and handle external attacks or faults that occur within the system.

- Maintainability

The interface of the Internet of Things (IoT) system should have the capabilities of configuration management, maintenance and upgrade. Specifically as follows:

a)  The interface shall support parameter configuration and firmware (or driver) updates, and it is advisable to provide a configuration management interface;

b)  Interface updates and upgrades shall be backward compatible to ensure that the new version is compatible with and supports functions and data of the old version;

c)  The interface shall have a version management function to ensure that both updates and upgrades have version numbers.

- Scalability

The interface of the Internet of Things (IoT) system should adapt to different usage scenarios and future demand changes, and possess flexibility and easy scalability. The interface design should allow for the support of new functions and requirements by adding parameters and other means.

### Enclosure And Housing Requirements

Add considerations for anti-biofouling to ensure long-term stability of data acquisition by underwater sensors: Sensor probes (especially temperature and salinity sensors for underwater applications) shall be designed with anti-biofouling measures (e.g., antifouling coating, ultrasonic antifouling or mechanical wiping device) to ensure long-term stability of data acquisition.

## Chapter four--CYBERSECURITY RISK MITIGATION

IoT systems and data are exposed to risks from various threat vectors including adversarial attack, system failure, data loss or corruption and simple human error. The following measures for improving resilience against deliberate and accidental cyber risks should be considered:

### Early stage of system deployment

- Identify critical data and systems, network connections and access points. Consider forming a written cyber security policy for risk mitigation which incorporates data back-up, continuity of operations planning and restoration management.

- All relevant modules and systems shall be assessed on the risk of cyber incidents to determine appropriate and proportional risk control(s) that Administrations should take.

### System Operation and Data Transmission

- Develop and implement a policy for physical key management to ensure proper authorization and logging.

- Implement strict security protocols with network segmentation and isolation;

- Apply authentication and encryption on the entire communication link to ensure the integrity of the data transmitted.

- Implement and maintain strict authorized user identification and access protection. Apply authentication where possible to be able to validate that data is actually originating from the device or system, and provide mariners with a way to verify the authenticity of the signals.

- Limit the number of network connections and data access points to only those necessary for routine and contingency operations.

- Disable unnecessary features of systems. Hydrographic sensors may be programmable by Bluetooth. Make sure the Bluetooth is disabled when not programming the sensor, and that this action is logged for reuse by the next sensor. Also, any unused (network) interfaces should be disabled at unmanned locations.

- Whenever possible, leverage data-management systems and programs which are user friendly and incorporate functions such as graphical displays that help prevent human error and allow users to identify and correct erroneous or missing data..

### System Update and Backup

- Establish procedures for the rapid recovery and/or replacement of sensors.

- Implement software/firmware updates and (security) patches on AtoN IoT sensors, but only after thorough acceptance testing;

- Conduct periodic penetration tests and/or vulnerability scanning to verify network cyber resilience.

- Documentation of AtoN information and configuration should continuously be verified to be accurate and up to date.

- Ensure that backups of firmware, software and configuration are periodically made and that their integrity is verified. Create new backups immediately following any change in software, firmware or configuration. Ensure proper protection, with regards to Confidentiality, Integrity and Availability (“CIA”) of backed-up information and documentation.

- Where possible, collect and save log files of all (sensor) information from AtoN for a minimum of  180 days to enable (forensic) inspection following abnormal behavior and cyber incidents.

## Chapter five-- Installation and Maintenance of Sensor Equipment

Add requirements of lightning protection for sensors to Section 5.1 INSTALLATION CONSIDERATIONS. Lightning protection devices shall be installed at the antenna feeder, power supply cable and other connection terminals connected to the sensors. The connection of lightning protection devices shall not affect the normal operation of the system.

Add the requirement for lifetime log management to Section 5.2 MAINTENANCE BEST PRACTICE to realize the standardized management of the full lifecycle of IoT sensors and provide data support for scientific operation and maintenance: A history log shall be established to record activities of IoT sensors including the installation, maintenance, adjustment, decommissioning, and remote operations that change the characteristics of AtoN. Records shall be made in a timely, accurate and detailed manner to facilitate scientific management.

# References

G1179 An introduction to the Internet of Things from an IALA perspective

IEC 63173-2-2022 Maritime navigation and radio communication equipment and systems - Data interface

G1182 Cyber security specifics from an IALA perspective

Requirements for Interfaces of Internet of Things Systems of Chinese Standard (GB/T 35319-2025)

G1190 Harmonised Internet of Things (IoT) Protocol For Visual AtoN

# Action requested of the Committee

The Committee is requested to:

- consider the proposal in section 3.

And incorporate the proposed chapter contents into the draft guideline of Use of Simple IoT Sensors on Physical AtoN if deemed appropriate.

Footer will automatically populate