Input paper:   ARM22-7.4.11

Input paper for the following Committee(s): 		Purpose of paper:

(Select as appropriate)

☒ ARM	☐  ENG		☐ PAP			☒ Input

☐ ENAV	☐ VTS					☐ Information

Agenda item 			7.4

Technical domain/ Task number 2	….

Author(s)/Submitter(s)			IHO NIPWG Chair

# IHO S-125 progress report

# Summary

This is a progress report on the actions related to S-125 within IHO.

# Background

Although S-125 is owned by NIPWG, the specification has, for many years, been developed by IALA ARM, as a subset of S-201. In 2025, a release candidate (RC) of S-125 1.0.0 was returned to IHO NIPWG.

Upon receiving the S-125 1.0.0 RC, IHO NIPWG regained an active role in regards of development of S-125. An S-125 Task Group (TG) was established, and expert contributors from both IHO NIPWG and IALA ARM were invited. S-125 TG Task Group lead is Caroline Johansson (Sjöfartsverket, Sweden) and co-lead is Dave Lewald (US Coast Guard).

S-125 TG reports to IHO NIPWG, and the main task is to finalize and support the IHO approval process of S-125. In March 2026, S-125 edition 1.0.0 was submitted to HSSC 18 for approval. Within IHO, edition 1.0.0 is the first recognized publication of a specification. This edition is aimed for extensive testing by stakeholders, before the production-ready (2.0.0) version is published.

Edition 1.0.0 of S-125 was submitted for approval by HSSC 18 (meeting in Poland, May 18-22 2026). As the edition 1.0.0 does not need Member state approval according to IHO resolution 2/2007, this edition can be approved by the HSSC- meeting.

Edition 1.0.0 is not yet a production-ready version, and this version is not yet considered mature enough for inclusion in ECDIS. It is assumed active development work continues towards the approval of a 2.0.0 version. Approval of the 2.0.0 version will need endorsement by HSSC followed by a Member State vote.

A release in GitHub reflects the "final" version which was submitted to HSSC;

https://github.com/iho-ohi/S-125-Product-Specification-Development/releases/tag/ed1.0.0

# Discussion

When established, S-125 TG was faced with some quite complex questions. The first big discussion was related to use-cases, and which use-cases to support. For the 1.0.0 edition of S-125, some temporary modifications were needed, as the discussion needs more time to mature.

## Technical validation and aligment to IHO GI-registry

During technical validation of S-125, concept-entries were added to IHO registry, and changes were made to the machine-readable files to reflect those changes. Definitions of concepts and casing of entries were updated according to IHO registry. This validation process has resulted in changes to the data model, which might be considered also in S-201.

## Multiple Use-cases and the ecdis use-case

Although multiple use-cases are identified and can be supported, the IHO specification shall focus on specifying use of S-125 within ECDIS / ENDS. S-125 is recognized in the ECDIS interoperability specification S-98, as an optional product. S-125 is optional, because it cannot be defined as mandatory in ECDIS before the production ready 2.0.0 edition is published. Mandatory / optional in this regard refers to whether a type-approved ECDIS must or may support display of S-125. S-98 only defines the requirements on technical capability of an ECDIS, and this does not relate to any need to use such a product when navigating.

## portrayal and reduced interoperability

HSSC 17 decided to reduce interoperability in the 1st Generation S-100 ECDIS. It is still assumed a 2nd Generation S-100 ECDIS will have extended interoperability. Therefore, there is a need to support ECDIS use in 1st Generation ECDIS (overlay) and consider future support for 2nd Generation ECDIS use (interoperability).

Even though ECDIS is the main use-case, support for secondary use-cases and alignment with S-201 (and S-101) shall be considered as far as practicable. In order to support ECDIS use as an overlay, some modification had to be made to the data model.

The current data model was left intact, but the old extensive portrayal had to be simplified. A new feature "AtonStatusIndication" and related simplified portrayal of only status indication was used instead.

This means, that S-125 1.0.0 RC can contain all the data, but only AtonStatusIndication is used in ECDIS to drive portrayal. The benefit of this approach is, that S-125 can support both use as an overlay in ECDIS and secondary use-cases. The use of the full data model in ECDIS will need further alignment between S-125 and S-101 (which also means consideration of S-201). There are still differences between S-101 and S-201, which must be further discussed.

## change types and the ecdis use case

Change types were discussed by the S-125 TG. The need for a new change type "Permanent Change" was identified, and such type added to S-125 Change Types. Anyway, due to restrictions related to ECDIS portrayal, the acronym PC would become similar for both the existing item Proposed Change and the new item Permanent Change. Alternative names for permanent change were considered, but it was decided to keep permanent change as most descriptive, and due to the registered entry.  It was determined, that a Proposed change, although important in some use-cases, might be of less importance in ECDIS- use, and therefore the new entry Permanent Change was given the PC- acronym in portrayal.

# Action requested of the Committee

The Committee is requested to:

- Note the progress of S-125 1.0.0 approval process

- Note the discussions related to S-125 use-cases and ECDIS portrayal

- Provide any feedback to NIPWG / S-125 TG

- Consider the impact on S-201 an consider the future management processes to ensure needed harmonization between S-101, S-125 and S-201

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain