Input paper:   ARM22-7.5.2

Date: 30 Mar 2025

Input paper for the following Committee(s): 		Purpose of paper:

(Select as appropriate)

☒ ARM	☐  ENG		☐ PAP			☐ Input

☐ ENAV	☐ VTS					☒ Information

Agenda item 			7.5

Technical domain/ Task number 2	7.1.14

Author(s)/Submitter(s)			Tuomas Martikainen, Finnish Transport Infrastructure Agency

# results from testbed for aton information service

# Summary

This paper presents the implementation and testing of the Finnish Aids to Navigation (AtoN) Information Service based on the IALA S-125 Technical Service Specification for the provision of AtoN information.

The piloting of the service included bridge simulator tests, onboard trials, and technical field measurements.

## Purpose of the document

The purpose of this document is to provide information on the implementation, testing, and performance of the AtoN Information Service and to support the ongoing development of the IALA Technical Service Specification for the provision of AtoN information.

## Related documents

Marine Aids to Navigation (AtoN): Technical Specification for the provision of AtoN information to end-users.

Product Specification for S-125 Marine Aids to Navigation (AtoN)

# Background

The Finnish Transport Infrastructure Agency (FTIA) has conducted piloting of an AtoN Information Service based on the IALA Technical Service Specification for the provision of AtoN information.

The work was carried out as part of the EU Interreg Baltic Sea Region co-funded MaDaMe project.

The pilot activities conducted in 2025 comprised of:

- Bridge simulator tests at Novia University of Applied Sciences

- Onboard tests on M/V Finnsirius

- Technical field tests and mobile measurements onboard an FTIA fairway maintenance vessel

# Discussion

## Implementation of Finnish Aids to Navigation (AtoN) INformation service

The implemented pilot service is based on the IALA Technical Service Specification for the Provision of AtoN Information, version 1.0, released in June 2024. This specification offers a holistic, technology-independent overview of the service and its core components. It outlines the key aspects at a logical level, including the operational and business context, service description, and the processes for service provision and validation.

A more detailed service design is still under development. The piloted service developed during the MaDaMe project is based on SECOM.

### Converting data from legacy DB to S-125 data format

AtoN data in Finland is maintained in FTIA’s Waterway Registry. This legacy system is not directly compatible with S-201/S-125 formats.

The initial development phase included:

- Analysis of compatibility between the legacy data model and S-201

- Implementation of data conversion services within FTIA’s Service-Oriented Architecture (SOA)

The service converts legacy data into S-201/S-125 format and stores it in a cloud-based database.

Due to the large volume of data (approx. 25,000 AtoNs), datasets were divided into smaller entities. In the pilot phase, datasets were structured by fairway. The service currently includes approximately 200 datasets covering merchant shipping fairways (fairway classes 1 and 2) along Finnish coast, but it is worth noting that service consumers can retrieve multiple datasets at once—for example, based on the port of destination.

### Overview of S-125 service architecture

The S-125 service consists of three main components:

- Operational AtoN register (Waterway database)

- SOA REST service for data conversion

- S-125 producer providing SECOM-compatible interfaces

Figure 1 FTIA's S-125 Service Architecture Overview Diagram

The service has been implemented as a cloud service, which includes its own instances for production and test data.

### Message Flow

AtoN data is maintained in FTIA operational systems and stored in the legacy database.

The S-125 service:

- Monitors changes at regular intervals (currently every 5 minutes)

- Updates the S-125 database accordingly

- Distributes updated datasets to subscribed SECOM consumers

Detailed message flow is depicted in the sequence diagram below:

Figure 2 Sequence Diagram of information flow in the S-125 service

### S-125 Datasets used in the tests

As the S-125 Product Specification is still under development, test datasets were based on Product Specification S-201 Edition 2.0 using only attributes aligned with S-125.

Datasets were structured by fairway and could be queried by:

- Fairway identifier or name

- UN/LOCODE of the destination port

During the pilots onboard Electronic Chart System (ECS) received full datasets, while only AtoNs with status changes were displayed in the ECS.

Figure 3 Screenshot of ECS dialog for dataset subscriptions

## bridge Simulator AND onboard tests

In addition to the S-125 AtoN information also S-124 Navigational Warnings were tested during the bridge simulator and onboard tests to evaluate overlaps and usability of different technical services.

Simulator scenarios included situations where:

- Buoys were temporarily repositioned

- Lights were extinguished

- Fairway conditions were altered

Information was delivered via both S-124 and S-125 services.

Figure 4 Screenshot of ECS dialog for AtoN discrepancies

Onboard tests were conducted between Naantali (Finland) and Kapellskär (Sweden) under normal operational conditions without affecting navigation.

Feedback was collected through:

- Observation

- Interviews

- Questionnaires

Participants included experienced navigation officers, masters, and pilots with seagoing experience ranging between 10-40 years.

Results:

- Overall feedback was positive

- Integration of AtoN status information and Navigational Warnings into ECS/ECDIS improved situational awareness

- Minor concerns were raised regarding symbology and colour usage, particularly for Navigational Warnings

Figure 5 Results from questionary

Figure 6 Results from questionary

## Technical tests and mobile measurements

Field tests were conducted onboard an FTIA’s fairway inspection vessel in the Archipelago Sea using 5G connectivity.

The main goal of these tests was to validate the promulgation of up-to-date AtoN status information in real operational environment using the technical service for the provision of AtoN information, especially the ‘subscription’ -functionality of the service.

Key findings:

- Near real-time transmission of AtoN updates was achieved

- Subscription functionality minimized data transfer volume

- Network coverage was generally sufficient

Performance results:

- All datasets covering all merchant fairways in the Finnish coast (202 datasets, ~6000 AtoNs): 2.7–56.5 seconds

- Single dataset, one fairway (23-172 AtoNs): 0.2–2.8 seconds

- Subscription updates: 0.13–0.26 seconds

These results confirm that up-to-date AtoN information can be delivered to ships using the Technical Service for the provision of AtoN information using IP-connections, providing that the area in which vessels are sailing has sufficient network coverage.

## conclusions

- Nautical charts do not include detailed AtoN status information, yet such data is critical for safe navigation. Discrepancies—such as unlit or off-position AtoNs—should be delivered to vessels in near real-time.

- Only changes in AtoN status need to be transmitted onboard. Re-transmitting data already present in enc’s is unnecessary.

- S-125 information can be presented as an additional chart layer, provided symbology is harmonized

- Overlap with Navigational Warnings was not found to be problematic

However, it should be noted that after these pilots were concluded the use cases for S-125 and the status types of AtoN’s have been further developed. This development is inline with the results of these pilots, particularly regarding the focus on real-time status updates.

# Action requested of the Committee

The Committee is requested to:

Note the information provided; and

Take the findings into account in the further development of the IALA Technical Service Specification for the provision of AtoN information.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain

Footer will automatically populate