Input Paper:   ARM22-8.1.2

Input paper for the following Committee(s):  		     Purpose of paper:

(check as appropriate)

☒ ARM	☐  ENG		☐ PAP		      	             ☒Input

☐ DTEC	☐ VTS				                             	☐ Information

Agenda item 			 	8.1.2

Technical Domain / Task Number 	1.4.7

Author(s) / Submitter(s)			 	China Maritime Safety Administration (China MSA)

# A Risk Assessment Method Based on DCPA/TCPA

# Abstract

China MSA has always attached great importance to the assessment of navigation environment risk. This paper introduces a risk assessment method based on Distance to Closest Point of Approach(DCPA)/Time to Closest Point of Approach(TCPA) monitoring technology developed by China MSA, along with the practical experience base on this risk assessment tool, aiming to provide proposal for further enriching the IALA Risk Management Toolbox.

## Purpose of Paper

To provide a risk assessment method based on DCPA/TCPA monitoring technology, as a potential candidate tool for the IALA Risk Management Toolbox.

### Related Documents

- ARM21-12.1 Report of ARM21

- ARM21-9.1.1 Maritime Hydro-Meteo Fusion Index (MHFI)

### 2.  Background

In recent years, China MSA has been working on the development of a risk assessment method based on DCPA/TCPA monitoring technology and has applied it in practice in the coastal waters of the Pearl River Estuary. Collision risk assessments have been conducted for encounters between cargo vessels and fishing vessels, as well as between cargo vessels. The risk assessment results have generally been consistent with historical accident data, showing that it is valuable in waterway risk assessment.

### Discussion

In the fields of vessel collision avoidance and Vessel Traffic Services (VTS), DCPA and TCPA are quantitative indicators widely used to measure the collision risk between vessels. The combination of these two parameters allows for precise judgment of the collision risk, and is often a key basis used for decision-making by officers, pilots, or VTS operators in collision avoidance.

DCPA refers to the closest distance of approach between two vessels if they maintain their current speed and course. A smaller DCPA value indicates a closer passing distance and a higher collision risk.

TCPA refers to the time to closest point of approach between two vessels if they maintain their current speed and course. It measures the time urgency of the collision risk. A smaller TCPA value indicates less time until the closest point of approach, necessitating immediate collision avoidance action.

### 3.1. Risk Assessment Tool Based on DCPA/TCPA Monitoring Technology

3.1.1.  Theoretical Model

During an encounter between vessel i and vessel j, DCPA and TCPA values change as the encounter situation evolves. When both DCPA and TCPA fall below set safety thresholds, the two vessels are identified as being at risk. The collision risk between vessel i and vessel j at time t can be expressed as:

Riskijt=h(g(DWTi, VTi, DISPj, VTj),e(VISt,windt,currentt),f(DCPAijt,DCPAth_ij,TCPAijt,TCPAth_ij))

Where:

g represents the risk coefficient function related to the displacement (DISP) and vessel type (VT) of vessels i and j.

e represents the risk coefficient function related to visibility (VIS), wind speed, and current.

f is the risk function concerning the DCPA and TCPA variables.

DCPAijt is the DCPA value between vessel i and vessel j at time t. A smaller DCPA leads to a higher risk value.

DCPAth_ij represents the threshold value for the rate of change in DCPA between vessel i and vessel j. If the change in DCPA per unit time exceeds this threshold, the risk increases.

TCPAijt is the TCPA value between vessel i and vessel j at time t. A smaller TCPA leads to a higher risk value.

TCPAth_ij represents the threshold value for the rate of change in TCPA between vessel i and vessel j. If the change in TCPA per unit time exceeds this threshold, the risk increases.

3.1.2.  Risk Determination

When assessing the risk of collision between vessels, it is necessary to set the minimum search time, "safe approaching time" and "safe approaching distance", the safe change rate of DCPA and the safe change rate of TCPA.

For vessel i, set a search radius and take the greater of the search radius and vessel i's speed multiplied by the minimum search time as the collision risk assessment range. When vessel j enters this risk range, begin calculating the DCPA and TCPA between the two vessels. A collision risk is determined when TCPA falls below the configured “safe meeting time” and DCPA is less than the safe meeting distance. The safe meeting distance between Vessel i and Vessel j is calculated as: Speed × (Length of Vessel i + Length of Vessel j) × 0.25.

During the encounter between vessel i and vessel j, if the rate of decrease in DCPA or TCPA values exceeds safe change rate, the risk value increases; conversely, it decreases. The maximum risk value during the entire encounter is taken as the risk peak for the encounter between the two vessels.

3.1.3.  Risk Plotting

The collision risk peak points generated during an encounter between vessel i and vessel j are plotted on a nautical chart using a color code: green (risk peak 1-4), yellow (risk peak 4-7), orange (risk peak 7-13), and red (risk peak >13). This generates a distribution map of regional vessel collision risk, as shown in the figure below.

Figure 1 The distribution of risk points

All risk points are aggregated using a 7-precision geohash grid (approximately 150m x 150m). The average risk value for each grid (total risk value / total number of points) is calculated. A heat map generated based on these average values is then overlaid on the nautical chart, providing an intuitive visualization of the regional vessel collision risk distribution, as shown below.

Figure 2 The heat map of risk points

3.1.4.  Risk Assessment Process

Based on the theoretical foundation above, China MSA has developed a web-based vessel collision risk assessment tool. This tool can monitor vessel collision risk situations within a selected area over a specific period. The main assessment process is as follows:

- Define the assessment area and time period;

- Set parameters, including search range, and minimum DCPA/TCPA thresholds;

- Select the vessel types to be assessed, allowing for risk assessment between different types as required;

- Generate a distribution map of collision risk points;

- Propose risk mitigation measures.

Additionally, the tool can assess the collision risk levels of different vessel types within a selected area over a specified period, and rank collision risks across different areas for the same period. Functions for assessing grounding and contact risks are also under development.

### 3.2. Application Case

Since October 2025, China MSA has regularly used this risk assessment tool to analyze the collision risk between merchant vessels and fishing vessels in high-risk areas along the coast of Guangdong Province. One of the cases is presented below.

3.2.1.  Area Overview

The assessment area covers 168.5 square kilometers, located south of Biaojiao, Shantou City, Guangdong Province. It is near busy fishing ports such as Haimen Fishing Port and Nan'ao Fishing Port, where inbound and outbound merchant vessels and fishing vessels intensively converge. This area is one of the 38 "high-risk warning zones for merchant vessel-fishing vessel collisions" along the Chinese coast published by China MSA, and collisions in this area have been frequent in recent years.

Figure 3 the study area

3.2.2.  Assessment Process

Selecting the above area as the assessment area and January 2026 as the assessment period, the collision risk between merchant vessels and fishing vessels within this area and period was assessed. After setting the assessment parameters, the cumulative number of merchant vessel-fishing vessel collision risk points generated was 1,032. This included: 1 red risk point (0.1%), 12 orange risk points (1.16%), 116 yellow risk points (11.24%), and 903 green risk points (87.5%).

By plotting the risk points on the nautical chart, it can be intuitively observed that the merchant-fishing vessel collision risk points in this area are concentrated south of the 20-meter isobath, within a 0.8 nautical mile range on either side of the central line of the coastal route. Dense risk areas extend to the northeast and southwest waters, as well as near the Guang'ao Terminal and the entrance/exit channels of Shantou Port.

3.2.3.  Risk Mitigation Recommendations

Recommendations for preventing merchant-fishing vessel collision risks include:

- Strengthen the organization and guidance of merchant and fishing vessels in high-risk collision areas. Use VHF, SSB, and other means to coordinate safety reminders and broadcast relevant safety information to vessels in the area.

- Promote the above risk analysis results and accompanying charts to shipping companies, advising them to optimize vessel routing and operational planning to avoid trackpotential high-risk collision areas as much as possible.

- Enhance the maintenance and operational status monitoring of AtoN in high-risk merchant-fishing vessel collision waters to ensure the stability and reliability of AtoN services.

- Vessels navigating through high-risk areas should keep vigilant lookout, paying particular attention to nighttime fishing vessel operations that may affect navigation.

- Conduct outreach and education targeting fishing companies, fishing vessels, and fishermen to raise awareness of relevant safety risks.

## 3.3.  A Valuable Candidate to the Risk Assessment Toolbox

Compared to the current IALA risk assessment tools, the tool based on DCPA/TCPA monitoring technology differs fundamentally. By monitoring changes in DCPA/TCPA and incorporating factors such as vessel type, speed, load, and hydro-environmental conditions, it quantifies the risk of incidents occurring and can serve as a new candidate tool within the current IALA Risk Management Toolbox. This risk assessment tool offers the following advantages:

- Compared to the IWRAP risk assessment tool, this tool does not rely on specialized personnel to define routes or estimate probability factors, making it simpler and more convenient to operate;

- The risk assessment results are plotted using different colors, allowing for direct visualization of the waterway's risk distribution on electronic nautical charts;

- Beyond application in the AtoN field, DCPA/TCPA monitoring technology is widely used in marine domains such as vessel collision avoidance and VTS management, giving it stronger compatibility;

- This DCPA/TCPA-based monitoring technology can serve as a potential tool to support collision avoidance for Maritime Autonomous Surface vessels (MASS).

## 3.4.  Work plan for the Risk Tool in China

Currently, the preliminary application of this tool in China has yielded positive results. Optimization of the tool is ongoing. In the next stage, China MSA will focus on refining the algorithm model based on DCPA/TCPA monitoring, enhancing the algorithm for the influence of hydro-environmental factors, improving the risk visualization interface, and exploring its application in the analysis of vessel grounding and contact accidents.

### 3.5.  Proposal

It is proposed that the Committee take note of the content of this paper, and consider the probabilities of inclusion the new risk assessment tool during the development of IALA Risk Management Toolbox.

### 4.  References

N/A

### Action Requested of the Committee

- The Committee is invited to consider the proposal in section 3.5 and take appropriate action.