Input paper:   ARM22-7.4.2

Input paper for the following Committee(s): 		Purpose of paper:

(Select as appropriate)

☒ ARM	☐  ENG		☐ PAP			☒ Input

☐ DTEC	☐ VTS					☐ Information

Agenda item 			7.4

Technical domain/ Task number 2	7.1.4

Author(s)/Submitter(s)			 China Maritime Safety Administration (China MSA)

# Proposals for the Revision and Clarification of IALA S-201

# Summary

IALA has finalized the development of S-201 and released Version 2.0 of S-201 in April 2025. Currently, S-201 has entered the application and testing phase, with the product specification hosted on GitHub to solicit public feedback. During the research and testing of this specification, the China Maritime Safety Administration (MSA) identified several issues and developed corresponding revision recommendations. These are hereby submitted to this meeting for deliberation, and the relevant comments will also be submitted to GitHub for further consultation.

## Purpose of the document

This proposal aims to revise and improve the IALA S-201 Product Specification, clarify and standardize technical details in the document, and provide a reference for the Committee in carrying out standard revision work.

## Related documents

- IALA S-201 Aids to Navigation Information Product Specification, Ed.2.0.0, 2025

- ITU-R M.1371-6  Technical characteristics for VHF automatic identification system using time division multiple access in the maritime mobile service, 2026

# Background

The S-201 Aids to Navigation (AtoN) Information Product Specification provides a universal structure for the exchange of AtoN information. The specification applies to various types of AtoN, including lighthouses,  buoys, RACONs, AIS AtoN, and sound beacons, as well as their related information such as position, attributes, and operational status. The S-201 Product Specification can be used by AtoN authorities, hydrographic organizations, and other organizations (including commercial and professional organizations) to exchange AtoN information in a consistent format. IALA has officially released S-201 Edition 2.0, which constitutes the first operational version of the specification. Meanwhile, the newly released ITU-R M.1371-6 standard in February 2026 officially supplemented the technical operational characteristics of "Mobile AIS AtoN", which requires synchronous support from S-201 in its AtoN information exchange architecture.

# Discussion

## Discussion on the Application Modes of AIS AtoN

Section 4.2 of the S-201 main document specifies that Virtual and Synthetic AIS AtoN use a RadioStation as the broadcasting station, and describes their broadcasting logic via association relationships. However, it fails to define the relationship between Physical AIS AtoN and the RadioStation. In addition, Figure 4 in the main S-201 document defines the associations among virtual, physical and synthetic AIS AtoN and RadioStation, , with the multiplicity at the RadioStation end for all AIS AtoN types is (0..1), which conflicts with actual AIS mechanisms. In practical applications,  Virtual/Synthetic types rely entirely on RadioStation to broadcast, By contrast, a physical AIS AtoN has autonomous transmitting capability (i.e., it is an independent AIS station itself). The relationship there is logical/management-based (e.g., VDL channel coordination or monitoring by a Base Station) rather than a physical broadcast dependency. The existing UML diagram should be revised and improved to avoid ambiguity.

Recommendation:

1.Add restrictive notes in Section 4.2 or the UML diagram to clarify that for Physical AIS AtoN, the RadioStation acts as a Base Station (logical management) rather than a Broadcasting Station, and clearly stating that the PhysicalAIS association only represents a logical management relationship.

2.Change the multiplicity of VirtualAISAidToNavigation and SyntheticAISAidToNavigation at the RadioStation end from (0..1) to (1..*).

AIS Aids to Navigation

## Discussion on adding Mobile AIS AtoN in the AIS AtoN model

In February 2026, the International Telecommunication Union (ITU) released Recommendation M.1371-6, which officially introduced the concept and related technical characteristics of "Mobile AIS AtoN". However, the current AIS AtoN model in S-201 has not yet covered this new type of AtoN with dynamic positional characteristics. If they are not specifically defined and modeled, receiving end systems could easily misinterpret the Mobile AIS AtoN, whose real-time positions are constantly changing, as a fixed AtoN that is off-position, or even as marine hazards, thereby triggering continuous false alarms.

Recommendation:

Incorporate operational logic support for 'Mobile AIS AtoN' in Figure 4. AIS Aids to Navigation, considering establishing it as an independent geographic Feature Type parallel to Physical, Virtual, and Synthetic AIS AtoN. Furthermore, it is recommended to further study and implement specific data modeling, attribute classification refinement, and spatial geometry encoding rules for Mobile AIS AtoN in subsequent version revisions.

## Definition of Bridge Features

There is a geometric inconsistency between the main document (Figure 2/Annex C), which labels Bridge as noGeometry, and Annex A (Section 6.8), which defines it as Point, Curve, or Surface. This causes confusion for data producers and risks interoperability failures. In practice, bridge aids to navigation (e.g., marks indicating the best point of passage for navigable spans, or prohibited transit marks) are highly specialized in both function and shape. Since a bridge is an independent structural element, the existing options under the 'Category of special purpose' attribute for the Daymark feature are insufficient to intuitively convey their core semantics of 'bridge guidance'.

Recommendation:

1.Amend Figure 2 by removing the “no geometry” label  next to  the Bridge feature.

2. Update the Feature Catalogue, redefine Bridge as a geographic Feature Type with spatial attributes, so as to maintain consistency and machine readability of the standard.

3. Add bridge mark to the code list for the “Category of special purpose mark” attribute in section 8.10 in Annex A of S-201, to identify port/starboard span marks, best points of passage, span restriction markers and pier warning marks.

## Adaptability of Cardinal Buoys in Special Purpose Models

In the Figure 7 ("Mooring buoy & emergency wreck buoy"), currently CardinalBuoy is included in the model design, which conflicts with maintenance logic, as Cardinal Buoys are used to indicate navigable water directions, whereas Mooring and Emergency Wreck buoys serve highly specific, non-directional purposes. In the current model, this may easily lead to ambiguous data expression and logical conflicts.

Recommendation:

Clarify the role of this element in Figure 7 within the product specification. If it is only intended as a reference to a generic mark shape, it is recommended to remove CardinalBuoy from this specific -purpose of UML diagram to ensure logical rigor in functional classification.

Mooring buoy & emergency wreck buoy

# References

IHO S-100 Universal Hydrographic Data Model, Edition 5.2.0, 2024

IALA S-201 Aids to Navigation Information Product Specification, Ed. 2.0.0, 2025

ITU-R M.1371-6  Technical characteristics for VHF automatic identification system using time division multiple access in the maritime mobile service, 2026

# Action requested of the Committee

The Committee is requested to consider the document and provide clarifications and amendments based on the recommendations provided.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain

Footer will automatically populate