Input paper:  ARM22-7.4.5

Date: 13 April 2026

Input paper for the following Committee(s): 		Purpose of paper:

(Select as appropriate)

☒ ARM	☐  ENG		☐ PAP			☒ Input

☐ ENAV	☐ VTS					☐ Information

Agenda item			7.4.5

Technical domain/ Task number 	7.1.3

Author(s)/Submitter(s)			Canadian Coast Guard

# Proposal to Add “Cautionary” as Part of the Enumerated List of “Category of Special Purpose Mark” in S-201

# Summary

The Canadian Coast Guard (CCG) is currently working on implementing the S-201 Product Specifications.

They are facing issues regarding their terms of business not being well reflected in what is available in the specifications. CCG has evaluated that certain terminologies can be renamed and reworked, but others are embedded in Canadian law and therefore cannot be approximated without additional explanations to the mariner to bridge the gap.

## Purpose of the document

S-201 is at Edition 2.0.0 and is to be updated following the work on S-125. CCG would like to propose the addition of “cautionary” to be added to “category of special purpose mark” for the next edition of S-201.

## Related documents

- IALA S-201 Aids to Navigation Information Product Specification, Ed. 2.0.0, 2025

- IALA S-201 AtoN Information GitHub

# Background

CCG, in conjunction with Transport Canada, determines the categories of special aids to navigation (AtoN). These are defined in the Vessel Operation Restriction Regulations (VORR) which fall under the Canada Shipping Act, 2001. The VORR governs the markings of any buoy used to restrict navigation, e.g. speed limits, keep-out areas, etc. Cautionary AtoNs, particularly buoys, are described in this document.

Cautionary buoys are defined as marking an area where mariners are to be warned of:

aquaculture facilities;

dangers (such as firing ranges, racing courses, seaplane bases, and underwater structures);

areas where no safe through channel exists; and

traffic separations.

A cautionary buoy is coloured yellow and displays identification letter(s).

If it carries a topmark, the topmark is a single yellow “X” shape.

If it carries a light, the light is yellow and has a flashing (Fl) light characteristic with a signal period of four seconds (4 s).

If it carries retroreflective material, such material is yellow.

Drawing of a Cautionary Buoy with its Light Characteristics

# Discussion

## Consideration of Cautionary as their own Class of AtoN

In Canada, cautionary AtoN have predetermined and specific characteristics. Although some functionality of cautionary buoys can be mapped to more specific options found in the “category of special purpose mark” (e.g. traffic separation and firing ranges), not all cautionary buoys require that specificity.

“Cautionary” clearly identifies a middle road between the highly defined choices of special purpose marks and the catch-all “general warning mark,” which on charts is often applied to certain Canadian AtoN due to the lack of other applicable options.

The defined flash characteristics and colour of this AtoN provides additional information that would not be shared with other special purpose general mark AtoNs, e.g. white beacons with white lights.

Cautionary AtoN is one example that CCG would like to put forward. With that consideration, the proposal of a way forward to include additional entries to the “category of special purpose mark” enumerated list is also worth discussing as there are other examples from the CCG that could be defined and added to it.

## Consideration of a Process for Discussions Leading to Proposals to be Incorporated into Future S-201 Editions

As this standard matures over the course of testing and implementation by the various member states and interested stakeholders, there should be a process for discussing and proposing modifications for future editions of S-201. The IALA S-201 AtoN Information GitHub appears to be the best place to bring up questions and incite discussion with other member states and interested stakeholders that will lead to clarity and even proposals to be incorporated into future S-201 editions.

# References

- IALA S-201 Aids to Navigation Information Product Specification, Ed. 2.0.0, 2025

- The Canadian Aids to Navigation System 2023

- Canada Shipping Act, 2001

- An Owner’s Guide to Private Buoys (Transport Canada)

- Signage Requirements and Recommendations: Vessel Operation Restriction Regulations (Transport Canada)

# Action requested of the Committee

The Committee is requested to:

- Note this input paper;

To consider adding “cautionary” to the enumerated list of “category of special purpose mark”; and

To consider the IALA S-201 AtoN Information GitHub as a way to bring up questions, incite discussion with other members states and interested stakeholders, and lead to proposals to be incorporated into future S-201 editions.