Input paper:   ARM22-7.4.9

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

X  ARM	□  ENG		□   PAP					X  Input

□  DTEC	□  VTS							□  Information

Agenda item 			7.4

Technical Domain / Task Number 2	7.1.13

Author(s) / Submitter(s)			Sewoong OH (KRISO)

# Guidance on the symbology and portrayal of AtoN for other use than navigational charting

# Summary

The ARM Committee has defined Task 7.1.13 for the development of symbols and portrayal guidance for AtoN beyond charting purposes. While symbols and portrayal for AtoN management purposes are required, there has been no established IALA guideline to support this. Although IHO S-100 Part 9 (Portrayal) defines feature-based portrayal mechanism, it is highly technical in nature, and there is a need for technical guideline that can define symbols and portrayal for other use than navigational charting. This paper presents the components and considerations for developing symbols and portrayal for non-navigational purposes and requests discussion on approaches for developing the IALA guideline.

## Purpose of the document

This paper proposes the components and considerations for guidelines to develop AtoN symbols and portrayal methods for purposes other than charting.

## Related documents

S-100	IHO Universal Hydrographic Data Model (Edition 5.2.0, June 2024)

# Background

AtoN symbols can be displayed for AtoN management purposes, and IALA Member States have independently defined their own methods for AtoN symbols and portrayal for other use than navigational charting. As the IHO has established a feature-based portrayal mechanism, there is a need to develop guidelines that enable the definition of AtoN symbols and portrayal for non-charting purposes.

# Discussion

## Examples of AtoN Symbol and Portrayal for other use than navigational charting

Symbols for navigational charts are defined in the IHO S-4 Regulations for International (INT) Charts and Chart Specifications of the IHO, and a feature-based portrayal mechanism is defined in S-100 Part 9. AtoN authorities are responsible for AtoN management, inspection, and maintenance, and when operating information systems for AtoN management, they have independently defined their own AtoN symbols and portrayal methods. While the symbols and portrayal rules defined in S-57 ENC and S-101 ENC can be referenced when displaying AtoN types and positions on maps, there are no dedicated symbols for AtoN management purposes. Therefore, there is a need to develop IALA guidelines for non-navigational AtoN symbols and portrayal.

Examples of AtoN management purposes are as follows, and it is considered that non-navigational AtoN symbols and portrayal rules can be further developed based on future requirements:

- (Case 1) AtoN inspection cycle: Different symbol representations based on maintenance/inspection schedules—sufficient time remaining, approaching due date, or overdue

- (Case 2) AtoN change information: Classification of AtoN changes into Advance notice of change, Discrepancy, Permanent change, Proposed change, and Temporary change. While S-125 discusses simplified representations aligned with S-101 symbols, management purposes may require more intuitive and distinguishable symbol designs

- (Case 3) AtoN AIS: S-201 includes Physical, Synthetic, and Virtual AtoN, whereas S-101 ENC includes only Physical and Virtual; therefore, new symbols aligned with S-201 purposes may be introduced

- (Case 4) Major Light: Beyond identification by nominal range, development of a dedicated symbol for Major Lights for management use

- (Case 5) Monitoring status: Indication of whether AtoN is remotely monitored via AIS or maritime communication networks

- (Case 6) Public/Private classification: Symbols indicating whether an AtoN is government-owned or privately maintained

- (Case 7) Special purpose marks: Design and development of symbols suitable for frequently used special-purpose (e.g., construction) AtoN

- (Case 8) Power supply type: Representation of power sources such as shore power, batteries, or renewable energy

- (Case 9) Failure prediction: Visualization of fault scores or predictive maintenance information based on failure prediction algorithms

## IHO S-100 Part 9 - Portrayal

S-100 Edition 5.2 Part 9 defines Portrayal, This part of the standard describes a Portrayal Catalogue and its contents. The concept in this standard is that feature data is modelled with a focus on content and portrayal of a feature is accomplished using rules or functions that map the content to the appropriate symbols and display characteristics. This concept allows the same content to be displayed in different ways and allows the display mapping rules to be maintained without having to modify all the content data.

The Portrayal Catalogue contains portrayal functions that map the features to symbology it also contains symbol definitions, colour definitions, portrayal parameters and portrayal management concepts such as viewing groups. The goal in S-100 is to provide a mechanism where, for a given product, the Portrayal Catalogue can be delivered as data in a machine readable form such that a compliant implementation can display the product feature data using the given Portrayal Catalogue.

This part of S-100 defines a feature-centred function-based portrayal mechanism. Instances of features are portrayed based on portrayal functions, which make use of geometry and attribute information. The relationship between the feature instances, attributes, and the underlying spatial geometry is specified in a Product Specification based on the General Feature Model of S-100.

Portrayal information is needed to portray a dataset containing geographic data. The portrayal information is defined as drawing instructions created by specific portrayal functions. The portrayal mechanism makes it possible to portray the same dataset in different ways without altering the dataset itself.

The drawing instructions are intermediate data used by the rendering engine to produce the portrayal output. During the rendering process, the rendering engine uses the symbol definitions to create the output according to the output device. The symbol definitions contain the details of all graphical elements used for the portrayal.

Fig. 1 General Portrayal Model of S-100 Ed.5.2

## Components of Guidelines for AtoN Symbols and Portrayal for other use than navigational charting

The portrayal mechanism based on the S-100 standard can refer to S-100 Ed. 5.2. IALA guidelines for AtoN symbols and portrayal for non-charting purposes should follow the S-100 portrayal mechanism procedures, while defining the processes for symbol design and development, preparation of the Portrayal Catalogue, and registration of portrayal items in the IHO GI Registry Portrayal Register. The main components that may be included in the IALA guidelines are as follows:

  - Definition of AtoN symbols and portrayal for non-charting purposes: Describe the background and purpose of developing guidelines to support AtoN management objectives and non-charting symbol and portrayal requirements

  - S-100 Portrayal mechanism: Explain the fundamental concept of feature-based AtoN symbolization and portrayal procedures, including the components and examples of a Portrayal Catalogue

  - Drawing Instructions concept: Describe the components and procedures of Drawing Instructions

  - AtoN symbol design: Methods for designing SVG point symbols, line symbols, pattern symbols, and text symbols

  - Rule definition: Methods for designing XSLT-based portrayal rules

  - Portrayal Catalogue: Catalogue components and methods for structuring the catalogue package

  - Registry registration: Methods for registering portrayal items in the IHO GI Registry Portrayal Register

## Use of Colors for AtoN Symbol

IALA intends to define symbols and portrayal rules for AtoN management purposes; however, harmonized representation with electronic navigational charts should be considered. Therefore, the colour conventions used by the IHO for chart production need to be taken into account. The IHO S-4 Regulations for International (INT) Charts and Chart Specifications of the IHO define principles for the use of colours in chart production, with the main points as follows:

- Use of Colour: All charts should be printed in a minimum of four colours: black, magenta, buff (or grey) and blue. Additional colours may be used. They may be useful in clarifying local navigational complexities, for example, on some charts the light sectors marking intricate inshore channels in Scandinavian waters are shown in red, green, and yellow. Charts using additional colours are often called ‘multicoloured’ charts

- BLACK: The general principles for the use of black are that it must normally be used: For all the details which provide the basic cartographic framework of charts, For all physical (solid) features, including depth information

- MAGENTA: The Specifications state which individual features are to be shown in magenta. The general principles for the use of magenta are that it should be reserved for: Drawing attention to symbols for features which have a significance extending beyond their immediate location. Distinguishing information superimposed on the physical features and not implying any permanent physical obstruction

- BUFF (YELLOW) OR GREY: A colour, usually buff or grey, must be used as a land tint. If the minimum four colours are used, the colour may be carefully selected so that a satisfactory colour over inter-tidal areas is derived from printing the land tint over the shallow water blue tint.

- BLUE: The colour blue must be used as a tint to emphasize shallow water. Two (or more) densities of blue tint may be used to show different depth bands of shallow water, the darkest tint showing the shallowest water. The blue tint may be combined with that used for land, to produce an appropriate colour for inter-tidal areas. Blue may also be used for depth contours, particularly in intricate waters.

- GREEN: The colour green may be used as a tint for inter-tidal areas. This may be achieved by combining the land colour with the shallow water blue colour. Green may also be used, instead of magenta, for environmental information and limits

## Testing Symbology and Portrayal of AtoN

The S-201 Test & Validation Tool, developed by IALA and Korea (MOF/KRISO/Bluemap), enables the registration of Feature Catalogues (FC) and Portrayal Catalogues (PC) for S-200 series product specifications, allowing users to create feature-based sample data and verify symbol portrayal. When developing AtoN symbols and portrayal standards for non-navigational purposes, the IALA S-201 Test & Validation Tool can be used to verify the application of symbols and portrayal. It is also considered useful for evaluating the appropriateness of symbol portrayal methods (e.g., color and shape) and for obtaining feedback.

# Action requested of the Committee

The Committee is requested to consider this input paper, and take actions as appropriate.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain