IALA GUIDELINE

GXXXX

IALA MArine Aids To Navigation (ATON) SERVICE DESIGN

provision of AtoN information to End-Users over IEC 63173-2 (SECOM)

Edition 1.0

March 2026

urn:mrn:iala:pub:gXXXX:ed.1.0

Revisions to this document are to be noted in the table prior to the issue of a revised document.

| Date | Details | Approval |
| --- | --- | --- |
| March 2026 | First edition |  |

IALA Marine Aids to Navigation (AtoN) Service Design	5

1.	Introduction	5

1.1.	Purpose of this Guideline	5

1.2.	Intended Readership	5

1.2.1.	Inputs from Other Sources	5

2.	Service Identification	6

3.	Technology Introduction	7

3.1.	General	7

3.2.	Service Technology and Service Transportation Protocol	7

3.3.	Security	7

3.3.1.	Communication Channel Security	7

3.3.2.	Data Protection	7

3.3.3.	Data Signature	7

3.3.4.	Data Encryption	7

4.	Service Overview	9

4.1.	General	9

4.2.	Service Interfaces	9

4.3.	Service Discovery	10

5.	Service Data Model	11

5.1.	Service Internal Data Model	11

6.	Service Interface Behaviour	12

6.1.	Common	12

6.1.1.	Common parameter values	12

6.1.2.	Error codes	12

6.2.	Applicable SECOM Interfaces	12

6.2.1.	Operation Get	13

6.2.2.	Operation Get by Link	15

6.2.3.	Operation Get Summary	16

6.2.4.	Operation Subscription	18

6.2.5.	Operation Remove Subscription	21

6.2.6.	Operation Subscription Notification	21

6.2.7.	Operation Upload	22

6.2.8.	Operation Upload by Link	24

6.2.9.	Operation Acknowledgment	26

7.	Service Dynamic Behaviour	28

7.1.	Generic sequence for signatures	28

7.2.	Client-initiated Retrieval of AtoN Information	28

7.3.	Subscription-based AtoN Information Propagation	30

8.	Definitions	32

9.	Acronyms	33

10.	References	33

11.	Annex A. OpenAPI Template	34

12.	Annex B. G1128 Instance Description XML Template	57

List of Tables

Table 1	Technical Design Identification Information.	6

Table 2	AtoN Information Service interfaces	9

Table 3	The Get Operation Parameters.	14

Table 4	The Get By Link Operation Parameters.	15

Table 5	The Get Summary Operation Parameters.	17

Table 6	The Subscription Operation Parameters.	19

Table 7	The Remove Subscription Operation Parameters.	21

Table 8	The Subscription Notification Operation Parameters.	22

Table 9	The Upload Operation Parameters.	23

Table 10	The Upload Operation Parameters.	25

Table 11	The Acknowledgment Operation Parameters.	27

Table 12	List of Definitions.	32

Table 13	List of Acronyms.	33

List of Figures

Figure 1	S-125 dataset coupled with service metadata in an S-100 Exchange Set.	11

Figure 2	Client-Initiated AtoN Information Retrieval through the Get operation.	29

Figure 3	Client-Initiated AtoN Information Retrieval through the Get Summary operation.	29

Figure 4	Subscription requested by external service.	31

# IALA Marine Aids to Navigation (AtoN) Service Design

Provision of AtoN information to End-Users using IEC 63173-2 (SECOM)

# Introduction

This document specifies the design of the “Provision of AtoN Information to End-Users” service according to the technical service specification [1] using the IEC SECOM [2] standard as the transport mechanism for route exchange. This one of the core technical services included in the “MS-2 - Aids to Navigation” maritime service, as this was defined by IMO [3].

This document was produced as part of the work of IALA ARM committee task group on development of technical service specifications for the provision of AtoN Information. The document is structured according to the IALA Guideline G1128 The Specification of e-Navigation Technical Services [4] and is based on the SECOM template found therein [5]

## Purpose of this Guideline

The purpose of this document is to define a service design using SECOM for the “Provision of AtoN Information to End-Users” service defined in the technical service specification [1]. This is done by using SECOM and the S-125 XML representation as defined in the specification. It defines the actual way the interactions and logical interfaces defined in the specification are to be implemented. It also describes how the requirements for authentication etc are to be implemented.

It describes the actual requirements for both ship and shore side systems in order to support the AtoN information provision as defined in the specification.

Note that this design will undergo changes between versions 1.0 and 2.0 as the SECOM and S-125 standard should be updated during that transition period and the design will be changed to reflect the latest changes. Some of the anticipated changes to SECOM and S-125 have already been included in this design and as such this design may not be fully compatible with SECOM 1.0 and S-125 1.0.

## Intended Readership

This Service Design is intended to be read by service architects, system engineers and developers in charge of designing and developing an instance of the route exchange service.

### Inputs from Other Sources

This service design follows the requirements, use cases and dynamic behaviour outlined in [1]. The interfaces, parameters and logic of the API is compliant with IEC 63173-2 SECOM and borrows much of its content from there.

“The authors thank the International Electrotechnical Commission (IEC) for permission to reproduce Information from its International Standards. All such extracts are copyright of IEC, Geneva, Switzerland. All rights reserved. Further information on the IEC is available from www.iec.ch. IEC has no responsibility for the placement and context in which the extracts and contents are reproduced by the author, nor is IEC in any way responsible for the other content or accuracy therein.”.

# Service Identification

This section provides a unique identification of the service and describes where the service is in the engineering lifecycle.

- Technical Design Identification Information.

| Name | Provision of AtoN Information Service to End-users over IEC 63173-2 (SECOM) |
| --- | --- |
| Implements | IALA Marine Aids to Navigation (AtoN) Service Specification |
|  | Provision of AtoN Information to End-Users |
|  | urn:mrn:iala:techsvc:ss:arm:atoninfo:1.0 |
| Version | 0.0.1 |
| Description | Technical Design for the provision of AtoN information to end-users via web services, using the IEC 63173-2 (SECOM) communication standard. |
| Keywords | REST, SECOM, MS2, ARM, AtoN, Aids to Navigation, S-125 |
| Architects | Nikolaos Vastardis |
| Status | Provisional |

# Technology Introduction

## General

This design template realizes the service specification [1] using SECOM as defined in IEC 63173-2 [2].

The services conforming to this design must be implemented with REST APIs using HTTPS with TLS protection to encrypt all communication in transit.

## Service Technology and Service Transportation Protocol

Reference: IEC 63173-2 SECOM v1.0.0 Clause 5.3 Service Technology

The technology (architectural style) chosen is REST (REpresentational State Transfer) upon HTTP/1.1 (RFC 7231).

## Security

### Communication Channel Security

Reference: IEC 63173-2 SECOM v1.0.0 Clause 6 SECOM communication channel security

The channel security between the SECOM REST service and a consumer are

- HTTP/1.1 according to RFC-7231

- HTTPS over TLS according to RFC-2818

Valid versions of TLS for this version of service design template are

- TLS version 1.2 and 1.3 (RFC-8446)

X.509 Certificates are used in the TLS according to RFC 5280 and RFC 2459.

Certificates shall be verified with OCSP and/or CRL methods.

### Data Protection

Reference: IEC 63173-2 SECOM v1.0.0 Clause 4.3.4 SECOM data protection

Reference: IHO S-100 ed5.2.0 Part 15 Data Protection Scheme

The data is mandatory to be signed by the sender to enable data authentication and integrity check by the receiver.

The data can optionally be encrypted by the sender, and the sender is responsible for exchanging the encryption key with the receiver.

While sending standalone S-421 documents without the additional metadata of datasets or exchange sets is preferred, if datasets or exchange sets are used the data (one or more data files) can optionally be packaged and compressed before being signed and sent.

### Data Signature

Reference: IEC 63173-2 SECOM v1.0.0 Clause 7.3 Data authentication and signing

Reference: IHO S-100 ed5.2.0 Part 15-8 Data Authentication

Reference: NIST Digital Signature Standard (DSS–FIPS Publication 186

The algorithm for signing data must be ECDSA-384-SHA2.

The signature is transported in HEX.

### Data Encryption

Reference: IEC 63173-2 SECOM v1.0.0 Clause 7.4 Data encryption

Reference: IHO S-100 ed5.2.0 Part 15-6 Data Encryption

The encryption algorithm for encryption is AES and CBC mode.

The symmetric encryption key can be exchanged by different means, including using the SECOM REST API to exchange the encryption key.

# Service Overview

## General

The design uses SECOM defined APIs. As such, it is important to understand that both the AtoN information provision service and its consumers must be able to function as client and server as understood in the traditional HTTP world. As such from here on when the term service is used, it applies to the on-shore “Provision of AtoN Information to End-Users” service and the consumer is the on-board ship system that is communicating with it.

This design does not concern itself with how the service instances will communicate with on-board ship systems. The instances of this design may be developed as integrated components directly or as independent microservices that communicate with the onboard ship systems via different integration mechanisms e.g., APIs or by emitting and consuming events.

It should also be noted that all AtoN information provision service instances, the advertised S-125 datasets, as well as the included AtoNs, must support the IALA Maritime Resource Naming (MRN) scheme, as per [6]. Valid MRN identifiers should be made available for each of these entities. The implementation methodology however, falls outside the scope of this service design.

## Service Interfaces

Reference: IEC 63173-2 SECOM v1.0.0 Clause 5.7 SECOM service interface definitions

SECOM does not require that all the interfaces defined in the standard must be implemented. Thus, for the purposes of this service and its consumers, only the following interfaces are required:

AtoN Information Service interfaces

| Interface | SECOM Reference | Comment |
| --- | --- | --- |
| Get | IEC 63173-2 SECOM v1.0.0 Clause 5.7.5 service interface - Get | This interface is called when the client gets (pulls) data from the service. |
| Get By Link | IEC 63173-2 SECOM v1.0.0 Clause 5.7.7 service interface - Get By Link | This interface is called when the client downloads (pulls) large data by reference given from interface Upload Link. |
| Get Summary | IEC 63173-2 SECOM v1.0.0 Clause 5.7.6 service interface - Get Summary | This interface is called when the client gets a summary of available data from the service. The data is retrieved (pulled) using the interface Get. |
| Subscription | IEC 63173-2 SECOM v1.0.0 Clause 5.7.10 service interface - Subscription | This interface is called when the client or server initiates subscription on data from the service. Response is given with interface Upload and Subscription Notification. |
| Remove Subscription | IEC 63173-2 SECOM v1.0.0 Clause 5.7.11 service interface - Remove Subscription | This interface is called when the client or server removes subscription. Response is given with interface Subscription Notification. |
| Subscription Notification | IEC 63173-2 SECOM v1.0.0 Clause 5.7.12 service interface - Subscription Notification | This interface is called as response from Subscription or Remove Subscription. |
| Upload | IEC 63173-2 SECOM v1.0.0 Clause 5.7.2 service interface - Upload | This interface is called when the client uploads (pushes) data to the service. The sender (client) decides the format and protection of the data. |
| Upload Link | IEC 63173-2 SECOM v1.0.0 Clause 5.7.3 service interface - Upload Link | This interface is called when the client uploads (pushes) a reference pointer to large data. The data is downloaded using interface Get By Link. |
| Acknowledgement | IEC 63173-2 SECOM v1.0.0 Clause 5.7.4 service interface - Acknowledgement | This interface is called as response to Acknowledgement request in Upload. |
| Capability | IEC 63173-2 SECOM v1.0.0 Clause 5.7.13 service interface - Capability | This interface is called when the client asks for the service capabilities. |
| Ping | IEC 63173-2 SECOM v1.0.0 Clause 5.7.14 service interface - Ping | This interface is called when the client checks the availability of the service. |
| PublicKey | IEC 63173-2 SECOM v1.0.0 Clause 5.7.16 service interface - PublicKey | This interface is called when the client gets (pulls) the public certificate(s) from the service. |

All unused interfaces of SECOM should be implemented to return HTTP 501 as specified in [2].

There are three components that are of interest from the perspective of the service design:

- The data provider’s service has a SECOM-component which supports the SECOM REST APIs defined in the table above. All other components of the service are left to the decisions of the implementing party.

- The data consumer client has a SECOM-component which will accept the incoming connections from the service and store all messages until delivered to the vessel. This interface is typically on a shoreside server as it must be always available and at a reachable address.

- The data consumer client also supports a SECOM client which allows it to make direct SECOM calls to the service without having to proxy all calls via the SECOM-component on shore.

In this service design we will not define the communication between the data provider and the AtoN information SECOM service or between the ship and the ship’s shoreside SECOM-component. These are specific for each implementation and depend on the general system architecture and implementation decisions.

## Service Discovery

Services implemented according to this design must submit their instance description to a valid service registry that follows the IALA G1129 Maritime Service Registry (MSR) [7] technical specification.

An XML template for the instance description is provided as an annex to this design ANNEX B Section 12.

# Service Data Model

This section describes the logical data structures to be exchanged between providers and consumers of the service. The service data model is bound by the current definition of the IHO S-125. For complete and updated documentation refer to the latest S-125 Product Specification which can be found at:

https://iho.int/en/nipwg-product-specifications.

Included in the service data model is a full description followed by specific extracts for;

- AtoN Information Features and Information types

- Enumerations

- Complex Attributes

Note that the S-100 specification [8] describes in Appendix 9-B how S-100 based data models shall be formulated in XML schema format.

Since the main purpose of the discussed service is to provide AtoN information to ECDIS, the S-125 information should be packaged as S-100 datasets, alongside any necessary metadata and other support information. The S-100 data model specification [8] in Appendix 4aS-100 Part 17, introduces the Exchange Set data structure, precisely for supporting this functionality. To indicate that S-125 datasets are coupled with additional service metadata, we refer to the type S100ExchangeSet (see Figure 1).

- S-125 dataset coupled with service metadata in an S-100 Exchange Set.

All AtoN information that is sent for use and display on an Electronic Chart Display and Information System (ECDIS) should always be packaged using an S-100 Exchange Set structure. All other cases, including third-party Electronic Chart Systems (ECS), may make use of the plain S-100 Dataset packaging option.

## Service Internal Data Model

As the S-125 data model, used to represent the transmitted data, is developed independently from this service specification, the internal data model of the service should be adequate to generate the required S-125 datasets. In addition, a way to store service metadata that are not directly related to the data model (internal service identifies, signatures, etc.) is required. For further information, refer to IALA Guideline 1157. However, these metadata are mostly implementation specific and therefore are not discussed in this service specification. Examples on how to implement this functionality can also be found in the IMO’s S-100 data model specification [8], as well as the SECOM standard [2].

# Service Interface Behaviour

As defined by SECOM, all communication between the components of the service that are in the scope of this design document is achieved via REST calls.

Additional information on the interfaces and parameters is available in the attached OpenAPI template of service in the Annex of Section 11.

## Common

The definitions of the service interfaces in this section, assume the following actors:

- An AtoN information provision service, identified as the Data Provider/Server.

- A digital communication device onboard the ship (interconnected with the ship’s ECDIS), identified as the Data Receiver/Consumer.

All parameters that are not set, should be omitted or can passed with a null value.

### Common parameter values

For all interfaces, the following parameter values must be used when applicable:

- containerType - should match the container type used to package the AtoN information (i.e. “0” for S-125 datasets and “1” for S-100 Exchange-Sets).

- dataProductType - must always be “S-125”.

- dataProductVersion - must be the S-125 version expected.

- exchangeMetadata - must contain all fields mandated by SECOM for the secure exchange of AtoN information.

- protectionScheme - must always be “SECOM”.

- transactionIdentifier (transactionId) - must be a UUID. It must be unique for each transaction.

For SECOM v2.0 services only:

- callbackEndpoint - The endpoint of the consumer’s SECOM v2.0 service that accepts the interface calls. The endpoint must be defined in a way that appending the SECOM standard REST endpoints (e.g. /v2/subscription) produces a valid URL. If the callbackEndpoint is not a part of the subscription, the service must search for the vessel’s SECOM service from a service registry.

### Error codes

If incoming route does not pass business rule validation, the SECOM_ResponseCode must be 0.

If the signature verification fails, the SECOM_ResponseCode must be 1.

If certificate provided is invalid, the SECOM_ResponseCode must be 2.

If incoming route does not pass schema validation, the SECOM_ResponseCode must be 3.

In all of these cases, the response returned to the client must have the HTTP status 400.

## Applicable SECOM Interfaces

All interface descriptions below only have the information necessary for the business logic of this service. All definitions and descriptions that are directly derived from the SECOM standard are left to be read from the SECOM standard and the API documentation template in the Annex of Section 11.

All SECOM communication is done via REST APIs with JSON as the data format for the SECOM data and XML for the S-125 data.

For all operations only those parameters that have predefined values or their usage needs to be explained are described below. Otherwise refer to SECOM [2].

The Capability, Ping, EncryptionKey and PublicKey interfaces are not discussed in this design as they follow the requirements defined in SECOM and further elaboration is necessary.

As per the SECOM standard, the provider must sign the data response using a valid certificate generated by the employed SECOM PKI. The signature, the public certificate, the root certificate and any intermediate certificates of the PKI, must all be present in the response.

### Operation Get

The Get operation is used by data consumers for retrieving AtoN information directly from a data provider, i.e. S-125 datasets that are applicable for a specified geographical area. The consumer can filter the requested S-125 dataset results through parameters like the dataset identifier, validity period and/or geometry etc.

The Get operation interface must be implemented only by the data provider.

Use of the interface is according to the SECOM specification.

- Name: Get Interface

- Sub-path: /v2/object

- HTTP-Method: GET

- Cookies: Session-Cookies (optional)

- Content-Type: application/json

#### Operation Functionality

All instances of this service design must provide access to all S-125 datasets made available for their advertised coverage area. Each individual S-125 dataset should have a fixed, globally unique, MRN identifier associated with it. In addition, each update on any of the datasets must be identified using a different, locally unique, UUID.

The operation supports multiple filter parameters, presented in more detail in Table 2. To ensure the authenticity of the operation request, the filter parameters are included within a signed SECOM envelope structure. Data provider instances are required to support all referenced filter parameters.

If multiple filter parameters are provided, only results that match all requested filters, combined with logical AND, must be returned. If no parameters are given, the response is either an empty list or all available datasets for which has been given access to by the data provider. A data provider must offer the appropriate encoding of AtoN information in accordance with the S-125 standard.

Depending on the combination of the containterType and validFrom/validTo input parameter values, the service must respond differently:

- For the containterType set to 0 (S-100 Dataset): the service must respond with the matching S-125 datasets containing all the latest information on the AtoNs included in them. The validFrom/validTo should not be taken into account, as the data provider must never return out-of-date information. If however, the specified dataReference UUIDs represent the past state of a valid S-125 dataset (with the same MRN), which has not been cancelled yet, then the data provider must return that exact dataset, once again disregarding the validFrom/validTo parameters. The service must never return S-125 delta files if the containterType value is set to 0.

- For the containterType set to 1 (S-100 Exchange-Set): the service must return the dataset using the S-100 Exchange-Set structure. That involves the original content of the S-125 dataset (update 0), along with any incremental updates (for the same dataset MRN) in the form of S-100 delta files. If the validFrom/validTo parameters are utilised, any of the original content of the S-125 dataset and delta files should only be included in the Exchange-Set, if they falls within the specified time interval.

#### Operation Parameters

The Get Operation Parameters.

| Parameters (in) | Encoding | Mult. | Descriptions |
| --- | --- | --- | --- |
| containerType | Integer | 0..1 | Requested encoding of the data as described in [2]. If not provided, the service provider determines the standard encoding. |
| dataProductType | String | 0..1 | Data product type as described in [2]. |
| productVersion | String | 0..1 | S-100 based Product type version, e.g. 1.0.0. |
| dataReference | UUID | 0..1 | Dataset reference identifier to a specific S-125 dataset. The reference can be provided as search criteria. A list of references can be retrieved via Get Summary interface. |
| geometry | String (WKT) | 0..1 | Geometry condition for geo-located information objects in WKT format. This can be used to filter on geometric shapes (e.g., filter S-125 datasets by a bounding box). |
| unlocode | String | 0..1 | UN/LOCODE (United Nations Code for Trade and Transport Locations) of a defined object (e.g., “CN” for China). |
| validFrom | DateTime | 0..1 | Describes the start of the validity time for the requested data in ISO 8601 UTC formatted as yyyy-mm-dd hh:mm:ssZ. |
| validTo | DateTime | 0..1 | Describes the end of the validity time for the requested data in ISO 8601 UTC formatted as yyyy-mm-dd hh:mm:ssZ. |
| page | Integer | 0..1 | The number of the page for retrieving paged results. If not provided, the default value of this parameter is set to zero (0). |
| pageSize | Integer | 0..1 | The size of each page in a paged result. If not provided, the default value of this parameter is not set and the resulting S-125 datasets are not paged. |

#### Interpretation Guidelines

If no data is returned, the response is HTTP code 404.

If the size of the data to be returned in more than the limit set by SECOM (i.e. 350 kbs), an HTTP 413 error code is expected from the data provider. In that case the data consumer should make use of the Get By Link operation, described in Section 6.2.2.

The dataReference is used when a known data object is retrieved. The dataReference is provided in the response from Get Summary and is typically an UUID.

The containterType describes how the data is packaged, e.g. 0 for a S-100 DataSet, 1 for a S-100 Exchange-Set or 2 for no specific container, e.g. RTZ in XML.

The dataProductType is a string describing which data product that is in focus in the operation. For the purpose of this service design, it must always be “S-125” as per the format specified in S-100 [8].

The productVersion describes the specific version of the data product specification of the data requested.

The geometry describes geographical criteria for the data requested. The geometry is given as a one-line WKT formatted string and can contain one or more geospatial objects.

The unlocode describes with 5 characters the UN code for city or other area, such as a port. The unlocode is a list administered by UN and can be found online. Many unlocodes have a position in longitude/latitude attached, but not all. In most cases the unlocode will be used as text only but this may depend on the service developer.

The validFrom/validTo describes the requested validity time for the requested data in ISO 8601 UTC formatted as yyyy-mm-dd hh:mm:ssZ.

### Operation Get by Link

The Get by Link operation is used by data consumers for retrieving large AtoN information datasets directly from a data provider. This interface shall be used instead of the Get, when the size of the data to be exchanged exceeds the SECOM limit (i.e. 350kbs). The consumer in this case cannot filter the pre-prepared results any further.

The Get by Link operation interface must be implemented only by the data provider.

Use of the interface is according to the SECOM specification.

- Name: Get by Link Interface

- Sub-path: /v2/object/link

- HTTP-Method: GET

- Cookies: Session-Cookies (optional)

- Content-Type: application/json

#### Operation Functionality

The operation validates the given transactionIdentifier against the caller’s identity and the timeToLive given for the data. The data is then packaged according to agreement in Upload By Link and returned in the call.

#### Operation Parameters

The Get By Link Operation Parameters.

| Parameters (in) | Encoding | Mult. | Descriptions |
| --- | --- | --- | --- |
| transactionIdentifier | UUID | 1 | Identifier uploaded in Upload Link |
| envelopeSignatureCertificate | String | 1..* | The public certificate(s) of the sender. Used to verify the envelope signature. |
| envelopeRootCertificateThumbprint | String | 1 | Claimed Thumbprint for Signed Root Key (X.509 Certificate) SHA256 and base64 as format. |
| envelopeSignatureTime | DateTime | 1 | Timestamp when the envelope is signed envelope. |
| SignatureReference | String | 1 | Specifies the algorithm used to compute digitalSignatureValue. For example “ECDSA-384-SHA2”. |

#### Interpretation Guidelines

The unique transactionIdentifier given by the Upload By Link call.

If the provided transactionIdentifier is not valid, the response is HTTP code 404.

There is not specific time constraint for a data consumer to retrieve the results through the Get by Link operation. If however, the involved datasets have in the meantime been cancelled, the data provider is allowed to drop the pending transactions.

### Operation Get Summary

The Get Summary operation is used by data consumers to discover the AtoN information that can be made available (and is accessible) by a data provider. The data provider will respond by listing the available (and accessible) S-125 datasets that are of interest to the consumer. This list should only contain summary information on each S-125 dataset, not the dataset content itself. UUID-based dataset identifiers can be used to retrieve the full S-125 datasets via the Get interface. The consumer can filter the requested S-125 dataset results through parameters like the dataset identifier, validity period and/or geometry etc.

The Get Summary operation interface must be implemented only by the data provider.

Use of the interface is according to the SECOM specification.

- Name: Get Summary Interface

- Sub-path: /v2/object/summary

- HTTP-Method: GET

- Cookies: Session-Cookies (optional)

- Content-Type: application/json

#### Operation Functionality

All instances of this service design must provide access to the summary information of all S-125 datasets made available for their advertised coverage area. Each individual S-125 dataset should have a fixed MRN identifier associated with it, while each S-125 dataset update must be identified using a different unique UUID.

The operation supports multiple filter parameters, presented in more detail in Table 4. To ensure the authenticity of the operation request, the filter parameters are included within a signed SECOM envelope structure. Data provider instances are required to support all referenced filter parameters.

If multiple query parameters are provided, only results that match all requested filters, combined with logical AND, must be returned. If no parameters are given, the response is either an empty list or all available dataset summaries the consumer has been given access to by the service provider.

Depending on the combination of the containterType and validFrom/validTo input parameter values, the data provider must respond differently:

- For the containterType set to 0 (S-100 Dataset): the service must respond with the latest summary information for all matching S-125 datasets. The validFrom/validTo should not be taken into account, as the data provider must never return out-of-date information. If however, the specified dataReference UUIDs represent the past state of a valid S-125 dataset (with the same MRN), which has not been cancelled yet, then the data provider must return the summary information for that exact dataset, once again disregarding the validFrom/validTo parameters. The service must never return summary information regarding S-125 delta files if the containterType value is set to 0.

- For the containterType set to 1 (S-100 Exchange-Set): the service must return the dataset summary information using the S-100 Exchange-Set structure. That involves the original content of the S-125 dataset (update 0), along with any incremental updates (for the same dataset MRN) in the form of S-100 delta files. If the validFrom/validTo parameters are utilised, any of the original content of the S-125 dataset and delta files should only be included in the Exchange-Set, if they fall within the specified time interval.

#### Operation Parameters

The Get Summary Operation Parameters.

| Parameters (in) | Encoding | Mult. | Descriptions |
| --- | --- | --- | --- |
| containerType | Integer | 0..1 | Requested encoding of the data as described in [2]. If not provided, the service provider determines the standard encoding. |
| dataProductType | String | 0..1 | Data product type as described in [2]. |
| productVersion | String | 0..1 | S-100 based Product type version, e.g. 1.0.0. |
| geometry | String (WKT) | 0..1 | Geometry condition for geo-located information objects in WKT format. This can be used to filter on geometric shapes (e.g., filter S-125 datasets by a bounding box). |
| unlocode | String | 0..1 | UN/LOCODE |
| validFrom | DateTime | 0..1 | Describes the start of the validity time for the requested data in ISO 8601 UTC formatted as yyyy-mm-dd hh:mmZ. |
| validTo | DateTime | 0..1 | Describes the end of the validity time for the requested data in ISO 8601 UTC formatted as yyyy-mm-dd hh:mmZ. |
| page | Integer | 0..1 | The number of the page for retrieving paged results. If not provided, the default value of this parameter is set to zero (0). |
| pageSize | Integer | 0..1 | The size of each page in a paged result. If not provided, the default value of this parameter is not set and the resulting S-125 datasets are not paged. |

#### Interpretation Guidelines

In the response, the dataReference is given as a UUID for each data object listed. The dataReference can then be provided as input to a Get call.

The dataProtection flag indicates whether the data is encrypted or not. If encrypted, an encryptionKey is required to decrypt the data.

The compression flag indicates whether the data is compressed with ZIP or not.

The dataProductType and productVersion indicate type and version of the data specification for the data object. For the purpose of this service design, the data product type must always be “S-125” as per the format specified in S-100 [8].

The info_identifier, info_name, info_status, info_description, and info_lastModifiedData are describing each matching S-125 dataset.

The info_size represents the file size of the S-125 dataset, as returned by Get data. The size does not include the envelope.

The validFrom/validTo describes the requested validity time for the requested data in ISO 8601 UTC formatted as yyyy-mm-dd hh:mm:ssZ.

### Operation Subscription

The purpose of the Subscription operation is for service consumers to request a new or cancel an existing subscription on AtoN information. Subscriptions are made either on specific parameters, otherwise these are decided by the data provider.

The Subscription operation interface must be implemented only by the data provider.

Use of the interface is according to the SECOM specification.

- Name: Subscription Interface

- Sub-path: /v2/subscription

- HTTP-Method: POST

- Cookies: Session-Cookies (optional)

- Content-Type: application/json

#### Operation Functionality

In SECOM, the data provider may then use the URL available in the <callbackEndpoint> parameter to contact the data consumer directly and send any available subscription updates. In the absence of a valid callback endpoint value, the data provider may discover the consumer’s URL utilising an appropriate Service Registry (e.g. MSR). The data consumer can be identified through its MRN, contained in the HTTPS request certificate provided during the TLS/SSL authentication step. This workaround is compatible with the specifications of SECOM v1.0 and allows the service to identify the data consumer and locate its uploading interface. More information on the dynamic behaviour of the subscription operation and its relation to the SECOM-compliant service registry is provided in Section 7.3.

The service needs to keep track of the following operations:

- New subscriptions

- Changes between two points in time (S-125 delta files)

- Cancellation of datasets

Once the subscription request is received and processed successfully, the service will create a new subscription record for the respective client and return a unique UUID identifier for that record. The response containing with the generated are presented in the example service response.

A subscription can be cancelled through the Remove Subscription operation. More information can be found in Section 6.2.5.

The operation interface consumes the following consumer interfaces:

- Upload

- Subscription Notification

The Upload consumer interface can be used by the data provider to push new and/or updated S-125 datasets to the subscribed service consumer. The Subscription Notification consumer interface on the other hand, is used to inform the consumer that on the status of the subscription, i.e., when it has been successfully activated or removed. More information on this operation can be found in Section 7.2. Note that for each new subscription, a Subscription Identifier should be made available to the consumer though the interface’s response.

The operation supports multiple filter parameters, presented in more detail in Table 5. To ensure the authenticity of the operation request, the filter parameters are included within a signed SECOM envelope structure. Data provider instances are required to support all referenced filter parameters.

All criteria for the subscription are optional. If none are provided, all authorized data from the data provider will be included in the subscription. If more than one criteria is provided, these must be combined with a logical AND, hence all criteria must be met.

#### Operation Parameters

The Subscription Operation Parameters.

| Parameters (in) | Encoding | Mult. | Descriptions |
| --- | --- | --- | --- |
| containerType | Integer | 0..1 | Requested encoding of the data as described in [2]. If not provided, the service provider determines the standard encoding. |
| dataProductType | String | 0..1 | Data product type as described in [2]. |
| productVersion | String | 0..1 | S-100 based Product type version, e.g. 1.0.0. |
| dataReference | UUID | 0..1 | Dataset reference identifier to a specific S-125 dataset. The reference can be provided as search criteria. A list of references can be retrieved via Get Summary interface. If no references are provided. |
| geometry | String (WKT) | 0..1 | Geometry condition for geo-located information objects in WKT format. This can be used to filter on geometric shapes (e.g., filter S-125 datasets by a bounding box). |
| unlocode | String | 0..1 | UN/LOCODE (United Nations Code for Trade and Transport Locations) of a defined object (e.g., “CN” for China). |
| subscriptionPeriodStart | DateTime | 0..1 | The start date and time for which the subscription is active. |
| subscriptionPeriodEnd | DateTime | 0..1 | The end date and time for which the subscription is active. |
| callbackEndpoint | URL | 0..1 | Base URL to the requestor’s SECOM service Attribute Messages within subscription are sent to callbackEndpoint/object. If not available, the baseUrl needs to be available through search service. |
| pushAll | Boolean | 0..1 | Flag for sending an initial set of data within requested subscription and then start the subscription. If false, only data updates from the start of the subscription will be sent. |

#### Interpretation Guidelines

The dataReference criteria means that only new updates of the specific S-125 dataset will be sent in the subscription. If the dataReference dataset is cancelled or deleted, the subscription ends.

The containterType describes how the data is packaged, e.g. 0 for a S-100 DataSet, 1 for a S-100 Exchange-Set or 2 for no specific container, e.g. RTZ in XML.

The dataProductType and productVersion criteria means that only information of the type and version will be sent in the subscription. For the purpose of this service design, the data product type must always “S-125” as per the format specified in S-100 [8].

The geometry describes geographical criteria for the data requested. The geometry is given as a one-line WKT formatted string and can contain one or more geographical objects.

The unlocode criteria means that only information related to the unlocode will be sent in the subscription. If there is no unlocode related to the information provided by the service, the unlocode is ignored.

The subscriptionPeriodStart and subscriptionPeriodEnd criteria mean that data produced in the given time period will be sent in the subscription. When subscriptionPeriodEnd is reached, the subscription shall be removed automatically. The time period is recommended to be formatted in ISO 8601 UTC as yyyy-mm-dd hh:mm:ssZ.

The callbackEndpoint optionally contains the URL of the data consumers. The service may use that information to contact the data consumer when updates are available for an established subscription. If that information is not available, the data provider should search for the data consumer using the provided MRN in the appropriate Service Registry (e.g. MSR).

The pushAll flag signals that the service is expected push the latest S-125 datasets covered by a subscription to the data consumer, right after the subscription has been established. This removes the requirement for data consumers to always pull the data manually when creating a subscription.

### Operation Remove Subscription

The Remove Subscription operation can be used to remove subscriptions that were created earlier through the Subscribe operation. It follows a one-way paradigm (ONE_WAY), where a client sends data to consumer without expecting information back, other than a technical response.

The Remove Subscription operation interface must be implemented only by the data provider.

Use of the interface is according to the SECOM specification.

- Name: Remove Subscription Interface

- Sub-path: /v2/subscription

- HTTP-Method: DELETE

- Cookies: Session-Cookies (optional)

- Content-Type: application/json

#### Operation Functionality

The SECOM interface specification is not clear on whether the MRN of the data consumer that originally requested the subscription is to be checked against the MRN of the actor who performed this request. It is therefore assumed that any valid data consumer can terminate an existing subscription. A notification is then expected to inform the subscribed data consumer. If no subscription identifier parameter is specified, the service will respond with an error code and an appropriate message.

The operation interface consumes the following consumer interfaces:

- Subscription Notification

The Subscription Notification consumer interface is used to inform the consumer that on the status of the subscription, i.e. when it has been successfully removed.

#### Operation Parameters

The Remove Subscription Operation Parameters.

| Parameters (in) | Encoding | Mult. | Descriptions |
| --- | --- | --- | --- |
| subscriptionIdentifier | UUID | 1 | Specific identifier of the active subscription to be removed. Typically received by the data consumer after a subscription request or through a subscription notification. |

#### Interpretation Guidelines

The incoming subscriptionIdentifier given when the subscription was created and provided in the Subscription Notification, is used to remove the corresponding subscription.

### Operation Subscription Notification

The Subscription Notification operation receives notifications by data provider, when a subscription is created or removed.

The Subscription Notification operation interface must be implemented only by the data consumer.

Use of the interface is according to the SECOM specification.

- Name: Subscription Notification Interface

- Sub-path: /v2/subscription/notification

- HTTP-Method: POST

- Cookies: Session-Cookies (optional)

- Content-Type: application/json

#### Operation Functionality

The data provider is not required to call the subscriptionNotification of the consumer unless the creation or removal of the subscription requires asynchronous action or is the result of a maintenance operation. If the subscription and removeSubscription are synchronous processes when called from the data provider, then the subscriptionNotification is not required.

The operation two parameters, presented in more detail in Table 7. To ensure the authenticity of the operation request, these parameters are included within a signed SECOM envelope structure.

#### Operation Parameters

The Subscription Notification Operation Parameters.

| Parameters (in) | Encoding | Mult. | Descriptions |
| --- | --- | --- | --- |
| subscriptionIdentifier | UUID | 1 | Specific identifier of the subscription for which the status has being updated. |
| eventEnum | SubscriptionEventEnum as defined in [2]. | 1 | Type of subscription status update event e.g., Created or Removed. |

#### Interpretation Guidelines

The subscriptionIdentifier is the reference to the specific subscription in focus.

The eventEnum describes the type of event, e.g. 1 when the subscription is created and “2” when the subscription is removed.

### Operation Upload

The Upload operation enables data providers to upload (push) AtoN information to the data consumers. It allows consumers to receive S-125 datasets, while in a subscription or a due to a broadcast.

The Upload operation interface must be implemented only by the data consumers.

Use of the interface is according to the SECOM specification.

- Name: Upload Interface

- Sub-path: /v2/object

- HTTP-Method: POST

- Cookies: Session-Cookies (optional)

- Content-Type: application/json

#### Operation Functionality

In most cases this operation will take place as part of a subscription, originally initiated by the data consumer. In those cases the fromSubscription parameters is expected to be set to true. There might however, be cases where urgent AtoN information might need to be broadcasted, therefore data consumer uploads may also take place outside active subscriptions. These cases however are not covered by this service design.

This interface operation may on request consume the following provider interfaces:

- Acknowledgement

In an upload operation, an acknowledgement can be requested via the <ackRequest> parameter. The acknowledgment is expected to be received when the uploaded S-125 dataset has been delivered to the end‑system (technical acknowledgement), and/or (if supported) when the message has been opened/read by the end‑user (operational acknowledgement).

The data consumer may then use the URL available in the <callbackEndpoint> parameter to contact the data provider directly and send the required acknowledgement. In the absence of a valid callback endpoint value, the data consumer may discover the server URL utilising an appropriate Service Registry (e.g. MSR). The data provider can be identified through its MRN, contained in the HTTPS request certificate provided during the TLS/SSL authentication step, as in Section 6.2.4.1. More information on the dynamic behaviour of this operation and its relation to the SECOM-compliant service registry is provided in Section 7.2.

The operation supports multiple other parameters, which are presented in more detail in Table 8. To ensure the authenticity of the operation request, these parameters are included within a signed SECOM envelope structure.

#### Operation Parameters

The Upload Operation Parameters.

| Parameters (in) | Encoding | Mult. | Descriptions |
| --- | --- | --- | --- |
| data | Base64 | 1 | The S-125 XML payload, Base64 encoded. Optionally, the data can protected and/or compressed. This should be reflected in the exchangeMetadata structure. |
| containerType | Integer | 0..1 | Requested encoding of the data as described in [2]. If not provided, the service provider determines the standard encoding. |
| dataProductType | String | 0..1 | Data product type as described in [2]. |
| exchangeMetadata | SECOM_ExchangeMetadata as defined in [2] | 1 | The exchange metadata that contains information regarding the protection scheme, compression, signature and claimed identity. |
| fromSubscription | Boolean | 1 | A flag that indicates whether the data has been uploaded within an active subscription or not. |
| subscriptionIdentifier | UUID | 0..1 | Subscription identifier is specified if the object is uploaded within a subscription. |
| ackRequest | AckRequestEnum as defined in [2] | 1 | Flag to indicate that acknowledgement is expected to be returned when the data is delivered to the end user, and/or when the content of the data is processed (opened) by the end user. |
| callbackEndpoint | URL | 0..1 | Base URL without trailing slash to the requestor SECOM service Endpoint where to send an acknowledgement. If not available, the endpoint needs to be available through the search service. |
| transactionIdentifier | UUID | 1 | Unique transaction UUID to be used e.g. in acknowledgement. |
| envelopeSignatureCertificate | String | 1..* | The public certificate(s) of the sender. Used to verify the envelope object signature. |
| envelopeRootCertificateThumbprint | String | 1 | Claimed Thumbprint for Signed Root Key (X.509 Certificate) SHA256 and base64 as format |
| envelopeSignatureTime | DateTime | 1 | Timestamp when the envelope is signed. |
| envelopeSignatureReference | String | 1 | (S-100) Specifies the algorithm used to compute the envelopeSignature For example “ECDSA-384-SHA2”. |

#### Interpretation Guidelines

If any of the upload request S-125 datasets is too large, the returned HTTP status code must be 413. The S-125 datasets must then be shared using the upload link interface.

The data consumer may return HTTP status 403 if the uploaded AtoN information it receives that it has not requested.

### Operation Upload by Link

The Upload by Link operation enables data providers to provide a link to large volumes of AtoN information to data consumers. This interface shall be used instead of the Upload operation, when the size of the data to be exchanged exceeds the predefined SECOM limits. It allows data consumers to receive large S-125 datasets, while in a subscription or a due to a broadcast.

The Upload by Link operation interface must be implemented only by the data consumers.

Use of the interface is according to the SECOM specification.

- Name: Upload by Link Interface

- Sub-path: /v2/object/link

- HTTP-Method: POST

- Cookies: Session-Cookies (optional)

- Content-Type: application/json

#### Operation Functionality

In most cases the Upload by Link operation will take place as part of a subscription, initiated by the data consumer. It must be used instead of the Upload operation when the S-125 datasets to be transmitted exceed 350 kbs. In those cases, the fromSubscription parameters is expected to be set to true. There might however, be cases where urgent AtoN information might need to be broadcasted, therefore data consumer uploads may also take place outside active subscriptions. These cases however are not covered by this service design.

This interface operation may on request consume the following provider interfaces:

- Acknowledgement

In an upload by link operation, an acknowledgement can be requested via the ackRequest parameter. The acknowledgment is expected to be received when the uploaded S-125 dataset has been delivered to the end‑system (technical acknowledgement), and/or (if supported) when the message has been opened/read by the end‑user (operational acknowledgement).

The data consumer may then use the URL available in the callbackEndpoint parameter to contact the data provider directly and get the advertised S-125 datasets, as well as send the required acknowledgement. In the absence of a valid callback endpoint value, the data consumer may discover the server URL utilising an appropriate Service Registry (e.g. MSR). The data provider can be identified through its MRN, contained in the HTTPS request certificate provided during the TLS/SSL authentication step, as in Section 6.2.4.1. More information on the dynamic behaviour of this operation and its relation to the SECOM-compliant service registry is provided in Section 7.2.

As in the Upload operation described previously, this operation supports multiple other parameters, which are presented in more detail in Table 9. To ensure the authenticity of the operation request, these parameters are included within a signed SECOM envelope structure.

#### Operation Parameters

The Upload Operation Parameters.

| Parameters (in) | Encoding | Mult. | Descriptions |
| --- | --- | --- | --- |
| containerType | Integer | 0..1 | Requested encoding of the data as described in [2]. If not provided, the service provider determines the standard encoding. |
| dataProductType | String | 0..1 | Data product type as described in [2]. |
| exchangeMetadata | SECOM_ExchangeMetadata as defined in [2] | 1 | The exchange metadata that contains information regarding the protection scheme, compression, signature and claimed identity. |
| fromSubscription | Boolean | 1 | A flag that indicates whether the data has been uploaded within an active subscription or not. |
| subscriptionIdentifier | UUID | 0..1 | Subscription identifier is specified if the object is uploaded within a subscription. |
| ackRequest | AckRequestEnum as defined in [2] | 1 | Flag to indicate that acknowledgement is expected to be returned when the data is delivered to the end user, and/or when the content of the data is processed (opened) by the end user. |
| callbackEndpoint | URL | 0..1 | Base URL without trailing slash to the requestor SECOM service Endpoint where to send an acknowledgement. If not available, the endpoint needs to be available through the search service. |
| transactionIdentifier | UUID | 1 | Unique transaction UUID to be used e.g. in acknowledgement. |
| envelopeSignatureCertificate | String | 1..* | The public certificate(s) of the sender. Used to verify the envelope object signature. |
| envelopeRootCertificateThumbprint | String | 1 | Claimed Thumbprint for Signed Root Key (X.509 Certificate) SHA256 and base64 as format |
| size | Integer | 1 | Size of the data in kBytes to be downloaded. |
| timeToLive | DateTime | 1 | Timestamp when data will be deleted on server. The data need to be fetched before this time. |
| envelopeSignatureTime | DateTime | 1 | Timestamp when the envelope is signed. |
| envelopeSignatureReference | String | 1 | (S-100) Specifies the algorithm used to compute the envelopeSignature For example “ECDSA-384-SHA2” |

#### Interpretation Guidelines

The Upload by Link operation should always be used when the server identifies that the size of the data to be transmitted exceed the predefined SECOM limit, or when the previously attempted upload operation failed with an HTTP 413 error code.

The data consumer may return HTTP status 403 if the uploaded AtoN information it receives that it has not requested.

### Operation Acknowledgment

The Acknowledgment operation allows data consumers to asynchronously confirm whether an uploaded S-125 dataset has been successfully received and/or opened by an end-user.

It is not yet clear on whether acknowledgements are necessary for the provision of AtoN information to end-users. Regardless, for the purposes of this service design,the Acknowledgment operation interface must be implemented only by the data consumers.

Use of the interface is according to the SECOM specification.

- Name: Acknowledgment Interface

- Sub-path: /v2/acknowledgement

- HTTP-Method: POST

- Cookies: Session-Cookies (optional)

- Content-Type: application/json

#### Operation Functionality

The operation validates the given transactionIdentifier and handles the acknowledgement of sent data. The acknowledgement can be either for when the S-125 dataset has been delivered, opened or both. The acknowledgement contains a reference to the object delivered and has no time limit.

#### Operation Parameters

The Acknowledgment Operation Parameters.

| Parameters (in) | Encoding | Mult. | Descriptions |
| --- | --- | --- | --- |
| createdAt | DateTime | 1 | Creation time for the acknowledgement. |
| envelopeCertificate | String | 1..* | The public certificate of the sender. used to verify the envelope signature |
| envelopeRootCertificateThumbprint | String | 1 | Claimed Thumbprint for Signed Root Key (X.509 Certificate) SHA256 and base64 as format. |
| transactionIdentifier | String | 1 | Reference identifier given in upload. |
| ackType | AckTypeEnumType as defined in [2] | 1 | Type of of acknowledgement. |
| nackType | NackTypeEnum as defined in [2] | 0..1 | NackTypeEnum Information details if error. |
| envelopeSignatureTime | DateTime | 0..1 | Timestamp when the envelope is signed |
| envelopeSignatureReference | String | 1 | Specifies the algorithm used to compute envelope signature. For example “ECDSA-384-SHA2” |

#### Interpretation Guidelines

The transactionIdentifier describes the reference to what sending transaction of data that is acknowledged.

The response can be either positive acknowledgement (ackType = 1 or 2) or negative acknowledgement (ackType = 3 (error)). If negative acknowledgement is given, the error message is expected to be provided in nackType (0-4) describing the error.

# Service Dynamic Behaviour

In the following diagrams and descriptions, the communication between the vessel’s onboard systems and its SECOM service is out of scope for the purposes of this design.

Similarly, the communication and interfaces between the service and the on-board ship system is out of scope of this design.

## Generic sequence for signatures

This sequence assumes the following actors:

Data Provider → Data Server → Data Receiver → Data Consumer

The sequence describes the data from its source through transport using SECOM to the end-user.

Data Provider preparation:

- The data provider must have an identity in trusted PKI system

- The data provider creates a private-public key pair

- The data provider sends the public key to PKI system to get it signed and connected to a specific identity/identifier in the PKI system (Certificate Signing Request)

Data Sender preparation:

- The data sender must have an identity in trusted PKI system

- The data sender creates a private-public key pair

- The data sender sends the public key to PKI system to get it signed and connected to a specific identity/identifier in the PKI system (Certificate Signing Request)

Data Provider and Data Sender:

- The data provider selects identifier and certificate to use for the data signing

- The data provider creates a signature for the data

- The data provider transfers the data with its signature to the data sender.

- The data sender protects the data transferring by the selected communication mechanism, e.g. SECOM. SECOM Upload wraps the data, the signature and metadata into an envelope. The envelope is signed by the data sender.

- The data sends contacts the data receiver and transfers the data

Data Receiver and Data Consumer:

- The data receiver receives the transferred object with envelope. The envelope signature is verified.

- The data receiver forwards the data and data signature to the data consumer

- The data consumer verifies the data signature

## Client-initiated Retrieval of AtoN Information

The client-initiated retrieval is triggered by the consumer, when they issue a request on an appropriate AtoN information data receiver (i.e. Get and Get Summary). This functionality covers Use Case #1, Use Case #2 and Use Case #4, as outlined in relevant service specification [1]. All three use cases can be achieved by two methods:

- By directly requesting specific S-125 datasets, using the operation parameters of the Get operation, or;

- By requesting the S-125 dataset summary information through the Get Summary operation first, and then using the provided dataset reference identifiers, to retrieve the corresponding S-125 datasets.

The first method is presented in Figure 2, where the service consumer contacts directly the Get operation of the service provider. Utilising the available operation parameters such as the geometry, area name and time-period, only specific S-125 datasets matching the specified criteria and/or have updates on the specified time-period interval, will be retrieved, packaged into an S-100 Exchange Set, and returned as a response.

Client-Initiated AtoN Information Retrieval through the Get operation.

In cases where the service consumer requires more information, before making a decision on which datasets should be downloaded, the second retrieval method can be employed, utilising the Get Summary operation. This is demonstrated in Figure 3, where the data consumer initially requests the list of the available S-125 dataset metadata via the summary operation and directly receives a response from the data provider. The service consumer can then select a dataset reference identifier from the returned list and request the detailed information (i.e. the complete S-125 dataset) back, through the Get operation of the data provider. Each of the corresponding S-125 dataset (and/or its corresponding updates) will be packaged as an individual S-100 Exchange Set and will be sent back to the data consumer as direct response.

Client-Initiated AtoN Information Retrieval through the Get Summary operation.

The Get Summary operation accepts the same operation parameters as Get, which can be employed to filter the summary response list. This feature improves the flexibility of the whole operation. For example, even if a provided geometry operation parameter matches more than one S-125 datasets, only the required dataset will eventually be retrieved by the service consumer, by using its respective dataset reference identifier.

## Subscription-based AtoN Information Propagation

The subscription on AtoN Information is an alternative method of data propagation, in which a data consumer actively registers its interest to receive updates either on a specific S-125 dataset or alternatively, on all available S-125 datasets. This operation covers Use Case #3, as outlined in [1].

Figure 3 illustrates the typical behaviour of the subscription operation. Initially, the data consumer is required to initiate the subscription through the Subscription operation of the data provider. Utilising the available operation parameters such as the geometry, the area name and the subscription period, updates only on the S-125 datasets matching the specified criteria will be provided. Once the subscription request has been received, processed and accepted by the data server, a notification may be sent back to the data consumer, using its Subscription Notification operation. This notification will indicate that the requested subscription has commenced and should include the UUID identifier reference of the established subscription.

Whenever an update is made available on the set of S-125 datasets covered by a subscription, the Upload operation is used to deliver those updates, packaged as S-100 Exchange Sets, to the data consumer. Depending on the implementation of the service, the data consumer may be required to respond to this operation by sending an acknowledgement message back to the service provider. The subscription updates will continue to be provided until one of the following conditions are met:

- The subscription period initially set by the data consumer has expired.

- The valid and authorised actor requests an end to the active subscription, using the Remove Subscription operation of the data provider.

- The data provider removes the information objects covered by the subscription.

- An expected incident/unexpected error forces the data provider to terminate the subscription.

Subscription requested by external service.

In the sequence diagram presented in Figure 4, the established subscription is terminated though a service consumer request. This should include the identifier reference of the subscription to be terminated. If no subscription identifier is provided, the service server must not terminate any of the active subscriptions of the data consumer performing the request. Once the data consumer’s request has been received, processed, and accepted by the data server, another notification will be sent back to the service consumer’s Subscription Notification operation, to inform them of the successful termination of the subscription.

Note that the full contents of the S-125 datasets covered by a subscription, are not send back to the service consumer, unless a new dataset is created within the active period of an existing subscription that covers it. In all other cases, only the S-125 dataset changes will be transferred, in order to conserve bandwidth. Therefore, the service consumer needs to utilise the Get (and Get Summary) operations, if the full contents of the S-125 datasets are required.

# Definitions

The definitions of terms used in this IALA Guideline can be found in the International Dictionary of Marine Aids to Navigation (IALA Dictionary) and were checked as correct at the time of going to print. Where conflict arises, the IALA Dictionary shall be considered as the authoritative source of definitions used in IALA documents.

List of Definitions.

| Term | Definition |
| --- | --- |
| External Data Model | Describes the semantics of the ‘maritime world’ (or a significant part thereof) by defining data structures and their relations. This could be at logical level (e.g. in UML) or at physical level (e.g. in XSD schema definitions), as for example standard data models, or S-100 based data produce specifications. |
| Message Exchange Pattern | Describes the principles two different parts of a message passing system (in our case: the service provider and the service consumer) use to interact and communicate with each other. Examples: In the Request/Response MEP, the service consumer sends a request to the service provider to obtain certain information; the service provider provides the requested information in a dedicated response. In the Publish/Subscribe MEP, the service consumer establishes a subscription with the service provider to obtain certain information; the service provider publishes information (either in regular intervals or upon change) to all subscribed service consumers. |

# Acronyms

List of Acronyms.

| Acronum | Definition |
| --- | --- |
| API | Application Programming Interface |
| ECDIS | Electronic Chart Display and Information System |
| ECS | Electronic Chart System |
| HO | Hydrographic Office |
| IEC | International Electrotechnical Commission |
| IMO | International Maritime Organization |
| JSON | JavaScript Object Notation |
| MIR | Maritime Identity Registry |
| MRN | Maritime Resource Name |
| MS | Maritime Service |
| MSI | Maritime Safety Information |
| MSR | Maritime Service Registry |
| RENC | Regional ENC Coordination Centre |
| REST | REpresentational State Transfer |
| XML | Extensible Markup Language |

# References

[1] 	International Organization for Marine Aids to Navigation (IALA), “IALA marine aids to navigation (AtoN) service specification - provision of AtoN information to end-users,” IALA G1128 Service Specification ssXXXX.

[2] 	International Electrotechnical Commission (IEC), “Maritime navigation and radiocommunication equipment and systems - data interfaces - part 2: Secure communication between ship and shore (SECOM),” Guideline 63173-2. Available: https://webstore.iec.ch/en/publication/64543

[3] 	International Maritime Organization (IMO), “Descriptions of maritime services in the context of e-navigation,” Circular IMO-MSC.1/Circ.1610/Rev.1, 2024.

[4] 	International Organization for Marine Aids to Navigation (IALA), “G1128 - the specification of e-navigation technical services,” Guideline 1128.

[5] 	International Organization for Marine Aids to Navigation (IALA), “G1128 - annex d - service design template for SECOM service,” Guideline 1128-D.

[6] 	International Organization for Marine Aids to Navigation (IALA), “G1143 - unique identifiers for maritime resources,” Guideline 1143.

[7] 	International Organization for Marine Aids to Navigation (IALA), “G1191 - maritime service registry (MSR) technical specification,” Guideline 1191.

[8] 	International Hydrographic Organization (IHO), “S-100 – universal hydrographic data model,” Standard 5.2.0. Available: https://iho.int/uploads/user/pubs/standards/s-100/S-100_5.2.0_Final_Clean.pdf

# Annex A. OpenAPI Template

{
  "openapi": "3.0.1",
  "info": {
    "title": "AtoN Service - SECOM v1.0 Interfaces",
    "description": "The SECOM interfaces of the AtoN Service",
    "termsOfService": "https://gla-rad.org/",
    "contact": {
      "email": "Nikolaos.Vastardis@gla-rad.org"
    },
    "license": {
      "name": "Apache 2.0",
      "url": "http://www.apache.org/licenses/LICENSE-2.0.html"
    },
    "version": "v0.0.1"
  },
  "externalDocs": {
    "description": "SpringShop Wiki Documentation",
    "url": "https://springshop.wiki.github.org/docs"
  },
  "servers": [
    {
      "url": ".."
    },
    {
      "url": "."
    },
    {
      "url": "/api/secom"
    }
  ],
  "paths": {
    "/v1/acknowledgement": {
      "post": {
        "tags": [
          "SECOM"
        ],
        "operationId": "acknowledgment",
        "requestBody": {
          "content": {
            "application/json": {
              "schema": {
                "$ref": "#/components/schemas/AcknowledgementObject",
                "exampleSetFlag": false
              },
              "exampleSetFlag": false
            }
          }
        },
        "responses": {
          "default": {
            "description": "default response",
            "content": {
              "application/json": {
                "schema": {
                  "$ref": "#/components/schemas/AcknowledgementResponseObject",
                  "exampleSetFlag": false
                },
                "exampleSetFlag": false
              }
            }
          }
        }
      }
    },
    "/v1/capability": {
      "get": {
        "tags": [
          "SECOM"
        ],
        "operationId": "capability",
        "responses": {
          "default": {
            "description": "default response",
            "content": {
              "application/json": {
                "schema": {
                  "$ref": "#/components/schemas/CapabilityResponseObject",
                  "exampleSetFlag": false
                },
                "exampleSetFlag": false
              }
            }
          }
        }
      }
    },
    "/v1/object": {
      "get": {
        "tags": [
          "SECOM"
        ],
        "operationId": "get",
        "parameters": [
          {
            "name": "dataReference",
            "in": "query",
            "schema": {
              "type": "string",
              "format": "uuid",
              "exampleSetFlag": false,
              "types": [
                "string"
              ]
            }
          },
          {
            "name": "containerType",
            "in": "query",
            "schema": {
              "type": "string",
              "exampleSetFlag": false,
              "types": [
                "string"
              ],
              "enum": [
                "0",
                "1",
                "2"
              ]
            }
          },
          {
            "name": "dataProductType",
            "in": "query",
            "schema": {
              "type": "string",
              "exampleSetFlag": false,
              "types": [
                "string"
              ],
              "enum": [
                "OTHER",
                "S57",
                "S101",
                "S102",
                "S104",
                "S111",
                "S122",
                "S123",
                "S124",
                "S125",
                "S126",
                "S127",
                "S128",
                "S129",
                "S131",
                "S201",
                "S210",
                "S211",
                "S212",
                "S401",
                "S402",
                "S411",
                "S412",
                "S413",
                "S414",
                "S421",
                "RTZ",
                "EPC"
              ]
            }
          },
          {
            "name": "productVersion",
            "in": "query",
            "schema": {
              "type": "string",
              "exampleSetFlag": false,
              "types": [
                "string"
              ]
            }
          },
          {
            "name": "geometry",
            "in": "query",
            "schema": {
              "type": "string",
              "exampleSetFlag": false,
              "types": [
                "string"
              ]
            }
          },
          {
            "name": "unlocode",
            "in": "query",
            "schema": {
              "pattern": "[A-Z]{5}",
              "type": "string",
              "exampleSetFlag": false,
              "types": [
                "string"
              ]
            }
          },
          {
            "name": "validFrom",
            "in": "query",
            "schema": {
              "pattern": "(\\d{8})T(\\d{6})(Z|\\+\\d{4})?",
              "type": "string",
              "exampleSetFlag": false,
              "types": [
                "string"
              ]
            },
            "example": "20200101T123000"
          },
          {
            "name": "validTo",
            "in": "query",
            "schema": {
              "pattern": "(\\d{8})T(\\d{6})(Z|\\+\\d{4})?",
              "type": "string",
              "exampleSetFlag": false,
              "types": [
                "string"
              ]
            },
            "example": "20200101T123000"
          },
          {
            "name": "page",
            "in": "query",
            "schema": {
              "minimum": 1,
              "type": "integer",
              "format": "int32",
              "exampleSetFlag": false,
              "types": [
                "integer"
              ]
            }
          },
          {
            "name": "pageSize",
            "in": "query",
            "schema": {
              "minimum": 0,
              "type": "integer",
              "format": "int32",
              "exampleSetFlag": false,
              "types": [
                "integer"
              ]
            }
          }
        ],
        "responses": {
          "default": {
            "description": "default response",
            "content": {
              "application/json": {
                "schema": {
                  "$ref": "#/components/schemas/GetResponseObject",
                  "exampleSetFlag": false
                },
                "exampleSetFlag": false
              }
            }
          }
        }
      }
    },
    "/v1/object/summary": {
      "get": {
        "tags": [
          "SECOM"
        ],
        "operationId": "getSummary",
        "parameters": [
          {
            "name": "containerType",
            "in": "query",
            "schema": {
              "type": "string",
              "exampleSetFlag": false,
              "types": [
                "string"
              ],
              "enum": [
                "0",
                "1",
                "2"
              ]
            }
          },
          {
            "name": "dataProductType",
            "in": "query",
            "schema": {
              "type": "string",
              "exampleSetFlag": false,
              "types": [
                "string"
              ],
              "enum": [
                "OTHER",
                "S57",
                "S101",
                "S102",
                "S104",
                "S111",
                "S122",
                "S123",
                "S124",
                "S125",
                "S126",
                "S127",
                "S128",
                "S129",
                "S131",
                "S201",
                "S210",
                "S211",
                "S212",
                "S401",
                "S402",
                "S411",
                "S412",
                "S413",
                "S414",
                "S421",
                "RTZ",
                "EPC"
              ]
            }
          },
          {
            "name": "productVersion",
            "in": "query",
            "schema": {
              "type": "string",
              "exampleSetFlag": false,
              "types": [
                "string"
              ]
            }
          },
          {
            "name": "geometry",
            "in": "query",
            "schema": {
              "type": "string",
              "exampleSetFlag": false,
              "types": [
                "string"
              ]
            }
          },
          {
            "name": "unlocode",
            "in": "query",
            "schema": {
              "pattern": "[A-Z]{5}",
              "type": "string",
              "exampleSetFlag": false,
              "types": [
                "string"
              ]
            }
          },
          {
            "name": "validFrom",
            "in": "query",
            "schema": {
              "pattern": "(\\d{8})T(\\d{6})(Z|\\+\\d{4})?",
              "type": "string",
              "exampleSetFlag": false,
              "types": [
                "string"
              ]
            },
            "example": "20200101T123000"
          },
          {
            "name": "validTo",
            "in": "query",
            "schema": {
              "pattern": "(\\d{8})T(\\d{6})(Z|\\+\\d{4})?",
              "type": "string",
              "exampleSetFlag": false,
              "types": [
                "string"
              ]
            },
            "example": "20200101T123000"
          },
          {
            "name": "page",
            "in": "query",
            "schema": {
              "minimum": 1,
              "type": "integer",
              "format": "int32",
              "exampleSetFlag": false,
              "types": [
                "integer"
              ]
            }
          },
          {
            "name": "pageSize",
            "in": "query",
            "schema": {
              "minimum": 0,
              "type": "integer",
              "format": "int32",
              "exampleSetFlag": false,
              "types": [
                "integer"
              ]
            }
          }
        ],
        "responses": {
          "default": {
            "description": "default response",
            "content": {
              "application/json": {
                "schema": {
                  "$ref": "#/components/schemas/GetSummaryResponseObject",
                  "exampleSetFlag": false
                },
                "exampleSetFlag": false
              }
            }
          }
        }
      }
    },
    "/v1/ping": {
      "get": {
        "tags": [
          "SECOM"
        ],
        "operationId": "ping",
        "responses": {
          "default": {
            "description": "default response",
            "content": {
              "application/json": {
                "schema": {
                  "$ref": "#/components/schemas/PingResponseObject",
                  "exampleSetFlag": false
                },
                "exampleSetFlag": false
              }
            }
          }
        }
      }
    },
    "/v1/subscription": {
      "post": {
        "tags": [
          "SECOM"
        ],
        "operationId": "subscription",
        "requestBody": {
          "content": {
            "application/json": {
              "schema": {
                "$ref": "#/components/schemas/SubscriptionRequestObject",
                "exampleSetFlag": false
              },
              "exampleSetFlag": false
            }
          }
        },
        "responses": {
          "default": {
            "description": "default response",
            "content": {
              "application/json": {
                "schema": {
                  "$ref": "#/components/schemas/SubscriptionResponseObject",
                  "exampleSetFlag": false
                },
                "exampleSetFlag": false
              }
            }
          }
        }
      },
      "delete": {
        "tags": [
          "SECOM"
        ],
        "operationId": "removeSubscription",
        "requestBody": {
          "content": {
            "application/json": {
              "schema": {
                "$ref": "#/components/schemas/RemoveSubscriptionObject",
                "exampleSetFlag": false
              },
              "exampleSetFlag": false
            }
          }
        },
        "responses": {
          "default": {
            "description": "default response",
            "content": {
              "application/json": {
                "schema": {
                  "$ref": "#/components/schemas/RemoveSubscriptionResponseObject",
                  "exampleSetFlag": false
                },
                "exampleSetFlag": false
              }
            }
          }
        }
      }
    }
  },
  "components": {
    "schemas": {
      "secom-v1": {
        "type": "object",
        "exampleSetFlag": false,
        "types": [
          "object"
        ],
        "jsonSchema": {}
      },
      "AcknowledgementResponseObject": {
        "type": "object",
        "properties": {
          "message": {
            "type": "string",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          },
          "SECOM_ResponseCode": {
            "type": "integer",
            "format": "int32",
            "exampleSetFlag": false,
            "types": [
              "integer"
            ]
          }
        },
        "exampleSetFlag": false
      },
      "AcknowledgementObject": {
        "required": [
          "digitalSignature",
          "envelope"
        ],
        "type": "object",
        "properties": {
          "envelope": {
            "$ref": "#/components/schemas/EnvelopeAckObject",
            "exampleSetFlag": false
          },
          "digitalSignature": {
            "type": "string",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          }
        },
        "exampleSetFlag": false
      },
      "EnvelopeAckObject": {
        "required": [
          "ackType",
          "createdAt",
          "envelopeCertificate",
          "envelopeRootCertificateThumbprint",
          "envelopeSignatureTime",
          "transactionIdentifier"
        ],
        "type": "object",
        "properties": {
          "envelopeRootCertificateThumbprint": {
            "type": "string",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          },
          "envelopeSignatureTime": {
            "pattern": "(\\d{8})T(\\d{6})(Z|\\+\\d{4})?",
            "type": "string",
            "description": "The envelope signature date-time",
            "example": "19850412T101530",
            "exampleSetFlag": true,
            "types": [
              "string"
            ]
          },
          "createdAt": {
            "pattern": "(\\d{8})T(\\d{6})(Z|\\+\\d{4})?",
            "type": "string",
            "description": "The creation date-time",
            "example": "19850412T101530",
            "exampleSetFlag": true,
            "types": [
              "string"
            ]
          },
          "transactionIdentifier": {
            "type": "string",
            "format": "uuid",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          },
          "ackType": {
            "type": "integer",
            "format": "int32",
            "exampleSetFlag": false,
            "types": [
              "integer"
            ]
          },
          "nackType": {
            "type": "integer",
            "format": "int32",
            "exampleSetFlag": false,
            "types": [
              "integer"
            ]
          },
          "envelopeCertificate": {
            "type": "string",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          }
        },
        "exampleSetFlag": false
      },
      "CapabilityObject": {
        "required": [
          "containerType",
          "dataProductType",
          "implementedInterfaces",
          "productSchemaUrl",
          "serviceVersion"
        ],
        "type": "object",
        "properties": {
          "containerType": {
            "type": "integer",
            "format": "int32",
            "exampleSetFlag": false,
            "types": [
              "integer"
            ]
          },
          "dataProductType": {
            "type": "string",
            "exampleSetFlag": false,
            "types": [
              "string"
            ],
            "enum": [
              "OTHER",
              "S57",
              "S101",
              "S102",
              "S104",
              "S111",
              "S122",
              "S123",
              "S124",
              "S125",
              "S126",
              "S127",
              "S128",
              "S129",
              "S131",
              "S201",
              "S210",
              "S211",
              "S212",
              "S401",
              "S402",
              "S411",
              "S412",
              "S413",
              "S414",
              "S421",
              "RTZ",
              "EPC"
            ]
          },
          "productSchemaUrl": {
            "type": "string",
            "format": "url",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          },
          "implementedInterfaces": {
            "$ref": "#/components/schemas/ImplementedInterfaces",
            "exampleSetFlag": false
          },
          "serviceVersion": {
            "type": "string",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          }
        },
        "exampleSetFlag": false
      },
      "CapabilityResponseObject": {
        "type": "object",
        "properties": {
          "capability": {
            "type": "array",
            "exampleSetFlag": false,
            "items": {
              "$ref": "#/components/schemas/CapabilityObject",
              "exampleSetFlag": false
            },
            "types": [
              "array"
            ]
          }
        },
        "exampleSetFlag": false
      },
      "ImplementedInterfaces": {
        "required": [
          "access",
          "encryptionKey",
          "get",
          "getByLink",
          "getSummary",
          "subscription",
          "upload",
          "uploadLink"
        ],
        "type": "object",
        "properties": {
          "upload": {
            "type": "boolean",
            "exampleSetFlag": false,
            "types": [
              "boolean"
            ]
          },
          "uploadLink": {
            "type": "boolean",
            "exampleSetFlag": false,
            "types": [
              "boolean"
            ]
          },
          "get": {
            "type": "boolean",
            "exampleSetFlag": false,
            "types": [
              "boolean"
            ]
          },
          "getSummary": {
            "type": "boolean",
            "exampleSetFlag": false,
            "types": [
              "boolean"
            ]
          },
          "getByLink": {
            "type": "boolean",
            "exampleSetFlag": false,
            "types": [
              "boolean"
            ]
          },
          "subscription": {
            "type": "boolean",
            "exampleSetFlag": false,
            "types": [
              "boolean"
            ]
          },
          "access": {
            "type": "boolean",
            "exampleSetFlag": false,
            "types": [
              "boolean"
            ]
          },
          "encryptionKey": {
            "type": "boolean",
            "exampleSetFlag": false,
            "types": [
              "boolean"
            ]
          }
        },
        "exampleSetFlag": false
      },
      "DataResponseObject": {
        "required": [
          "ackRequest",
          "data",
          "exchangeMetadata"
        ],
        "type": "object",
        "properties": {
          "data": {
            "type": "string",
            "format": "byte",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          },
          "exchangeMetadata": {
            "$ref": "#/components/schemas/SECOM_ExchangeMetadataObject",
            "exampleSetFlag": false
          },
          "ackRequest": {
            "type": "integer",
            "format": "int32",
            "exampleSetFlag": false,
            "types": [
              "integer"
            ]
          }
        },
        "exampleSetFlag": false
      },
      "DigitalSignatureValue": {
        "required": [
          "digitalSignature",
          "publicCertificate"
        ],
        "type": "object",
        "properties": {
          "publicRootCertificateThumbprint": {
            "type": "string",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          },
          "publicCertificate": {
            "type": "string",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          },
          "digitalSignature": {
            "type": "string",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          }
        },
        "exampleSetFlag": false
      },
      "GetResponseObject": {
        "required": [
          "pagination"
        ],
        "type": "object",
        "properties": {
          "dataResponseObject": {
            "type": "array",
            "exampleSetFlag": false,
            "items": {
              "$ref": "#/components/schemas/DataResponseObject",
              "exampleSetFlag": false
            },
            "types": [
              "array"
            ]
          },
          "pagination": {
            "$ref": "#/components/schemas/PaginationObject",
            "exampleSetFlag": false
          },
          "responseText": {
            "type": "string",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          }
        },
        "exampleSetFlag": false
      },
      "PaginationObject": {
        "required": [
          "maxItemsPerPage",
          "totalItems"
        ],
        "type": "object",
        "properties": {
          "totalItems": {
            "type": "integer",
            "format": "int32",
            "exampleSetFlag": false,
            "types": [
              "integer"
            ]
          },
          "maxItemsPerPage": {
            "type": "integer",
            "format": "int32",
            "exampleSetFlag": false,
            "types": [
              "integer"
            ]
          }
        },
        "exampleSetFlag": false
      },
      "SECOM_ExchangeMetadataObject": {
        "required": [
          "compressionFlag",
          "dataProtection",
          "digitalSignatureReference",
          "digitalSignatureValue",
          "protectionScheme"
        ],
        "type": "object",
        "properties": {
          "dataProtection": {
            "type": "boolean",
            "exampleSetFlag": false,
            "types": [
              "boolean"
            ]
          },
          "protectionScheme": {
            "type": "string",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          },
          "digitalSignatureReference": {
            "type": "string",
            "exampleSetFlag": false,
            "types": [
              "string"
            ],
            "enum": [
              "dsa",
              "ecdsa-256-sha2-256",
              "ecdsa-256-sha3-256",
              "ecdsa-384-sha2",
              "ecdsa-384-sha3",
              "cvc_ecdsa"
            ]
          },
          "digitalSignatureValue": {
            "$ref": "#/components/schemas/DigitalSignatureValue",
            "exampleSetFlag": false
          },
          "compressionFlag": {
            "type": "boolean",
            "exampleSetFlag": false,
            "types": [
              "boolean"
            ]
          }
        },
        "exampleSetFlag": false
      },
      "GetSummaryResponseObject": {
        "required": [
          "pagination"
        ],
        "type": "object",
        "properties": {
          "summaryObject": {
            "type": "array",
            "exampleSetFlag": false,
            "items": {
              "$ref": "#/components/schemas/SummaryObject",
              "exampleSetFlag": false
            },
            "types": [
              "array"
            ]
          },
          "pagination": {
            "$ref": "#/components/schemas/PaginationObject",
            "exampleSetFlag": false
          },
          "responseText": {
            "type": "string",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          }
        },
        "exampleSetFlag": false
      },
      "SummaryObject": {
        "required": [
          "containerType",
          "dataCompression",
          "dataProductType",
          "dataProtection",
          "dataReference"
        ],
        "type": "object",
        "properties": {
          "dataReference": {
            "type": "string",
            "format": "uuid",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          },
          "dataProtection": {
            "type": "boolean",
            "exampleSetFlag": false,
            "types": [
              "boolean"
            ]
          },
          "dataCompression": {
            "type": "boolean",
            "exampleSetFlag": false,
            "types": [
              "boolean"
            ]
          },
          "containerType": {
            "type": "integer",
            "format": "int32",
            "exampleSetFlag": false,
            "types": [
              "integer"
            ]
          },
          "dataProductType": {
            "type": "string",
            "exampleSetFlag": false,
            "types": [
              "string"
            ],
            "enum": [
              "OTHER",
              "S57",
              "S101",
              "S102",
              "S104",
              "S111",
              "S122",
              "S123",
              "S124",
              "S125",
              "S126",
              "S127",
              "S128",
              "S129",
              "S131",
              "S201",
              "S210",
              "S211",
              "S212",
              "S401",
              "S402",
              "S411",
              "S412",
              "S413",
              "S414",
              "S421",
              "RTZ",
              "EPC"
            ]
          },
          "info_identifier": {
            "type": "string",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          },
          "info_name": {
            "type": "string",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          },
          "info_status": {
            "type": "string",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          },
          "info_description": {
            "type": "string",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          },
          "info_lastModifiedDate": {
            "pattern": "(\\d{8})T(\\d{6})(Z|\\+\\d{4})?",
            "type": "string",
            "description": "The last modified date-time",
            "example": "19850412T101530",
            "exampleSetFlag": true,
            "types": [
              "string"
            ]
          },
          "info_productVersion": {
            "type": "string",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          },
          "info_size": {
            "type": "integer",
            "format": "int64",
            "exampleSetFlag": false,
            "types": [
              "integer"
            ]
          }
        },
        "exampleSetFlag": false
      },
      "PingResponseObject": {
        "type": "object",
        "properties": {
          "lastPrivateInteractionTime": {
            "pattern": "(\\d{8})T(\\d{6})(Z|\\+\\d{4})?",
            "type": "string",
            "description": "The last private interaction date-time",
            "example": "19850412T101530",
            "exampleSetFlag": true,
            "types": [
              "string"
            ]
          }
        },
        "exampleSetFlag": false
      },
      "RemoveSubscriptionResponseObject": {
        "type": "object",
        "properties": {
          "message": {
            "type": "string",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          }
        },
        "exampleSetFlag": false
      },
      "RemoveSubscriptionObject": {
        "required": [
          "subscriptionIdentifier"
        ],
        "type": "object",
        "properties": {
          "subscriptionIdentifier": {
            "type": "string",
            "format": "uuid",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          }
        },
        "exampleSetFlag": false
      },
      "SubscriptionResponseObject": {
        "type": "object",
        "properties": {
          "subscriptionIdentifier": {
            "type": "string",
            "format": "uuid",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          },
          "message": {
            "type": "string",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          }
        },
        "exampleSetFlag": false
      },
      "SubscriptionRequestObject": {
        "type": "object",
        "properties": {
          "containerType": {
            "type": "integer",
            "format": "int32",
            "exampleSetFlag": false,
            "types": [
              "integer"
            ]
          },
          "dataProductType": {
            "type": "string",
            "exampleSetFlag": false,
            "types": [
              "string"
            ],
            "enum": [
              "OTHER",
              "S57",
              "S101",
              "S102",
              "S104",
              "S111",
              "S122",
              "S123",
              "S124",
              "S125",
              "S126",
              "S127",
              "S128",
              "S129",
              "S131",
              "S201",
              "S210",
              "S211",
              "S212",
              "S401",
              "S402",
              "S411",
              "S412",
              "S413",
              "S414",
              "S421",
              "RTZ",
              "EPC"
            ]
          },
          "dataReference": {
            "type": "string",
            "format": "uuid",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          },
          "productVersion": {
            "type": "string",
            "exampleSetFlag": false,
            "types": [
              "string"
            ]
          },
          "geometry": {
            "pattern": "^([A-Z]+\\s*\\(\\(?\\s*(-?\\d+(\\.\\d+)?)\\s+-?\\d+(\\.\\d+)?(?:\\s+-?\\d+(\\.\\d+)?)?\\s*(,\\s*(-?\\d+(\\.\\d+)?)\\s+-?\\d+(\\.\\d+)?(?:\\s+-?\\d+(\\.\\d+)?)?\\s*)*\\)\\)?\\s*)+$",
            "type": "string",
            "description": "The subscription geometry",
            "example": "POLYGON ((0.65 51.42, 0.65 52.26, 2.68 52.26, 2.68 51.42, 0.65 51.42))",
            "exampleSetFlag": true,
            "types": [
              "string"
            ]
          },
          "unlocode": {
            "pattern": "[A-Z]{5}",
            "type": "string",
            "description": "The subscription area as UNLOCODE",
            "example": "GBHRW",
            "exampleSetFlag": true,
            "types": [
              "string"
            ]
          },
          "subscriptionPeriodStart": {
            "pattern": "(\\d{8})T(\\d{6})(Z|\\+\\d{4})?",
            "type": "string",
            "description": "The subscription period start",
            "example": "19850412T101530",
            "exampleSetFlag": true,
            "types": [
              "string"
            ]
          },
          "subscriptionPeriodEnd": {
            "pattern": "(\\d{8})T(\\d{6})(Z|\\+\\d{4})?",
            "type": "string",
            "description": "The subscription period end",
            "example": "19850412T101530",
            "exampleSetFlag": true,
            "types": [
              "string"
            ]
          }
        },
        "exampleSetFlag": false
      }
    }
  }
}

# Annex B. G1128 Instance Description XML Template

<?xml version="1.0" encoding="UTF-8" standalone="yes"?>
<serviceInstance
  xmlns="http://iala-aism.org/g1128/v1.6/ServiceInstanceSchema.xsd"
  xmlns:ns2="http://iala-aism.org/g1128/v1.6/ServiceSpecificationSchema.xsd">
    <id>urn:mrn:mcp:service:mcc:grad:instance:aton-service</id>
    <version>0.0.1</version>
    <name>Test AtoN Service</name>
    <status>provisional</status>
    <description>An example of a G1128 instance description XML</description>
    <keywords>aton service g1128 example</keywords>
    <endpoint>https://localhost:8443/api/secom</endpoint>
    <serviceType>other:atonservice</serviceType>
    <requiresAuthorization>true</requiresAuthorization>
    <offersServiceLevel>
        <availability>0.0</availability>
        <name>Development</name>
        <description>This service implementation is under active development, 0% availability is guaranteed.</description>
    </offersServiceLevel>
    <coversAreas>
        <coversArea>
            <name>Example</name>
            <description>Description on the example area</description>
            <geometryAsWKT>POLYGON ((0.0 0.0, 0.0 1.1, 1.1 1,1, 1.1 0.0 ))</geometryAsWKT>
        </coversArea>
    </coversAreas>
    <implementsServiceDesign>
        <id>urn:mrn:iala:si:atonservice:0.0.1</id>
        <version>0.0.0</version>
    </implementsServiceDesign>
    <producedBy>
        <ns2:id>urn:mrn:iala</ns2:id>
        <ns2:name>IALA</ns2:name>
        <ns2:description>International Organization of Marine Aids to Navigation</ns2:description>
        <ns2:contactInfo>https://www.iala.int/</ns2:contactInfo>
        <ns2:isCommercial>false</ns2:isCommercial>
    </producedBy>
    <providedBy>
        <ns2:id>urn:mrn:iala</ns2:id>
        <ns2:name>IALA</ns2:name>
        <ns2:description>International Organization of Marine Aids to Navigation</ns2:description>
        <ns2:contactInfo>https://www.iala.int/</ns2:contactInfo>
        <ns2:isCommercial>false</ns2:isCommercial>
    </providedBy>
</serviceInstance>