Input paper:   ARM22-8.1.1

Date: 23 Mar 2025

Input paper for the following Committee(s): 		Purpose of paper:

(Select as appropriate)

☒ ARM	☐  ENG		☐ PAP			☒ Input

☐ ENAV	☐ VTS					☐ Information

Agenda item 			8.1

Technical domain/ Task number 2	1.4.5

Author(s)/Submitter(s)			Im Namkyun, Kim Ahra / Mokpo National Maritime University

# PROPOSAL FOR INTRODUCING A RISK MANAGEMENT FRAMEWORK BASED ON THE MARITIME HYDRO-METEO FUSION INDEX (MHFI) INCLUDING SEA ICE INFORMATION

# Summary

Effective maritime risk assessment requires consideration not only of vessel traffic patterns but also of environmental conditions that directly affect navigational safety. IALA Guideline G1018 provides a range of risk assessment tools — including PAWSA, IWRAP, and SIRA — that are effective for analysing collision, grounding, and contact risks in congested waters. However, the current Toolbox lacks a framework for quantitatively assessing environmental hazard factors such as wave height, swell period, tidal current, wind speed, and visibility. In practice, maritime accidents frequently occur when such environmental factors combine with traffic conditions, underscoring the need for an environment-based assessment tool.

To address this gap, ARM 21 session document 9.1.1 proposed the Maritime Hydro-Meteo Fusion Index (MFI) as an environment-based risk assessment tool. At the WG3 meeting, participants identified the following areas for improvement:

- Addition of sea ice information (ice density map visualisation and data availability review)

- Provision of a means to identify the highest-risk contributing factor (major contributor display)

- Visualisation of directional (vector) information for input variables

- Clarification of terminology (factor, parameter, oceanographic, etc.)

- Review of data accessibility and spatial availability

- Development of differentiated index systems by vessel size (small vessel / port / coastal)

- Reference to the Portuguese Navy Impact Diagram case study

- Parallel development of risk reduction measures using AtoN

- Phased implementation approach by country

In response, this paper proposes an enhanced MFI incorporating sea ice as the sixth environmental variable. The key features are:

- Six environmental variables — wave height, swell period, tidal current, wind speed, visibility, and sea ice — integrated into a single standardised 0–4 index.

- Results visualised on electronic navigational charts using LV0–LV4 colour coding (green → yellow → orange → red → crimson).

- Function added to identify the highest-risk contributing factor among all variables.

Incorporating the MHFI into the IALA Risk Management Toolbox would extend the current traffic-centric framework to encompass environmental factors, supporting safe navigation in coastal and port waters, providing objective decision criteria for both small and large vessels, and enabling application in polar environments.

# Background

As proposed in ARM 21 session document 9.1.1, the IALA G1018 Risk Management Toolbox currently operates primarily around AIS traffic-data-based assessment tools — including IWRAP, PAWSA, and SIRA — which are effective for traffic-volume-oriented risk assessment. However, maritime accidents frequently arise from the combination of traffic factors and hydro-meteorological conditions such as wave height, swell, tidal current, wind speed, and visibility, yet no method for quantitatively assessing these environmental risk factors exists within the Toolbox.

ARM 21 session document 9.1.1 proposed the MFI as an environment-based risk assessment tool. The present paper proposes an enhanced version adding sea ice as a sixth variable. The original five-variable MFI considered wave height, swell period, tidal current, wind speed, and visibility; the updated MHFI integrates all six factors into a unified index.

The MHFI is intended to complement the existing traffic-centric tools and to serve as a practical and innovative instrument supporting safety for coastal and small vessel navigation, and should therefore be considered in the context of revising the Risk Assessment Toolbox within Guideline G1018.

# Discussion

The MHFI quantitatively assesses various hydro-meteorological conditions affecting vessel operations and presents corresponding risk levels. Six core variables are quantified individually, and a composite risk score is derived. Each variable is classified into five levels (Safe – Caution – Warning – Danger – Extreme Danger) based on international standards, accident statistics, and navigation restriction regulations. The final composite score is calculated by aggregating the three highest-risk variables and displayed on an S-100-based navigational chart.

## Sea Ice Data and POLARIS-Based Risk Quantification

The sea ice navigational hazard component of the MHFI is quantified using the POLARIS Risk Index Outcome (RIO), defined in IMO MSC.1/Circ.1519 (§1.2.2). For a given grid cell:

RIO = Σ (Cᵢ × RIVᵢ)

Cᵢ is the concentration of ice type i (0–10, in tenths), and RIVᵢ is the Risk Index Value for that ice type for the reference vessel class (PC7 for MHFI). A positive RIO indicates conditions within the vessel's design tolerance; a negative RIO indicates exceedance.

POLARIS defines three operational levels. For the purposes of MHFI, this has been expanded to five levels to align with the existing framework. The POLARIS three-level framework is illustrated in Figure 1.

Figure 1. POLARIS three-level operational framework (IMO MSC.1/Circ.1519)

The resulting five-level MHFI classification is presented in Table 1.

Table 1. MHFI Sea Ice Risk Classification (5 Levels) — PC7 POLARIS Reference Class

| Condition | RIO Range (PC7) | Level | Navigational Meaning (Aice) |
| --- | --- | --- | --- |
| Safe (LV0) | RIO ≥ +10 | 0–1 | No restriction; safe navigation |
| Caution (LV1) | 0 ≤ RIO < +10 | 1–2 | Minor ice resistance; navigation generally manageable |
| Warning (LV2) | −10 ≤ RIO < 0 | 2–3 | Recommended speed limits; elevated operational risk |
| Danger (LV3) | −20 ≤ RIO < −10 | 3–4 | Severe resistance; ice-strengthened vessels recommended |
| Extreme Danger (LV4+) | RIO < −20 | > 4 | Icebreaker assistance required; operation under special consideration |

The boundary values are derived from POLARIS thresholds in IMO MSC.1/Circ.1519 §1.6.4. The upper boundary b₀ = +10 corresponds to the Normal Operation lower bound corrected by the §1.6.4 escort correction; b₃ = −20 represents the Special Consideration boundary. Intermediate boundaries b₁ = 0 and b₂ = −10 are placed symmetrically to yield five operationally meaningful levels.

## Integration into the MHFI Framework

All six MHFI variables are independently standardised to a 0–4 scale and combined through a weighted composite formula:

MHFI  =  A_max  +  0.2 × A_2nd  +  0.1 × A_3rd

A_max, A_2nd, and A_3rd are the highest, second-highest, and third-highest individual variable scores, respectively, among the six variables at a given location and time. The weighting structure reflects findings from maritime accident analysis: adverse conditions rarely cause accidents in isolation, and risk increases substantially when two or more hazard factors occur simultaneously.

## MHFI Risk Classification Criteria for All Six Variables

Classification criteria for wave height, swell period, tidal current speed, wind speed, and visibility are described in ARM 21 input paper 9.1.1. The sea ice component is newly introduced in this paper.

Table 2. MHFI Risk Classification Criteria for All Six Variables

| Variable | Vessel | LV0 Safe | LV1 Caution | LV2 Warning | LV3 Danger | LV4 Extreme |
| --- | --- | --- | --- | --- | --- | --- |
| Wave Height (Hs) | General | ≤ 0.5 m | ≤ 1.25 m | ≤ 2.5 m | 2.5–4.0 m | > 4.0 m |
|  | Small Vessel | ≤ Hs_IMO | ≤ Hs IMO | ≤ Hs IMO×1.2 | ≤ Hs IMO×1.5 | > Hs IMO×1.5 |
| Swell Period (sec) | All | ≤ 4 s | ≤ 6 s | ≤ 8 s | 8–11 s | > 11 s |
| Current Speed (kts) | All | ≤ 0.6 kts | ≤ 1.6 kts | ≤ 2.6 kts | ≤ 3.2 kts | ≥ 4.0 / ≥4.5 (Δv≥2.06 m/s) |
| Wind Speed (m/s) | General | ≤ 5.4 m/s | ≤ 10.7 m/s | ≤ 13.8 m/s | ≤ 17.1 m/s | ≥ 17.2 m/s |
|  | Small Vessel | ≤ 5.4 m/s | ≤ 7 m/s | ≤ 10 m/s | ≤ 14 m/s | > 14 m/s |
| Visibility (km) | All | > 11.1 km | 11.1–5.5 km | ~5.5–1.85 km | 1.85–1 km | < 1 km |
| Sea Ice (PC7 POLARIS RIO) | All | RIO ≥ +10 | 0 ≤ RIO < +10 | −10 ≤ RIO < 0 | −20 ≤ RIO < −10 | RIO < −20 |
| Individual Aᵢ Index |  | 0–1 | 1–2 | 2–3 | 3–4 | > 4+ |

# Application and Utilisation of the Model

The figure below shows part of the developed MHFI visualisation program. As shown, the overall MHFI risk level for the area is differentiated by colour, allowing easy identification of risk factors for each weather variable — that is, the risk index for each of the six elements (wave height, swell, tidal current, wind speed, visibility, and sea ice) can be individually assessed. By selecting and clicking on a specific sea area, the overall MHFI value together with the risk index for each weather factor are displayed, making it possible to identify which weather element is most contributing to the current hazardous situation and its value.

The primary purpose of this MHFI is to provide a risk assessment map by integrating various meteorological factors during navigation. The ultimate goal is to display it on VTS or onboard navigation equipment such as ECDIS.

The MHFI can be visualised on S-100-based ECDIS. Risk levels are represented using the LV0–LV4 colour scheme (green → yellow → orange → red → crimson), enabling navigators and administrators to intuitively assess risk levels directly on the chart.

Incorporating the MHFI into the IALA Risk Management Toolbox would extend the current traffic-centric framework to encompass environmental factors comprehensively, supporting safe navigation in coastal and port waters, and providing objective decision criteria for both small and large vessels. Given the inclusion of sea ice information, the MHFI is also applicable as a risk assessment index for polar environments.

Figure 2. Example MHFI visualisation on S-100-based ECDIS — LV0–LV4 colour scheme (Mokpo coastal waters)

# References

1.  Bué, I., Semedo, Á., and Catalão, J. (2020). Evaluation of HF Radar Wave Measurements in Iberian Peninsula. Remote Sensing, 12(21), 3623.

2.  COLREGs (1972, as amended). Convention on the International Regulations for Preventing Collisions at Sea. IMO, London.

3.  Hwang, S. M. et al. (2018). Analysis of East Coast Swell Accident Characteristics. Journal of Korean Society of Coastal and Ocean Engineers, 30(6), 235–241.

4.  IACS (2001). Guidance on the Assessment of Ship Loading Conditions. London.

5.  IALA, Guideline G1018, Risk Management, Edition 4.0, June 2022.

6.  IMO (2007). MSC/Circ.1228 — Revised Guidance to the Master for Avoiding Dangerous Situations in Adverse Weather and Sea Conditions. London.

7.  IMO (2016). MSC.1/Circ.1519 — Guidance on the Polar Code: POLARIS. London.

8.  ITTC (2014). Guidelines for Sea-keeping Assessment of Ships. Recommended Procedures 7.5-02-07-02.2.

9.  Japan Coast Guard (2010). Official Website. https://www6.kaiho.mlit.go.jp

10.  JCOMM / GOOS (2017). Recognition of the Global HF Radar Network as a GOOS Observing Network.

11.  KMA. Official Website. https://www.kma.go.kr/kma

12.  Korea Hydrographic and Oceanographic Agency (KHOA). https://www.khoa.go.kr

13.  Korea Maritime Safety Tribunal (KMST) (2023). Marine Accident Statistics 2022. Ministry of Oceans and Fisheries.

14.  Korean Ministry of Oceans and Fisheries. Marine Safety Act (해사안전법). Republic of Korea.

15.  NOAA. New York / New Jersey Operational Forecast System (NYOFS). https://tidesandcurrents.noaa.gov/ofs/nyofs/nyofs.html

16.  Paduan, J.D. and Washburn, L. (2013). High-Frequency Radar Observations of Ocean Surface Currents. Annual Review of Marine Science, 5, 115–136.

17.  Ricker, R. et al. (2017). A weekly Arctic sea-ice thickness data record from merged CryoSat-2 and SMOS satellite data. The Cryosphere, 11, 1607–1623.

18.  Roarty, H. et al. (2019). The Global High Frequency Radar Network. Frontiers in Marine Science, 6, 164.

19.  Stevens Institute of Technology, New York Harbor Observing and Prediction System (NYHOPS). https://hudson.dl.stevens-tech.edu/maritimeforecast/info/

20.  IALA, Vessel Traffic Services (VTS) Manual, Edition 8.4, June 2024.

21.  WMO (2014). WMO No. 259 — Sea Ice Nomenclature. Geneva.

22.  WMO (2018). WMO No. 702 — Guide to Wave Analysis and Forecasting, 2nd edition. Geneva.

23.  WMO (2020). WMO No. 306 — Manual on Codes. Geneva.

24.  WMO (2021). WMO No. 8 — Guide to Instruments and Methods of Observation, 2021 edition. Geneva.

# Action Requested of the Committee

The Committee is invited to:

- note the proposed Maritime Hydro-Meteo Fusion Index (MHFI) framework, including sea ice information, as an environment-based tool for maritime risk assessment;

- consider the potential role of environmental risk indicators — in particular the six-variable MHFI — in complementing the traffic-oriented approaches currently described in Guideline G1018;

- provide comments or guidance regarding the possible future development and incorporation of the MHFI methodology within the IALA Risk Management Toolbox.

........

The following tables reproduce the POLARIS Risk Index Value (RIV) source data from IMO MSC.1/Circ.1519, provided for reference to support verification of the MHFI sea ice classification boundaries in Tables 1 and 2.

PC7 Risk Index Values

PC7 Risk Index Values (RIV) by ice type. (Based on IMO MSC.1/Circ.1519, Table 1.3.)

| Ice Type | Thickness | RIV (PC7) | Navigational Implication |
| --- | --- | --- | --- |
| Ice-Free | 0cm | +3 | No restriction; safe navigation |
| New/Grey/Grey White Ice | 0~30cm | +2 | Very low resistance; normal vessel operation possible |
| Thin First-Year Ice(1-2stage) | 30~70cm | +1 | Minor ice resistance; navigation generally manageable |
| Medium First Year Ice less than 1m thick | 70~100cm | 0 | Noticeable resistance; reduced speed and careful route selection required |
| Medium First-Year Ice | 70~120cm | −1 | Significant resistance; manoeuvring performance reduced |
| Thick First-Year Ice | >120 cm | −2 | Severe resistance; ice-strengthened vessels recommended |
| Second-Year / Heavy(light) Multi year Ice | >200cm(Typical) | −3 | Beyond normal PC7 operating envelope; icebreaker assistance required |

POLARIS Vessel Ice Classes

POLARIS vessel ice classes. PC7 row is the MHFI reference class. IACS Unified Requirements for Polar Class Ships, UR I1, Table 1 (2006/Rev.3 2016)

Full POLARIS Risk Index Value Matrix

Full POLARIS RIV matrix (winter conditions). Green: within design limits; amber: design boundary (RIV = 0); red: exceeds design limits. Source: IMO MSC.1/Circ.1519, Table 1.3.

| Ice Class | Open Water | New Ice | Grey Ice | Grey- White | Thin FY St.1 | Thin FY St.2 | Med FY <1m | Med FY | Thick FY | 2nd Year | Lt. MYI | Hvy. MYI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PC1 | +3 | +3 | +3 | +3 | +2 | +2 | +2 | +2 | +2 | +2 | +1 | +1 |
| PC2 | +3 | +3 | +3 | +3 | +2 | +2 | +2 | +2 | +2 | +1 | +1 | 0 |
| PC3 | +3 | +3 | +3 | +3 | +2 | +2 | +2 | +2 | +2 | +1 | 0 | -1 |
| PC4 | +3 | +3 | +3 | +3 | +2 | +2 | +2 | +2 | +1 | 0 | -1 | -2 |
| PC5 | +3 | +3 | +3 | +3 | +2 | +2 | +1 | +1 | 0 | -1 | -2 | -2 |
| PC6 | +3 | +2 | +2 | +2 | +2 | +1 | +1 | 0 | -1 | -2 | -3 | -3 |
| PC7 | +3 | +2 | +2 | +2 | +1 | +1 | 0 | -1 | -2 | -3 | -3 | -3 |
| IA Super | +3 | +2 | +2 | +2 | +2 | +1 | 0 | -1 | -3 | -3 | -4 | -4 |
| IA | +3 | +2 | +2 | +2 | +1 | 0 | -1 | -2 | -3 | -4 | -5 | -5 |
| IB | +3 | +2 | +2 | +1 | 0 | -1 | -2 | -3 | -4 | -5 | -6 | -6 |
| IC | +3 | +2 | +1 | 0 | -1 | -2 | -3 | -4 | -5 | -6 | -7 | -8 |
| Not Ice Str. | +3 | +1 | 0 | -1 | -2 | -3 | -4 | -5 | -6 | -7 | -8 | -8 |

MHFI Variable Classification Criteria

The following tables are from the ARM21 input paper 9.1.1, which originally defined the classification criteria for environmental variables.

Wave Height Classification

Swell Index Classification

Current Speed Classification

Wind Speed Classification

Visibility Classification

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain

Footer will automatically populate