Input paper:   ARM22-8.1.3

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

☒ ARM	☐  ENG		☐ PAP		                  	☐ Input

☐ DTEC	☐ VTS			         	               	☒ Information

Agenda item  			8.1

Technical Domain / Task Number 	1.4.7

Author(s) / Submitter(s)			China Maritime Safety Administration (China MSA)/ Jimei University

Risk assessment in sea crossing bridge waters based on iwrap

SUMMARY

To enhance the understanding of ship collision risks in sea-crossing bridge waters, this proposal conducts a risk assessment using the IWRAP (IALA Waterway Risk Assessment Program) model, with case studies on the sea areas of Zhanjiang Bay Bridge and Tiaoshun Bridge. By importing annual AIS data from 2022 to 2024 into the IWRAP MKII software, this study estimates and analyzes ship traffic flow, ship collision probability, and pier impact probability in the studied waters. The analysis identifies high-risk navigation segments, waypoints, and major risk ship types. Results indicate a year-on-year increase in ship traffic within the Zhanjiang Bay Bridge area, and alongside upward trends in both pier impact probability and cumulative collision probability. High-risk areas are primarily concentrated in narrow segments where the main channel approaches the bridge zone and in the vicinity of berthing points. Based on these findings, targeted recommendations are proposed for optimizing the layout of AtoNs and enhancing VTS (Vessel Traffic Services) risk prevention and control, thereby providing valuable reference for navigation safety management in sea-crossing bridge waters.

Purpose of the document

The purpose of this document is to verify the applicability of the IWRAP MKII tool for risk assessment in sea-crossing bridge waters and to provide a practical use case for risk assessment in similar scenarios.

Related documents

Burmeister, H-C., et al. "Assessing the frequency and material consequences of collisions with vessels lying at an anchorage in line with IALA iWrap MkII." TransNav: International Journal on Marine Navigation and Safety of Sea Transportation 8.1 (2014)

IALA Standard 1010 on Marine Aids to Navigation Planning and Service Requirements

IALA Guideline 1018 on Risk Management

IALA Guideline 1123 on The Use of IALA Waterway Risk Assessment Program (IWRAP)

Background

- In the past three decades, frequent collision accidents involving sea-crossing bridges worldwide have underscored the critical importance of navigation safety in bridge waters. Although the IALA Waterway Risk Assessment Program (IWRAP)  is a well-established tool for assessing navigational risk in open waters and port areas, its systematic application to sea-crossing bridge scenarios remains relatively underexplored. Existing studies largely focus on macroscopic collision risks across the entire bridge waterway areas and consequently lack targeted risk assessments at critical navigation points.

- The Zhanjiang Bay Bridge and Tiaoshun Bridge are two key sea-crossing infrastructure projects in Zhanjiang City, located within the core area of Zhanjiang Bay—a waterway characterized by complex navigational conditions involving multiple intersections with tributary channels and ferry routes. Figures 1, 2, and 3 present the vessel traffic heat maps for waters surrounding the Zhanjiang Bay Bridge and Tiaoshun Bridge in 2022, 2023, and 2024, respectively. The navigational environment in this area is characterized by strong tidal currents that significantly influence vessel manoeuvring, along with a high susceptibility to tropical cyclones and fog, all of which compound the complexity of navigation and collision avoidance. The coexistence of mixed passenger and cargo traffic, coupled with high vessel density, renders this waterway a critical focus for navigation safety oversight.

| Figure 1 | Figure 2 | Figure 3 |
| --- | --- | --- |

Note:

Figure 1. Traffic heat map of the Zhanjiang Bay Bridge and Tiaoshun Bridge waters in 2022.

Figure 2. Traffic heat map of the Zhanjiang Bay Bridge and Tiaoshun Bridge waters in 2023.

Figure 3. Traffic heat map of the Zhanjiang Bay Bridge and Tiaoshun Bridge waters in 2024.

discussion

Application and processing Procedure of the IWRAP Model

This study employs the IWRAP MKII software to perform a risk assessment of the waters around the Zhanjiang Bay Bridge and Tiaoshun Bridge. By constructing a traffic flow model based on historical AIS data, the software quantifies the probability of ship collisions and groundings. Its core principle lies in abstracting vessel navigation routes into a network composed of "legs" and "nodes." Collision frequencies are then calculated using IALA- recommended mathematics, which integrate parameters such as vessel distribution within the fairway, speed, and course.

Data Import and reprocessing

The annual AIS data for the Zhanjiang Port area from 2022 to 2024 were first imported into the IWRAP MKII software. During the import process, key data fields—including time, longitude, latitude, ship type, speed, and course—were defined to ensure accurate data parsing. The software supports batch import and automatically generates a data distribution time chart, facilitating the verification of data integrity and time coverage. See Figures 4, 5, and 6 for details.

Figure 4 Selected Operation Area Diagram

Figure 5 Schematic Diagram of AIS Data Import

Figure 6 Data Distribution Time Chart

Traffic Flow Heat Map and Fairway Network Construction

After data import, the software automatically generates a traffic flow heat map, as shown in Figure 7, which intuitively presents the density of vessel trajectories. Based on the heat map, the researchers manually delineate or automatically extract the main fairways and waypoints to construct a traffic network model consisting of legs and nodes, as illustrated in Figure 8. This process requires adjusting the width and alignment of the legs according to the actual fairway conditions and vessel navigation practices.

Figure 7 Traffic Flow Heat Map

Figure 8 Schematic Diagram of Fairway and Node Definition

Model Parameter Configuration and Risk Type Selection

Following the construction of the traffic network, the analysis proceeds to the risk assessment module. During this phase, key parameters must be configured, including vessel type classification, fairway geometric parameters, and navigation rules (e.g., traffic separation schemes). The software enables separate calculations for various collision types, including overtaking, head-on, crossing, bend, and bridge pier collisions. Taking pier collision as an example, relevant parameters such as pier coordinates, span widths, and pier protection capacity must be defined at the bridge location, with the pier area selected as the analysis object. See Figures 9-13 for details.

Figure 9 Schematic Diagram of Bridge Editing Options

Figure 10 Schematic Diagram of Adding a Bridge

Figure 11 Schematic Diagram of Model Extraction

Figure 12 Model Histogram

Figure 13 Speed and Quantity Analysis Chart

Collision Probability Calculation and Result Output

Based on critical navigational parameters derived from AIS data—including vessel traffic flow intensity, speed distribution patterns, and ship type composition—the software systematically calculates collision probabilities for each designated leg and bridge pier area. These calculations integrate geometric probability models that account for the spatial distribution of vessel traffic and the geometric characteristics of the waterway. The resulting probability values are comprehensively presented in both tabular and graphical formats to facilitate intuitive interpretation and comparative analysis of risk levels across different locations, as illustrated in Figure 14.

Figure 14 Risk Analysis Results Diagram

Application of IWRAP Model and Traffic Analysis

This study utilized three years of AIS data (2022, 2023, and 2024) provided by the Zhanjiang Aids to Navigation Service to ensure the reliability of the assessment. Using the IWRAP MKII software, collision probabilities were calculated by analyzing key risk factors during vessel navigation.  Based on this analysis, the annual vessel traffic volume in the waters between the Zhanjiang Bay Bridge and Tiaoshun Bridge was found to increase from 3,415 transits in 2022 to 7,199 in 2023, before declining to 6,613 in 2024. Notably, vessel traffic in the Zhanjiang Bay Bridge area exhibited a continuous upward trend, rising from 2,972 transits in 2022 to 5,494 in 2024. Passenger ships, fishing vessels, and support vessels were identified as the predominant types in this area.

High-Risk Segments and Collision Probabilities

- Based on operational data analysis, the locations of high-risk segments remained largely consistent from 2022 to 2024, primarily situated where the main channel approaches the areas of the Zhanjiang Bay Bridge and Tiaoshun Bridge. These segments are characterized by relatively narrow waterways, which increase the likelihood of head-on and overtaking collisions. The vessel types most susceptible to collisions in these areas are primarily general cargo ships, support vessels, and fishing vessels (See Table 1).

- The results also indicate that the cumulative collision probability increased steadily over the study period, rising from 0.009855 per year in 2022 to 0.03155 per year in 2024. Among all collision types, head-on collisions exhibited the highest risk, recording the greatest probability in 2024.

Table 1 High-risk collision sections in the waters of the Gulf Bridge and Tiaoshun Bridge

| Serial Number | Navigation Segment (Coordinates), Location Map, Annual Ship Collision Probability in the Segment (times/year) | Vessel Types Prone to Collisions, Collision Probability Between These Vessel Types (times/year) |
| --- | --- | --- |
| 1 | LEG63 (110°25.417′E,21°12.090′N-110°25.992′E,21°13.190′N)/2022 year | General Cargo Ship and General Cargo Ship (0.00011) Support Vessel and General Cargo Ship (9.18×10-5) |
|  | 0.0011 |  |
| 2 | LEG137 (110°25.944′E,21°13.015′N-110°25.927′E,21°14.011′N)/2023 year | General Cargo Ship and Fishing Vessel (9.14×10-5) |
|  | 0.0011 |  |
| 3 | LEG186 (110°25.959′E,21°13.042′N-110°25.901′E,21°14.048′N)/2024 year | Support Vessel and Fishing Vessel (0.00029) |
|  | 0.0020 |  |

vessel Impact Probability to Bridge Piers

In parallel with the increasing vessel traffic flow, the probability of vessels impacting the piers of the Zhanjiang Bay Bridge has exhibited a year-on-year increase. The annual impact probabilities for each pier are presented in Table 2. The simulation results indicate that piers AREA-6188 and AREA-6190 have the highest and most rapidly increasing impact likelihood over the study period. The AREA-6188 pier is located in the middle of the navigational channel under the Zhanjiang Bay Bridge, where traffic flow is concentrated, while AREA-6190 is situated on the right side of the channel.

Table 2 Statistical Table of Probability of Vessel Collisions in the Gulf Bridge Area (units: times per year)

| Year | AREA-6187 | AREA-6188 | AREA-6189 | AREA-6190 | AREA-6191 | AREA-6192 |
| --- | --- | --- | --- | --- | --- | --- |
| 2022 | 0.003862 | 0.008297 | 0.006182 | 0.007313 | 0.003514 | 0.002985 |
| 2023 | 0.005045 | 0.008897 | 0.007114 | 0.009547 | 0.005086 | 0.004255 |
| 2024 | 0.007616 | 0.01396 | 0.008533 | 0.01128 | 0.006497 | 0.005760 |

risk mitigation Recommendations for Navigation Aids and VTS

Optimization of the layout of AtoNs

To address the increasing probability of vessel impacts, particularly concerning piers AREA-6188 and AREA-6190, optimizing the layout of AtoNs is recommended. The visibility around these high-risk piers should be enhanced through the installation prominent markers, establishing a clearly delineated warning zone to define vessel navigation boundaries. Furthermore, advanced AtoNs equipped with nighttime illumination and radar reflectors are recommended to improve visibility during adverse weather conditions such as heavy rain or fog. For high-risk navigation segments, conspicuous directional markers should be deployed at entrances and key nodes to guide vessels along the correct channel, thereby mitigating the risk of head-on and overtaking collisions in narrow waterways.

VTS Risk Prevention and Control

In light of the concentration of high-risk points near the bridge area where the main channel meets port berths, strengthening VTS risk prevention and control is essential. To enhance situational awareness, VTS centers should strengthen real-time monitoring by deploying advanced equipment-such as high-definition cameras and radar-in the identified high-risk segments and points. Furthermore, a vessel traffic warning mechanism should be established, enabling the VTS center to issue immediate alerts when the system detects dangerous behaviors, such as course deviations. Additionally, the VTS center should enhance communication with passing vessels to disseminate channel and weather information promptly, while proactively managing vessel traffic sequences to prevent congestion in high-risk areas.

Action requested of the Committee

The Committee is requested to:

- Note the findings on the application of IWRAP MKII for risk assessment in sea-crossing bridge waters.

Provide guidance on how to further deepen the application of IWRAP in the safety management of similar waterways.

Provide guidance on the operational methodology and analytical depth of IWRAP MKII in this case study.