Input paper:   ARM22-8.1.5

Date: 13 April 2026

Input paper for the following Committee(s): 		Purpose of paper:

(Select as appropriate)

☒ ARM	☐  ENG		☐ PAP			☒ Input

☐ ENAV	☐ VTS					☐ Information

Agenda item 			8.1

Technical domain/ Task number 2	…………………………………

Author(s)/Submitter(s)			Vincent Du Sablon – Canadian Coast Guard

# risk management approach to periodic review of aids to navigation systems

# Summary

This paper proposes for discussion and feedback a risk management approach for the periodic review of aids to navigation (AtoN) systems, currently under development by the Canadian Coast Guard (CCG) AtoN Program. This approach aims to replace the current CCG approach of a fixed five-year review cycle applicable to each system with a more flexible, risk-based process that distinguishes between critical and non-critical AtoN systems. Review cycles will be tailored based on system criticality and risk assessment. Criticality is assessed using likelihood and impact criteria, including socio-economic and navigational factors, resulting in a risk acceptability score that determines review frequency and resource prioritization. Ultimately, the approach will lead to CCG guidelines intended to improve resource allocation and planning, support management decisions in terms of implementation of AtoN-related projects, and ensure the program meets its mandate, standards and levels of service efficiently, while considering risk tolerance.

## Purpose of the document

The purpose of this paper is to seek input and feedback from the committee regarding a risk management approach for the periodic review of AtoN systems, currently being developed by the CCG AtoN Program.

## Related documents

ARM 22-5.0 Input Paper Related document - Risk Management Approach to Periodic AtoN System Review.pdf

# Background

The CCG AtoN Program manages more than 17,000 AtoN, providing services across three ocean coasts and Canadian inland waters, including the St. Lawrence and Greak Lakes waterways. In compliance with IALA Guideline 1004 (2017) which requires competent authorities to conduct periodic reviews of established levels of service, an internal CCG directive states each AtoN systems must undergo a complete review process every five years. This target has long been proved unpracticable and unrealistic, and would require a substantial increase of resources and personnel to achieve. As part of the revision of this directive, the CCG AtoN Program proposes a new approach to this periodic review target, based on risk management in relation to the program’s ability to meet its mandate and standards. The approach is still under development and will result in internal guidelines, the draft of which is summarized and discussed here.

These guidelines detail a process intended for the AtoN program management, allowing for some degree of risk tolerance in less vulnerable systems, where a longer and more flexible review cycle can apply. Such process also seeks to support decision making in planning and prioritizing AtoN implementation and related projects, resources, efforts and funding.

# Discussions

The revision of CCG AtoN Program’s directive on review of AtoN systems called for replacing of the fixed five-years review cycle with a more flexible, risk-based approach. The updated directive establishes as a general principle the ranking of AtoN systems based on their criticality to the Program’s ability to meet its mandate and standards related to AtoN provision. The application of these principles results in a risk acceptability assessment process described in the guidelines currently under development, which outlines a five-step approach.

## General Principles

### Critical and non-critical aton systems

The core principle of this approach is the distinction between critical and non-critical AtoN systems:

Critical systems are subject to periodic review at least once every five years.

Non-critical systems are subject to periodic review at a frequency determined by the regional AtoN program offices.

Such distinction is similar to that found in the USCG Aids to Navigation Manual – Administration (USCG, 2015). In the proposed approach, when the period of time determined for revision of an AtoN system comes to its term, the system will be included in the annual workplan for a complete review if changes have occurred since the last complete review that are likely to affect the relevance, efficiency, or effectiveness of the AtoN provided. Otherwise, the periodicity determined for complete system review will be reset or reassessed, as required.

System criticality is understood from the perspective of the program management, not from the mariners and users of the AtoN: it allows the management some degree of risk tolerance in systems where they are least vulnerable to not meeting the mandate or standards. The AtoN program can thus focus efforts, resources and funding on AtoN systems presenting the greatest threat to proper fulfilment of its mandate.

To apply this principle, guidelines detail a structured approach to differentiate between critical and non-critical systems, and to determine flexible review cycle targets accordingly.

### likelihood and impacts

This approach is based in part on IALA guidelines 1098 and 1138 (2022a and 2022b). Risk is defined as a combination of the likelihood an event occurring and the severity of its impacts.

In the context of AtoN management, likelihood is defined as the possibility that a given AtoN system becomes ineffective in mitigating threats or inadequate to its user’s needs. The impacts of such a scenario are considered from two perspectives. Socio-economic impacts consider consequences on users and marine activities supported by the AtoN. Navigational impacts consider the effect on vessel safety, the environment, and human life. This correspond to the two main aspects of the AtoN Program’s mandate: facilitating safe, economical and efficient movement of vessels. A systems can be assessed socio-economically critical, navigationally critical, or both. A risk acceptability matrix and its related score scale (see tables 4 and 5 below) are used to rate the criticality.

## Risk acceptability process

The proposed approach consists of five steps for each AtoN system, from the likelihood and impact rating using defined criteria, to the determination of their risk acceptability score and selection of a review cycle target.

### Step 1: Rating likelihood

The first step is to rate the likelihood that a given AtoN system becomes ineffective in mitigating threats for which it was designed for, or inadequate to the users’ needs. The criteria and rating scale (low, medium, high) to evaluate likelihood are presented in Table 1. For each of the five criteria, a short statement must be produced to explain the reasoning behind its rating. The overall rating of the likelihood will be determined, using the same scale and based on the individual rating of these criteria and on managerial judgment. CCG Regional AtoN offices may assign any weighting factor to the criteria, as appropriate, to reflect regional or area-specific considerations. This final likelihood rating must be supported with a short statement explaining the reasoning.

Criteria for likelihood rating

| Rating | Recency of LOS reviews | Navigational conditions | Information available | AtoN asset conditions | Overall system reliability |
| --- | --- | --- | --- | --- | --- |
| 1 Low | AtoN system fully reviewed within the last 3 years, or ad hoc reviews covering 50% or more of its aids completed within the last year. | Navigational conditions, degree of risk, or volume, mix and type of traffic remain unchanged, and are unlikely to change. | Regular communication with main users, including reporting of navigational hazard, and/or reliable and timeliness data or information available from knowledgeable users, supporting a continuous watch on the area. | Asset condition assessment program reveal that fixed aids components are in a good or satisfactory state, considering their life cycle. | Overall AtoN reliability of the system meets the availability target. |
| 2 Medium | AtoN system not fully reviewed within the last 3 years, but ad hoc reviews completed within this period, covering between 25% and 50% of the aids. No approved recommendations are pending for implementation. | Navigational conditions, degree of risk, or volume, mix and type of traffic have changed, or are likely to change in a foreseeable future. | Little or no regular communication with main users, or data available not up-to-date, fully accurate or easily available, making it difficult to keep an accurate portrait of the area. | Asset condition assessment program reveal that fixed aids components are in fair state, considering their life cycle. | Overall AtoN reliability of the system meets the target, but one or a few individual AtoN are below their availability targets, considering their aid’s category. |
| 3 High | AtoN system not reviewed for an extended period, or implementation of the latest approved recommendations are pending for more than 3 years. | Navigational conditions, degree of risk, or volume, mix and type of traffic have changed, or will likely change significantly. | No communication with main users or lack of data available, resulting in insufficient knowledge of the area. | Asset condition assessment program reveal that fixed aids components are in fair or poor state, considering their life cycle. | Overall AtoN reliability of the system does not meet the target, or many individual AtoN are below their availability targets, considering their aid’s category. |

### Step 2: Rating socio-economic impacts

The consequences of an AtoN system being inadequate to users’ needs must next be assessed, starting with socio-economics. Impacts are to be evaluated in a similar manner as for likelihood, based on defined criteria and also using a three-level scale (minor, moderate, severe), with a short statement recorded to explain the reasoning behind the individual criteria ratings and the overall socio-economic and navigational impacts ratings.

In a given area, socio-economic impacts of an AtoN system’s efficiency are defined in relation to the importance of marine activities on both society and the economy. An AtoN system not able to maintain economical and efficient movement of vessels can impact at different degrees commercial or other socio-economic activities essential to the country, to a specific region, area, community, industry, or site. For example, it can jeopardize essential marine resupply of an isolated community, or impact the efficiency of trade or supply chains to varying degrees. This means that consideration must be given, on the one hand, to the users of the serviced waterways, as well as the individuals, groups, or communities who benefit from marine activities in terms of living conditions, standards of living and related factors, such as access to essential goods or services, commuting in the area, employment, etc. Must also be considered, on the other hand, revenues or incomes, exchange of goods and services, industry competitiveness, and alike, generated or supported by marine activities.  The criteria to rate socio-economic impacts are presented in Table 2.

Inputs for assessing socio-economic impacts will rely to a large extent on regional knowledge about the users (vessels categories, types of vessels) and the local environment (communities, industries, marine facilities, etc.) around the waterways serviced by AtoN. It can be based on experience, managerial judgment, and collaboration and discussions with partners or with users.

Criteria for socio-economic impacts rating

| Rating | Users and purpose of their use | Socio-economic value and dependency |
| --- | --- | --- |
| 1 Minor | AtoN system essentially serves users for recreational purposes and boaters. | Area serviced by the AtoN system has a mix of economic activities, and marine activities have minor local impacts on incomes, resupplying or supply chains, or commuting in the area. |
| 2 Moderate | AtoN system serves more than one vessel category, or is essentially used by small commercial vessels, or for socio-economic reasons such as employment or to maintain the necessities of life. | Area serviced by the AtoN system relies mainly on marine activities considered essential for the local or regional economy, or marine activities have a moderate impact on incomes, resupplying or supply chains, or commuting in the area. |
| 3 Severe | AtoN system serves more than one vessel category, or is essentially used by large commercial vessels, or for socio-economic reasons in an isolated community. | Area serviced by the AtoN system relies mainly on marine activities considered essential for the regional or national economy, or marine activities have a major impact on incomes, resupplying or supply chains, or commuting in the area. |

### Step 3: Rating navigational impacts

The navigational impacts of an AtoN system’s efficiency are defined in relation to the threats it mitigates and its contribution to marine safety, along with other existing local means. This means not only the threats and the sensitivity of the area must be considered, but also the impact of AtoN systems on other parties that might be required to respond in the event of a marine incident. If an AtoN system fails to ensure the safe movement of vessels within a specific area, it can adversely affect transiting vessels, maritime or shore-based infrastructures, environment, or human life to varying degrees.  The criteria for rating navigational impacts are presented in Table 3.

Inputs for assessing navigational impacts rely to a large extent on the regional AtoN offices knowledge of threats to marine navigation through previous reviews conducted by AtoN program officers and should also consider discussions with other CCG programs (Marine environmental and hazards response, Search and rescue) involved in response in case of a marine incident.

Criteria for navigational impacts rating

| Rating | Composite threats | Hazardous cargos transiting | Other local means | Expenses in case of incident |
| --- | --- | --- | --- | --- |
| 1 Minor | 1 to 4 composite threats present, or AtoN in place mitigate threats rated no more than significant. Clear danger to the vessel, but no danger to life or environment. | No significant hazardous material or dangerous cargo transiting in the area, limited short-term damage to the environment. | Other local means to mitigate are present and most users have good local knowledge of the area. | Little or minor expenses and resources required in the event of a necessary incident response by the CCG or other partners. |
| 2 Moderate | 5 or more composite threats present, or at least 25% of AtoN in place mitigate threats rated highly significant or overriding importance. At least one clear danger to life or environment due to navigational conditions. | Hazardous materials and dangerous cargo transiting occasionally in the area. Short-term damage to the environment in a small area, or long-term irreversible damage to the environment in a limited area. | Lack of other means than AtoN to mitigate threats, but compensated with local knowledge of most users. | Moderate to significant expenses and resources required in the event of a necessary incident response by the CCG or other partners. |
| 3 Severe | 5 or more composite threats present, and at least 50% of AtoN in place mitigate threats rated highly significant or overriding importance. Multiple clear dangers to life or environment due to navigational conditions. | Hazardous materials and dangerous cargo transiting frequently in the area. Long-term to irreversible damage to the environment to a large area or in a remote area. | Lack of other means or of local knowledge to mitigate threats. | Significant to major expenses and resources required in the event of a necessary response by the CCG or other partners. |

### Step 4: Determining risk acceptability score

The next step is to combine the overall likelihood rating (low, medium or high) with the socio-economic impact and navigational ratings (minor, moderate or severe). This will determine the final risk acceptability score for the given AtoN system, thus indicating to what extent it should be considered as critical or not. This is done by using the risk acceptability matrix (Table 4), for each type of impacts. The product of likelihood and impact results in a three-level risk scale: acceptable risk, acceptable risk with caution, and unacceptable risk.

Risk acceptability matrix

|  | 1- Low likelihood | 2- Medium likelihood | 3- High likelihood |
| --- | --- | --- | --- |
| 1- Minor impact | 1 | 2 | 3 |
| 2- Moderate impact | 2 | 4 | 6 |
| 3- Severe impact | 3 | 6 | 9 |

A critical system is a system having an unacceptable level of risk from a management perspective. This means that the AtoN Program considers the risk of not being able to fulfil its mandate adequately as being too high, and not tolerable. An AtoN system can be assessed as socio-economically critical, navigationally critical, or both. The meaning of each risk acceptability level for the management of AtoN systems’ complete review cycle and related efforts is summarized in Table 5 and described below. This scale is intended to guide regional CCG AtoN program in determining the two main measures that could reduce the likelihood or limit the impact of an AtoN system becoming ineffective or inadequate:

Determine a functional complete review cycle, or add a complete review of a system in the annual planning.

Adjust or request the proper level of resources and efforts required to maintain a system adequately or prioritize implementation of recommendations in that system.

Risk acceptability score scale

| Score | Level of risk |
| --- | --- |
| 1-2 | Acceptable: AtoN system to be considered as non-critical, subject to periodical reviews, the frequency of which is regionally determined, and to low cost in terms of time, money and effort, when required. |
| 3-4 | Acceptable with caution: AtoN system to be considered as non-critical, but subject to be reviewed once every 5 years or at a longer frequency regionally determined, and to additional effort and funding when required. |
| 6-9 | Unacceptable: AtoN system to be considered as critical, subject to be reviewed at least once every 5 years and to consistent or substantial effort and funding when required. |

An acceptable level of risk means that the AtoN system is considered as non-critical, and that the AtoN Program considers risk related to fulfilment of its mandate, levels of service or other AtoN related standards and requirements, to be sufficiently low. Control measures such as review cycle target regionally determined and low cost in terms of time, money and effort when required are deemed sufficient to maintain adequate services. Additional efforts may still be implemented to further reduce the risk for the program or to meet the essential minimum requirements, or to monitor the system to detect any changes that may indicate an increased risk.

An acceptable level of risk with caution means that the AtoN system can be considered as non-critical, but that the AtoN Program tolerates risks related to its mandate, levels of service, or other AtoN related standards and requirements, as long as they are maintained as low as reasonably practicable. This tolerance to risk must be controlled with review cycle targets of once every 5 years, or at a longer frequency deemed reasonable by the region. Additional effort and funding must be prioritized for such systems when required. Such efforts should not exclude capital project investments to meet the essential minimum requirements in order to maintain the risk as tolerable, or to reduce it to acceptable.

An unacceptable level of risk means that the AtoN system must be considered as critical and that it requires extra attention and resources for the risk to be reduced. In other words, it is a system for which the program is at risk not to fulfill its mandate, or not to meet its levels of service or other AtoN related standards and requirements. The program’s vulnerability to risk must be controlled with a shorter review cycle target of 5 years or less. Consistent or substantial effort and funding must be prioritized for such systems, when required, as long as it remains technically practicable and cost-effective. Such efforts to meet the essential minimum requirements could include, but are not limited to, prioritizing of implementations or major AtoN improvements, or, as required, including implementation projects in the national prioritization plan for capital project investments in order to reduce the risk to a tolerable or acceptable level.

### Step 5: Selecting a review cycle target and final statement

Once the risk acceptability level of an AtoN system has been decided, a short final statement must be produced to explain its reasoning. It must be stated if the system is critical or not from a socio-economic or a navigational perspective, or both. The review cycle target of the AtoN system as determined based on its assessed criticality must be explicitly stated and followed. Table 6 provides guidance for deciding the review cycle target. It combines the risk acceptability score scale obtained for both socio-economic and navigational perspectives.

Guidance matrix for complete review cycle targets

|  | Acceptable | Acceptable with caution | Unacceptable |
| --- | --- | --- | --- |
| Acceptable | Regionally determined | ≥ 5 years | ≤ 5 years |
| Acceptable with caution | ≥ 5 years | 5 years | ≤ 5 years |
| Unacceptable | ≤ 5 years | ≤ 5 years | < 5 years |

## Final considerations: a dynamic process

As a planning, prioritization and monitoring method, the results of the proposed risk acceptability approach and the rating of an AtoN system, including the complete review cycle, must not be considered as permanent, but rather a snapshot of an AtoN system at a given point in time. It is a dynamic process intended to consider the evolution of waterways, their conditions and users, as well as of the different assets and their components providing AtoN services in regards to the ability of the AtoN Program to meet its standards and levels of service.

If the program becomes aware, through a review or other means, of a new situation or a change in an AtoN system, its risk acceptability rating should be adapted to reflect the new requirements in terms of complete review cycle, effort and funding.

The risk acceptability of an AtoN system should be reassessed after any complete or ad hoc review has been   conducted based on its conclusions and recommendations. For example:

If a review conducted on an AtoN system that has been given a rating of “acceptable,” or “acceptable with caution,” reveals significant gaps in service provision or major improvement required to the service, the rating could be raised to the appropriate level until the approved recommendations are implemented.

If a review conducted on an AtoN system that has been given a rating of “unacceptable,” or “acceptable with caution,” concludes and recommends status quo or minor improvement to most of its AtoN, the rating could be reduced to the appropriate level.

Once implemented, these guidelines will help the CCG AtoN Program to plan and prioritize level of service reviews on an annual basis more effectively. They will also serve as a consistent method for the ranking and prioritizing of individual AtoN projects — particularly capital projects — based on each system’s risk acceptability rating within a national prioritization framework. In doing so, the guidelines are expected to become a key tool for expressing the Program’s risk tolerance when engaging with partners and enablers responsible for implementing its AtoN recommendations.

# References

IALA (2022a), Guideline 1098, Risk Management, Edition 4.0.

IALA (2022b), Guideline 1138, The Use of Simplified IALA Risk Assessment Method (SIRA), Edition 2.0.

IALA (2017), Guideline 1004, Level of Service, Edition 3.1.

USCG (2015), Aids to Navigation Manual – Administration, February 2015 (March 2005).

# Action requested of the Committee

The Committee is requested to:

- Note the work and take action as deemed appropriate.

- Provide comments or guidance on the proposed risk management approach, assessment criteria or any other factors to consider.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain