Input paper:   ARM22-8.1.6

Date: 14 April 2026

Input paper for the following Committee(s): 		Purpose of paper:

(Select as appropriate)

☒ ARM	☐  ENG		☐ PAP			☐ Input

☐ ENAV	☐ VTS					☒ Information

Agenda item 			8.1

Technical domain/ Task number 2	1.4.7

Author(s)/Submitter(s)			Y. Koldenhof (MARIN), E. Bolt (RWS)

# Modernisation Safety Assessment model SAMSON

# Summary

This document informs the committee about recent developments and future plans for the Safety Assessment Model SAMSON and seeks guidance on the requirements for including SAMSON within the IALA Risk Management Toolbox. SAMSON is a generic maritime risk assessment model originally developed for the North Sea but applicable worldwide. It estimates the probability, location, and consequences of maritime accidents using historical casualty data and has been extensively applied and enhanced by MARIN together with RWS over several decades.

To ensure long-term sustainability and relevance in a rapidly evolving maritime environment—particularly due to offshore wind development—a modernization program was launched in 2024. Key changes include redeveloping the software from Fortran to C++, improving traffic modelling by better representing route-bound and non-route-bound vessel behaviour, and updating model assumptions to enable validation against current operational and policy needs. SAMSON 2.0 will combine an open-source core with a licensed, supported version.

In parallel, increased cooperation with IWRAP is foreseen, following recent comparative studies that showed differences in absolute risk values but consistent conclusions regarding spatial planning choices. Strengthened collaboration is expected to improve mutual understanding of model assumptions and highlight complementary strengths.

## Purpose of the document

The purpose of the input paper is to inform the committee on the developments and future plans on the Safety Assessment Model SAMSON and to ask the committee what is needed to consider SAMSON as part of the IALA Risk management toolbox.

# Background

## SAMSON

SAMSON stands for: Safety Assessment model for Shipping and Offshore on the North Sea. With the model, various risk assessment calculations can be performed regarding maritime safety. Although the name suggests SAMSON is only applicable for the North Sea, it is a generic model can be applied to any defined geographic location. The model was developed to determine the probabilities, locations and consequences of various maritime accidents, taking into consideration various mitigation measures that could be used to reduce the likelihood of a maritime accident (e.g.: pilotage).

At the core of SAMSON [GP2.1] lies a geometric modelling approach that determines, based on a representation of a given shipping traffic system, the possible dangerous situations. These include encounters between moving vessels, vessels deviating from their intended course due to navigational errors, or vessels becoming uncontrollably adrift as a result of technical failure. By multiplying these “collision candidates” or other possible dangerous situations with the probabilities derived from historical incident databases (causation factors or basic accident rates), the model yields the expected number of accidents per time period.

The parameters of the accident models are derived from the worldwide casualty data. The SAMSON model was originally developed over 40 years ago and since that time it has been extended, validated and improved by MARIN in various studies performed for Rijkswaterstaat, the EU and Transport Canada.

A detailed system diagram of the current original version of SAMSON model is presented in Figure A-1  highlighting the numerous parameters, systems, and impacts that can be considered with SAMSON.  More information on the current version (1.0) SAMSON can be found in ANNEX A and on https://www.iala-aism.org/wiki/iwrap/index.php/SAMSON

## Modernisation of the model

SAMSON has a development history that dates back to the late 1970s. In the years that followed, the initially theoretical model was gradually implemented into software code, with the aim of enabling the calculation of —among other things —collision frequencies between vessels, as well as between vessels and fixed offshore objects.

As the model has evolved over several decades, both software and the associated workflows have developed incrementally, with a focus/emphasis on extending functionality while maintaining the model in line with its original design. Over time, however, the architecture of the model has become outdated. In addition, not all sub-models have been applied to the same extent, resulting in uneven maintenance. Several assumptions within the model also lack thorough validation and, in light of significant developments in offshore infrastructure—such as the expansion of offshore wind energy areas—have become due for reassessment.

For these reasons, a modernization program for SAMSON was initiated in 2024. The objective of this program is to bring the software up to contemporary standards, accommodate changes in data usage, and improve or adapt the computational models to enable validation and alignment with current and future operational contexts and policy questions.

# Discussion

Some of the major developments of the past years regarding the SAMSON-model are described in this chapter, this include the changing of the code and some adjustment to the mathematical models as well as the fact that the model will be provided as “open source”. Finally a suggestion is made for a further cooperation with IWRAP.

## Code language

The software in which the models within the SAMSON framework are implemented is currently being modernized. This modernization effort covers both the source code and the overall software architecture. The original SAMSON model was developed in Fortran. Although Fortran remains well suited for large-scale numerical computations, sufficient in-house expertise is no longer available at MARIN to adequately maintain and further modernize the software. To enhance the long-term sustainability and maintainability of the code and its architecture, adopting a more widely used programming language is a logical step. It was therefore decided to redevelop the SAMSON software in C++. While other programming languages could offer comparable performance, MARIN has extensive positive experience with C++ for applications involving large volumes of data.

## Modelling route-bound and non-route-bound traffic

The development of offshore wind has only accelerated in recent years, meaning that the existing models have not yet been adapted accordingly. As a result, modelling traffic in and around wind farms is currently possible only with a high degree of uncertainty, because the behaviour of this traffic does not align with the assumptions embedded in the current model. Within the existing model the actual behaviour of the vessel is not fully taken into account when defining the abstract representation of the traffic as input for the model.

Within the current version of SAMSON (version 1.0) the distinction between route-bound and non-route-bound traffic was made solely on the basis of vessel type, e.g. container, passenger, fishing or work boat. In the revised modelling approach within SAMSON 2.0, differences in behaviour are also taken into account independently of vessel type. “Route-bound” behaviour (e.g. sailing in transit ) is translated into a route network, comparable to the approach used within IWRAP. “Non-route-bound” behaviour (e.g. active fishing, dredging, working inside windfarms)  is translated into traffic intensity per grid cell. Within SAMSON 2.0, the observed distribution of sailing directions within each grid cell is also taken into account. In addition, different speed profiles can be incorporated for each of the vessel types (type and behaviour).

More detailed modelling of the vessels with “non-route-bound” behaviour lead to an overall improved modelling of vessel traffic behaviour, particularly in a maritime environment that is changing rapidly due to the expansion of offshore wind farms and other offshore activities.

## Adjustment in ship-object model

Some of the basic assumptions within the model for ship–object collisions have been adjusted to better prepare for changing demands. In the updated model, objects can be of arbitrary shape; we are no longer restricted to lines or rectangles. This makes it possible to easily provide areas (e.g. wind farm of solar constructions) as input to the model. In addition, the model can account for the fact that one object can be blocked by another object, meaning that shielding effects are taken into account. This is particularly relevant when modelling a wind farm or when automatically including shallow areas in the calculations. Furthermore, a height can now be assigned to an object, relevant when modelling detailed bridge collisions.

Finally, the drift model has been updated so that drift direction is now a function of tide, current and wind, where it was previously assumed that the drift direction was purely dependent on wind direction.

## Open source

The developments on the model will be completed around the summer of 2026. In addition to model documentation, that will be freely available, also the core of the model will be released as open-source software. The details concerning publication, governance and maintenance, still needs to be finalized. The code will be made available under a European Union Public Licence  (EUPL) version 1.2. The motivation to  for this approach is that open publication of the core code can support transparency, facilitate external review and enable broader contribution to future model development.

This approach also offers mutual benefits for both IALA and SAMSON. For IALA, it could provide access to an additional risk assessment approach and support wider exchange of knowledge on model assumptions and applications. For SAMSON, engagement through IALA could increase international visibility, encourage expert feedback and external review, and support further development through broader practical use.

Alongside the open-source code, compiled version of the program, including documentation and training will be made available under a paid license with a yearly fee.

## Cooperation with IWRAP

In 2016, a study was carried out  to for IALA to compare  IWRAP and SAMSON. Since then, both models have  undergone further development . In 2023 and 2024, an extensive Formal Safety Assessment (FSA) study was performed for a major international traffic route on the North Sea. As a part of that assessment, the impact of offshore wind development was investigated, including a quantitative risk analysis using both IWRAP and SAMSON. IWRAP was used by ABL on behalf  of the German authorities and MARIN used SAMSON at the request of the Dutch authorities. Both consultants used the same input data. The main conclusion was that the absolute values of the expected accident frequencies differed, however the conclusions that were drawn concerning the impact of different geographical choices for the wind farm were comparable.

These differences call for a better understanding of the different model choices and the consequences of these assumptions. This can also help to better understand the strength of both models and how and where they can be complementary.

Results of the quantitative risk assessment using SAMSON and IWRAP

# Action requested of the Committee

The Committee is requested to:

- Take note of the information provided regarding the development and modernisation of the SAMSON model.

- Take note of the information provided regarding making the computational core of the SAMSON model available as open source.

- Consider the inclusion of the new open source SAMSON in the IALA Risk Management Toolbox.

- Consider advising on a comparison study IWRAP and SAMSON to investigate the strength of both models and how they can reinforce each other.

BACKGROUND SAMSON

Introduction

Maritime traffic and its behaviour is essential for a successful nautical risk assessment. For many years, such knowledge has been built up and improved. The knowledge has been acquired within Dutch national projects for the Maritime Safety Agency (DGLM with former names DGTL, DGG and DGSM) and the North Sea Directorate (Dienst Noordzee/Dienst Zee en Delta), both part of the Ministry of Infrastructure and Water Management, and within the European projects, COST 301, EURET, SAFESHIP, EMBARC, MarNIS, BE-AWARE and OpenRisk, for the European Commission. Based on this knowledge the SAMSON-model has been developed. The core of this model is a very detailed statistical traffic image surrounded by a number of accident models. The models predict the frequency of different accident types e.g. collisions, contacts, foundering, grounding, ramming and drifting against offshore platforms, wind farms etc. The models predict the frequency of occurrence combined with geographical position. This is essential for answering questions dealing with contingency planning.

Model use in policy evaluation

SAMSON is used as a decision support system for the assessment of the effect of policy measures concerning:

- Re-routeing of traffic or part of the traffic to avoid sensitive areas. Is the new situation safer and what are the ship costs?

- What is the effect of training on the accident rate and what does that mean in terms of benefits?

- What is the effect of changes in the construction and/or loading regulations of vessel on the frequency of spills as consequence of a accident?

- What is the effect of the introduction of a “clearways system” in the North Sea and other waters of interest?

Examples of application of the model

- Presenting traffic images in sea areas.

- Assessment of the spillage of oil and chemicals in case of collisions, grounding and foundering on the basis of the expected hull damage.

- Assessment of the pollution of the environment due to normal operations.

- Collision risk assessment for offshore platforms. How safe is the intended position? What is the probability that the platform will be collided by a ship and what will be the distribution of the ship sizes in that case? What are the possible risk control measures and what does it yield?

- Frequency of hitting pipelines by containers, anchors and sinking ships.

- Assessment of the optimum location and capacity for the salvage tug and oil recovery vessels for a required safety level.

- Assessment of the optimum location and capacity of units for the Search and Rescues capacity plan.

- Collision Risk Assessment for offshore wind farms.

Accident models in SAMSON

The frequency in which a certain type of (collision) accident occurs at sea is modelled by multiplying a certain exposure measure with a bare accident rate.

- An exposure measure can be explained as a certain elementary “traffic situation” which is representative for a certain type of accident.

- The basic accident rate is the probability that the situation leads to a real collision. The accident rate is determined from accident data from the different accident databases.

The main thought behind all models (except the ship/ship-model) is that accidents can be divided in accidents at low speed and accident occurring at high speed. An accident occurring at low speed is caused by an engine failure, a failure in the propulsion engine or in the steering equipment. Because of this engine failure the ship slowly becomes uncontrollable as it loses speed, the combined effect of wind, waves and current may cause it to drift. An accident occurring at high speed is caused by a human error or navigational error.

The different type of accidents defined in SAMSON with their cause and exposure measure can be found in Table A-1.

- Table A-1	Overview of the different types of accidents distinguished in SAMSON

| Type | Subtype | Cause | Exposure measure |
| --- | --- | --- | --- |
| Ship/Ship collision | Sailing ships | Human error | Encounter |
|  | Ramming ship/anchored ship anchor area | Human error | Anchored ship ramming opportunity |
|  | Ramming ship/anchored ship anchor area | Engine failure | Danger miles |
| Contacts | Objects with fixed position | Engine failure | Danger miles |
|  | Objects with fixed position | Navigational error | Ramming opportunity |
| Sinking |  |  | Ship miles |
| Fire and Explosion |  |  | Ship miles |
| Stranding |  | Engine failure | Danger miles |
|  |  | Navigational error | Stranding opportunity |
| Incident Hull/Machinery |  |  | Ship Miles |

- Figure A-1 System diagram SAMSON

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain

Footer will automatically populate