# PERFORMANCE REQUIREMENTS FOR  
DIFFERENTIAL GLOBAL NAVIGATION SATELLITE SERVICE (DGNSS) IN VTS

Secretarially reformat to reflect existing Radar requirement Annex to V-128

### INTRODUCTION

In 1997 the IMO Maritime Safety Committee adopted Regulations for Vessel Traffic Services (VTS) that have since been included in SOLAS Chapter V (Safety of Navigation) as Regulation 8-2.This Regulation specifies the responsibilities of contracting governments to arrange for the establishment of VTS in certain vulnerable areas under their control.

### BACKGROUND

The positioning, navigation, and timing (PNT) systems in a VTS, also referred to as differential global navigation satellite service (DGNSS) systems, include the U.S. Global Positioning System (GPS), GLONASS (Russian Federation Global Navigation Satellite System), Galileo (future European navigation satellite system),  and their augmentations (e.g., differential GPS, wide area augmentation system, etc.).  Typical DGNSS variables provided by these systems include latitude, longitude, course, and speed.   Additionally, these systems can provide accurate timing references for other applications.  DGNSS signals may be obtained through navigation receivers designed for this purpose.  DGNSS is an input to other systems, most notably Automatic Identification Systems (AIS).  At the VTS centre, graphical and/or numeric information (ship positions, course and speed vectors, etc.) are presented for use by the operators.

### OBJECTIVES OF DGNSS EQUIPMENT

DGNSS information may be integrated into VTS applications to provide the operator a real-time position, navigation, and timing information in the VTS area of responsibility.  Information collected from this equipment can be provided to ships to assist in assessing the waterway conditions.  DGNSS information can be referenced to a common geographical reference point (e.g., WGS ’84).

### RELATED REQUIREMENTS INVOLVED

In this section all additional applicable IMO requirements for the respective navigational equipment should be listed. This may be:

Relevant SOLAS requirements; SOLAS Chapter V (Safety of Navigation) - Regulation 12

IMO Assembly Resolution A.694(17) - General Requirements for Shipborne Radio Equipment forming Part of the Global Maritime Distress and Safety System (GMDSS) and for Electronic Navigational Aids;

IMO Assembly Resolution A.688(17) - Code on Alarms and Indicators;

Resolution MSC.39(63) - Adoption of amendments to the Code on Alarms and Indicators.

IEC 61108-1  Edition 2  Maritime navigation and radiocommunication equipment and systems –

Global navigation satellite systems (GNSS) Part 1: Global positioning system (GPS) – Receiver equipment – Performance standards, methods of testing and required test results

NMEA 0183 – Standard for Marine Electronic Devices (IEC equivalent)

RTCM SC-104, Recommended Standards for Differential GPS

### DEFINITIONS

As defined by World Geodetic Survey (WGS).

### FUNCTIONAL REQUIREMENTS

6.1	DGNSS equipment and availability

### DGNSS receivers should be installed and located according to recommendations from a VTS authority in consultation with radio engineers.  Differential broadcast systems (e.g., DGPS LF/MF) should be sited with due regard for antenna height and placement, as well as appropriate frequency allocations.

DGNSS measurements may include:

Latitude
Longitude

Course
Speed

6.2	Reliability, accuracy, range, and resolution

The reliability, accuracy, range, resolution, and units of the measurements should satisfy the minimum requirements as determined by IEC 61108-1 Ed. 2.

6.3	Malfunctions, warnings, alarms and indications

In this section reference should be made to relevant requirements of resolution A.686(17) and resolution MSC.39(63). Depending on the individual type of the navigational equipment requirements in addition to resolutions A.686(17) and MSC.39(63) should be determined if necessary.

6.4	Software requirements

Not applicable

### OPERATIONAL REQUIREMENTS

7.1 Ergonomics

Not applicable

7.2	Operational controls

Not applicable.

7.3	Presentation of information

The results of the measurements must be transmitted in WMO standard units and displayed in user-selectable format.

The measurements must be available to the traffic operators at all times through an integrated display or separate instruments.  Data may be presented numerically and/or graphically.  A log of the latest 24 hour measurements should be available to the operators either numerically or graphically.

7.4	Software requirements

Not applicable

### DESIGN AND INSTALLATION

8.1	Durability and resistance to environmental conditions

Electronics installed externally should be in an environmental enclosure .  As regards environmental conditions, the IEC requirements should be applied as far as relevant.

8.2 	Interference

These sensors are industrial equipment and therefore comply with applicable international standards and regulations.  Refer to the IEC requirements (IEC 60945 and 61108-1 Ed. 2) as far as relevant.

8.3	Power supply

IEC requirements should be applied as far as relevant.  In remote locations, due to the low power consumption of DGNSS receivers, authorities should consider use of alternative power (e.g., solar panels, wind vanes, etc.) in lieu of generators when commercial power is not available.  When differential augmentations are installed (e.g., DGPS LF/MF transmitters), the higher power consumption may preclude this approach.

8.4	 Installation

Requirements concerning the installation of receivers and transmitters, wiring and the arrangement of the equipment providing DGNSS information for the VTC should be determined.  Operational requirements will determine where remote sites are to be located and how many are required.  Sites should be selected based upon optimizing data relevant to the VTS.  Other considerations include availability of power, protection against vandalism, housing, and collocation with existing VTS, AtoN, or other suitable infrastructure

'IEC 529 "Degrees of protection provided by enclosures (IP Code)"

IEC 721-3-6 "Classification of environmental conditions, Part 3: Classification of groups of environmental parameters and their severities; Ship environment"

IEC 60945 "Maritime navigation and radiocommunication equipment and systems - General requirements, methods of testing and required test results"

8.5 Maintenance

Requirements in addition to IMO Assembly Resolution A.694(17) concerning maintenance should be determined if necessary. Citing considerations for sensors should include the maintenance, repair, and accessibility requirements.

### INTERFACING

The data to be interfaced for the DGNSS service are described in Section 6.

For the interfacing of DGNSS services to VTS equipment, several different standards are in use.  Among those standards, the Standard for Marine Electronic Devices, NMEA 0183, has been applied for these applications.

For the interface between a VTS and its users, DGNSS data should follow standardized data exchange formats, e.g., XML.  (See Guidelines on the Exchange of Real Time Data between VTS Systems.)  In addition, a time stamp and source should be provided.

### BACK-UP AND FALL-BACK ARRANGEMENTS

Depending on the individual type of the equipment, requirements concerning back-up and fall-back arrangements should be determined if necessary.

### SAFETY PRECAUTIONS

Depending on the individual type of the equipment, requirements in addition to resolution A.694(17) concerning safety precautions should be determined if necessary.

### MARKING AND IDENTIFICATION

Depending on the individual type of the equipment, requirements in addition to resolution A.694(17) should be determined if necessary, e.g., for the identification of software.

### DOCUMENTATION

In this section the contents of the documentation (e.g. Operator's Manual, Installation Manual, Service Manual) necessary for the operation of the equipment should be determined. The documentation should contain the following items:

General:

manufacturer, type of equipment and the serial number; system description;

specification of the equipment; supplementary equipment;

Installation:

physical characteristics of the equipment;

requirements concerning installation (e.g. supply voltage and frequency, earthing, power consumption, heat dissipation);

safety information;

Operation:

operational controls, functions and presentation of information; startup and shutdown procedures;

tests, failures, calibration procedures and alarms; interfaces and software description;

of updating procedures; safety information;

Failure Elimination, Maintenance and Service:

required tools and materials for failure elimination and maintenance on board; access for maintenance;

preventive maintenance;

replacement, repair, failure elimination, adjustment; information concerning external service.

### MISCELLANEOUS

Not applicable