# CHAPTER 6:   PRINCIPLES OF TRAFFIC MANAGEMENT

0601	Introduction

This chapter discusses the Principles of Traffic Management.  These principles may be considered by an authority responsible for traffic management in local or regional areas.  The process includes a review of the geography, meteorology, hydrology and environmental issues of the port or regional area, an assessment of the types and numbers of vessels operating within it, consideration of commercial factors and other activities, a review of the waterspace management techniques and concludes with an evaluation of the types of VTS service and how they can contribute to safety and efficiency of marine traffic operating within the area.  The primary issues are outlined below and may need to be taken into account in determining whether a VTS is required to enhance safety, a process that is described more fully in Chapter 7.

0602	Geography, Meteorology, Hydrology and Environmental Issues

The geography, meteorology and hydrology of the local or regional area (Comment: Regional Area should be reviewed as a wider concept within the manual) will determine the way in which traffic operates within the area and how it may be managed.

Geography.  This mainly involves the assessment of the water available for navigation, identification of the fairways or channels and how they might be marked.  Guidance on assessing the criteria for safe shipping movements have been published the by the Permanent International Association if Navigation Congress (PIANC) and include the relationship between factors such as vessel draught, underkeel clearance (UKC) and channel width.  UKC is a key risk management and safety feature.  Its calculation includes an allowance for factors such as: vessel construction, water density, squat, wave & swell allowance and bottom type.  It may also include an additional safety margin following risk assessment.  Consideration should also be given to the proximity of isolated dangers and the quality/availability of primary and alternative methods of navigation.

Meteorology.  Factors such as the speed and direction of the prevailing wind, direction and height of the waves, visibility and the formation of ice may impact on the assessment of the safe operating patterns in a particular area, fairway or channel and the types of vessels that may be permitted to operate within the area.

Hydrology.  The establishment of safe operating areas, fairways and channels should take into account the hydrology of the area.  This will include factors such as the stability of the seabed, the accuracy of surveys, tidal ranges, tidal streams, prevailing currents and swell.

Environmental Issues.  There are areas where the consequences of an incident would be such that extra safety provision, over that normally applied, may be appropriate.  These areas must be identified so that the VTS can accommodate them.

0603	Vessel Types and Traffic Density

The geography, meteorology and hydrological considerations above will be closely linked with an assessment of the types of vessels, their size and manoeuvrability, traffic density, traffic patterns and the trade being conducted in the regional or port area. The inter-relationship between the environmental factors and the vessel size is self-evident but special consideration may also have to be given to the type of vessel and the cargo that it is carrying.  International guidance such as SIGTTO recommends options for some high-risk ships; national regulations may dictate the need for additional restrictions in the management of certain Hazardous Cargoes.

The Society of International Gas Tanker & Terminal Operators (SIGTTO) document “LNG Operations in Port Areas” gives guidance on the size of domain that should be considered for use with liquefied gas shipping when in a narrow channel.

Key Point

0604	Commercial Factors and Other Activities

Ports must operate in an efficient and timely manner and meet the needs of the users but this must be done without impinging on the safe operation of the port.  Unannounced arrivals and departures can have a considerable and adverse effect on the viability of a port and its operations.  Recreational activities, issues associated with oil and gas production and Naval operations may take place within the area to be covered by a Vessel traffic Service.  These activities will also influence the operation on the scheme and must be taken into account.  A good working relationship needs to be established and maintained with other users of the area and Allied Services.  A VTS must make due allowance for any potential conflict between safety, commercial operations and other activities, and pre-empt such conflicts before they arise.

0605	Waterspace Management Techniques

Having established the available waterspace and the type of vessels that will be operating within the port or area, a number of techniques are available to manage the traffic operating within it.  These include:

- Channel and Fairway Width.  Safety of navigation may be enhanced by establishing a deep water channel within a buoyed fairway which would permit shallower draught vessels to navigate safely outside of the deep draught channel, whilst remaining within the buoyed fairway.

- Traffic Separation Schemes.  Traffic Separation Schemes (TSS) may be established to regulate traffic where traffic patterns and traffic flows indicate that this may be desirable.  TSS may be established by national authorities within territorial sea but those in international waters must be adopted by IMO.

- Two –Way Traffic Occupancy.  Within a channel, normal two way traffic flows may be permitted.  This may include the need for approval for overtaking and for encounters involving vessels carrying hazardous cargoes.  Special consideration should be given to additional restrictions involving overtaking and encounters at pinch points such as bends in the channel.

- One – Way Traffic Occupancy.  Risk assessment may indicate the desirability of limiting the flow of traffic to one – way occupancy only for all vessels or for vessels of a particular type or carrying specific cargoes.  The need for this will be determined through risk assessment.

- Point of No Return.  Ports with significant tidal ranges may require to identify “points of no return” or “abort” points to ensure that a vessels can return to safe water, a lay-by berth or an anchorage in the event that the planned berth is unable to accept the vessels as planned.  This will need to form part of the Passage Planning process.

- Anchorages.  In establishing anchorage areas, consideration should be given to factors such as shelter, depth, holding ground and proximity to channels and fairways.  Specific anchorages may be reserved for use by large vessels or those carrying dangerous goods that are unable to proceed to their planned berth.

- Slot Management.[technical AIS term, need a revised term]  Two and One – Way Management techniques may be combined with the requirement for slot management.  This is the process whereby a vessels is allocated a turn to make passage through the whole or part of a passage within a designated channel.

- Ship Domain.  A vessel may be allocated a “domain” within which all other should remain clear.  The size of this domain may be dictated by international guidance and risk assessment. [note – may need to establish the concept of Domain, note possible conflict with Exclusion zone]

- Exclusion Zone.  An exclusion zone may be established to keep vessel traffic clear of an incident or accident.  These may form part of a contingency plan and should take into account potential incidents involving shore installations as well as vessel emergencies.

- Authorisation of Ship Movements.  Traffic movements may be managed within a port through the authorisation of ship movements.  This requires vessels to seek permission before sailing from a berth or anchorage and may include the provision for advanced notice to enable the managing authority to assess the situation and veto the movement should this be necessary.

- Control of Arrivals and Departures.  The control of arrival and departure times to and from the berth or pilot station or port approach point is an effective way of managing traffic movements and establishing priorities for individual vessels.  This is frequently achieved through negotiation with allied services.

The “Maritime Assistance Service” concept has been established by IMO for the control and direction of serious maritime accidents and incidents.  This could require the VTS centre to make provision for government nominated staff and to accept direction by an external authority.  (Check reference!)

Key Point

0606	Types of Service

Assessment of the issues above are fundamental considerations in determining the need for a VTS (see Chapter 7) and in selecting the Type of Service (see Chapter 5) appropriate to that VTS if deemed appropriate.  Waterspace Management Techniques such as the establishment and marking of channels and fairways and the imposition of Traffic Separation Schemes are passive measures that may be used in conjunction with a VTS but may be used in isolation.  Techniques involving the closer management of vessel traffic, however, will invariably involve the establishment of a VTS.

Where it is decided to establish a VTS, the Waterspace Management Techniques will be a key consideration in determining the type of service that will be required.  The control of departures and arrivals may be achieved through allied services and other passive measures such as channels/fairways and TSS may complement VTS which may only need to provide an Information Service.  The measures in 0604 c) – j) above are active measures that would normally dictate the requirement for a Traffic Organisation Service.  The complexity of the navigational environment will determine, through risk assessment, whether there is a need for a Navigational Assistance Service to be overlaid on the Information or Traffic Organisations Services.  In all cases, the training and qualifications of VTS operators providing the service and their authorisations should be clearly identified and clear operating procedures established (see Chapter 14).