RECOMMENDATION ON THE OPERATION PERFORMANCE REQUIREMENTS FOR VTS EQUIPMENT

The purpose of this document is to give an overview of the operational performance standards and requirements for VTS equipment including: radar, AIS, communications, hydrometeo equipment, data systems and CCTV.

- VTS Radar System

- Closed Circuit TV Cameras as a VTS sensor (CCTV)

- Radio Communication

- Automatic Identification System (AIS)

- Hydrometeo Equipment

- VTS Data System

1. VTS RADAR SYSTEM

  - General

The performance requirements placed on the VTS radar service vary a great deal depending on traffic density, levels of VTS, regional features and the coverage of the VTS area. Basic functions will be provided by the VTS Radar, enhanced functions may be provided by the VTS processing system. The purpose of this document is to describe the general performance requirements of the radar service. By meeting these general requirements, it is possible to obtain the basic level required of the VTS radar equipment in the VTS area.

Requirements of the radar equipment may have a high impact on acquisition and life-cycle costs of a VTS and therefore the performance recommendations are divided into three different levels, where:

- Basic level are applicable to VTS information service and/or navigational assistance service for areas with low traffic density and without any major navigational hazards.

- Standard level are applicable to all levels of VTS as identified by IMO – information service, navigational assistance service and traffic organizational service – for areas with medium  traffic density and/or without major navigational hazards.

- Advanced level are applicable to VTS areas with high traffic density and/or specific major navigational hazards.

These three levels of recommendation may be used as applicable within a VTS, e.g. part of a VTS area may call for basic level and another part may call for standard level.

The VTS Radar System should be capable of performing the following Table 1 functions on the display:

| Parameters / Level | Basic | Standard | Advanced |
| --- | --- | --- | --- |
| path, time and track prediction, |  |  | X |
| CPA, | X | X | X |
| TCPA, | X | X | X |
| anchor watch, |  |  | X |
| vessels vector, | X | X | X |
| course, speed and identity, | X | X | X |
| collision alerts. | X | X | X |

Table 1: Performance Functions

These parameters will assist in the development of the traffic image.

1.2 Characteristics of the Radar Target

The radar system’s detection and measurement capacity depends on the characteristics of the radar and the target. These include the target’s average reflective area and its oscillation in relation to the time and the measuring frequency used. The distribution of the target’s reflective parts, the incidence angle and turning speed have a major impact on the power reflected back from the target.

The detection capacity required of the system is determined according to the service level, traffic density and potential navigational hazards. Table 2 below describes the typical reflection features and service level requirements of VTS targets.

The purpose of the following table is to illustrate typical VTS target parameters, including reflective characteristics, for appropriate radar equipment selection, to enable a VTS to function efficiently in accordance with the performance recommendations and recommended service level.

| TARGET |  | Recommended Service Level |  |  | Corresponding (point) target model |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Basic | Standard | Advanced | radar cross section |  | Height |
|  |  |  |  |  | S-band | X-band |  |
| 1 | Aids to Navigation etc. –without radar reflector. Small open boats, fiberglass, wood or rubber with outboard motor and at least 4 meters long, small speedboats, small fishing vessels, small sailing boats and the like. |  |  | X |  | 1 m2 | 1 m ASL* |
| 2 | Inshore fishing vessels, sailing boats, speedboats and the like. |  |  | X |  | 3 m2 | 2 m ASL |
| 3 | Aids to Navigation with radar reflector. |  | X | X | 4 m2 | 10 m2 | 3 m ASL |
| 4 | Small metal ships, fishing vessels, patrol vessels and the like. | X | X | X | 40 m2 | 100 m2 | 5 m ASL |
| 5 | Coasters and the like. | X | X | X | 400 m2 | 1.000 m2 | 8 m ASL |
| 6 | Large coasters, bulk carriers, cargo ships and the like. | X | X | X | 4.000 m2 | 10.000 m2 | 12 m ASL |
| 7 | Container carriers, tankers etc. | X | X | X | 40.000 m2 | 100.000 m2 | 18 m ASL |

Note: The figures in the above table are typical examples and as an average only.

*ASL = Above Sea Level

Table 2. Typical Reflection Features and Service Level Requirements

1.3 Radar Detection Performance and Disturbances

When selecting a radar system, attention should be paid to the fact that the target types defined by the VTS authority should enable reliable detection even in demanding weather and environmental conditions, in visibility of 0.5 Nm or rain up to 10 mm/hr and in sea conditions of Sea State 1. The system should be designed in such a way that the defined target types could be detected reliably in the entire area covered by the VTS service.

Factors disturbing and restricting the performance of radar systems include noise as well as interference and clutter signals from various sources. Every radar should be designed and equipped with devices to reduce the adverse effects of rain and sea clutter and enhance the probability of target detection per scan. The radar should also be designed and installed so as to eliminate to the maximum extent possible, false echoes caused by side lobes or reflections from nearby structures. When selecting a radar system and the measuring frequency, regional special conditions such as heavy rainfall should be taken into account.

There are several technological solutions available to raise the performance of the radar system to the required level. Most typical solutions e.g. include higher average transmission power, larger antennas, the use of circular polarization, reducing the receiver noise, sector blanking and more effective processing and filtering of the transmitted and received signal.

1.4 Radar Accuracy and Discrimination

The measuring accuracy and discrimination of the system is determined by the VTS authority based on recommendation levels. The recommendations for target separation for different service levels are listed in Table 3.

| Radar accuracy and physical separation between small point targets for discrimination in display and tracking |  |  | Recommendation level |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Basic |  | Standard |  | Advanced |  |
|  |  |  | Display | Tracking | Display | Tracking | Display | Tracking |
| In range |  | Short range applications (<5 Nm coverage – include waterways, harbors etc) | 25 m | 40 m | 20 m | 30 m | 15 m | 25 m |
|  |  | Long range applications (up to 20 Nm coverage – littoral waters, offshore etc) | 75 m | 100m | 60 m | 75 m | 50 m | 60 m |
|  |  | Very long range applications (>20 Nm coverage) | N/A |  | 100 m | 125 m | 80 m | 100 m |
| In azimuth | X-band | Angle between targets as seen from the radar | 1.2o | 1.3o | 0.7o | 0.8o | 0.55o | 0.6o |
|  |  | Or distance in meters, whichever is the greater | 25 m | 40 m | 20 m | 30 m | 15 m | 25 m |
|  |  | Corresponding –3 dB antenna horizontal beam width | 0.6 - 0.7o |  | 0.4 - 0.45o |  | 0.35 - 0.4o |  |
|  | S-band | Angle between targets as seen from the radar | N/A |  | 3.5o | 4o | 1.8o | 2o |
|  |  | Or distance in meters, whichever is the greater | N/A |  | 20 m | 30 m | 15 m | 25 m |
|  |  | Corresponding –3 dB antenna horizontal beam width | N/A |  | 1.8 - 2o |  | 1 - 1.25o |  |

Table 3. Target Separation and Accuracy

The system should be designed in such a way that the defined radar accuracy and discrimination can be achieved in the entire area(s) covered by the VTS service. In long measuring distances, the impact of the height and type of antenna on the measuring accuracy and resolution should be taken into account.

1.5 Recording and Replaying of Data

The extraction of plot data and voice recording should be automatic. Track initiation should be either automatic for all areas but with the ability to de-select specific areas or manual, depending on the concept of operations. The maximum number of confirmed tracks for each individual radar sensor and other tracking performance parameters are defined by the VTS authority, based on traffic density and types of tracks.

1.6 Availability

The requirements for the availability of the radar service should be defined by the VTS authority. The recommendations for availability are listed in Table 4 below.

| Availability for radar coverage |  |  |  |
| --- | --- | --- | --- |
|  | Recommended system level |  |  |
|  | Basic | Standard | Advanced |
| Recommended availability for the radar coverage | 99 % | 99.6 % | 99.9 % |

Table 4. Availability

Availability is defined in IMO Resolution A.915 (220 Ref.40 as:

“The percentage of time that an aid, or system of aids, is performing a required function under stated conditions.  The non-availability can be caused by scheduled and/or unscheduled interruptions

Where the coverage from two or more radar sites provides overlapping coverage, as long as at least one radar is available in the area of waterway normally covered, the requirements for radar coverage is met.  Thus from a coverage perspective, unusable time for a waterway can be calculated as the time when any portion of the waterway is without a usable radar.

The radar coverage provided by adjacent countries, may, if agreed, be taken into account when calculating the performance of the VTS radar.  In addition, in areas of high traffic where two waterways meet with converging traffic the location of radar sites should be determined in a manner that minimises shadowing.

Single coverage: Unless dual coverage is provided throughout the whole coverage area, some VTS sites provide a mix of overlapping and single radar coverage. VTS sites that provide single coverage to at least some portion of a waterway are considered to be single coverage sites,

### Calculation

Administrations may choose to calculate service availability using one of two methods:

- by waterway model, or

- by radar site/ radar site combination model

Waterway availability model: In this model administrations need to define which waterways are high risk and which waterways are low risk. Separate calculations for high and low risk are required, providing both exist within the coverage area. Individual waterway availability calculations are then averaged to produce one figure for each waterway risk category. If desired, a figure for each waterway may be reported.

Radar site availability model: In this model, administrations must define which radar sites serve low risk waterways and which serve high risk waterways. The overall availability is calculated by averaging the availability of the associated individual radr sites as illustrated in the examples below.

Example 1: An advanced level site receives complete radar coverage. For example, a VTS area having two radar sites where at Radar site A the availability was 99% and Radar site B’s availability was 99,7%, but both radar sites suffered coincidental outages during 0,1% of the period. Although neither radar site individually met the availability target the combined availability was 99.9% and met the performance goal achieved. Example 2: A basic level site receives coverage from radar site A (as above) covering a low risk waterway. Radar site C, adjacent to radar site A, provides completely overlapping coverage to the low risk waterway portion served by radar site A. Radar site C’s performance was 99%, but neither unusable period (of radar sites A and C) coincided. The low risk waterway covered by radar site A received usable signals for 100% of the time.

While these two distinctly different situations have different availability requirements based upon the level of risk, the concept of availability remains consistent – the minimum requirement is met for the coverage area.

Note: Higher level availability targets may be applicable to more critical parts of areas.the VTS area.  Risk assessment may support lower availability targets in less critical

2. CLOSED CIRCUIT TV CAMERAS AS A VTS SENSOR (CCTV)

2.1	General

The performance requirements placed on the CCTV service varies depending on traffic density, levels of VTS, special regional features and the coverage of the VTS area. The purpose of this document is to describe the general performance requirements of these sensors.

Special surveillance cameras such as infrared are not covered by this Manual, as they are considered to be outside of the scope of a VTS.

2.2 Detection Performance

Depending upon the circumstances, cameras should be capable of identifying the type and possibly the name of the vessels concerned.

The type of vessel should be capable of being identified normally at a minimum range of 3 Nm from the camera location, but this will depend on the individual circumstances, including local topography. The identity of a vessel by shape, color and other features should be capable of being determined normally at a minimum range of 1 NM, but again this will depend on the individual circumstances. These ranges are based on where the nominal visibility is in excess of 10 NM.

2.3 Characteristics of the CCTV

Cameras may be used either in low traffic density areas where this is a more cost effective solution to radar or in conjunction with radar as an additional sensor, depending on the level of risk.

The camera should be capable of automatically tracking a vessel manually selected by the VTS Operator via the VTS Screen, if required by the VTS Authority. In addition, there should be the possibility for the VTS Operator to manually de-select the automatically acquired target and manually select another target and/ or area of the acquired target, in order for the VTS Operator to perform specified monitoring tasks, such as pilot embarkation/disembarkation.

Where more than one camera is installed to cover a VTS area it is desirable for the output from both to be fused together to provide one composite picture.

There may be a requirement for a zoom, pan and tilt facilities.

2.4 Recording and Replaying of Data

The data should be recorded automatically and capable of being replayed onto a separate replay system, if required by the VTS Authority.

2.5 Availability

The requirements for the availability of CCTV is a matter for the VTS Authority to determine.

3. RADIO COMMUNICATION

  - Very High Frequency (VHF)

It is common for the VTS to have its own independent VHF network, for the use within specifically designated VHF Channels.

This network may comprise of one or more VHF Channels in different sectors of the VTS Area.

The VTS Authority may require specific VHF Channels to be designated by the National Radio Authority for specific types of operations.

The VHF equipment must comply with national and international regulations.

  - Long Range Communication

In the case where a VTS Authority requires pre-arrival information, the normal maritime communication systems should be used and therefore an independent network is not required.

  - Radio Direction Finder (RDF)

A number of VTS Authorities require RDF receivers to receive information on the identity of the vessels.

In order to ensure accurate identification the use of two or more separate RDF bearing stations are required.

Bearing accuracy should be better than +/- 2 degrees.

All bearings should be automatically displayed on the chosen screen when the signal has been received after a delay of no more than 3 seconds. The bearings should remain visible on the chosen screen as long as the vessel is transmitting a signal.

It is foreseen that this requirement may decrease as AIS usage by VTS Authorities increases, but will not become obsolete as, depending on national requirements, some vessels may not be equipped with AIS.

RDF is not suitable of being used for continuous tracking.

  - Communication with Allied Services

VTS Centres should be equipped with the ability to communicate with allied services by the use of reliable and secure communication networks. It was usual for a VTS Centre to be equipped with a point to point line with MRCCs for this purpose. However, this means have been found to be inadequate and unnecessarily expensive to establish and maintain. Therefore it is recommended that VTS Centres should be equipped with a digital switched network, with caller identification.

- AUTOMATIC IDENTIFICATION SYSTEM (AIS)

The MP (Maritime Picture) provided by a AIS should be subject to track fusion. The resulting (Recognized Maritime Picture) should be a fused situation picture where both track information obtained from AIS, and

Radar is presented as one unambiguous RMP. However, the VTS Operator should have the ability to choose whether to display the information on a sensor basis i.e. whether  targets are to be displayed using the AIS derived data only or from the radar display only or from both subject to track fusion, in accordance with the requirements of the VTS Authority.

- HYDROMETEO EQUIPMENT

It is essential that a VTS Centre has access to local Hydrometeo information relevant to the VTS Area(s) and can, if required by the VTS Authority, disseminate this to their users and allied services.

Where a VTS Authority determines a need to establish their own monitoring stations, it should be noted that the individual VTS Authorities should determine the accuracy and availability requirements for each VTS Centre, as these will be based on individual circumstances. The following Table 5 gives an indication of typical minimum accuracy and availability requirements.

| Parameter | Availability | Accuracy |
| --- | --- | --- |
| Height of Tide | 99.9% | < +/- 0.10 m |
| Rate of Tidal Stream/ Current | 99.9% | < +/- 0.5 knots |
| Direction of Tidal Stream/Current | 99.9% | < 10 Degrees |
| Wave height | 99.9% | +/- 10% of the height |
| Wave Direction | 99.9% | +/- 20 Degrees |
| Wind speed | 99.9% | < 1m per sec |
| Wind Direction | 99.9% | < 10 Degrees |
| Visibility | 99.9% | +/- 10% of the distance |
| Air Temperature | 99.9% | < +/- 1 Degree |
| Air Humidity | 99.9% | < +/- 5 % |
| Air Pressure | 99.9% | < +/- 2 hPa |

Table 5: Indication of typical minimum accuracy and availability.

The VTS Authority should specify the time periods over which the various data parameters should be updated and may be averaged, if required, as these factors will depend upon the local circumstances pertaining to the VTS Centre.

- VTS DATA SYSTEM

6.1	General

A VTS Data System should be flexible and easily up-datable alongside of the routine operations of the VTS Centre without the need for interrupting these VTS operations.

6.2	Emergency Situations

To optimize the operations of the VTS Centre, this data information should be capable of being easily transferred to another location, in the event of an emergency situation resulting in temporary closure of the VTS Centre. The need for an alternative site for the VTS Centre is covered under a separate IALA Recommendation.

  - Gathering and Recording of information

The period of time for which the information gathered by a VTS is required to be stored, should be identified in internal procedures. This time period should be such that it allows for the full retrieval of data post-incident/accident, in compliance with national requirements and those of the incident/accident investigation procedures of the VTS authority and other interested parties. This type of information may include:

- Communications, internal and external.

- Sensor data, i.e. data used to generate the traffic image such as radar, CCTV, AIS.

- Shipping information data, i.e. vessel and cargo data, including vessel movement information.

- Meteorological and hydrological data; and

- Data from other sources if relevant.

Provision should be made for the storage, security, retrieval and presentation of this information.

6.4 	List of Participating Vessels

The list of participating vessels should include static information and dynamic information concerning the vessel, for example Vessels Name, Call Sign, IMO Number, MMSI, ETA, ETD, Course, Speed and Position, if required.

An example of a typical list is shown below. This example is taken from the Baltic AIS Network which is integrated to the NAVTEX VTS system.