DRAFT GUIDELINES ON THE EXCHANGE OF DATA BETWEEN VTS SYSTEMS

# Introduction

There is increasing interest in exchanging data between VTS systems, although it needs to be recognised that it will form only a part of the overall data flow into and out of a VTS.  These systems may be locally or regionally grouped and will be using a variety of sources (including external databases) and which may be site dependent / application specific.

Even if several VTS systems exchange data, there is currently no standard available covering the different types of data available in a VTS.  To make a standard describing data exchange between VTS systems, it is important to choose a way forward that will take into account the ever evolving needs of new types of information required to be exchanged.  In addition, a fixed format will lead to stand-alone proprietary solutions for different types of data not covered by it.  However, it is necessary to create a baseline so that interfaces between different vendors will work together without further refinement.

# Objective

This paper provides in section 4.1 guidance on the content and type of real-time data which may be exchanged between VTS systems and in section 4.2 et seq, guidance on the exchange of other data between VTS and/or data bases.

# Data VALIDATION AND EXCHANGE

It is highly recommended that only validated data be exchanged to avoid operational conflicts between VTS systems.  It is essential that the system network architecture prevents data loops.

# TYPES / CLASSES OF DATA THAT MAY BE EXCHANGED

These may be contained in the following types of data :

## Real-Time Track  Message

This is dynamic information related to a target being followed by a tracking system, which may be derived from multiple sensors and other sources.  Currently, the most common sources are radar and AIS.  Position, course and speed over ground, and identification with a few additions comprise the parameter set for the real time track specific message involved in tracking.

### Considerations relating to track data exchange

Track data needs special consideration because latency is critical.

Track data exchange is used to :

- Extend the coverage of own system;

- Avoid repeated identification of targets which have already been identified by the neighbouring system.

- Maintain the security of VTS data.

The following data set has been identified as typical for the integration, correlation and display in the receiving VTS system as well as for the storage in an associated database system :

- Ship identification (e.g., IMO number or MMSI)

- Name of vessel;

- Call Sign;

- Position (Latitude / Longitude);

- Course (COG);

- Speed (SOG);

- Heading (if available from AIS);

- Time stamp (time must be synchronized between VTCs);

- Data source;

- Quality figure.

The minimum data set is considered to be ship identification, position, and time.

A quality figure is an assessment of the reliablity of the transmitted track data.  There is currently no existing standard for such a quality figure.  However, as a preliminary indicator, the sensor data source should suffice. (See ATC reference on ADS-B MOPS)

The message is described in the table below. The message purpose is to enable continuous tracking and handover between the adjacent VTS systems.

The following messages are identified

- Dynamic target data

- Basic static target data

The output is based on XML.  Compression may be required because XML is bandwidth intensive.  There are two approaches to data compression.  This may be done at the application level or the link level, but the compression method needs to be understood for both the sending and receiving parties. Similar considerations must be given to data encryption.

Extended static information (e.g. sailing plans) is not part of this interface but requires a different interface preferably compatible with existing systems (e.g., SafeSeaNet).

### Dynamic data

| Header | Units | Description | Range |
| --- | --- | --- | --- |
| Timestamp | mm.ss | Minutes and seconds from UTC timestamp | 00.00 - 59.59 |
| Message ID |  |  |  |
| Datablock |  |  |  |
| Target ID type | Char | MMSI, IMO, Callsign, or Unknown | M, I, C, U |
| Target ID data/internal Target ID | <string> | Either MMSI, IMO number Callsign, or internal track id | up to 9 char |
| Target timestamp | mm.ss.s | Minutes, seconds and tenths from UTC timestamp | 00.00.0 - 59.59.9 |
| Target source | Char | AIS, Radar, Fused, Coasted, Other, and Terminated | A,R,F,C,O,T |
| Target quality | Char | Low, Medium, High | L,M,H |
| Target position (lat/lon) | pDDD:MM.mmmm, | p=+/- Degrees,minutes, fraction of minutes | -180:00.0000 – +179.59.9999 |
| Target SOG | KKK.k | speed over ground (knots) | 000.0 – 999.9 |
| Target COG | DDD.d | course over ground | 0.0 – 359.9 |

VTS Centres are encouraged to define a zone of cooperation.  The messages should be repeated for every target when within this zone.

Note: Update rates and bandwidth will be effected by traffic density, sensor rates, and size of zone of cooperation.

### Basic static and voyage related data message in combination with real time tracking message

Static message (taken from AIS msg. 5) (to be sent prior to the first dynamic message, and then every 6th minute):

- MMSI

- IMO Number

- Callsign

- Name

- Type of ship and cargo

- Length

- Beam

- Draught

- Destination (per UNLOCODE)

- ETA

- Version number for the interface

VTS Authorities should analyse the data traffic and the requirements for data bandwidth and select a suitable data link, taking into account traffic growth.

## Non-Real-Time Data

### Meteorological and hydrological information data class

This information can vary according to the different types of sensors and information, although NMEA has provided a form of standardisation.  Transfer of this class of data requires a different interface preferably compatible with existing systems.

### Voyage related and static ship information data class

Although non-real-time data, voyage related and static ship information are subject to  change and may be useful to adjacent VTSs. This information is not necessarily restricted to that provided by AIS and may be application-specific to regional or national requirements.  For a comprehensive list of AIS data elements, refer to ITU-R Recommendation M.1371-1.   Transfer of this class of data requires a different interface preferably compatible with existing systems (e.g., SafeSeaNet).

### Chart data class

### Cargo data class

### Port data class

# TECHNOLOGIES

There are several non-proprietary and internationally recognized methods of data exchange, including NMEA, EDI, XML, TCP/IP, as well as NATO standards.  A format suitable for real time data exchange needs to be selected.

The eXtensible Markup Language (XML) -- has emerged as a standard for data representation and exchange. The basic ideas underlying XML are simple: tags on data elements identify the meaning of the data, rather than, e.g., specifying how the data should be formatted, and relationships between data elements are provided via simple nesting and references.  Servers and applications encoding their data in XML can quickly make their information available in a simple and usable format, and such information providers can interoperate easily. Information content is separated from information rendering, making it easy to provide multiple views of the same data.

Using XML schemas for  data class validation will make the transfer of data easier. The schemas can be made part of the standard.  Data classes can be checked against schemas, helping to validate the data.

# Abbreviations and definitions

AIS	Automatic Identification System

COG	Course Over Ground

EDI	Electronic Data Interchange

IMO	International Maritime Organisation

ITU-R	International Telecommunications Union – Radiocommunications Bureau

NATO	North Atlantic Treaty Organisation

NMEA	National Marine Electronics Association

SOG	Speed Over Ground

TCP/IP	Transmission Control Protocol / Internet Protocol

VTS	Vessel Traffic Services

XML	eXtensible Markup Language

# References

- IALA VTS Manual, 2002	3.3.4.2

- IALA VTS Manual, 2002	4.3.2

- ITU-R M.1371-1 Technical Characteristics of Automatic Identification Systems using Self-organizing Time Division Multiple Access

- Regional Directives, e.g. EU Directive 2002/59

- XML - Recommendation of W3C dated 4 Feb 2004

# NOTES

During last meeting we gathered in the ad hoc group to deal with the real time track data exchange and described the results in same add  on to paper “VTS22-TWG-WP07- Data Exchange”(attached). The issue "real time track data exchange" in this paper is 70 % finalised. I made a copy of this paper I put in my addition and comments and called it “VTS22-TWG-WP07- Data Exchange-changed by Eckhoff” (attached). The missing 30 % are recommendations to the format of the data to be exchanged, some questions concerning data traffic addressed already last time and agreements to XML-protocol. Example for data format definition see below.

| Name of date | Abbr. | Desciption | Format | ## Coding | Unit | Presentation |
| --- | --- | --- | --- | --- | --- | --- |
| Position Latitude | POS LAT | Geographical Position Latituide (Reference WGS-84) | String | dd.mm.ss | Degrees. Minutes. Seconds | UNI-Code |

To ease the job it is useful to copy most of the data definition from Safe Sea Net, NMEA or IEC 61162-1, ITU- R1371 (AIS). If lucky the part “Real Time Data” can be finished during the next meeting.

Aside from the real time exchange there is data that not to be time critical at exchange between VTS´s. This data will be exchanged preferably the data bases on both ends of the VTS link and not be fed directly into VTS-Systems like track data will. This recommendations may even by used for data exchange of a VTS with not VTS data bases ( security-, port-, commercial data bases)

In the future this data exchange will increase a lot like the installation of network servers like Safe Sea Net (SSN), HELCOM (Baltic Sea data exchange) or LR-Monitoring data exchange shows. Therefore an IALA recommendation would be helpful. I assume the industrial members will agree.

The data definition may be copied from sources like ISO, SSN , ITU, IEC etc.

What not should be copied from those sources are the entire application specific messages like SSN “hazardous cargo” messages. Those messages are designed specific for SSN-purposed. IALA should not recommend a number of application specific messages. With any new application (and the will come lot in the future) VTS committee would have to start a new definition process.

I recommend therefore to define the attributes of data classes like vessels, cargo, passengers, crew, charts, environment (met,hydro). This objects are used in any VTS world wide. Any application can be supported by set of data attributes to be exchanged. The data classes be identified by a data class identifier, each attribute or set of attributes by a attribute identifier in combination with the data object id . Please see the example se below.

Data Class:  Vessel

- dynamic ship data attributes: Position (Lat, Long),  COG , SOG ,..

- static ship data attributes: ( IMO NR, MMSI, ...)

- voyage related data attributes: ( Cargo Type, ETA...)

Data classes: environment (Water and Air)

- environmental data attributes: (Met) temperature, humidity, wind speed,.... (hydro) stream, water temperature,...)

Data Class : Chart

- chart object attributes:  Position and identification of  Buoys, lighthouse, way points, fair way line, sector, berth...

Data Class: Cargo

- Cargo attributes: UN-Code, hazard classification,...

Data Class: Crew

- Crew attributes: t.b.d.