Performance requirements for Automatic Identification Systems (AIS) Service in VTS

Table of Contents

1	Introduction	3

1.1	Abbreviations	3

1.2	References	3

2	General Provisions	5

2.1	Objectives of AIS	5

2.2	Application	7

3	Description of AIS as a VTS sensor	8

3.1	General	8

3.2	Coverage aspects	8

4	AIS Service	9

4.1	Equipment	9

4.2	AIS Interfaces	9

4.2.1	Data interface to the AIS Base Station	9

4.2.2	AIS Service Interface	9

5	Operational aspects in relation to the VTS	10

5.1	General requirements	10

5.2	Use of virtual MMSI	10

5.3	Use of the VTS Targets message	11

5.4	Short safety-related messages	11

5.5	Use of binary messages for data exchange	11

5.6	Assigned Mode	12

5.7	Graphical presentation	12

5.7.1	Symbol usage	12

5.7.2	Interaction with radar tracks	13

5.8	Data Validity	13

6	Security	13

# Introduction

AIS is intended to enhance: safety of life at sea; the safety and efficiency of navigation; and the protection of the marine environment. In addition, AIS may contribute to maritime security.  SOLAS regulation V/19 requires that AIS exchange data ship-to-ship and with shore-based facilities. Therefore, the purpose of AIS is to help identify vessels; assist in target tracking; simplify information exchange (e.g. reduce verbal mandatory ship reporting); and provide additional information to assist situation awareness. In general, data received via AIS will improve the quality of the information available to the VTSO or OOW, whether at a shore surveillance station or on board a ship. AIS should become a useful source of supplementary information to that derived from navigational systems (including radar) and therefore an important ‘tool’ in enhancing situation awareness of traffic confronting users.

## Abbreviations

AIS		Automatic Identification System

ASM 		AIS Service Management

BSC		Base Station Controller

FATDMA	Fixed Allocation Time Division Multiple Access

LSS		Logical Shore Station

OOW		Officer of the Watch

PSS		Physical Shore Station

SOTDMA	Self Organised Time Division Multiple Access

VTSO		Vessel Traffic Services Operator

## References

| [1] | IMO MSC. 74(69), Annex 3 | IMO Recommendation on Performance Standards for a Universal Shipborne Automatic Identification System (AIS) |
| --- | --- | --- |
| [2] | IMO SOLAS | Chapter V Safety of Navigation Regulation 19 |
| [3] | ITU Radio Regulations | Appendix S18, Table of Transmitting Frequencies in the VHF Maritime Mobile Band |
| [4] | ITU-R M.1371 | ITU Recommendation on the Technical Characteristics for a Universal Shipborne Automatic Identification System (AIS) Using Time Division Multiple Access in the Maritime Mobile Band |
| [5] | IEC Standard 61993 Part 2 | Universal Shipborne Automatic Identification System (AIS) Operational and Performance Requirements, Methods of Testing and required Test Results. |
| [6] | IALA Recommendation A-?? | IALA Recommendation on the Technical Clarifications of Recommendation ITU-R M.1371-1 |
| [7] | IALA Recommendation A-124 | IALA Recommendation on AIS Shore Stations and Networking Aspects Related to the AIS Service |
| [8] | Resolution A.694(17) | General Requirements for Shipborne Radio Equipment forming Part of the Global Maritime Distress and Safety System (GMDSS) and for Electronic Navigational Aids |
| [9] | Resolution A.953(23) | World-Wide Radionavigation System |
| [10] | Resolution A.688(17) | Code on Alarms and Indicators |
| [11] | Resolution MSC.39(63) | Adoption of amendments to the Code on Alarms and Indicators |

# General Provisions

## Objectives of AIS

AIS should:

- Provide information automatically to appropriately equipped shore stations, other vessels and aircraft, including the craft’s identity, type, position, course, speed, navigational status and other safety-related information

- Receive information automatically from similarly fitted vessels and craft

- Monitor and track vessels

- Exchange data with shore based facilities.

- Assist in ensuring the highest possible level of safety and efficiency for the vessel traffic in the designated area

The requirements should not be applied to cases where International agreements, rules or standards provide for the protection of navigational information.  AIS should be operated taking into account the guidelines developed by the International Maritime Organization.

## Application

These performance requirements are for the use of AIS in a Vessel Traffic Service (VTS).  The AIS should improve the safety of navigation by assisting in the efficient navigation of ships, protection of the environment, and operation of VTS, by satisfying the following functional requirements:

- in a ship-to-ship mode for collision avoidance;

- as a means for littoral States to obtain information about a ship and its cargo; and

- as a VTS tool, i.e. ship-to-shore (traffic management).

The AIS provides to ships and competent authorities, information from the ship, automatically and with the required accuracy and frequency, to facilitate accurate tracking.

Mandating AIS carriage and establishing a service to receive, process and distribute the AIS signals received from vessels enhances safety and security and improves a shore facility’s ability to manage traffic.

The shore facility that receives and processes the AIS signal can vary in size and mission ranging from a small station, a VTS, a Captain of the Port (COTP), or a regional data fusion centre.

Some shore facilities may need to act on the information, another may need to monitor AIS and maintain an information database or archive.  For these reasons, a nationwide network may be set up for this purpose.  If established, the network must be scalable and adaptable to a range of demands and requirements.

The service must also be capable of information exchange and distribution among several users ashore and afloat.  Government agencies outside the VTS and other commercial maritime interests all may have justifiable needs for AIS data.

In order for VTS to take full advantage of AIS, access to the capabilities of an AIS Base Station is required.  This access may be through an AIS network or directly from the Base Station itself.

# Description of AIS as a VTS sensor

## General

Automatic Identification System (AIS) is a system that makes it possible to monitor ships from other ships, and from shore based stations.  AIS equipped ships continuously transmit a short message containing information of position, course over ground (COG), speed over ground (SOG), gyro course (heading), etc. Ships equipped with AIS meeting anywhere on earth will be able to identify and track each other without being dependent of shore stations.

Shore stations receive the same information from AIS equipped ships within the VHF area of the station when monitoring the coastal areas and the ports. The AIS uses a broadcast and interrogation technology that operates ship-to-ship and ship-to-shore including limited communication capabilities.

The International Maritime Organisation (IMO) has established carriage requirements for merchant ships.  The International Telecommunication Union (ITU) has defined the technical standard and ratified the global frequencies.  In addition, the International Electrotechnical Commission (IEC) has developed methods for testing AIS for global interoperability.

AIS is making navigation safer by enhancing situational awareness and the possibility of detecting the location of other ships, even if they are behind a bend in a channel or river or behind an island in an archipelago. AIS also solves the inherent problem with all radars, by detecting boats and smaller ships, fitted with AIS, in sea-clutter and in heavy rain.

## Coverage aspects

In general, AIS design coverage ranges should approximate VHF voice communication ranges.  However, actual vessel traffic density or geographic considerations (i.e., mountains or other VHF occlusions) may determine the need for additional base stations.  As a general rule, an AIS Base Station should be able to accommodate about 300 active AIS units.

# AIS Service

The purpose of an AIS service is to provide AIS information from one or several base stations to users. An AIS service will provide status on AIS equipment, and the maintenance and control of data.  The AIS Service may consist of one or more Base Stations or AIS Networks, or a combination of both.

## Equipment

AIS equipment is described in detail in IALA Recommendation A-124 on Automatic Identification System (AIS) Shore Station and networking aspects relating to the AIS Service.

## AIS Interfaces

### Data interface to the AIS Base Station

The AIS Base Station will be interfaced to the VTS system via the hardware interface provided by the manufacturer of the unit. The primary use of the unit is to receive AIS data as received from mobile units, but there are additional important interface requirements that need to be supported such as how the unit supports firmware upgrades, base station configuration and data exchange. There is currently no standard which allows for the interchange of base stations of different manufacturers without modification.  However, the following interfaces are common:

- Serial

A connection to the unit through a serial connection is the traditionally most common way of interfacing. The interfaces in use are RS232 and RS422/485. The differences between these interfaces are solely hardware related where the balanced (RS422/485) has a better transmission characteristic, while RS232 are more commonly supported for computer equipment.

- Ethernet

Ethernet support is becoming widely supported either directly from the Base Station or via Base Station controllers. The support for Ethernet directly out of the base station simplifies the set up of a site, and has advantages over serial as there is no need to keep a hardware connection (like a serial cable) from the unit to another converting device. Currently the support differs between the manufacturers as some requires an extra device in addition to the base station to support Ethernet, some support only one protocol (UDP or TCP) while some manufacturers supports configuration of the protocol on the interface port.

Detailed technical interface requirements can be found in IALA Recommendation A-124 on Automatic Identification System (AIS) Shore Station and networking aspects relating to the AIS Service.

### AIS Service Interface

The VTS will select features of the AIS Service..  These features are the Basic 
AIS Services defined in A-124.

# Operational aspects in relation to the VTS

## General requirements

The AIS will provide timely, relevant and accurate information to users to support their decision-making processes.  The AIS also provides information to VTS and allied services to support their missions.

a. Basic information needs of the VTS.  Representative users identified the following as the basic information needs for vessel tracking and port security related missions.

- Up-to-date knowledge and/or information regarding the route to be transited.

- Timely, relevant, and accurate information about other vessels within the area that might affect safety, security, or the decision making of the VTSO.

- Timely information about emergency and environmental conditions that might affect safety or the decision making of the VTSO.

- Where required, the transmission of relevant information to the mariner in a manner that does not distract from the task at hand, particularly in narrow, confined channels where there is heavy traffic.

- Existing navigation aids and tools, pilotage systems, navigation management systems, and regulations provide information to the mariner but these systems require integrity monitoring to ensure the information they impart is accurate.

b. Integrated Operations.  Where applicable, AISs will support regional Vessel Traffic Services between adjoining VTSs, both domestic and foreign.

c. Incident Analysis Support.  The AIS service shall support incident response and analysis.

- Immediately after notification of an incident, the AIS service shall alert vessels in or planning to enter the area of concern with particulars of the incident.

- The AIS service will provide for review of supporting information and events that occurred before, during, and after an incident.

- The AIS service will be able to support replay of a recorded incident.

d. Operations Support.  The AIS service may support port operations by providing information to appropriate shore facilities.  It provides automatic vessel position reports and movement information as it is received at remote sites throughout the service area. In support of incident response, the AIS service, in conjunction with the port authority, will provide information about traffic and the corresponding situational information.  Utilizing the AIS Service, the VTS will monitor safety and security zone boundaries that are established in connection with an incident.

e. Allied Services Support.  The AIS service will provide information to other agencies and allied services when a particular vessel of interest has been identified.

## Use of virtual MMSI

Every base station will have a unique Maritime Mobile Service Identity (MMSI) number. This number defines the AIS base station. When operating from a VTS, the VTS might have its own virtual MMSI number. This will enable the VTS to send messages to vessels using one MMSI regardless of the number of base stations connected, and even when operating through an AIS Network. The feature has to be supported through either the AIS Network or the software used by the VTS vendor when sending data from the VTS.

## Use of the VTS Targets message

The set of standard AIS messages comprises a message called VTS Targets, which contains information on targets derived by means other than AIS. Typically this will be targets that are tracked by a VTS-system based on radars. Sometimes the VTS Targets message is also referred to as VTS Footprint.

The AIS message for VTS Targets can be filled with information for only a few targets, each comprising:

- Target id (MMSI, IMO number or Callsign)

- Latitude

- Longitude

- Course (COG)

- Speed (SOG)

- UTC time stamp

A VTS target should only be included in the message if it contains latitude and longitude values, while all the other data elements may be unknown.

A VTS Targets message can either be sent as an addressed message to one or more target(s), or as a broadcast message sent to all targets that are inside the VHF coverage area of one or more AIS base station(s).

For additional information, refer to IALA AIS Guidelines.

## Short safety-related messages

Short safety-related messages are fixed or free format text messages addressed either to a specified destination (MMSI) or all ships in the area. Their content should be relevant to the safety of navigation, e.g. an iceberg sighted or a buoy not on station. Messages should be kept as short as possible. At present these messages are not further regulated, to keep all possibilities open.

## Use of binary messages for data exchange

Those messages identified for use during a 4-year trial period are:

- meteorological and hydrological data

- dangerous cargo indication

- fairway closed

- tidal window

- extended ship static and voyage related data.

- number of persons on board

- pseudo-AIS targets.

For further details, see IMO SN/Circ.236, dated 28 May 2004, Guidance on the Application of AIS Binary Messages.

## Assigned Mode

VTS may use the AIS Service capability to change the reporting mode of selected shipboard AIS units by changing the reporting rate.

## Graphical presentation

The presentation of AIS-data can be done in several ways, spanning from the target list on a typical MKD-unit (Minimal Keyboard and Display) that is the minimum required user interface for an AIS transponder to the more advanced chart- and symbol based displays on modern VTS- and ECDIS-systems.  A VTS-operator is most concerned about the overall traffic situation inside his/her area of responsibility, and the detection of any deviances from the normal and safe navigation of any of the ships.

### Symbol usage

The IALA Guidelines on AIS as a VTS Tool (December 2001) includes a description of the recommended AIS target symbols, but these are originally intended for the onboard ECDIS/ECS systems. As the guideline points out the given symbols are not necessarily adequate in the VTS context. The main difference is that a VTS-operator may prefer a much wider range of information than is necessary onboard a ship. For example traffic management may necessitate the use of symbols which depict different types and sizes of vessels. Alternatively, it may be necessary to show which vessel have pilots embarked, and which do not.

A VTS-system should include a configurable set of AIS target symbols that corresponds to the following 6 categories:

- Vessels with Class A AIS transponder

- Vessels with Class B AIS transponder

- AIS base stations

- SAR aircrafts with AIS transponder

- AtoNs with AIS transponder

- VTS Targets

When a new target shall be displayed the VTS-system can select one of the 6 categories by looking at the type of AIS messages that is received. When the category of a new target is established the VTS could also select among a list of targets dependent on the type of ship as given by for instance the AIS-message for Ship Static and Voyage Related Data. If the VTS-system comprises a vessel database, the type of target can also be looked up from there based on the target ID (MMSI, IMO no or Callsign). This means that dedicated symbols can be used for different ship types like passenger ship, cargo ships, tanker, high-speed crafts, tugs, etc.

A relative indication of the length and beam (or even the draught and height over keel for 3D graphical presentations) can be given similarly by using AIS-provided data optionally supplemented by database lookups. Such size indications must be scaled according to the range (usage level) of the current chart view.

The symbol orientation on the chart should be according to the heading indicator, and the speed-vector should indicate the course (COG), the speed (SOG) and the rate of turn (if applicable). The length of the speed-vector should depict where the target will be positioned after a certain amount of time, i.e. xx minutes from current time where xx should be easy to change when ranging in or out in the chart.

A target should be marked as lost when no AIS-messages have been received from that target in the last 200 seconds, and the symbol should be removed from the chart after 6 minutes. A target is marked as lost by adding a solid line across the symbol or by replacing the symbol by an x-sign. No speed-vector or rate of turn indicator will be shown for lost targets.

### Interaction with radar tracks

A target that is tracked by radar and also carries an AIS transponder may be displayed with one symbol based on a fusion of the dynamic information received from the two sensor types. However, the user should have the option to override the integration and display the input from each sensor with two different symbols. It should also be possible to decide whether to use both sensors integrated, both sensors independently, only AIS input or only radar input. It should be clearly indicated if the user has chosen to override the default functionality, which is the integration (fusion).

In a VTS environment, the data fusion and presentation is important for the safety within the area. When erroneous information occurs, this might impact the operator’s ability to get a correct view of the situation. Typical situations are when certain on board sensors are malfunctioning, or the AIS mobile has been mis-configured. This could lead to potential dangerous situations where the Vessel reports a wrong position or heading. The information from a radar based tracking system is important as a confirmation of the AIS information received. Even if the different systems might not have the same coverage, a VTS should not rely solely on AIS for critical areas of the VTS.

## Data Validity

Implementation of AIS in several VTS has indicated instances of erroneous data received from AIS Mobile units. Erroneous data has been seen to have the correct checksums and transmits on the AIS channels. However, decoding of the data leads to useless information where the messages are proved to not follow the AIS standard.  This is an important issue as it implies caution has to be taken when using AIS data in a VTS environment.  The validity of AIS data received from ships is dependent on its proper installation, correctly interfaced and functioning ship’s equipment, and correct manual inputs of static and voyage-related data.

Until further regulations dictate what data shall be correct in the AIS mobile units, caution has to be taken when using AIS data for processing.

# Security

VTS may contribute AIS service data to other government agencies that focus on security issues.  This determination will be made by the competent authority.