To identify the importance to navigation safety of VHF-FM communication in the International Maritime Mobile IMM band, the shortage of available channels and the need to coordinate the use of the spectrum.

- Background

VHF will continue to be the primary means of ship-to-ship and ship-to-shore communication. AIS complements voice communications and, whilst it may help reduce the overall loading, the requirement for VHF voice communication will remain.

It is recognised that the number of channels available in the International Maritime Mobile (IMM )band is severely restricted. There is evidence that the effectiveness of VTS is being compromised by mutual interference.

- Current use and allocations

The current allocation for the use of specific channels is promulgated in ITU Radio Regulations [need reference IALA].  This gives a broad allocation of channels that are recommended for ship to ship and ship to shore communication. There is no specific allocation for VTS communication.

VHF voice communication is an important means of guaranteeing real time exchange of information with an immediate assurance of receipt. Many VTS centres will use VHF voice communications as a means to evaluate general marine competence and, specifically, competence in the official language of that VTS.

- Current / projected problems and consequences

A VTS centre may require to break down its area of responsibility into separate sectors. There may also be a need to establish additional VTS Areas to accommodate competitive offshore usage and the establishment of ship reporting systems or traffic separation schemes.  Each sector will need a dedicated frequency and, to ensure that all vessels in that sector are aware of the movements of others, there are benefits in using a simplex channel as the primary working frequency. There are a limited number of simplex channels available which give rise to the selection of the same frequency by VTS centres that are not significantly geographically displaced. This problem is compounded by the lack of channels dedicated to VTS and the limited overall number of channels available in the IMM as a whole. This gives rise to VTS channels being overloaded with other general communications. The potential consequence of this interference is that important information between a vessel and VTS may be missed at a critical moment which may impact on navigational safety, security and efficiency.

- Solutions and actions required

IALA perceives the need for the allocation of a sufficient number of specific channels dedicated to the use in VTS. It is recognised that there are limited options for achieving this in the short term save for the reallocation of duplex channel frequencies to achieve more simplex channels but this is unlikely to be accepted due to practical reasons.

It is understood that work is ongoing to achieve a more efficient use of the IMM band but pressure is growing to reallocate part of this spectrum to non-maritime users. COMSAR is requested to note the requirement for the allocation of a number of channels specifically dedicated to VTS in any review.