Chapter 10		VTS - Equipment

Introduction

Traffic density, navigation hazards, local climate, topography and extent of a VTS area sets the requirements to VTS equipment and these factors will have substantial impact on life-cycle costs of a VTS and acquisition of VTS equipment e.g. including:

- Communications

- VTS Radar System

- Automatic Identification System (AIS)

- Closed Circuit TV Cameras (CCTV)

- Hydrometeo Equipment

- VTS Data System

The required features and in particular the need for coverage by sensors, e.g. radar, should be determined by an assessment of the service to be provided, the safety level to be reached and the user requirements of the VTS system.  Subsequently suitable positions for the equipment should to be determined by site survey, analysis, simulations and/or site-tests to ensure, as far as practicable, that the required functions and coverage will be provided.

Detailed guidance is given by IALA recommendation V-119 on the Implementation of Vessel Traffic Services and V-128, Operational and Technical Performance Requirements for VTS Equipment.  V-128 is divided into three different levels of capabilities:

- Basic - applicable to VTS information service and, where applicable, navigational assistance service.

- Standard - applicable to all types of VTS as identified by IMO – information service, navigational assistance service and traffic organizational service – for areas with medium traffic density and/or without major navigational hazards.

- Advanced - applicable to VTS areas with high traffic density and/or specific major navigational hazards.

The individual VTS authority should decide on appropriate levels. Different levels may be used as applicable within a VTS.  E.g. part of a VTS area may call for a Basic capability and another part may call for a Standard capability. The essence of the previous edition of the VTS manual, restructured and updated on the basis of the recommendations in V-128 follows.

Communication

Communications in a VTS area are presently heavily biased towards voice systems, mostly using VHF and data.  This requires a complex combination of procedural and language skills, which must be thoroughly understood by all VTS Personnel.

A VTS Operator will need to communicate with shipping for which he has responsibility and may also need to communicate with other operators or allied services.  The ideal situation would be for the control of all communication equipment to be integrated into each VTS Operators console.

#### Communication with shipping in the VTS area

Normally the communication coverage of the VTS area will be achieved by VHF using telephony or data transmissions and will involve:

Communication between shore and participating vessels, using appropriate international VHF channels.  This communication can take place on simplex as well as duplex channels.  In the case of duplex channels, re-transmission from the shore may need to be carried out if the information received is of interest to other vessels to enable them to better comprehend the traffic situation.

Private channels may also be available for use between VTS centres and local service craft. However, in this case local service craft should always be able to communicate with shipping on the appropriate international VHF channel.

It is of the utmost importance that the VHF communication network of a VTS is set up properly, with high quality and reliable equipment and interconnection.  The number of VHF channels required for specific VTS usage should be assessed during the Project Definition phase and arranged with the Telecommunication Authorities well in advance of the Commissioning and Acceptance phase.  Often a VTS has, apart from the sub-area or sector frequencies, one or more general reporting or emergency frequencies.  The number of channels required is dependent upon the number of the various sub-areas and sectors that will be used and the overall traffic density. The VTS Authority should also seek permission from their Telecommunication to use VHF Channel 16 for emergencies and for calling ships which are not participating in the VTS.

Ship borne VHF equipment is normally capable of transmitting and receiving on the international channels 1-29 and 60-89 at full power (25W) and reduced power (1W).  Where practicable it is advisable to request low power transmission from ships.  Communications with vessels should be recorded, for accident investigation, disputes, etc.

Very High Frequency (VHF) radio communication

It is common for the VTS to have its own independent VHF network, for the use within specifically designated VHF Channels. This network may comprise of one or more VHF Channels in different sectors of the VTS Area. The VTS Authority may require specific VHF Channels to be designated by the National Radio Authority for specific types of operations. The VHF equipment must comply with national and international regulations.

Long Range Communication

In the case where a VTS Authority requires pre-arrival information, the normal maritime communication systems should be used and therefore an independent network is not required.

Communication with Allied Services

VTS Centres should be equipped with the ability to communicate with allied services by the use of reliable and secure communication networks. It was usual for a VTS Centre to be equipped with a point to point line with MRCCs for this purpose. However, point to point lines have been found to be inadequate and unnecessarily expensive to establish and maintain. Therefore it is recommended that VTS Centres should be equipped with a digital switched network, with caller identification.

Other Communication

The following general types of communication should be available within a VTS:

- between VTS operators in the same VTS centre

- where appropriate, between VTS operators in different VTS centres belonging to the same VTS;

- between VTS operators of any adjacent VTS;

- with vessels about to enter a VTS area

- with Pilotage, tugboats, SAR, Port Authorities and other Authorities.

These types of communication can be performed by using the following equipment:

- public telephone lines

- private telephone lines

- telex

- facsimile

- radio telephony and microwave links

- automated data transfer systems.

Where there is a need for communication between a VTS and a shore authority to have a high degree of operational importance, consideration should be given to providing direct telephone lines to increase the reliability and decrease the time of the connection.

The internal communication within a VTS is of the utmost importance.  If there is more than one VTS Centre it may be necessary for the operators in one VTS centre to communicate easily with operators in other centres so that specific traffic information can be passed from one operator to another with the minimum of delay.

VTS RADAR SYSTEM

Depending on the services that a VTS is to carry out the radar coverage can be:

- nil (automatic identification systems, voice communication and reporting only)

- partly (covered areas chosen intentionally with some blind sectors)

- totally by one radar sensor (without any blind sectors)

- totally by two or more radar sensors (for large VTS areas and to cover for shadow effects of other vessels). Stereographic processing of images from 2 or more radars may also be utilised for elimination of false (ghost) echoes.

In principle VTS radars typically function like ships radars, but they will I most cases need to operate simultaneously on short and long range, preferably without the need for operator adjustments. Weather related phenomena such as sea clutter and ducting will furthermore influence shore based radars more than ships’ radars.

In addition new challenges have developed over recent years, including:

- Increased requirements as introduction of new technologies, especially AIS further develops VTS, and the presentation of radar information needs to be sufficiently accurate to avoid ambiguity. In addition antenna side lobes and ghost targets (multiple reflections) may lead to false and dangerous results when radar returns and AIS plots are associated.  High precision, low sidelobe antennas and careful location of VTS radars is therefore required to allow for unambiguous correlation of position obtained from the two information sources.

- The influence from off-shore wind farms. VTS radars will normally not be dependent on Doppler shift and they are therefore not affected by the rotation of wind turbines, but the large towers will reflect radar signals resulting in false echoes, often moving with ships movements even far away from wind farms and possibly impossible to distinguish from ships. The symmetrical lay out of off-shore wind farms add to the problem.

- Increasing demands to see small targets in rough weather, if objectives include detection of targets for security purposes.

- Requirements to reduce spurious / out of band emissions

This leads to dynamic and resolution requirements that far exceed those required in other onboard ships, reduced spurious/out of band emissions, better sea clutter suppression and, in most parts of the world, better rain clutter suppression.

Radar functions

When radar is required, it should be able to detect and track, for subsequent display, all specified moving or stationary targets which satisfy the detection criteria within specified coverage areas and during all specified operating circumstances.

Each radar should be equipped to reduce the adverse effects of rain and sea clutter and enhance the probability of target detection. The radar should also be designed and installed so as to eliminate to the maximum extent possible, false echoes caused by side lobes or reflections from nearby structures.

The VTS Radar System should furthermore assist in the development of the traffic image, by performing the functions shown in Table 1.

Table 1 Radar Functions

| Parameters / Capability | Basic | Standard | Advanced |
| --- | --- | --- | --- |
| Path, time and track prediction, |  |  | X |
| CPA, | X | X | X |
| TCPA, | X | X | X |
| Anchor watch, |  |  | X |
| Vessels vector, | X | X | X |
| Course, speed and label/identity, | X | X | X |
| Collision alerts. | X | X | X |

Radar frequencies selected for VTS lie typically within the S-band and X-band frequencies, however, higher frequencies such as Ku band may also be utilised. The majority of VTS services use X-band radars as a best compromise, especially since technologies for rain clutter suppression have matured.  Also, as a result of production volume, X-band radars are the least expensive.

The second most used frequency is S-band, due to better weather penetration in heavy rainfall. A radar operating in the S-band is typically needed if precipitation rates frequently is greater than 25mm/h and required detection distance exceed few nautical miles.

Frequency band allocation is granted by the ITU, whereas permissions to transmit on given frequencies are granted on a national basis.

#### Characteristics of Radar Targets.

The characteristics of VTS radar targets are defined by its height above sea level, its radar cross-section (RCS) and its fluctuations in RCS.

Formally, the RCS is defined as the ratio between the power [in W] scattered by the target back towards the radar receiver and the power density [in W/m2] hitting the target.  Thus RCS is measured in m2 and has the dimension of an area.  There is, however, no simple relation between the physical area of the target and the RCS, as the reflected power depends on the angle of incidence as well as target properties such as material and physical shape.

Targets may fluctuate severely in RCS and the returned energy is highly dependant on propagation conditions. Table 2 provides recommended data to be used when defining requirements to a VTS.

Table 2 Radar target characteristics

| TARGET |  | Type of Capability |  |  | Design Requirements |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Basic | Standard | Advanced | Radar cross section |  | Height of Target |
|  |  |  |  |  | S-band | X-band |  |
| 1 | Aids to Navigation etc. –without radar reflector. Small open boats, fibreglass, wood or rubber with outboard motor and at least 4 meters long, small speedboats, small fishing vessels, small sailing boats and the like. |  |  | X |  | 1 m2 | 1 m ASL |
| 2 | Inshore fishing vessels, sailing boats, speedboats and the like. |  |  | X |  | 3 m2 | 2 m ASL |
| 3 | Aids to Navigation with radar reflector. |  | X | X | 4 m2 | 10 m2 | 3 m ASL |
| 4 | Small metal ships, fishing vessels, patrol vessels and the like. | X | X | X | 40 m2 | 100 m2 | 5 m ASL |
| 5 | Coasters and the like. | X | X | X | 400 m2 | 1,000 m2 | 8 m ASL |
| 6 | Large coasters, bulk carriers, cargo ships and the like. | X | X | X | 4,000 m2 | 10,000 m2 | 12 m ASL |
| 7 | Container carriers, tankers etc. | X | X | X | 40,000 m2 | 100,000 m2 | 18 m ASL |

#### Detection Performance

Table 3 and Table 4 provide examples of calculated range performance typical for radar equipment suitable for the three recommendation levels. The calculations in this table were performed using the software programme CARPET.

Table 3  Typical range performance, X-band

| Antenna elevation | Target type | Modelled as fluctuating point target |  | Detection and tracking ranges for standard atmosphere and rain/sea state as indicated |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RCS | Height | Basic recommendation |  |  | Standard recommendation |  | Advanced recommendation |  |
|  |  |  |  | Clear | 2 mm/h rain |  | Clear | 4 mm/h rain | Clear | 10 mm/h rain |
| 20 m ASL | 1 | 1 m2 | 1 m ASL | N/A |  |  | N/A |  | 5 NM | NIL |
|  |  |  |  |  |  |  |  |  | Up to sea state 4 |  |
|  | 2 | 3 m2 | 2 m ASL | N/A |  |  | 7 NM | 4NM | 7 NM | 6 NM |
|  |  |  |  |  |  |  | Up to sea state 3 |  | Up to sea state 5 |  |
|  | 3 | 10 m2 | 3 m ASL | 7 NM |  | 4 NM | 8 NM | 5NM | 9 NM | 7 NM |
|  |  |  |  | Up to sea state 3 |  |  | Up to sea state 4 |  | Up to sea state 6 |  |
|  | 4 | 100 m2 | 5 m ASL | 9 NM |  | 8 NM | 11 NM | 9NM | 12 NM | 10 NM |
|  |  |  |  | Up to sea state 4 |  |  | Up to sea state 5 |  | Up to sea state 7 |  |
|  | 5 | 1000 m2 | 8 m ASL | 12 NM |  | 10 NM | 13 NM | 11 NM | 14 NM | 13 NM |
|  |  |  |  | Up to sea state 5 |  |  | Up to sea state 6 |  | Up to sea state 8 |  |
| 50 m ASL | 1 | 1 m2 | 1 m ASL | N/A |  |  | N/A |  | 10 NM | NIL |
|  |  |  |  |  |  |  |  |  | Up to sea state 4 |  |
|  | 2 | 3 m2 | 2 m ASL | N/A |  |  | 10 NM | 7 NM | 12 NM | 9 NM |
|  |  |  |  |  |  |  | Up to sea state 3 |  | Up to sea state 5 |  |
|  | 3 | 10 m2 | 3 m ASL | 10 NM |  | 6 NM | 12 NM | 8 NM | 14 NM | 12 NM |
|  |  |  |  | Up to sea state 3 |  |  | Up to sea state 4 |  | Up to sea state 7 |  |
|  | 4 | 100 m2 | 5 m ASL | 13 NM |  | 12 NM | 15 NM | 13 NM | 17 NM | 15 NM |
|  |  |  |  | Up to sea state 4 |  |  | Up to sea state 5 |  | Up to sea state 7 |  |
|  | 5 | 1000 m2 | 8 m ASL | 16 NM |  | 15 NM | 18 NM | 17 NM | 20 NM | 18 NM |
|  |  |  |  | Up to sea state 5 |  |  | Up to sea state 6 |  | Up to sea state 8 |  |
| 100 m ASL | 1 | 1 m2 | 1 m ASL | N/A |  |  | N/A |  | 12 NM | NIL |
|  |  |  |  |  |  |  |  |  | Up to sea state 4 |  |
|  | 2 | 3 m2 | 2 m ASL |  |  |  | 13 NM | 5 NM | 16 NM | 10 NM |
|  |  |  |  |  |  |  | Up to sea state 3 |  | Up to sea state 5 |  |
|  | 3 | 10 m2 | 3 m ASL |  |  |  | 17 NM | 10 NM | 18 NM | 16 NM |
|  |  |  |  |  |  |  | Up to sea state 4 |  | Up to sea state 7 |  |
|  | 4 | 100 m2 | 5 m ASL |  |  |  | 20 NM | 19 NM | 22 NM | 20 NM |
|  |  |  |  |  |  |  | Up to sea state 5 |  | Up to sea state 7 |  |
|  | 5 | 1000 m2 | 8 m ASL |  |  |  | 23 NM | 22 NM | 25 NM | 23 NM |
|  |  |  |  |  |  |  | Up to sea state 6 |  | Up to sea state 8 |  |

Table 4  Typical range performance, S-band

| Antenna elevation | Target type | Modelled as fluctuating point target |  | Detection and tracking ranges for standard atmosphere and rain/sea state as indicated |  |
| --- | --- | --- | --- | --- | --- |
|  |  | RCS | Height | Standard recommendation |  |
|  |  |  |  | Clear | 16 mm/h rain |
| 20 m ASL | 3 | 4 m2 | 3 m ASL | 4 NM | 3 NM |
|  |  |  |  | Up to sea state 4 |  |
|  | 4 | 40 m2 | 5 m ASL | 7 NM | 5 NM |
|  |  |  |  | Up to sea state 5 |  |
|  | 5 | 400 m2 | 8 m ASL | 10 NM | 8 NM |
|  |  |  |  | Up to sea state 6 |  |
| 50 m ASL | 3 | 4 m2 | 3 m ASL | 7 NM | 4 NM |
|  |  |  |  | Up to sea state 4 |  |
|  | 4 | 40 m2 | 5 m ASL | 11 NM | 8 NM |
|  |  |  |  | Up to sea state 5 |  |
|  | 5 | 400 m2 | 8 m ASL | 14 NM | 13 NM |
|  |  |  |  | Up to sea state 6 |  |
| 100 m ASL | 3 | 4 m2 | 3 m ASL | 10 NM | NIL |
|  |  |  |  | Up to sea state 4 |  |
|  | 4 | 40 m2 | 5 m ASL | 14 NM | 12 NM |
|  |  |  |  | Up to sea state 5 |  |
|  | 5 | 400 m2 | 8 m ASL | 18 NM | 19 NM |
|  |  |  |  | Up to sea state 6 |  |

For detailed analysis, the recommended method for determination of radar coverage and range performance is a combination of site inspections and radar system performance calculations, made by experts with a sound operational and technical knowledge about the subject.

Calculation of performance should be focussed on the smallest targets of interest in poor weather conditions. All applicable losses should be included in the calculations. The probability of detection and false alarm rates used should comply with that required to meet the performance required for the individual VTS.

#### Propagation conditions

Performance should, in all cases, be evaluated assuming standard atmospheric conditions. In addition, for each individual VTS the influence from adverse propagation effects should be analysed in detail for areas of the world having tropical climate and dry and hot climate.

Ducting may occur almost anywhere, and all systems should be designed to eliminate adverse effects from this. For most parts of the world evaporation ducting tends to persist most of the time, giving extended range, especially for low mounted antennas.  The effect will give average improvement in detection performance and may therefore be very useful in respect to security applications, if required.  The effect is usually not stable enough to be calculated as a benefit in safety applications.

#### Accuracy and target discrimination

Accuracy as well as range and bearing resolution/precision is necessary in order to have a clear and distinct appreciation of the movement of vessels, including those which are at anchor.   Consult V128 for further guidance.

#### Tracking

Prevision of target tracking, where computers automatically follow radar plots and provide information in synthetic form, is done by a plot extraction process followed by an automatic tracking process.

Plot extraction should be automatic in the entire VTS area covered by radar. Track initiation should be automatic, except in selected areas or manual depending on the concept of operations.

In automatic track initiation modes, all plots in a scan should be considered potential targets.  Some of the plots will be associated with previously established tracks, while the remaining plots should be considered as candidates for new tracks, i.e. tentative tracks.

Tentative tracks will become confirmed tracks if plots from consecutive scans “fit into the picture” within reasonable physical manoeuvrability limits, otherwise the tentative tracks are discarded.

The tracking system should be able to handle at least a certain number of tentative tracks and to initiate tracks and eventually to confirm tracks under certain conditions of Pd (probability of detection) and Pfa (probability of false alarm).

It should also be possible to initiate a track manually.  In manual track initiation a plot on the radar display is selected by the operator using a graphical tool.  When selected this plot should form the starting point for a tentative track which eventually should be confirmed or discarded, as in the case of automatic initiation.

If automatically or manually created tentative tracks persist over a certain length of time the tracks should be promoted to confirmed tracks.  Confirmed tracks should be shown on the display.  The tracking system should be able to handle at least a certain number of confirmed tracks as recommended by V-128.

If a confirmed track either moves outside a user defined maximum range, into a user defined non-tracking area, if the quality of the track falls below a predefined minimum, or if the track cannot be updated with new plots over a certain length of time, then the track should be terminated.  In certain cases the operator should receive a warning as defined by the VTS Authority.

False tracks may appear as a result of noise, clutter (including wakes) and ghost echoes.  The maximum number of false tracks allowed is dependant on role of the VTS, however, false tracks should be avoided in safety critical areas.

There is a trade-off between the time for confirmation of tentative track and the number of false tracks.  A longer confirmation time implies less false tracks and it should be possible to balance this trade-off in the setup of the VTS.

Track loss may occur as a result of Pd < 1 in combination with targets manoeuvring, especially in the vicinity of obstructions such as bridges. A level generally accepted is that each operator should correct up to one track loss per hour

Swapping of track identity may occur as a result of targets moving close together or even merging for a period of time, especially if targets are overtaking with small difference in speed and course. A simple method of manual correction should be employed. In the case of AIS information being available for the radar track(s) in question, automatic correction should be performed. The problem may also be addressed by implementing operational procedures to separate targets or to prevent overtaking in critical areas.

The VTS authority should address critical areas, such as the vicinity of bridges, and explain expectations to tracking to allow VTS suppliers to make solutions accordingly.

AUTOMATIC IDENTIFICATION SYSTEM (AIS)

AIS should be integrated into the VTS. However, the VTS Operator should have the ability to choose whether to display the information and whether  targets are to be displayed using the AIS derived data only or from the radar display only or from both, in accordance with the requirements of the VTS Authority

Any good suggestions for further descriptions?

Radio Direction Finder (RDF)

A number of VTS Authorities require RDF receivers to receive information on the identity of the vessels.

In order to ensure accurate identification the use of two or more separate RDF bearing stations are required.

All bearings should be automatically displayed on the chosen screen when the signal has been received after a delay of no more than 3 seconds. The bearings should remain visible on the chosen screen as long as the vessel is transmitting a signal.

It is foreseen that this requirement may decrease as AIS usage by VTS Authorities increases, but will not become obsolete as, depending on national requirements, some vessels may not be equipped with AIS.

RDF is not suitable of being used for continuous tracking.

HYDROMETEO EQUIPMENT

It is essential that a VTS Centre has access to local Hydrometeo information relevant to the VTS Area(s) and can, if required by the VTS Authority, disseminate this to their users and allied services.

Where a VTS Authority determines a need to establish their own monitoring stations, the individual VTS Authorities should determine the accuracy and availability requirements for each VTS Centre..

V-128 gives an indication of typical minimum accuracy and availability requirements.

Closed circuit TV (CCTV) CAMERAS

Any good suggestions for a description?

VTS DATA SYSTEM

A VTS Data System should be flexible and easily up-datable alongside of the routine operations of the VTS Centre without the need for interrupting these VTS operations.

#### VTS Operator positions

A console should be provided at every VTS operator position with the equipment integrated in the most ergonomic arrangement.  The illumination of the operator position should be such that all relevant equipment such as the VTS monitor, communication equipment, target analyser etc can be monitored effectively and administrative tasks can be carried out at the same time.  Daylight displays for radar equipment have the advantage that they can be operated efficiently in normal room light levels.

Care should be when choosing the site for the consoles that VTS operator positions do not interfere with each other especially regarding sound.

In addition to the sensors and the communication equipment, access to data base information is necessary at a VTS operator’s position for reference purposes and for providing information to shipping.

Work Environment

Air supply

Lighting

Workstations

Equipment spaces

Fire/lightning protection

EMC

Security

#### Gathering and Recording of information

The period of time for which the information gathered by a VTS is required to be stored, should be identified in internal procedures. This time period should be such that it allows for the full retrieval of data post-incident/accident, in compliance with national requirements and those of the incident/accident investigation procedures of the VTS authority and other interested parties. This type of information may include:

- Communications, internal and external.

- Sensor data, i.e. data used to generate the traffic image such as radar, CCTV, AIS.

- Shipping information data, i.e. vessel and cargo data, including vessel movement information.

- Meteorological and hydrological data; and

- Data from other sources if relevant.

Provision should be made for the storage, security, retrieval and presentation of this information.

The procedures which should be applied to the custody and operation of the recorders are as follows:

- Radar and other electronic displays of the VTS traffic image should be recorded continuously;

- Tapes or discs should be changed daily;

- Recorded tapes or discs should be numbered, dated and placed in locked storage, the control and access to which should be under the control of the VTS;

- All recorded tapes or discs should be kept for a minimum of 30 days;

Tapes or discs should be retained by the VTS whenever an investigation or judicial inquiry has been ordered or is anticipated.

#### Information Management

Database management systems

Standards, Accuracy, Quality.

#### Display

A VTS display mapping graphics, analytical graphics and overlay information should be provided to enable a VTS Operator to have a concise picture of the geographical features, waterways and navigational lanes.  When this information is being displayed, in many cases the radar video echoes of the coastline should be suppressed beyond the coastline, making this area of the display available for other synthetic information.  Definitions of the requirements and accuracy of all the graphic possibilities need to be developed during the Project Definition phase.

If VHF direction finders are included in the system, the bearing lines should be shown on the VTS display, with the possibility for the operator to switch them off.

#### HMI, Symbology

#### List of Participating Vessels

The list of participating vessels should include static information and dynamic information concerning the vessel, for example Vessels Name, Call Sign, IMO Number, MMSI, ETA, ETD, Course, Speed and Position, if required.

An example of a typical list is shown below. This example is taken from the Baltic AIS Network

#### Pilot aids - carryaboard equipment

#### Operational conditions, Redundancy and emergency precautions

#### The operational conditions for VTS equipment will vary from one place to another.  In some countries the availability and reliability requirements need to take into account extremes of temperature or precipitation, in others special attention may need to be paid to wind force (including gusts) or earthquake resistance. The maintenance and availability of remote sensors should be given special consideration and adequate redundancy should be provided.

#### To ensure adequate availability and reliability, vital parts of the VTS, e.g. all operational VTS communication services should have back-up systems with a power source independent from the normal power supply.

#### The need for redundant sensors and possibly an alternative site for the VTS Centre should be considered in accordance with the guidelines provided by IALA Recommendation. V-119.

#### When determining the position and range of radar equipment, the possibility of malfunction of a radar should be taken into account, and where practicable the arrangements should enable another radar, or AIS where appropriate, to provide cover for the sub-area or sector affected.

#### In the case of an alternative site for the VTS centre the operation should be capable of being easily transferred to the alternative location, in the event of an emergency or maintenance situation resulting in temporary closure of the primary VTS Centre

#### One or more spare VTS operator console should also be considered to:

- Substitute for any console that is unserviceable

- Be used for On-the-Job training

- Where a need exists, be used by a VTS supervisor

- Where appropriate, be used in the co-ordination of emergencies

Finally, when computers are used to establish and maintain the VTS traffic image the VTS display may be dependent completely on the information supplied by the target analysis computer and only synthetic images of the traffic are displayed.  In the case of a malfunction of this major sub-system the information available on the VTS display is not likely to be of use.  Therefore a combination of radar video and information supplied by a target analysis computer is recommended. A combination display will also provide the operator with means to verify track positions and the experienced operator will often be able to classify the individual target on the basis of the radar video.

### Availability and reliability of equipment

The equipment performance parameters are strongly dependant upon the services to be provided which influences the Mean Time Between Failures (MTBF) and the availability of the service.  Information on reliability methods is given in the IALA Guide to the Reliability and Availability of Aids to Navigation.