DRAFT

IALA Recommendation on Generic Operational Procedures for Vessel Traffic Services July 2003

THE COUNCIL

NOTING the function of the Council with respect to Safety of Navigation, the efficiency of maritime traffic flow and the protection of the environment,

NOTING ALSO the International Maritime Organization resolution A.857(20) on

Guidelines for Vessel Traffic Services (VTS), in particular paragraph 2.5.3,

NOTING FURTHER:

.1 the work program of the VTS Committee, in particular the item Generic Operational Procedure for VTS, including the use of Standard Marine Communication Phrases,

.2 the IALA VTS Manual, in particular paragraph 6.1

.3 the IALA Guidelines on VTS Operating Procedures

HAVING CONSIDERED the proposals made by the VTS Committee:

ADOPTS the guidance on Generic Operational Procedures as set out in the annex

to this recommendation; and

RECOMMENDS:

.1 National Members and other appropriate Authorities providing, or intending

to provide, vessel traffic services use the guidance as set forth in the annex;

.2 the guidance as set forth in the annex be promulgated to IALA Members.

# Annex: Generic Operational Procedures for Vessel Traffic Services

# Introduction

# The purpose of a VTS is to improve the maritime safety and efficiency of navigation, safety of life at sea and the protection of the marine environment and/or the adjacent shore area, work sites and offshore installations from the possible adverse effects of marine traffic in a given area. The VTS may also, to a lesser or greater extent have a role to play in maritime security issues.

In order to meet its purpose operational objectives are laid down, and tasks and activities that must be performed, in order to meet these objectives, must be identified. The nature of the tasks and activities to be performed will depend on the type of VTS, the area of its jurisdiction, the services to be provided and the competence of the VTS Authority. In general these tasks and activities all involve the collecting, processing and dissemination of information. The collecting and dissemination of this information will involve both internal and external communications, while the processing of the information will be carried out within the VTS Centre itself and will therefore only involve internal communications. The VTS Authority should therefore decide on the level of decision making that can be taken within the VTS Centre before reference is made by the VTS Centre to either an outside organisation, to the VTS Authority itself and/ or the Competent VTS Authority/ national government.

Successful pursuit of operational objectives is dependent upon the co-operation and trust among service users, VTS personnel and allied services. This can only be achieved through the reliability of the VTS information and is based on the assured availability, continuity and quality of the declared type and level of service provided to all the stakeholders.

To ensure the effective and efficient execution of all the identified tasks and activities to be performed by the VTS, operational procedures should be laid down, with which VTS personnel should normally comply. It is therefore recommended that all such procedures for the VTS Centre be retained within a single Manual. However, as it may be decided, due to local circumstances, to give the VTS Centre secondary tasks to perform for the area, which may give rise to a different set of procedures, this single manual may not be restricted to just VTS operations but encompass all the procedures which fall within the safety and quality management system of the organisation as a whole. Again, depending on the local circumstances, there may be a requirement to hold contingency planning procedures that are required to be acted upon in the event of an emergency, within a separate Manual. Whilst it is not considered feasible for IALA to develop a comprehensive set of specific operating procedures that would apply to all VTSs, the following sections provide guidance to VTS Authorities upon the tasks and activities that are considered likely to be encountered within a generic VTS. In addition, the Annexes to this Recommendation contain some generic procedures that may be applicable to individual VTS Centres. These have been compiled from Manuals held by VTS Centres situated within the countries of a number of IALA members and are included, for reference purposes, as examples of best practice.

General

Where operating procedures are concerned, a distinction should be made between internal and external procedures. Internal procedures address operating instruments, interactions among the staff and the internal routing and distribution of data. External procedures govern the interactions with users and allied services. A further distinction should be made for procedures governing the daily routine and procedures governing contingency planning such as search and rescue and environmental protection activities. All operational procedures, routine or contingency, should be laid down in handbooks or manuals and be an integral part of regular training exercises. Adherence to procedures should be monitored. An overview of relevant internal and external operational procedures is contained at Annexes 1 and 2, respectively.

Any Manual incorporating the operating procedures for the VTS Centre should include sections containing:

- Definitions of the terms used;

- General information, giving details of the purpose and objectives of the VTS and its general duties. For example, the general duties of the VTS Centre covering such topics as:

- Maintenance of an information service to ensure that essential information becomes available in time for on-board navigational decision-making.

- Maintenance of a navigational assistance service to assist on-board navigational decision-making and to monitor its effects.

- Maintenance of a traffic organisation service to prevent the development of dangerous maritime traffic situations and to provide for the safe and efficient movement of vessel traffic within the VTS area.

- Co-ordination and facilitation of the supply of pilots to vessels.

- Provision of necessary information to allow a passage plan to be formulated for the vessel.

- Provision of information to ships outside the VTS’s Area, either if requested or if it is likely to be beneficial to navigational safety.

- Monitoring shipping movements, tidal and weather conditions within the VTS Area and to provide timely information concerning possible hazardous situations. Such timely information may under certain circumstances be upgraded to a Warning if the VTS Centre has sound reason to believe that such a warning may assist the master or pilot of a vessel to avert a hazardous situation.

- Acting as coordinating authority in the event of an incident in the VTS Authority’s jurisdiction that requires the involvement of other organisations, services or authorities.

- Alerting senior staff within the VTS Authority to any situation, which may require the implementation of the Major Incident Procedure; carrying out initial alerting and staff call-out role and assisting the designated person with the command and control of the incident.

- Acting as the VTS Authority’s central point of contact outside normal office hours for all incoming inquiries or information, and for the provision of essential advice and information to the shipping and port community.

- The reviewing and updating of all Operational procedures in force within the VTS Centre.

- The legal basis for the operation of the VTS, including extracts of all relevant international, national and local legislation;

- The various responsibilities of all the staff within the VTS Centre, which should be clearly laid down; and

- Details of the staff chain of command within the Centre and how this links into other VTS Centres under the responsibility of the Competent VTS Authority. Where the Competent VTS Authority has additional responsibilities this information is likely to encompass all staff within the organization and detail where the VTS Authority fits into the organization as a whole. A sample organisation diagram is shown in figure 1 below.

VTS Authority

VTS Centre

VTS Centre

VTS Centre

VTS Manager

VTS Manager

VTS Manager

VTS Supervisor

Sub-Area 1

VTS Supervisor

Sub-Area 2

VTS Supervisor

Sub-Area 3

Administrative staff

VTS Operator

Sub-Area 2

VTS Operator

Sub-Area 2

VTS Operator

Sub-Area 2

Trainee VTS Operator

Figure 1: VTS Authority organisational structure

The general information section may also include details that are of benefit to the day to day running of the VTS Centre, such as:

- Area of operation of the VTS Centre, including boundaries of any sectors/ sub-areas.

- The type of VTS i.e. a VTS should be divided into the following types:

  - offshore	-	international waters

  - coastal	-	in territorial waters but covering

traffic transiting a sea area

  - approach-	in esturial or river situations

  - port 	-	entry into specific ports

- The level of service provided i.e.

  - Information Service

  - Navigational Assistance Service

  - Traffic Organisation Service

- Under what circumstances the service is provided, particularly in relation to the issuing of assistance/advice to shipping within the VTS area. For instance the VTS Operator may have the authority to give advice of a routine nature, e.g. ship to wait alongside, slow down, etc. at any time, as part of their normal duties, but they may be required to seek guidance before giving more detailed assistance/advice where the situation is more complicated or serious. Neither a VTS passage plan, nor requested or agreed changes to the passage plan can supersede the decisions of the master concerning the actual navigation and manoeuvring of the vessel because under international law the Master is and remains in charge of the vessel even with a pilot on board. However, the VTS Operator also has a duty of care. If he/she notices a potentially hazardous situation developing, he/she may assist with timely information which the master or pilot may wish to utilise.  It must be borne in mind, that such information should be result orientated, and given in good time. If unavoidably given at a late stage, a response from the vessel may not be expected. In a case where action is clearly necessary e.g. because a vessel is in the wrong channel, deliberate inactivity may be considered to be as negligent as a wrong advice. Even if a vessel, which strayed off course, was in no danger and posed no threat to other vessels, the VTS Centre could not automatically ignore that it was in the wrong position.  The VTS Centre would have to have regard to the obligation to keep the port/ channel open.  If one assumes that designed channels serve a purpose, a vessel that is in the wrong place could, clearly, cause difficulties thereafter in terms of other vessels using that port/channel. Reports of the action taken and the outcome may be required, for any assistance/advice given, where these are of an urgent or serious nature.

The Manual should then go on to encompass the relevant internal and external procedures specific to the VTS Centre. For ease of reading in this recommendation examples of the scope of the types of procedures to be covered in any such manual have been sub-divided into Routine Procedures and Special Situation Procedures, as a number of the examples contained in the annexes to this Recommendation cover both internal and external communication aspects.

Routine procedures:

- Manning of the VTS Centre as a whole and any specific requirements for the sectors/ sub-areas. It should be noted that IALA are currently developing Guidelines on VTS Operator Workload Staffing Levels.

- VTS Watchkeeping covering duties, responsibilities, professional conduct, performance standards, relief cover and handover procedures.

- Verbal Communications.

  - Official language(s) for communications with the VTS, clearly stating if there are any exceptions to this for certain areas/sectors of the VTS.

  - Frequencies to be used: Providing information on the frequencies to be used for different types of communications within the VTS Area and its approaches and under what circumstances; requirements for listening watches to be maintained; call sign to be used for all external communications; messages to be relayed from another station which may be of general or particular interest; Navigational Information Broadcasts; repeat messages/ broadcasts.

  - Conduct: External Communication between a VTS Authority and a participating vessel should be conducted in accordance with the Guidelines and Criteria for Ship Reporting systems and should be limited to information essential to achieve the objectives of the VTS. All VHF radio messages should therefore be factual, as brief as possible and given in a professional manner in accordance with standard radio communications procedure and the IMO Standard Marine Vocabulary.   MMSC 67/22/Add.2 Annex 20 2.4.1 states “Communications between a VTS Authority and a Participating vessel should be conducted in accordance with the Guidelines and Criteria for VTS.  IMO standard Marine Communication Phrases should be used where practicable.” Draft IMO Standard Communication Phrases, (MSC/Circ.794) stipulates the use of 8 Message Markers. These are, INSTRUCTION, ADVICE, WARNING, INFORMATION, QUESTION, ANSWER, REQUEST, INTENTION. It is therefore recommended that all VTS Centres use VHF communications strictly in accordance with these message markers. Examples of typical communications are contained in Annex 3. VTS procedures should therefore clearly all stipulate what communications are required and which frequencies are to be monitored.

  - Responsibility: As a general rule, the VTS Operator/Supervisor will be directly responsible for all communications concerned with traffic management.

- Written communications. The use of standard templates for all written communications from the VTS Centre is desirable, be these transmitted electronically by fax or e-mail or by letter. For example, a VTS Centre may be empowered to issue operational notices on their own authority and relay them to vessels and shore side facilities, dispatch offices and other organizations.

- Participating vessels. Procedures should clearly indicate which vessels are required to participate and those whose participation is voluntary unless required to do so by the VTS Authority. Specific procedures for different classes of vessel/cargo may be put in place, for instance:

  - Tankers

  - LPG & Explosive Vessels

  - Ultra Large Vessels

  - Vessels Over “X” Metres

  - High Speed Craft

  - Fishing Vessels

      - Environmental Protection. Environmental protection procedures covering, for example, bunkering, waste management, dangerous goods notification, use of the information and tracking of vessels with nominated hazardous cargoes. An example of a procedure for tracking of vessels with nominated hazardous cargoes is attached at Annex 4.

- VTS Equipment operating procedures and equipment checks. An example of a generic procedure for use of surveillance equipment and Equipment checks are at Annexes 5 and 6.

- Provision of information to a pilot about the vessel to which he/she is being assigned in order for the pilot to be able to prepare a passage plan.

- Prioritisation and co-ordination of vessel movements and the availability of tugs.

    - Reporting requirements for vessels including any special reporting requirements for certain types of vessel and/or specific destinations within the VTS area and vessels on passage through the VTS Area. For example, when the traffic participant is a pilotage exemption certificate holder the VTS Authority may require a report that circumstances on board the vessel are in accordance with the rules and restrictions laid down in the exemption.

    - Information required to be given to departing/ arriving vessels. It is recommended that this information be given in a set order, as laid down by the VTS authority. Such information may include details of the situation at the berth, number of tugs ordered and availability, movements within the VTS Area, turn position for mooring gang, tidal information etc.

    - Specific information required to be given to vessels on passage through the VTS Area, such as traffic movements affecting their passage and any limitations of vessels in their vicinity with restrictions on ability to manoeuvre freely due to draft restriction, etc.

    - Content, format and frequency of reduced visibility information broadcasts.

    - Content, format and frequency of navigational information broadcasts.

    - Content, format and frequency of emergency broadcasts.

      - Use of Traffic Separation Schemes, Deep Water Routes, Anchorages etc within the VTS Area; and permitted variations to their use. An example of a procedure permitting variation to the use of an anchorage is attached at Annex 7.

      - Given the role of VTS in the provision of safety and efficiency services to shipping and in the protection of the environment, the need to avoid confusion on the part of users travelling from one VTS to another and the importance of professionalism on the part of VTS operators in determining the extent of trust placed in the functioning and effectiveness of a VTS, it is essential that VTS personnel be adequately qualified and trained to carry out their functions. Procedures covering training, which incorporate the IALA Recommendation V-103 on Standards for Training and Certification of VTS Personnel and its annex and associated model courses should be drawn up and implemented.

      - VTS Information retrieval.  In order for the VTS to function effectively through the use of traffic management measures, information must flow into, as well as out of, the traffic centre.  Just as most of the information that flows from the centre is transmitted via radio as advisories, most of the information flowing into the centre is also received by radio, as vessel reports.  There are however, other sources of information that are nearly as vital to the operation of the centre as the vessel reports.  Marine Exchange vessel arrival and departure reports, weather and visibility reports, pilot lists and COTP/HM message traffic are just as essential to operations.  VTSs must develop and maintain an efficient and effective method for gathering and dispensing information necessary for VTS operations. If necessary for the operation of the service, a database with the capacity to retain, update, supplement and retrieve data once collected should be accurately maintained. Any data retained in a system for further use should be made available only on a selective and secure basis. However, due account must be taken of relevant national legislation, such as Freedom of Information and Data Protection. Procedures should therefore be in place governing the following aspects:

- Keeping of accurate and comprehensive records of all commercial vessel movements, pilotage acts and other activities, which will be made available to the VTS Authority, including vessel details, any limitations which the vessel may have, such as structure, deficiencies, etc., weather conditions, tug availability, reason for any previous refusal of the vessel by the VTS Centre.

- Keeping of accurate and comprehensive records of any new information relevant to the safe passage of the vessel either through the VTS area or to the berth, such as changes/ casualties to aids to navigation, berth/ bollard number, distance from next ship, availability of berth/ mooring gang and weather conditions at berth.

- Keeping of accurate and comprehensive records of the status of vessels, such as ETA, whether berthing on arrival, ETD.

- Keeping of and completing an authentic record of all vessel movements, ship/ shore and ship/ ship communications etc. See also Special Situations below.

Special situations including emergencies:

There are instances when routine procedures for handling vessel traffic situations will not suffice and special procedures must be implemented.  While not all situations can be addressed here, some common examples of non-routine traffic management issues and procedures for handling them are identified below:

- Request for Navigational Assistance.  The term "Navigation Assistance" is a service in which the VTS contributes to the navigational decision-making process on board the vessel and monitors its effects.  It is provided by the VTS at the request of the vessel and is most common in cases of defects or deficiencies on the vessel or during difficult navigational or meteorological circumstances.  Navigational assistance shall only be given if positive identification of the vessel and uninterruptible communications have been established and can be maintained throughout the process.  The assistance may include information or advice such as course and speed made good, position relative to fairway axis, waypoints, landmarks or other vessels.

- Captain of the Port (COTP)/ Harbour Master’s(HM) Orders.  The nature of its mission and capabilities make VTS a natural management tool for COTP/ HM to relay orders to vessels.  In addition to relaying COTP/HM orders, the VTS may also become involved in other COTP/HM generated events, such as hazardous cargo escorts and security/safety zone monitoring.  Since procedures for handling these types of events are extremely dependent upon equipment, staffing and port geography, the VTS Authority may recommend that each VTS Centre should develop and promulgate these in conjunction with the COTP/HM.

- Marine Events. VTS Operators must be aware of marine events taking place within their area.  While not all marine events directly affect vessel traffic, those that do can impact the safe navigation of large vessels.  VTSs should take an active role in coordinating ship movements in and around marine events and must continually evaluate the risks to shipping and to those involved in the marine event.  Communications between the marine event on-site coordinator and the VTS are critical.  Every effort must be made by the VTS to coordinate scheduled ship movements with the on-site coordinator.  While suspension of the marine event should be viewed as a last resort, it must be considered if the risk of loss of life and/or property damage is too great.

- Deviations from Regulations.  Deviations from any published Regulations are likely to require special handling of vessel traffic.  A verbal deviation request considered by the VTS watch supervisor must be thoroughly evaluated for its impact on overall vessel traffic safety.  Recurring requests for deviation from a particular Regulation/provision may be considered for a permanent change to the regulations if the deviation can be shown to enhance overall safety.

- Emergencies/ incidents. Investigating an incident and the action to be taken, when a vessel in the charge of, or acting under the specific advice or instruction of a VTS operator has touched the ground, or has been in collision with, or dangerous proximity to any other vessel, or any fixed or floating object in the waters under the jurisdiction of the VTS Authority, including

  - The reporting of the occurrence to the VTS Supervisor, giving all the circumstances leading up to the occurrence;

  - The gathering of available reports, evidence and records. This procedure should also clearly indicate the person(s) designated to investigate such incidents and the subsequent action they are authorised to take, including the person responsible for closing out any such investigation and re-opening an investigation if subsequently, new information comes to light;

  - The circumstances under which any inquiries are to be held.

Keeping of and completing an authentic record of all vessel movements, ship/ shore and ship/ ship communications etc mentioned above under Routine Procedures is particularly vital in connection with any subsequent investigation of an incident or operation. Due care must also be taken in respect of the release of such information. Annex 8 provides an example of a generic Post Incident Procedure and an example of Guidance to Staff on the release of Verbal, Recorded and Written Information subsequent to an incident can be found at Annex 9.

Any incident occurring within the VTS Area is normally graded depending upon the seriousness and nature of the incident and the level of response required by the Authority, the emergency services and other agencies.  An example of a procedure where such incidents have been graded into Minor incidents, VTS Emergencies and Major incidents is contained in Annex 10. Annex 11 provides an example of a Medical Assistance procedure, which may or may not arise out of such incidents.

The Competent Authority/VTS Authority may require standard checklists to be completed in respect of certain incidents. One example of which is the Flare Sighting Check List at Annex 12. In addition, the VTS Centre may be required to participate in a national incident reporting scheme and submit information in a standard format. An example of such a procedure and report form is contained in Annex 13.

In the event of serious deficiencies in a ship’s equipment or condition reporting to external agencies may be required.

    - Environmental Conditions.  Adverse weather conditions may affect the normal transit of vessels in the VTS area.  VTS Operators must make every effort to ensure mariners are aware of weather conditions that may affect their transit.  VTSs shall ensure that procedures for handling vessel traffic in adverse weather conditions have been developed, are understood by all VTS Operators, and have been promulgated to the users.  These procedures may include the recall of Operators to augment the watch until conditions have improved, specialist radar watches, awareness of variation to standard Pilotage procedures, introduction of standard emergency or other situations or their variation. Such procedures should including any pre-determined conditions that may need to be applied by the VTS Centre, for example, before the VTS provides radar assistance to a vessel on request. An example of a typical Reduced Visibility procedure is attached at Annex 14. An example of a procedure covering tides and storm surges is at Annex 15. The weather forecast should be checked at the beginning of each watch monitored frequently throughout the watch.

    - VTS Equipment malfunctions/failure. In the event of equipment failure, VTSs should have emergency plans in place to continue operations with as little interruption as possible.  Should the failure be catastrophic, i.e., complete power failure, or one that requires evacuation of the VTC, such as fire, explosion, etc., emergency plans shall be prepared to move VTS operations to an alternate location and make necessary notifications as soon as possible.

- Aids to navigation defects. Monitoring and reporting on the status of aids to navigation within the VTS Area. An example of a typical procedure is attached at Annex 16.

    - Anchorage administration. Vessels at anchor should be continuously monitored by the VTS watch to ensure that they remain within a prescribed area.  When high wind or tide, strong current or other environmental conditions warrant, it is recommended that the VTS watch increases its vigilance in monitoring anchorage areas.  When unannounced or unexpected movement of a vessel at anchor is detected by the VTS, the VTS should contact the vessel immediately and direct that action be taken to stop the movement and re-anchor.    Other vessels at anchor or transiting nearby should be notified of the movement.  The VTS authority may require each VTS Centre to develop local procedures for addressing in addition to routine anchorage management (see Annex 7), foul weather anchorage procedures and handling of anchorage incidents.

    - Vessel Casualties/Vessel Emergencies. Reporting to external agencies in the event of serious deficiencies in a ship’s equipment or condition.

      - Suspension or limitation of the VTS. Under certain circumstances it may become necessary to limit or suspend a VTS. In such circumstances it is highly desirable for VTS Authorities to have in place a clearly laid down procedure that should be followed by the VTS Centre(s). An example of such a procedure can be found at Annex 17.

      - Support of Other Missions, Agencies, Allied Services, Governments and Organizations, including Security incidents, SAR incidents and Port State Control issues. A VTS may perform numerous tasks and functions in support of other missions and government activities.  While support of other missions may be encouraged, the VTS Authority/ Centre Manager must carefully weigh the impact of these support operations on vessel traffic safety in the VTS Area.

Annex 1

Overview of Internal Procedures addressing Operating Instruments, Interactions among the Staff And the Internal Routeing and Distribution of Data

These should be established by the VTS Authority to include the following:

- Routine Procedures

  - recording and retaining communication, surveillance and other data – this data shall be stored for a minimum of 30 days

  - ensuring correct staffing for operational positions

  - equipment operation, maintenance and updating

  - interactions with allied services (contingency plans)

  - media relations

  - security

  - access to information affecting vessels in VTS area

  - quality system (e.g. ISO 9000)

  - training / revalidation

2	Emergency Procedures

2.1	loss of functionality of communication equipment due to:

  - power failure

  - component failure etc.

    - secure emergency power supply

    - inform local VTS manager

2.2	loss of functionality of surveillance equipment due to

  - power failure

  - component failure etc.

    - promulgate information to vessels in VTS area

    - inform local VTS manager

2.3	internal emergencies due to

  - fire

  - flood etc.

    - comply with General Emergency Plan

    - inform local VTS manager

2.4	forced evacuation of VTS centre

  - if possible promulgate information to vessels in VTS area

  - if possible inform local VTS manager

  - if possible inform emergency services

2.5	personnel medical emergencies

  - call emergency service

  - inform local VTS manager

  - call up back-up VTS personnel

2.6	loss of landline communications

  - inform local VTS manager

Annex 2

Overview of External Procedures

External Procedures govern the interactions with users and allied services and should be established by the VTS Authority to include the following:

      - Routine Procedures

1.1	Arrival

1.1.1	Collect advance information in accordance with local rules and regulations.

1.1.2	Identify and establish communications with approaching vessels.

1.1.3	Confirm required advance information has been obtained.

1.1.4	Acquire further information if necessary prior to entry.

1.2	Vessels Entering VTS Area

1.2.1	Provide relevant information

- traffic

- navigational / fairway

- confirm participation

1.3	Vessels Transiting VTS Area

1.3.1	Monitor the movements of non-participating vessels only if considered significant to the movement of participating vessels and attempt to identify and establish communications with them.

    - Monitor the movements of non-participating vessels only if considered

significant to the movement of participating vessels and attempt to identify and establish communications with them.

1.3.3	Provide relevant information to participating vessels at regular intervals

- environmental conditions

- traffic situation

- navigational conditions

- warnings and restrictions concerning the movement of traffic in the area

1.4	Vessels at Anchor

1.4.1	If surveillance is available monitor their position.

1.4.2	Maintain communications with the vessels.

1.4.3	Obtain reports from vessels prior to them leaving the anchorage.

1.5	Vessels at Berth

1.5.1	Obtain reports from vessels on arrive at berth.

1.5.2	Communication with VTS is generally unnecessary but conditions may

require them to be maintained constantly.

1.5.3	Their presence may need to be declared to other vessels transiting the area.

1.5.4	Obtain reports from vessels prior to them leaving the berth.

1.6	Vessels Departing the VTS Area

1.6.1	Reports are required from vessels when departing the area.

1.7	Transition between Adjacent VTS Area

1.7.1	To ensure that the handover of identification, communication procedures

and monitoring responsibility are concluded satisfactorily.

2	Emergency Procures in the VTS Area

2.1	In case of collision, man overboard, capsize, sinking, grounding, fire on

vessel, search and rescue

- alert to MRCC

- inform local ship safety inspector

- inform local VTS manager

- call up back-up VTS personnel

- promulgate information concerning incident to vessels in VTS area

  - In case of pollution or risk of

- inform local ship safety inspector

- alert authority responsible for pollution control measures

2.3	2.3	In case of medical emergency

- contact appropriate authority for arrangement of radio medical assistance

- contact MRCC if MEDIVAC assistance required

2.4	In case of ship NUC

- promulgate information concerning incident to vessels in the VTS area

- obtain detailed information about on board situation

- if possible maintain communication with vessel in question

- assist vessel in question and comply with master’s request for assistance as necessary

2.5	In case of protest action or terrorist action against any vessel transiting the VTS area

- alert MRCC

- inform local VTS manager

- promulgate information concerning incident to vessels in the VTS area

2.6	In case of natural disaster such as forest fire, exceptional weather conditions etc.

- promulgate information accordingly to vessels in the VTS area

- inform local VTS manager

- inform MRCC as necessary

Annex 3

Example of Standard Communication messages to a Vessel

- A	Advice:        		Prefix   	 “ADVICE”

This indicates that the following message implies the intention of the sender to influence others by a recommendation.

Eg.	“ADVICE, Motor Vessel Nonesuch, the Large Container Vessel ‘LCV Europa’, has passed the “x” Buoy inward bound and will be taking her tugs at “x” location.  Recommend you slow down and await ‘LCV Europa’ clear of “x” Buoy”.

- All advisory messages should be communicated in an unobtrusive manner and at times when mariners can be expected to devote their full intention. Brevity is paramount.  Advisories must not contain extraneous information, nor should advisories be repeated except for clarification or when requested.  As circumstances warrant, VTS shall update information provided to mariners to ensure that the most recent information is available on which to base navigation related decisions.

## B	Information:	 	Prefix  	“INFORMATION”

- This indicates that the following message is restricted to observed facts, situations etc.  This marker is preferably used for navigational and traffic information, etc. Action taken as a result of receiving such an information message can be at the discretion of the recipient.

- Eg.	“INFORMATION, The dredger “Westminster Giant” is operating in the area close to the East of the “x” Buoy with moorings projecting into the fairway.”

- C	Instructions:		Prefix		 “INSTRUCTION”

This indicates that the following message implies the intention of the sender to influence the intention of others by a regulation. This means that the sender, e.g. the VTS Centre, must have the full authority to send such a message. The recipient has to follow this legally binding message unless he/she has contradictory safety reasons, which then have to be reported to the sender.

Eg.	“INSTRUCTION, The large Container Vessel ‘LCV Europa’, is experiencing some difficulty swinging in the fairway off your berth, you are to remain alongside until a further instruction is passed

D.	Warning:     		 Prefix		“ WARNING.”

This indicates that the following message implies the intention of the sender to inform others about danger. This means that any recipient of a WARNING should pay immediate attention to the danger mentioned. Consequences are up to the recipient.

Eg.	 “WARNING,  ‘Prospero’, VTS radar indicates you are heading towards shallow water”.

Annex 4

Example of a Generic Procedure dealing with the Tracking of Vessels with Nominated Hazardous Cargo

As the majority of vessels using the “name” VTS area now carry hazardous cargoes, only NOMINATED HAZARDOUS VESSELS must be tracked in red (Dangerous) and a Guard Zone applied when navigating in the VTS Area.

Nominated hazardous vessels are defined as:

- Liquefied Gas Carrier

- Explosive Vessels

- Specified Vessels

The Dangerous Goods Officer will deliver to the VTS Operator/Supervisor a NOMINATED HAZARDOUS file for any vessel meeting the criteria, with all information pertaining to that vessel.  The file must be maintained by the “name” VTS until the vessel is clear of the “name” Area.

The movements of NOMINATED HAZARDOUS vessels in the “name” VTS Area of Jurisdiction are governed by the following regulations:

- List relevant regulations

Annex 5

Generic Procedure for the use of Surveillance Equipment

VTS uses various equipment systems to maintain surveillance of the VTS Area.  These systems generally include radar, closed circuit television, and VHF-FM radio frequencies.

Although the Vessel Movement Reporting System (VMRS) is the "backbone" of monitoring the movement of vessel traffic, the proper use of surveillance both confirms and enhances this information.  Observations of channel and vessel traffic conditions allow the VTS Centre to collect, analyze and disseminate information that may not otherwise be reported.

VTS Operators will use the following operational doctrine for utilizing surveillance equipment to monitor waterways.

| RADAR | At the beginning of each watch the VTS Supervisor will insure that the radars are properly adjusted in accordance with the outlined specifications. Picture Presentation - Upon relieving the sector watch, each subsequent controller shall insure that the video presentation on each ODP is adjusted for optimum radar performance. Operational Setup - Radar indicators shall be monitored for the purpose of enhancing the Sector I and portions of sector II traffic plot. ODP 2 monitor 1 shall normally be configured to monitor “ name” Radar from the “ named lower limit” to “ named upper limit”. Monitor 2 shall normally be configured to monitor “name” Radar from “named lower Limit” to “ named upper limit”. Monitor 3 shall normally be configured to monitor “ named” Radar from “named lower limit” to “named upper limit”. VTS Operators are reminded that these are normal areas of coverage and may be adjusted according to traffic density or weather conditions. |
| --- | --- |
| RADAR (cont) | Operating Procedures - When monitoring the Radar, the Sector I controller shall note the movement of the reported traffic as well as any radar targets which by their movements or location may present a potential traffic or navigational hazard. Special attention shall be given to vessels stopped in a channel, vessels aground or anchored in a channel, and to crossing or intersecting traffic situations. |
| CCTV | Picture Presentation - Upon relieving the sector watch, each subsequent controller shall insure that the video on each monitor is adjusted for optimum presentation according to ambient lighting conditions. Operational Setup - When the cameras at a CCTV site are not being actively used to monitor a particular channel situation and to locate, identify, or track vessel traffic, they shall be positioned to monitor the appropriate reporting point for that remote site. |
| VHF | Operational Setup - The communications console at each sector shall be selected to monitor the appropriate VTS working frequency (Initial Vessel Check-In VHF-FM Channel 5A, Sector I/II VHF-FM channel 12, Sector III VHF-FM channel 11). At each sector position, VHF-FM channel 13 bridge-to-bridge communications, for that sector’s particular sites, will be placed on the “UN-SELECT” speaker. The audio level shall be adjusted to monitor all communications transmitted. Vessel movement information obtained should be correlated with other sources of surveillance to enhance the accuracy of vessel positions and their intentions. |

Annex 6

Example of a VTS Equipment Check/ Fault Rectification Generic Procedure

1.	VTS Centre

  - EQUIPMENT CHECKS

    - The Designated person within the VTS Centre is to ensure that the following equipment checks are carried out and to record the fact in a log.  Details of the nature of any defects should be entered in the log and action taken to rectify these defects, if appropriate.

    - At midnight every night:

      - Check all equipment system times are synchronised.

      - Change VTS Centre and Security video tapes.

      - Carry out archiving of radar and audio data.

      - Check functionality of all radars, communication systems and data sensors.

      - Note any system faults.

    - At midday every day:

      - Check all equipment systems times are synchronised.

      - Carry out archiving of radar and audio data.

      - Check functionality of all radars, communication systems and data sensors.

      - Note any system faults.

    - Every week check:

      - VTS Centre fire alarm system.

      - Lifeboat Pager

      - Fire Brigade radio

      - Emergency Hand Held Radios

      - Police telephone link

      - Note any system faults

  - Equipment Fault

    - In the event of an equipment fault, the following details should be recorded:

      - Time and details of the fault occurring in the equipment.

      - Status of equipment.

      - Time service engineer called (if required).

      - Time and details of repair.

      - Status of equipment.

Annex 7

Example of a Generic VTS Anchorage Procedure

All parameters for anchorages are guidelines only.  VTS Operators have discretion to vary these guidelines taking into account tide, weather, length of stay and the general circumstances or permit larger vessels to anchor for operational reasons.

“A” ANCHORAGE

Recommended Maximum length	Example:  	91.5 metres

Recommended Maximum draft	Example: 	3.7 metres

“B” Anchorage

Recommended Maximum length	Example:	85.0 metres

Recommended Maximum Draft         Example:	  5.0 metres

“C” Anchorage

This anchorage should be used as an extension of the “B” Anchorage and the same length and draught restrictions apply. The holding ground is not good and vessels using this anchorage should be reminded to keep a listening watch on VHF channel “number”.

Vessels carrying dangerous goods are not normally allowed to anchor in “name”, but the VTS Operator may, at his/her discretion permit such a vessel to anchor at the “B” Anchorage.

“A” Anchorage is licensed for explosives up to 100,000 Kgs Class 1.1 on any one vessel, provided that no other vessel containing dangerous goods is present. For Class 1.2, 1.3, or 1.4 the amount is unlimited, provided vessels are moored at least 170 metres apart.

“D” Anchorage (Hazardous Anchorage)

Recommended Maximum length	Example:	180.0 metres

Recommended Maximum draft	Example:	    9.0 metres

“E” Anchorage

Recommended Maximum length	Example:	130.0 metres

Recommended Maximum draft	Example:	   5.5 metres

“F” Anchorage

Under 8m Maximum draft:

Recommended Maximum length	Example:	225 metres

Up to 9m Maximum draft:

Recommended Maximum length	Example:	170 metres		Suitable for short term stay, and emergency use only.

Annex 8

Example of Generic Post Incident Procedure

(i)	VTS Operator/Supervisor to ensure tight control of all outgoing information and to refer all inquiries to the designated person.

(ii)	Remind all staff that statements must be given through the agreed channels

(iii)	Collect and retain all scrap notes.

- VTS Operator/Supervisor to be relieved in order to produce written statements as “Initial Report”.

- (v)	Consider and list for future contact all others who may have witnessed or heard anything relating to the incident.

Annex 9

Example of Generic Guidance for Staff on Release of Verbal, Recorded or Written Information

The permission of the “name” should normally be obtained before any form of information on an incident is released to press, public, or interested parties.

The following guidelines set out the general principles to be followed in the release of information.

(i)	Marine Accident Investigation Unit or equivalent

Under national law there may be a requirement to inform a Marine Accident Investigation unit of any serious incident. In which case all forms of information may be released to them at their request.  The amount of information provided should be sufficient for the purpose.

(ii)	Pollution Reports

There is a duty to report all cases of oil or chemical pollution to the relevant body.  A number of IALA members have laid down procedures and standard forms for this purpose. Any subsequent requests for information should be satisfied.

- Deficiencies in Ship’s Equipment for Navigating/Failure of Equipment whilst Transiting the Port

Pilots have a duty to report to the VTS any experience or the discovery of serious deficiencies in a vessel’s equipment for navigating. This information must be passed by the VTS Centre onto the relevant Administration responsible for Port State Control.

- Reports and Information Concerning Collisions/Groundings/Near Misses/Founderings etc.

A wide variety of information is generated by a serious marine incident.  The VTS Centre will normally be involved in gathering the available information and passing this on to the designated person.  Requests for the subsequent release of this information can come from a wide variety of sources including official bodies, lawyers representing various interested parties, Underwriter, P & I Clubs, towing companies, salvors, port and berth operators etc.

a.	Pilots’ Reports

Pilots’ Incident Reports are made to the Authority usually on a confidential basis, but they may generally be released to persons or bodies with a legitimate reason for requesting them.  The Pilot should be consulted before release of his/her confidential report, and his/her permission obtained.

NOTE:	  It should be remembered that the Pilot is, in law, the servant of the Shipowner or Charterer.  The Master therefore has a legitimate right to require the Pilot to furnish him with a report.  However, this does not necessarily need to contain the same level of detail as the confidential report submitted to Authority, so long as it is truthful and accurate.

- Tidal/Meteorological and Traffic Data

These items are a matter of public record and there can be no objection to making them publicly available on request.

- VHF Radio Transcripts

VHF messages are broadcast into the public domain and routinely listened to/overheard/recorded by a wide spectrum of listeners.  This also applies to so-called “private channels” which are not private, but rather restricted to certain users.  There can be no objection in principle to making radio recordings or transcripts available to shipping companies, their Underwriters or Legal Advisors.

d.	VTS Centre Radar Recordings

Radar recordings are the private property of the VTS Centre and should not routinely be provided without good reason.  The extent to which the radar recording will be made available will depend on the circumstances.  A copy would normally only be made available to the Marine Accident Investigation unit or a court of law.  Lawyers seeking information may be permitted to view the tape on the VTS premises and make notes.  Similarly, towing companies, Underwriters, P & I Clubs, union representatives etc. would receive the same treatment.  Actual copies of the radar tape would only normally be made available in certain circumstances where the liability of the Authority and its employees is not at risk, e.g. a collision or grounding well beyond the jurisdiction of the VTS in which the Authority had no involvement.

e.	Written Statements Provided by VTS Operators/ Supervisors/Managers.

These are the private property of the Authority and discretion is normally exercised in making them available.  In some cases permission may be given for the member of staff to be interviewed instead, with a legal representative present to protect the interest of the employee and the Authority.

f.	Technical Documents Provided by a Vessel

In the course of an investigation, copies of a ship’s log book, cargo or bunker information, course recorder and engine order record etc. may be obtained.  These may be made available to the proper authorities but should not be released otherwise, except in response to a court order for disclosure.

g.	Master or PEC Holder’s Statement

Similarly, this document may be released to the proper authorities but lawyers or others seeking the information should be directed to the Master/Shipping line.

h.	Requests for Information from Press/Media

It is normal practice for the VTS Manager/Supervisor in charge of any incident to prepare a holding statement.  Thereafter, all press enquiries are usually addressed to the Centre’s/Authority’s designated press spokesperson.

i.	Survey Information

The Authority has a public duty to provide a safe system of fairways and channels, and to ensure that berths are regularly surveyed.  This information is a matter of public record and can be provided on request.

(vi)	Costs of Providing Information

This is a matter for the VTS Authority to determine.

Annex 10

Example Of a Generic Minor Incident, VTS Emergency and Major Incident Procedure

  - MINOR INCIDENTS

Minor Incidents are those that can be handled within the every day resources of the VTS. Although the Emergency Services or other organisations may be notified or required to assist, such assistance will be routine. An example of a minor incident is where a casualty to a vessel occurs that does not immediately threaten the vessel's safety but reduce its manoeuvrability or navigation capability to a point below that specified by regulation or accepted as prudent seamanship. Permission to transit, under impaired conditions, may be requested by the vessel. In these cases the VTS should exercise extra vigilance in monitoring the vessel’s transit and may issue a COTP/HM order directing the vessel to conduct itself in a manner prescribed by the COTP/HM.

1.2	VTS EMERGENCY

VTS Emergencies are incidents of greater severity, which require a pre-planned, structured response.  These responses can be sub-divided into a number of likely scenarios (e.g. collision, fire, grounding, etc.) and are normally included in a VTS Emergency Procedures Manual.

1.3	MAJOR INCIDENTS

A Major Incident within the VTS Area is usually defined as meeting one or more of the following criteria:

“Any emergency which causes or threatens one or more of the following:-

- Death or serious injury to numbers of people;

- Extensive damage or contamination to the environment;

(iii)	Extensive damage to vessels, installations, berth facilities, river structures, plant or equipment;

(iv)	Serious disruption to vessel traffic

”

The definition of major incident cannot be precise and may be at the discretion of the VTS Operator/Supervisor on duty at the time. As a guide, but by no means as a rule, a major incident may be defined as one of scale, which immediately involves the intervention of the towage, salvage or emergency services and may require the putting into effect of the full scale emergency procedures.

Alternatively, it may be defined as an incident being on a scale beyond the normal day-to-day capacity of the VTS Centre/Authority and the emergency services, and requiring the special and extensive mobilisation and organisation of those resources.

A VTS Authority may decide to classify their major incidents into tiers, depending upon the seriousness of the incident and level of response required. For example some VTS Authorities have written tiered response procedures covering different levels of oil spillage.

Level of Response

In the event of any incident or emergency broadcast by a vessel or involving vessels within the “name” VTS area on VHF Channel “number”, the VTS Operator must use the following procedure:

- Assess the situation by collating initial information covering such items as:

-	Time and date of incident

-	Vessel identity or area affected

-	Type of incident

- Position of incident

- Number of persons involved

-	Any injuries or people requiring medical assistance

-	Hazardous cargoes on the vessel

-	Initial damage to vessels and then

brief VTS Supervisor/Manager on incident details

- Alert VTS Authority/Allied Services by providing a brief on the incident details. The services called will depend upon the severity of the incident and whether of a specific nature. Alerting other authorities may also depend on the severity of the incident or its location.

- Initial response and actions may involve:

-	Co-ordination of VTS communications using a secondary VHF channel if necessary

- Emergency broadcast (if directed)

- Liason with neighbouring VTS centres

- Setting up of the VTS Emergency Room

Whether the full initial response team is called or just some members will depend on the severity and type of the incident. It is however essential that key personnel are on site to assist before the incident escalates. Again depending on the seriousness of the incident/emergency the VTS Emergency Room may need to be manned up and assume all communications for the incident from the VTS Centre itself. Consideration may therefore need to be given to the use of additional VHF channels and telephone lines.

- Further responses and actions that may be required may include:

- Suspension of pilotage

- Control of vessel traffic

- Declaration of an Exclusion Zone

- Implementation of Emergency procedures

If the incident, from its type and severity, is classed as major, or has escalated to that of a major incident, then additional procedures may need to be implemented.

Annex 11

Example of a Generic Procedure for Medical Assistance

If a request for immediate medical assistance comes from a vessel without shore access the following procedure will apply.

Inform Doctor/ Duty Doctor

Arrange pick-up point for Doctor.

Arrange transport to the vessel by most appropriate means of transport immediately available.

NB The transporting of the doctor without delay is the first consideration, but suitable arrangements must be made for the evacuation of the patient.

Arrange ambulance if needed at the rendezvous point.

Inform vessel’s local agent of the situation.

Customs/ Immigration to be advised of any person being landed from a vessel, which is not berthed.

If medical assistance is requested by a vessel already berthed or by a vessel on arrival at a berth, the berth operator concerned should be instructed to make the necessary arrangements. Should there be any delay or difficulty in advising the berth operator, summon medical assistance without further delay and endeavour to advise the berth operator of the action taken.

Annex 12

Example of a Flare Sighting Check-sheet

OBTAIN A CLEAR MENTAL PICTURE OF WHAT THE R/S OBSERVED

DTG of Report:________ Reporting Source Name:________________________ Address:_______________________________Phone:_____________________

Flare color: RED AMBER WHITE GREEN OTHER:________ # Observed:____ Type of flares: PARACHUTE HANDHELD METEOR OTHER:__________ Time interval between flares:______________ Duration of burn:_____________ Trajectory: RISE FALL ARC STEADY OTHER:

Additional Information: R/S Position: Position Uncertainty:

DETERMINE THE ANGLE OF ELEVATION

To determine the angle of elevation, particularly from the inexperienced R/S, ask the R/S to hold his/her arm at arm’s length, make a fist, and place the bottom of the fist on the horizon. If the elevation of the flare is ABOVE the fist, the angle is greater than 8 degrees. Any elevation above 8 degrees can be approximated as the distance to the flare is within 1 NM. If the elevation is BELOW the top of the fist, ascertain how high up the fist, i.e. ¼, ½, ¾ or number of fingers. The distance to the source of the flare is much greater for any angle below 8 Degrees. Angle of Elevation:__________deg NLT:_________deg NMT:_________deg How Determined:____________________________________________________ Distance from R/S based on Bearing from R/S: angle of elevation ______deg T / M + / -_____deg NLT:______nm + / -______nm NMT:______nm + / - ______nm

Apparent origin of flare: SURFACE AIR OTHER:________________ Relation to the horizon: ABOVE BELOW ON OTHER:__________ R/S height of eye:_______ Any vsls or aircraft sighted in vic:__________ Obstruction in line of sight: TREES BLDGS OTHER:________ O/S Weather: Wind:_____/_____ Seas:_____/_____ Vis:_____ Sea temp:____ Additional notes:

If R/S is on a vessel, will R/S respond to sighting: Y / N ETA O/S:______ Intended action by R/S:________________________________________ If R/S is on land, will R/S remain by the phone to vector SRU: Y / N Advise R/S of intentions:

Annex 13

Generic Procedure covering National VTS Accident and Incident Reporting on Vessels

- OBJECTIVES

VTS Accident and Incident Reporting on Vessels can assist VTS authorities to take measures to avoid accidents in the VTS area and provides a valuable source of ’Lessons Learned’ to both VTS authorities and individual VTS operators.

1.1	To achieve this a national centrally organised VTS Accident and Incident Reporting System on Vessels shall be established.

1.2	This national VTS Accident and Incident Reporting System will collect the reported data and will disseminate summaries.

1.3	The System will use the information for the purpose of accident prevention only.

1.4	Before dissemination of information, it will be ensured that individuals or vessels cannot be identified.

- THE REPORTING SYSTEM

2.1	This system provides an opportunity for the VTS Authority to report VTS addressable accidents of VTS-participating vessels. Accidents are: collisions with a vessel, collisions with a non-vessel object and groundings.

2.2	VTS addressable incidents of VTS-participating vessels may be reported if they are considered important for safety and accident prevention.

2.3	The system will utilise a database where the reported accidents and incidents will be stored and processed for dissemination.

- PROCEDURE

  - Types of report are

- The VTS Accident / Incident and ’Lessons Learned’ Report

For the VTS Accident / Incident and ’Lessons Learned’ Report on Vessels and VTS Centres are provided with a special form- see Appendix 1.

  - The data provided for the VTS Authority will be used for statistical analysis only.

  - The data provided by the VTS Authority may be distributed periodically to IMO and IALA without any interpretation by the System, with the results of the statistical analysis of all reports.

- ACCIDENTS AND INCIDENTS

  - An accident has occured when a vessel is involved in

- collision with another vessel

- collision with a non-vessel object

- grounding

  - An incident is an occurrence, other than an accident, associated with the operation of a vessel, which affects or could affect safety of operation.

The VTS Authority may also forward to IMO and IALA data on significant incidents to be considered for safety recommendations and ’Lessons Learned’.

| VAIREP FORM A |  |  | PRELIMINARY DRAFT |  |  |  | Appendix 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Organization: |  | Reference: |  |  | Date: |  |
|  |  |  |  | Page: |  | of | Pages: |  |
| VTS Accident/Incident and 'Lessons Learned' report (VAIREP Form A) |  |  |  |  |  |  |  |  |
| Type of Report: |  |  | Accident | Incident |  |  |  |  |
| Type of Accident: |  |  | Collision with vessel | Collision with non- vessel object | Grounding |  |  |  |
| Vessel: |  |  |  |  |  |  |  |  |
| Type of Vessel: |  |  |  |  |  |  |  |  |
| Length of Vessel: |  |  |  |  |  |  |  |  |
| Draught of Vessel: |  |  |  |  |  |  |  |  |
| Loading Condition |  |  | Light | partly loaded | loaded |  |  |  |
| Under Pilotage |  |  | no | yes |  |  |  |  |
| Area |  |  | Open sea | Restricted waters |  |  |  |  |
| Personnel Casualties: |  |  |  |  |  |  |  |  |
| Injuries |  |  | no | yes | major | minor |  |  |
| Deaths |  |  | no | yes | major | minor |  |  |
| Pollution: |  |  | no | yes | major | minor |  |  |
| Damage: |  |  | no | yes | major | minor |  |  |
| Contributing Causes: |  |  |  |  |  |  |  |  |
| Vessel: |  |  | Comms failure | Location failure | Procedural failure | Machinery failure |  |  |
| VTS: |  |  | Comms failure | Localisation failure | Procedural failure |  |  |  |
| Communication Interference: |  |  | no | yes |  |  |  |  |
| Meteorological and Hydrological Conditions: |  |  |  |  |  |  |  |  |
| Wind |  |  | Insignificant | Significant |  |  |  |  |
| Gale Warning |  |  | no | yes |  |  |  |  |
| Visibility |  |  | good | bad |  |  |  |  |
| Light Conditions |  |  | day | twilight | night |  |  |  |
| Tide |  |  | Insignificant | Significant |  |  |  |  |
| Current |  |  | Insignificant | Significant |  |  |  |  |
| Sea Estate and Swell |  |  | Insignificant | Significant |  |  |  |  |
| Aids to Navigation operating |  |  | yes | no |  |  |  |  |
| Narrative: 'Lessons Learned' |  |  |  |  | Please use additional pages if required for the text |  |  |  |

Annex 14

Example of a VTS Reduced Visibility Generic Procedure

- General Reduced Visibility Procedures – Instructions to Duty VTS Staff

  - Definitions

  - In the event of reduced visibility (to be defined in 1.1 above) the following procedure is to be followed: -

    - An INFORMATION Warning Broadcast will be made on Channel “X”, stating that the Reduced Visibility Procedure is in operation for the VTS Area. This information will include the status of any special Radar Watch required.  A further broadcast should be made on resumption of normal operations.

    - The VTS Centre (which is responsible for VHF discipline at all times), will exercise particular vigilance concerning VHF discipline during the period when the Reduced Visibility Procedure is in operation.

    - Vessels alongside will exercise extreme caution when communicating with a vessel underway.

    - No vessel will leave the berth without clearance from the VTS Centre Designated Person.

    - The VTS Centre Designated Person will consider the inward traffic disposition and will, when necessary, delay the arrival or departure of vessels to ensure a one-way flow of traffic is achieved in any such designated channels. Ships proceeding through any one-way traffic flow areas are to maintain a minimum separation distance of “x” metres.

    - Action should be taken to ensure that any designated tracks for specific types of vessel remain clear.

    - The VTS Centre may allow use of the areas surrounding designated tracks, by certain other vessels, requiring an escape route from the main channel.

    - Vessels must be directed to the outer anchorages or to remain alongside, if the VTS Centre Designated Person has reason to believe that they may not be equipped to transit the harbour safely.

  - Special Radar Watch service: The specific purpose of the service provided by any Special Radar Watch service in reduced visibility is to monitor traffic movements, ensure close liaison between the VTS Operator(s) and other relevant staff, to advise of anticipated problems, provide safe, reliable, navigational advice to any vessel requesting shore assistance.

    - If such assistance is requested because of serious on-board equipment failure or navigational problems, such advice should be designed to place the vessel in a stable situation, e.g. in a safe anchorage clear of channels and not normally bring it to a berth.

    - Radar assistance as far as the off- berth position may be given to well found vessels if, in the judgement of persons designated by the VTS Authority, such assistance would enhance the safe passage of such a vessel.

- Conditions for Providing Special Radar Assistance in Reduced Visibility

  - In reduced visibility, a request will normally only be considered under certain conditions laid down by the VTS Authority.  However, in case of emergency, the VTS Designated person(s) should always provide assistance. The conditions contained in 1.2 above will normally also apply.

  - The identity of the radar target must be established beyond reasonable doubt before radar advice commences.

  - The VTS Authority should specify the number of vessels that can safely be advised by the Special Radar Watch Designated Person at any given time. However, it is accepted that the Designated Person may safely monitor a number of vessels.

- PROCEDURES

  - Operation of VTS Centre Radar Advice in Reduced Visibility

    - Immediately before the Radar advice commences, the VTS Centre will assign a VHF channel exclusively for the use of the Vessel and the Designated Person within the VTS Centre undertaking the Radar Watch. The designated person on board the vessel will then monitor only the assigned frequency, all other communications being relayed to him/her by the Designated Person within the VTS Centre undertaking the Radar Watch.

    - The VTS Centre will then make a warning broadcast on channel “x”, informing all other stations that a vessel is using VTS Centre Radar Assistance and will not answer on channel “x”. All calls to the assisted vessel will be passed via the Designated Person within the VTS Centre undertaking the Radar Watch on the assigned frequency. No other stations will be allowed to use this frequency. The Designated Person within the VTS Centre undertaking the Radar Watch will keep the vessel under advice fully informed about other vessel movements, as necessary.

    - The VTS Centre will report to vessels in the VTS Area the progress of the assisted vessel, at appropriate intervals, with the repeated reminder that the assisted vessel is not monitoring channel “x”.

    - Any tug rendezvous arrangements will be passed via the Designated Person within the VTS Centre undertaking the Radar Watch but when tug manoeuvres commence, the vessel and the Designated Person within the VTS Centre undertaking the Radar Watch will change to the tug frequency which will be used for advice and tug communication until the designated person on-board the vessel is satisfied that further assistance is not required from the Designated Person within the VTS Centre undertaking the Radar Watch.

    - Designated Person within the VTS Centre undertaking the Radar Watch must observe VHF discipline and prefix all advice with the name of the vessel being addressed, even though only one vessel may be being advised. This is to eliminate the possibility that another vessel could construe the advice as applicable to her.

  - Information Required from Vessel

    - The Designated Person within the VTS Centre undertaking the Radar Watch should satisfy himself/herself that he/she has the following information from the requesting vessel before giving advice.

      - Heading and speed and any change in either.

      - Proposed helm-over positions.

      - Rate of turn, where available.

  - Information to Inward Vessel by Designated Person within the VTS Centre undertaking the Radar Watch

    - Bearing and distance to selected AtoN. Care should be taken when using floating AtoN for this purpose, as the assigned position/ charted position of the AtoN will invariably differ from the position shown on the VTS radar screen due to the influence of tide and wind on the body of the floating AtoN and its mooring. Fixed AtoN should therefore be used in preference to floating AtoN for this purpose, wherever possible.

    - When passing selected AtoN.

    - Vessel’s position relative to the centre line of the channel.  LEFT or RIGHT only.

    - Vessel’s track tendency, e.g. “holding steady track”, “tending to drift left”, etc.

    - Value judgement on vessel’s progress, e.g. “maintaining good track” or “deviation to right needing immediate correction”, etc.

    - Predicted traffic movements, and when likely to pass the vessel.

    - Any other information as requested, e.g. clearing bearings/transits as vessel approaches wheel-over position.

  - Information to Outward Vessel by Designated Person within the VTS Centre undertaking the Radar Watch

    - Bearing and distance to selected AtoN- see paragraph   above

    - Situation at selected AtoN when passing.

    - Vessel’s position relative to the centre line of the channel. LEFT or RIGHT only.

    - Predicted traffic movements and when likely to pass the vessel.

    - Any other information as requested, e.g. clearing bearings/transits as vessel approaches wheel-over position.

  - Positional Guidance

    - When giving vessel’s position relative to Centre Line of channel, it may be useful to divide the half channel into 3, i.e.

Centre Line				Centre Line

1/3 Left of centre			1/3 Right of centre

2/3 Left of centre			2/3 Right of centre

On Left AtoN line			On Right AtoN line

  - Warning of Serious Situation Or Trend

    - If a potentially serious situation is developing, early warning of a dangerous trend is essential.  Use the message marker “WARNING” to prefix messages about a serious or dangerous situation,

e.g. “WARNING, Nonsuch, you are on left AtoNline, “X” metres from “ Name of AtoN”.

or

“WARNING, Nonsuch, you are now rapidly tending towards the “ X        Jetty”.

Advice To Vessels Without A Pilot.

The VTS Authority should provide specific instructions to the Designated Person within the VTS Centre undertaking the Radar Watch, as to the action they are to take when advice is being given to a vessel in difficulty without a Pilot on board.

Arrival or Departure under VTS Centre Radar Advice

If a vessel is entering or leaving the port under shore radar advice, the VTS Authority should provide specific instructions to the VTS Centre on what procedures are to be followed in relation to other vessels in the VTS Area, including those at berth.

Annex 15

Example of a Generic Procedure for Tides and Surges

(i)	High Tide Warnings

Where appropriate, the Tidal prediction for each high and low water should be carefully monitored and any predicted abnormally high or low water should be brought to the attention of users of the VTS.

## (ii)	Storm Tide Warnings

- Some member countries of IALA may have in place a Storm Tide Forecasting Service.  The issue of warnings of abnormally high sea levels may be the responsibility of the VTS Centre. Formal Warnings may come in two stages.

1.	First stage – Preliminary Situation Report	“Advice”

( issued 12 hours before the

high water considered to be a threat.)

2.	Second Stage - Warning  either  (a)Flood watch

(Issued 6 hours before the            (b)Flood Warning

High water considered	      (c)Severe Flood Warning

to be a threat.)

- Upon receipt of a warning, the VTS Centre should arrange for all relevant users/authorities/personnel to be informed.

- Acknowledgement of receipt of these warnings may be required.

- The VTS Centre may also be requested to advise relevant users/authorities/personnel an “ALL CLEAR” when warnings are stood down.

Annex 16

Example Generic Procedure for dealing with reported casualties and defects to aids to navigation.

Normally the Authority should have laid down availability targets for its aids to navigation. Based on these targets, the Authority should have drawn up both casualty response and defect response priorities, indicating the urgency and nature of the response required, including the resources to be called upon to rectify the casualty/defect and whether, in the case of a casualty, a radio navigation broadcast is required.  These casualty/defect response priorities are usually drawn up and made available for the guidance of VTS personnel. They are not necessarily intended to indicate a timescale for rectification of the casualty/defect. These casualty/defect response priorities may be contained within the overall VTS Emergency Plan(s).

When a report is received, from either an internal or external source, of a casualty or defect to an aid to navigation, the time of the report and the details should be recorded by the relevant VTS personnel. Depending upon the considered reliability of the source of the report the Authority may require VTS personnel to obtain confirmation of the casualty/defect from another source. Any such reports should then be actioned in accordance with the laid down response priorities.

In addition to the above sources of information, the positions of all major floating marks may be mapped on the VTS radar and any serious discrepancy between the video mapping and the radar echo should be checked, recorded and dealt with in accordance with the laid down casualty response priorities.

Casualties to aids to navigation should normally promulgated to users of the VTS Area(s), in the form of Navigation Warning Broadcasts and “Notices to Mariners”. Standard message formats are normally required to facilitate notification and reduce confusion in passing information on updates and corrections.

Where the AtoN belongs to another authority or private individual the VTS Centre will be responsible for passing the details of the casualty onto the relevant Authority/individual in order that they can make arrangements for the AtoN casualty to be restored.

VTS Operators/Supervisors are responsible for recording every discrepancy, update, and correction in the ATON Discrepancy Log, which will show every outstanding discrepancy within the VTS Area.

VTS Operators/Supervisors will use the ATON Discrepancy “check list” to ensure all administrative and operational requirements are met.

The Authority may have also delegated to the VTS Centre the task of preparing reports on all AtoN defects, including the analysis of the same. In addition, completion of additional forms and/or surveys may occasionally be required.  Their purpose could range from quality management measurement of VTS's involvement in this support function, to providing data for documenting the need or success of various AtoN projects or VTS participation in the Level of Service requirements.

In cases of damage, the Authority may be able to reclaim the cost of repairs or replacement from the individual, vessel or corporation responsible for damaging ATON.  If the vessel making the report is the vessel that caused the damage or the vessel causing the damage can be identified, the VTS Operator should obtain as much information as possible concerning the damage, the operator’s name, address, and phone number, the owner’s name, address, and phone number, and the vessel’s call sign. This information should then be forwarded to the appropriate designated person in order that the matter can be fully investigated and all the costs involved in rectification of the damage reclaimed.

Annex 17

Example of Generic Procedure for the Limitation or Suspension of “Name” VTS

If in the event of emergency or for any other reason it is not possible to operate the “name” VTS to the service level stated, or it is required to suspend the service, the following procedures must be followed:

- Initial alert by VTS Centre

- Vessels within the “name” VTS area

A broadcast is to be made by “name” VTS on VHF channel “number” informing vessels of the limitation or suspension of the VTS Information/Navigational Assistance/Traffic Organisation (delete as appropriate) Service.

[(B)	VTS Authority

The VTS Authority at “x” is to be given brief details and approximate length of the limitation or suspension.]

(C)	Neighbouring VTS Centres

Neighbouring VTS Centres are to be given brief details of the limitation or suspension.

- Emergency Operation

- Equipment failure

Initiate back-up equipment and resume “name” VTS service.

- Operate “name” VTS Information/Navigational Assistance/Traffic Organisation Service (as appropriate) on back-up systems.

- Operate “name” VTS Information/Navigational Assistance/Traffic Organisation Service (as appropriate) providing traffic broadcasts at fixed intervals on VHF Channel “number”.

- Operate “name” VTS Information/Navigational Assistance/Traffic Organisation Service (as appropriate) via another VTS Centre/Area on VHF channel “number”.

- Evacuation Of VTS Centre

If, in the event of fire or other reason, the Operations Centre has to be abandoned, the following procedures must be followed:

VTS Operator/Supervisor should:

- Alert Emergency Services (if required)

- Emergency Broadcast

- Suspend VTS and Pilotage operations

- Establish communications with the VTS Officer team

- Evacuate Operations Centre

- If fire, ascertain position from alarm repeater panel

- If necessary release extinguishing gas to Equipment Room

- Check each floor has been evacuated

- Muster in designated area

- Liase with Emergency Services

- Hand over to Senior Officer (if present)

- Proceed to designated Emergency VTS Centre

VTS  Officer Team

- Establish communication with VTS Operator/Supervisor

- Evacuate VTS Centre

- Proceed to Emergency VTS Centre

- Activate VTS equipment

- Reinstate VTS

- Maintain operations from Emergency VTS

- Continuation of limitation or suspension

- VTS Authority

Maintain liaison with the VTS Authority.

- Neighbouring VTS Centres

Maintain liaison with neighbouring VTS Centres and request update broadcast to be transmitted by them, if required.

For possible inclusion