Input paper:   ENAV24-6.1.12

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM		□  ENG		□  PAP						□  Input

X  ENAV		□  VTS								X  Information

Agenda item 				6

Technical Domain / Task Number 2	…………………………………

Working Group				WG3

Author(s) / Submitter(s)			Wonyong KIM(COMESTA), Juntae KIM(Korea Register),

Jaeheon KIM(Ace Antenna), Jeonghyun KIM(I-Storm),

# Initial VDES sea trial and test plan under Smart-Navigation Project

# Summary

As the part of the SMART-NAVIGATION project in Republic of Korea, Korean Register (KR) in cooperation of COMESTA, Ace Antenna and I-Storm is proceeding with the development of VDES communication equipment, and construction of VDES Testbed by 2020.

This paper introduces the test results regarding our first sea trial of VDES communication system conducted in Mokpo from 19 to 20 August 2019, and future test plan for VDES equipment developed under Smart-Navigation.

The sea trial was conducted with support of Mokpo National Maritime University on ASM and VDE communication links of VDES Base Station equipment.

## Purpose of the document

This paper is aiming to provide the information on the test results of our first VDES sea trial and future test plans at sea for VDES equipment developed through the Smart-Navigation Project.

## Related documents

[1]	IALA Guideline G1139 Ed.2, The technical specification of VDES

# Background

Smart-Navigation project funded by Korea’s Ministry of Oceans and Fisheries, was initiated in 2016, and is now underway for the development and construction of Korean e-Navigation testbed.

KR together with COMESTA, Ace Antenna and I-Storm is developing VDES Base Station and Mobile Station Equipment in the core task III of the Smart-Navigation Project.

Figure 1 SMART-NAVIGATION Project

The VDES Base Station (Model Type: KEVSS-100) has been developed under Smart-Navigation Project, and which integrates the functions of ASM, VDE-TER, AIS Receiver and GPS, and is complied with IALA Guideline G1139 ed.2 issued in December 2018 by IALA.

The below Figure 2 shows the external appearance and internal configuration of KEVSS-100. The main hardware configuration consists of RF front end subsystem, baseband subsystem and control subsystem. In addition, the AIS receiver module and GPS module are included to enable independent operation by itself.

Figure 2 KEVSS-100

Basic specification of KEVSS-100 is as follows:

[Features of KEVSS-100]

- Software Defined Radio (SDR) Solution

- Complete VDES solution

- True two channel receiver, continuously receiving on AIS channels A(161.975 MHz) and B (162.025 MHz)

- Fully compliant VDES (ASM + VDE-TER)

- Upgradeable and future proof

- Output power: 12.5W(25W peak)

- RF Range covered: 156.025 - 162.025 MHz

- Interfaces:

- Ethernet and serial ports

- LCD display and keypad

- USB Debug port

# VDES SEA TRIAL OVERVIEW

## General

The first sea trial of KEVSS-100 was conducted in nearby waters with the support of Mokpo National Maritime University, and the test was performed by dividing into ASM service test according to predetermined scenario and ASM and VDE-TER transmission/reception test.

The KEVSS-100 used in the test is VDES Base Station equipment, the KEVSS-100 equipment installed on the ship station was used and simulated for ship station since the ship station is still under development. The test was carried out using only coast station frequencies.

The sea trial confirmed the ASM message transmission/reception function to generate and AIS-ASM messages from the ship station and deliver it to the coast station, and the VDE message transmission/reception function to generate text-based short message from the ship station and deliver it to the coast station. Also, the Packet Error Rate (PER) of both ASM and VDE message received from the coast station was measured.

- ASM Message Interworking : ASM Link ID 7

- ASM Performance Test : Measure Packet Error Ratio

- VDE Message Interworking : VDE Link ID 11

- VDE Performance Test : Measure Packet Error Ratio

KEVSS-100 equipment are installed in both shore station and ship station. The shore station was installed on the navigational bridge in training vessel “SAENURI” of Mokpo National Maritime University, and the ship station was installed on the yacht “NEPTUNE” to conduct communication tests between the ship station and the shore station.

Figure 3 Test Configuration

## Test location and environment

The VDES sea trial site is located near Mokpo National Maritime University, in Mokpo, Republic of Korea. As shown in Figure 4, in the test area, a number of islands and bridges are located nearby which may bring out the multipath propagation.

In addition to this, there is possibility to be interfered from the AIS signal transmitted from nearby VTS.

Figure 4 Test location

The ship station's route in the sea trial is shown in the above figure, and the test results could be derived through several repeated tests.

## Shore station installation for testing

The coast station of VDES was installed at the Navigational Bridge of M/V “SAENURI”, which is anchored at Mokpo National Maritime University. The default settings of the coast station for sea trial are shown in the table below.

| M/V “SAENURI” | VDES shore station installation in the bridge |
| --- | --- |

| Shore Station Parameter Setting for Testing |  |
| --- | --- |
| Location of Shore Station | Training Vessel “SAENURI” |
| RF Transmit Power | 12.5 W |
| Link ID for ASM and VDE | Link ID 7, Link ID 11 |
| Bandwidth | 25kHz |
| Antenna Height | 1 m(10 m above sea level) |

## Ship station installation for Testing

The ship station of VDES was installed in Yacht “NEPTUNE” and the default settings of the ship station for sea trial are shown in the below table.

| Yacht “NEPTUNE” and antenna installation | VDES ship station installation inside the vessel |
| --- | --- |

| Ship Station Parameter Setting for Testing |  |
| --- | --- |
| Location of Shore Station | Yacht “Neptune” |
| RF Transmit Power | 12.5 W |
| Link ID for ASM and VDE | Link ID 7, Link ID 11 |
| Bandwidth | 25kHz |
| Antenna Height | 1 m(2 m above sea level) |

# TEST RESULT

## Test results for Ship to Shore communication of ASM

In order to measure the performance of ASM packet transmission/reception in the marine environment, the AIS packet was periodically transmitted from the ship station to the shore station, and the measured test results are as follows.

| Ship to Shore Sea Trial Results (Link ID 7) |  |
| --- | --- |
| Measured ASM Packet Error Ratio | 0.13 % |

## Test results for Ship to Shore communication of VDE

In order to measure the performance of VDE packet transmission/reception in the marine environment, VDE messages were periodically transmitted from the ship station to the shore station. The measured test results are as follows.

| Ship to Shore Sea Trial Results (Link ID 11) |  |
| --- | --- |
| Measured ASM Packet Error Ratio | 0.38% |

## Future test plan for VDES

### Test plan in 2019

The sea trial for the KEVSS-100 regarding overall function and performance is planned to be perform in Busan, Republic of Korea from October to November this year. The VDES shore station will be installed on the rooftop of the Korea Register head office building.

- Test Site

- Location of VDES Shore Station : Korean Register Head Office in Busan City, Republic of Korea

- Ship Station : Fishing vessel or Yacht (TBD)

- Test Methods

- ASM message transmission test (Shore to ship, Ship to Shore)

- VDE message transmission test (Shore to ship, Ship to Shore)

- PER Test (%)

- Data Rate Test (kbps)

- Coverage Test

- AIS Co-Existence

Figure 5 VDES Test Site in 2019

Figure 6 VDES Test Configuration

### Test plan in 2020

In 2020, It is planned to test the operational functionality and performance of the VDES communication for e-Navigation service data transmission through the VDES testbed containing VDES Test Center.

Figure 7 VDES Testbed in 2020

# Conclusion

With the support of Mokpo National Maritime University, The test was conducted for VDES Base Station equipment (KEVSS-100) developed under the Smart-Navigation Project for the first time in the real maritime environment. However, due to the test location and time constraints, simple message transmission and reception functions and its performances were verified for link ID 7 & 11 of ASM and VDE-TER at a short distance. In this regard, we could confirm the possibility of the VDES operability in the location where the AIS signal from the adjacent VTS and the multipath effect by the structure and the geography may exist.

We are planning to carry out sea trial on KEVSS-100 in Busan from October to November this year. It is expected that detailed test results of the function and performance of ASM and VDE-TER can be obtained and meaningful analysis may be possible. In this connection, it is our plan to provide detailed results of our VDES testing at sea for the next IALA ENAV meeting.

# References

IALA Guideline G1139 Ed.2, The Technical Specification of VDES.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain