Input paper: ENAV24-6.1.21

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM		□  ENG		□  PAP						X  Input

X  ENAV	□  VTS								□  Information

Agenda item 	(from agenda)			9

Workplan Task Number / Technical Domain 2	…………………………………

Working Group					WG 2

Author(s) / Submitter(s)				WTI(Wen Jie)

DNSA（Zhou Xuyao, Lv Xuwei）

# Summary of LTE Private Network for the Yangtze River Estuary E Navigation Construction Project

# Summary

In the Yangtze River Estuary E navigation construction project, the China MSA(Maritime Safety Administration) needs to supervise the ship in real time to ensure the safe navigation of the Yangtze River estuary waters. At present, the mobile public network in the waters of the Yangtze River estuary cannot meet the demand for use. The long-distance voice and video communication is an urgent problem to be solved at this stage. Through the construction of LTE dedicated network, the project aims to achieve signal coverage within 50 km of the Yangtze River estuary, which not only meets the low-speed transmission requirements of ship dynamic information, environmental hydrometeorology, but also meets the high-speed voice image of fixed stations and official ships. The project uses LTE 1.8G wireless private network to achieve long-distance coverage, and the frequency adopts 1785-1805MHZ. The main applications of the network include: 1. Fixed-point video transmission service; 2. Shipborne video transmission service; 3. Ship Internet access service.

# construction plan

## Overview of LTE network

The LTE private network in the Yangtze River estuary has built 6 LTE base stations in Dagushan Island, Huaniao Island, Sheshan Island, Jigu Reef, Hengsha Island and Wusongkou, thus achieving full coverage of wireless broadband signals in the Yangtze River estuary and the main channel. The system consists of three parts: control center, transport layer and access layer.

1.Control Center: consists of 1 set of core network, 1 set of network management system, and application server;

2.Transport layer: realized by wired fiber link and microwave transmission link;

3.Access layer: consists of 6 sets of base stations (the base station consists of two parts: BBU and RRU, each base station is composed of 1BBU+3RRU) and a terminal (CPE);

The overall networking structure is as follows:

Figure 1. Overall composition of the system

## Base station location

Base station information:

Figure 2. Base station location and base station distance

Table 1. Base station main information

| Site Name | Height | bandwidth（M） | Station Power（W） | Terminal power（W） | Terminal antenna height（m） | Theoretical cell coverage distance |
| --- | --- | --- | --- | --- | --- | --- |
| Dajishan | 93 | 10 | 20 | 0.2 | 6 | 43.8 |
| Hengsha | 57 | 10 | 20 | 0.2 | 6 | 34.2 |
| Huaniao | 100 | 10 | 20 | 0.2 | 6 | 43.8 |
| Jigu Reef | 52 | 10 | 20 | 0.2 | 6 | 34.2 |
| Sheshan | 50 | 10 | 20 | 0.2 | 6 | 34.2 |
| Wusongkou | 50 | 10 | 20 | 0.2 | 6 | 34.2 |

## Device installation

Figure 3. BBU unit installation

Figure 4. RRU unit installation

Figure 5. Antenna unit installation(Mobile operator tower)

Figure 6. Antenna unit installation(Radar tower)

# Coverage test

After the completion of the construction of the project, we conducted signal coverage tests in accordance with the channel of the Yangtze River Estuary. The test paths are as shown in the A, B, and C channels:

Figure 7. The main channel of the Yangtze River estuary

The main test content includes SINR(Signal to Interference plus Noise Ratio) and RSRP(Reference Signal Receiving Power)，The test results are as follows：

1. Test record along channel A (Hengsha-Shehan-Jigu reef direction)

| Location |  | RSRP | SINR |
| --- | --- | --- | --- |
| A1 |  | -114 | 3.39 |
| A2 |  | -101 | 6.71 |
| A3 |  | -84 | 16.68 |
| A4 |  | -85 | 18.2 |
| A5 |  | -86 | 16.16 |
| A6 |  | -93 | 12.53 |
| A7 |  | -97 | 6.63 |
| A8 |  | -103 | 4.91 |
| A9 |  | -105 | 5.6 |

2. Test record along channel B (Hengsha - north trough - Niupi Reef - Jigu reef direction)

| Location |  | RSRP | SINR |
| --- | --- | --- | --- |
| B1 |  | -75 | 25 |
| B2 |  | -88 | 22 |
| B3 |  | -90 | 20 |
| B4 |  | -95 | 18 |
| B5 |  | -88 | 30 |
| B6 |  | -85 | 28 |
| B7 |  | -83 | 25 |
| B8 |  | -72 | 30 |

Figure 8. Test Record of Channel A and Channel B

3. Test record along channel C (Dajishan-Huaniao-Jigu Reef direction)

| Location |  | RSRP | SINR |
| --- | --- | --- | --- |
| C1 |  | -121 | 0 |
| C2 |  | -116 | 5 |
| C3 |  | -95 | 18 |
| C4 |  | -90 | 27 |
| C5 |  | -75 | 30 |
| C6 |  | -88 | 26 |
| C7 |  | -82 | 28 |
| C8 |  | -88 | 24 |
| C9 |  | -86 | 28 |
| C10 |  | -75 | 30 |
| C11 |  | -81 | 28 |

Figure 9. Test Record of Channel C

Conduct wireless signal coverage test along channel A, The wireless signal strength of the channel is between -80db and -105db. Conduct wireless signal coverage test along channel A, The wireless signal strength of the channel is between -70db and -100db. Fixed point camera video can be transferred to the shore based management center. The ship that has installed the terminal can access the Internet through the access network, which fully meets the needs of various applications.

# Main application

1. Fixed point and shipborne video transmission service: In the area where wireless signal coverage has been achieved, the fixed point camera or the onboard camera accesses the LTE private network through the CPE, and the video picture is transmitted back to the monitoring hall;

Figure 10. Device connection (Fixed point and shipborne video transmission service)

Figure 11. Fixed point video information

Figure 11. Fixed point and shipborne  video information

2. Official ship Internet access service: When the official ship is in the wireless signal coverage waters, the handheld terminal and the computer access the LTE network by connecting the CPE to realize Internet access.

:

Figure 10. Device connection (Official ship Internet access service)

# Action requested of the Committee

1.This paper serves as a references  to the Initial Assessment of LTE and allows participants to know more about LTE technology.

2.This paper requires no action by the Committee.

Input document number, to be assigned by the Committee Secretary

Input papers should be assigned to a work task as listed in the Committee work plan which is available in  input papers. Leave open if uncertain but consider how the paper is to be processed if not relevant to a work task