Annex 5

Performance requirements for Hydrological and Meteorological Equipment in VTS

Table of Contents

1.	Introduction	2

1.1	Background	2

1.2	Scope	2

1.3	Objectives of Hydrological / Meteorological Equipment	2

2.	References	3

3.	Definitions	3

3.1	Abbreviations	3

4.	Functional Requirements	5

4.1	Sensors	5

4.2	Reliability	5

4.3	Malfunctions and Indicators	5

5.	Operational Requirements	6

5.1	Information Presentation	6

6.	Design and Installation	7

6.1	Durability and Resistance to Environmental conditions	7

6.2	Interference	7

6.3	Power Supply Requirements / Options	7

6.4	Installation	7

6.5	Maintenance	8

7.	Interfacing	9

8.	Backup Arrangements	9

9.	Safety Precautions	9

10.	Marking and Identification	9

11.	Documentation	9

# Introduction

In 1997 the IMO Maritime Safety Committee adopted Regulations for Vessel Traffic Services (VTS) that have since been included in SOLAS Chapter V (Safety of Navigation) as Regulation 12.This Regulation specifies the responsibilities of contracting governments to arrange for the establishment of VTS in certain vulnerable areas under their control.

## Background

The environmental systems in a VTS, also referred to as the hydrological / meteorological (hydro/meteo) systems, include sensors and readouts of various meterological or hydrographical variables.  Typical meteo variables are those provided by weather stations and include air temperature and humidity, wind velocity and direction, and visibility.   In certain locations, hydro variables such as tidal level, and current direction and velocity may be required.  This data may be obtained through sensors or available in tables/databases from national authorities.  Sensors providing this data, usually located at remote sites, communicate the variables to a Vessel Traffic Services Centre (VTS centre) via a telecommunications link.  At the VTS centre, graphical and/or numeric information is presented for use by the operators.

## Scope

The aim of Annex 5 of this recommendation is to:

- identify functional and operational requirements for hydrographical and meteorological equipment in VTS; and

- provide guidance on design and installation of such equipment.

## Objectives of Hydrological / Meteorological Equipment

Hydrological and meteorological information may be integrated into VTS applications to provide the operator a real-time assessment of the environmental situation in the VTS area of responsibility.  Information collected from this equipment can be provided to ships to assist in assessing the waterway conditions.

A number of countries operate tide gauges and current meters to assist the prediction of tidal heights and streams or for the broadcast of real-time information to shipping.  The Intergovernmental Oceanographic Commission (IOC) is responsible for co-ordinating the Global Sea Level Observing System (GLOSS) program to establish global and regional networks of sea level stations for providing essential information for international oceanographic research programmes.  GLOSS operates under the Global Ocean Observing System (GOOS) http://ioc.unesco.org/goos.  IALA supports and encourages participation in the GLOSS program.

# References

There are many applicable IMO, IEC, WMO and other requirements.  These include, but are not limited to:

NMEA 0183 – Standard for Marine Electronic Devices (IEC equivalent)

| IMO Resolution A.686(17) | Code on Alarms and Indicators (and MSC.39(63) Adoption of amendments to the Code on Alarms and Indicators) |
| --- | --- |
| IMO Resolution A.694(17) | General Requirements for Shipborne Radio Equipment forming Part of the Global Maritime Distress and Safety System (GMDSS) and for Electronic Navigational Aids |
| IMO | SOLAS (i.e. Chapter V, Regulation 12) |
| IEC 529 | Degrees of protection provided by enclosures (IP Code) |
| IEC 721-3-6 | Classification of environmental conditions |
| IEC 60945 | Maritime Navigation and Radiocommunication Equipment and Systems |
| WMO | International Meteorological Vocabulary Guide to Meteorological Instruments and methods of Observation |
| NMEA 0183 | Standard for Marine Electronic Devices (IEC equivalent) |

# Definitions

For general terms used throughout this document refer to the World Meteorological Organization (WMO).

## Abbreviations

GLOSS	Global Sea Level Observing System

GOOS	Global Ocean Observing System

IALA	International Association of Marine Aids to Navigation and Lighthouse Authorities

IEC	International Electro-Technical Commission

IMO	International Maritime Organisation

IOC	International Oceanographic Commission

IP	Ingress Protection

NMEA	National Marine Electronics Association

WMO	World Meteorological Organization

XML	Extensible Mark-up Language

# Functional Requirements

Briefly stated, the functional requirements state what the system should be able to do, the functions it should perform.

## Sensors

For hydro/meteo systems within a VTS, the measurement sensors should be installed and located according to recommendations from a VTS authority in consultation with hydrologist/meteorologist(s).  The Sensor identification and location should be provided.

The measurements/sensors may include:

- Wind speed / Wind direction / Wind gust

- Air temperature / Relative humidity

- Precipitation

- Barometric Pressure (atmospheric pressure)

- Visibility

- Water temperature / Water level / Salinity

- Height of tide

- Current speed (may be required at various depths)

- Current direction (may be required at various depths)

- Wave height / direction

- Ice coverage / thickness

- Ice coverage

## Reliability

The reliability, accuracy, range, resolution, and units of the measurements should satisfy the minimum requirements as determined by WMO.

## Malfunctions and Indicators

As a minimum, malfunctions, warnings, alarms and indicators should respond to the requirements of IMO Resolution A.686(17) and MSC.Circ.39(63).  Additional requirements may be required, depending on the individual type / purpose of sensor.

# Operational Requirements

Briefly stated, operational requirements are qualitative and quantitative parameters that specify the desired capabilities of a system and serve as a basis for determining operational effectiveness.

Operational Requirements may include ergonomics, operational controls and information presentation.  Due to the varied nature of hydrological and meteorological equipment in VTS, it is not possible to specify ergonomic or operational control requirements.

## Information Presentation

The results of the measurements should be transmitted in WMO standard units and displayed in user-selectable format.

The measurements should be available to the VTS operators through an integrated display or separate instruments.  Data may be presented numerically and/or graphically.  A log of the latest 24 hour measurements should be available to the VTS operators either numerically or graphically.

# Design and Installation

Key aspects to design and installation include:

- Durability and resistance to environmental conditions;

- Interference;

- Power supply requirements /options;

- Installation; and

- Maintenance.

## Durability and Resistance to Environmental conditions

Electronics installed externally should be in an environmental enclosure.  IEC requirements for environmental conditions should be applied as practicable.

## Interference

These sensors are industrial equipment and therefore comply with applicable international standards and regulations.  IEC requirements (IEC 60945) refer.

## Power Supply Requirements / Options

Relevant IEC requirements should be applied.  In remote locations, due to the low power consumption of hydro/meteo sensors, authorities should consider use of alternative power (e.g., solar panels, wind vanes, etc.), in lieu of generators, when commercial power is not available.

## Installation

Requirements concerning the installation of sensors, wiring and the arrangement of the equipment providing hydro/meteo information to the VTS centre should be determined.  Operational requirements will determine where sensors are to be located and how many are required.  Sites for sensors should be selected based upon optimizing data relevant to the VTS.  Other considerations include:

- availability of power,

- protection against vandalism,

- housing, and collocation with existing VTS, AtoN, or other suitable infrastructure

Relevant IEC requirements should be applied.  For example:

- IEC 529 "Degrees of protection provided by enclosures (IP Code)"

- IEC 721-3-6 "Classification of environmental conditions, Part 3: Classification of groups of environmental parameters and their severities; Ship environment"

- IEC 60945 "Maritime navigation and radiocommunication equipment and systems - General requirements, methods of testing and required test results"

## Maintenance

Possible requirements, in addition to IMO Assembly Resolution A.694(17) concerning maintenance, should be determined. Citing considerations for sensors should include maintenance, repair, and accessibility requirements.

# Interfacing

The data to be interfaced for the hydro/meteo service are described under ‘Functional Requirements / Sensors’.

For the interfacing of hydro/meteo services to VTS equipment, several different standards are in use.  Among those standards, the Standard for Marine Electronic Devices, NMEA 0183, has been applied for these applications.  In addition, the WMO has developed an interface standard for hydro/meteo applications.

For the interface between a VTS and its users, hydro/meteo data should follow standardized data exchange formats, e.g., XML.  (In addition, a time stamp and source should be provided.

# Backup Arrangements

Depending on the individual type of the equipment, requirements concerning back-up and fall-back arrangements should be determined.

# Safety Precautions

Depending on the individual type of the equipment, requirements in addition to IMO Resolution A.694(17) should be determined.

# Marking and Identification

Depending on the individual type of the equipment, requirements in addition to IMO Resolution A.694(17) should be determined.  (e.g. for the identification of software)

# Documentation

Documentation should be provided in accordance with the IALA VTS Manual.