FROM:  The IALA e-Navigation Committee (e-NAV1, Sept 2006)

To:   The IALA VTS Committee (VTS25, March 2007)

THE INTERFACING OF ‘OLDER’ GNSS UNITS

The following is an extract from the report of the IALA E-Navigation Committee’s first meeting (E-NAV1).

------------------------------------------------------------------------------------------------------

## Interfacing older GNSS units with AIS

Relevant input documents: e-NAV1/9/1, e-NAV1/9/2

Regarding e-NAV1/09/01, the Committee recognizes the unresolved issue of integrity monitoring – RAIM – in legacy GNSS units, a 'latent pathogen' for AIS.  Contrary to ships with newer GNSS' which will isolate a faulty satellite, calculate, and transmit a correct position (as long as sufficient satellites are visible); older receivers may rely upon faulty satellites, and, thus transmit an inaccurate position.  Unless the OOW is vigilant he/she will be unaware that incorrect own ship’s positions are being transmitted.  Similarly, a VTS operator may assume the ship’s position is accurate, when it may not be; particularly if the VTS operator’s system does not highlight the AIS RAIM flag and/or AIS position accuracy.  The e-Navigation Committee wishes to bring the latter to the attention of the VTS Committee, as a potential matter for training and design of VTS displays and workstations.  In addition, the United States, Australia, United Kingdom and Sweden, agreed to bring this matter to IMO’s attention via a request for the promulgation of a Safety of Navigation Circular.

Reference is made to Section 9.4 of this report.

Action Item

The Secretariat was asked to forward the issue of the interfacing of older GNSS units to the attention of the VTS Committee.

-------------------------------

## Draft IMO Safety of Navigation Circular regarding interfacing older GNSS units with AIS.

The Committee was charged with developing the draft for an IMO Safety of Navigation Circular regarding interfacing older GNSS units with AIS for national administrations to submit to IMO.  This task was completed and approved by the Committee (e-NAV1/output/06).  J Arroyo of the USCG indicated that, with additional development work, the USA would sponsor this document at IMO, and would seek support from UK, Australia, Sweden and other administrations.  Reference is made to Section 10.1 of this report.

Action Item:

Jorge Arroyo of the USCG is requested to carry out additional work in developing the draft of an IMO Safety of Navigation Circular and then arrange for it to be co-sponsored for submission to IMO NAV.